3GPP TSG RAN WG1 Meeting #103-e		                                                      R1-2007800

e-Meeting, October 26th – November 13th, 2020

Source:	CATT

Title:	Discussion on the LS from RAN2 on sidelink configured grant handling

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

In LS R1-2007522 (R2-2008586) [1], RAN2 asked RAN1 the following question and except feedback on it.

- Is it possible to use the retransmission opportunities for initial transmission for a sidelink configured grant in case when the data was not available for the transmission opportunity for initial transmission?

In this contribution, how to handle initial transmission using retransmission resources is discussed.

Discussion

For a configured grant, the transmission and re-transmission resources can be allocated through configured grant and dynamic grant. A TB can be allocated with multiple resources for its initial transmission and re-transmissions, and the maximum number of re-transmission is configured by higher layer. As it was agreed, a TB cannot be transmitted on the resource that is allocated for other TB’s transmissions. It is possible that the data is not available yet when it is the time for its initial transmission. The initial transmission of the TB can use the following re-transmission opportunity that is indicated for this TB’s re-transmission. For example, a configured grant configures 3 transmission opportunities for each TB within a period. The data is not available at the initial transmission opportunity, and the TB can be transmitted by using the second or third transmission opportunity as its initial transmission whenever this data is available.

Proposal 1: For a sidelink configured grant, the initial transmission of a TB can use one of this TB’s re-transmission opportunities whenever the data is available for transmission.

Conclusion

In this contribution, the question in the LS from RAN2 to RAN1 is discussed, and the following proposals can be concluded on whether a TB’s re-transmission opportunity can be used for its initial transmission if the initial transmission opportunity is missed since unavailable data.

Proposal 1: For a sidelink configured grant, the initial transmission of a TB can use one of this TB’s re-transmission opportunities whenever the data is available for transmission.

References

  - R1-2007522, “LS to RAN1 on sidelink configured grant handling”, 3GPP TSG RAN WG1 #103-e e-meeting, October 26th – November 28th, 2020.