3GPP TSG RAN WG1 #103-e											R1-2008285

e-Meeting, October 26th – November 13th, 2020

Source:			OPPO

Title:			Discussion on multi-cell PDSCH scheduling via a single DCI

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

Based on discussions in [102-e-NR-DSS-DC_enh2-01] email thread, following agreements were made in RAN1#102-e for the study on PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI

Agreements:

- For the study on single DCI scheduling PDSCH on two cells

  - Consider the following scenarios as baseline for evaluation

    - UE configured with Inter-band CA with PCell and an SCell

      - PCell for the UE is operated on a DSS carrier (i.e.,  same carrier is also used for serving LTE users)

      - Case 1: Different SCS for PCell and SCell

      - Case 2: Same SCS for PCell and Scell

  - Additional scenarios can also be evaluated, e.g. as below

    - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers)

    - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers)

    - Note: other combinations not precluded

Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

If one DCI can schedule two or more cells, overhead reduction of PDCCH can be expected. In this contribution, we evaluate and analyze single DCI scheduling PDSCH on two cells performance.

Discussion

Carrier aggregation provides large bandwidth to support high data rates. However, PDCCH capacity becomes a bottleneck for data transmission due to limited control region in some scenarios, e.g. LTE carrier in dynamic spectrum sharing (DSS). In addition, large PDCCH overhead also impacts physical resource for data transmission. Therefore, PDCCH overhead reduction is a meaningful technical. One-to-two scheduling is a direct solution.

One-to-two scheduling scheme

Currently, one DCI can schedule a single PDSCH only, shown in Figure 1 (left). One-to-two scheduling, to be specifically, one DCI schedules two PDSCHs in two carriers, is shown in Figure 1 (right). To minimize DCI overhead in one-to-two scheduling, some DCI fields for two PDSCHs will be shared and correspondingly, scheduling will be restricted to some extent, e.g. the same MCS level, time domain resource, PUCCH resource and so on.

Figure 1 NR scheduling schemes

System-level Evaluation

Generally, PDCCH overhead reduction will increase throughput due to more resource can be used for PDSCH. However, scheduling restriction may decrease spectrum efficiency. So in system-level evaluation, taking the above parameters into account, we evaluate gain from one-to-two scheduling.

In system level evaluation, two schemes are evaluated.

- Scheme1 (baseline): One PDCCH schedules one PDSCH in any carrier. DCI size including CRC 82bits is used.

- Scheme2: One PDCCH schedules one or two PDSCHs. If two PDSCHs in two carriers are allocated for one UE, one PDCCH is used. Otherwise, one PDCCH schedules one PDSCH. MCS information and time domain resource are the same for two PDSCHs scheduled by one PDCCH. DCI size including CRC 82bits is used for one-to-one scheduling, and 130 bits is used for one-to-two scheduling.

The details simulation assumptions are summarized in Table A-1. From simulation result in Figure2, we could see that scheme 2 has 13.5% throughput gain due to PDCCH overhead reduction.

Figure 2 System level simulation result for SE

Observation 1: One-to-two scheduling has 13.5% throughput gain due to PDCCH overhead reduction.

Link-level Evaluation

For one DCI scheduling two PDSCHs, larger DCI size is benefit for more flexible scheduling. However, larger DCI size need to configure larger aggregation level to meet reliability requirement, which will neutralize the gain from DCI number decrease. Therefore, a new DCI design is the trade-off between the flexibility of scheduling and PDCCH overhead reduction.

SINR gap between adjacent aggregation levels is larger than 3dB, it means that DCI size variation range between adjacent aggregation levels is not small. Figure 3 shows different DCI size cases. Taking DCI size including CRC 130 bits as an example, the SINR for BLER=1% is around -3.6 dB for AL =16, while for reference DCI size including CRC, i.e. 82bits, the SINR for BLER=1% is around -2 dB for AL =8. So for DCI size including CRC 130bits, gain from PDCCH overhead reduction can be achieved in SINR range (-3.6dB,-2dB).To be specific, the gain from one-to-two scheduling can be achieved when a DCI with 130bits @ AL=16 replaces two DCIs with 82bit @ AL=16. However, when a DCI with 130bits @ AL=16 or two DCIs with 82bit @ AL=8 can work, there is no gain from PDCCH overhead reduction. So we define SINR range for gain is (-3.6dB,-2dB).

Figure 3 PDCCH performance

Assuming that SINR distribution for UE is uniform. For new DCI size including CRC 130bits, SINR range size for gain is around 50% SINR gap between adjacent aggregation levels, so it can be seen that around 50% UE could achieve the gain from one-to-two scheduling. For other DCI size cases, the ratio of UE achieving gain from one-to-two scheduling is listed in Table 1. From Table 1, we could see that the smaller DCI size increases, the more UEs achieve gain from one-to-two scheduling

Table 1. Ratio for UE achieving gain from one-to-two scheduling

| New DCI size(bits) | Ratio for UE achieving gain from one-to-two scheduling |
| --- | --- |
| 92 | 90% |
| 106 | 70% |
| 116 | 60% |
| 130 | 50% |

Observation 2: The smaller DCI size increases, the more UEs achieve gain from one-to-two scheduling.

Conclusions

In this contribution, we show our views on multi-cell PDSCH scheduling via a single DCI enhancement with following observations:

Observation 1: One-to-two scheduling has 13.5% throughput gain due to PDCCH overhead reduction.

Observation 2: The smaller DCI size increases, the more UEs achieve gain from one-to-two scheduling.

Appendix

Table A-1. System-level simulation scenario

| parameters | assumption |
| --- | --- |
| Carrier frequency | 700/800 MHz |
| Simulation bandwdith | 10+10 MHz |
| UE number | 10+10 |
| Antenna configuration at TRxP (M, N, P, Mg, Ng; Mp, Np) | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1;1,1) (dH,dV) = (0.5, 0.8)λ |
| Antenna configuration at UE | (M,N,P,Mg,Ng; Mp,Np) = (2,1,1,1,1; 1,1) (dH,dV) = (0.5, 0.8)λ |
| TRxP number per site | 3 |
| Transmit power per TRxP | 46 dBm |
| ChannelModel | Channel3DMIMO |
| Traffic | FullBuffer |
| CSIFeedbackPeriod | 5slot |
| Scenario | UMA |
| ISD | 500 |
| UENoiseFigure | 7 |

Table A-2. Link-level simulation scenario

| parameters | assumption |
| --- | --- |
| DCI Size(including 24bits CRC) | 82/92/106/116/130 |
| Carrier frequency | 4 GHz |
| SCS | 15kHz |
| PDCCH symbols | 3 |
| channel model | TDL-B DS=100ns |
| number of BS antennas | 2Tx |
| number of UE antennas | 2Tx |
| Transmission type | interleaved |
| bundle size | 6 |
| modulation | QPSK |
| channel coding | polar code |
| UE speed | 3km/h |

References

- RP-193260 New WID on NR Dynamic spectrum sharing (DSS)

- R1-2006064 Multi-cell PDSCH scheduling via a single DCI

- R1-2007442 Summary of NR Dynamic spectrum sharing (DSS) in Email discussion [102-e-NR-DSS-DC_enh2-01]