3GPP TSG RAN WG1 Meeting #103-e	R1-2008390

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Remaining issues on synchronization mechanism for NR sidelink

Agenda Item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this document, we share our views on remaining issues on synchronization mechanism for NR sidelink.

Discussion

Indication of the non-TDD case in sl-TDD-Config

In RAN1#102-e meeting, changes to TS 38.213 were approved in R1-2007454 [2], including the following for SL synchronization procedures:

## *** Unchanged text is omitted *** ## 16.1 Synchronization procedures ## *** Unchanged text is omitted *** For transmission of an S-SS/PSBCH block, a UE includes a bit sequence a0, a1, a2, a3, …, a11 in the PSBCH payload to indicate sl-TDD-Config-r16 and provide a slot format over a number of slots. For paired spectrum, or if tdd-UL-DL-ConfigurationCommon and sl-TDD-Configuration-r16 are not provided for a spectrum indicated with only PC5 interface in Table 5.2E.1-1 in [TS 38.101-1], - a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11 are set to ‘1’; else - a0=0 if pattern1 is provided by tdd-UL-DL-ConfigurationCommon; a0=1 if both pattern1 and pattern2 are provided by tdd-UL-DL-ConfigurationCommon as described in Clause 11.1 ## *** Unchanged text is omitted ***

It seems the case of shared SL carrier and SUL carrier is still not correctly specified, i.e. when tdd-UL-DL-ConfigurationCommon is provided in a cell configured with a SUL carrier, and spectrum is shared between the SL carrier and the SUL carrier. In our view this case should be treated in the same way as other “non-TDD” cases.

Proposal 1: In case of shared SL carrier and SUL carrier, the sl-TDD-Config bits are set as follows,

- a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11=(1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1).

Conclusion

In this contribution, we discuss remaining issues on synchronization for NR sidelink, and make the following proposal:

Proposal 1: In case of shared SL carrier and SUL carrier, the sl-TDD-Config bits are set as follows,

- a0, a1, a2, a3,a4, a5, a6, a7,a8, a9, a10, a11=(1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1).

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.

R1-2007454, “Corrections on Sidelink”, Samsung, 3GPP RAN1#102-e.