3GPP TSG RAN WG1 #103-e			R1-2008465

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.4.1

Source:	Apple

Title:	Timing Relationship Enhancement in NTN

Document for:	Discussion/Decision

# Introduction

One of the RAN1-lead objectives in the NTN WID [1] is timing relationship enhancements for NTN. The timing relationship enhancement is related to introducing an additional time offset to compensate the large propagation delay in NTN. A list of topics regarding timing relationship enhancements was discussed [2]. The following agreements have been made in RAN1 #102-e meeting [3].

Agreement:

- Introduce K_offset to enhance the following timing relationships:

  - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH).

  - The transmission timing of RAR grant scheduled PUSCH.

  - The transmission timing of HARQ-ACK on PUCCH.

  - The CSI reference resource timing.

  - The transmission timing of aperiodic SRS.

- Note: Additional timing relationships that require K_offset of the same or different values can be further identified.

Agreement:

For Koffset used in initial access, the information of Koffset is carried in system information.

- FFS implicit and/or explicit signaling of Koffset in system information.

- FFS a cell specific Koffset value used in all beams of a cell and/or each beam in a cell uses a beam-specific Koffset value.

- FFS whether/how to update Koffset after initial access.

In this contribution, we provide our views on the topics of the additional timing relationships that require Koffset, as well as the details of Koffset in initial access and after initial access.

# Discussion

It was agreed [3] that Koffset used in initial access is carried in system information. It is open whether the Koffset is implicitly or explicitly signaled in system information. The explicit signaling of Koffset via system information is preferred.

It is open whether the Koffset is cell specific or beam specific. It is agreed [3] that both one-beam per cell and multiple-beam per cell are supported in existing NR specifications and are baseline for NR NTN. If beam specific Koffset is carried in system information, then the signaling overhead for the system information is large in the multiple-beam per cell deployment. In other words, the system information will carry multiple Koffset, one per beam. Hence, we prefer a cell specific Koffset used in initial access.

Proposal 1: A cell specific Koffset is used in initial access. This cell specific Koffset is explicitly indicated in system information.

The time offset Koffset is introduced for UE’s downlink-uplink timing interaction, which is aimed to compensate the large propagation delay in NTN. Due to large cell size, the propagation delay of UEs in a NTN cell can have large variation. For example, the maximum differential delay in a GEO-based cell is up to 10.3 ms [4]. The cell specific time offset Koffset used in initial access is not accurate for each individual UE. Considering the maximum value of K1 or K2 in NR is 15 slots or 32 slots, a cell specific time offset Koffset used together with K1 and K2 is unable to match the maximum differential delay. Hence, we prefer to introduce a UE specific or a beam specific time offset Koffset after initial access.

Proposal 2: A UE specific or a beam specific Koffset is used after initial access.

A UE specific or a beam specific Koffset may need to be updated, e.g., due to satellite movement. In NGEO scenarios, the timing drift rate is up to 40 s/s. For example, the timing advance value changes 1 ms every 25 seconds. Considering that maximum value of K1 or K2 is 15 or 32 slots respectively, a UE specific or a beam specific Koffset needs to be updated frequently in order to ensure K1 or K2, together with Koffset, is able to indicate a proper uplink slot. The signaling overhead is large in case a dedicated RRC message is used to update UE specific Koffset. Hence, we have the following proposal.

Proposal 3: RAN1 strives to reduce the signaling overhead of updating Koffset.

In terrestrial network, the MAC CE activation timing is the first slot after slot n+3Nslotsubframe,µ, where UE transmits HARQ-ACK for the MAC CE command in UL slot n. In general, the time gap between an UL slot and a DL slot of the same slot index is equal to timing advance (TA). Since the TA value is relatively small in terrestrial network, there is no big difference of activating MAC CE at UL slot n+3Nslotsubframe,µ or at DL slot n+3Nslotsubframe,µ.

However, the TA value is large in NTN. Based on the extensive discussions on MAC CE activation timing in RAN1 #102-e meeting [2], it seems beneficial to consider the MAC CE activation timing separately for UL MAC CE command and DL MAC CE command.

For an UL MAC CE command, a UE sends HARQ-ACK for the MAC CE command in UL slot and activates the MAC CE command also in UL slot. There is no uplink-downlink timing interaction. Hence, the legacy 3 ms gap is re-used for the UE to process the MAC CE command. Here, the time offset Koffset is not applied.

Proposal 4: On MAC CE timing relationship, UE assumes UL MAC CE command is activated 3 ms after it transmits HARQ-ACK corresponding to a received PDSCH carrying the UL MAC CE command.

For a DL MAC CE command, a UE sends HARQ-ACK for the MAC CE command in UL slot and activates the MAC CE command in DL slot. Hence, there exists an uplink-downlink timing interaction, in which the time offset Koffset plays a role. Specifically, for DL MAC CE timing relationship, UE assumes MAC CE command is activated Y ms after it transmits HARQ-ACK corresponding to a received PDSCH carrying the DL MAC CE command. The value Y is equal to Koffset'+X, where Koffset' is the absolute time converted from Koffset and X is less than 3 ms corresponding to gNB’s processing time for activating the MAC CE command.

Proposal 5: On MAC CE timing relationship, UE assumes DL MAC CE command is activated Y ms after it transmits HARQ-ACK corresponding to a received PDSCH carrying the DL MAC CE command. The value Y is equal to Koffset'+X, where K'offset is the absolute time converted from Koffset and X is less than 3 ms.

Type 1 configured grant is configured and activated by RRC (i.e., ConfiguredGrantConfig). In type 1 configured grant, the N-th uplink grant occurs in the symbol for which

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =

(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot).

In the formula, the time reference SFN is based on the downlink timing. Considering the downlink-uplink timing interaction, we think Koffset should be introduced to the timing relationship for type 1 configured grant. It can be further studied whether Koffset is contained in the type 1 configured grant configuration, or whether Koffset is additionally applied to type 1 configured grant.

Proposal 6: Introduce Koffset to the timing relationship for type 1 configured grant.

# Conclusion

In this contribution, we provided our views on timing relationship enhancement for NTN. Our proposals are as follows:

Proposal 1: A cell specific Koffset is used in initial access. This cell specific Koffset is explicitly indicated in system information.

Proposal 2: A UE specific or a beam specific Koffset is used after initial access.

Proposal 3: RAN1 strives to reduce the signaling overhead of updating Koffset.

Proposal 4: On MAC CE timing relationship, UE assumes UL MAC CE command is activated 3 ms after it transmits HARQ-ACK corresponding to a received PDSCH carrying the UL MAC CE command.

Proposal 5: On MAC CE timing relationship, UE assumes DL MAC CE command is activated Y ms after it transmits HARQ-ACK corresponding to a received PDSCH carrying the DL MAC CE command. The value Y is equal to Koffset'+X, where K'offset is the absolute time converted from Koffset and X is less than 3 ms.

Proposal 6: Introduce Koffset to the timing relationship for type 1 configured grant.

# References

- RP-201256, Solutions for NR to support non-terrestrial networks (NTN), Jun. 2020.

- R1-2007074, “Feature lead summary on timing relationship enhancements,” Aug. 2020.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #102-e Meeting, Aug. 2020.

- 3GPP TR 38.821, Solution for NR to support non terrestrial networks (NTN), v. 16.2.0, Dec. 2019.