3GPP TSG-RAN WG1 e-Meeting #103	R1- 2008722

November 2nd-13th, 2020

Title:	Discussion on Timing Relationship Enhancements for NTN

Source: 	Fraunhofer IIS, Fraunhofer HHI

Agenda Item:	8.4.1 Timing relationship enhancements

TDoc Type:	Discussion

Document for:	Decision

# Introduction

In RAN1#102-e, several agreements were achieved regarding the agenda item (AI) 8.4.1, timing relationship enhancement. The summary of the achieved agreements is listed below:

Agreement:

- Introduce K_offset to enhance the following timing relationships:

  - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH).

  - The transmission timing of RAR grant scheduled PUSCH.

  - The transmission timing of HARQ-ACK on PUCCH.

  - The CSI reference resource timing.

  - The transmission timing of aperiodic SRS.

- Note: Additional timing relationships that require K_offset of the same or different values can be further identified.

Agreement:

For K_offset used in initial access, the information of K_offset is carried in system information.

- FFS implicit and/or explicit signaling of K_offset in system information.

- FFS a cell specific K_offset value used in all beams of a cell and/or each beam in a cell uses a beam-specific K_offset value.

- FFS whether/how to update K_offset after initial access.

# Discussion

One of the main outcomes of the previous meeting was the decision of RAN1 to distinguish between the configuration of Koffset in initial access and the potential update of Koffset after initial access. Furthermore, it was decided that the information of  Koffset is conveyed in system information for the initial access.

## Initial Access

The value of Koffset used in initial access is either cell specific or beam specific. The details is discussed in subsection 2.3. However, irrespective of cell specific or beam specific design of Koffset, due to the reason that the information of Koffset is carried in system information, the value of Koffset requires to be the same for all UEs in the cell or at least in the beam if cell specific design or beam specific design is adopted, respectively.

It is clear that for each individual UE, the corresponding value of Koffset, if UE specific signaling was adopted, is closely related to the value of timing advance (TA) applied by the UE, which in turn depends on the experienced round trip delay (RTD) of the UE (w.r.t the reference point used for the TA calculation). However, since for initial access, as mentioned above, the value of Koffset must be the same for all UEs, rather conservative approach should be adopted for the value of Koffset. In particular, the value of Koffset shall not be smaller than the maximum RTD of the cell/beam. Here, we assume the reference point for TA calculation to be at gNB, see Fig. 1.

Observation 1: The minimum configurable value of Koffset used in initial access shall not be smaller than the maximum RTD of the serving cell for cell specific signaling and the maximum RTD of the serving beam for beam specific signaling.

Figure 1: Initial Koffset used in PUSCH scheduled by random access response

In addition to the value of Koffset, it is important to consider a degree of flexibility for the network to choose the value of the Koffset in initial access, for instance to select an arbitrary value greater than or equal to the maximum RTD of cell/beam. Having the above mentioned degree of freedom has several advantages. This includes, among others, introducing a unified signaling framework with respect to the value of TA applied by each UE, i.e., full TA or differential TA. In other words, irrespective of the mechanism adopted by UE to calculate its TA for uplink (UL) timing synchronization, i.e., autonomous or non-autonomous acquisition of TA, and whether UE acquires full TA or differential TA, the value of Koffset can be adopted independently by the network. Another important advantage of the flexible choice of Koffset is that it facilitates the signaling of the value of Koffset in system information, as it is discussed in subsection 2.2. We note that adopting values of Koffset larger than the maximum RTD of cell/beam may increase the delay of UEs performing random access procedure. However, this delay should not be of much concern for initial access procedure.

Observation 2: It is beneficial with respect to unified signaling framework design, i.e.,  to support both full TA or differential TA, to leave the particular choice of the value of Koffset to the network/gNB.

Proposal 1: It must be left to gNB/network to select a value of Koffset greater than or equal to the maximum RTD of cell or beam depending on cell specific or beam specific signaling.

Moreover, we recommend to adopt the unit of the value of Koffset in millisecond (and not in number of slot such as K1 and K2). Then, Koffset value can be translated by the gNB and the UE to the number of slots according to the numerology in use.

Proposal 2: RAN1 to adopt millisecond as the unit of the Koffset.

## Implicit Versus Explicit Signaling

In this subsection, we discuss the implicit signaling and explicit signaling of Koffset. As highlighted in FL summary in [3], explicit signaling of Koffset is clean and forward compatible. However, it was mentioned by several companies that explicit signaling may lead to redundancy or duplicate signaling. In the following, we propose several methods for calculation of Koffset from other system information parameters as potential candidates for implicit signaling of Koffset.

The rationale behind implicit signaling is to obtain a value of Koffset from other system information parameters broadcast in, e.g., SIB1. As mentioned above, the value of Koffset is tightly related to the maximum RTD of the cell/beam. So, let us first take a look at the typical values of maximum RTD in NTN. It is stated in TR 38.821 [2] that the maximum satellite beam size can reach up to 3500 km and 1000 km for GEO and LEO, respectively, which results in maximum differential delay of 10.3 ms for GEO and 3.2 ms for LEO within a satellite beam. Considering a cell with one beam, the corresponding values of differential RTD for different UEs may differ up to 20.6 ms for GEO and 6.4 ms for LEO. In FL summary [3], the differential RTDs are translated in terms of slots as follows:

- 20.6 ms is equivalent to 20.6 / 51.2 / 102.4 / 204.8 slots for SCS 15 / 30 / 60 / 120 kHz, respectively.

- 6.4 ms is equivalent to 6.4 / 12.8 / 25.6 / 51.2 slots for SCS 15 / 30 / 60 / 120 kHz, respectively.

Common values of RTD must be added to the mentioned values above to obtain absolute RTD values. The absolute values of RTD are reported in TR 38.821 [2] as follows: For transparent GEO the absolute RTD can be up to 541.46 ms and for transparent LEO (600 Km) up to 25.77 ms.

In the broadcast system information, e.g., in SIB1, there are some parameters that are related to RTD, in that, their corresponding values are adopted taking the experienced RTD into account. These parameters can be potentially exploited for deriving a value of Koffset in NTN as well. In principle, two different approaches can be adopted for implicit signaling design. In the first approach, the parameter(s) of interest capture the differential value of RTD in NTN. In the first approach, however, the common part of RTD must be retrieved from other broadcast system information parameters as well. In the second approach, the parameter(s) of interest, in the broadcast system information, capture the absolute value of RTD in NTN.

- First Design Approach:

Given the progress in RAN2 meetings w.r.t MAC aspects in NTN, and in particular TA, the majority of the companies agreed on broadcasting of the information related to common delay (e.g. feeder link delay) or common delay from gNB to cell/beam reference point via network, see [6]. Furthermore, it was agreed in the last RAN2 meeting [7] that an offset is added to the start of the parameter ra-ContentionResolutionTimer for NTN. Several companies suggested that the same offset which is broadcast via network for indication of common delay can also be adopted here.

In the following, we use the variable NToffset to refer to either a) the common/minimum RTD obtained from common/minimum delay broadcast via network or b) the offset to the start of the parameter ra-ContentionResolutionTimer. Furthermore, ra-ContentionResolutionTimer is a parameter configured within RACH-ConfigCommon parameter structure which is broadcast via SIB1. In particular, ra-ContentionResolutionTimer takes values in the range of {8, 16, 24, 32, 40, 48, 56, 64} subframe [ms] which, in turn, captures the range of the values of differential RTDs in NTN. As a result, ra-ContentionResolutionTimer  together with NToffset can be employed for deriving a value for the initial Koffset as follows

Koffset=RAConResTimer+ NToffset,

where RAConResTimer is the specific value of ra-ContentionResolutionTimer broadcast in SIB1. Here, for simplicity, we assumed that all parameters have the unit of millisecond.

Observation 3: NTN UE acquires ra-ContentionResolutionTimer and the value of common/minimum RTD via broadcast system information before performing random access procedure.

Proposal 3: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., ra-ContentionResolutionTimer and an offset to the start of ra-ContentionResolutionTimer or common/minimum delay.

Figure 2: Implicit signaling of initial Koffset based on system information parameters, e.g. common RTD.

- Second Design Approach:

As stated above, an alternative design approach can also be adopted which is based on system information parameter(s) that capture the range of the values of absolute RTDs in NTN. An example of such parameters includes RRC timers T300, T301, T319, and T310. The RRC timers are configured within system information parameter ue-TimerAndConstants and broadcast via SIB1. In particular, the RRC timers take the following range of values:

T300/T301/T319 = { 100, 200, 300, 400, 600, 1000, 1500, 2000 }  ms, T310 = { 0, 50, 100, 200, 500, 1000, 2000 } ms. We can observe that the RRC timers can provide values greater than maximum experienced RTD in GEO and LEO satellites, i.e., 541.46 and 25.77, respectively. As a result of this, the RRC timers can also be exploited for derivation of the value of initial Koffset as follows

Koffset=T3xx,

where T3xx is the certain value of RRC timer T3XX broadcast in SIB1, XX = {00,01,19,10}.

Observation 4: NTN UE acquires RRC timers T300, T301, T319, and T310 before performing random access procedure.

Proposal 4: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., RRC timers T300, T301, T319, and T310.

Observation 5: RRC timers T300/301/319 are most suited for GEO satellites, while RRC timer T310 is suited for LEO scenario.

## Cell Specific Versus Beam Specific Signaling

In this subsection, we discuss the cell specific and beam specific signaling of Koffset. Taking the number of the beams that may be supported by an NTN cell into account (e.g. up to 64 beams in FR2), broadcasting many Koffset values in system information per beam increases the signaling overhead.

Observation 6: Beam specific signaling of initial Koffset increases the signaling overhead.

Proposal 5: RAN1 to consider cell specific signaling of initial Koffset.

## Updating Initial Koffset

Updating initial Koffset after RRC connection is very crucial, as using the initial Koffset impacts the overall system delay and average throughput of UEs. Taking this into account, two different approaches can be assumed for updating initial Koffset with respect to the number of targeting UEs, UE common and UE specific update of Koffset. For UE common update of Koffset, it was proposed by several companies that, e.g., for all UEs located in one beam, the value of Koffset be updated for the corresponding beam. Irrespective of the signaling mechanism, given the maximum satellite beam size, i.e., 3500 Km for GEO and 1000 Km for LEO, differential RTD of 20.6 ms for GEO and 6.4 ms for LEO is observed between different UEs located in one beam. Given that the updated value of Koffset shall be greater than or equal to the maximum RTD of the beam (for UE common Koffset updating in one beam), the updated value of Koffset impacts the overall system delay and reduces the average throughput of UEs that are experiencing RTDs much smaller than the maximum RTD of the beam. Furthermore, it was shown in [4] that values of Koffset greater than UE TA would make a set of slots equal to the differential delay of the UE non-schedulable, see Fig. 2 of [4].  As a result, UE specific update of Koffset is preferable with respect to minimization of system delay and maximization of individual UE average throughput.

Observation 7: UE specific update of Koffset  results in minimum system delay and maximum UE average throughput.

Proposal 6: The value of Koffset should be updated/reconfigure after RRC connection in UE specific manner.

We have shown in our previous contribution [5] that UE specific update of Koffset can be performed according to the value of TA for the corresponding UE. Given the progress made in AI 8.4.2, UEs with GNSS capability can autonomously acquire full TA based on their location and satellite ephemeris information. In order to be able to update Koffset in UE specific manner, the acquired TA of the UE should be reported to gNB.

Proposal 7: For UE specific update of Koffset, NTN UE should report its acquired TA to gNB.

In particular, we propose that UE reports its first TA report as part of MSG3 in RACH procedure. Details related to UE TA report can be further studied.

Proposal 8: NTN UE should report its first TA report as part of MSG3.

Proposal 9: RAN1 to further study the details of NTN UE TA report.

# Others

It was agreed in the previous RAN2 meeting that both 2-step and 4-step RACH are supported in Rel-17 NTN [7]. As a result of this, we recommend that RAN1 also considers 2-step RACH timing relationship enhancement. Details of the enhancement can be for further study.

Proposal 10: RAN1 to further study the enhancement of 2-step RACH timing relationship for NTN.

# Summary

Observation 1: The minimum configurable value of Koffset used in initial access shall not be smaller than the maximum RTD of the serving cell for cell specific signaling and the maximum RTD of the serving beam for beam specific signaling.

Observation 2: It is beneficial with respect to unified signaling framework design, i.e.,  to support both full TA or differential TA, to leave the particular choice of the value of Koffset to the network/gNB.

Observation 3: NTN UE acquires ra-ContentionResolutionTimer and the value of common/minimum RTD via broadcast system information before performing random access procedure.

Observation 4: NTN UE acquires RRC timers T300, T301, T319, and T310 before performing random access procedure.

Observation 5: RRC timers T300/301/319 are most suited for GEO satellites, while RRC timer T310 is suited for LEO scenario.

Observation 6: Beam specific signaling of initial Koffset increases the signaling overhead.

Observation 7: UE specific update of Koffset  results in minimum system delay and maximum UE average throughput.

Proposal 1: It must be left to gNB/network to select a value of Koffset greater than or equal to the maximum RTD of cell or beam depending on cell specific or beam specific signaling.

Proposal 2: RAN1 to adopt millisecond as the unit of the Koffset.

Proposal 3: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., ra-ContentionResolutionTimer and an offset to the start of ra-ContentionResolutionTimer or common/minimum delay.

Proposal 4: NTN UE should derive the initial value of Koffset from the broadcast system information, e.g., RRC timers T300, T301, T319, and T310.

Proposal 5: RAN1 to consider cell specific signaling of initial Koffset.

Proposal 6: The value of Koffset should be updated/reconfigure after RRC connection in UE specific manner.

Proposal 7: For UE specific update of Koffset, NTN UE should report its acquired TA to gNB.

Proposal 8: NTN UE should report its first TA report as part of MSG3.

Proposal 9: RAN1 to further study the details of NTN UE TA report.

Proposal 10: RAN1 to further study the enhancement of 2-step RACH timing relationship for NTN.

# References

RP-193234, “3GPPWork Item Description: Solutions for NR to support non-terrestrial networks (NTN)

3GPP TR 38.821 v16.0.0 (2019-12): 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) (Release 16)

R1-2007323, “Feature lead summary#4 on timing relationship enhancements”, Meeting #102-e

R1-2006464, “On basic assumptions and timing relationship enhancements for NTN”, 3GPP TSG RAN1 #102e, August 2020.

R1- 2005548, “NR-NTN: Timing Relationship Enhancements” 3GPP TSG RAN1 #102e, August 2020.

R2-2008188, “Summary of [AT111][107][NTN] Pre-compensation and other MAC issues”, 3GPP TSG RAN2 #111e, August 2020.

R2-2008122, “Report from Break-out session on R16 eMIMO, CLI, PRN, RACS and R17 NTN and REDCAP”, 3GPP TSG RAN2 #111e, August 2020.