3GPP TSG RAN WG1 #103-e		R1-2008755

e-Meeting, October 26th – November 13th, 2020

Source:	OPPO

Title:	Analysis of Control Signaling for multi-beam operation

Agenda Item:	8.1.5

Document for:	Discussion and Decision

- Introduction

NR system specify multi-beam operation since 3GPP release 15 mainly to support FR2 systems. New WID for further enhancing NR MIMO [1] includes the following objectives of enhancement on multi-beam operations.

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

In RAN1#102-e meeting [2], we reached the following agreements on unified TCI state framework:

Agreement Note: the enumeration for issues (such as “issue 1a), 1b), 6) in the proposal below refers to the enumeration within the proposals, not Table 1 in the FL summary. - [Issue 1] For Rel.17 NR FeMIMO, on the unified TCI framework - Support joint TCI for DL and UL based on and analogous to Rel.15/16 DL TCI framework - The term “TCI” at least comprises a TCI state that includes at least one source RS to provide a reference (UE assumption) for determining QCL and/or spatial filter - The source reference signal(s) in M TCIs provide common QCL information at least for UE-dedicated reception on PDSCH and all or subset of CORESETs in a CC - FFS: Optionally this common QCL information can also apply to CSI-RS resource for CSI, CSI-RS resource for BM, and CSI-RS for tracking - FFS: Applicability on PDSCH includes PDSCH default beam - Working Assumption: Select between M=1 and M>=1 - The source reference signal(s) in N TCIs provide a reference for determining common UL TX spatial filter(s) at least for dynamic-grant/configured-grant based PUSCH, all or subset of dedicated PUCCH resources in a CC, - Optionally, this UL TX spatial filter can also apply to all SRS resources in resource set(s) configured for antenna switching/codebook-based/non-codebook-based UL transmissions - FFS: applicability of this UL TX spatial filter to SRS configured for beam management (BM) - FFS: PUSCH port determination based on the TCI, e.g., to be mapped with SRS ports analogous to Rel.15/16 - Working Assumption: Select between N=1 and N>=1 - FFS: extension to common QCL information applied to only some of the CORESETs or PUCCH resources in a CC, e.g. for mTRP - FFS: When used for the purpose of joint beam indication for UL and DL, whether a joint TCI pool for DL and UL dedicated for the purpose is used, or the same TCI pool as that used for the purpose of separate DL/UL beam indication is used - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - FFS (RAN1#103-e): The supported number of active TCI states considering factors such as multi-TRP and issue 6 - FFS (RAN1#103-e): Applicable QCL types, and co-existence with DL TCI and spatial relation indication in Rel.15/16 - In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives for accommodating the case of separate beam indication for UL and DL - Alt1. Utilize the joint TCI to include references for both DL and UL beams - Alt2. Utilize two separate TCI states, one for DL and one for UL. The TCI state for the DL is the same as agreed in 1a. The TCI state for the UL can be newly introduced. - Alt 2-1: The UL TCI state is taken from the same pool of TCI states as the DL TCI state - Alt 2-2: The UL TCI state is taken from another pool of TCI states than the DL TCI state - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - Note: This may be related to issue 5 as well as other reasons for different TCIs such as network flexibility/scheduling - Support the use of SSB/CSI-RS for BM and/or SRS for BM as source RS to determine a UL TX spatial filter in the unified TCI framework - Whether the UL TX spatial filter corresponds to UL TCI (separate from DL TCI) depends on the outcome of 1b) above - FFS: Support the use of non-BM CSI-RS and/or non-BM SRS in addition - In RAN1#103-e, decide if SRS for BM can be configured as a source RS to represent a DL RX spatial filter in the unified TCI framework - In RAN1#103-e, decide/finalize all other parameters included in or concurrent with (but not included in) the TCI, e.g. UL-PC-related parameters (involving P0/alpha, PL RS, and/or closed loop index), UL-timing-related parameters - In RAN1#103-e, identify issues pertaining to alignment between DL and UL default beam assumptions using the unified TCI framework - [Issue 3] For Rel.17 NR FeMIMO, on dynamic TCI state update signaling medium: - In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives: - Alt1. DCI - Alt2. MAC CE - Note: Combination between DCI and MAC CE for, e.g. different use cases or control information partitioning can also be considered - Note: The study should consider factors such as feasibility for pertinent use cases, performance (based on at least the agreed EVM), overhead (including PDCCH capacity), latency, flexibility, reliability including the support of retransmission - Note: This may be related to outcome of issue 1a), 1b), and 6a) - In RAN1#103-e, depending on the outcome of 3a), identify candidates for more detailed design issues for the dynamic TCI state update such as - Exact content - Signaling format - Reliability aspects including the support of retransmission - Extensions, including the support of UE-group (in contrast to UE-dedicated) signaling

In this contribution, we present detailed analysis on the methods of control signaling for TCI state update, including comparing DCI-based method and MAC CE-based method TCI state update from the perspective of latency and overhead.

- Discussion on TCI state update methods

For dynamic TCI state update, we agreed to investigate the following two alternatives of control signaling in RAN1#102-e meeting:

- Alt1: DCI-based TCI state update

- Alt2: MAC CE-based TCI state update

In DCI-based TCI state update method, the gNB indicates one common TCI state through a DCI signaling and the UE shall apply that common TCI state on the reception of UE-specific PDCCH, PDSCH and some CSI-RS resource. The UE shall also apply the spatial setting included in the common TCI state on the transmission of PUSCH, PUCCH and some SRS resources. In contrast, in MAC CE-based method, the gNB indicates one TCI state through a MAC CE message.

These two alternatives can be analyzed from the perspective of latency and overhead. The timeline of DCI-based TCI state update and MAC CE based TCI state are shown in Figures 1.

Figure 1 Time line of MAC CE-based and DCI-based beam switch

Given the timeline of each method TCI state indication, the latency analysis is listed in Table 1.

Table 1: Latency evaluation of TCI state signaling methods

| Latency Components | DCI-based method | MAC CE-based method |
| --- | --- | --- |
| Trigger Time | The gNB decides TCI state switch | The gNB decides TCI state switch |
| Component #1: prepare MAC CE or DCI for indicating TCI state | 1 slot: SCS=60kHz: 0.25ms SCS=120kHz: 0.125 ms | 2ms |
| Component #2: DL transmission and HARQ-ACK | 1 slot, SCS=60kHz: 0.25ms SCS=120kHz: 0.125 ms | 1 slot SCS=60kHz: 0.25ms SCS=120kHz: 0.125 ms |
| Component #2: Process the TCI state switch | 1ms | 3ms |
| End time | The UE starts to assume the new TCI state | The UE starts to assume the new TCI state |
| Total latency | SCS=60kHz: 1.5ms SCS=120KHz: 1.25ms | SCS=60KHz: 5.25ms SCS=120KHz: 5.125ms |

We can observe that DCI-based TCI state update could reduce the latency of TCI state update by ~4ms in comparison with MAC CE based TCI state update.

Next, the signaling overhead is analyzed for these two methods. In MAC CE-based TCI state update, a MAC CE indicating one TCI state shall be transmitted in a PDSCH. In contrast, DCI-based TCI state update only needs one DCI indicating one TCI state. Therefore, DCI-based TCI state update would require less overhead than MAC CE-based method. To analyze the signaling overhead, the following assumptions are used:

- PDSCH carrying one MAC CE PDU: one MAC CE PDU contains one MAC CE subheader that generally includes 4 bytes for header fields LCID/eLCID/L and one MAC CE message that contains 2 bytes. Including 2 bytes of CRC, the total length for payload is 8 bytes. Assume QSPK are used for PDSCH with code rate 679/1024 or 379/1024. 1/5 DMRS overhead is assumed.

- PDCCH carrying DCI scheduling the PDSCH or PDCCH carrying DCI indicating TCI state: 1/2/4/8/16 CCEs can be used for PDCCH.

The comparison of signaling overhead of these two methods is shown in Table 2.

Table 2: signaling overhead comparison: DCI-based vs MAC CE-based

Based on the analysis, we can observe that DCI-based TCI state update can reduce both latency and signaling overhead significantly in comparison with MAC CE-based TCI state update. Thus, DCI-based method is preferred.

Observation 1: DCI-based TCI state method has lower latency than MAC CE based method:

- The latency of DCI-based TCI state is 1.5ms for 60KHz

- The latency of MAC CE based method is 5.25 ms for 60KHz

Observation 2: The signaling overhead of MAC CE based method could be 120%~260% of that of the DCI based method.

- Conclusion

In this contribution, we analyzed the control signaling for TCI state update. Based on the discussion, the following observations are provided:

Observation 1: DCI-based TCI state method has lower latency than MAC CE based method:

- The latency of DCI-based TCI state is 1.5ms for 60KHz

- The latency of MAC CE based method is 5.25ms for 60KHz

Observation 2: The signaling overhead of MAC CE based method could be 120%~260% of that of the DCI based method.

- Reference

RP-193133 New WID: Further ehancements on MIMO for NR

Chairman’s Notes RAN1#102-e