3GPP TSG RAN WG1 Meeting #103e	R1-2008831

e-Meeting, October 26th – November 13th, 2020

Source: 	ZTE

Title: 	Discussion on Multi-cell PDSCH Scheduling via a Single DCI

Agenda item:	8.13.2

Document for:	Discussion/Decision

# Introduction

During RAN #86 meeting, a new work item on DSS (dynamic spectrum sharing) was approved for Rel-17 [1]. In this WI, two main objectives are included, i.e., SCell-schedule-PCell and one-to-two scheduling. The two objectives are mainly introduced to reduce the PDCCH blockage issue under DSS scenario.

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

During RAN1 #102e meeting, the following agreements were approved for the study on single DCI scheduling PDSCH on two cells [2].

Agreements: - For the study on single DCI scheduling PDSCH on two cells - Consider the following scenarios as baseline for evaluation - UE configured with Inter-band CA with PCell and an SCell - PCell for the UE is operated on a DSS carrier (i.e., same carrier is also used for serving LTE users) - Case 1: Different SCS for PCell and SCell - Case 2: Same SCS for PCell and Scell - Additional scenarios can also be evaluated, e.g. as below - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers) - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers) - Note: other combinations not precluded - Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

In this contribution, we provide our analysis for one-to-two scheduling, focusing on the necessity of one-to-two scheduling, simulation results and potential open issues.

# Discussion

## Relationship between SCell-schedule-PCell and one-to-two scheduling

As mentioned in the justification section of the WID, the main motivation of this WI is to ensure sufficient scheduling capacity for NR UEs on the shared carriers under DSS scenario. Take Fig.1 as an example, LTE and NR share the same carrier. In order to avoid interference to the LTE system, NR scheduling is not expected to overlap with the LTE PDCCH region and symbol with LTE CRS. In this case, the available resources for NR PDCCH and NR PDSCH are pretty constrained. This WI aims to mitigate the NR PDCCH constraints and ensure sufficient scheduling capacity for NR UEs.

Fig.1 A typical scenario of DSS.

SCell-schedule-PCell

Most typically, the cell with lower frequency is configured as the primary cell to guarantee the DL/UL coverage. In case of DSS, the shared cell is most likely to be configured as PCell. To mitigate the PDCCH resource constraint in PCell, cross-carrier scheduling can be considered to move the PDCCH in one cell to another cell. However, in Rel-15/16, SCell cross-carrier schedules PCell is not allowed. Thus, SCell cross-carrier schedules PCell has been approved as one solution in this WI to resolve the PDCCH constraint issue on PCell. With this, network can ensure sufficient scheduling capacity for NR UEs in shared carriers. In our companion contribution [3], we provide our analysis and consideration on Scell-schedule-PCell.

One-to-two scheduling

Some companies mentioned that even with the SCell-schedule-PCell, the PDCCH constraint issue is still there as the resources occupied by PDCCH don’t reduce while they are just moved from PCell to SCell. However, the NR carrier is expected to have much larger bandwidth than the LTE carrier. Moving the PDCCH from PCell to NR SCell won’t incur any PDCCH resource constraint in the NR carrier. One may argue that if the bandwidth of SCell is still limited, then the PDCCH resource constraint issue is still there. However, any carrier with limited bandwidth would cause PDCCH resource constraint issue, which is not an issue specific to DSS. In this sense, SCell-schedule-PCell can already resolve the PDCCH resource constraint issue for DSS and the necessity of additionally introducing one-to-two scheduling is not justified.

Observation 1: SCell-schedule-PCell can effectively resolve the PDCCH capacity issue on PCell i.e. usually a shared carrier in DSS scenario, which is the major issue to be addressed in this WI. On top of it, one-to-two scheduling may potentially further improves scheduling capacity but it is more for NR-only carrier i.e. not a DSS specific enhancement. With SCell-schedule-PCell already in the scope of this WI, the necessity of additionally introducing one-to-two scheduling needs to be further justified.

## Preliminary simulation results and analysis

Based on the agreements in RAN1 #102e meeting, the scenario where UE configured with inter-band CA with PCell and an SCell is consider as baseline in the study of one-to-two scheduling, and the scenario where intra-band CA with PCell and an SCell scenario can be also considered. Same SCS for PCell and SCell is a typical case for both of the two scenarios above, and the simulation assumptions and theoretical analysis are made based on this assumption.

In common sense, the size of the single DCI will increase significantly when it schedules PDSCH on two cells especially in inter-band CA scenario, which will lead to a higher aggregation level (AL). It may increase PDCCH blocking rate with high aggregation level. If PDCCH blocking rate of using one bigger size DCI for scheduling PDSCH on two cells is higher than or close to that of using two DCIs with normal size for legacy scheduling, the enhancement of scheduling PDSCH on two cells using a single DCI is meaningless.

To compare the performance of using one bigger size DCI and two DCIs with normal size under typical scenarios, PDCCH blocking rate and the average cell throughput are evaluated via two simulations.

Simulation of PDCCH Blocking Rate

The following steps are adopted to perform the simulation of PDCCH blocking rate comparison.

Step1: Perform system level simulation to get geometry results in Fig. A-1 and Fig. A-2 for urban scenario;

Step2: Perform link level simulation to get the BLER-SNR results in Fig. A-3 and Fig. A-4 for PDCCH;

Step3: Calculate the probability of each aggregation level targeting 10^-2 BLER based on the simulation results in Step1 and Step2;

Step4: Simulate the PDCCH blocking rate for different cases (i.e., different DCI sizes) based on the probability of each aggregation level in Step3.

Two cases are simulated, i.e., the legacy DCI case and the one-to-two scheduling case. For the baseline legacy scheduling mechanism, two DCIs with normal size are used to schedule PDSCH on two cells respectively. For the one-to-two scheduling case, one bigger size DCI is used to schedule PDSCH on two cells. For the baseline legacy scheduling mechanism, DCI size (excluding CRC) is assumed to be 60 bits with all UEs assumed with legacy scheduling. DCI size of 72, 84, 85, 96 and 110 bits are used to simulate for one-to-two scheduling with all UEs assumed with enhanced scheduling (one-to-two scheduling). The remaining details of simulation assumptions (Table A-1) are listed in the appendix Section 4.1.

Simulation of average cell throughput

The average cell throughput is evaluated via a system level simulation under different DCI overhead assumption. The detailed simulation assumptions are listed in Table A-2 in Appendix Section 4.1. Overall, three cases are simulated.

Case A (Baseline): Legacy scheduling mechanism, each DCI schedules one PDSCH on one carrier, DCI size = 60 bits.

Case B: One DCI schedules two PDSCHs on two carriers, where most of the fields related to resource allocation are separated indicated for two carriers. This case is mainly applied for inter-band CA because the cell configuration is likely to be different in the two carriers and it is difficult to share the corresponding indication within two carriers.

Case C: One DCI schedules two PDSCHs on two carriers, where most of the fields related to resource allocation are shared between two carriers. This case is mainly applied for intra-band CA because it is possible that the configurations between the two carriers could be similar or the same.

The detailed assumptions of each DCI field for Case A, Case B and Case C are summarized in Table A-3 in Appendix Section 4.2.

Simulation results for 700MHz DSS carrier

The simulation results of blocking rate are showed in Fig.2, and the simulation results of the average cell throughput are showed in Table 1.

Fig.2 PDCCH blocking rate for 700MHz

Table 1. Average cell throughput for 700MHz DSS cases

| Cases | Average Cell Throughput(Mbps/Sec) |
| --- | --- |
| Case A (Baseline) | 46.8609 |
| Case B | 46.8609 |
| Case C | 40.4571 (13.7% loss) |

Note: In this simulation, the time-frequency resource region reserved for PDCCH is the same for Case A, Case B and Case C, and the unused PDCCH resources are not reused for PDSCH. Thus, the cell throughput is same for Case A (Baseline, two legacy DCIs) and Case B (One DCI with most of the fields separate indicated).

In Case B, DCI fields related to resource indication such as FDRA are independent to ensure the flexibility of PDSCH scheduling, and the DCI size is 108 bits as summarized in Table A-3 in Appendix Section 4.2. As show in Fig.2, although the PDCCH blocking rate using one bigger DCI is lower than that using two normal DCIs, the gain becomes smaller and smaller with the increase of the DCI size. For example, the gain of PDCCH blocking rate is marginal when the DCI size is 110 bits. Because of the increased DCI size, more than half of DCIs with 110 bits are transmitted through AL = 4/8 CCEs, which makes the average CCE is similarly as that of two DCIs with 60 bits. In this simulation, the gain of PDCCH blocking rate for DCI size 110 bits is less than 5%.

Case C has a 13.7% throughput loss compared with case A or Case B as shown in Table 1. The DCI size is 83 bits for Case C as summarized in Table A-3 in Appendix Section 4.2. The main reason for Case C performance loss is that DCI fields related to resource indication is shared, so at least one CC will be restricted in PDSCH scheduling. Although the gain of PDCCH blocking rate is considerable, the average cell throughput will decrease.

Simulation results for 2GHz DSS carrier

The simulation results of blocking rate for 2GHz DSS scenario are showed in Fig.3 and the simulation results of the average cell throughput are showed in Table 2.

Fig.3 PDCCH blocking rate for 2GHz

Table 2 Average cell throughput for 2GHz DSS cases

| Cases | Average Cell Throughput(Mbps/Sec) |
| --- | --- |
| Case A (Baseline) | 56.2344 |
| Case B | 56.2344 |
| Case C | 47.3849 (-15.7% loss) |

Note: In this simulation, the time-frequency resource region reserved for PDCCH is the same for Case A, Case B and Case C, and the unused PDCCH resources are not reused for PDSCH. Thus, the cell throughput is almost the same for Case A (Baseline, two legacy DCIs) and Case B (One DCI with most of the fields separate indicated).

The throughput of Case B is same as that of Case A because most of the DCI fields are separated indicated for two carriers. However, the gain of PDCCH blocking rate is marginal if the size of the DCI in case B is larger than 84 bits as shown in Fig.3. For Case A, most of DCIs are transmitted through AL = 1 CCE because of the smaller DCI size. When the DCI size is larger than 84 bits, DCI can only be transmitted through AL >= 2 CCEs because the max available payload of AL = 1 CCE is exceeded, which will lead to a significant increase in CCE utilization rate of case B, and further increase the PDCCH blocking rate.

For Case C, although the gain of PDCCH blocking rate is considerable when the DCI size is smaller than 85 bits, the average cell throughput will decrease because PDSCH scheduling is limited. As show in Table 2, case C has a loss of 15.7% compared with case A in terms of the average cell throughput. The performance loss is mainly due to the inappropriate scheduling parameters that have to be shared between the two carriers.

Based on the analysis above, we have the following observation.

Observation 2:

For Case B (most of the DCI fields are separate indicated for two carriers), the cell throughput performance is almost the same as the baseline and the gain of PDCCH blocking rate is marginal.

For Case C (most of the DCI fields are shared between two carriers), the cell throughput performance is decreased by 13~16% compared with the baseline though the PDCCH blocking rate is decreased.

## Open issues

If agreed to specify PDCCH of P(S)Cell/SCell scheduling PDSCH on up to two cells using a single DCI, then each DCI field needs to be decided whether to adopt shared indication or separate indication. For shared indication, the two PDSCHs on two different carriers can share the same valued indicated by the DCI field. While for separate indication, two separate values are indicated to the two PDSCHs on two different carriers, respectively.

Then for this single DCI with enhanced scheduling, whether using existing DL assignment(s) or new DL format should be determined. Furthermore, for this single DCI with enhanced scheduling, the total PDCCH blind decoding budget should not be changed as a result of this work.

### DCI fields

Firstly, we assume the baseline DCI format for legacy PDSCH scheduling is Rel-15 DCI format 1_1 with cross-carrier scheduling function. All Rel-15 DCI fields of DCI format 1_1 are listed in Table 1 below. For each field of the single DCI with enhanced scheduling, whether to adopt shared or separate indication should be determined. For example, Identifier for DCI formats can apply shared indication because the cells are both scheduled with PDSCH. The Frequency/Time resource on the two cells can be indicated shared or separately due to different requirement/configuration on the two cells. In order to maintain system efficiency, AMC related fields should be indicated separately. The feedback of the PDSCH on the two cells can be in same codebook/channel or different codebook/channel, as a result the HARQ feedback related fields can apply either shared or separate indication accordingly. For MIMO related fields, it may depend on the detailed configuration of the two carriers and UE feature which can be also shared or separate indication.

Observation 3: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to discuss whether to adopt shared indication or separate indication for each DCI field.

Table 3 DCI fields for the single DCI with enhanced scheduling (assume BWP = 100 PRBs)

| - DCI fields of Format 1_1 | - Bit size of Format 1_1 (bits) | - Baseline size (bits) | - Single DCI with enhanced scheduling (Shared or Separate indication) | - Bit size of Single DCI with enhanced scheduling compared with baseline size (bits) |
| --- | --- | --- | --- | --- |
| - Identifier for DCI formats | - 1 | - 1 | - Shared | - 1 |
| - Carrier indicator | - 0 or 3 | - 3 | - Shared or separate | - 3 or 6 |
| - Bandwidth part indicator | - 0 – 2 | - 1 | - Separate | - 2 |
| - Frequency domain resource assignment | - 13 | - 13 | - Shared or separate | - 13 or 26 |
| - Time domain resource assignment | - 0 - 4 | - 4 | - Shared or separate | - 4 or 8 |
| - VRB-to-PRB mapping | - 0 or 1 | - 1 | - Shared or separate | - 1 or 2 |
| - PRB bundling size indicator | - 0 or 1 | - 1 | - Shared or separate | - 1 or 2 |
| - Rate matching indicator | - 0 - 2 | - 1 | - Separate | - 2 |
| - ZP CSI-RS trigger | - 0 - 2 | - 1 | - Shared or separate | - 1 or 2 |
| - TB1: Modulation and coding scheme | - 5 | - 5 | - Separate | - 10 |
| - TB1: New data indicator | - 1 | - 1 | - Separate | - 2 |
| - TB1: Redundancy version | - 2 | - 2 | - Separate | - 4 |
| - TB2: Modulation and coding scheme | - 5 | - - | - Separate | - - |
| - TB2: New data indicator | - 1 | - - | - Separate | - - |
| - TB2: Redundancy version | - 2 | - - | - Separate | - - |
| - HARQ process number | - 4 | - 4 | - Separate | - 8 |
| - Downlink assignment index | - 0, 2, or 4 | - 4 | - Shared or separate | - 4 or 8 |
| - TPC command for scheduled PUCCH | - 2 | - 2 | - Shared or separate | - 2 or 4 |
| - PUCCH resource indicator | - 3 | - 3 | - Shared or separate | - 3 or 6 |
| - K1 timing indicator | - 0 - 3 | - 3 | - Shared or separate | - 3 or 6 |
| - Antenna port(s) | - 4, 5, or 6 | - 4 | - Shared or separate | - 4 or 8 |
| - Transmission configuration indication | - 0 or 3 | - 3 | - Shared or separate | - 3 or 6 |
| - SRS request | - 2 or 3 | - 2 | - Shared or separate | - 2 or 4 |
| - CBG transmission information (CBGTI) | - 0, 2, 4, 6, or 8 | - - | - Separate | - - |
| - CBG flushing out information (CBGFI) | - 0 or 1 | - - | - Separate | - - |
| - DMRS sequence initialization | - 1 | - 1 | - Shared or separate | - 1 or 2 |
| - Total size (excluding CRC) | - 38 - 83 | - 60 | - Total size (bits) | - 74 - 119 |

Ideally, all the DCI fields adopt separate indication for the one-to-two scheduling DCI. In this case, network can have fully flexibility and can guarantee the PDSCH throughput. However, the DCI size of this one-to-two scheduling DCI is pretty large, which will result with high PDCCH blockage and thus negate the benefits of one-to-two scheduling. On the other hand, if all the DCI fields adopt shared indication for the one-to-two scheduling DCI. In this case, the PDSCH scheduling would have lots of limitations, which may lead to performance degradation in the end. It is hard to figure out the tradeoff between flexibility and DCI size considering that different companies may have different views.

Based on our preliminary analysis, to guarantee at least the moderate flexibility, the size of this enhanced DCI for one-to-two scheduling is at least 74 bits compared with baseline size of 60 bits for legacy scheduling in Table 3. If we want to have fully flexibility, the DCI size can go up to 119 bits. Furthermore, if two TBs or CBG transmission are supported on each carrier, then the DCI size will be increased further.

Furthermore, other DCI fields introduced in Rel-16 of DCI format 1_1 are listed in Table 4 below. For simplicity, Rel-16 NRU/URLLC/Power saving related DCI fields can be ignored in Rel-17 DSS, thus the fields in Table 4 can be all ignored. However, if one or both of cells are configured with NRU/URLLC/Power saving related functionality, then each field in Table 4 should be determined whether to adopt shared or separate indication for the single DCI with enhanced scheduling. For SCell dormancy indication field, it can be shared indication because it originally used for a group of cells of each bit. While for other fields, more studies are needed.

Observation 4: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to handle the Rel-16 newly introduced DCI fields in DCI format 1_1.

Table 4 DCI fields introduced in Rel-16 for the single DCI with enhanced scheduling

| - DCI fields of Format 1_1 | - Bit size of Format 1_1 (bits) | - Single DCI with enhanced scheduling (Shared or Separate indication) |
| --- | --- | --- |
| - One-shot HARQ-ACK request | - 0 or 1 | - Shared or separate |
| - PDSCH group index | - 0 or 1 | - Shared or separate |
| - New feedback indicator | - 0 – 2 | - Shared or separate |
| - Number of requested PDSCH group(s) | - 0 or 1 | - Shared or separate |
| - ChannelAccess-CPext | - 0 - 4 | - Shared or separate |
| - Priority indicator | - 0 or 1 | - Shared or separate |
| - Minimum applicable scheduling offset indicator | - 0 or 1 | - Shared or separate |
| - SCell dormancy indication | - 0 - 5 | - Shared |

### DCI format

The single DCI with enhanced scheduling can be designed by using DCI format 1_1/1_2 as a starting point. If the enhanced scheduling is enabled, then the format 1_1/1_2 with bigger size will be used for scheduling PDSCH on up to two cells. Note that scheduling PUSCH on two cells is out of the scope of this WI, then more padding bits are needed if size of DCI format 0_1/0_2 are required to align with the size of DCI format 1_1/1_2 once DCI size budget is exceeded. Meanwhile, scheduling PDSCH on two cells are not always needed, then dynamic fallback to single cell scheduling should be compatible as well. In this case, the bigger DCI size keep the same between scheduling two carriers and scheduling one carrier to avoid additional blind decoding. In other words, even if the DCI fallbacks to scheduling PDSCH only on one cell, its DCI size has to be aligned with the bigger DCI size too.

Alternatively, new DCI format, e.g., format 1_3, can be also considered. In this case, the new DCI format 1_3 is introduced only for scheduling PDSCH on two cells, and all the existing DCI formats are used for legacy scheduling. With this, dynamic fallback to single cell scheduling is not needed for the new DCI format 1_3. And UL grant corresponding to this new DL assignment is not needed too. But this will challenge the DCI size budget/alignment.

Observation 5: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study whether to reuse DCI format 1_1/1_2 or introduce a new DCI format for one-to-two scheduling.

### How to schedule the two cells

With the current CA mechanism, each CIF index corresponds to one scheduled cell. In case that scheduling PDSCH on two cells via a single DCI is supported, how to indicate the two scheduled cells should be studied.

If dynamic combination of the two cells is supported, separated indication of CIF is needed. Since different cells may have different configurations, the size of this single DCI need to be determined according the carrier combination with maximum DCI size. Another way is to consider predefined combination of the two scheduled cells. For example, cell corresponding to CIF=x and cell corresponding to CIF=0 are always bundled together. In this case, only one CIF field is sufficient.

If semi-static combination of the two cells is supported, then only one CIF indication is enough. While CIF configuration needs to be updated accordingly.

Observation 6: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to indicate the two scheduled carriers.

### Blind decoding issues

Furthermore, for this single DCI with enhanced scheduling, the total PDCCH blind decoding budget should not be changed as a result of this work.

In case additional DCI size is introduced based on the new DCI format, e.g., DCI format 1_3, the blind decoding number may be increased, but the total blind decoding number can be controlled to avoid beyond the BD budget by reasonable candidate configuration.

Another issue is in which USS to blind decode the single DCI with enhanced scheduling. In current spec, USS on the scheduling cell for each scheduled cell is determined by CIF value. Then for this single DCI with enhanced scheduling, using the USS of one of the two cells or both cells should be determined. And it is better to determine after the issue of how to schedule the two cells is solved. Also the blind decoding budget should not be changed in case of additional search space introduced to blind decoding.

Observation 7: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to guarantee the current BD/CCE budget.

### HARQ-ACK feedback

Another open issue is how to construct the HARQ-ACK codebook for one-to-two PDSCH scheduling and how to determine the PUCCH resource for the corresponding HARQ-ACK codebook.

If all the related DCI fields are all separately indicated to the UE for each carrier, e.g., K1, PRI and TPC, the specification could be minimized. However, this would significantly increase the DCI size. Another approach is to adopt shared indication for the related DCI fields. Then, some further studies are needed to finalize the detailed mechanism.

Observation 8: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to perform the corresponding HARQ-ACK feedback.

## Summary

Based on our simulation results in Section 2.2, to ensure scheduling flexibility, most of the DCI fields are separately indicated for two carriers. However, this may end up with a marginal gain of PDCCH blocking rate. On the other hand, if most of the DCI fields are shared between two carriers, the PDCCH blocking rate is reduced while the average cell throughput is decreased by about 14% due to the inappropriate DCI field settings for these two carriers. The more DCI fields shared between two carriers, the less gain of PDCCH blocking rate. The less DCI fields shared between two carriers, the higher system throughput performance loss.

Observation 9: It is difficult to obtain the gain of single DCI scheduling two PDSCHs on two carriers considering the trade-off between PDCCH blocking rate and system performance throughput.

Besides, the TU reserved for DSS WI in RAN1 is 6 TUs in total with 1 TU per meeting. Except for the first meeting, which may be used to study the motivation and potential gain of one-to-two scheduling, there are only 5 meetings left for further discussion on one-to-two scheduling. Considering the all the contentious issues raised in Section 2.3, it is impossible for companies to finalize all the detailed design with only 1 TU reserved for DSS WI, especially considering that SCell-schedule-PCell and Rel-17 MRDC share the same 1 TU with one-to-two scheduling. In this sense, it is preferred to put off the one-to-two scheduling to later release or increase the TU for DSS WI. Based on our understanding, increasing TUs for DSS WI means take out TUs from other WIs as the whole Rel-17 package WIs have been already finalized in last year. Thus, it is more appropriate to put off one-to-two scheduling to future release.

Observation 10: Considering all the contentious issues on one-to-two scheduling and the limited TUs for DSS WI, the timely completion of one-to-two scheduling and other more important enhancements may be jeopardized. It is more desirable to prioritize and focus more on the standardization on Scell-schedule-PCell which is more important for resolving PDCCH capacity issue in DSS scenarios.

Overall, from our perspective, it seems more practical to skip the one-to-two scheduling in Rel-17 DSS WI.

Proposal 1: RAN1 further discusses the necessity, gain, open issues and possibility of timely completion of single DCI scheduling two PDSCHs on two carriers.

If TU permits, one alternative for one DCI scheduling two PDSCHs on two carriers is to use one DCI to schedule two PDSCHs on the same carrier. Similar mechanism for PUSCH has been specified in NR-U, where one DCI can be used to schedule two or more PUSCHs on the same carrier in consecutive slots. If one DCI is used to schedule up to two PDSCHs on the same carrier, only additional 1 - 2 bits are needed on top of the existing DCI format 1_1 or DCI format 1_2 if similar enhancement of PUSCH multi-TTI scheduling by DCI format 0_1 is adopted. In this case, the PDCCH blocking rate would be reduced. Besides, throughput performance can also be guaranteed as the two scheduled PDSCHs shared the same cell configuration. Furthermore, the spec impact is minor for up to two PDSCHs scheduling by one DCI on the same carrier, according to the current mechanism of one DCI scheduling multiple PUSCHs on one carrier in NRU, maybe we only need to double the bits of NDI field, and the size of RV field is still 2 bits with 1 bit for each TTI.

Proposal 2: If TU permits, RAN1 considers one DCI scheduling two PDSCHs on the same carrier instead of one DCI scheduling two PDSCHs on two carriers.

# Conclusion

In this contribution, analysis on the necessity, gain, open issues and possibility of timely completion of single DCI scheduling two PDSCHs on two carriers are presented with the following observations and proposals.

Relationship between SCell-schedule-PCell and one-to-two scheduling

Observation 1: SCell-schedule-PCell can effectively resolve the PDCCH capacity issue on PCell i.e. usually a shared carrier in DSS scenario, which is the major issue to be addressed in this WI. On top of it, one-to-two scheduling may potentially further improves scheduling capacity but it is more for NR-only carrier i.e. not a DSS specific enhancement. With SCell-schedule-PCell already in the scope of this WI, the necessity of additionally introducing one-to-two scheduling needs to be further justified.

Preliminary simulation results and analysis

Observation 2:

For Case B (most of the DCI fields are separate indicated for two carriers), the cell throughput performance is almost the same as the baseline and the gain of PDCCH blocking rate is marginal.

For Case C (most of the DCI fields are shared between two carriers), the cell throughput performance is decreased by 13~16% compared with the baseline though the PDCCH blocking rate is decreased.

Open issues

Observation 3: If single DCI scheduling two PDSCHs on two carriers is supported, to guarantee at least the moderate flexibility, the minimum size of this enhanced DCI for one-to-two scheduling is 93bits compared with 60 bits for legacy scheduling.

Observation 4: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to handle the Rel-16 newly introduced DCI fields in DCI format 1_1.

Observation 5: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study whether to apply DCI format 1_1/1_2 or introduce a new DCI format for one-to-two scheduling.

Observation 6: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to indicate the two scheduled carriers.

Observation 7: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to guarantee the current BD/CCE budget.

Observation 8: If single DCI scheduling two PDSCHs on two carriers is supported, RAN1 needs to further study how to perform the corresponding HARQ-ACK feedback.

Summary

Observation 9: It is difficult to obtain the gain of single DCI scheduling two PDSCHs on two carriers considering the trade-off between PDCCH blocking rate and system performance throughput.

Observation 10: Considering all the contentious issues on one-to-two scheduling and the limited TUs for DSS WI, the timely completion of one-to-two scheduling and other more important enhancements may be jeopardized. It is more desirable to prioritize and focus more on the standardization on Scell-schedule-PCell which is more important for resolving PDCCH capacity issue in DSS scenarios.

Proposal 1: RAN1 further discusses the necessity, gain, open issues and possibility of timely completion of single DCI scheduling two PDSCHs on two carriers.

Proposal 2: If TU permits, RAN1 considers one DCI scheduling two PDSCHs on the same carrier instead of one DCI scheduling two PDSCHs on two carriers.

# Reference

RP-193260, New WID on NR Dynamic spectrum sharing (DSS), RAN #86.

RAN1 #102e meeting, chairman notes.

R1-2008830, Discussion on Cross-Carrier Scheduling from SCell to PCell, ZTE

# Appendix Simulation assumptions

## LLS and SLS assumption

Table A-1 PDCCH blocking rate simulation assumptions

| Parameters | Value |
| --- | --- |
| DCI payload (excluding 24 bits CRC) | Legacy DCI: 60 bits, One-to-two scheduling DCI: 72, 84, 85, 96 and 110 bits |
| System bandwidth | 20 MHz |
| Carrier Frequency | 700MHz/2 GHz |
| UE speed | 3 km/h |
| Number of UE antennas | 2 Rx |
| Number of symbols for CORESET | 2 |
| Subcarrier spacing | 15 KHz |
| Transmission type | Interleaved(R=3 for 3OS,others,R=2) |
| REG bundling size | 6 |
| Modulation | QPSK |
| Channel coding | Polar code (DCI) |
| Transmission scheme | 1-port precoder cycling |
| Channel model | TDL-C (delay spread: 300ns) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Note: CORESET size is 16 CCEs, and number of candidates for AL=1, 2, 4, 8, 16 CCEs are configured as 4, 2, 2, 1, 1. |  |

Table A-2 System level simulation assumptions

| Parameters | Value |  |
| --- | --- | --- |
| Carrier frequency | 700MHz | 2 GHz |
| BS antenna configurations | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 1) | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 1) |
| UE antenna configuration | 2 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) | 2 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| Simulation bandwidth | CC1/CC2:20 MHz | CC1/CC2:20 MHz |
| Inter-BS distance | 500m |  |
| UE Tx power | 23dBm |  |
| BS antenna element gain + connector loss | 8 dBi |  |
| BS receiver noise figure | 5dB |  |
| BS antenna height | 25 m |  |
| UE antenna height | 1.5m |  |
| UE antenna gain | 0dBi |  |
| BS Tx power | 49 dBm per 20 MHz |  |
| BS receiver | MMSE-IRC |  |
| UE receiver noise figure | 9 dB |  |
| SCS | 15 kHz |  |
| Channel model | UMa in TR 38.901 |  |
| UE distribution | 80% of users are indoor: 3 km/h |  |
| HARQ | Max number of transmissions = 4 with target BLER = 0.1 |  |
| Channel estimation | Ideal |  |
| CSI feedback Period and scheduling delay | 5 , 2 |  |
| CSI feedback mode | Sub-band feedback |  |
| Traffic model | Full buffer |  |

## DCI size and DCI fields assumption

Table A-3 DCI fields assumption for system level simulation

| - DCI fields of Format 1_1 | - Bit size of Format 1_1 (bits) | - Case A - Baseline size (bits) | - Case B |  | - Case C |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | - Single DCI with enhanced scheduling (Shared or Separate indication) | - Bit size of Single DCI with enhanced scheduling compared with baseline size (bits) | - Single DCI with enhanced scheduling (Shared or Separate indication) | - Bit size of Single DCI with enhanced scheduling compared with baseline size (bits) |
| - Identifier for DCI formats | - 1 | - 1 | - Shared | - 1 | - Shared | - 1 |
| - Carrier indicator | - 0 or 3 | - 3 | - Shared | - 3 | - Shared | - 3 |
| - Bandwidth part indicator | - 0 – 2 | - 1 | - Separate | - 2 | - Separate | - 2 |
| - FDRA | - 13 | - 13 | - Separate | - 26 | - Shared | - 13 |
| - TDRA | - 0 - 4 | - 4 | - Separate | - 8 | - Shared | - 4 |
| - VRB-to-PRB mapping | - 0 or 1 | - 1 | - Shared | - 1 | - Shared | - 1 |
| - PRB bundling size indicator | - 0 or 1 | - 1 | - Shared | - 1 | - Shared | - 1 |
| - Rate matching indicator | - 0 - 2 | - 1 | - Separate | - 2 | - Separate | - 2 |
| - ZP CSI-RS trigger | - 0 - 2 | - 1 | - Separate | - 2 | - Shared | - 1 |
| - TB1: Modulation and coding scheme | - 5 | - 5 | - Separate | - 10 | - Separate | - 10 |
| - TB1: New data indicator | - 1 | - 1 | - Separate | - 2 | - Separate | - 2 |
| - TB1: Redundancy version | - 2 | - 2 | - Separate | - 4 | - Separate | - 4 |
| - HARQ process number | - 4 | - 4 | - Separate | - 8 | - Separate | - 8 |
| - Downlink assignment index | - 0, 2, or 4 | - 4 | - Separate | - 8 | - Shared | - 4 |
| - TPC command for scheduled PUCCH | - 2 | - 2 | - Shared | - 2 | - Shared | - 2 |
| - PUCCH resource indicator | - 3 | - 3 | - Shared | - 3 | - Shared | - 3 |
| - K1 timing indicator | - 0 - 3 | - 3 | - Separate | - 6 | - Shared | - 3 |
| - Antenna port(s) | - 4, 5, or 6 | - 4 | - Separate | - 8 | - Separate | - 8 |
| - Transmission configuration indication | - 0 or 3 | - 3 | - Separate | - 6 | - Separate | - 6 |
| - SRS request | - 2 or 3 | - 2 | - Separate | - 4 | - Separate | - 4 |
| - DMRS sequence initialization | - 1 | - 1 | - Shared | - 1 | - Shared | - 1 |
| - Total size (excluding CRC) | - 38 - 83 | - 60 | - Total size (bits) | - 108 | - Total size (bits) | - 83 |

## Geometry

- Fig.A-1. CDF of SNR for PDCCH in Urban scenario (700MHz)

- Fig.A-2. CDF of SNR for PDCCH in Urban scenario (2GHz)

## PDCCH LLS results

- Fig.A-3. Link level simulation results (700MHz)

- Fig.A-4. Link level simulation results (2GHz)