3GPP TSG-RAN WG1 Meeting #103-e	R1-2008865

e-Meeting, Oct. 26th—Nov. 13th, 2020

Agenda Item:	8.6.3

Source:	Ericsson

Title:	Coverage recovery and capacity impact for RedCap

Document for:	Discussion, Decision

# Introduction

This contribution is a revision of R1-2007531. The revisions are made in Section 2.2 to reflect more accurate antenna gain component correction factors.

In the study item on the support of reduced capability NR devices for use cases such as industrial wireless sensors, video surveillance, and wearables [1], lower device cost and complexity, as compared to high-end eMBB and URLLC devices of Rel-15/16, are among the requirements for these three use cases, as described in [1].

One potential consequence of device cost and complexity reduction is coverage loss. Thus, in one of the study item objectives, it is considered to investigate coverage recovery to compensate for potential decrease in the coverage due to the device complexity reduction. It is worth noting that the levels of coverage recovery needed are likely different for different physical channels. Thus, the first step is to assess the coverage situation of Rel-15/16 NR and identify physical channels that are limiting the NR coverage, as they can potentially also limit the RedCap coverage. These coverage-limiting channels then need coverage-recovery solutions. Different coverage-recovery solutions for different physical channels can be considered.

Related to study of coverage loss/recovery the following agreements have been made in RAN1#101e and RAN1#102e [2] meetings.

RAN1 #101e Agreements: If/when coverage evaluations outside the CE SI are needed, - The basic evaluation methodology is based on link-level simulation for FR1. - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements. - Step 2: Obtain the baseline performance based on required SINR and link budget template. - Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately - The evaluation methodology for FR2 is the same as FR1. Agreements: If/when link-level coverage evaluations outside the CE SI are needed, - The CE SI link-level simulation assumptions can be used as a starting point. - For calibration purposes, the following settings can be used: Parameters; FR1 values; FR2 values. Scenario and frequency; Urban: 2.6 GHz (TDD) (primary choice) 4 GHz (TDD) (secondary choice) Rural: 700 MHz (FDD); Indoor: 28 GHz (TDD). Frame structure for TDD; For 2.6 GHz: DDDDDDDSUU (S: 6D:4G:4U) For 4 GHz: DDDSUDDSUU (S: 10D:2G:2U); DDDSU (S: 10D:2G:2U). Channel model; TDL-C; TDL-A. UE velocity; 3 km/h; 3 km/h. RAN1 #102 e: Agreements: For the channel(s) affected by complexity reduction, the following methodology can be used to determine the target performance for coverage recovery. - Step 1: Obtain the link budget performance of the channel based on link budget evaluation - Step 2: Obtain the target performance requirement for RedCap UEs within a deployment scenario - FFS on the target performance requirement - Step 3: Find the coverage recovery value for the channel if the link budget performance is worse than the target performance requirement Agreements: Link budget evaluation for RedCap should include at least PDCCH/PDSCH and PUCCH/PUSCH. Agreements: For initial access related channels, at least Msg2, Msg3, Msg4 and PDCCH scheduling Msg2/4 are included for link budget evaluation - Other initial access related channels are not precluded Agreements: The impact of small form factor is considered for all the uplink and downlink channels - A 3dB loss of antenna gain is included in link budget calculation for FR1 - FFS on the application to both FDD and TDD bands or only FDD bands [revised, see below] Agreements: For link budget evaluation, the antenna gain loss due to the small form factor can be applied to all the FR1 bands - For RedCap coverage analysis, the agreements in the Rel-17 CE SI regarding link budget template and antenna array gain are reused. - Continue to discuss and decide the performance metric in RAN1-103 e-meeting Agreements: Down-selection on the following options for the target performance requirement for RedCap UEs in RAN1#103-e (aim for early in the e-meeting): - Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment - Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL - The details for the target performance requirement are FFS Agreements: For RedCap UE, adopt the following target data rates for link budget evaluation for FR1 Rural. - 1 Mbps on DL and 100kbps in UL Agreements: For RedCap UE, adopt the following target data rates for link budget evaluation for FR1 Urban. - 2 Mbps on DL and 1Mbps in UL Note: The 2Mbps target data rate in downlink is the scaled value of the 10Mbps in the CE SI by a factor of 0.2 Agreements: For RedCap UEs, the target data rates for link budget evaluation for FR2 are as follows: - 25Mbps for BW 50MHz/100MHz on DL and 5Mbps in UL - Optionally, 12.5Mbps for BW 50MHz as the target data rate for DL, assuming the same DL PSD as that of BW 100MHz - Note: in case of 50MHz BW, the maximum supported DL data rate is half that of the 100MHz BW in DL Agreements: For RedCap coverage evaluation, the Rel-17 CE SI agreements on gNB antenna configuration, # gNB Tx/Rx chains, channel model and delay spread are reused with the following revision and/or addition Parameters; FR1 values; FR2 values. Channel model; TDL-C; TDL-A CDL-A(optional). Delay spread; 300ns; 30ns. UE velocity; 3 km/h; 3 km/h. Antenna correlation; Low; Low. # gNB Tx chains; 2 or 4; 2. # gNB Rx chains; 2 or 4; 2. For RedCap coverage evaluation, adopt the following table for the reference NR UE. Parameters; FR1 values; FR2 values. # UE Tx chains; 1; 1. # UE Rx chains; Urban: 4 and Rural: 2; 2. UE BW; Urban: 100 MHz (273 PRBs) Rural: 20 MHz (106 PRBs); 100 MHz (66 PRBs). For RedCap coverage evaluation, adopt the following table for the RedCap UE. - Other UE BWs are not precluded Parameters; FR1 values; FR2 values. # UE Tx chains; 1; 1. # UE Rx chains; 1 or 2; 1 or 2. UE BW; Urban: 20 MHz (51 PRBs) Rural: 20 MHz (106 PRBs); 50 MHz (32 PRBs) or 100 MHz (66 PRBs). Agreements: For RedCap coverage evaluation, reuse the Rel-17 CE SI agreements on channel specific parameters with the following revision and/or addition - TBS/PRB/MCS of PDSCH (except for Msg2)/PUSCH for the RedCap UE are based on the agreed target data rates or message sizes and reported by companies - Adopt the following table for Msg2 evaluation - Note: the TBS scaling is not precluded in the table entry “PRBs/TBS/MCS” Parameters; Values. PRBs/TBS/MCS; MCS is fixed to zero. Companies to report the used number of PRBs and corresponding TBS value. PDSCH duration; 12 OS. DMRS configuration; Type I, 3 DMRS symbol, no multiplexing with data. Waveform; CP-OFDM. HARQ configuration; No retransmission.

In relation to our general intention to investigate the impact of complexity reduction, we have performed link-level simulations based on the above-mentioned agreements. Then, we have obtained the required SINR for the considered physical channels under target scenarios and service/reliability requirements. These results are used to perform link-budget evaluation to identify the bottleneck channels which need coverage recovery.

The outcomes of our link-level simulations and the link-budget evaluation are summarized in Section 2. In this section we have also listed several possible coverage-recovery solutions for different channels.

Also, considering the newly added objective to the SID, “The study includes evaluations of the impact to network capacity and spectral efficiency”, and based on the agreements on system-level simulation assumptions (listed below), we have performed system-level simulations, and the results are presented in Section 3.

Agreements: - For SLS based capacity evaluation, use the assumption in TR 38.802, Table A.2.1-1 as the baseline. - For calibration purposes, the following settings can be used: Parameters; FR1 values; FR2 values. Layout; Single layer Macro layer: Hex. Grid; Single layer Indoor floor: (12BSs per 120m x 50m) Candidate TRP numbers: 3, 6, 12. Inter-BS distance; 500m; 20m. Scenario and frequency; Dense Urban: 2.6 GHz (TDD) (primary choice) 4 GHz (TDD) (secondary choice) Other scenarios (e.g. Rural 700MHz) are not precluded.; Indoor: 28 GHz (TDD). Frame structure for TDD; For 2.6 GHz: DDDDDDDSUU (S: 6D:4G:4U) For 4 GHz: DDDSUDDSUU (S: 10D:2G:2U); DDDSU (S: 10D:2G:2U). Channel model; 3Duma; 5GCM office. UE distribution; 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h; 100% Indoor: 3km/h. Traffic model; Full buffer (Optional) Non-full buffer traffic, e.g. FTP traffic model 3 for the reference NR UEs and the IM traffic model from TR 38.840 for RedCap UEs. Traffic load; Full buffer traffic (Optional): 10 users per cell including both RedCap and reference NR UEs Non-full buffer traffic: Low (e.g. <30%) and medium (e.g. 30%-50%) loading (resource utilization). Percentage of RedCap UEs among total number of UEs Note: Other UEs are the reference NR UEs; Full buffer traffic (Optional): 0, 20%, 50% (i.e. 0, 2 or 5 RedCap UEs per cell), 100% (as applicable) Non-full buffer traffic: 0, 25%, 50%, 100% (optional, as applicable).

# UE complexity reduction and coverage evaluation

## Link-level simulations

In order to evaluate the impact of the UE complexity reduction, we have performed link-level simulations to obtain the required SINR for the physical channels under performance target for the reference UEs and RedCap UEs. For that, we have considered four different scenarios:

- FR1, Urban with the carrier frequency of 2.6 GHz,

- FR2, Indoor with the carrier frequency of 28 GHz,

- FR1, Rural with the carrier frequency of 0.7 GHz, and

- FR1, Urban with the carrier frequency of 4 GHz.

Our simulation assumptions and performance targets are based on the agreements from RAN1-101e and RAN1-102e meetings and listed in Appendix in detail. Considering the corresponding performance target for the different physical channels, the required SINRs—to fulfil these targets—are reported in Table 1, Table 2, Table 3, and Table 4, respectively, for each of the above-mentioned scenarios. Note that to obtain the required SINRs for PDSCH-eMBB and PUSCH-eMBB, we have considered both data rate and BLER targets.

Table 1: Required SINR (dB); FR1, Urban with the carrier frequency of 2.6 GHz.

|  | PRACH | SSB | PDCCH | PDSCH | Msg 2 | Msg 4 | PUCCH (22 bits PF3, 1% BLER) | PUSCH | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW= 100 MHz #Rx= 4 | -17.5 | -11 | -9.2 | -5.7 | -4.1 | -6.6 | -5.4 | -10.5 | -6.0 |
| BW= 20 MHz #Rx= 2 | -17.5 | -8 | -6 | -2.7 | -0.4 | -3.1 | -5.4 | -10.5 | -6.0 |
| BW= 20 MHz #Rx= 1 | -17.5 | -4.1 | -3 | 0.5 | 4.5 | 0.9 | -5.4 | -10.5 | -6.0 |

Table 2: Required SINR (dB); FR2, Indoor with the carrier frequency of 28 GHz.

|  | PRACH | SSB | PDCCH | PDSCH | Msg 2 | Msg 4 | PUCCH (22 bits PF3, 1% BLER) | PUSCH | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW= 100 MHz #Rx= 2 | -12.2 | -8.2 | -6 | -2.3 | -2.1 | -1.9 | -0.9 | -9.4 | -1.8 |
| BW= 100 MHz #Rx= 1 | -12.2 | -4.5 | -2.1 | 1.7 | 3.6 | 2.5 | -0.9 | -9.4 | -1.8 |
| BW= 50 MHz #Rx= 2 | -12.2 | -8.2 | -4.1 | -2 | -2 | -1.9 | -0.6 | -6.2 | -1.8 |
| BW= 50 MHz #Rx= 1 | -12.2 | -4.5 | 0 | 2.2 | 3.7 | 2.6 | -0.6 | -6.2 | -1.8 |

Table 3: Required SINR (dB); FR1, Rural with the carrier frequency of 0.7 GHz.

|  | PRACH | SSB | PDCCH | PDSCH | Msg 2 | Msg 4 | PUCCH (22 bits PF3, 1% BLER) | PUSCH | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW=20 MHz #Rx= 2 | -9.1 | -7.3 | -6.6 | -5.6 | -3.2 | -5.9 | -0.9 | -2.4 | -1.5 |
| BW=20 MHz #Rx= 1 | -9.1 | -2.9 | -3.1 | -2 | 0.9 | -2.2 | -0.9 | -2.4 | -1.5 |

Table 4: Required SINR (dB); FR1, Urban with the carrier frequency of 4 GHz.

|  | PRACH | SSB | PDCCH | PDSCH | Msg 2 | Msg 4 | PUCCH (22 bits PF3, 1% BLER) | PUSCH | Msg3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW= 100 MHz #Rx= 4 | -17.3 | -11.1 | -9.2 | -5.9 | -3.8 | -6.7 | -5.4 | -10.6 | -6.1 |
| BW= 20 MHz #Rx= 2 | -17.3 | -8.2 | -6 | -2.7 | -0.4 | -3.1 | -5.4 | -10.6 | -6.1 |
| BW= 20 MHz #Rx= 1 | -17.3 | -4.6 | -2.9 | 1 | 4.4 | 0.8 | -5.4 | -10.6 | -6.1 |

For all the considered scenarios, the impact of the complexity reduction is more considerable for DL channels.

## Link budget

On RAN1#102e meeting, it was agreed to take the following steps to identify the channels which need coverage recovery and the corresponding coverage-recovery values.

- Step 1: Obtain the link-budget performance of the channel based on link budget evaluation

- Step 2: Obtain the target performance requirement for RedCap UEs within a deployment scenario

- FFS on the target performance requirement

- Step 3: Find the coverage recovery value for the channel if the link budget performance is worse than the target performance requirement

Regarding the first step, we have used the simulation assumptions and results presented in Section 2.1 and Appendix to perform the link budget evaluation based on the link budget template agreed on RAN1#102-e post-meeting [4]. Our link budget evaluation outcomes in terms of maximum coupling loss (MCL), available path loss and maximum isotropic loss (MIL) are shown in Figure 2 to Figure 13, and are discussed in the following subsections.

Regarding the second step, to identify the target performance requirement and the bottleneck channels, in RAN1#102e meeting, it was agreed to down select between the following options:

- Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment

- Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario

  - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL

As it is not clear how a reasonable deployment is defined in the RedCap coverage study, we think it is more straightforward to progress with Option 3.

To determine the target performance requirement and the bottleneck channel(s), Option 3 is used, i.e.,
The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario.
Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL

Maximum coupling loss (MCL) and maximum isotropic loss (MIL) are two commonly used metrics in coverage evaluation. The difference between the two is that MCL does not include antenna gain components 3 and 4 in Figure 1. Antenna gain component 3 is however channel and procedure dependent. In the agreed link budget template this aspect is captured through an antenna gain correction factor, which is channel and procedure dependent. Not including such a channel and procedure dependent aspect prevents MCL from properly identifying the coverage bottleneck channels among all physical channels and for all the relevant procedure, e.g. initial access. Furthermore, antenna component 4 may be reduced to reflect the antenna efficiency loss in a RedCap UE. Therefore, we propose to use the MIL as the coverage evaluation metric.

Maximum isotropic loss (MIL) is used as the coverage evaluation metric.

Figure 1: Antenna gain components

In the following subsections, we have presented the results of link-budget evaluation and listed the channels or messages that potentially need coverage recovery separately for the considered scenarios. It is worthy to mention that in our evaluations, we have considered 3 dB antenna gain loss due to the small form factor for all the FR1 bands.

We have considered the channel that has the lowest MIL (MCL or MPL) for the reference UE as the bottleneck channel, and the corresponding MIL (MCL or MPL) as a threshold to identify the channels that need coverage recovery and the corresponding coverage recovery values. For the cases in them, either UL or DL data channel has the lowest MIL (MCL or MPL), assuming that the coverage of the data channel can be improved by decreasing data rate, we have considered the channel with second-lowest MIL (MCL or MPL), as the bottleneck channel.

### FR1, Urban with the carrier frequency of 2.6 GHz

Figure 2, Figure 3, and Figure 4 show respectively MCL, available path loss, and MIL for different downlink and uplink control and data channels, at 2.6 GHz carrier frequency.

Figure 2: Link budget performance, MCL, 2.6 GHz.

Figure 3: Link budget performance, available path loss, 2.6 GHz.

Figure 4: Link budget performance, MIL, 2.6 GHz.

Considering the coverage recovery metrics MCL, MPL and MIL, Table 5 shows the bottleneck channel for the reference UE and the channels that potentially need coverage recovery for the RedCap UE at 2.6 GHz carrier frequency.

Table 5: Bottleneck channel for the reference UE, and the channels that need coverage recovery for the RedCap UE, 2.6 GHz.

|  | Bottleneck channel for reference UE | Need coverage recovery RedCap (2 Rx) | Need coverage recovery RedCap (1 Rx) |
| --- | --- | --- | --- |
| MCL | PUSCH, MCL=139.1 | - | - |
| MPL | PUSCH, MPL=113.2 | PUSCH, MPL=110.2 | PUSCH, MPL=110.2 |
| MIL | PUSCH, MIL=143.9 | PUSCH, MIL=140.9 | PUSCH, MIL=140.9 |

For a RedCap UE, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 3.0 dB is observed for PUSCH at 2.6 GHz carrier frequency.

However as shown in Table 5, the bottleneck channel is PUSCH. For data channels coverage can be improved by simply lowering the data rates. Thus, it might be more interesting to consider the next worst channel as the bottleneck channel. This is reflected in the summary in Table 6. First, Msg3 with 154.1 dB MIL then becomes the bottleneck channel. Then, for RedCap UEs with 2 Rx, approximately 2.0 dB and 3.0 dB coverage recovery would be needed for Msg2 and Msg3, respectively. For RedCap UEs with 1 Rx, approximately 6.9 dB, 3.3 dB and 3.0 dB coverage recovery would be needed for Msg2, Msg4 and Msg3, respectively.

Table 6: Bottleneck channel for the reference UE, based on the worst channel other than PUSCH, and the channels that need coverage recovery for the RedCap UE, 2.6 GHz.

|  | Bottleneck channel for reference UE | Need coverage recovery RedCap (2 Rx) | Need coverage recovery RedCap (1 Rx) |
| --- | --- | --- | --- |
| MCL | Msg3, MCL=146.4 | Msg 2, MCL= 144.4 | Msg 2, MCL= 139.5 |
| MPL | PUCCH (22 bits), MPL=119.7 | PRACH, MPL=118.2 PUCCH (2 bits), MPL=118 PUCCH (11 bits), MPL=118.6 PUCCH (22 bits), MPL=116.8 Msg3, MPL=117.4 Msg2, MPL=119.4 | PRACH, MPL=118.2 Msg2, MPL=114.5 Msg4, MPL=118.1 PUCCH (2 bits), MPL=118.0 PUCCH (11 bits), MPL=118.6 PUCCH (22 bits), MPL=116.8 Msg3, MPL=117.4 PDCCH CSS, MPL=118.9 PDCCH USS, MPL=118.9 |
| MIL | Msg3, MIL=151.1 | Msg2, MIL=150.1 Msg3, MIL=148.1 PUCCH (22 bits), MIL=150.6 | Msg2, MIL=145.2 Msg4, MIL= 148.8 Msg3, MIL=148.1 PUCCH (22 bits), MIL=150.6 |

For a RedCap UE with 2 Rx antennas, considering MIL of the worst channel other than data channels, a coverage degradation of approximately 1.0 dB, 3.0 dB and 0.5 dB, respectively, is observed for Msg2, Msg3 and PUCCH (22 bits) at 2.6 GHz carrier frequency.

For a RedCap UE with 1 Rx antenna, considering MIL of the worst channel other than data channels, a coverage degradation of approximately of 5.9 dB, 2.3 dB, 3.0 dB and 0,5 dB, respectively, is observed for Msg2, Msg4, Msg3 and PUCCH (22 bits) at 2.6 GHz carrier frequency.

### FR2, Indoor with the carrier frequency of 28 GHz

Figure 5, Figure 6, and Figure 7 show respectively MCL, available path loss, and MIL for different downlink and uplink control and data channels, at 28 GHz carrier frequency.

Figure 5: Link budget performance, MCL, 28 GHz.

Figure 6: Link budget performance, Available path loss, 28 GHz.

Figure 7: Link budget performance, MIL, 28 GHz.

For 28 GHz, considering the coverage recovery metrics MCL, MPL and MIL, Table 7 shows the bottleneck channel for the reference UE and the channels that need coverage recovery for the RedCap UE.

Table 7: Bottleneck channel for the reference UE and the channels that need coverage recovery for the RedCap UE, 28 GHz

|  | Bottleneck channel for reference UE | Need coverage recovery RedCap (100 MHz, 1 Rx) | Need coverage recovery RedCap (50 MHz, 1 Rx) |
| --- | --- | --- | --- |
| MCL | Msg 4, MCL=109.9 | Msg2, MCL=104.3 Msg4, MCL=105.4 PDSCH, MCL=106.3 | PDCCH CSS, MCL=108 PDCCH USS, MCL=108 Msg2, MCL=104.3 Msg4, MCL=105.4 PDSCH, MCL=105.8 |
| MPL | Msg 4, MPL=127.9 | Msg2, MPL=122.3 Msg 4, MPL=123.4 PDSCH, MPL=124,3 | PDCCH CSS, MPL=127 PDCCH USS, MPL=127 PDSCH, MPL=123.8 Msg2, MPL=122.3 Msg 4, MPL=123.4 |
| MIL | Msg 4, MIL=127.9 | Msg2, MIL=122.3 Msg 4, MIL=123.4 PDSCH, MIL=124.3 | PDCCH CSS, MIL=127 PDCCH USS, MIL=127 PDSCH, MIL=123.8 Msg2, MIL=122.3 Msg 4, MIL=123.4 |

For a RedCap UE with 100 MHz BW and 1 Rx antenna, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 5.6 dB, 4.5 dB and 3.6 dB, respectively, is observed for Msg2, Msg4 and PDSCH at 28 GHz carrier frequency.

For a RedCap UE with 50 MHz BW and 1 Rx antenna, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 0.9 dB, 0.9 dB, 4.1 dB, 5.6 dB and 4.5 dB, respectively, is observed for PDCCH USS, PDCCH CSS, PDSCH, Msg2 and Msg4 at 28 GHz carrier frequency.

### FR1, Rural with the carrier frequency of 0.7 GHz

Figure 8, Figure 9, and Figure 10 show respectively MCL, available path loss, and MIL for different downlink and uplink control and data channels, at 0.7 GHz carrier frequency.

Figure 8: Link budget performance, MCL, 0.7 GHz.

Figure 9: Link budget performance, Available path loss, 0.7 GHz.

Figure 10: Link budget performance, MIL, 0.7 GHz.

For 700 MHz, considering the coverage recovery metrics MCL, MPL and MIL, Table 8 shows the bottleneck channel for the reference UE and the channels that need coverage recovery for the RedCap UE.

Table 8: Bottleneck channel for the reference UE and the channels that need coverage recovery for the RedCap UE, 700 MHz.

|  | Bottleneck channel for reference UE | Need coverage recovery RedCap (2 Rx) | Need coverage recovery RedCap (1 Rx) |
| --- | --- | --- | --- |
| MCL | PUSCH, MCL=133.8 | - | - |
| MPL | PUSCH, MPL=125.2 | PRACH, MPL=123.9 PUSCH, MPL=122.2 PUCCH (22 bits), MPL=123.4 Msg3, MPL=124.3 | PRACH, MPL=123.9 PUSCH, MPL=122.2 PUCCH (22 bits), MPL=123.4 Msg3, MPL=124.3 |
| MIL | PUSCH, MIL=142.8 | PUSCH, MIL=139.8 Msg3, MIL=141.9 | PUSCH, MIL=139.8 Msg3, MIL=141.9 |

For a RedCap UE, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 3.0 dB and 0.9 dB is observed for PUSCH and Msg3, respectively, at 700 MHz carrier frequency.

As shown in Table 8, the bottleneck channel is PUSCH. As mentioned before, for data channels coverage can be improved by simply lowering the data rates. Thus, it might be more interesting to consider the next worst channel as the bottleneck channel. This is reflected in the summary in Table 9. First, Msg3 with 148.9 dB MIL then becomes the bottleneck channel. For RedCap UEs, approximately 3.0 dB coverage recovery would be needed for Msg3.

Table 9: Bottleneck channel for the reference UE, based on the worst channel other than PUSCH, and the channels that need coverage recovery for the RedCap UE, 700 MHz.

|  | Bottleneck channel for reference UE | Need coverage recovery RedCap (2 Rx) | Need coverage recovery RedCap (1 Rx) |
| --- | --- | --- | --- |
| MCL | Msg3, MCL=135.9 | - | - |
| MPL | PUCCH (22 bits), MPL=126.4 | PRACH, MPL=123.9 PUCCH (2 bits), MPL=125.4 PUCCH (22 bits), MPL=123.4 Msg3, MPL=128.3 | PRACH, MPL=123.9 PUCCH (2 bits), MPL=125.4 PUCCH (22 bits), MPL=123.4 Msg3, MPL=124.3 |
| MIL | Msg3, MIL=144.9 | Msg3, MIL=141.9 | Msg3, MIL=141.9 |

For a RedCap UE, considering the MIL of the bottleneck channel other than the data channels of the reference UE, a coverage degradation of approximately 3.0 dB is observed for Msg3 at 700 MHz carrier frequency.

### FR1, Urban with the carrier frequency of 4 GHz

Figure 11, Figure 12, and Figure 13 show respectively MCL, available path loss, and MIL for different downlink and uplink control and data channels, at 4 GHz carrier frequency.

Figure 11: Link budget performance, MCL, 4 GHz.

Figure 12: Link budget performance, Available path loss, 4 GHz.

Figure 13: Link budget performance, MIL, 4 GHz.

For 4 GHz, considering the coverage recovery metrics MCL, MPL and MIL, Table 10 shows the bottleneck channel for the reference UE and the channels that need coverage recovery for the RedCap UE.

Table 10: Bottleneck channel for the reference UE, the channels that need coverage recovery for the RedCap UE, 4 GHz.

|  | Bottleneck channel for reference UE | Need coverage recovery RedCap (2 Rx) | Need coverage recovery RedCap (1 Rx) |
| --- | --- | --- | --- |
| MCL | Msg2, MCL=138.8 | Msg2, MCL=135.4 Msg4, MCL=138.1 | PDCCH CSS, MCL=137.9 Msg 2, MCL= 130.6 Msg4, MCL=134.2 PDSCH, MCL=138 |
| MPL | Msg2, MPL=112.8 | PDCCH CSS, MPL=108.9 Msg 2, MPL= 106.4 Msg4, MPL=109.1 PUSCH, MPL=110.3 PDSCH, MPL=112.7 | SSB, MPL=110.6 PDSCH, MPL=109 PDCCH CSS, MPL=105.8 PDCCH USS, MPL=109.9 Msg 2, MPL= 101.6 Msg4, MPL=105.2 PUSCH, MPL=110.3 |
| MIL | Msg2, MIL=143.5 | Msg 2, MIL= 137.1 Msg4, MIL=139.8 PUSCH, MIL=141 PDSCH, MIL=143.4 PDCCH CSS, MIL=142.7 | SSB, MIL=141.3 PDSCH, MIL=139.7 PDCCH CSS, MIL=139.6 Msg 2, MIL=132.3 Msg4, MIL=135.9 PUSCH, MIL=141 |

For a RedCap UE with 2 Rx antennas, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 0.8 dB, 6.4 dB, 3.7 dB and 2.5 dB, respectively, is observed for PDCCH CSS, Msg2, Msg4 and PUSCH at 4 GHz carrier frequency.

For a RedCap UE with 1 Rx antenna, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 2.2 dB, 3.8 dB, 3.9 dB, 11.2 dB, 7.6 dB and 2.5 dB, respectively, is observed for SSB, PDSCH, PDCCH CSS, Msg2, Msg4 and PUSCH at 4 GHz carrier frequency.

## Potential coverage recovery techniques

Our link-budget evaluation show that depending on frequency band, we may potentially need coverage recovery for different channels, signals or messages. However, considering some combinations of complexity reduction techniques may lead to unfeasible needed coverage recovery values. As an example, for carrier frequency of 4 GHz, considering RedCap UE with 1 Rx antenna and reduced antenna efficiency, a coverage compensation of 11.2 dB is required for Msg2.

Some combinations of complexity reduction techniques may lead to unfeasibly large required coverage recovery measures.

In this subsection, we discuss potential coverage recovery solutions that may be considered. Here, we itemize solutions for different channels, signals, and messages in Table 11. However, depending on the findings from the coverage evaluation study, coverage enhancement is needed only for channels and/or messages that end up having RedCap UE coverage lower than the target NR coverage.

Table 11: Potential coverage recovery techniques

| Channel | Coverage recovery techniques |
| --- | --- |
| SSB and system information acquisition | - For RedCap use cases, the acquisition time needed for SSB acquisition can be relaxed. The longer acquisition time allows multiple trials of SSB acquisition. This improves coverage. - With relaxed SSB acquisition time, the UE may also try to accumulate the detection metrics over multiple SSB transmissions in the same beam to improve coverage. - The “keep trying” method and metric accumulation can be also used for improving the coverage of system information acquisition. |
| ## PDCCH | - Reduce DCI sizes to allow for lower code rates for a given aggregation level. Since RedCap UEs have smaller BW and reduced capability, some of the DCI fields may be either reduced or removed. - Increase the largest aggregation level beyond 16, e.g. 24 or 32 (also, AL 8 and 16 for the cases that are not already supported), when possible. Also, as the RedCap UEs have reduced BW, using a higher aggregation level may be achieved e.g., by time repetition. - Consider frequency-hopped CORESET for RedCap UEs to increase frequency diversity. - Take the advantage of PDCCH repetition. |
| ## PDSCH | - Consider frequency hopping to increase frequency diversity. - Take the advantage of PDSCH repetition. |
| ## PRACH | - Repeat random access attempts. - Use longer PRACH preambles. |
| ## PUCCH | - Use a longer PUCCH format. - Take the advantage of PUCCH repetition. |
| ## PUSCH | - Use slot aggregation. Rel-16 already introduces slot aggregation level up to 16, which allows the same payload to be repeated in 16 slots. - Consider frequency hopping to increase frequency diversity. |

# System-level simulation

## Simulator and assumptions

To meet the new objective on the need for capacity and spectral efficiency evaluations (see introduction) we have carried out system simulations using multiple cells with wrap-around. For FR1 a hexagonal 7x3 deployment has been used, and for FR2 a rectangular deployment with 12 indoor base stations.

Figure 14: Examples of 2.6 GHz and 28 GHz frequency band deployments.

Users are distributed uniformly, and a gain-matrices set up for all the radio links. The table below lists the simulated cases:

Table 12: System simulation cases.

| Case: | 2.6 GHz | 28 GHz |
| --- | --- | --- |
| System BW: | 100 MHz | 100 MHz |
| Ref UE: | 100 MHz 4Rx Max 256QAM in DL Max 64QAM in UL | 100 MHz 2Rx Max 64QAM in DL Max 64QAM in UL |
| RedCap UE: | 20 MHz 1Rx and 2Rx Max 64QAM in DL Max 16QAM in UL | 100 MHz 1Rx and 2Rx Max 16QAM in DL Max 16QAM in UL |

Further, power control is used in the uplink, no carrier-aggregation is used, MIMO is used for reference users and for RedCap users with 2Rx, and MU-MIMO is used in the downlink. For the indoor base stations in the office scenario for FR2 a down-tilt of 6 degress is applied.

## SLS results

For the simulation case with 2Rx for RedCap in the 2.6 GHz frequency band, the downlink user throughput (for all users) is shown in Figure 15.

Figure 15: DL user throughput in 2.6 GHz band with reference UEs and RedCap UEs (2Rx).

It is seen that the 95th-percentile of the user throughput for RedCap (green curve=100% RedCap) is considerably lower than for MBB reference case (blue curve=100% MBB) due to the reduction of BW to 20 MHz, lower 64QAM modulation, and half the number of Rx antennas. Taking also the mixed traffic simulations into account (red and yellow curves), it is seen that the 95th-percentile indistinguishable from the MBB 95th-precentile (yellow curve is on top of the blue and red in the figure), and is therefore unaffected by the introduction of RedCap users.

Looking at the median, it is seen that also is unaffected by the introduction of RedCap (dashed curves). With a 50% RedCap user fraction (yellow curve), it would be expected that the median would be impacted more. However, in our simulator the percentiles are rather for ‘packet throughput‘ and therefore they are more dependent on the traffic load from RedCap users. That is, according to the assumed traffic model a MBB user will in average generate an offered load of 2x107 bits/s (0.5 MB payload every 200 ms), whereas a RedCap user will in average generate an offered load of 4x105 bits/s (0.1 MB payload every 2 s). That is, a MBB user will generate 50 times more traffic than a RedCap user, so even with a 50-50 split of MBB and RedCap users, the RedCap users will only produce a data volume which is less than 2% of the total data volume.

The 5th-percentile, however, is decreasing with the increase of the fraction of RedCap users. Again, this is the ‘data packet‘ percentile and the decline is reflecting that an increasing part of the 5th-percentile traffic comes from RedCap users. This does not, however, imply that there is any decrease in MBB user performance, and this will be become clear further down in the tables where we look at at the RedCap and MBB user throughput separately.

Figure 16: DL user throughput for 1Rx for RedCap in 2.6 GHz band.

Figure 16 is also showing the 2.6 GHz simulation case but for RedCap users with 1Rx. Mainly it is the 100% RedCap case (green curve) which is impacted, the 95th-percentile drops from 137 Mbps to 74 Mbps due to the antenna reduction and the loss of MIMO. However, the mixed traffic cases are not affected much due to the low relative data volume from RedCap.

Figure 17: UL user throughput for 1Rx and 2Rx for RedCap in 2.6 GHz band.

Looking at the uplink user throughput in Figure 17, it is seen first of all the the 95th-percentiles are not very high due the small UL fraction of 23% from the TDD configurations. Second, it is seen that very much in the same way as for the DL case the mixed traffic cases coincide with the pure MBB reference case (blue curves which are again underneath the yellow curves). The reason is the same, even with a 50-50 split of MBB reference users and RedCap users, the RedCap contribution to the overall traffic volume is only 2%.

Table 13: 2.6GHz simulation case.

|  |  | DL- 2Rx: |  |  |  | DL – 1Rx: |  |  |  | UL: |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RedCap user fraction: | 0 | 0.25 | 0.5 | 1.0 | 0 | 0.25 | 0.5 | 1.0 | 0 | 0.25 | 0.5 | 1.0 |
| Low Loading RU~30% | RU | 0.30 | 0.30 | 0.29 | 0.29 | 0.30 | 0.30 | 0.30 | 0.31 | 0.28 | 0.28 | 0.28 | 0.34 |
|  | 50% UPT (eMBB UEs) | 631 | 631 | 637 |  | 629 | 632 | 634 |  | 47 | 47 | 47 |  |
|  | 5% UPT (eMBB UEs) | 315 | 312 | 321 |  | 313 | 313 | 317 |  | 3.0 | 3.0 | 3.0 |  |
|  | Cell avg. SE (bps/Hz) (eMBB UEs) | 6.3 | - | - |  | 6.3 | - | - |  | 0.4 | - | - |  |
|  | 50% UPT (RedCap UEs) |  | 85 | 87 | 85 |  | 63 | 63 | 62 |  | 12 | 12 | 11 |
|  | 5% UPT (RedCap UEs) |  | 37 | 38 | 38 |  | 29 | 28 | 28 |  | 2.7 | 2.7 | 2.2 |
|  | Cell avg. SE (bps/Hz) (RedCap UEs) |  | - | - | 4.2 |  | - | - | 2.9 |  | - | - | 0.4 |
|  | 50% UPT (All UEs) | 631 | 628 | 632 | 85 | 629 | 631 | 630 | 62 | 47 | 46 | 46 | 11 |
|  | 5% UPT (All UEs) | 315 | 301 | 274 | 38 | 313 | 302 | 275 | 28 | 3.0 | 3.0 | 3.0 | 2.2 |
|  | Cell avg. SE (bps/Hz) (All UEs) | 6.3 | 6.2 | 6.2 | 4.2 | 6.3 | 6.2 | 6.1 | 2.9 | 0.4 | 0.4 | 0.4 | 0.4 |

Looking specifically at the MBB user throughput degradation in the table above, it is seen that for low load coresponding to cell throughput of 180 Mbps (30% RU), and the DL case with 2Rx for RedCap, the median does not drop from 631 Mbps  when the fraction of RedCap users is increased from 0% to 50% (the slight increase is due to the slightly lower load for 50% RedCap users). Unlike from the figures showing the average user throughput above (for all types of users), it is here seen when looking at the MBB reference specifically that there is no drop in performance neither for the 5th-percentile with increaseing RedCap fraction. Also for the cases of DL with 1Rx for RedCap and for UL, the MBB performance for both the median and the 5-percentile is unaffected by the introduction of RedCap users. Again, this is due to the relatively very low data volume produced by RedCap with the assumed traffic models.

The spectral efficiency in DL has a minor degradation with the introduction of small RedCap data volume, but is really only negatively impacted when there is only 100% RedCap users in the system, which is not surprising. In UL the spectral efficiency is essentially unchanged.

From the discussion above we make the following observation:

Even with a 50% fraction of RedCap users, the impact on MBB user performance is negligible since the data volume for RedCap users is relatively small (2%).

More results will be provided in the ongoing email discssion for collecting initial evaluation results.

# Conclusion

In the previous sections, we discussed link budget evaluation, the impact of the complexity reduction on coverage, and possible coverage recovery solutions for different physical channels. We made the following observations:

Observation 1	For all the considered scenarios, the impact of the complexity reduction is more considerable for DL channels.

Observation 2	For a RedCap UE, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 3.0 dB is observed for PUSCH at 2.6 GHz carrier frequency.

Observation 3	For a RedCap UE with 2 Rx antennas, considering MIL of the worst channel other than data channels, a coverage degradation of approximately 1.0 dB, 3.0 dB and 0.5 dB, respectively, is observed for Msg2, Msg3 and PUCCH (22 bits) at 2.6 GHz carrier frequency.

Observation 4	For a RedCap UE with 1 Rx antenna, considering MIL of the worst channel other than data channels, a coverage degradation of approximately of 5.9 dB, 2.3 dB, 3.0 dB and 0,5 dB, respectively, is observed for Msg2, Msg4, Msg3 and PUCCH (22 bits) at 2.6 GHz carrier frequency.

Observation 5	For a RedCap UE with 100 MHz BW and 1 Rx antenna, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 5.6 dB, 4.5 dB and 3.6 dB, respectively, is observed for Msg2, Msg4 and PDSCH at 28 GHz carrier frequency.

Observation 6	For a RedCap UE with 50 MHz BW and 1 Rx antenna, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 0.9 dB, 0.9 dB, 4.1 dB, 5.6 dB and 4.5 dB, respectively, is observed for PDCCH USS, PDCCH CSS, PDSCH, Msg2 and Msg4 at 28 GHz carrier frequency.

Observation 7	For a RedCap UE, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 3.0 dB and 0.9 dB is observed for PUSCH and Msg3, respectively, at 700 MHz carrier frequency.

Observation 8	For a RedCap UE, considering the MIL of the bottleneck channel other than the data channels of the reference UE, a coverage degradation of approximately 3.0 dB is observed for Msg3 at 700 MHz carrier frequency.

Observation 9	For a RedCap UE with 2 Rx antennas, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 0.8 dB, 6.4 dB, 3.7 dB and 2.5 dB, respectively, is observed for PDCCH CSS, Msg2, Msg4 and PUSCH at 4 GHz carrier frequency.

Observation 10	For a RedCap UE with 1 Rx antenna, considering the MIL of the bottleneck channel of the reference UE, a coverage degradation of approximately 2.2 dB, 3.8 dB, 3.9 dB, 11.2 dB, 7.6 dB and 2.5 dB, respectively, is observed for SSB, PDSCH, PDCCH CSS, Msg2, Msg4 and PUSCH at 4 GHz carrier frequency.

Observation 11	Some combinations of complexity reduction techniques may lead to unfeasibly large required coverage recovery measures.

Observation 12	Even with a 50% fraction of RedCap users, the impact on MBB user performance is negligible since the data volume for RedCap users is relatively small (2%).

Based on the discussion in the previous sections we propose the following:

Proposal 1	To determine the target performance requirement and the bottleneck channel(s), Option 3 is used, i.e., The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario. Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL

Proposal 2	Maximum isotropic loss (MIL) is used as the coverage evaluation metric.

# References

RP-201677, “Revised SID on Study on support of reduced capability NR devices”, Ericsson, RAN#89e, Electronic Meeting, Sept. 14 – 18, 2020.

RP-201676, “Status report for Study on support of reduced capability NR devices”, Ericsson, RAN#89e, Electronic Meeting, Sept. 14 – 18, 2020.

RP-200860, “Status report for Study on NR coverage enhancements”, China Telecom, RAN#88e, Electronic Meeting, June 29 – July 3, 2020.

R1-2007481, “Email discussion summary #4 for Study on support of reduced capability NR devices”, Rapporteur (Ericsson), RAN1#102-e post-meeting, Electronic Meeting, Agu. 17th – Agu. 28th, 2020.

# Appendix

### Simulation assumptions

Table 14: Link-level simulations assumptions for reference UE.

| Parameter name | FR1, Urban | FR2, Indoor | FR1, Rural | FR1, Urban |
| --- | --- | --- | --- | --- |
| # Carrier Frequency | # 2.6 GHz (TDD) | # 28 GHz (TDD) | # 0.7 GHz (FDD) | # 4 GHz (TDD) |
| # BWP BW | # 100 MHz (273 PRBs) | # 100 MHz (66 PRBs) | # 20 MHz (106 PRBs) | # 100 MHz (273 PRBs) |
| # SCS | # 30 kHz | # 120 kHz | # 15 kHz | # 30 kHz |
| # Frame structure for TDD | # DDDDDDDSUU (S: 6D:4G:4U) | # DDDSU (S: 10D:2G:2U) |  | # DDDDDDDSUU (S: 6D:4G:4U) |
| # # of gNB TX chains | # 4 | # 2 | # 2 | # 4 |
| # # of gNB RX chains | # 4 | # 2 | # 2 | # 4 |
| # # of UE TX chains | # 1 | # 1 | # 1 | # 1 |
| # # of UE RX chains | # 4 | # 2 | # 2 | # 4 |
| # Channel Model | # TDL-C, NLOS | # TDL-A, NLOS | # TDL-C, NLOS | # TDL-C, NLOS |
| # UE antenna correlation | # Low | # Low | # Low | # Low |
| # delay spread | # 300 ns | # 30 ns | # 300 ns | # 300 ns |
| # UE velocity | # 3 km/h | # 3 km/h | # 3 km/h | # 3 km/h |

### RedCap assumptions

Table 15: Link-level simulations assumptions for RedCap.

| RedCap parameter | FR1, Urban | FR2, Indoor | FR1, Rural |
| --- | --- | --- | --- |
| # BW | # 20 MHz (51 PRBs) | # 50 (32 PRBs) or 100 MHz (66 PRBs) | # 20 MHz (106 PRBs) |
| # # of UE TX chains | # 1 | # 1 | # 1 |
| # # of UE RX chains | # 1 or 2 | # 1 or 2 | # 1 or 2 |

### Channel-specific assumptions and link-level simulation results

### SSB

Table 16: SSB parameters.

| Parameter | Value |
| --- | --- |
| (Residual) frequency offset (UE) | 0.1 ppm |
| SS burst set periodicity | 20 ms |
| Precoder | Precoder cycling |
| Number of transmissions (shots) | 4 |
| BLER target for PBCH | 1% |

| Figure 18: PBCH performance for 700 MHz. | Figure 19: PBCH performance for 2.6 GHz. |
| --- | --- |

Figure 20: PBCH performance for 4 GHz.

| Figure 21: PBCH performance for 28 GHz. (120 kHz SCS) | Figure 22: Impact of bandwidth reduction on PBCH performance for 240 kHz SSB SCS. |
| --- | --- |

### PRACH

The channel-specific parameters and performance targets for PRACH are shown in Table 17.

Table 17: PRACH parameters.

| Parameter | Value |
| --- | --- |
| Performance target | 1% missed detection at 0.1% false alarm probability |
| MCS index/TBS (or modulation and code rate, or PRACH format) | 0.7 GHz: Format 0 (1.25 KHz SCS); BW = 1.04875 MHz 2.6 GHz: Format B4 (30 KHz SCS); BW = 4.17 MHz 4 GHz: Format B4 (30 KHz SCS); BW = 4.17 MHz 28 GHz: Format B4 (120 KHz SCS); BW = 16.68 MHz |
| Number of transmissions | 1 |
| Rx combining | non-coherent combining of branches |
| Propagation delay (RTT) | 0.7 GHz (rural): Uniformly distributed [0, 23] µs (ISD 6 km) 2.6 GHz (urban): Uniformly distributed [0, 2.7] µs (ISD 700 m) 28 GHz (indoor): Uniformly distributed [0, 0.077] µs (ISD 20 m) |
| Frequency error | 0.10 ppm at the UE, none at the gNB |

Figure 23: Missed detection rates of PRACH for different bands.

### PDCCH

Table 18: PDCCH parameters.

| Parameter | Value |
| --- | --- |
| DCI payload size | 40 bits+CRC |
| Aggregation level (AL) | 16 |
| CORESET | 2 symbols x 48 PRBs |
| Precoding | Precoder cycling at CCE level (REG bundle=6) |
| BLER target for PDCCH | 1% |

| Figure 24: PDCCH BLER performance for 700 MHz. | Figure 25: PDCCH BLER performance for 2.6 GHz. |
| --- | --- |

Figure 26: PDCCH BLER performance for 4 GHz.

| Figure 27: PDCCH BLER performance for 28 GHz. | Figure 28: Impact of bandwidth reduction on PDCCH BLER performance in FR2 (1.9 MHz each side guardband). |
| --- | --- |

### PDSCH

Table 19 shows the channel-specific parameters and performance targets for PDSCH (eMBB data), Msg 2, and Msg 4. We have adjusted the TBS and MCS accordingly, so that we can obtain the targeted data rate for the reference UE. Then, the target data rates for RedCap UEs are adjusted lower to reflect the BW constraint. As an example: for 2.6 GHz with 30 kHz SCS, there are 51 PRBs in limited bandwidth of 20 MHz. We use the MCS value from the reference UE, i.e. MCS=0. So, we end up with TBS=1480. Considering the targeted BLER, this then give 1480/0.005*(1-0.1) = 2.7 Mbps. In essence, for eMBB data, the coverage is assessed based on the same spectral efficiency expected from the reference and RedCap UEs.

Table 19: PDSCH (eMBB data), Msg 2, and Msg 4 parameters.

| Parameter | Value for PDSCH eMBB data | Value for Msg 2 | Value for Msg 4 |
| --- | --- | --- | --- |
| FDRA (reference UE) | 2.6 GHz and 4 GHz: 200 PRBs 28 GHz: 60 PRBs for 25 Mbps and 30 PRBs for 12.5 700 MHz: 40 PRBs | 3 PRBs | 2.6 GHz and 4 GHz: 36 PRBs 28 GHz: 18 PRBs 700 MHz: 36 PRBs |
| TDRA | 12 OFDM symbols |  |  |
| Waveform | CP-OFDM |  |  |
| DMRS | Type I, 2 DMRS symbol, no multiplexing with data. | Type I, 3 DMRS symbol, no multiplexing with data | Type I, 3 DMRS symbol, no multiplexing with data |
| Payload | target data rate 10 Mbps (30 kHz SCS): TBS =5640 target data rate 1 Mbps (15 kHz SCS): TBS = 1128 target data rate 25 Mbps (120 kHz SCS): TBS = 3624 target data rate 12.5 Mbps (120 kHz SCS): TBS = 1800 | 9 bytes | 130 bytes |
| MCS index/TBS | Use Table 5.1.3.1-1 target data rate 10 Mbps (30 kHz SCS): MCS=0 target data rate 1 Mbps (15 kHz SCS): MCS = 0 target data rate 25 Mbps (120 kHz SCS): MCS = 3 | MCS=0 | Use Table 5.1.3.1-1 MCS=0 for FR1 MCS = 3 for FR2 |
| Number of transmissions | No HARQ |  |  |
| Rx combining | MRC |  |  |
| Diversity scheme | Precoder cycling; PRB bundle size of 2 |  |  |
| Performance target | FR1: 10 Mbps (Urban), 1 Mbps (Rural); 10% iBLER FR2: 25 Mbps; 10% iBLER | 10% iBLER | 10% iBLER |

| Figure 29: PDSCH BLER performance for 700 MHz. | Figure 30: PDSCH BLER performance for 2.6 GHz. |
| --- | --- |

| Figure 31: PDSCH BLER performance for 28 GHz. | Figure 32: PDSCH BLER performance for 4 GHz. |
| --- | --- |

| Figure 33: Msg2 BLER performance for 700 MHz. | Figure 34: Msg2 BLER performance for 2.6 GHz. |
| --- | --- |

| Figure 35: Msg2 BLER performance for 28 GHz. | Figure 36: Msg2 BLER performance for 4 GHz. |
| --- | --- |

| Figure 37: Msg 4 BLER performance for 700 MHz | Figure 38: Msg 4 BLER performance for 2.6 GHz. |
| --- | --- |

| Figure 39: Msg 4 BLER performance for 28 GHz | Figure 40: Msg 4 BLER performance for 4 GHz. |
| --- | --- |

| Figure 41: Data rate, 2.6 GHz. | Figure 42: Data rate, 28 GHz. |
| --- | --- |
| Figure 43: Data rate, 0.7 GHz. | Figure 44: Data rate, 4 GHz. |

### PUCCH

Table 20: PUCCH parameters.

| Parameter | Value for PUCCH format 1 | Value for PUCCH format 3 |
| --- | --- | --- |
| Payload and format | 2 bits (A/N) | 4/11/22 bits (A/N+SR/UCI) |
| Number of symbols | 14 | 14 |
| Number of PRBs | 1 | 1 |
| Frequency hopping | At UL BWP edge | At UL BWP edge |
| DMRS | every even symbol according to the specification | Additional DMRS configured (4 symbols) |
| Performance target for PUCCH | 1% D2A and Aerr, 0.1% N2A | BLER 1% for 4 bits, 10% and 1% for the rest |

| Figure 45: PUCCH format 1 performance for 700 MHz. | Figure 46: PUCCH format 3 performance for 700 MHz. |
| --- | --- |

| Figure 47: PUCCH format 1 performance for 2.6 GHz. | Figure 48: PUCCH format 3 performance for 2.6 GHz. |  |
| --- | --- | --- |
| Figure 49: PUCCH format 1 performance for 28 GHz. |  | Figure 50: PUCCH format 3 performance for 28 GHz. |

| Figure 51: PUCCH format 1 performance for 4 GHz. | Figure 52: PUCCH format 3 performance for 4 GHz. |
| --- | --- |

Table 21: Required SNR (dB) to achieve performance target for PUCCH.

| Carrier frequency | 700 MHz | 2.6 GHz |  | 4 GHz |  | 28 GHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BW (MHz) | 20 | 20 | 100 (ref) | 20 | 100 (ref) | 50 | 100 | 200 (ref) |
| PF1 (2 bits) | -2.92 | -6.75 | -6.62 | -6.81 | -6.78 | -3.02 | -3.02 | -3.02 |
| PF3 (4 bits) | -5.03 (1%) | -8.50 (1%) | -8.41 (1%) | -8.49 (1%) | -8.44 (1%) | -4.69 (1%) | -4.98 (1%) | -4.80 (1%) |
| PF3 (11 bits) | -3.38 (1%) -7.23 (10%) | -7.34 (1%) -10.03(10%) | -7.30 (1%) -9.97 (10%) | -7.32 (1%) -9.99 (10%) | -7.30 (1%) -9.97 (10%) | -3.04 (1%) -6.95 (10%) | -3.37 (1%) -7.20 (10%) | -3.15 (1%) -7.00 (10%) |
| PF3 (22 bits) | -0.91 (1%) -4.53 (10%) | -5.43 (1%) -7.81 (10%) | -5.35 (1%) -7.76 (10%) | -5.38 (1%) -7.77 (10%) | -5.35 (1%) -7.77 (10%) | -0.57 (1%) -4.21 (10%) | -0.91 (1%) -4.46 (10%) | -0.67 (1%) -4.28 (10%) |

### PUSCH

Table 22: PUSCH (eMBB data) and Msg3 parameters.

| Parameters | Value for PUSCH eMBB data | Value for Msg3 |
| --- | --- | --- |
| Performance target | FR1: 1 Mbps (Urban), 100 kbps (Rural); 10% iBLER FR2: 5 Mbps; 10% iBLER | 10% iBLER |
| FDRA (reference UE) | 1 Mbps: 30 PRBs 100 kbps: 4 PRBs 5 Mbps (100 MHz): 66 PRBs 5 Mbps (50 MHz): 32 PRBs | 2 PRBs |
| TDRA | 14 OFDM symbols | 14 OFDM symbols |
| Waveform | DFT-s-OFDM | DFT-s-OFDM |
| DMRS | Type 1, 1+1 DMRS, no multiplexing with data. | Type 1, 1+1+1 DMRS, no multiplexing with data |
| Payload | target data rate 1 Mbps (30 kHz SCS): TBS = 552 target data rate 100 kbps (15 kHz SCS): TBS = 128 target data rate 5 Mbps (100 MHz, 120 kHz SCS): TBS = 736 target data rate 5 Mbps (50 MHz, 120 kHz SCS): TBS = 736 | 56 bits |
| MCS index/TBS (or modulation and code rate) | target data rate 1 Mbps (30 kHz SCS): MCS=3 target data rate 100 kbps (15 kHz SCS): MCS = 6 target data rate 5 Mbps (100 MHz, 120 kHz SCS): MCS = 1 target data rate 5 Mbps (50 MHz, 120 kHz SCS): MCS = 4 MCS Table 6.1.4.1-2 (TS38.214) | MCS0 MCS Table 6.1.4.1-1 (TS38.214) |
| Number of transmissions | 1 | 1 |
| Rx combining | MRC | MRC |
| Diversity scheme | no frequency hopping | no frequency hopping |
| Comments | Required SNR used in the link budget table is the minimum SNR where both BLER (10%) and data rate requirements can be met. | - |

| Figure 53: Data rates of PUSCH (data) for different bands. | Figure 54: Block error rates of PUSCH (data) for different bands. |
| --- | --- |

Figure 55: Block error rates of Msg3 for different bands.