3GPP TSG RAN WG1 Meeting #103-E					      R1-2008994

e-Meeting, October 26th – November 13th, 2020

Source:	Intel Corporation

Title:	                       On PDCCH monitoring reduction techniques during active time

Agenda item:	8.7.1.3

Document for:	Discussion and Decision

Introduction

The RAN WG approved work item on Power Saving Enhancements [1]. The work item includes the following objective for power saving enhancements during active time.

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4] - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1] - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

Moreover, the following was agreed for evaluation of DCI based power saving adaptations during active time:

Agreements: - Reusing power model in TR38.840 for evaluation of DCI-based power saving adaptation schemes. - Note: company reporting additional power model for missing state or update is not precluded. Agreements: - Company should report assumptions used for periodic measurement activities for the Rel-17 DCI-based power saving adaptation evaluation. - The periodic activities defined in TR38.840 can be reused. - Measurement for RLM/BFD every C-DRX cycle can be optionally modelled Update from 8/28 GTW: Agreements: - The performance metrics described in TR38.840 section 8.2 is reused for power saving evaluation of Rel-17 DCI-based power saving adaptation during ActiveTime. - The following Rel-15 / 16 features is recommended of the power consumption as reference for baseline. Company can report the feature(s) being used in the baseline. - DRX - C-DRX cycle 40msec for VoIP - 10ms IAT, 8ms On-duration - Assume max two packets bundled - C-DRX cycle 160msec for FTP - Alt 1: 20 msec IAT, 8ms On-duration - Alt 2: short DRX - 20 ms [or 40ms as optional] IAT, 8ms On-duration - 20 ms for short DRX cycle, 4 cycles - Note: 100 msec IAT, 8ms On-duration can also be used with sufficient justifications that available Rel-15/16 Techniques being used to reduce UE power saving - DCP for DRX adaptation, - DCP offset to DRX ON = 2 ms, other values are not precluded - Cross-slot scheduling adaptation - Minimum K0 can be adapted from 0 to 1 for FR1, 0 to [4] for FR2 - BWP switching, including - MIMO layer adaptation, - Max # of MIMO layer can be adapted from 4 layer to 2 layer for FR1, 2 layer to 1 layer for FR2 - PDCCH monitoring period adaptation - PDCCH monitoring period can be adapted from per slot monitoring to X slot monitoring - X = [2] for FR1 and [8] for FR2 - Bandwidth adaptation - Bandwidth can be adapted from 100MHz to 20MHz for FR1,FFS for FR2 - Note: - BWP transition time type 2 is assumed, BWP transition duration is - 5 slot @ 30kHz SCS for FR1, - 18 slot@120kHz SCS for FR2 - the slot-average power level for BWP transition duration is according to TR38.840 - BWP transition time type 1 can be optional modelled - BWP switching is Y (ms) after last packet/data burst. - Y = [8], other values are not precluded - Whether BWP switching is modeled depends on the assumed UE capability and evaluated schemes. - Scell dormancy assumption for CA capable UEs - FR1 & FR2: SCell dormancy with [160 ms] periodic CSI measurement and reporting - Other settings - CA assumption if configured for CA capable UEs - For FR1, FFS - For FR2, 4*100MHz can be considered. - Assumptions for scheduler - For FR1, no restriction on the beam assumptions being used in each slot - For FR2, up to each company, e.g., gNB equally schedule the slots for UEs targeting to different beams. - Note: the assumptions does not necessary mean to restrict or precluded any implementation. Other assumptions are not precluded and can be reported by companies. - Company to report the used assumption for the interruption and also power savings impact due to presence/absence of interruptions Agreements: Legacy traffic models in TR38.840 can be considered for Rel-17 DCI-based power saving adaptation evaluation, other traffic models can be optionally modelled and company report which traffic model(s) is used.

In this contribution, we express our views on DCI based power saving adaptation and PDCCH monitoring reduction schemes during active time.

# PDCCH monitoring reduction techniques during active time

PDCCH monitoring accounts for major power consumption during active time. In Rel-16, Scell dormancy was introduced which allows for sparse or no PDCCH monitoring activity in one or more activated Scells. However, in the scheduling or Pcell, no PDCCH monitoring reduction was considered. To this end, one of the objective of the Rel17 Power saving WI is to identify suitable techniques to achieve more power saving during active time.

In RAN1 # 102-e, the following baselines were agreed for considerations into evaluation for checking performance of proposed solutions:

  - C-DRX

  - Wake up Signal, DCI format 2_6

  - Cross-slot scheduling adaptation

  - BWP switching adaptation, with one or more of

    - MIMO layer adaptation

    - PDCCH monitoring period adaptation, i.e., periodicity is adapted when UE switches BWP

    - BWP size adaptation

  - SCell dormancy

In order to reduce PDCCH monitoring during active time, PDCCH skipping signal or go-to-sleep (GTS) signal can be considered. DCI may indicate the UE to not monitor PDCCH for a certain duration. UE may optionally monitor measurement signals, such as in a sparse manner, when the UE is not monitoring PDCCH. Alternatively, search space switching scheme, such as the procedure adopted in NR-U, can be considered to achieve similar effect. In our view, PDCCH skipping signal is more flexible and duration can be dynamically adapted.

Following options can be considered for DCI based indication of PDCCH skipping signal:

  - Scheduling DCI formats can be enhanced to include indication of PDCCH skipping signal

    - SCell dormancy indication field can be leveraged

  - DCI format 2_6 can be used, i.e., if the DCI is received during active time, it is used for triggering PDCCH skipping signal

Next, we observe the relative gain of PDCCH skipping signal or GTS with respect to the Rel15 and Rel16 baseline schemes.

## Performance comparison with respect to Rel-15/16 Schemes

We performed SLS evaluations assuming the following parameters in Table 1. Figures 1a, 1b show power consumption and throughput performances for baseline DRX configuration on top of NR Rel-16 power saving schemes. The results are presented for different types of UEs: 5% (cell edge), 50% and 95% (cell center) from SINR CDF statistics. IMT-2020 assumptions are listed in the Appendix in Table 3. As can be seem from Figure 1, go-to-sleep signal provides significant additional power saving gain compared to Rel-15/16 schemes.

Table 1: Simulation parameters

| Simulation parameters | Value |
| --- | --- |
| DRX parameters | DRX cycle length = 160ms On Duration = 8ms Inactivity timer = 20ms, 100ms |
| Traffic model | FTP model 3: 0.5 Mbyte packet size 200ms mean inter-arrival time |
| K0 (Cross-slot scheduling offset) | 1 |
| BWP adaptation | BWP for data – 100 MHz BWP for PDCCH monitoring – 20MHz Switching delay - 5 slot Switching delay after the last PDSCH – 8ms Optionally, PDCCH monitoring periodicity adapted from 1 to 2 slots, when BWP is 100MHz and 20MHz, respectively |
| PDCCH reduction schemes | GTS: 2ms/8ms sleep duration |

Results for C-DRX with IAT 20ms

(a)

(b)

(c)

(d)

Figure 1. Power consumption (a, b) and UPT (c, d) performance for IAT 20ms for GTS with different sleep duration

Observation 1: For C-DRX with IAT = 20ms, when BWP switching is OFF, GTS with 8ms duration provides additional power saving gain in the range of  13 - 24% for cell edge UEs and 29 - 41% for cell center UEs with 0-3% UPT loss for cell edge UEs and 0-4% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Observation 2: For C-DRX with IAT = 20ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain  in the range of  7 - 10% for cell edge UEs and 20 - 24% for cell center UEs with 0-3% UPT loss for cell edge UEs and 0-3% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Results for C-DRX with IAT 100ms

(a)

(b)

(c)

(d)

Figure 2. Power consumption (a, b) and UPT (c, d) performance for IAT 100ms for GTS with different sleep duration

Observation 3: For C-DRX with IAT = 100ms, when BWP switching is OFF, GTS with 8ms duration provides additional power saving gain in the range of  29 - 42% for cell edge UEs and 38 - 52% for cell center UEs with 6-10% UPT loss for cell edge UEs and 5-12% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Observation 4: For C-DRX with IAT = 100ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain  in the range of  20 - 30% for cell edge UEs and 30 - 42% for cell center UEs with 5-8% UPT loss for cell edge UEs and 4-10% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Next, we consider comparison between PDCCH skipping signal and DRX with short DRX.

## Performance comparison with respect to short DRX

For comparison with respect to short DRX, following parameters are used as shown in Table 2.

Table 2: Simulation parameters

| Simulation parameters | Value |
| --- | --- |
| DRX cycle | DRX cycle length = 160ms On Duration = 8ms Inactivity timer = 20ms |
| Traffic model | FTP model 3: 0.5 Mbyte packet size 200ms mean inter-arrival time |
| K0 (Cross-slot scheduling offset) | 1 |
| BWP switching | Default BWP – 100 MHz Reduced BWP – 20MHz Switching delay - 5 slot Switching delay after the last PDSCH – 8ms Optionally, PDCCH monitoring periodicity adapted from 1 to 2 slots, when BWP is 100MHz and 20MHz, respectively |
| Short DRX cycle | Short DRX cycle length = 20ms, Short DRX timer = 4 |
| Short DRX triggering | off / on with MAC CE with 3ms delay |
| PDCCH reduction schemes | GTS with 2ms sleep duration, 8ms sleep duration Reduced PDCCH monitoring periodicity = 2 slots |

Figures 3a,b,c,d shows comparison of power consumption and throughput performances for configuration with IAT=20ms and IAT=40ms.

(a)

(b)

(c)

(d)

Figure 3. Power consumptions and UPT comparison of GTS and Short DRX

Observation 5: For C-DRX with IAT = 20ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain in the range of  9-11% for cell edge UEs and 22-25% for cell center UEs with 0-12% UPT loss for cell edge UEs and 0-23% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS, DRX + Short DRX.

Observation 6: For C-DRX with IAT = 40ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain in the range of  0-11% for cell edge UEs and 8-17% for cell center UEs with 0-15% UPT loss for both cell edge and cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS, DRX + Short DRX.

Hence, it can be observed that GTS in most occasions provides significant power saving gain and UPT loss can be small, depending on scenario and baselines.

Proposal 1: Support DCI-based PDCCH skipping signal in Rel17.

- Either Scheduling DCI format or DCI format 2_6 can be enhanced to include the indication.

Conclusions

In summary, we have following list of observations and proposals:

Observation 1: For C-DRX with IAT = 20ms, when BWP switching is OFF, GTS with 8ms duration provides additional power saving gain in the range of  13 - 24% for cell edge UEs and 29 - 41% for cell center UEs with 0-3% UPT loss for cell edge UEs and 0-4% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Observation 2: For C-DRX with IAT = 20ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain  in the range of  7 - 10% for cell edge UEs and 20 - 24% for cell center UEs with 0-3% UPT loss for cell edge UEs and 0-3% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Observation 3: For C-DRX with IAT = 100ms, when BWP switching is OFF, GTS with 8ms duration provides additional power saving gain in the range of  29 - 42% for cell edge UEs and 38 - 52% for cell center UEs with 6-10% UPT loss for cell edge UEs and 5-12% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Observation 4: For C-DRX with IAT = 100ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain  in the range of  20 - 30% for cell edge UEs and 30 - 42% for cell center UEs with 5-8% UPT loss for cell edge UEs and 4-10% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS.

Observation 5: For C-DRX with IAT = 20ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain in the range of  9-11% for cell edge UEs and 22-25% for cell center UEs with 0-12% UPT loss for cell edge UEs and 0-23% UPT loss for cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS, DRX + Short DRX.

Observation 6: For C-DRX with IAT = 40ms, when BWP switching is ON, GTS with 8ms duration provides additional power saving gain in the range of  0-11% for cell edge UEs and 8-17% for cell center UEs with 0-15% UPT loss for both cell edge and cell center UEs with respect to the baselines, which include DRX, DRX + Cross-slot scheduling, DRX + WUS, DRX + Short DRX.

Proposal 1: Support DCI-based PDCCH skipping signal in Rel17.

- Either Scheduling DCI format or DCI format 2_6 can be enhanced to include the indication.

References

- RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek.

- R1-1902507, “System level evaluation of UE power saving schemes”, RAN1 96.

Appendix

## Table 3: IMT-2020 Simulation assumptions

| Simulation Parameters | Scenario |
| --- | --- |
|  | Dense Urban Macro FR1 DL |
| Carrier Frequency | 4GHz |
| Simulation BW | 100MHz (TDD) |
| Sub-carrier Spacing | 30 kHz |
| Channel Model | IMT UMa A |
| Inter-Site Distance | 200m |
| BS Antenna Configuration | (1,1,4,1,2;2,8) 32 TXRU |
| BS Antenna Spacing | (0.8, 0.5) |
| UE Antenna Configuration | (1,1,1,2,2; 1,2) 4 TXRU (1,1,1,1,2; 1,2) 2 TXRU |
| UE Antenna Spacing | (0.5, 0.5) |
| Beam set at BS | Single beam at 102° electronic downtilt |
| Beam Set at UE | Single Beam Omni-Directional |
| UE Deployment | 80% Indoor; 20% Outdoor |
| Highest Modulation | 256 QAM |
| Channel Coding | LPDC |
| Transmission/Reception Scheme | Multi-user MIMO with 12 layers at BS |
| Scheduler | MU-PF rank adaptation and beam sweeping at BS |
| Feedback | Type II Codebook based CSI |