3GPP TSG RAN WG1 #103-e		                                    		   R1-2009169

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.8.1.1

Source:	InterDigital Inc.

Title:	FR1 baseline coverage performance using LLS

Document for:	Discussion and Decision

# Introduction

In RAN1#102e, simulation assumptions for FR1 were agreed [1]. In this contribution, baseline techniques for both downlink and uplink channels are evaluated. In addition, the bottleneck channel is identified from relative performance analysis using MCL, MIL or MPL.

# FR1 baseline coverage performance

## Link level simulation results for VoIP

In this section, detailed link level simulation results for VoIP scenarios are shown due to variations in the HARQ parameters for VoIP. Evaluation assumptions for the simulation results are shown in Appendix.

VoIP evaluation results : FDD

In Figure 1, rBLER simulation results for VoIP for latency requirements of 50ms and 100ms in the FDD 700MHz are shown. From the figure, it is clear that the target rBLER=2% is reached at SNR=-5.6dB and -7.6dB, respectively.  It is also clear from the figure that longer latency requirement allows more opportunities for retransmission, improving the rBLER performance. The performance is improved nearly by 2dB by extending the latency requirement to 100ms. As explained in our companion contribution [2], extending the latency requirement comes with a risk of incoming VoIP packets to be dropped, if the retransmission continues. Thus, the status of the buffer at the UE should be evaluated.

VoIP evaluation results : TDD

In Figure 2, rBLER simulation results for VoIP for latency requirements of 20ms, 50ms and 100ms in the TDD 4GHz rural scenario are shown. In the simulation, 2 repetitions are assumed for all configurations. As discussed in our companion contribution [3], in the current specification, 2 repetitions is the maximum number allowed for the frame format such as DDDSU. For 20ms, 1 HARQ process and 4 retransmissions are assumed. For the latency requirement of 50ms, 1 and 2 HARQ processes, and 10 and 5 retransmissions are assumed, respectively. Finally, for the latency requirement of 100ms, 1 and 2 HARQ processes, and 20 and 10 retransmissions are assumed, respectively. SNR values required to reach the target rBLER are summarized in Table 1.

From the table it is clear that as the latency requirement increases, lower SNR is required to reach the target value, thanks to more opportunities for retransmission. However, as explained in our companion contribution [2], it should be noted that longer latency requirement allows new VoIP packets to accumulate in the buffer at UE, possibly causing the timer to expire for transmission. Thus, extending the latency requirement longer than necessary may cause degradation in rBLER in practice.

From the rBLER analysis, the following observation is made.

Observation 1: Extending latency requirement from 20ms to 100ms for VoIP improves rBLER performance.

Table 1 SNR required to reach the target rBLER=2%, TDD, 4GHz, VoIP

| Latency requirement | 20ms | 50ms 2 HARQ | 50ms 1 HARQ | 100ms 2HARQ | 100ms 1HARQ |
| --- | --- | --- | --- | --- | --- |
| SNR at rBLER=2% (dB) | -3.5 | -5.3 | -7.3 | -7.7 | -9.7 |

Figure 1 FDD VoIP, 700MHz, 3km/hr, latency requirement = 50ms and 100ms

Figure 2 TDD VoIP, 4GHz, rural channel, 3km/hr, latency requirement = 20ms, 50ms and 100ms

## Relative MCL/MIL/MPL analysis for identification of bottleneck channel

In this section, we summarize the MCL, MPL and MIL obtained using the template agreed in the discussion [4]. For computation of the metrics, antenna gain correction factor ((4b) and (11b) in the template) of 0 dB is assumed. Evaluation assumptions for PUSCH, PDCCH and PDSCH are included in Appendix. Simulation parameters for the FDD and TDD scenario are shown in Table 4 and Table 5, respectively. Details of PDCCH  and PUCCH parameters are shown in Table 6 and Table 7, respectively. The parameters used to compute MCL, MIL and MPL are shown in Table 8. The pathloss parameters were computed using methods described in [5].

Table 2 MCL/MIL/MPL analysis for FR1 FDD 700MHz, 3km/hr

|  | MCL | MIL | MPL |
| --- | --- | --- | --- |
| PUSCH VoIP 50ms | 132.06 | 145.09 | 127.21 |
| PUSCH VoIP 100ms | 134.06 | 147.09 | 129.21 |
| PUSCH eMBB, 100 kbps | 127.86 | 140.89 | 123.01 |
| PUCCH PF1 | 134.90 | 147.94 | 126.32 |
| PUCCH PF3 | 131.41 | 144.44 | 122.82 |
| PDCCH | 148.19 | 158.21 | 136.59 |
| PDSCH, 1 Mbps | 142.79 | 152.81 | 134.93 |

The results in Table 2 indicate that in the FR1 FDD 700MHz scenario, according to the relative comparison, PUCCH PF3 and PUSCH eMBB yield lowest coverage metrics among data and control channels.

Table 3 MCL/MIL/MPL analysis for FR1 TDD 4GHz, rural, 3km/hr

|  | MCL | MIL | MPL |
| --- | --- | --- | --- |
| PUSCH VoIP 20ms | 142.00 | 146.00 | 126.57 |
| PUSCH VoIP 50ms 1 HARQ | 143.80 | 147.80 | 128.37 |
| PUSCH VoIP 50ms 2 HARQ | 145.80 | 149.80 | 130.37 |
| PUSCH VoIP 100ms 1 HARQ | 146.20 | 150.20 | 130.77 |
| PUSCH VoIP 100ms 2 HARQ | 148.20 | 152.20 | 132.77 |
| PUSCH eMBB, 100 kbps | 136.83 | 140.83 | 121.40 |
| PUCCH PF1 | 146.89 | 150.89 | 128.22 |
| PUCCH PF3 | 143.15 | 147.15 | 124.48 |
| PDCCH | 155.54 | 162.55 | 139.88 |
| PDSCH, 1 Mbps | 153.06 | 160.07 | 140.64 |

The results in Table 3 indicates that for 4GHz rural scenario, according to the relative analysis, PUSCH eMBB channel is identified as the bottleneck channel. PUCCH PF1 exhibits superior performance compared to the data channels due to its compact bandwidth in the frequency domain. In addition, the downlink channel exhibits superior performance compared to the VoIP channel due to its use of 64 TXRUs. The performance of PDSCH can be improved by enabling frequency hopping, use of repetitions or enabling HARQ. From the above relative analysis, the following observation is made.

Observation 2: Using relative MCL, MIL or MPL analysis, for FR1 rural FDD,  PUCCH PF3 and PUSCH eMBB are identified as bottleneck data and control channels

Observation 3: Using relative MCL, MIL or MPL analysis, for FR1 rural TDD, PUCCH PF3 and PUSCH eMBB are identified as bottleneck data and control channels

Observation 4: Assuming the short latency requirement, PUSCH VoIP yields inferior coverage performance compared to PUCCH PF1

Based on the above observation, the following proposal is made:

Proposal 1 : For Rel. 17 NR coverage enhancement, study enhancements for PUSCH and PUCCH

# Discussion on the number of HARQ processes for TDD VoIP

As explained in our companion contribution [2], the buffer status at UE for TDD VoIP needs close attention during evaluation. An example of the buffer status is shown in Figure 3 and Figure 4 where the number of packets served by 1 HARQ processes with the maximum transmission of 10 attempts, when 50ms latency requirement is assumed. For 2 HARQ processes, the maximum transmission of 5 attempts is assumed. In the evaluation, 4GHz TDD rural scenario with 30kHz SCS and DDDSU TDD format are assumed. Repetition factor of 2 is assumed. The number of packets in the buffer are analyzed over 100 TDD uplink slots and averaged over 20 iterations. In the figure, the SNR values at which buffer accumulation did not occur are not shown.

From the figure, it is clear that up to -9dB, simulation indicate that the number of packets accumulate in the buffer, if only 1 HARQ process is assumed. Therefore, with 1 HARQ processes, buffer accumulation cannot be avoided at SNR=-9dB. In Figure 2, by 2 HARQ processes with the maximum transmission of 5 attempts, when 50ms latency requirement is assumed. It is clear from the evaluation results that multiple HARQ processes decrease the average number of packets significantly.

Figure 3 Number of packets accumulated in the buffer for 1 HARQ process, 50ms latency requirement, rural scenario with 2DMRS symbols, buffer status tracked over 100 uplink slots, averaged over 100 iterations, 2 repetitions

Figure 4 Number of packets accumulated in the buffer for 2 HARQ processes, 50ms latency requirement, rural scenario with 2DMRS symbols, buffer status tracked over 100 uplink slots, averaged over 100 iterations, 2 repetitions

Observation 5: For low SNR, VoIP packets accumulate at the UE if the number of HARQ processes is set to low value

Based on the above observation, the following proposal is made:

Proposal 2 :  Multiple HARQ processes should be considered for evaluation of baseline simulations and enhancements for VoIP

# Conclusion

In this contribution, the following observations and proposals are made.

Observation 1: Extending latency requirement from 20ms to 100ms for VoIP improves rBLER performance.

Observation 2: Using relative MCL, MIL or MPL analysis, for FR1 rural FDD,  PUCCH PF3 and PUSCH eMBB are identified as bottleneck data and control channels

Observation 3: Using relative MCL, MIL or MPL analysis, for FR1 rural TDD, PUCCH PF3 and PUSCH eMBB are identified as bottleneck data and control channels

Observation 4: Assuming the short latency requirement, PUSCH VoIP yields inferior coverage performance compared to PUCCH PF1

Proposal 1 : For Rel. 17 NR coverage enhancement, study enhancements for PUSCH and PUCCH

Proposal 2 :  Multiple HARQ processes should be considered for evaluation of baseline simulations and enhancements for VoIP

Reference

[1] RAN1 Chairman’s notes, RAN1#102e, Aug., 2020.

[2] R1-2008487, “Discussion on simulation assumptions for VoIP,” InterDigital, RAN1#103-e, Aug. 2020.

[3] R1-2008483, “PUSCH coverage enhancements,” InterDigital, RAN1#103-e, Aug. 2020.

[4] 3GPP, “[102-e-Post-NR-CovEnh-02] Email discussion/approval of Phase 1 - link budget template,” Softbank, 3GPP RAN1 email discussion, Aug. 2020.

[5] InterDigital, “IMT-2020 Evaluation: Link Budget analysis for eMBB and UMa-URLLC scenarios,” ATIS WTSC-RAN meeting, March, 2019.

Appendix

Table 4 FDD simulation parameters for eMBB and VoIP for PUSCH and PDSCH

| Simulation Assumptions (to maximize MCL) | eMBB Values | VoIP Values |
| --- | --- | --- |
| Frequency Region | FR1 |  |
| Duplexing mode | FDD |  |
| Carrier Frequency | 700MHz |  |
| Subcarrier Spacing (kHz) | 15 |  |
| Tx Bandwidth Configuration (MHz) | 20 |  |
| Waveform | W CP DFT-s-OFDM for UL, W CP OFDM for DL |  |
| CP type | Normal |  |
| PHY channel | PUSCH, PDSCH |  |
| Allocation (# of PRB) | 4 for UL, 17 for DL | 4 for UL |
| DM-RS Type | Configuration Type 1 |  |
| DM-RS (# of OFDM symbols) | Type B for uplink (14), Type A for downlink (12) |  |
| MCS index, table, VoIP packet size | #0, Table 6.1.4.1-1 for UL, #9, Table 5.1.3.1-3 for DL, TS 38.214 | #4, Table 6.1.4.1-1, TS 38.214, VoIP packet size=320 bits |
| HARQ sequence | N/A | 0,2,3,1 |
| Repetition/HARQ scheme | N/A | 50ms latency requirement : 4 repetitions, 3 HARQ processes, 5 maximum retransmissions 100ms latency requirement : 4 repetitions, 3 HARQ processes, 9 maximum retransmissions |
| Frequency Hopping | Disabled |  |
| Antenna Configuration | 1x2 uplink channels (i.e., 1 TX at UE and 2 RX at BS), (M,N,P,Mg,Ng) = (4,2,2,1,1) at BS 2x2 for downlink channels |  |
| PA impairment | ideal |  |
| Channel model (TDL type, DS) | TDL-C, DS=300ns |  |
| TX pattern per frame (for TDD) | N/A |  |
| Speed | 3 km/hr |  |
| Receiver: CHEST | Realistic |  |
| Data rate | 100 kbps for rural UL, 1 Mbps for rural DL, iBLER=10% | N/A |
| Residual BLER | N/A | 2% |

Table 5 TDD simulation assumptions for eMBB and VoIP for PUSCH and PDSCH

| Simulation Assumptions (to maximize MCL) | eMBB Values | VoIP Values |
| --- | --- | --- |
| Frequency Region | FR1 |  |
| Duplexing mode | TDD |  |
| Carrier Frequency | 4GHz |  |
| Subcarrier Spacing (kHz) | 30 |  |
| Tx Bandwidth Configuration (MHz) | 100 |  |
| Waveform | W CP DFT-s-OFDM for uplink, W CP OFDM for downlink |  |
| CP type | Normal |  |
| PHY channel | PDSCH/PUSCH |  |
| Allocation (# of PRB) | 4 for uplink 30 for downlink | 4 |
| DM-RS Type | Configuration Type 1 (according to Table 6.4.1.1.3-1 in TS 38.211) |  |
| DM-RS (# of OFDM symbols) | Type B for uplink (14), Type A for downlink (12) |  |
| Precoder | None for uplink, Precoder cycling for downlink |  |
| MCS index, table, VoIP packet size | #3, Table 6.1.4.1-1, TS 38.214 for uplink #6, Table 5.1.3.1-3, TS 38.214 for downlink | #4, Table 6.1.4.1-1, TS 38.214, VoIP packet size=320 bits |
| HARQ sequence | N/A | 0,2 |
| Repetition/HARQ scheme | N/A | Configurations for PUSCH 20ms latency requirement : 2 repetitions, H=1 HARQ processes, 4 maximum retransmissions 50ms latency requirement : 2 repetitions, H=1 or 2 HARQ processes, 10 or 5 maximum retransmissions for H=1 and 2, respectively 100ms latency requirement :2 repetitions, H=1 or 2 HARQ processes, 20 or 10 maximum retransmissions for H=1 and 2, respectively |
| Frequency Hopping | Disabled |  |
| Antenna Configuration | UL : 1x2 (i.e., 1 TX at UE and 2 RX at BS), 64 TXRUs, (M,N,P,Mg,Ng) = (8,4,2,1,1) for rural DL : 2 x 4 |  |
| PA impairment | ideal |  |
| Channel model (TDL type, DS) | TDL-C, DS=300ns |  |
| TX pattern per frame (for TDD) | DDDSU |  |
| Speed | 3 km/hr |  |
| Receiver: CHEST | Realistic |  |
| Data rate | 100 kbps for rural uplink, iBLER=10% 1 Mbps for rural downlink, iBLER=10% | N/A |
| Residual BLER | N/A | 2% |

Table 6 PDCCH parameters

| - Parameters | - Values |
| --- | --- |
| - Payload | - 40 bits + CRC |
| - Aggregation level | - 16 |
| - CORESET | - 2 symbols x 48 PRBs |
| - Precoder | - Precoder cycling, CCE level REG bundle = 6 |

Table 7 PUCCH parameters

| - Parameters | - Values PUCCH format 1 | - Values PUCCH format 3 |
| --- | --- | --- |
| - Payload | - 2 bits | - 22 bits |
| - PUCCH symbols | - 14 | - 14 |
| - # of RB | - 1 | - 1 |
| - Frequency hopping | - Disabled |  |
| - Repetition | - Disabled |  |
| - Performance | - ACK missed detection probability : 1%, NACK to ACK probability : 0.1%, DTX to ACK probability : 1% | - BLER 1% |

Table 8 Link budget parameters for MCL/MIL/MPL computation

| - Parameters | - Values (700MHz, FDD) | - Values (4GHz, TDD, rural) |
| --- | --- | --- |
| - Antenna gain correction factor (4b), (11b) | - 0 dB |  |
| - Cell area reliability | - 90% for data, 95% for control |  |
| - BS antenna heights, UT antenna heights | - 35m, 1.5m |  |
| - Total transmit power uplink/downlink (3) | - 23 dBm/49dBm |  |
| - Cable, connector, body loss (8)/(12) | - 1 dB (UE) / 3dB (BS) |  |
| - Gain of antenna elements - (11c) for uplink, (4c) for downlink | - 8 dB |  |
| - Receiver noise figure (13), uplink/downlink | - 5 dB / 7dB |  |
| - Thermal noise density (14) | - -174 dBm/Hz |  |
| - Receiver interference density (15) | - -165.7 dBm/Hz PUSCH, - -161.7dBm/Hz PUCCH, - -169.3 dBm/Hz PDCCH, PDSCH |  |
| - Receiver implementation margin (20) | - 2 dB |  |
| - Pathloss (25)+(26)+(27)+(28) - ((25),(27)) | - 17.88 dB PUSCH (5.92 dB, 11.96dB) - 21.62 dB PUCCH (9.66dB, 11.96dB) - 21.62 dB PDCCH (9.66dB, 11.96dB) - 17.88 dB PDSCH (5.92 dB, 11.96dB) | - 19.43 dB PUSCH (4.93dB ,14.5 dB) - 22.67 dB PUCCH (8.17dB, 14.5dB) - 22.67 dB PDCCH (8.17dB, 14.5dB) - 19.43 dB PDSCH (4.93dB ,14.5 dB) |