3GPP TSG RAN WG1 Meeting #103-e	                                                                          R1-2009362

e-Meeting, October 26th – November 13th, 2020	(revision of R1-2008889)

Agenda item:    8.2.2

Source:              Qualcomm Incorporated

Title:                  Channel Access Mechanism and System Level Evaluations for NR in 52.6 to 71GHz band

Document for:  Discussion and Decision

# Introduction

This paper presents a collection of system level evaluation results for study of NR deployment in 52.6 to 71 GHz band.  In addition, we discuss aspects of channel access mechanism along with possible enhancements for NR deployment in the 52.6 to 71GHz band.

In this paper, evaluations in accordance with the agreed evaluation methodology are presented. Additional results are presented to provide a deeper understanding of interference in the spectrum of interest.

# Simulation Studies

## Performance of Baseline LBT Scheme

This section covers baseline LBT scheme as agreed in the evaluation methodology. One objective of the simulation studies is to study the impact of spectrum sharing mechanisms on system performance. In accordance with the agreements in R1 #102e, we discuss below an indoor deployment scenario with 2 operators. The simulation layout is as depicted in the Figure 1 – and corresponds to the full indoor open office layout with 12 gNBs/operator. Each gNB is connected to 5 UEs/ cell. Additional details of the simulation are presented in Table 1. We undertake comparison study of COT based channel access under different sensing assumptions and parameters.

Figure 1.  Indoor Office Layout with 2 Operators

### COT based channel access with LBT sensing

We look at a collection of COT based channel access schemes that deploy channel sensing LBT mechanisms. The detailed assumptions of the LBT mechanism are captured in the table below.

Table 1:  Common assumptions on the channel access mechanism

| Contention Slot and Contention Window | 8us + (1-5) random number of contention slots of duration 5 us for the gNB, UE transmissions are contention free within the COT |
| --- | --- |
| SCS, BW | 960KHz, 2GHz |
| Layout | InH Open office model – with 2 operators, 12gNB/operator in 120m x 50m |
| {BS, UE} Antenna Configuration | Baseline: (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) Higher Directivity at gNB: (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), (Mg,Ng,M,N,P) = (1,2,2,2,2) with (0.5 dv, 0.5 dH) Transmit power at gNB adjusted to achieve Max EIRP of 40 dBm. |
| Energy Sensing Beams | - Omni directional (b) Directional matched to transmission beam |
| COT duration | 0.25ms = 16 NR Slots. 100% DL or 100% UL frame structure per COT. DL and UL directions for COTs alternate if both types of traffic need to be scheduled at gNB. |
| Delay assumptions | K1=12 NR Slots, K2=0, K3=32 NR Slots |
| Multi-user scheduling | 1 User per COT with beam persistence throughout the COT |
| DL -UL Traffic | 50%-50%, FTP Model 3 with file sizes 2MB (and 8MB where stated) |

The LBT/COT based channel access schemes have been simulated with the assumptions on the [K1,K2,K3] as [12,0,32] to accommodate simulator considerations. It can be observed that the deployment has wide variation in the channel conditions seen by users. As the offered load increases the tail users start seeing instabilities while the median users continue to observe good UPT performance.  This variation can be attributed to both variation in path gain and interference. We next focus on the sensing aspects and their performance under COT based access in the next section.

### Sensing at the gNB with varying thresholds

Figure 2. DL and UL User perceived throughput for simulations with different versions of LBT mechanisms with sensing thresholds at the transmitter. 5 UEs/Cell for InH Model (120x50m) with 2 operators. Ki = [12,0,32] slots. SCS 960 KHz. 2GHz. 40dBm/Pol, 25dBm/Pol EIRP at gNB/UE: Antenna Configuration at gNB (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), Traffc: FTP Model 3 with short file size of 2MB. [Solid lines – Median user, Dashed – Tail user]

Figure 2 depicts the overall performance of DL and UL when sensing is performed at the gNB with varying thresholds, and omni vs directional sensing. A threshold of -47 dBm corresponds to almost no back-off at the gNB. With variation in thresholds the LBT based back-off improves the performance at higher loading levels marginally. This is in line with the observation that for majority of cases, LBT at the transmitter has small, if any, performance improvements. For this layout, a threshold of -67 dBm does not appear to create significant silencing. Consequently, the relative benefit/demerits of directionality of sensing are diminished as well. Further, directional sensing done at the gNB appears to help at higher loads in the aggregate in the uplink. Note that, for uplink transmissions, gNB is the receiver and hence sensing at the gNB is expected to be beneficial since it is effectively receiver assisted. The situation can be different for specific drops as opposed to in the aggregate. For example, in Section 3, the benefit of silencing is larger for the median user than the aggregate statistics.

### Rx Assistance to combat Tx-Rx Mismatch

Figure 3. DL User perceived throughput for simulations with LBT mechanisms that make use of Rx assistance. 5 UEs/cell for InH model (120m x 50m) with 2 operators. Ki = [12,0,32] slots. SCS 960 KHz. 2GHz. 40 dBm/pol, 25 dBm/pol EIRP at gNB/UE, Traffc: FTP Model 3 with short file size of 2MB [Solid lines – Median user, Dashed – Tail user]

It is well established that transmitter energy sensing may be highly mismatched with the interference conditions at the receiver. This is borne out by the results for downlink performance as shown in Figure 3 where receiver assistance in the form of silencing transmissions can help considerably. But note that given the large beam widths used at the UE, directional sensing of the UE transmissions provides smaller benefits.  The performance improvements available by directional sensing and receiver assistance can be more pronounced in specific drops, indicating a relatively stuck situation despite the directionality, likely due to beam collisions. This is depicted in Figure 4 below which indicates a situation where receiver assistance provides benefits in the median, even for the case with 5 active UEs per cell.

Figure 4. For a specific drop, drop-A, User Perceived Throughput for Simulations with different versions of LBT mechanisms with Rx Assistance. Rx-Assistance is shown to have benefits for the COT based access mode, indicating a stuck situation and a remedy based on LBT

Note in all the results that a reliably backoff based on a threshold of -72 dBm is not readily achieved by energy detection as it is close to the noise floor. Hence the results with -72 dB can be considered to represent the performance of a something akin to a sequence detection with some processing gain available for reliable detection. On the other hand, higher level of sensing thresholds is more amenable to raw energy detection and Rx-assistance facilitating energy detection of transmissions done by a receiver.

### Directional Sensing and Rx Assistance under highe beamforming gains

The impact of directionality vs. omni sensing increases when the directionality and gain of underlying beams increase.

This is seen in the case of indoor deployment of gNBs with more antennas and directivity than the baseline case.

Figure 5 and Tables in section 8.2 show the performance of directional sensing and Rx assistance when antenna configuration (1,1,8,16,2) is used at the gNBs. Note that although gNB beams are more directional, the interference level created by gNB is capped by the EIRP constraint.

As expected, the higher directivity of narrower beams and consequent interference rejection increases the system capacity. Thus, the same loading levels – buffer occupancies- are achieved at much higher offered load. LBT schemes based of sensing performed at the transmitter without receiver assistance provide only a small improvement over No-backoff dynamic TDD type system irrespective of ED thresholds.

On the other hand, in an environment with more directional transmission beams. Rx Assistance appears to provide a useful improvement systemwide in the tail and the median performance as loading levels increase.  Further at higher loads, Rx assistance when used with directional sensing outperforms or matches Rx Assistance with omni sensing, although not by much. For the simulations in Figure 5 and Table 8.2, a matched directional sensing is modeled – i.e. the sensing beam for LBT is the same as that used for data transmission/reception.  At low loads, directional sensing when used with appropriate threshold adjustment, can reduce unnecessary silencing.

Some of these observations are in contrast to the baseline scenario, where relatively low directionality in the gNB beams may hide distinctive behaviors of directional sensing in comparison to omni-directional sensing.

Figure 5. Indoor Scenario 2 operator  gNB deployment with higher beamforming gains –(antenna configuration   (1,1,8,16,2)).  Rx Assistance provides useful benefits in median and tail performance as loading increases. Impact of directional sensing is also observed  under this scenario. Note that the system has higher capacity than baseline configuration.

Observation 1: Shared spectrum operation with high directivity systems experiences low interference and good performance on the aggregate. Moreover, the gain of LBT schemes over no LBT schemes is minimal on the aggregate.

Observation 2: Aggregate performance may not be representative of individual drops due to the highly directional nature of links. In the events where interference becomes an issue, Rx-assistance based LBT schemes outperform Tx-only LBT schemes as well as not doing LBT at all.

# Interference characteristics in stuck situations

It is understood that the nature of interference in this band is different from the corresponding Sub 7 GHz scenarios due to analog beamforming.  The interference may be a rare occurrence, but when it appears, it can be damaging, mainly arising from beam collisions between interfering links.

We highlight these situations by looking at the indoor layout with fewer users per cell, in order to clearly isolate the interference problem from scheduling diversity. In the simulation studies in this section, the 2-operator indoor layout is deployed with 2 users per cell and 50:50 DL and UL FTP3 traffic with file sizes of 8 MB. The other assumptions are the same as used in previous section. We simulated both no-LBT scheme and a receiver assisted LBT scheme.

The aggregated performance of 10 drops is depicted in Figure 6.

Figure 6. Aggregated performance of deployment with 2 UE/Cell in InH Open office scenario. Solid lines -Median user performance, dotted lines – tail user (5th percentile user)

The aggregated performance computed over 10 drops confirms a graceful degradation in performance as a function of increased offered load. A per-transmission receiver assisted LBT mechanism provides a small improvement in aggregated performance at high load.

On the other hand, the overall statistics may hide scenarios where interference is severe. This is depicted in perceived throughput performance of particular drops. Figure 7 shows the performance of a vulnerable drop out of the 10 drops simulated.  The performance of median users is affected by interference which is addressed by a receiver assisted LBT scheme.

Figure 7. Performance of a vulnerable drop with 2 UE/Cell in InH Open office scenario. [Solid lines -Median user, dotted lines – tail (5th percentile) user.

Figure 8. Median and Tail SINR for scheduled transmissions in a vulnerable drop.

Figure 8 further shows the median and tail SINR distribution.  Due to the semi-stationary interferer beam collision, the degradation in SINR for the no-LBT (Dyn. TDD) scenario can be clearly observed, as reflected as low SINR or outage. For this drop, the interferer may affect median users as well as tail users.

Observation 3: Under scenarios with low interference diversity, there can be scenarios (depends on locations of transmitters and receivers) persistent interferers present and cause significantly reduced SINR or outage. A receiver assisted LBT mechanism can help those scenarios.

Proposal 1. Consider receiver assisted LBT mechanism for scenarios with low interference variation.

# Motivating a Long-Term-Sensing framework

Based on the simulation results above, it can be argued that conventional coexistence mechanisms such as per-TxOP LBT have limited performance benefit. This is largely attributed to the directionality of transmissions and receptions due to analog beamforming and hence minimal interference. On the other hand, it has also been demonstrated that interference, when present, can be damaging. When designing a technology for the future in an unlicensed band, it would still be prudent to have a mechanism to resolve damaging interference when it is present, since it may not always be an operator deployed scenario in unlicensed bands. It is worth noting that the regulatory bodies still require some form of adequate spectrum sharing mechanism to resolve such situations.

## Considerations for Directional Sensing

The goal of channel sensing for medium access in a shared spectrum channel is two-fold:

Protect the on-going transmissions from being interfered by the intended transmission

Protect the intended transmission from being interfered by the on-going transmission

Given the (1) high directionality of transmissions at the transmitter and (2) high directionality of receiver, sensing for channel access in FR2x therefore, cannot be decoupled from directionality and the transmission / reception roles of the node in the ensuing transaction.

As outlined in [7], sensing based channel access procedures are implicitly based on an ‘interference-reciprocity’ assumption, i.e. a node expected to cause more interference should back-off/silence more. In highly beamformed systems such as FR2x unlicensed is expected to be, the relationship between a sensing operation and beamformed transmission at a node can be discussed in terms of its ‘interference footprint’ -which roughly identifies the locations affected by the intended transmission,  and its ‘sensing footprint’ – which identifies the set of locations that channel sensing can pick up interference from.

Figure 9. The interference footprint of transmission from contending node N and the (omnidirectional) sensing footprints at thresholds T1 and T2  for the sensing unit at node N.

As indicated in Figure 9, an omnidirectional sensing footprint at any threshold may mismatch with the interference footprint of highly beamformed directional transmission. This can contribute to (1) interference e.g. the victim device in the interference footprint but not in sensing footprint under threshold T1 (2) over-silencing e.g. the aggressor device not in the interference footprint but in the sensing footprint under threshold T2.

It is important to study what is an appropriate relationship between the sensing beam, sensing threshold and the ensuing transmission beams. Stated another way, we need to define the basic principles on the relationship between T1 and T2.

The sensing beam shape and gain may be different from the eventually used transmit beam’s gain and directivity. For example, this can prominently happen in the following use cases depicted in Figure 10:

- A gNB contends and senses the medium using a sensing beam B0, and on winning the medium under the channel access procedure, transmits a group of SSBs, each beamformed by a separate beam Bi , that cannot necessarily  be assumed to be QCL with respect to each other. (Figure 10: left)

- A gNB wins the medium using channel access procedure with a sensing beam B0, and goes to serve multiple users in the same COT, using separate non-QCL beams Bi. (Figure 10: right)

Figure 10. Sensing for channel access, for a known sequence of SSB transmissions (left) or for a dynamic sequence of data transmissions to different UEs (right).

The distinction of QCL versus not-QCL may not be suitable for sensing beam and transmission relationship, which may instead be more amenable to a ‘soft’ quantification as opposed to a binary criterion.  In order to capture this in a more natural way, we could consider the use antenna gain as a possible metric to assess suitability of using sensing beam A in conjunction with transmission beam B, for example. In particular, a wider beam sensing may pick up less energy along the direction of eventual transmission using a narrow beam. This should likely be reflected the energy detection threshold used with wider beam sensing as opposed to narrow beam sensing for an ensuing narrow beam transmission. For instance, a wider beam sensing with narrow beam transmission should use a lower energy detection threshold compared to a narrow beam sensing for the same narrow beam transmission.

Proposal 2:  Consider the use of antenna gain of sensing beam and transmission beam to determine the suitability of using a given sensing beam in conjunction with another transmission beam.

## Tx sensing and Tx-Rx mismatch and Considerations for Rx Assistance

Figure 11. Sensing a beamformed transmission.

Figure 12. Understanding Tx Sensing-Rx mismatch: The simulations scatterplots show measured energy (received power in omnidirectional sensing)  from interfering downlink transmissions measured at gNB-UE pairs in 60 GHz and 6 GHz environments respectively in an Indoor Office Scenario: [left: 60GHz/2.16GHz BW, 40dBm EIRP, 16x8 Antennas/Pol@gNB, right: 6GHz/20MHz BW, 23 dBm TxPower, 4 Antennas@gNB, ].A common energy threshold of θ= -78 dBm is used for depiction.

As noted in [7], sensing at a contending transmitter does not necessarily represent the eventual interference impact to the receiver. However, this phenomenon is amplified under beamformed operation of FR2, i.e., greater fraction of hidden and exposed nodes for a given nominal sensing threshold is observed, although the overall interference level is less due to the very same reason.

Dealing with this Tx-sensing and Rx mismatch typically requires some involvement from the receiver in the sensing process. Moreover, mismatches in sensing can reduce the effectiveness of Tx-sensing and back-off/silencing procedures, consequently diminishing the purpose of medium reuse traded off to gain SINR.

Traditionally, receiver involvement has typically implied some form of message exchange between transmitter and receiver. However, these techniques have had their challenges in the past, owing to a multitude of factors which can be placed under the umbrella of complexity.

## Considerations for Long Term Sensing Paradigm

Long term sensing paradigm permits use of multiple sensing opportunities to coordinate interference management over a longer term and adapt channel access accordingly. In its simplest form, long term sensing could have the following components:

- Sensing part: Periodic channel measurements are provisioned per channel

  - This is different from LBT in the sense that it is not per-transmission or per-channel occupancy. The concept is closer to RRM measurements in the sense that it is predictable and periodic.

- Response part:

  - If no nearby node is detected, no additional restriction on medium access, i.e., the system operates as if there are no restrictions on channel access and subsequent transmissions

  - If nearby node is detected, additional restriction on medium access can be imposed, such as constraints on transmission or requirement of an LBT procedure

The following figures show how long-term sensing could help identifying and resolving collisions without incurring the cost of per-COT LBT. The first observation is the provisioning of measurement occasions. The measurement occasions may be for gNBs and UEs alike within a network. Measurement occasions are expected to be periodic and configured every T ms. During this period, if there is a node of another operator in the vicinity and is received with a large signal power above some pre-configured threshold, that indicates a potential interfered or interfering node.

Figure 13. Long-term Sensing: If no nearby node is detected above threshold

Figure above represents the ‘sunny-day’ scenario where the node in question periodically performs measurements at the measurement occasion and does not detect significant energy during these instances. The node interprets this as the absence of colliding interferer and continues to operate without any restrictions on its channel access. Furthermore, no additional constraints are assumed on its transmit power (other than the regulatory max limit), duty cycle etc.

Figure 14. Long-term Sensing: If nearby node is detected above threshold

Figure above represents the ‘colliding beam’ scenario where the node in question detects a signal during the measurement occasion(s) that exceeds a certain pre-configured energy threshold. As suggested here, this could be configured to be multiple measurement occasions to ensure robustness. The node then interprets this as the presence of a colliding interferer and switches to mode where per-COT listen-before-talk is employed for the next pre-configured amount of time. Alternatively, instead of introducing per-COT LBT, one could consider other constraints in the form of transmit power or duty cycle limitations to ensure the interference impact is kept to a minimum.

Further discussion could be pursued on how to extend this type of a framework to a multiple beam scenario. For example, the sensing occasions could be beam specific, which allows for directional sensing, i.e., a node only needs to sense on the beam direction(s) on which it plans to transmit in the next T ms. Moreover, the extension can largely borrow from the well-understood principles of RRM measurements in NR.

In addition, long-term sensing provides the potential benefits of receiver assistance as well, allowing the interfered receiver opportunities to indicate to the interfering transmitter via measurement occasions. In particular, the measurement occasions provide gaps for a transmitter to sense the medium during which other receivers of uncoordinated deployments could transmit an indication of some kind to activate LBT on the transmitting node.

Long-term sensing could, in summary:

- Provide a low-cost, predictable way of resolving beam-colliding nodes

- In the event of minimal beam collisions, fallback to licensed-like operation without LBT overhead and complexity

- Provide the framework for beam-based sensing and receiver assistance.

Proposal 3: Consider measurements intervals for long term sensing as designated transmission opportunities for detection of potentially interfered nodes and enabling collision resolution selectively. The collision resolution techniques can be per-COT LBT, TX power limitation, and/or duty cycle limitation.

# Conclusions

The proposals and observations made in this contribution are summarized below.

Observation 1: Shared spectrum operation with high directivity systems experiences low interference and good performance on the aggregate. Moreover, the gain of LBT schemes over no LBT schemes is minimal on the aggregate.

Observation 2: Aggregate performance may not be representative of individual drops due to the highly directional nature of links. In the events where interference becomes an issue, Rx-assistance based LBT schemes outperform Tx-only LBT schemes as well as not doing LBT at all.

Observation 3: Under scenarios with low interference diversity, there can be scenarios (depends on locations of transmitters and receivers) persistent interferers present and cause significantly reduced SINR or outage. A receiver assisted LBT mechanism can help those scenarios.

Proposal 1. Consider receiver assisted LBT mechanism for scenarios with low interference variation.

Proposal 2:  Consider the use of antenna gain of sensing beam and transmission beam to determine the suitability of using a given sensing beam in conjunction with another transmission beam.

Proposal 3: Consider measurements intervals for long term sensing as designated transmission opportunities for detection of potentially interfered nodes and enabling collision resolution selectively. The collision resolution techniques can be per-COT LBT, TX power limitation, and/or duty cycle limitation.

# References

- RP-193259, Study on supporting NR from 52.6 GHz to 71 GHz.

- RP-193229, Extending current NR operation to 71GHz.

- Chairman’s Notes, RAN1#92, Agreements for NR-U study.

- CEPT ECC, ERC Recommendation 70-03 for Short Range Devices, June 2019.

- ETSI EN 302 567, Harmonised European standard for Multi-Gigabit radio equipment in 60GHz.

- ETSI EN 303 722, WDTS for Fixed Networks.

- R1-2004489, Channel Access Mechanisms for NR in 52.6GHz to 71GHz, Qualcomm.

- ETSI EN 303 753, New ETSI WI for c2, work in progress.

# Deployment metrics

## Downlink RSRP Distribution for Indoor Deployment

Figure 15 shows the distribution of serving cell downlink RSRP for indoor scenarios. The scenarios depicted include

- Baseline assumptions on antenna configuration – 32 gNB antenna elements per polarization

- A case with larger number of antenna elements (128 per polarization) and higher beamforming gain

Figure 15. Downlink serving Cell RSRP Distributions for Indoor scenarios

# Reporting Metrics for LBT Mechanisms

The following table summarises the layout assumptions of the tables in the following subsections.

| Case No. | Antenna Configuration @gNB | LBT Scheme | Sensing Threshold and Directionality |
| --- | --- | --- | --- |
| 1 | (1,1,4,8,2) | No Backoff, ~Dynamic TDD | -47 dBm |
| 2 | (1,1,4,8,2) | Tx Sensing | -67 dBm, Omni |
| 3 | (1,1,4,8,2) | Rx Assistance | -67 dBm, Omni |
| 4 | (1,1,4,8,2) | Rx Assistance | -72 dBm, Omni |
| 5 | (1,1,4,8,2) | Tx Sensing | -67 dBm, Directional |
| 6 | (1,1,4,8,2) | Tx Sensing | -72 dBm, Directional |
| 7 | (1,1,4,8,2) | Rx Assistance | -67 dBm, Directional |
| 8 | (1,1,4,8,2) | Rx Assistance | -72 dBm, Directional |
| 9 | (1,1,8,16,2) | No Backoff, ~Dynamic TDD | -47 dBm |
| 10 | (1,1,8,16,2) | Tx Sensing | -67 dBm, Omni |
| 11 | (1,1,8,16,2) | Tx Sensing | -67 dBm, Directional |
| 12 | (1,1,8,16,2) | Tx Sensing | -72 dBm, Directional |
| 13 | (1,1,8,16,2) | Rx Assistance | -67 dBm, Omni |
| 14 | (1,1,8,16,2) | Rx Assistance | -72 dBm, Omni |
| 15 | (1,1,8,16,2) | Rx Assistance | -67 dBm, Directional |
| 16 | (1,1,8,16,2) | Rx Assistance | -72 dBm, Directional |

## Indoor Office Scenario for (Mg,Ng,M,N,P) = (1,1,4,8,2)

| Tdoc / Source | Cases |  | Case 1: -47dBM@gNB, Dynamic TDD (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, |  |  | Case 2: -67dBM@gNB, (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Omni Listening |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6381 | 3753 | 758 | 6176 | 3738 | 781 |
|  |  | 50%ile | 11297 | 8593 | 4511 | 11335 | 8607 | 4841 |
|  |  | 95%ile | 13685 | 12550 | 10440 | 13661 | 12503 | 10659 |
|  |  | mean | 10738 | 8387 | 4992 | 10734 | 8420 | 5170 |
|  | DL delay (ms) | 5%ile | 1.22 | 1.428 | 1.919 | 1.226 | 1.422 | 1.87 |
|  |  | 50%ile | 1.712 | 2.525 | 5.854 | 1.712 | 2.48 | 5.472 |
|  |  | 95%ile | 3.406 | 6.704 | 83.257 | 3.362 | 6.698 | 47.683 |
|  |  | mean | 1.942 | 3.172 | 24.875 | 1.944 | 3.111 | 20.491 |
|  | UL UPT (Mbps) | 5%ile | 4997 | 3438 | 935 | 4921 | 3506 | 972 |
|  |  | 50%ile | 8755 | 6996 | 4086 | 8731 | 7023 | 4281 |
|  |  | 95%ile | 10395 | 9591 | 8161 | 10372 | 9575 | 8316 |
|  |  | mean | 8407 | 6826 | 4300 | 8397 | 6841 | 4450 |
|  | UL delay (ms) | 5%ile | 1.597 | 1.791 | 2.357 | 1.604 | 1.806 | 2.287 |
|  |  | 50%ile | 2.091 | 2.921 | 6.543 | 2.099 | 2.881 | 6.041 |
|  |  | 95%ile | 3.822 | 6.702 | 37.443 | 3.872 | 6.7 | 36.987 |
|  |  | mean | 2.292 | 3.418 | 12.621 | 2.299 | 3.394 | 11.065 |
|  | Arrival rate (files/s) |  | 10 | 15 | 20 | 10 | 15 | 20 |  |
|  | 𝜌DL |  | 0.999 | 0.999 | 0.984 | 0.999 | 0.999 | 0.985 |  |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.994 | 0.999 | 0.999 | 0.995 |  |
|  | BO |  | 0.18 | 0.33 | 0.508 | 0.18 | 0.326 | 0.502 |  |
|  | Additional report/notes: Case 1 and Case 2: - LBT procedure and parameters: Baseline LBT Procedure at gNB: 8us+(1-3)*5us, at the gNB. Only gNBs perform extended CCA. - All Results with 2 operator Indoor scenario. Main assumptions provided in Column header and Table 1. Omni Directional LBT with specified thresholds. - No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling -1 user per COT with beam persistence. |  |  |  |  |  |  |  |

| Tdoc / Source | Cases |  | Case 3: -67dBM@gNB, Rx Assist (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, Omni Listening |  |  | Case 4: -72dBM@gNB, Rx Assist (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Omni Listening |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6275 | 3713 | 851 | 6368 | 3855 | 1146 |
|  |  | 50%ile | 11237 | 8686 | 5063 | 11235 | 8751 | 5497 |
|  |  | 95%ile | 13473 | 12340 | 10634 | 13480 | 12444 | 10730 |
|  |  | mean | 10631 | 8364 | 5340 | 10670 | 8492 | 5719 |
|  | DL delay (ms) | 5%ile | 1.239 | 1.442 | 1.844 | 1.231 | 1.425 | 1.799 |
|  |  | 50%ile | 1.719 | 2.464 | 5.203 | 1.69 | 2.429 | 4.489 |
|  |  | 95%ile | 3.355 | 6.728 | 43.559 | 3.326 | 6.33 | 26.962 |
|  |  | mean | 1.948 | 3.153 | 19.907 | 1.931 | 3.054 | 12.928 |
|  | UL UPT (Mbps) | 5%ile | 5023 | 3443 | 1132 | 5066 | 3521 | 1343 |
|  |  | 50%ile | 8740 | 7035 | 4438 | 8724 | 7142 | 4726 |
|  |  | 95%ile | 10362 | 9580 | 8331 | 10328 | 9614 | 8390 |
|  |  | mean | 8408 | 6850 | 4593 | 8405 | 6881 | 4796 |
|  | UL delay (ms) | 5%ile | 1.603 | 1.8 | 2.272 | 1.609 | 1.8 | 2.245 |
|  |  | 50%ile | 2.084 | 2.862 | 5.932 | 2.093 | 2.838 | 5.189 |
|  |  | 95%ile | 3.861 | 6.806 | 30.388 | 3.773 | 6.608 | 21.325 |
|  |  | mean | 2.29 | 3.368 | 9.494 | 2.292 | 3.316 | 7.919 |
|  | Arrival rate (files/s) |  | 10 | 15 | 20 | 10 | 15 | 20 |
|  | 𝜌DL |  | 0.999 | 0.999 | 0.987 | 0.999 | 0.999 | 0.993 |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.996 | 0.999 | 0.999 | 0.997 |
|  | BO |  | 0.18 | 0.331 | 0.5 | 0.179 | 0.325 | 0.504 |
|  | Additional report/notes: Case 3 and Case 4: - LBT procedure and parameters: ECCA based Contention at gNB: 8us+(1-3)*5us, at the gNB. - Rx-Assistance: Silencing signals sent by gNB and UE after winning the medium. Only gNBs perform extended CCA. - All results are for 2 operator Indoor office scenarios. Main Setup described in the column header and Table 1. - Only gNBs perform extended CCA. No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling 1 user.COT with beam persistence |  |  |  |  |  |  |  |

| Tdoc / Source | Cases |  | Case 5: -67dBM@gNB, Tx Sensing (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, Directional Sensing |  |  | Case 6: -72dBM@gNB, Tx Sensing (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Directional Sensing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6133 | 3698 | 791 | 6212 | 3710 | 874 |
|  |  | 50%ile | 11328 | 8385 | 4730 | 11344 | 8503 | 4888 |
|  |  | 95%ile | 13610 | 12535 | 10512 | 13632 | 12465 | 10476 |
|  |  | mean | 10725 | 8301 | 5038 | 10722 | 8347 | 5143 |
|  | DL delay (ms) | 5%ile | 1.23 | 1.436 | 1.944 | 1.237 | 1.445 | 1.889 |
|  |  | 50%ile | 1.715 | 2.547 | 5.659 | 1.718 | 2.595 | 5.524 |
|  |  | 95%ile | 3.452 | 6.646 | 81.718 | 3.393 | 6.992 | 52.664 |
|  |  | mean | 1.949 | 3.21 | 22.175 | 1.946 | 3.205 | 21.199 |
|  | UL UPT (Mbps) | 5%ile | 4999 | 3466 | 1001 | 5030 | 3454 | 996 |
|  |  | 50%ile | 8726 | 6979 | 4169 | 8779 | 6989 | 4296 |
|  |  | 95%ile | 10363 | 9573 | 8298 | 10373 | 9575 | 8255 |
|  |  | mean | 8397 | 6809 | 4357 | 8409 | 6798 | 4411 |
|  | UL delay (ms) | 5%ile | 1.606 | 1.812 | 2.292 | 1.602 | 1.806 | 2.312 |
|  |  | 50%ile | 2.082 | 2.912 | 6.481 | 2.071 | 2.89 | 6.168 |
|  |  | 95%ile | 3.881 | 6.662 | 36.804 | 3.858 | 6.578 | 33.331 |
|  |  | mean | 2.301 | 3.412 | 11.291 | 2.287 | 3.407 | 10.714 |
|  | Arrival rate (files/s) |  | 10 | 15 | 20 | 10 | 15 | 20 |
|  | 𝜌DL |  | 0.999 | 0.999 | 0.986 | 0.999 | 0.999 | 0.986 |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.995 | 0.999 | 0.999 | 0.995 |
|  | BO |  | 0.18 | 0.334 | 0.518 | 0.18 | 0.333 | 0.496 |
|  | Additional report/notes: Case 5 and Case 6: - LBT procedure and parameters: Baseline LBT Procedure at gNB: 8us+(1-3)*5us, at the gNB. Only gNBs perform extended CCA - All Results with 2 operator Indoor scenario. Main assumptions provided in Column header and Table 1. Directional LBT – Energy Detection in the direction of UE to serve, with same beam as the transmission beam. - Only gNBs perform extended CCA. No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling -1 user per COT with beam persistence. |  |  |  |  |  |  |  |

| Tdoc / Source | Cases |  | Case 7: -67dBM@gNB, Rx Assistance (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, Directional Sensing |  |  | Case 8: -72dBM@gNB, , Rx Assistance (Mg,Ng,M,N,P) = (1,1,4,8,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Directional Sensing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6136 | 3692 | 874 | 6139 | 3809 | 1148 |
|  |  | 50%ile | 11231 | 8642 | 4954 | 11270 | 8782 | 5675 |
|  |  | 95%ile | 13448 | 12371 | 10582 | 13498 | 12450 | 10806 |
|  |  | mean | 10605 | 8346 | 5312 | 10687 | 8510 | 5783 |
|  | DL delay (ms) | 5%ile | 1.24 | 1.43 | 1.859 | 1.238 | 1.448 | 1.819 |
|  |  | 50%ile | 1.729 | 2.478 | 5.159 | 1.692 | 2.434 | 4.455 |
|  |  | 95%ile | 3.463 | 6.799 | 52.919 | 3.466 | 6.218 | 29.454 |
|  |  | mean | 1.957 | 3.174 | 18.912 | 1.93 | 3.016 | 13.05 |
|  | UL UPT (Mbps) | 5%ile | 4990 | 3485 | 1206 | 5088 | 3556 | 1335 |
|  |  | 50%ile | 8758 | 7055 | 4383 | 8765 | 7116 | 4783 |
|  |  | 95%ile | 10351 | 9612 | 8236 | 10350 | 9620 | 8455 |
|  |  | mean | 8405 | 6836 | 4566 | 8399 | 6916 | 4860 |
|  | UL delay (ms) | 5%ile | 1.602 | 1.792 | 2.338 | 1.608 | 1.799 | 2.223 |
|  |  | 50%ile | 2.079 | 2.867 | 5.735 | 2.08 | 2.802 | 4.957 |
|  |  | 95%ile | 3.906 | 6.877 | 30.671 | 3.766 | 6.4 | 23.645 |
|  |  | mean | 2.295 | 3.384 | 9.428 | 2.299 | 3.261 | 8.354 |
|  | Arrival rate (files/s) |  | 10 | 15 | 20 | 10 | 15 | 20 |
|  | 𝜌DL |  | 0.999 | 0.999 | 0.988 | 0.999 | 0.999 | 0.994 |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.996 | 0.999 | 0.999 | 0.997 |
|  | BO |  | 0.18 | 0.329 | 0.495 | 0.179 | 0.326 | 0.497 |
|  | Additional report/notes: Case 7 and Case 8: - LBT procedure and parameters: ECCA based Contention at gNB: 8us+(1-3)*5us, at the gNB. - Rx-Assistance: Silencing signals sent by gNB and UE after winning the medium. Only gNBs perform extended CCA. - Directional LBT – Sensing done at gNB in the direction of the intended UE, with same beam as the transmission beam. All results are for 2 operator Indoor office scenarios. Main Setup described in the column header and Table 1. - Only gNBs perform extended CCA. No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling -1 user per COT with beam persistence. |  |  |  |  |  |  |  |

## Indoor Office Scenario for (Mg,Ng,M,N,P) = (1,1,8,16,2)

The transmit power at gNB is adjusted to maintain Max EIRP of 40 dBm.

|  | Cases |  | Case 9: -47dBM@gNB, Tx Sensing (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, Omni Sensing |  |  | Case10: -67dBM@gNB, , Tx Sensing (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Omni Sensing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6576 | 4888 | 1848 | 6635 | 4929 | 1997 |
|  |  | 50%ile | 11495 | 10028 | 5942 | 11428 | 10011 | 6094 |
|  |  | 95%ile | 13400 | 12560 | 10568 | 13382 | 12576 | 10522 |
|  |  | mean | 10892 | 9474 | 6139 | 10860 | 9467 | 6178 |
|  | DL delay (ms) | 5%ile | 1.258 | 1.394 | 1.875 | 1.261 | 1.394 | 1.853 |
|  |  | 50%ile | 1.627 | 2.006 | 3.9 | 1.635 | 2.022 | 3.859 |
|  |  | 95%ile | 3.1 | 4.438 | 15.855 | 3.229 | 4.351 | 14.397 |
|  |  | mean | 1.853 | 2.352 | 6.46 | 1.857 | 2.353 | 6.204 |
|  | UL UPT (Mbps) | 5%ile | 7106 | 5859 | 2909 | 7040 | 5796 | 2915 |
|  |  | 50%ile | 9406 | 8610 | 6309 | 9416 | 8613 | 6369 |
|  |  | 95%ile | 10297 | 9812 | 8582 | 10281 | 9838 | 8533 |
|  |  | mean | 9126 | 8344 | 6125 | 9124 | 8340 | 6151 |
|  | UL delay (ms) | 5%ile | 1.615 | 1.734 | 2.129 | 1.613 | 1.725 | 2.149 |
|  |  | 50%ile | 1.86 | 2.141 | 3.439 | 1.861 | 2.141 | 3.443 |
|  |  | 95%ile | 2.733 | 3.569 | 10.374 | 2.739 | 3.556 | 9.89 |
|  |  | mean | 1.985 | 2.32 | 4.523 | 1.984 | 2.32 | 4.389 |
|  | Arrival rate (files/s) |  | 15 | 20 | 30 | 15 | 20 | 30 |
|  | 𝜌DL |  | 0.999 | 1 | 0.997 | 0.999 | 1 | 0.998 |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 |
|  | BO |  | 0.232 | 0.338 | 0.56 | 0.231 | 0.338 | 0.569 |
|  | Additional report/notes: Case 9 and Case 10: - LBT procedure and parameters: Baseline LBT Procedure at gNB: 8us+(1-3)*5us, at the gNB. Only gNBs perform extended CCA. - All Results with 2 operator Indoor scenario. Main assumptions provided in Column header and Table 1. Omni Directional LBT with specified thresholds. - No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling -1 user per COT with beam persistence. |  |  |  |  |  |  |  |

|  | Cases |  | Case 11 -67dBM@gNB, Tx Sensing (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, Directional Sensing |  |  | Case12: -72dBM@gNB, Tx Sensing (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Directional Sensing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6369 | 4880 | 1971 | 6644 | 4834 | 2003 |
|  |  | 50%ile | 11443 | 10011 | 6033 | 11446 | 9933 | 6041 |
|  |  | 95%ile | 13377 | 12579 | 10475 | 13341 | 12519 | 10518 |
|  |  | mean | 10853 | 9468 | 6156 | 10841 | 9414 | 6152 |
|  | DL delay (ms) | 5%ile | 1.264 | 1.392 | 1.843 | 1.27 | 1.397 | 1.903 |
|  |  | 50%ile | 1.632 | 2.014 | 3.932 | 1.64 | 2.041 | 3.897 |
|  |  | 95%ile | 3.179 | 4.328 | 14.559 | 3.164 | 4.359 | 14.482 |
|  |  | mean | 1.859 | 2.348 | 6.579 | 1.864 | 2.372 | 6.193 |
|  | UL UPT (Mbps) | 5%ile | 7070 | 5837 | 2947 | 7146 | 5716 | 2900 |
|  |  | 50%ile | 9422 | 8604 | 6272 | 9409 | 8582 | 6296 |
|  |  | 95%ile | 10286 | 9818 | 8570 | 10280 | 9815 | 8573 |
|  |  | mean | 9126 | 8337 | 6133 | 9122 | 8321 | 6152 |
|  | UL delay (ms) | 5%ile | 1.615 | 1.73 | 2.157 | 1.618 | 1.73 | 2.162 |
|  |  | 50%ile | 1.859 | 2.144 | 3.493 | 1.861 | 2.137 | 3.491 |
|  |  | 95%ile | 2.81 | 3.495 | 9.853 | 2.764 | 3.72 | 9.7 |
|  |  | mean | 1.984 | 2.326 | 4.435 | 1.984 | 2.332 | 4.332 |
|  | Arrival rate (files/s) |  | 15 | 20 | 30 | 15 | 20 | 30 |
|  | 𝜌DL |  | 0.999 | 1 | 0.997 | 0.999 | 1 | 0.997 |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 |
|  | BO |  | 0.232 | 0.337 | 0.565 | 0.231 | 0.338 | 0.554 |
|  | Additional report/notes: Case 11 and Case 12: - LBT procedure and parameters: Baseline LBT Procedure at gNB: 8us+(1-3)*5us, at the gNB. Only gNBs perform extended CCA - All Results with 2 operator Indoor scenario. Main assumptions provided in Column header and Table 1. Directional LBT – Energy Detection in the direction of UE to serve, with same beam as the transmission beam. - Only gNBs perform extended CCA. No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling -1 user per COT with beam persistence. |  |  |  |  |  |  |  |

|  | Cases |  | Case 13: -67dBM@gNB, Rx Assistance (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, Omni Sensing |  |  | Case14: -72dBM@gNB, Rx Assistance (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Omni Sensing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6526 | 4771 | 2133 | 6523 | 5001 | 2347 |
|  |  | 50%ile | 11384 | 9959 | 6366 | 11381 | 10040 | 6746 |
|  |  | 95%ile | 13214 | 12429 | 10545 | 13169 | 12359 | 10641 |
|  |  | mean | 10755 | 9429 | 6362 | 10764 | 9491 | 6675 |
|  | DL delay (ms) | 5%ile | 1.272 | 1.409 | 1.833 | 1.286 | 1.416 | 1.799 |
|  |  | 50%ile | 1.628 | 2.002 | 3.63 | 1.623 | 1.966 | 3.312 |
|  |  | 95%ile | 3.208 | 4.331 | 12.167 | 3.153 | 4.227 | 11.129 |
|  |  | mean | 1.866 | 2.33 | 5.678 | 1.856 | 2.302 | 5.038 |
|  | UL UPT (Mbps) | 5%ile | 7036 | 5738 | 2997 | 7092 | 5748 | 3242 |
|  |  | 50%ile | 9414 | 8592 | 6479 | 9396 | 8616 | 6591 |
|  |  | 95%ile | 10283 | 9853 | 8648 | 10265 | 9813 | 8634 |
|  |  | mean | 9132 | 8358 | 6283 | 9121 | 8344 | 6408 |
|  | UL delay (ms) | 5%ile | 1.615 | 1.724 | 2.105 | 1.618 | 1.733 | 2.107 |
|  |  | 50%ile | 1.853 | 2.141 | 3.262 | 1.865 | 2.12 | 3.184 |
|  |  | 95%ile | 2.76 | 3.572 | 9.22 | 2.735 | 3.507 | 7.599 |
|  |  | mean | 1.98 | 2.306 | 4.126 | 1.979 | 2.309 | 3.851 |
|  | Arrival rate (files/s) |  | 15 | 20 | 30 | 15 | 20 | 30 |
|  | 𝜌DL |  | 0.999 | 1 | 0.998 | 0.999 | 1 | 0.998 |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 |
|  | BO |  | 0.231 | 0.338 | 0.564 | 0.232 | 0.334 | 0.553 |
|  | Additional report/notes: Case 13 and Case 14: - LBT procedure and parameters: ECCA based Contention at gNB: 8us+(1-3)*5us, at the gNB. - Rx-Assistance: Silencing signals sent by gNB and UE after winning the medium. Only gNBs perform extended CCA. - All results are for 2 operator Indoor office scenarios. Main Setup described in the column header and Table 1. - Only gNBs perform extended CCA. No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling -1 user per COT with beam persistence. |  |  |  |  |  |  |  |

|  | Cases |  | Case 15: -67dBM@gNB, Rx Assistance (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators, Directional Sensing |  |  | Case 16: -72dBM@gNB, Rx Assistance (Mg,Ng,M,N,P) = (1,1,8,16,2) with (0.5 dv, 0.5 dH), 2Mbytes, 2 Operators Directional Sensing |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-xxxxxxx / Source 1 | Traffic load Metrics |  | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO | Low load 10%~25% BO | Medium load 35%~50% BO | High load above 55% BO |
|  | DL UPT (Mbps) | 5%ile | 6443 | 4782 | 2134 | 6752 | 5076 | 2408 |
|  |  | 50%ile | 11399 | 10083 | 6367 | 11400 | 10152 | 6836 |
|  |  | 95%ile | 13200 | 12406 | 10584 | 13162 | 12364 | 10668 |
|  |  | mean | 10759 | 9443 | 6378 | 10807 | 9514 | 6704 |
|  | DL delay (ms) | 5%ile | 1.276 | 1.394 | 1.829 | 1.284 | 1.412 | 1.8 |
|  |  | 50%ile | 1.62 | 1.991 | 3.594 | 1.618 | 1.945 | 3.275 |
|  |  | 95%ile | 3.179 | 4.415 | 12.388 | 3.093 | 4.215 | 10.824 |
|  |  | mean | 1.861 | 2.328 | 5.809 | 1.844 | 2.29 | 4.734 |
|  | UL UPT (Mbps) | 5%ile | 7010 | 5810 | 3013 | 7098 | 5882 | 3220 |
|  |  | 50%ile | 9411 | 8625 | 6454 | 9419 | 8644 | 6648 |
|  |  | 95%ile | 10290 | 9838 | 8654 | 10260 | 9838 | 8644 |
|  |  | mean | 9134 | 8363 | 6282 | 9122 | 8370 | 6444 |
|  | UL delay (ms) | 5%ile | 1.613 | 1.733 | 2.112 | 1.615 | 1.729 | 2.095 |
|  |  | 50%ile | 1.86 | 2.132 | 3.286 | 1.857 | 2.119 | 3.143 |
|  |  | 95%ile | 2.739 | 3.577 | 9.195 | 2.704 | 3.499 | 7.806 |
|  |  | mean | 1.975 | 2.302 | 4.15 | 1.976 | 2.293 | 3.819 |
|  | Arrival rate (files/s) |  | 15 | 20 | 30 | 15 | 20 | 30 |
|  | 𝜌DL |  | 0.999 | 1 | 0.998 | 0.999 | 1 | 0.999 |
|  | 𝜌UL |  | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 | 0.999 |
|  | BO |  | 0.23 | 0.336 | 0.571 | 0.228 | 0.335 | 0.551 |
|  | Additional report/notes: Case 15 and Case 16: - LBT procedure and parameters: ECCA based Contention at gNB: 8us+(1-3)*5us, at the gNB. - Rx-Assistance: Silencing signals sent by gNB and UE after winning the medium. Only gNBs perform extended CCA. - Directional LBT – Sensing done at gNB in the direction of the intended UE, with same beam as the transmission beam. All results are for 2 operator Indoor office scenarios. Main Setup described in the column header and Table 1. - Only gNBs perform extended CCA. No COT sharing from UL to DL. - Common: DL-UL Traffic:50:50, FTP Model 3, 2MB file.{SCS,BW=960Khz,2GHz},{[k1,k2,k3]=[12,0,32] NR slots}, COT duration 0.25ms, Multi-user scheduling -1 user per COT with beam persistence. |  |  |  |  |  |  |  |