3GPP TSG RAN WG1 #104-e		R1-2100347

e-Meeting, Jan. 25th – Feb. 5th, 2021

Source:	CATT

Title:	Discussion on enhancements for HST-SFN deployment

Agenda Item:	8.1.2.4

Document for:	Discussion and Decision

- Introduction

HST-SFN deployment scenario is one of the most important scenarios for high speed train deployment. In order to have better performance in HST-SFN deployment scenario, the following objective was approved in the WID for Rel-17 NR FeMIMO [1]:

Enhancement to support HST-SFN deployment scenario: i. Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission ii. Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In RAN1 #103 e-meeting, the following agreements on DL transmission schemes and TRP-based frequency offset pre-compensation schemes for HST-SFN deployment were achieved:

Agreement Support at least the following configuration for HST scenario in Rel-17 - The same DMRS port(s) can associate with multiple TCI states - FFS other details Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner Agreement At most two TCI states are supported for HST scenario in Rel-17 - FFS: Whether to support more than two TCI states for FR2 - FFS configuration/signalling details of the TCI states Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. - This agreement is for the purpose of evaluation and does not imply the support or lack of support of scheme 1 and/or TRP based pre-compensation

In this document, we share our views on the enhancements for HST-SFN deployment scenario.

- DL Transmission schemes

In HST-SFN deployment scenario, DL signals can be transmitted in SFN manner from multiple TRPs that deployed along the railway. When a signal is SFN-ed transmitted, the propagation channel of the signal comprises multiple paths with different Doppler shifts and the channel cannot be well estimated by a single SFN-ed transmitted reference signal. Hence, TRP-specific/non-SFN-ed reference signals for Doppler profile estimation are expected.

Mapping from layer to DMRS port

In RAN1 #103 e-meeting, the following scheme was agreed to be supported:

Support at least the following configuration for HST scenario in Rel-17 - The same DMRS port(s) can associate with multiple TCI states Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner

Since both DMRS and PDCCH/PDSCH are transmitted in SFN manner, the mapping from PDSCH/PDCCH layer to DMRS antenna port(s) can be the same as that in Rel-15, i.e. the layer(s) and DMRS port(s) are one-to-one mapped. For the agreed scheme, the Doppler profiles of the channel that convey the DMRS port(s) can be inferred from the combined channel of multiple RSs that transmitted in TRP-specific/non-SFN manner and indicated by the multiple TCI states. Then the performance of channel estimation can be improved.

Definition of QCL

In Rel-15, the definition of QCL is specified as follows:

Two antenna ports are said to be quasi co-located if the large-scale properties of the channel over which a symbol on one antenna port is conveyed can be inferred from the channel over which a symbol on the other antenna port is conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

For the enhanced scheme, the large-scale properties of the channel that convey DMRS port(s) can be inferred from the combination of channels of the RS ports that indicated by multiple TCI states. However, the large-scale properties of the channel of each RS port that indicated by the multiple TCI states cannot be inferred from the channel of the DMRS port. Hence, a definition on QCL association relationship of an antenna port and an antenna port group is needed. One example of the definition is as follows:

Antenna port x and antenna port group Y are said to be quasi co-located if the large-scale properties of the channel over which a symbol on antenna port x is conveyed can be inferred from the combination of channels over which the symbols on the antenna ports of the antenna port group Y are conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

Observation 1:

- A new definition on QCL association relationship of one antenna port and one antenna port group is needed in Rel-17.

Proposal 1:

- A new definition on QCL association relationship of one antenna port and one antenna port group is introduced.

  - E.g. Antenna port x and antenna port group Y are said to be quasi co-located if the large-scale properties of the channel over which a symbol on antenna port x is conveyed can be inferred from the combination of channels over which the symbols on the antenna ports of the antenna port group y are conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

Number of TCI states for FR2

Due to the high path loss in mmWave, the coverage area of a TRP in FR 2 is smaller than that of the TRP in FR 1. Therefore, it is more possible that PDSCH/PDCCH is transmitted from no more than two TRPs in HST scenario for FR 2.

Proposal 2:

- Up to two TCI states are supported in HST scenario for FR 2.

QCL indication

For PDSCH with the transmission scheme agreed in RAN1 #103 e-meeting,  similar TCI states indication solution as that for M-TRP transmission schemes in Rel-16 can be used, i.e. the TCI field of the DCI scheduling the PDSCH indicates a codepoint associated with two TCI states, and each TCI state indicates one of the following quasi co-location type(s) separately:

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with the same CSI-RS resource, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition,or

-	QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'QCL-TypeD' with the same CSI-RS resource.

Proposal 3:

- For PDSCH transmission scheme enhancement in HST scenario for Rel-17,

  - At least the QCL types and QCL source RS resource types as that of M-TRP transmission schemes in Rel-16 are supported.

For PDCCH with the transmission scheme agreed in RAN1 #103 e-meeting, since a DMRS antenna port for the PDCCH in a CORESET is quasi co-located with two TCI states, the MAC CE activation command for the CORESET associated with the PDCCH should be enhanced to be able to indicate two TCI states for the CORESET. At least same QCL types and the types of the QCL source RSs for each TCI state as that in Rel-16 are supported.

Proposal 4:

- For PDCCH transmission scheme enhancement in HST scenario for Rel-17,

  - The MAC CE activation command for CORESET should be enhanced to be able to indicate two TCI states;

  - At least the same QCL types and QCL source RS resource types for each TCI state as that in Rel-16 are supported.

M-TRP scheme identification

For M-TRP PDCCH transmission, the association relationship of CORESET/search space and TCI states is under discussion in agenda 8.1.2.1. If more than one transmission scheme is to be supported, the differentiation of the enhanced scheme in HST scenario for PDCCH and other enhanced schemes for PDCCH should be considered in agenda 8.1.2.1.

In Rel-16, a PDSCH can be indicated with a TCI codepoint with two TCI states for M-TRP transmission schemes of SDM 1a, FDM 2a, FDM 2b, TDM 3, and TDM 4. For SDM 1a, two DMRS ports within two different CDM groups are indicated, and the two DMRS ports are associated with two TCI states respectively. For FDM 2a, FDM 2b, TDM 3, and TDM 4, each DMRS port is associated with two TCI states and the DMRS ports are within the same CDM group when multiple DMRS ports are indicated. For scheme differentiation, FDM 2a, FDM 2b, and TDM 3 are identified by RRC parameter RepetitionScheme explicitly. TDM 4 is identified when the TDRA of DCI indicates an entry contains repetitionNumber-r16. SDM 1a is identified when the TDRA of DCI doesn’t indicate an entry contains repetitionNumber-r16 and a TCI codepoint with two TCI states is indicated and the DMRS port(s) within two CDM groups are indicated. The enhanced scheme for HST scenario also can be identified according to a RRC parameter explicitly or be identified by the configuration of DMRS ports of PDSCH, TCI indication and TDRA indication. The following two alternatives for the identification of the enhanced scheme for HST scenario can be considered:

- Alt 1: The enhanced scheme is identified according to RRC signaling explicitly. E.g, a new value of “SDMSchemeA” is added for RRC parameter RepetitionScheme, when SDMSchemeA is indicated and a TCI codepoint associated with 2 TCI state is indicated, the PDSCH is SFN-ed transmitted with the enhanced scheme.

- Alt 2: The enhanced scheme is identified according to the configuration of DMRS ports of PDSCH, TCI indication and TDRA indication. For this alternative, only when the following conditions are satisfied, the PDSCH is SFN-ed transmitted with the enhanced scheme:

  - RepetitionScheme is not configured;

  - the TDRA of DCI doesn’t indicate an entry contains repetitionNumber-r16

  - a TCI codepoint associated with 2 TCI state is indicated; and

  - the DMRS ports of the PDSCH are within the same CDM group.

For Alt 1, the procedure of M-TRP transmission schemes identification is as follows:

- If RepetitionScheme is set to SDMSchemeA and a TCI codepoint associated with 2 TCI states is indicated, the PDSCH is SFN-ed transmitted with the enhanced scheme of HST scenario in Rel-17;

- If RepetitionScheme is set to FDMSchemeA and a TCI codepoint associated with 2 TCI states is indicated and the DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with FDM 2a;

- If RepetitionScheme is set to FDMSchemeB and a TCI codepoint associated with 2 TCI states is indicated and the DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with FDM 2b;

- If RepetitionScheme is set to TDMSchemeA and a TCI codepoint associated with 2 TCI states is indicated and the DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with TDM 3;

- If the TDRA of DCI indicates an entry contains repetitionNumber-r16 and a TCI codepoint associated with 2 TCI states is indicated and DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with TDM 4;

- If the TDRA of DCI indicates an entry contains repetitionNumber-r16 and a TCI codepoint associated with 1 TCI state is indicated and DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with the fallback mode of TDM 4;

- If the TDRA of DCI doesn’t indicate an entry contains repetitionNumber-r16 and a TCI codepoint associated with 2 TCI states is indicated and DMRS port(s) of PDSCH are within two CDM group, the PDSCH is transmitted with SDM 1a.

Alt 1 allows PDSCH to be transmitted with the enhanced scheme of HST scenario with more than 2 layers.

For Alt 2, when PDSCH is transmitted with the enhanced scheme of HST scenario, its DMRS ports should be within one CDM group. The procedure of M-TRP transmission schemes identification is as follows:

- If the TDRA of DCI doesn’t indicate an entry contains repetitionNumber-r16 and a TCI codepoint associated with 2 TCI states is indicated and DMRS port(s) of PDSCH are within one CDM group, the PDSCH is transmitted with the enhanced scheme of HST scenario in Rel-17；

- If the TDRA of DCI doesn’t indicate an entry contains repetitionNumber-r16 and a TCI codepoint associated with 2 TCI states is indicated and DMRS port(s) of PDSCH are within two CDM group, the PDSCH is transmitted with SDM 1a;

- If RepetitionScheme is set to FDMSchemeA and a TCI codepoint associated with 2 TCI states is indicated and the DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with FDM 2a;

- If RepetitionScheme is set to FDMSchemeB and a TCI codepoint associated with 2 TCI states is indicated and the DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with FDM 2b;

- If RepetitionScheme is set to TDMSchemeA and a TCI codepoint associated with 2 TCI states is indicated and the DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with TDM 3;

- If the TDRA of DCI indicates an entry contains repetitionNumber-r16 and a TCI codepoint associated with 2 TCI states is indicated and DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with TDM 4;

- If the TDRA of DCI indicates an entry contains repetitionNumber-r16 and a TCI codepoint associated with 1 TCI state is indicated and DMRS ports of PDSCH are within the same CDM group, the PDSCH is transmitted with the fallback mode of TDM 4.

Since the enhanced scheme of HST scenario of Rel-17 also can be used by eMBB or other scenarios, restriction on the number of CDM group of DMRS port(s) is not preferred, Alt 1 is slightly preferred.

Proposal 5:

- For the identification of the enhanced transmission scheme in HST scenario for Rel-17,

  - For PDCCH, how to identify the scheme should be considered together with the enhanced schemes for PDCCH in agenda 8.1.2.1;

  - For PDSCH, the following schemes are considered:

    - Alt 1: Identified according to RRC signaling explicitly. E.g, a new value of “SDMSchemeA” is added for RRC parameter RepetitionScheme;

    - Alt 2: Identified according to the configuration of DMRS ports of PDSCH, TCI indication and TDRA indication, similar as Rel-16 SDM 1a. If this alternative is adopted, the DMRS port(s) of PDSCH should be within one CDM group.

- Enhancements with frequency offset pre-compensation

It was agreed in RAN1 #102 e-meeting that the following three steps can be assumed for frequency offset pre-compensation schemes:

- 1st step: Transmission of the TRS resource(s) from TRP(s) without pre-compensation

- 2nd step: Transmission of the uplink signal(s)/channel(s) with carrier frequency determined based on the received TRS signals in the 1st step

- 3rd step: Transmission of the PDCCH/PDSCH from TRP(s) with frequency offset pre-compensation determined based on the received signal/channel in the 2nd step

For simplicity, in this section, we present the transmission schemes with PDSCH under the scenario with 2 TRPs as example. Note that the schemes discussed in this section apply to both PDCCH and PDSCH, and the transmission schemes can be extended to the scenarios with multiple TRPs.

The following two options were listed to be considered for frequency offset pre-compensation:

- Option 1: Explicit reporting of the Doppler shift(s) acquired in the 1st step using CSI framework;

- Option 2: Implicit indication of the Doppler shift(s) using uplink signal(s) transmitted on the carrier frequency acquired in the 1st step;

Although the Doppler spread for PDSCH/DMRS can be eliminated by option 1, the overhead for CSI reporting and the design of CSI reporting are needed and UE-specific TRS resource(s) with frequency offset pre-compensation may be needed in the 3rd step for PDSCH/DMRS channel estimation. Hence option 1 is not preferred.

There are 2 alternatives for option 2:

  - Alt 1: In the 2nd step, multiple SRS resources are transmitted and each SRS resource is transmitted on the carrier frequency acquired by a corresponding TRS in the 1st step. The TRS is transmitted in TRP-specific/non-SFN manner.

  - Alt 2: In the 2nd step, one SRS is transmitted on a carrier frequency acquired by any TRS transmitted in the 1st step.

The procedure of Alt 1 is provided in Fig. 1, with

- 1st Step: 2 TRP-specific TRS resources are transmitted at fc without frequency offset pre-compensation, where fc is the DL center frequency determined by gNB;

2nd Step: 2 SRS resources are transmitted according to the carrier frequency acquired in the 1st step respectively, i.e. SRS 1 is transmitted at fc+f1+fUL-DL and SRS 2 is transmitted at fc+f2+fUL-DL, where f1, and f2 are Doppler shifts from TRP 1 and TRP 2 respectively, and fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS/TRS is SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of f1 and f2 respectively.

In the 3rd step, the frequency offset pre-compensation for each TRP is determined as a half of frequency difference of the SRS corresponding to the TRP and the configured UL carrier frequency.

Figure 1 Procedure of Alt 1, option 2

Although the above procedure can eliminate the Doppler spread for PDSCH/DMRS, it has the following drawbacks:

  - UE-specific & TRP-specific SRS resources are needed;

  - Multiple UL signals with different carrier frequency may cause inter-subcarrier interference among UL signals, which would damage the UL performance;

  - UE-specific TRS resource(s) with frequency offset pre-compensation may be needed in the 3rd step for PDSCH/DMRS channel estimation;

  - A QCL-like association relationship between UL RS in the 2nd step and DL RS in the 1st step is needed.

Based on whether the TRS resource(s) transmitted in the 3rd step is frequency offset pre-compensated or not, the following two schemes can be considered for Alt 2, Option 2:

- Scheme 1: The TRS resources transmitted in the 3rd step are frequency offset pre-compensated;

- Scheme 2: The TRS resources transmitted in the 3rd step are not frequency offset pre-compensated;

For scheme 1, we consider the following schemes:

- Scheme 1-1: SFN-ed DMRS /TRS transmission with UE-specific frequency pre-compensation applied in each TRP in the 3rd step.

Figure 2 Procedure of scheme 1-1

Scheme 1-1 can be seen as Rel-15 SFN-ed transmission scheme + UE-specific frequency offset pre-compensation. A possible transmission procedure for scheme 1-1 is shown in Fig. 2, with

- 1st Step: TRS resource(s) are transmitted at fc without frequency offset pre-compensation, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS/TRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+f1-f2 respectively, where ∆f' is a frequency determined by gNB.

For this scheme, gNB estimates Doppler shifts for each TRP with the SRS transmission, and determines the pre-compensation frequency values for each TRP accordingly. The transmission frequency of the SRS can be the UL carrier frequency that determined by UE according to any downlink signals. In particular, if ∆f'=0, i.e. there is no frequency offset pre-compensation for TRP1, the reception frequency of PDSCH/DMRS/TRS would be fc+f1.

Scheme 1-1 can eliminate the Doppler spread for PDSCH/DMRS. However, it requires UE-specific TRS resources and UE-specific SRS resource(s), which cause large overhead and is not expected.

- Scheme 1-2: SFN-ed DMRS/TRS transmission with UE-group common frequency pre-compensation applied in each TRP in the 3rd step.

The procedure of scheme 1-2 is the same as scheme 1-1 except that UE-group common frequency offset pre-compensation is used instead of the UE-specific frequency offset pre-compensation. So scheme 1-2 can be seen as Rel-15 SFN-ed transmission scheme + UE-group common frequency offset pre-compensation.

A possible transmission procedure for scheme 1-2 is shown in Fig. 3:

- 1st Step: TRS resource(s) are transmitted at fc without pre-compensation, where fc is the DL center frequency determined by gNB;

- 2nd Step: An SRS resource is transmitted according to the carrier frequency determine by a reference UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆fRUE + fUL-DL, where ∆fRUE is the frequency difference of the DL center frequency determined by the reference UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS/TRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+ f1,RUE-f2,RUE respectively, where ∆f' is a frequency determined by gNB.

Figure 3 Procedure of scheme 1-2

For scheme 1-2, gNB estimates Doppler shifts for each TRP with the SRS of a reference UE and determines the pre-compensation frequency values for each TRP accordingly. Assuming that for the reference UE, the frequency difference of the DL center frequency determined by the UE and fc is ∆fRUE, the Doppler shifts of TRP 1 and TRP 2 without Doppler pre-compensation are f1,RUE and f2,RUE respectively; and for a target UE, the frequency difference of the DL center frequency determined by the UE and fc is ∆f, the Doppler shifts of TRP 1 and TRP 2 without Doppler pre-compensation are f1 and f2 respectively. Then the reception frequency of the SRS transmitted by the reference UE is fc+∆f+ fUL-DL +f1,RUE in TRP 1 and fc+∆f+fUL-DL+f2,RUE in TRP 2. If the pre-compensation frequency value is ∆f' in TRP 1 and ∆f'+f1,RUE-f2,RUE in TRP 2, the reception frequency of PDSCH/DMRS/TRS transmitted to the target UE would be fc+∆f'+f1 for TRP 1 and fc+∆f'+f1+f1,RUE-f2,RUE for TRP 2 respectively. In particular, if ∆f'=∆fRUE, the reception frequency of PDSCH/DMRS/TRS transmitted to the target UE would be fc+∆fRUE+f1 for TRP 1 and fc+∆fRUE+f2+f1,RUE-f2,RUE for TRP 2 respectively. Then compared to the DL frequency fc+∆f that locked by the target UE, the Doppler shift of TRP 1 is ∆fRUE+f1-∆f and the Doppler shift of TRP 2 is ∆fRUE-∆f +f2+f1,RUE-f2,RUE, i.e. there is still Doppler spread of the target UE. gNB estimates Doppler shifts for each TRP with the SRS resource, and determines the pre-compensation frequency values for each TRP accordingly. In particular, if ∆f'=0, the reception frequency of PDSCH/DMRS/TRS from TRP 1 would be fc+f1, and the reception frequency of PDSCH/DMRS/TRS from TRP 2 would be fc+f2+f1,RUE-f2,RUE.

For scheme 1-2, the differences of the Doppler shifts among different TRPs are reduced and UE-specific TRS resource is not needed. However, since the pre-compensation frequency value is determined by the SRS of a reference UE, the Doppler spread for PDSCH cannot be well eliminated.

- Scheme 1-3: SFN-ed DMRS transmission and TRP-specific TRS transmission in the 3rd step, with UE-specific frequency pre-compensation applied in each TRP.

The procedure of scheme 1-3 is the same as scheme 1-1, except that TRP-specific TRS resources are transmitted in the 3rd step in scheme 1-3.

Scheme 1-3 can be seen as enhanced scheme in section 2 + UE-specific frequency offset pre-compensation. Fig. 4 shows a possible transmission procedure for scheme 1-3, where:

- 1st Step: TRS resource(s) are transmitted at fc without pre-compensation, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where fc is the DL center frequency determined by gNB, ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+f1-f2 respectively, 2 TRS resources are transmitted from the two TRPs with frequency offset pre-compensation of ∆f' and ∆f'+f1-f2 respectively, where ∆f' is a frequency determined by gNB.

Figure 4 Procedure of scheme 1-3

For scheme 1-3, gNB estimates Doppler shifts for each TRP with the SRS resource, and determines the pre-compensation frequency values for each TRP accordingly. In particular, if ∆f'=0, the reception frequency of PDSCH/DMRS/TRS would be fc+f1.

Similar as scheme 1-1, the Doppler spread for PDSCH/DMRS can be eliminated by scheme 1-3, and UE-specific TRS resources is required in the 3rd step.

- Scheme 1-4: SFN-ed DMRS transmission and TRP-specific TRS transmission in the 3rd step, with UE-group common frequency pre-compensation applied in each TRP.

The procedure of scheme 1-4 is the same as scheme 1-2 except that TRP-specific TRS is transmitted in the 3rd step in scheme 1-4. Compared to scheme 1-3, in the 3rd step of scheme 1-4, UE-group common frequency offset pre-compensation is used instead of UE-specific frequency offset pre-compensation, so scheme 1-4 can be seen as enhanced scheme in section 2 + UE-group common frequency offset pre-compensation.

In scheme 1-3 and scheme 1-4, two TRP-specific TRS resources are transmitted and they are frequency pre-compensated with same frequencies as that for PDSCH/DMRS, therefore the Doppler profile and delay profile of the channel of PDSCH can be estimated by the TRS resources, and QCL assumption variant E in RAN1 #103 meeting can be used, i.e., the same DMRS port(s) are associated with two TCI states, where both TCI states can be associated with QCL-TypeA.

UE-specific TRS requires large TRS overhead in the system, so scheme 1-1 and scheme 1-3 might not be good solutions in practice. Since scheme 1-2 can be seen as Rel-15 SFN-ed transmission scheme + UE-group common frequency pre-compensation and scheme 1-4 can be seen as Rel-17 enhanced transmission scheme in section 2 + UE-group common frequency pre-compensation, whether frequency pre-compensation is used is up to gNB’s implementation.

In scheme 2, TRS resources are not frequency pre-compensated, therefore UE-specific TRS transmission is not needed. The following alternatives can be considered for scheme 2:

Scheme 2-1: DMRS is SFN-ed transmitted from all TRPs with UE-specific frequency pre-compensation in each TRP; one TRS is transmitted from a reference TRP for Doppler properties estimation, and another TRS is SFN-ed transmitted for delay properties estimation, both TRS resources are transmitted without frequency offset pre-compensation.

Fig. 5 shows a possible transmission procedure for frequency offset pre-compensation scheme 2-1:

- 1st Step: One TRS resource is transmitted from a reference TRP, and another TRS is SFN-ed transmitted from all TRPs. all TRS resources are transmitted without frequency offset pre-compensation at fc, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where fc is the DL center frequency determined by gNB, ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of 0 and f1-f2, respectively.

For this scheme, one TRS for Doppler properties estimation and one TRS for delay properties estimation are configured for PDSCH/PDCCH.

QCL assumption variant B in RAN1 #103 meeting can be considered for scheme 2-1, i.e. the same DMRS port(s) are associated with two TCI states, where one of the TCI state is associated with {average delay, delay spread} and another TCI state is associated with QCL-TypeB.

Variant C in RAN 1#103 meeting assumes that one TCI state is associated with {delay spread} and another TCI state is associated with QCL-Type B. In our opinion, the TRS that SFN-ed transmitted for delay spread estimation also can be used for average delay estimation, introduce a new QCL type only with delay spread is not preferred.

Figure 5 Procedure of scheme 2-1

- Scheme 2-2: DMRS is SFN-ed transmitted from all TRPs with UE-specific frequency offset pre-compensation in each TRP; one TRS is transmitted from a reference TRP for Doppler properties estimation and another TRS is transmitted from the other TRP, both TRS resources are transmitted without frequency offset pre-compensation and are used for delay properties estimation.

Fig. 6 shows a possible transmission procedure for frequency offset pre-compensation scheme 2-2:

- 1st Step: One TRS resource is transmitted from a reference TRP, and another TRS is transmitted from other TRP, all TRS are transmitted without frequency offset pre-compensation at fc, where fc is the DL center frequency determined by gNB;

- 2nd Step: 1 SRS resource is transmitted according to the carrier frequency determine by UE. Whether the carrier frequency of the SRS resource is acquired in the 1st step or not is up to UE’s implementation.

We assume that the SRS resource in this step is transmitted at frequency fc+∆f+ fUL-DL, where fc is the DL center frequency determined by gNB, ∆f is the frequency difference of the DL center frequency determined by the UE and fc, fUL-DL is the frequency offset between DL carrier and UL carrier and it is indicated by gNB.

- 3rd Step: PDSCH/DMRS are SFN-ed transmitted from the two TRPs with frequency offset pre-compensation of 0 and f1-f2 respectively.

Figure 6 Procedure of scheme 2-2

The difference of scheme 2-2 and scheme 2-1 is that 2 TRP-specific TRS resources are used for delay properties estimation in the 3rd step for scheme 2-2. For scheme 2-2, if the transmit frequency of UL RS is up to UE’s implementation, a residual Doppler shift may be observed by UE. The advantages and disadvantages of scheme 2-2 are similar as scheme 2-1.

QCL assumption variant A in RAN1 #103 meeting can be considered for scheme 2-2, i.e. the same DMRS port(s) are associated with two TCI states, where one of the TCI state is associated with {average delay, delay spread} and another TCI state with QCL-TypeA.

QCL assumption variant E in RAN1 #103 meeting also can be considered for scheme 2-2, i.e. the same DMRS port(s) are associated with two TCI states, where both TCI states are associated with QCL-TypeA. If Variant E is adopted, Doppler shift and Doppler spread in one TCI state should be omitted according to a rule. For example, Doppler shift and Doppler spread in the second TCI state is omitted.

Fig. 7 and Fig. 8 provide the performance comparison of different schemes when UE is located in the middle of 4 TRPs. In Fig. 7, CFO and frequency estimation error in TRPs are not considered. In Fig. 8, CFO and frequency estimation error are considered in each TRP, where the frequency estimation error in TRPs is used to characterize the error of the pre-compensated frequency. In the evaluations, 4 TRPs are assumed and PDSCH/DMRS is SFN-ed transmitted from 4 TRPs. Since up to 2 TCI states are supported, we assume that for the enhanced scheme in section 2 without frequency offset pre-compensation, pre-compensation scheme 1-3, 1-4 and 2-2, TRS 1 is SFN-ed transmitted from the 2 TRPs in front of the train and TRS 2 is SFN-ed transmitted from the 2 TRPs behind the train; in pre-compensation scheme 2-1, TRS 1 is SFN-ed transmitted from the 2 TRPs in front of the train, TRS 2 is SFN-ed transmitted from the 4 TRPs. More simulation assumptions can be found in Table I in the Appendix, and more simulation results can be found in [2]. It can be seen from the simulation results that

- For the performance of schemes with frequency offset pre-compensation, the effect of CFO and frequency estimation error is significant.

- When CFO and frequency estimation error in TRPs are not considered,

  - Compared to Rel-15 SFN-ed PDSCH transmission scheme, obvious performance gain can be achieved by the solutions with frequency offset pre-compensation;

  - The performance of the enhanced scheme in section 2 without frequency offset pre-compensation is similar as that of pre-compensation scheme 1-2 (Rel-15 SFN-ed transmission + UE-group common frequency offset pre-compensation) and is worse than that of other frequency offset pre-compensation schemes (pre-compensation scheme 1-1, 1-3, 2-1, 2-2 and 1-4);

  - Similar performance can be achieved by pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensation schemes (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2);

  - The performance of pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensated (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2) outperforms the performance of pre-compensation schemes with PDSCH transmitted with UE-group common frequency offset pre-compensated (pre-compensation scheme 1-2 and 1-4);

- When CFO and frequency estimation error are considered in each TRP,

  - Compared to Rel-15 SFN-ed PDSCH transmission scheme, obvious performance gain can be achieved by the solutions with frequency offset pre-compensated;

  - The performance of the schemes of enhanced scheme in section 2 + frequency offset pre-compensation (pre-compensation scheme 1-3 and 1-4) outperforms the performance of the enhanced scheme in section 2 without frequency offset pre-compensation;

  - The performance of pre-compensation schemes with PDSCH transmitted with UE-specific frequency offset pre-compensation (pre-compensation scheme 1-1, 1-3, 2-1 and 2-2) outperforms the performance of pre-compensation schemes with PDSCH transmitted with UE-group common frequency offset pre-compensated and SFN-ed TRS transmission (pre-compensation scheme 1-2);

  - Similar performance can be achieved by pre-compensation scheme 1-1, 2-1 and 2-2, and their performance is worse than that of pre-compensation scheme 1-3 and 1-4.

Figure 7 Performance comparison of different schemes, without CFO and without frequency estimation error in TRPs

Figure 8 Performance comparison of different schemes, with CFO = 0.05ppm and frequency estimation error = 0.05ppm in each TRP

Although scheme 1-3 outperforms option 2-1 and option 2-2 when CFO and frequency estimation error are considered, scheme 1-3 requires UE-specific frequency pre-compensated TRS resources. If scheme 1-3 is adopted, the overhead of TRS in the system can be large. Compared to scheme 1-3, the overhead of TRS in scheme 1-4 is reduced. However, UE-group common frequency pre-compensation is needed in scheme 4, its performance may not be ensured, especially when there are multiple trains to be served by same TRPs. Since scheme 2-1 and 2-2 can achieve significantly performance gain compared to Rel-15 SFN-ed transmission scheme and they don’t require UE-specific TRS transmission or UE-group common frequency offset pre-compensation, scheme 2-1 or 2-2 are preferred to be supported.

Based on the analysis and the simulation results, we have

Proposal 6:

- For the enhanced transmission scheme with frequency pre-compensation,

  - Frequency pre-compensation on TRS is not needed;

  - Explicitly reporting of the Doppler shift(s) acquired in the 1st step is not supported;

  - Scheme 2-1 or scheme 2-2 is supported:

    - Variant A or Variant B of TCI states indication for PDCCH/PDSCH is supported, and each TCI state can be associated with QCL-Type D optionally.

For the proposed schemes, gNB would choose one TRP as a reference TRP for frequency pre-compensation. It is obvious that best performance would be achieved if UE tracks the DL frequency of the carrier based on the signal from the reference TRP. In order to have the best performance, UE can be indicated with which TRS is used for DL frequency tracking/which TRP is used as the reference TRP explicitly or implicitly. In Rel-16, it’s up to UE’s implementation on how to track DL frequency. In general, for the initial access procedure, UE would track DL frequency by SSB. Then UE can track DL frequency by TRS or DMRS up to its implementation. Indicating UE which DL signal is used for DL frequency tracking is a big restriction on UE’s behavior, and its influence is not clear. We prefer not to indicate UE which DL signal is used for DL frequency tracking. UE can track DL frequency based on the TRS used for Doppler shift estimation.

Since PDSCH/PDCCH and its associated DMRS is not frequency offset pre-compensated in the reference TRP, and the pre-compensated frequency in the other TRP(s) is determined based on the difference of Doppler shift that estimated by the TRP(s) and the reference TRP. The SRS for Doppler shift estimation is transmitted in which frequency is not important. Association of DL and UL signals is not needed for HST-SFN scenario with frequency pre-compensation.

Proposal 7:

- For the enhanced transmission scheme with frequency pre-compensation,

  - The indication on which RS is used for DL frequency tracking is not needed;

  - Association of DL and UL signals is not needed.

- Conclusions

In this contribution, we provide our views on the enhancement for Rel-17 HST-SFN deployment scenarios. We have the following observations and proposals:

Observation 1:

A new definition on QCL association relationship of one antenna port and one antenna port group is needed in Rel-17.

Proposal 1:

- A new definition on QCL association relationship of one antenna port and one antenna port group is introduced.

  - E.g. Antenna port x and antenna port group Y are said to be quasi co-located if the large-scale properties of the channel over which a symbol on antenna port x is conveyed can be inferred from the combination of channels over which the symbols on the antenna ports of the antenna port group y is conveyed. The large-scale properties include one or more of delay spread, Doppler spread, Doppler shift, average gain, average delay, and spatial Rx parameters.

Proposal 2:

- Up to two TCI states are supported in HST scenario for FR 2.

Proposal 3:

- For PDSCH transmission scheme enhancement in HST scenario for Rel-17,

  - At least the QCL types and QCL source RS resource types as that of M-TRP transmission schemes in Rel-16 are supported.

Proposal 4:

- For PDCCH transmission scheme enhancement in HST scenario for Rel-17,

  - The MAC CE activation command for CORESET should be enhanced to be able to indicate two TCI states;

  - At least the same QCL types and QCL source RS resource types for each TCI state as that in Rel-16 are supported

Proposal 5:

- For the identification of the enhanced transmission scheme in HST scenario for Rel-17,

  - For PDCCH, how to identify the scheme should be considered together with the enhanced schemes for PDCCH in agenda 8.1.2.1;

  - For PDSCH, the following schemes are considered:

    - Alt 1: Identified according to RRC signaling explicitly. E.g, a new value of “SDMSchemeA” is added for RRC parameter RepetitionScheme;

    - Alt 2: Identified according to the configuration of DMRS ports of PDSCH, TCI indication and TDRA indication, similar as Rel-16 SDM 1a. If this alternative is adopted, the DMRS port(s) of PDSCH should be within one CDM group.

Proposal 6:

- For the enhanced transmission scheme with frequency pre-compensation,

  - Frequency pre-compensation on TRS is not needed;

  - Explicitly reporting of the Doppler shift(s) acquired in the 1st step is not supported;

  - Scheme 2-1 or scheme 2-2 is supported:

    - Variant A or Variant B of TCI states indication for PDCCH/PDSCH is supported, and each TCI state can be associated with QCL-Type D optionally.

Proposal 7:

- For the enhanced transmission scheme with frequency pre-compensation,

  - The indication on which RS is used for DL frequency tracking is not needed;

  - Association of DL and UL signals is not needed.

- References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, RAN #86.

[2] R1-2100350, Evaluation of enhancements for multi-TRP, CATT, RAN1 #104.

- Appendix

Table-I Link level simulation assumptions for HST-SFN

| Parameter | Value |
| --- | --- |
| Propagation condition | 4-tap channel model |
| TRP layout | Ds=700m, Dmin=150m |
| Carrier frequency | 3.5 GHz |
| Speed | 500 km/h |
| BW | 20 MHz |
| SCS | 30 KHz |
| Number of RBs for PDSCH | 50 PRBs |
| Duplexing | TDD |
| TDD pattern | DDDDDDDSUU |
| gNB antenna configuration | 2Tx |
| UE antenna configuration | 2Tx |
| DMRS configuration | DMRS type 1, 1+1+1 |
| TRS configuration | Periodic of 10ms, 2-slot pattern |
| MCS | MCS 13 based on 64QAM table |
| Rank | Rank 1 |