3GPP TSG RAN WG1 #104-e                               R1-2100886

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.13.2

Source: 	LG Electronics

Title: 	Discussion on multi-cell PDSCH scheduling via a single DCI

Document for:	Discussion and decision

- Introduction

In RAN1#103-e, there was several agreements on simulation assumptions/parameters for further study of multi-cell PDSCH scheduling via a single DCI [1]. In this contribution, we discuss and provide our views on the multi-cell PDSCH scheduling by using single DCI, in terms of the necessity and potential impacts (if introduced).

- Discussion

Regarding the multi-cell PDSCH scheduling (to schedule multiple PDSCHs transmitted on multiple cells) by using a single DCI, as clearly stated in the above WID, it is necessary to clarify/justify first on the technical motivation and benefits by introducing such kind of scheduling method, on top of specifying the cross-CC PDSCH/PUSCH scheduling from Scell to Pcell.

If potential reduction of the DCI overhead consumed for the PDSCH scheduling is considered as the motivation/benefits (by minimizing the increase of DCI payload size according to the WID), it might induce some scheduling restriction on the multiple cells in the gNB side due to the limited DCI size, in terms of flexibility and/or accuracy to indicate the scheduling information/parameters. Note that this multi-cell scheduling is quite different from the multi-TTI scheduling at least in terms of frequency domain resource allocation and TB-related parameters such as HARQ process ID and MCS value, which are likely to be independent between different cells while those information/parameters are likely to have dependency between different TTIs on a same cell. In summary, there seems to be no benefit with multi-cell scheduling by single DCI compared to multi-TTI scheduling by single DCI from the perspective of DCI overhead.

If potential congestion mitigation of the PDCCH transmissions on a single scheduling cell is considered as the motivation/benefits (by keeping the total PDCCH blind decoding budget according to the WID), the gNB could handle and avoid such situation, for example, by properly configuring PDCCH monitoring periodicity for multiple UEs (with different period) and/or by properly configuring cross-CC scheduling relationship for multiple UEs (with different pairing of scheduling/scheduled cells, e.g., scheduling from cell 1 to cell 2 for some UEs, scheduling from cell 2 to cell 1 for other UEs). Besides, the gNB could mitigate the PDCCH congestion on a cell by configuring single DCI based multi-TTI PUSCH scheduling (introduced in Rel-16 NR-U) for the UEs capable of that feature. Furthermore, it should be noted that multi-TTI PDSCH (and PUSCH) scheduling by single DCI was approved to be supported in Rel-17 as one of the objectives in WID for NR above 52 GHz.

Nevertheless, assuming the case where the single DCI based multi-cell PDSCH scheduling is introduced, at least following issues would need to be addressed, and relevant specification impacts (and standardization workload for them) are expected.

- How to indicate the multiple cells with PDSCH transmission by single scheduling DCI (e.g., by using CIF field (or other certain field), or adding a new field)

- How to compose (and signal) the DCI fields (e.g., TDRA field/information for two cells) in the multi-cell PDSCH scheduling DCI

- How to construct PDCCH region within CORESET for the multi-cell scheduling DCI transmission

- How to allocate (and handle) PDCCH BD candidates for the multi-cell scheduling DCI

Proposal #1: It is necessary to clarify/justify first on the technical motivation and benefits by introducing the single DCI based multi-cell PDSCH scheduling, on top of specifying the cross-CC PDSCH/PUSCH scheduling from Scell to Pcell.

Proposal #2: At least following issues would need to be addressed, and relevant specification impacts (and standardization workload for them) are expected, if the single DCI based multi-cell PDSCH scheduling is introduced.

- How to indicate the multiple cells with PDSCH transmission by single scheduling DCI

- How to compose (and signal) the DCI fields in the multi-cell PDSCH scheduling DCI

- How to construct PDCCH search space for the multi-cell scheduling DCI transmission

- How to allocate (and handle) PDCCH BD candidates for the multi-cell scheduling DCI

- Conclusions

In this contribution, the necessity and potential impacts of multi-cell PDSCH scheduling by using single DCI were discussed, and the followings are proposed.

Proposal #1: It is necessary to clarify/justify first on the technical motivation and benefits by introducing the single DCI based multi-cell PDSCH scheduling, on top of specifying the cross-CC PDSCH/PUSCH scheduling from Scell to Pcell.

Proposal #2: At least following issues would need to be addressed, and relevant specification impacts (and standardization workload for them) are expected, if the single DCI based multi-cell PDSCH scheduling is introduced.

- How to indicate the multiple cells with PDSCH transmission by single scheduling DCI

- How to compose (and signal) the DCI fields in the multi-cell PDSCH scheduling DCI

- How to construct PDCCH search space for the multi-cell scheduling DCI transmission

- How to allocate (and handle) PDCCH BD candidates for the multi-cell scheduling DCI

- References

RAN1 chairman’s notes, RAN1#103-e