3GPP TSG-RAN WG1 Meeting #104-e	R1-2102119

# E-meeting, January 25 – February 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0017 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Channel Access for Multi-Channel transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Nokia), LGE, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The agreements related to UL and DL transmission in the case of no intra-cell guard bands have not been captured correctly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For an active UL BWP configured with no intra-cell guard band, a UE is allowed to transmit UL transmission only if the UE succeeds LBT for all RB set(s) corresponding to the UL BWP. For the DL, if gNB transmits DL transmission to a UE configured with DL active BWP where intraCellGuardBandDL-r16 for the corresponding serving cell indicates to the UE that no intra-cell guard-bands are configured, gNB is allowed to transmit DL transmission to the UE only if gNB succeeds LBT for the whole DL BWP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect gNB and UE operation with no intra-cell guard bands. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.0.4; 4.1.6.1; 4.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact Analysis: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

##### 4.2.1.0.4	Channel access procedures for UL multi-channel transmission(s)

If a UE

-	is scheduled to transmit on a set of channels C, and if Type 1 channel access procedure is indicated by the UL scheduling grants for the UL transmissions on the set of channels C, and if the UL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C , or

-	intends to perform an uplink transmission on configured resources on the set of channels C with Type 1 channel access procedure, and if UL transmissions are configured to start transmissions on the same time all channels in the set of channels C, and

if the channel frequencies of set of channels C is a subset of one of the sets of channel frequencies defined in clause 5.7.4 in [2]

-	the UE may transmit on channel ci∈C using Type 2 channel access procedure as described in clause 4.2.1.2,

-	if Type 2 channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and

-	if the UE has accessed channel cj using Type 1 channel access procedure as described in clause 4.2.1.1,

-	where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C.

-	if a UE is configured without intra-cell guard band(s) on a UL bandwidthpart as described in clause 7 in [8], the UE may not transmit on a channel  within the bandwidth of the carrier, if the UE fails to access any of the channels of the UL bandwidthpart.

-	otherwise, the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured by UL resources.

<Unchanged parts are omitted>

#### 4.1.6.1	Type A multi-channel access procedures

An eNB/gNB shall perform channel access on each channel ci∈C, according to the procedures described in clause 4.1.1, where C is a set of channels on which the eNB/gNB intends to transmit, and i=0,1,…q-1, and q is the number of channels on which the eNB/gNB intends to transmit.

The counter N described in clause 4.1.1 is determined for each channel ci and is denoted as Nci. Nci is maintained according to clause 4.1.6.1.1 or 4.1.6.1.2.

If a gNB configures a carrier without intra-cell guard bands as described in clause 7 in [8], the gNB may not transmit on channel ci∈C within the bandwidth of the carrier, if the gNB fails to access any of the channels of the carrier bandwidth.

<Unchanged parts are omitted>

#### 4.1.6.2	Type B multi-channel access procedure

A channel cj∈C is selected by the eNB/gNB as follows:

-	the eNB/gNB selects cj by uniformly randomly choosing cj from C before each transmission on multiple channels ci∈C, or

-	the eNB/gNB selects cj no more frequently than once every 1 second,

where C is a set of channels on which the eNB/gNB intends to transmit, i=0,1,…q-1, and q is the number of channels on which the eNB intends to transmit.

To transmit on channel cj

-	the eNB/gNB shall perform channel access on channel cj according to the procedures described in clause 4.1.1 with the modifications described in clause 4.1.6.2.1 or 4.1.6.2.2.

To transmit on channel ci≠cj, ci∈C

-	for each channel ci, the eNB/gNB shall sense the channel ci for at least a sensing interval Tmc=25us immediately before the transmitting on channel cj, and the eNB/gNB may transmit on carrier ci immediately after sensing the channel ci to be idle for at least the sensing interval Tmc. The channel ci is considered to be idle for Tmc if the channel is sensed to be idle during all the time durations in which such idle sensing is performed on the channel cj in given interval Tmc.

The eNB/gNB shall not transmit a transmission on a channel ci≠cj, ci∈C, for a period exceeding Tmcot,p as given in Table 4.1.1-1, where the value of Tmcot,p is determined using the channel access parameters used for channel cj.

For the procedures in this clause, the channel frequencies of the set of channels C selected by gNB, is a subset of one of the sets of channel frequencies defined in [6].

If a gNB configures a carrier without intra-cell guard band(s) as described in clause 7 in [8], the gNB may not transmit on channel ci∈C within the bandwidth of the carrier, if the gNB fails to access any of the channels of the carrier bandwidth.

<Unchanged parts are omitted>