3GPP TSG RAN WG1 Meeting #104-e	R1-21xxxxx

e-Meeting, January 25 – February 05, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | xxxx | rev | - | Current version: | V16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on multicast gap in multi-TB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), Lenovo, Motorola mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5 -Core |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PDSCH mapping to BL/CE DL subframe(s) doesn’t consider the skipping the scheduling gaps when UE is configured with higher layer parameter multiTB-Gap and the PDSCH corresponds to an MPDCCH with DCI CRC scrambled by G-RNTI in subclause 7.1.11 in TS 36.213. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When UE is configured with higher layer parameter multiTB-Gap and the PDSCH corresponds to an MPDCCH with DCI CRC scrambled by G-RNTI, the PDSCH mapping to BL/CE DL subframe skips the scheduling gap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDSCH is not mapped to BL/CE DL subframe correctly when multicast gap is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

- 7.1.11	PDSCH subframe assignment for BL/CE UE

A BL/CE UE shall upon detection of a MPDCCH with DCI format 6-1A/6-1B/6-2 intended for the UE, decode the corresponding PDSCH in subframe(s) n+ki with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted and is determined from the starting subframe of MPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	the value of is the number of scheduled TB determined in the corresponding DCI if present, otherwise;

-	the value of  is determined by the repetition number field in the corresponding DCI, where  are given in Table 7.1.11-1, Table 7.1.11-2 and Table 7.1.11-3, respectively

-	if the UE is configured with higher layer parameter multiTB-Gap and the PDSCH corresponds to an MPDCCH with DCI CRC scrambled by G-RNTI,

-	subframe(s) ni = n+ki with i=0,1,…, NTBN-1 are NTBN BL/CE DL subframe(s), where, subframe n+x is the second BL/CE DL subframe after subframe n and for i=1,…,N×NTB-1, subframe ni is the first BL/CE DL subframe after subframe ni-1+Ngap×δ(i mod N), Ngap is given by higher layer parameter multiTB-Gap, and δd=1, d=00,d≠0.

-	otherwise,

-	subframe(s) ni = n+ki with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE DL subframe(s), where, , and subframe n+x is the second BL/CE DL subframe after subframe n.

-	for ,

-	if the UE is configured with higher layer parameter interleaving in ce-PDSCH-MultiTB-Config, and PDSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and  where  for BL/CE UE configured with CEModeA,  for BL/CE UE configured with CEModeB,

-	BL/CE DL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	BL/CE DL subframes  with  are associated with TBr+1 , .

<Unchanged parts are omitted>