3GPP TSG RAN WG1 #104-e	                                                                 R1-210XXXX

e-Meeting, January 25th – February 5th, 2021

Agenda item:	7.1

Source:	Moderator (CATT)

Title:	Summary of [104-e-NR-7.1CRs-15] Draft CR on Type-2 HARQ -ACK codebook in physical uplink shared channel

Document for:	Discussion and Decision

Introduction

This document is created to facilitate the email discussion of “[104-e-NR-7.1CRs-15] Draft CR on Type-2 HARQ-ACK codebook in physical uplink shared channel”. This email thread is triggered by draft CRs in [1][2][3].

According to TS38.213 clause 9.1.3.2, for a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by a DCI format 0_1, UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1 according to the value of  in the DCI format 0_1, which is used to indicate the total number of HARQ-ACKs in response to PDSCHs associated with PDCCH and PDCCH indicating SPS PDSCH release to be multiplexed in the PUSCH.

If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, with the following modifications: - For the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, after the completion of the and loops, the UE sets where is the value of the DAI field in DCI format 0_1 according to Table 9.1.3-2 - For the case of first and second HARQ-ACK sub-codebooks, DCI format 0_1 includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook - harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

The HARQ-ACK for SPS PDSCH, if any, is appended after the HARQ-ACK(s) in response to PDSCHs associated with PDCCH and PDCCH indicating SPS PDSCH release.

In addition, it is specified in TS38.213 clause 9.1.3.2 that for a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by DCI format 0_1, if  in the DCI format 0_1 and UE has no HARQ-ACK(s) in response to PDSCHs associated with PDCCH, PDCCH indicating SPS PDSCH release or SPS PDSCH, the UE does not multiplex HARQ-ACK in the PUSCH.

If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1 with DAI field value and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release on any serving cell and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission. If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1 with first DAI field value or with second DAI field value and the UE has not received any PDCCH within the monitoring occasions for PDCCH with DCI format 1_0 or with DCI format 1_1, respectively, for scheduling PDSCH receptions or SPS PDSCH release on any serving cell and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission.

However, for the similar case as above except that UE has HARQ-ACK for SPS PDSCH to be multiplexed in the PUSCH, UE would multiplex 5 bits in the PUSCH according to Rel-15 specifications. Therefore, the draft CRs in [1][2][3] propose to clarify that for a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by DCI format 0_1, if  in the DCI format 0_1 and UE has HARQ-ACK(s) for SPS PDSCH only (i.e. no HARQ-ACK(s) in response to PDSCHs associated with PDCCH or PDCCH(s) indicating SPS PDSCH release), the UE multiplexes HARQ-ACK(s) for SPS PDSCH only in the PUSCH.

Company views

Q1: In Rel-15, for a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by DCI format 0_1, if  in the DCI format 0_1 and UE has a HARQ-ACK for SPS PDSCH only (i.e. no HARQ-ACK(s) in response to PDSCHs associated with PDCCH or PDCCH(s) indicating SPS PDSCH release), what is the intended UE behaviour?

- Alternative 1: UE multiplexes 1-bit HARQ-ACK for SPS PDSCH in the PUSCH

- Alternative 2: UE multiplexes 5-bit HARQ-ACKs in the PUSCH

| Company | Alternative 1 or Alternative 2 | Comment |
| --- | --- | --- |
| OPPO | Alternative 2, if in the DCI format 0_1 | If only one SPS PDSCH is transmitted by gNB, the value of in the DCI format 0_1 should be smaller, e.g. 1. If the value of in the DCI format 0_1 is 4, then the intended behavior is alternative 2. |
| vivo | Alternative 1 | DAI in the DCI format 0_1 is used to count DG PDSCHs/SPS release only without considering SPS PDSCH. is used to indicate there is no HARQ-ACK for DG PDSCH/SPS release (if the UE has not received any PDCCH within the monitoring occasions). |

Q2: In Rel-15, for a type-2 HARQ-ACK codebook transmission in a PUSCH scheduled by DCI format 0_1, if  in the DCI format 0_1 and UE has a HARQ-ACK for SPS PDSCH only (i.e. no HARQ-ACK(s) in response to PDSCHs associated with PDCCH or PDCCH(s) indicating SPS PDSCH release), what is the UE behaviour according to current specification?

- Alternative 1: UE multiplexes 1-bit HARQ-ACK for SPS PDSCH in the PUSCH

- Alternative 2: UE multiplexes 5-bit HARQ-ACKs in the PUSCH

| Company | Alternative 1 or Alternative 2 | Comment |
| --- | --- | --- |
| OPPO | Alternative 2 | The same comment as Q1 |
| vivo | Alternative 1 | The same comment as Q1 |

Q3: Do you agree with the principle of the CRs? If not, why?

| Company | Agree or not | Comment |
| --- | --- | --- |
| OPPO | No | Current specification works. It is not an essential issue. |
| vivo | agree |  |

Conclusion

To be added after the discussion.

- Reference

- R1-2100083		Draft CR on Type-2 HARQ-ACK codebook in physical uplink shared channel	ZTE

- R1-2100324		Correction on HARQ-ACK transmission in PUSCH	CATT

- R1-2100325		Correction on HARQ-ACK transmission in PUSCH	CATT