3GPP TSG RAN WG1 #104-e							    R1-210xxxx

e-Meeting, January 26th – February 5th, 2021

Source:	Moderator (Intel Corporation)

Title:	Discussion on [104-e-NR-5G_V2X-04]

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

This document provides discussion on the identified thread related to Mode-2 resource allocation in RAN1#104-e:

[104-e-NR-5G_V2X-04]: UE behaviour regarding non-monitored slots in mode 2, till 1/28, with potential CRs till 2/2 – Sergey (Intel)

- M2-17: Clarify that hypothetical SCI in step 5) assumes N=1 num of repetitions

- Changes for the uncaptured agreement (M2-3: Capture RAN1#103-e agreement on pre-emption) can be discussed during the CR preparation

Outcome Summary

TBD

Discussion

## Clarification on hypothetical SCI content during exclusion of slots related to non-monitored slots

In [15] (R1-2101533, Sharp) it is analysed that current implementation of step 5) in section 8.1.4 of TS 38.214 may be ambiguous. The issue is that this step invokes step 6)-c) for an assumption of hypothetical SCI to determine excluded slots and resource blocks. In the same time step 6)-c) refers to section 8.1.5 for determination of slots, but since a hypothetical SCI does not have a content it is impossible to derive the number of reserved slots N = 1 or 2 or 3.

To fix that, it is proposed to clarify in step 5) that the hypothetical SCI is assumed with N = 1.

Q1-1: Do you agree to clarify in step 5) of section 8.1.4 of TS 38.214, that the hypothetical SCI is assumed with N = 1?

| Source | Answer | Comment, if any |
| --- | --- | --- |
| NEC | Agree/Prefer | - N=1 seems reasonable for a hypothetical SCI with no TRIV. - The 2nd option may be to assume all the slots within the 32 slots from the tm+q×Prsvp_RX'SL are reserved. But this option sounds a bit excessive, we'd like to follow majority's view. |
| Huawei, HiSilicon | Agree | Clarification is needed, N=1 seems straightforward. |
| Sharp | Agree |  |
| Samsung | Agree |  |
| QC | Agree |  |
| Apple | Agree |  |
| CATT, GOHIGH | Not necessary | We think the current spec is clear, “all subchannels of the resource in this slot” only refers to the resource in one slot. It is unnecessary to further clarify N=1. |
| ZTE | No | Regarding the current spec, the hypothetical SCI received in slot t'mSL and indicates all subchannels of the resource pool in this slot, that means only one transmission is indicated in the slot, i.e. N=1. We agree with the intention, but we don’t think spec change is necessary. |
| OPPO | Not necessary | Same understanding as CATT and ZTE in that the hypothetical SCI only indicating all subchannels in this slot. There is no other slots indicated by this hypothetical SCI. So N must be 1. |
| Ericsson | Not necessary | As stated by others it is quite clear from the procedure text that N=1. |
| NTT DOCOMO | Agree | Clarification is needed. Regarding ‘in this slot’, the hypothetical SCI may indicate other slots additionally. Clarifying N=1 would be better. |
| Vivo | Not necessary | In step 5, it is clear to say ‘the hypothetical SCI indicates all subchannels of the resource pool in this slot’. Thus, no further clarification. |
| Nokia, NSB | Not needed | The intention of the existing text seems clear and no interpretation other than N=1 seems reasonable. While one can come up with interpretations which are not precluded by the text, these interpretations would clearly be unreasonable. |

Q1-2: If the answer in Q1-1 is positive, do you agree to implement the following change in step 5) of section 8.1.4 of TS 38.214 (TP#8 from R1-2101533)?

<<< unchanged parts omitted >>> 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot t'mSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList and a hypothetical SCI format 1-A received in slot t'mSL with 'Resource reservation period' field set to that periodicity value, indicating all subchannels of the resource pool in this slot and N=1 actual resource, condition c in step 6 would be met. <<< unchanged parts omitted >>>

| Source | Answer | Comment, if any |
| --- | --- | --- |
| NEC | Agree |  |
| Huawei, HiSilicon | Ok with some revision | Suggest to add the following red part to be clearer: - “and N=1 actual resource as defined in Clause 8.1.5” |
| Sharp | Agree | Agree with HW’s revision. |
| Samsung | Agree |  |
| Apple | Agree |  |
| CATT, GOHIGH | Not necessary | See comment in Q1-1 |
| ZTE | No | Refer to comment in Q1-1 |
| NTT DOCOMO | Agree |  |

Based on the discussion, it seems the change is not required since no other interpretations than N=1 is possible.

Intermediate conclusion:

- Current specification in section 8.1.4 of TS 38.214 is clear enough to interpret that in step 5) the hypothetical SCI 1-A is assumed with N=1

## Capturing RAN1#103-e agreement on pre-emption

At the last meeting, the following late agreement was made which needs to be implemented in specifications:

Agreements: - When a UE checks pre-emption for a resource, the UE identifies a candidate resource set based on steps 1-7 in clause 8.1.4 TS 38.214 - After the candidate set is identified, the UE checks SL-RSRP measurement and priority condition as per agreements, for resource(s) {r’} subject to pre-emption overlapping with received SCI 1-A and not included in the candidate set, where the RSRP threshold is the final threshold after executing steps 1-7 i.e. includes all necessary increments for reaching X%.

It was already discussed in RAN1#103-e that the agreement may be implemented by letting UE to check the RSRP threshold for a resource which is even not in the identified resource set. Two sources propose TPs for the agreement:

R1-2100204, Huawei, HiSilicon:

--------------------------- Start of Text Proposal for TS 38.214 ------------------------ <Unchanged parts omitted> 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <Unchanged parts omitted> The UE shall report set SA to higher layers. If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers. If a resource ri' from the set (r0',r1',r2',…) is not a member of SA, checks whether the RSRP measurement is higher than Th'prioRX,prioTX for the received SCI format 1-A which overlaps with ri' according to step 6) with an associated priority prioRX, where prioRX satisfies one of the following conditions, and Th'prioRX,prioTX is the final threshold after executing steps 1)-7), i.e. includes all necessary increments for reaching X%. - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX If the RSRP measurement is higher than Th'prioRX,prioTX, then the UE shall report pre-emption of the resource ri' to higher layers. <Unchanged parts omitted> ------------------------------------End of Text Proposal -------------------------------

R1-2100630, Intel Corporation:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <<< Unchanged parts omitted >>> If a resource ri' from the set (r0',r1',r2',…) - is not a member of SA, and - if the RSRP measurement performed according to clause 8.4.2.1 for a received SCI format 1-A overlapped with the resource ri' is higher than ThprioRX,prioTX including all increments after execution of steps 1-7 above, and - if an associated priority prioRX, satisfies one of the following conditions, then the UE shall report pre-emption of the resource ri' to higher layers. - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX <<< Unchanged parts omitted >>>

Q2-1: Which of the above text proposal could be a starting point for implementing RAN1#103-e agreement? Do you have any other suggestions?

| Source | Comments |
| --- | --- |
| NEC | Either is OK, the one from Intel seems more concise. |
| Huawei, HiSilicon | It’s better to add the following red part to be more accurate, so the TP provided by Huawei/HiSilicon can be considered as a starting point. - “…higher than Th'prioRX,prioTX for the received SCI format 1-A which overlaps with ri' according to step 6) …” |
| Sharp | Both TPs are OK to us. For the TP from Intel, it seems that “overlapped with …” needs to refer to Step 6) as Huawei’s TP/comment. In our understanding, the condition of “is not a member of SA” seems unnecessary , since ri' is excluded in Step 5) already and we believe this is the main reason why we had such an agreement. If companies are OK to this condition, we are fine. |
| Samsung | Intel’s TP looks better |
| QC | Agree with NEC |
| Apple | Either is fine. The second text proposal is a little preferred as a starting point. |
| CATT. GOHIGH | Both are fine, slightly prefer second TP as a starting point. |
| ZTE | Either is ok. |
| OPPO | Intel’s TP is clearer and Huawei’s TP is more concrete. We suggest to use the TP from Intel as the start point. We also propose to replace the description of “ThprioRX,prioTX including all increments after execution of steps 1-7 above” with “Th'prioRX,prioTX, and Th'prioRX,prioTXis the final threshold after executing steps 1)-7), i.e. includes all necessary increments for reaching X%.” |
| Ericsson | Agree with NEC |
| NTT DOCOMO | Either is fine. |
| vivo | Either is fine |
| Nokia, NSB | Prefer the structure of Intel’s text and hence to use Intel as starting point. Some suggested changes: - To make it clearer what the text is about move “the UE shall report pre-emption of the resource r_i^\prime to higher layers” to the first line, e.g. If a resource r_i^\prime from the set (r_0^\prime,r_1^\prime,r_2^\prime,\ldots) meets the conditions below then the UE shall report pre-emption of the resource ri' to higher layers - “received SCI format 1-A overlapped with the resource r_i^\prime” seems somewhat sloppy language and should be replaced by text based on the condition in 6c. |

It seems in terms of structure the second TP can be a starting point as preferred by majority of views. However, adjustments are required as also pointed out by several companies. Considering all the comments, the following TP is proposed:

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 <<< Unchanged parts omitted >>> If a resource ri' from the set (r0',r1',r2',…) meets the conditions below then the UE shall report pre-emption of the resource ri' to higher layers - ri' is not a member of SA, and - the RSRP measurement performed according to clause 8.4.2.1 for a received SCI format 1-A is higher than Th'prioRX,prioTX which overlaps with ri' according to step 6), and Th'prioRX,prioTX is the final threshold after executing steps 1)-7), i.e. includes all necessary increments for reaching X⋅Mtotal, and - the associated priority prioRX, satisfies one of the following conditionsri' - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX <<< Unchanged parts omitted >>>

Please indicate any further comments and suggestions regarding the above TP:

| Source | Comment |
| --- | --- |

- References

Contributions identified by FL to contain Mode-2 related issues:

- R1-2100137	Remaining open issues and corrections for mode 1 and mode 2 RA	OPPO

- R1-2100204	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2100334	Discussion and TPs on resource allocation in NR V2X	CATT, GOHIGH

- R1-2100411	Maintenance on resource allocation mechanisms for NR sidelink	vivo

- R1-2100515	Discussion on essential corrections in resource allocation for Mode 1 and 2	LG Electronics

- R1-2100630	Corrections to Mode-2 resource allocation	Intel Corporation

- R1-2100799	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2100938	The slot set for SL resource allocation procedure	ZTE, Sanechips

- R1-2100945	Remaining issues on resource allocation mode 2	NEC

- R1-2101073	Remaining issues on resource allocation mode 2 for NR V2X	ETRI

- R1-2101175	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

- R1-2101176	Maintenance for NR Sidelink Mode 2 Operation	Samsung

- R1-2101346	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2101437	Remaining Issues in Mode 2 Resource Allocation	Qualcomm Incorporated

- R1-2101533	Remaining issues on resource allocation for NR sidelink	Sharp

- R1-2101571	Remaining issues on sidelink mode 2	ASUSTeK

- R1-2101582	Maintenance for sidelink synchronization and mode 2	NTT DOCOMO, INC.

- R1-2101759	Remaining details for Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

Other Rel.16 NR V2X contributions

- R1-2100135	Draft TP on physical strucutre for NR sidelink	OPPO

- R1-2100136	Remaining open issues and corrections for physical layer procedure	OPPO

- R1-2100333	Discussion and TPs on sidelink synchronization mechanism and physical layer structure in NR V2X	CATT, GOHIGH

- R1-2100335	Discussion and TPs on physical layer procedures in NR V2X	CATT, GOHIGH

- R1-2100410	Maintenance on physical layer structure for NR sidelink	vivo

- R1-2100412	Maintenance on NR sidelink synchronization and procedures	vivo

- R1-2100514	Discussion on essential corrections in physical layer structure	LG Electronics

- R1-2100516	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2100629	Correction to FD-OCC for PSCCH	Intel Corporation

- R1-2100631	Corrections to sidelink procedures	Intel Corporation

- R1-2100734	A remaining issue on Mode-1 resource allocation for NR sidelink	Fujitsu

- R1-2100735	Remaining issues on physical layer procedures for NR sidelink	Fujitsu

- R1-2100800	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2100936	Remaining issues on sidelink synchronization	ZTE, Sanechips

- R1-2100937	Remaining issues on mode1	ZTE, Sanechips

- R1-2101174	Maintenance for NR Sidelink Physical Layer Structure	Samsung

- R1-2101344	Remaining Issues of Sidelink Physical Layer Procedures	Apple

- R1-2101345	Remaining Issue of Mode 1 Resource Allocation	Apple

- R1-2101436	Remaining Issues in Mode 1 Resource Allocation	Qualcomm Incorporated

- R1-2101438	Remaining Issues in Physical Layer Procedure	Qualcomm Incorporated

- R1-2101532	Remaining issues on physical layer structure and procedures for NR sidelink	Sharp

- R1-2101534	Remaining issues on synchronization mechanism for NR sidelink	Sharp

- R1-2101581	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

- R1-2101583	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2101649	Remaining issues on type-1 HARQ-ACK codebook considering multiple sidelink reosurce pools	ASUSTeK

- R1-2101650	Remaining issues on sidelink procedure	ASUSTeK

- R1-2101707	Draft_CR_TS38.212	Ericsson

- R1-2101708	Draft_CR_TS38.213	Ericsson

- R1-2101709	Draft_CR_TS38.306	Ericsson

- R1-2101732	Correction on PSBCH payload generation	Huawei, HiSilicon

- R1-2101733	Correction on determination of PSFCH resources based on a set of configured PRBs	Huawei, HiSilicon

- R1-2101760	Remaining details for Physical layer structure for sidelink	Nokia, Nokia Shanghai Bell