#3GPP TSG RAN WG1 #104-e						R1-210xxxx

e-Meeting, January 25th – February 5th, 2021

______________________________________________________________________Agenda item: 7.2.4

Source: Moderator (LG Electronics)

Title: Feature lead summary#1 for physical layer procedure aspects in AI 7.2.4

Document for: Discussion and decision

- Potential issues for email discussions

- Physical layer procedure

  - Thread 1

    - Issue #1

  - Thread 2

    - Issue #4

- Physical layer procedure

Issue #1: Remaining UL/SL prioritization rule

- Issue #1-1: How SL HARQ-ACK report is piggybacked on PUSCH

  - Aspects on priority index of the collided PUSCH [vivo,3] [LGE,5]

  - Aspects on non-overlapping PUCCH collided with a PUSCH [Apple,11] [DCM,14]

- Issue #1-2: Prioritization rule between PUSCH carrying SL HARQ-ACK reports and SL transmission [vivo,3] [LGE,5] [Fujitsu,7]

  - Option 1: No spec change is needed [LGE,5]

  - Option 2: SL transmission is prioritized when the SL transmission is prioritized over both UL-SCH and SL HARQ-ACK reporting [vivo,3] [Fujitsu,7]

- Issue #1-3: Further consideration on prioritization rule between PUCCH for the response of MsgB and SL TX [LGE,5]

- Issue #1-4: Further consideration on prioritization rule between SL reception and PUCCH carrying SL HARQ-ACK report [DCM,14]

- Issue #1-5: Further consideration on how to handle the case when a UE drops PSCCH/PSSCH retransmission due to lack of processing time [ASUSTeK,15]

Issue #2: Further clarification on SL power control

- Issue #2-1: Clarification on reference RS is used for pathloss derivation [vivo,4] [ASUSTeK,15]

- Issue #2-2: Whether or not to remove CBR part when DL pathloss is used [Spreadtrum,8]

Issue #3: Additional Prioritization rule between PSFCH TX and PSFCH RX or between PSFCH TX and PSFCH TX for tie-break [CATT,2]

- Not support: [Apple,11]

Issue #4: Interpretation of resource reservation period field in a SCI format 1-A [OPPO,1] [CATT,2] [vivo,3] [LGE,5] [Intel,6] [ZTE,9] [Samsung,10] [Apple,11] [Qulacomm,12] [Sharp,13] [DCM,14]

- Option 0: No additional agreement

- Option 1: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’ (following 8.1.7 of 38.214), [LGE,5] [Intel,6] [DCM,14]

  - P’ is counted in t'0SL, t'1SL, t'2SL,… (i.e. The number of slots in the resource pool between slot n and slot n+P’ is always the same as P’ (including slot n+P’ itself)).

- Option 1’: Option 1 + updated equation in 8.1.7 (N is the number of slots belonging to the resource pool) [Samsung,10] [Apple,11]

- Option 2: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’ (following 8.1.7 of 38.214), P’ is counted in t0SL,t1SL,t2SL,... (i.e. The number of slots in the resource pool between slot n and slot n+P’ can be less than P’).

  - Option 2-1: No further change [CATT,2]

  - Option 2-2: Add “UE expects tm+j×Prsvp_TX'SL belong to the set of slots assigned to the resource pool.” [CATT,2] [Intel,6] [Sharp,13]

  - Option 2-3: Add “If slot tm+j×Prsvp_TX'SL is not in the resource pool, the next slot in the resource pool should be used instead.” [OPPO,1] [vivo,3] [Intel,6]

- Option 3: If a UE transmits a SCI format 1-A in slot n in a resource pool, “Resource reservation period” P in the SCI format 1-A indicates the period in terms of ms. [Qulacomm,12]

  - If the physical slot after P ms is not in the resource pool, the next slot in the resource pool should be used instead.

- Option 4: If a UE transmits a SCI format 1-A in slot n in a resource pool, and if “Resource reservation period” in the SCI format 1-A indicates P’ (following 8.1.7 of 38.214), P’ is counted in the slots that can be used for SL transmission (i.e. the gap between the slots indicated in SCI within two periods is P). [ZTE,9]

  - Add “UE expects tm+j×Prsvp_TX'SL belong to the set of slots assigned to the resource pool.”

References

- R1-2100136	Remaining open issues and corrections for physical layer procedure	OPPO

- R1-2100335	Discussion and TPs on physical layer procedures in NR V2X	CATT, GOHIGH

- R1-2100410	Maintenance on physical layer structure for NR sidelink	vivo

- R1-2100412	Maintenance on NR sidelink synchronization and procedures	vivo

- R1-2100516	Discussion on essential corrections in physical layer procedure	LG Electronics

- R1-2100631	Corrections to sidelink procedures	Intel Corporation

- R1-2100735	Remaining issues on physical layer procedures for NR sidelink	Fujitsu

- R1-2100800	Remaining issues on sidelink physical layer procedure	Spreadtrum Communications

- R1-2100938	The slot set for SL resource allocation procedure	ZTE, Sanechips

- R1-2101175	Draft CR on Sidelink Physical Duration to Logical Slot Conversion	Samsung

- R1-2101344	Remaining Issues of Sidelink Physical Layer Procedures	Apple

- R1-2101438	Remaining Issues in Physical Layer Procedure	Qualcomm Incorporated

- R1-2101532	Remaining issues on physical layer structure and procedures for NR sidelink	Sharp

- R1-2101583	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2101650	Remaining issues on sidelink procedure	ASUSTeK

- R1-2101707	Draft_CR_TS38.212	Ericsson

- R1-2101708	Draft_CR_TS38.213	Ericsson

- R1-2101709	Draft_CR_TS38.306	Ericsson

- R1-2101733	Correction on determination of PSFCH resources based on a set of configured PRBs	Huawei, HiSilicon