Proposal 5’:

- When UE is configured to perform partial sensing, resource (re-)selection is triggered in slot n and resource reservation interval (Prsvp_TX) is NOT provided or Prsvp_TX=0 is set by higher layer,

  - Option 1: In slot n, UE performs random resource selection

    - For re-evaluation and pre-emption checking, the UE monitors slots after the random resource selection

      - FFS condition(s) in which re-evaluation and pre-emption checking can be performed

      - FFS details of the monitoring, including timing, duration and exceptions

  - Option 2: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA,n+TB] and performs resource selection based on sensing results.

    - FFS TA, TB and remaining details

    - For re-evaluation and pre-emption checking, the UE monitors additional slots

      - FFS condition(s) in which re-evaluation and pre-emption checking can be performed

      - FFS details of the additional monitoring, including timing, duration and exceptions

  - Other options are not precluded

  - FFS which one or multiple option(s) to be supported

    - If multiple options are supported, study the switching condition(s)

  - These options are in addition to random selection only without sensing or re-evaluation and pre-emption checking

Proposal 2’: If UE is configured to perform partial sensing and provided with a resource reservation interval (Prsvp_TX≠0) from higher layer in slot n, it is up to UE implementation to determine Y candidate slots within a resource selection window, where

- Down select to one:

  - Option 1: The resource selection window [n+T1,n+T2] is defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

  - Option 2: The resource selection window [n+T1, n+T2] is randomly selected by UE while satisfying:

    - T1 ≥ 0 and T2 ≤ remaining PDB

    - T2-T1 ≤ (pre-)configured threshold

- FFS whether A range of minimum Y values is (pre-)configured per priority level as in LTE-V

- FFS any restriction to determine Y candidate slots

- FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

Proposal 3’: If UE is configured to perform partial sensing and provided with a resource reservation interval (Prsvp_TX≠0) from higher layer, the UE monitors slots of a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to ty-k×PreserveSL if tySL is included in the set of Y candidate slots.

- Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one:

  - Option 1:  Preserve corresponds to all values from sl-ResourceReservePeriodList

  - Option 2:  Preserve corresponds to a subset of values from sl-ResourceReservePeriodList

    - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination, whether to include all values)

  - Option 3: Preserve is (pre-)configurable from values in sl-ResourceReservePeriodList

  - Option 3: Preserve is a common divisor among values in sl-ResourceReservePeriodList

- k equals to (down select to one)

  - Option 1: Only the most recent sensing occasion for a reservation period (k=1)

  - Option 2: The two most recent sensing occasions for a reservation period (k = [1, 2])

  - Option 3: All possible sensing occasions after n –T0

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured.

  - Option 5: k is (pre-)configured

  - Option 6: FFS others