3GPP TSG RAN WG1 #104-e														R1-200XXXX

e-Meeting, January 25th – February 5th, 2021

Agenda item:	8.12.3

Source: 			Moderator (BBC)

Title:	Summary # 2 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states

Document for:	Discussion and Decision

- Introduction

RAN1#104-e is the second meeting that discusses the AI 8.12.3 on Basic functions for broadcast/multicast for RRC_IDLE/ RRC_INACTIVE UEs. The information of the email thread on this topic and the check points on the discussion provided by RAN1 Chairman is shown below:

[104-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on Jan-28, Feb-02, Feb-05 – David (BBC)

A summary of the analysis and key issues identified from the technical inputs to this meeting to AI 8.12.3 can be found in R1-2101721 (Inbox).

Section 2 includes an initial proposal of High Priority issues for discussion at RAN1#104-e. Section 3 includes an initial proposal on Medium Priority issues for discussion at RAN1#104-e. Each Issue includes Initial FL proposals where companies are welcomed to provide their inputs.

Please use the “Navigation Pane” of Word to quickly find the proposals and the different rounds of discussions in this document.

Section 4 will include any agreements reached from the discussions.

- Discussion on High Priority Issues

Issue 1: MBS Common Frequency Resource: relation to the Initial BWP

Initial FL proposals for Issue 1

Proposal 1:  For RRC_IDLE/RRC_INACTIVE UEs, the defined/configured common frequency resource for group-common PDCCH/PDSCH contains the initial BWP and has the same SCS and CP as the initial BWP.

Proposal 2:  For RRC_IDLE/RRC_INACTIVE UEs, the initial BWP contains the defined/configured common frequency resource for group-common PDCCH/PDSCH.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | Fine with proposal 1 and proposal 2. |

Issue 2: Number of MBS Common Frequency Resources

Initial FL proposal for Issue 2

Proposal 3:  For RRC_IDLE/RRC_INACTIVE UEs, one common frequency resource for group-common PDCCH/PDSCH can be defined/configured.

- FFS: whether to define/configure more than one common frequency resources

- FFS: if more than one common frequency resource are configured, either the common frequency resource can be fully confined within the initial BWP and other configured common frequency resources, or the common frequency resource can be configured to contain the entire initial BWP and other common frequency resources.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | We are not sure about the motivation of configuring multiple common frequency resources, because the multiple broadcast services are common for all Idle/inactive UEs in the cell, gNB can configure a proper bandwidth of one common frequency resource to transmit multiple group-common PDCCHs/PDSCHs. |

Issue 3: Configuration/Definition of MBS Common Frequency Resources

Initial FL proposals for Issue 3

Proposal 4:  For RRC_IDLE/RRC_INACTIVE UEs, for the case that the common frequency resource for group-common PDCCH/PDSCH is larger than the Initial BWP (if supported), define a MBS specific BWP.

- FFS: details on start PRB, length PRB and reuse of SILV indication mechanisms.

Proposal 5:  For RRC_IDLE/RRC_INACTIVE UEs, for the case that the Initial BWP contains the common frequency resource for group-common PDCCH/PDSCH (if supported), configure a MBS frequency resource within the Initial BWP.

- FFS: details on start PRB, length PRB and reuse of SILV indication mechanisms.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | Not support proposal 4 and 5. We think a general definition for common frequency resource is enough, regardless the common frequency resource is larger or smaller than initial BWP, e.g., For RRC_IDLE/RRC_INACTIVE UEs, configure a MBS frequency resource larger than (if supported) or within (if supported) the Initial BWP for the case that the common frequency resource for group-common PDCCH/PDSCH is larger than the Initial BWP (if supported), define a MBS specific BWP. - FFS: details on start PRB, length PRB and reuse of SILV indication mechanisms. In addition, whether adopt option 2A (MBS specific BWP) or option 2B (MBS resource region) for RRC_CONNECTED UEs are still under discussion in AI 8.12.1, we can defer this issue after the process of RRC_CONNECTED UEs. |

Issue 4: CORESET configuration for group-common PDCCH/PDSCH

Initial FL proposals for Issue 4

Proposal 6:  For RRC_IDLE/RRC_INACTIVE UEs, network can configure the common CORESET configured by RRC signalling commonControlResourceSet for group-common PDCCH/PDSCH if the common frequency resource is the initial BWP and a CORESET is not configured.

Proposal 7:  For RRC_IDLE/RRC_INACTIVE UEs, multiple CORESETs can be configured for the defined/configured common frequency resource for group-common PDCCH/PDSCH.

- the same configured CORESET can be used to schedule MBS control information reception, broadcast, multicast and unicast.

- multiple CORESETs can be configured to independently schedule different MBS services and/or channels for control and traffic.

- FFS: definition of new RRC parameters to configure CORESET in a common frequency resource larger than Initial BWP (if supported).

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | Not support. We are not sure the motivation to support multiple CORESETs to differentiate multiple MBS services and/or channels for control and traffic. In addition, whether to support more than one CORESETs in addition to CORESET 0 is an optional capability of UE. For all IDLE/INACTIVE UEs in the cell, gNB can only configure one additional CORESET in addition to CORESET 0. Therefore, we think we can modify the proposal like this: Proposal 7: For RRC_IDLE/RRC_INACTIVE UEs, at most one multiple CORESETs can be configured for the defined/configured common frequency resource for group-common PDCCH/PDSCH in addition to CORESET 0. - the same configured CORESET can be used to schedule MBS control information reception, broadcast, multicast and unicast. - multiple CORESETs can be configured to independently schedule different MBS services and/or channels for control and traffic. - FFS: definition of new RRC parameters to configure CORESET in a common frequency resource larger than Initial BWP (if supported). |

Issue 5: Search Space (SS) for group-common PDCCH/PDSCH

Initial FL proposal for Issue 5

Proposal 8:  For RRC_IDLE/RRC_INACTIVE UEs, a new CSS type is defined for group-common PDCCH.

- FFS: monitoring priority with respect to existing CSS and USS.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | Support. |

Issue 6: Beam Sweeping for group-common PDCCH/PDSCH

Initial FL proposal for Issue 6

Proposal 9:  For RRC_IDLE/RRC_INACTIVE UEs, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB if configured.

- UE monitoring occasions are associated with a subset of the total SSB indexes in a timing window.

  - FFS: definition details of timing window such as periodicity and offset

  - FFS: association rules between SSB indexes and UE monitoring occasions.

- for broadcast reception, full beam sweeping is supported.

- FFS: (re)use of RRC_CONNECTED beam configuration for RRC_IDLE/RRC_INACTIVE UEs states.

- FFS: group-common PDCCH/PDSCH is QCL’d with TRS if configured.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | As RAN2 agreement, IDLE/INACTIVE UE only support broadcast sessions, can moderator clarify whether the first sub-bullet is also used for broadcast reception or only for multicast reception? For Rel-17, R2 specifies two modes: 1: One delivery mode for high QoS (reliability, latency) requirement, to be available in CONNECTED (possibly the UE can switch to other states when there is no data reception TBD) 2: One delivery mode for “low” QoS requirement, where the UE can also receive data in INACTIVE/IDLE (details TBD). R2 assumes (for R17) that delivery mode 1 is used only for multicast sessions. R2 assumes that delivery mode 2 is used for broadcast sessions. The applicability of delivery mode 2 to multicast sessions is FFS. |

Issue 7: HARQ feedback for RRC_IDLE/RRC_INACTIVE UE states

Initial FL proposal for Issue 7

Proposal 10:  For RRC_IDLE/RRC_INACTIVE UEs, study the potential support of HARQ feedback.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | We prefer not support HARQ feedback for RRC_IDLE/RRC_INACTIVE UEs. |

- Discussion on Medium Priority Issues

Issue 8: PDSCH repetition

Initial FL proposals for Issue 8

Proposal 11:  For RRC_IDLE/RRC_INACTIVE UEs, support slot-level repetition for group-common PDSCH.

- semi-static and dynamic slot-level repetition number configured by higher layer signalling.

- FFS: support of consecutive slot-level and RV-based time-interleaving for group-common PDSCH.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | We can defer this issue after the process of AI 8.12.2. |

Issue 9: PDSCH Semi Persistent Scheduling

Initial FL proposals for Issue 9

Proposal 12:  Support SPS group-common PDSCH for MBS for RRC_IDLE/RRC_INACTIVE UEs.

- FFS: whether to support more than one SPS group-common PDSCH configuration per UE

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | Support. |

Issue 10: MBS Common Frequency Resource: relation with Unicast BWP

Initial FL proposals for Issue 10

Proposal 13: for broadcast reception, the same group-common PDCCH and the corresponding scheduled group-common PDSCH can be received by both RRC_IDLE/RRC_INACTIVE UEs and RRC_CONNECTED UEs when UE-specific active BWP of RRC_CONNECTED UE contains the common frequency resource of RRC_IDLE/INACTIVE UEs.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | Support. |

Issue 11: Multicast reception by UEs in IDLE/INACTIVE states

Initial FL proposals for Issue 11

Proposal 14: For RRC_IDLE/RRC_INACTIVE UEs, Multicast reception with high QoS requirement (reliability, latency) is not supported for UEs in RRC_IDLE/RRC_INACTIVE states.

- FFS: multicast reception with low QoS requirement (reliability, latency) for For RRC_IDLE/RRC_INACTIVE UEs, subject to final RAN2 confirmation.

Please provide your company’s views and comments in the table below:

| company | comments |
| --- | --- |
| CMCC | We think this issue is up to RAN2’s decision, and is not necessary to discuss in RAN1. |

- Summary

References

- RP-201038	Revised Work Item on NR Multicast and Broadcast Services, Huawei, HiSilicon

- R1-2100108	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs, ZTE

- R1-2100146	Discussion on support for IDLE and INACTIVE state UEs, OPPO

- R1-2100191	Discussion on multicast support for IDLE/INACTIVE UEs, Huawei, HiSilicon

- R1-2100356	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, CATT, CBN

- R1-2100471	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE Ues, vivo

- R1-2100512	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues, Nokia, Nokia Shanghai Bell

- R1-2100615	Common frequency resource for NR PTM transmission, MediaTek Inc.

- R1-2100676	NR-MBS for RRC_IDLE/INACTIVE UEs, Intel Corporation

- R1-2100770	Basic functions for broadcast/multicast in idle/inactive states, Lenovo, Motorola Mobility

- R1-2100873	Considerations on MBS functions for RRC_IDLE UEs, Sony

- R1-2100908	Basic function for broadcast/multicast, LG Electronics

- R1-2101065	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states, CMCC

- R1-2101236	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, Samsung

- R1-2101361	Discussion on MBS for RRC_IDLE/RRC_INACTIVE  UEs, Apple

- R1-2101426	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs, Convida Wireless

- R1-2101489	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs, Qualcomm Incorporated

- R1-2101638	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs, CHENGDU TD TECH LTD.

- R1-2101728	Support for NR multicast reception in RRC Inactive/Idle, Ericsson

- R1-2009276	Discussion on broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs, Qualcomm Incorporated

- R1-2009307	Support for NR multicast reception in RRC Inactive/Idle, Ericsson