3GPP TSG RAN WG1 Meeting #104-E					      R1-210zzzz

e-Meeting, January 25th – February 5th, 2021

Source:	Moderator (Intel Corporation)

Title:	Feature Lead Summary #1 for NR Positioning UL-AoA Enhancements

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

In this document, we provide summary of contributions [1]-[19]submitted for the AI 8.5.2 on enhancements for UL-AOA positioning method as a part of the Rel.17 NR Positioning Enhancements work item. In addition, we provide initial set of proposals for RAN WG1 discussions/decisions at the RAN1#104E meeting.

# Summary of Proposed Enhancements

In this section, we provide summary of the major topics that have been identified based on review of the submitted contributions:

- Reporting and measurements enhancements for UL-AOA NR Positioning

  - [OPPO, [1]], [Huawei, [2]], [ZTE, [3]], [CATT, [4]], [vivo, [5]], [Futurewei, [6]], [InterDigital, [11]], [Sony, [12]], [CMCC, [13]], [Qualcomm, [17]], [DOCOMO, [18]], [Ericsson, [19]]

- Support of different antenna array configurations (e.g. ULA, different antenna spacing)

  - [Huawei, [2]], [vivo, [5]], [Futurewei, [6]], [Intel, [9]], [Ericsson, [19]]

- Calibration of gNB angle error and reference UE

  - [Huawei, [2]], [vivo, [5] - FFS], [CATT, [4], [BUPT,[7]]

- LOS/NLOS identification for UL-AOA measurements

  - [OPPO, [1]] (by implementation), [CATT, [4]], [Futurewei, [6]], [Intel, [9]]

- Assistance signaling for UL-AOA measurements (LMF assistance to gNB)

  - [CATT, [4]], [Nokia, [8]]

- SRS physical structure enhancements (sequence, RE mapping, multi-port SRS)

  - [LGE, [10]], [Fraunhofer, [14]]

- Power control enhancements of SRS for positioning

  - LGE, [10]], [Samsung, [15]]

- gNB/UE beamforming related aspects

  - [Nokia, [8]], [Samsung, [15]]

- SRS for positioning transmission priority

  - [InterDigital, [11]]

- Enhancements for UE timing advance

  - LGE, [10]]

- DL PRS-RSRP measurements enhancements

  - [Apple, [16]]

# Discussion on Proposed Enhancements

## Reporting and measurements enhancements

There are a lot of various proposals for gNB reporting and measurements enhancements as listed below:

- Study implementation based solution vs reporting of multiple AOA, associated UL-RTOA, associated SRS-RSRP [OPPO, [1]]

  - Motivation:

    - Improve the performance of UL AOA

- Per path UL AOA measurements/reporting [Huawei, [2]]

  - Motivation:

    - Positioning via reflections and spatial consistency checks

- Additional beam information reporting [ZTE, [3]] (e.g. antenna layout, beam width, beam vectors and corresponding gains etc.)

  - Motivation:

    - LMF can search for a right direction in an angle range, handling of sidelobes

- AOA & RTOA measurements associated to multi-paths to LMF for AOA positioning [CATT, [4]]

  - Motivation:

    - Discriminate AOA measurements corresponding to the LOS path from AOA measurements corresponding to the NLOS path

    - Use of advanced algorithms potentially to achieve a higher positioning accuracy

- Additional angle information of multiple paths [vivo, [5]] (RSRP, timing information per each angle)

  - Motivation:

    - Helps LMF to decide which angle is better for accurate positioning

- Detailed Tx/Rx beam information [Futurewei, [6]]

  - Motivation:

    - LMF to decide which angle is better for accurate positioning

- Reporting of UE orientation information from the UE [InterDigital, [11]

  - Motivation:

    - Network may be able to determine angle of transmission of SRSp depending on the UE orientation information and configured SRSp resource set and resource ID

- Uncertainty of the AOA and SRS Resource ID [Sony, [12]]

  - Motivation:

    - Helps to filter out some unreliable AOA measurements

    - Filter out undesired report from certain gNB

- Multiple UL AOA with associated ToA measurements [CMCC, [13]]

  - Motivation:

- Report multiple tuples (UL-AOA, UL-RSRP, RTOA/gNB Rx-Tx) within a single report [Qualcomm, [17]]

  - Motivation:

    - Helpful in advanced fusing algorithms

- Reporting multiple UL-AOAs w/ timing measurements [DOCOMO, [18]]

  - Motivation:

    - Single UL-AOA reporting may be insufficient to achieve a certain positioning accuracy for especially for NLOS

- Velocity of the UE should be reported to the network [Ericsson, [19]]

  - Motivation:

    - For estimation of AOA at TRPs

### Round – 1

Proposal 1-1

- NR supports combination of {UL-AOA, UL-RTOA or gNB Rx-Tx time difference, UL-RSRP} gNB/TRP measurements defined per channel path

  - FFS reporting details etc.

| Company Name | Comments |
| --- | --- |

## Support of different antenna array configurations

The following issues were raised for specific antenna configurations, e.g. Unilateral Linear Arrays (ULA):

- Ambiguity of angle measurements (e.g. due to different antenna spacings)

- Bias in angle measurements when gNB/TRP antenna array and UE are located at different heights

In order to address these issues, the following was proposed

- Firstly the use cases and scenarios for ULA antenna array should be discussed and confirmed [vivo, [5]]

- Reporting of multiple angle values per path or indication of the TRP antenna array orientation in the local coordinate system [Intel, [9]]

- Angle measurement defined with respect to the ULA antenna direction [Huawei, [2]]

- Enhanced AOA measurement with respect to the ULA antenna orientation [Futurewei, [6]]

- Reporting of the true AOA for ULA antenna configuration is supported by rel17 NR positioning enhancements [Ericsson, [19]]

### Round – 1

Proposal 2-1

- In case of UL-AOA measurements ambiguity,

  - gNB reports multiple values of UL-AOA measurements per path

- In case of ULA,

  - gNB reports UL-AOA measurement which is a function of the actual azimuth and zenith angles of arrival

- FFS details

| Company Name | Comments |
| --- | --- |

## Calibration of gNB angle error and reference UE

The following options were proposed to support calibration of gNB angle error measurements:

- Introduce a calibration/reference UE [Huawei, [2]], [vivo, [5] - FFS], [CATT, [4]

  - Motivation:

    - Use known location to mitigate the gNB angle error

- Support LMF assisted angle calibration [Huawei, [2]]

  - Motivation:

    - Calibrating the phase/amplitude error across antenna elements

- Correction of the fixed phase deviation between channels of gNB antenna [BUPT,[7]]

  - Motivation:

    - Accurate UL AOA measurements

### Round – 1

Proposal 3-1

- Further study the following two alternatives for gNB/TRP antenna array calibration to facilitate accurate UL-AOA measurements:

  - Alt.1: NR supports reference UE with known coordinates/orientation for UL-AOA measurement calibration

  - Alt.2: Procedure for calibration of UL-AOA measurements is up to gNB/TRP implementation

| Company Name | Comments |
| --- | --- |

## LOS/NLOS identification for UL-AOA measurements

The following proposals were discussed for with respect to LOS/NLOS identification to improve accuracy of UL-AOA positioning.

- AOA measurements associated with LOS/NLOS identifier and a confidence level [CATT, [4]]

  - Motivation:

    - Accuracy of AOA positioning

- LOS indicators as soft values to the network or LMF [Futurewei, [6]]

  - Motivation:

    - To improve positioning accuracy using regularization techniques

- NLOS link rejection by polarization [Futurewei, [6]]

  - Motivation:

    - Improve accuracy, identify NLOS direction (relative changes in their polarization orientation)

- Indication of link propagation type (LOS/NLOS) and reliability metric [Intel, [9]]

  - Motivation:

    - Improved UL-AOA positioning accuracy

### Round – 1

Proposal 4-1

- gNB/TRP reports estimate of LOS/NLOS link type identification to facilitate accurate UL-AOA positioning

  - FFS details for LOS/NLOS link type identification and signaling

| Company Name | Comments |
| --- | --- |

## Assistance signaling for UL-AOA measurements

The following options were proposed to assist UL-AOA measurements:

- Indication of expected AOA parameters [Nokia, [8]]

  - Motivation: Assist non-serving cell TRPs in UL-AOA measurements

- Indication of estimated UE position and the uncertainty [CATT, [4]]

  - Motivation: Aid the UE/gNB in the reception of the DL/UL reference signals and providing reliable NR timing and angular (especially AOA) measurements

### Round – 1

Proposal 5-1

- NR supports assistance signaling from LMF to gNB/TRP to facilitate UL-AOA measurements

  - FFS details of LMF assistance signaling

| Company Name | Comments |
| --- | --- |

## SRS physical structure enhancements

The following options were proposed to enhance SRS physical structure:

- Long ZC sequence (Length = existing sequence length * comb size) [LGE, [10]]

  - Motivation:

    - More cyclic shifts and root indexes can be used => cross-correlation can be improved

- Enhanced SRS sequence mapping rule [LGE, [10]]

  - Motivation:

    - Adjustment of phase are not necessary

- Multi-port SRS for positioning [Fraunhofer, [14]]

  - Motivation:

    - Reduced time-frequency resource overhead. TRP may take advantage of the multipath components received from multiple SRS resources coherently transmitted

### Round – 1

The proposed above enhancement seems to be a general one i.e. applicable to all UL NR positioning measurements.

Proposal 6-1

- FFS physical structure enhancements of SRS for positioning

| Company Name | Comments |
| --- | --- |

## Power control enhancements of SRS for positioning

The following open-loop power control enhancements were proposed

- Multiple DL RS resources as path-loss reference to each SRS resource set. [LGE, [10]]

- Association between the multiple path-loss reference RSs and SRS resource within a SRS resource set. [LGE, [10]]

  - Motivation:

    - Improve reception at neighbour gNBs/TRPs

- Power control enhancements [Samsung, [15]]

  - Motivation:

    - Reliable reception at neighbour TRPs/gNBs that are far from UE

### Round – 1

The proposed above enhancements seem to be general ones, i.e. applicable to all UL NR positioning measurements.

Proposal 7-1

- FFS association of pathloss reference RSs and SRS resources for positioning within an SRS resource set

- FFS power control enhancements for SRS for positioning

| Company Name | Comments |
| --- | --- |

## gNB/UE beamforming related aspects

The following beamforming related techniques were proposed for study to improve UL-AOA performance:

- Differential beamforming technique for UL-AOA positioning method [Samsung, [15]]

  - Motivation:

    - Accuracy enhancements, latency reduction, overhead reduction

- Beam interpolation based AOA estimation [Nokia, [8]]

  - Motivation:

    - Accurate measurement of UL-RSRP

### Round – 1

Proposal 8-1

- Further study performance of differential beamforming and beam interpolation-based AOA estimation for UL-AOA positioning and particular design aspects for specification work

| Company Name | Comments |
| --- | --- |

## SRS for positioning transmission priority

The SRS for positioning prioritization was discussed:

- Prioritization of SRS for positioning with respect to other signals and channels [InterDigital, [11]]

  - Motivation:

    - SRS for positioning transmission may be dropped when the UE has another data for UL transmission => degradation of low latency positioning services

### Round – 1

The proposed above enhancement seems to be a general one i.e. applicable to all UL NR positioning measurements.

Proposal 9-1

- Further study prioritization of SRS for positioning with respect to other signals and channels

| Company Name | Comments |
| --- | --- |

## Enhancements for UE timing advance

The following enhancements were proposed with respect to the SRS for positioning transmission timing:

- SRS resource-specific TA configuration [LGE, [10]]

  - Motivation:

    - For transmission intended to a neighbour cell, TA of the SRS resource should also be intended to the target neighbour cell.

### Round – 1

The proposed enhancements were discussed in Rel.16 and there was no consensus to introduce such functionality for NR SRS for positioning.

Proposal 10-1

- FFS support of SRS resource-specific TA configuration

| Company Name | Comments |
| --- | --- |

## DL PRS-RSRP measurements enhancements

- Association of DL-PRS-RSRP measurements with a time window, determined by UE [Apple, [16]]

  - Motivation:

    - PRS for DL-AoD, is not tied with the first detected path in time. Report may be often biased toward the NLOS path

  - NOTE: Proposal relates to DL-AoD enhancement and thus should be covered under another AI 8.5.3

### Round – 1

Proposal 11-1

- Discuss proposal under AI 8.5.3 unless the same approach is suggested for UL-AOA by proponent

| Company Name | Comments |
| --- | --- |

Conclusions

In this contribution, we have provided review of the submitted contributions for NR Positioning UL-AOA enhancements and prepared initial set of proposals to facilitate further discussion/decision.

References

- R1-2100129	Enhancements for UL AOA Positioning	OPPO

- R1-2100237	Enhancement for UL AOA positioning	Huawei, HiSilicon

- R1-2100294	Accuracy improvement for UL-AOA positioning solutions	ZTE

- R1-2100386	Discussion on accuracy improvements for UL-AOA positioning solutions	CATT

- R1-2100446	Discussion on potential enhancements for UL-AOA method	vivo

- R1-2100488	Discussion on improving the accuracy of UL AOA positioning solutions	FUTUREWEI

- R1-2100497	Accuracy improvements for UL-AOA positioning solutions	BUPT

- R1-2100549	Initial views on enhancing UL AOA	Nokia, Nokia Shanghai Bell

- R1-2100658	NR positioning enhancements for UL-AOA method	Intel Corporation

- R1-2100709	Discussion on accuracy improvement for UL-AOA positioning	LG Electronics

- R1-2100753	Discussions on techniques to improve accuracy for UL-AOA positioning solutions	InterDigital, Inc.

- R1-2100863	Discussion on accuracy improvements for UL-AOA positioning method	Sony

- R1-2101047	Discussion on UL-AOA enhancement	CMCC

- R1-2101132	UL-AOA positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

- R1-2101211	Discussion on accuracy improvements for UL-AOA positioning solutions	Samsung

- R1-2101388	Accuracy enhancements for DL-AoD positioning technique	Apple

- R1-2101469	Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

- R1-2101617	Discussion on UL-AOA positioning enhancements	NTT DOCOMO, INC.

- R1-2101755	Enhancements of UL-AOA positioning solutions	Ericsson