3GPP TSG RAN WG1 #104-bis-e	R1- 2102829

e-Meeting, April 12th – April 20th, 2021

Agenda item:      8.14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications

Document for:     Discussion

# Introduction

RAN#91 approved a revised SID on XR Evaluations for NR [1]. The XR SI is now ongoing with focus on related system-level performance evaluation to assess how well the currently standardized NR system can support such services. In this document, we present our preliminary results of XR and CG services in 5G NR. Two deployment scenarios are considered for evaluating the performance of XR and CG services, as agreed in [2]: (1) Indoor Hotspot (InH) and (2) Dense Urban (UMa). In these two deployments, we evaluate the DL system capacity of Cloud Gaming, Virtual Reality/Augmented Reality applications according to the DL traffic models discussed in [3].

# Statistical Models and KPI for XR Traffic

In this section, we discuss the parameters we used in our performance evaluation for the statistical model for XR traffic. Parameters have been set according to discussion in [2] and [4]. In particular, Table 1 and Table 2 illustrate the parameters for generating DL traffic (i.e., video traffic), whereas Table 3 shows the parameters for generating UL traffic (i.e., pose/control information).

Table 1 – Parameters of frame size distribution for DL traffic model computed as a function of data rate.

| Parameter | Proposed value |
| --- | --- |
| Frame Rate | 60 fps |
| Bitrate | 30 Mbps |
| Frame size distribution | Truncated Gaussian |
| Frame size (avg) | 62500 Bytes |
| Frame size (std) | 9375 Bytes |
| Frame size (max) | 93750 Bytes |
| Frame size (min) | 31250 Bytes |
| Transport protocol | UDP |
| Maximum transport packet size | 93750 Bytes 1 frame = 1 PDU |

Table 2 – Parameters of the interarrival time distribution for DL traffic model computed as a function of frame rate and jitter.

| Parameter | Proposed value |
| --- | --- |
| Frame Rate | 60 fps |
| Inter-arrival time distribution | Truncated Gaussian |
| Interarrival time (avg) | 16.67 ms |
| Interarrival time (std) | 2 ms |
| Interarrival time (max) | 20.67 ms |
| Interarrival time (min) | 12.67 ms |

Table 3 – Parameters of the UL traffic model (Pose/Control traffic model).

| Parameter | Proposed value |
| --- | --- |
| Packet size | 100 bytes |
| Inter-arrival time distribution | Constant |
| Inter-arrival time | 4 ms |

As agreed in [2,4], the system capacity is measured in terms on “number of satisfied UEs”. More specifically, a UE is marked as “satisfied UE” if more than X% of packets are successfully transmitted over the air interface within the PDB. The system capacity is defined as the maximum number of UEs per cell with at least Y% of UEs marked as satisfied UEs. In our evaluation, we adopt X={95%, 99%} as percentage of frames to be successfully received within the PDB to identify satisfied UEs, while Y=90% as percentage of satisfied UEs to measure the system capacity.

# Performance Evaluation of Indoor Hotspot

## Simulation settings

The carrier frequency is set to 4 GHz and 30GHz for FR1 and FR2, respectively. System bandwidth is assumed to be 100MHz for FR1 and for FR2 to compare achievable system capacity when propagation and antenna configuration change; additionally, 200MHz in FR2 is also analyzed to observe the change in network performance when doubling the system bandwidth. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 kHz and 120kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs is set as follows: 31dBm with 100MHz (24dBm per 20MHz) in FR1, and 24dBm with 100MHz and 27dBm with 200MHz (23dBm per 80MHz) in FR2. Table 4 lists the main parameters of the Indoor Hotspot deployment that are considered in this study.

Table 4 – Main parameters for Indoor Hotspot (InH) deployment

| Parameter | Value |  |  |
| --- | --- | --- | --- |
| Layout | - 120m x 50m, Single layer (indoor floor, open office) - 12 cells/TRPs - ISD: 20m |  |  |
| Channel model | InH |  |  |
| Carrier frequency | FR1: 4 GHz |  | FR2: 30 GHz |
| Subcarrier spacing | FR1: 30 kHz |  | FR2: 120 kHz |
| System bandwidth | FR1: 100 MHz |  | FR2: 100 MHz 200 MHz |
| BS height | 3 m |  |  |
| UE height | 1.5 m |  |  |
| BS noise figure | FR1: 5 dB |  | FR2: 7 dB |
| UE noise figure | FR1: 9 dB |  |  |
| BS receiver | LMMSE-IRC |  |  |
| UE receiver | LMMSE-IRC |  |  |
| Channel estimation | Ideal |  |  |
| UE speed | 3 km/h |  |  |
| MCS | Up to 256QAM |  |  |
| BS Tx power | FR1: 31dBm (24dBm per 20MHz) |  | FR2 (23dBm per 80MHz): - 24dBm for 100 MHz - 27 dBm for 200MHz |
| UE Tx max power | FR1: 23 dBm |  |  |
| TDD Frame structure | Option 1: DDDSU |  |  |
| Cell Selection | RSRP Slow Fading |  |  |
| BS antenna configuration | - Pattern: Ceiling-mount antenna radiation pattern - Gain: 5 dBi - Downtilt: 90° |  |  |
|  | Configuration in FR1: 32 TxRU (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) (dH, dV) = (0.5, 0.5)λ | Configuration in FR2: 2 TxRU (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1) (dH, dV) = (0.5, 0.5) Grid of Beams: - Azimuth angles (degrees): {90, 90, 90, 112.5, 112.5, 112.5, 67.5, 67.5, 67.5, 140, 140, 140, 40, 40, 40} - Elevation angles (degrees): {-30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30, -30, 0, 30} |  |
| UE antenna configuration | - Pattern : Omni-directional, - Gain : 0 dBi, - Configuration : 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, 0)λ |  |  |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |  |
| Scheduler | SU-MIMO, proportional fairness in time and frequency domain |  |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |  |
| PHY processing delay | PDSCH decoding: 4OFDM symbols |  |  |
| PDCCH overhead | Modelled |  |  |
| Target BLER | 10% for first transmission |  |  |
| Max HARQ transmission | 3 |  |  |
| HARQ scheme | Chase combining |  |  |

## Simulation results

In this section, we illustrate the results obtained with our system level simulator in the Indoor Hotspot deployment scenario. We illustrate the system capacity when UEs run either CG (Section 3.2.1) or VR/AR (Section 3.2.2) applications at 30Mbps. UEs are randomly dropped in the area according to a spatial uniform distribution, therefore, in each simulation run some cells get a higher number of connected UEs than other cells; this might affect the system capacity when compared to a deployment where all cells serve the exact same number of connected UEs. For a reference comparison, the reader can refer to sections 4.2.1 and 4.2.2 where such analysis is presented for the Dense Urban deployment. A similar trend is expected for the Indoor Hotspot deployment.

### Cloud Gaming: DL Capacity

Figure 1 shows CG results at 30Mbps, picturing the ratio of satisfied UEs computed with 95% and 99% of frames received within the PDB of 15 ms as a function of the average cell load, which is measured in number of UEs. Results correspond to a system bandwidth of 100MHz for both FR1 and FR2. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we conclude that in the Indoor Hotspot deployment in FR1 we can support 24 UEs running CG application at 30Mbps (i.e., 2 UEs per cell on average) for both considered values of X.  In FR2, we observe a significant improvement on network performance, supporting up to 60 UEs (i.e., 5 UEs per cell on average) assuming for both values of X. The higher capacity for FR2 is mainly due to the use of the more sophisticated gNB GoB configuration. When increasing the system bandwidth to 200 MHz in FR2, we can observe the corresponding increase in capacity in Figure 2. In this case we can support more than 120 UEs (i.e., more than 10 UEs per cell on average) running CG applications at 30 Mbps for both considered values of X.

| (a) CG in FR1 with X=95%. | (b) CG in FR1 with X=99%. |
| --- | --- |
| (c) CG in FR2 with X=95%. | (d) CG in FR2 with X=99%. |

Figure 1 – CG capacity evaluation of InH traffic in FR1 and FR2 with X={95%, 99%} of frames received within PDB and 100MHz system bandwidth. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

| (a) CG in FR2 with X=95%. | (b) CG in FR2 with X=99%. |
| --- | --- |

Figure 2 – CG capacity evaluation of InH traffic in FR2 with X={95%, 99%} of frames received within PDB and 200MHz system bandwidth. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell indicated in Table 5.

Table 5 – System capacity for CG (PDB=15ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | X=95% | X=99% |
| --- | --- | --- |
| FR1 (100 MHz) | 2 | 2 |
| FR2 (100 MHz) | 5 | 5 |
| FR2 (200 MHz) | >10 | >10 |

### Augmented/Virtual Reality: DL Capacity

In this section, we illustrate the results on the system capacity obtained for VR/AR applications, namely with PDB=10ms. Figure 3 shows the ratio of satisfied UEs computed with 95% and 99% of frames received within the PDB as a function of the average cell load, which is measured in number of UEs; results correspond to a system bandwidth of 100MHz for both FR1 and FR2. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Indoor Hotspot deployment in FR1 we can support 24 UEs running VR/AR applications at 30Mbps (i.e., 2 UEs per cell on average) for both considered values of X.  In FR2, we observe a significant improvement on network performance, supporting up to 60 UEs (i.e., 5 UEs per cell on average) assuming X=95%, and 48 UEs (i.e., 4 UEs per cell on average) when X=99%.

| (a) VR/AR in FR1 with X=95%. | (b) VR/AR in FR1 with X=99%. |
| --- | --- |
| (c) VR/AR in FR2 with X=95%. | (d) VR/AR in FR2 with X=99%. |

Figure 3 – VR/AR capacity evaluation of InH traffic in FR1 and FR2 with X={95%, 99%} of frames received within PDB and 100MHz system bandwidth. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

When increasing the system bandwidth to 200 MHz in FR2, we can observe the corresponding increase in capacity in Figure 4. In this case we can support more than 120 UEs (i.e., more than 10 UEs per cell on average) running VR/AR applications at 30 Mbps for both considered values of X.

| (a) VR/AR in FR2 with X=95%. | (b) VR/AR in FR2 with X=99%. |
| --- | --- |

Figure 4 – VR/AR capacity evaluation of InH traffic in FR2 with X={95%, 99%} of frames received within PDB and 200MHz system bandwidth. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 2: When we consider VR/AR traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell indicated in Table 6.

Table 6 – System capacity for VR/AR (PDB=10ms) at 30Mbps [#UEs/cell] in an InH deployment

|  | X=95% | X=99% |
| --- | --- | --- |
| FR1 (100 MHz) | 2 | 2 |
| FR2 (100 MHz) | 5 | 4 |
| FR2 (200 MHz) | >10 | 10 |

# Performance Evaluation in Dense Urban

## Simulation settings

The carrier frequency is set to 4 GHz and 30GHz for FR1 and FR2, respectively. System bandwidth has been fixed to 100MHz for both FR1 and FR2 to compare achievable system capacity when propagation and antenna configuration change. Time division duplexing (TDD) is configured according to the first option, thus using “DDDSU” as radio frame. The smallest schedulable radio resource is the physical radio blocks (PRBs) of 12 subcarriers, each is of 30 KHz and 120kHz for FR1 and FR2, respectively. The TTI size is set to 14 OFDM symbols, with one control symbol, always placed at the start of each TTI. The asynchronous HARQ Chase combing is adopted with maximum 3 HARQ retransmission before a packet is dropped (i.e., marked with an infinite radio latency). The transmit power of gNBs and the UE uplink power control settings are adjusted for Dense Urban, i.e., 50 dBm (i.e., 43dBM for 20MHz) for gNB transmit power and P0= -106 dBm and α= 1  for UE open loop power control. Table 7 lists the main parameters of the Dense Urban deployment that are considered in this study.

Table 7– Main parameters for Dense Urban (UMa) deployment

| Parameter | Value |  |
| --- | --- | --- |
| Layout | 21 cells with wraparound (ISD: 200m) |  |
| Channel model | UMa |  |
| Carrier frequency | FR1: 4 GHz | FR2: 30 GHz |
| Subcarrier spacing | FR1: 30 kHz | FR2: 120 kHz |
| System bandwidth | FR1: Option 1: 100 MHz | FR2: Option 1: 100 MHz |
| BS height | 25m |  |
| UE height | hUT = 3(nfl – 1) + 1.5 |  |
|  | Outdoor: nfl = 1 | Indoor: - nfl ~ uniform(1,Nfl) - Nfl ~ uniform(4,8) |
| BS noise figure | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Ideal |  |
| UE speed | 3 km/h |  |
| MCS | Up to 256QAM |  |
| BS Tx power | 44 dBm per 20 MHz 51 dBm per 100 MHz |  |
| UE Tx max power | FR1: 23 dBm |  |
| TDD Frame structure | Option 1: DDDSU |  |
| Mechanical Downtilt | Baseline: 12 degrees |  |
| Electrical Downtilt | 6 degrees |  |
| Cell Selection | RSRP Slow Fading |  |
| BS antenna configuration | FR1: 32TxRUs (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: 2TxRUs (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1), (dH, dV) = (0.5λ, 0.5λ) Grid of Beams - Azimuth angles: {33.75, 56.25, 78.75, 101.25, 123.75, 146.25, 33.75, 56.25, 78.75, 101.25, 123.75, 146.25} degrees - Elevation angles: {-22.5, -22.5, -22.5, -22.5, -22.5, -22.5, -67.5, -67.5, -67.5, -67.5, -67.5, -67.5} degrees |
| UE antenna configuration | FR1: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5λ, 0.5λ) | FR2: (M, N, P)=(1, 4, 2), 3 panels (left, right, top) (Mp, Np)=(1, 1) |
| Power control parameter | Open loop, Alpha = 1, P0 = -106 dBm |  |
| Scheduler | SU-MIMO, proportional fairness in time and frequency domain |  |
| CSI acquisition | Periodic CQI on 2 ms period |  |
| PHY processing delay | PDSCH decoding: 4OFDM symbols |  |
| PDCCH overhead | Modelled |  |
| Target BLER | 10% for first transmission |  |
| Max HARQ transmission | 3 |  |
| HARQ scheme | Chase combining |  |

## Simulation results

In the following, we illustrate the results obtained with our system level simulator in Dense Urban deployment scenario presented in the previous section. We illustrate the system capacity when UEs run either CG (Section 4.2.1) or VR/AR (Section 4.2.2) applications. Two random placements of UEs in the simulation area have been used as follows:

- Uneven load: All UEs are randomly dropped in the full simulation area according to a spatial uniform distribution. This essentially means that each cell will be serving different number of UEs, hence we call this Uneven load.

- Even load: UEs are randomly dropped in the simulated area one by one according to a spatial uniform distribution. This process is repeated for all UEs until each cell served exactly the same number of UEs. Note that cell selection for a UE is based on RSRP.

In the case of uneven load, some cell gets a higher number of connected UEs than other cells, whereas in the even load all cells get the same number of connected UEs. This affects the system capacity as we show hereafter.

### Cloud Gaming: DL Capacity

In this section, we illustrate the results on the system capacity obtained for CG application at 30Mbps, with PDB=15ms. Figure 5 shows the ratio of satisfied UEs when cells are unevenly loaded. The ratio of satisfied UEs has been computed with 95% and 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Dense Urban deployment in FR1 we can support 63 UEs running CG application at 30Mbps when traffic is not evenly distributed across cells (3 UE per cell). In FR2, we can support 63 (3 UE per cell) CG UEs at 30Mbps with UEs unevenly distributed across cells depending if we assume that a UE is fully satisfied when it receives 95% of frames within the PDB. In contrast, if we consider a percentage of frames received within the PDB equal to 99%, we can fully support 24 UEs (2 UE per cell) unevenly distributed across cells in FR2.

| (a) CG in FR1 with X=95%. | (b) CG in FR1 with X=99%. |
| --- | --- |
| (c) CG in FR2 with X=95%. | (d) CG in FR2 with X=99%. |

Figure 5 – Capacity evaluation of CG traffic in FR1 and FR2 with X={95%, 99%} of frames received within PDB and cells unevenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Figure 6 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 95% and 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that when UEs is evenly distributed across cells we can support more CG services since traffic load is balanced across cells. In all cases, as illustrated in the Figures we gain 1 or 2 UEs per cell.

| (a) CG in FR1 with X=95%. | (b) CG in FR1 with X=99%. |
| --- | --- |
| (c) CG in FR2 with X=95%. | (d) CG in FR2 with X=99%. |

Figure 6 – Capacity evaluation of CG traffic in FR1 and FR2 with X={95%, 99%} of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 3: When we consider CG traffic model at 30Mbps for a Dense Urban deployment, we can support a number of UEs per cell indicated in Table 8.

Table 8 – System capacity for CG (PDB=15ms) at 30Mbps [#UEs/cell]

|  | X=95% |  | X=99% |  |
| --- | --- | --- | --- | --- |
|  | Uneven Load | Even Load | Uneven Load | Even Load |
| FR1 | 3 | 5 | 3 | 4 |
| FR2 | 3 | 5 | 2 | 4 |

Observation 4: The method used for the random placement of UEs in the simulation area affects the system capacity. The even load random placement results in higher system capacity than uneven load placement by up to 2 UE/cell in Dense Urban deployments for CG applications at 30Mbps.

### Augmented/Virtual Reality: DL Capacity

In this section, we illustrate the results on the DL system capacity obtained for CG application, namely with PDB=10ms. Two placements of UEs in the simulation area have been used that results in cells unevenly and evenly loaded. In the case of uneven load, some cell gets a higher number of connected UEs than other cells, whereas in the even load all cells get the same number of connected UEs.

Figure 7 shows the ratio of satisfied UEs when cells are unevenly loaded. The ratio of satisfied UEs has been computed with 95% and 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. According to the 90% limit, we can conclude that in the Dense Urban deployment we can support 24 UEs running VR/AR application at 30Mbps both in FR1 and FR2 when traffic is not evenly distributed across cells.

| (a) AR/VR in FR1 with X=95%. | (b) AR/VR in FR1 with X=99%. |
| --- | --- |
| (c) AR/VR in FR2 with X=95%. | (d) AR/VR in FR2 with X=99%. |

Figure 7 – Capacity evaluation of AR/VR traffic in FR1 and FR2 with X={95%, 99%} of frames received within PDB and cells unevenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Figure 8 shows the ratio of satisfied UEs when cells are evenly loaded. The ratio of satisfied UEs has been computed with 95% and 99% of frames received within the PDB both in FR1 and FR2 configurations as a function of the average cell load, which is measured in number of UEs. The dashed black line indicates the 90% limit on the number of satisfied UEs to measure the system capacity. We can observe that when UEs is evenly distributed across cells we can support more CG services since traffic load is balanced across cells. In all cases, as illustrated in the Figures we gain 1 UEs per cell with respect to the uneven loaded case. More specifically, we can support 3 UEs per cell running AR/VR applications.

| (a) CG in FR1 with X=95%. | (b) CG in FR1 with X=99%. |
| --- | --- |

| (c) CG in FR2 with X=95%. | (d) CG in FR2 with X=99%. |
| --- | --- |

Figure 8 – Capacity evaluation of AR/VR traffic in FR1 and FR2 with X={95%, 99%} of frames received within PDB and cells evenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 5: When we consider AR/VR traffic model at 30Mbps for a Dense Urban deployment, we can support a number of UEs per cell indicated in Table 9.

Table 9 – System capacity for AR/VR (PDB=15ms) at 30Mbps [#UEs/cell]

|  | X=95% |  | X=99% |  |
| --- | --- | --- | --- | --- |
|  | Uneven Load | Even Load | Uneven Load | Even Load |
| FR1 | 2 | 3 | 2 | 3 |
| FR2 | 2 | 3 | 2 | 2-3 |

Observation 6: The method used for the random placement of UEs in the simulation area affects the system capacity. The even load random placement results in higher system capacity than uneven load placement by 1 UE/cell in Dense Urban deployments for AR/VR applications at 30Mbps.

### Pose/Control: UL Capacity

In this section, we illustrate the results on the UL system capacity obtained considering the Pose/Control traffic model used to capture the pose/control information periodically transmitted in UL by UEs hosting a CG or VR/AR application. We show here only the results with uneven load (worst case of traffic distribution) in FR1. Figure 9 clearly shows that 5G NR can serve up to 5 UEs per cell when we consider X={95%, 99%} and Y=90%. Therefore, the bottleneck to the system capacity is represented by DL for CG and AR/CR applications when only pose/control information is transmitted in UL.

| (a) Pose/control in FR1 with X=95%. | (b) Pose/control in FR1 with X=99%. |
| --- | --- |

Figure 9 – Capacity evaluation (UL) of Pose/Control traffic in FR1 and FR2 with X={95%, 99%} of packets received within PDB and cells unevenly loaded. The lower bound on the percentage of satisfied user is Y=90% (dashed line).

Observation 7: When we consider Pose/Control model for UL traffic with packet size equal to 100 bytes and constant inter-arrival time equal to 4ms, we can support at least 5 UEs per cell in UL. Further optimization of UL transmit power control settings is expected to result in additional improvements.

# Summary

In this contribution we have discussed performance results in indoor and outdoor deployments and bottlenecks that could be improved to increase 5G NR system capacity. Those naturally require further studies and discussions. In summary, following observations have been made:

Observation 1: When we consider CG traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell indicated in Table 5.

Observation 2: When we consider VR/AR traffic model at 30Mbps for an InH deployment, we can support the number of UEs per cell indicated in Table 6.

Observation 3: When we consider CG traffic model at 30Mbps for a Dense Urban deployment, we can support a number of UEs per cell indicated in Table 8.

Observation 4: The method used for the random placement of UEs in the simulation area affects the system capacity. The even load random placement results in higher system capacity than uneven load placement by up to 2 UE/cell in Dense Urban deployments for CG applications at 30Mbps.

Observation 5: When we consider AR/VR traffic model at 30Mbps for a Dense Urban deployment, we can support a number of UEs per cell indicated in Table 9.

Observation 6: The method used for the random placement of UEs in the simulation area affects the system capacity. The even load random placement results in higher system capacity than uneven load placement by 1 UE/cell in Dense Urban deployments for AR/VR applications at 30Mbps.

Observation 7: When we consider Pose/Control model for UL traffic with packet size equal to 100 bytes and constant inter-arrival time equal to 4ms, we can support at least 5 UEs per cell in UL. Further optimization of UL transmit power control settings is expected to result in additional improvements.

All previous observations are captured and summarized by the following tables:

|  | InH deployment |  |  |  |
| --- | --- | --- | --- | --- |
|  | CG (30 Mbps) |  | VR/AR (30 Mbps) |  |
|  | X=95% | X=99% | X=95% | X=99% |
| FR1 (100 MHz) | 2 | 2 | 2 | 2 |
| FR2 (100 MHz) | 5 | 5 | 5 | 4 |
| FR2 (200 MHz) | >10 | >10 | >10 | 10 |

|  | Dense Urban deployment |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CG (30 Mbps) |  |  |  | VR/AR (30 Mbps) |  |  |  |
|  | X=95% |  | X=99% |  | X=95% |  | X=99% |  |
|  | Uneven Load | Even Load | Uneven Load | Even Load | Uneven Load | Even Load | Uneven Load | Even Load |
| FR1 | 3 | 5 | 3 | 4 | 2 | 3 | 2 | 3 |
| FR2 | 3 | 5 | 2 | 4 | 2 | 3 | 2 | 2-3 |

# References

- RP-210460 Revised SID on XR Evaluations for NR, RAN#91

- R1-2009812 FL Summary of RAN1 103-e agreements and email discussions on Rel-17 SI on XR evaluations for NR

- RAN1 Chairman’s Notes (v018) 3GPP TSG RAN WG1 Meeting #104-e e-Meeting, January 25th – February 5th, 2021

- R1-2101866 Summary of [104-e-NR-XR-02]_Other Eval Methodology.