3GPP TSG RAN WG1 #104b-e																				R1-2102986

e-Meeting, April 12th – 20th, 2021

Agenda item:	8.4.2

Source:		Xiaomi

Title:	Discussion on UL time and frequency synchronization for NTN

Document for:	Discussion

Introduction

The work item on solutions for NR to support non-terrestrial networks (NTN) was approved at RAN#86 and the work item description (WID) was updated in [1]. One objective is to specify uplink (UL) time and frequency synchronization enhancements for NTN. The study item phase identified issues and made recommendations on NR UL synchronization in TR 38.821 [2]. In RAN1#104-e meeting, the following agreements were achieved for UL time and frequency synchronization for NTN [3].

Agreements:

An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

FFS: Operation of closed loop and open loop TA control

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

FFS: Details of the combination of open and closed loop TA control

Conclusion:

It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation

Agreement:

RAN1 should send an LS to RAN4 with the following questions:

Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

Conclusion:

If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary.

- FFS: support of DL frequency compensation for the service link Doppler.

Agreement:

- RAN1 to support satellite ephemeris broadcast based at least on one of the following format options:

  - Option 1: Ephemeris format based on satellite position and velocity state vectors

    - FFS: Details on state vectors formats

    - FFS: Details on time reference provisioning/format

  - Option 2: Ephemeris format based on orbital elements

    - FFS: Details on orbital elements formats

    - FFS: Details on time reference provisioning/format

- FFS: Whether down-selection is needed or both options are supported

UL timing synchronization

The following content was discussed in the last RAN1 meeting [4].

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Where:

  NTA and NTA,offset are defined as in Release-16.

  NTA,UE-specific  is UE self-estimated TA

  NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

   FFS:  Signaling and the granularity of NTA,common

   FFS: When common TA is not indicated

-        Note-1: Definition of NTA is different from that in RAN1 #103-e  agreement.

-        Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

-        Note-3: This agreement does not preclude aligned DL & UL timing at gNB or unaligned DL & UL timing at gNB with decision up to future discussion if any.

-        Note-4: NTA,common is the common timing offset as agreed in RAN1 #103-e

Discussion on the granularity of common TA

NTA,common is a common timing offset to be applied by the UE to deal with the RTD from the reference point to the satellite. It is derived by the UE from NTA,Common  if broadcasted by the gNB, it is calculated and expressed at unit of N∙Tc. Required bit number is shown in Table 1. With consideration on the tradeoff between accuracy and signalling overhead, the large granularity of NTA,common  i.e. 2048 Tc is preferred.

Proposal 1: With consideration on the signalling overhead, the large granularity of NTA,common i.e. 2048 Tc is preferred.

Table 1  Required bit number for common TA expression

| Scenarios | GEO based NTN | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Max RTD on the feeder link | 270.73 ms | 20.89 ms (1200km) | 12.89 ms |
| Required bit number (granularity is Tc) | 29 | 26 | 25 |
| Required bit number (granularity is 128 Tc) | 22 | 19 | 18 |
| Required bit number (granularity is 1024 Tc) | 19 | 16 | 15 |
| Required bit number (granularity is 2048 Tc) | 18 | 15 | 14 |
| Required bit number (granularity is 4096 Tc) | 17 | 14 | 13 |

NTA,common is a common timing offset to be applied by the UE to deal with the RTD from the reference point to the satellite. There are 3 options for the common TA discussed in RAN1#103e meeting and summarized in moderator’s summary document R1-2009697.

- RP OPTION 1: The RP is located at the gNB. Common TA indication shall be introduced.

- RP OPTION 2: The RP is located at the satellite. Common TA indication may be avoided.

- RP OPTION 3:  The RP localization is not specified and left to the implementation, i.e. network. Common TA indication shall be introduced to support all the foreseen deployment scenarios.

When RP OPTION 1 is used, the UE have to compensate the TA of feeder link, but without accurate information of feeder link delay. Network is required to indicate propagation delay information of feeder link, which will cause larger overhead. Configuring RP at satellite allows to avoid broadcasting the common TA. RP OPTION 3 can be seen as a generic option that can be used to support all the foreseen deployment scenarios. Adopting a flexible framework is the best option to reach consensus. At least RP OPTION 2 shouldn’t be precluded. With the consideration to supporting RP option 2, the UE should assume that   NTA,common=0, when it is not broadcasted.

Proposal 2: The UE should assume that   NTA,common=0, when common TA is not broadcasted by network.

TA calculation at UE side

In the last RAN1 meeting, the following was discussed.  The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:
TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc

Open and closed loop mechanisms for TA adjustment should be supported in NTN. NTA and NTA,offset are defined as in Release-16. NTA is informed by RAR and TAC in MAC-CE. Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode assume  NTA =0.

Proposal 3:  NTA  is controlled by RAR and TAC in MAC-CE as in Release-16.

Proposal 4:  Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode assume  NTA =0.

Discussion on the common timing drift rate

It was agreed in RAN1#98b [4] that indication of timing drift rate by gNB to the UE is beneficial. UE with GNSS capability can only acquire TA of service link based on UE location and satellite ephemeris, while the accuracy is affected by the error of acquired ephemeris and location information. When timing drift rate is signalled by gNB, the less TA update signaling is needed.

With the indication of timing drift rate, timing adjustment upon reception of a TAC or a timing adjustment indication is as follows

NTA_new=NTA_old+∆NTA∙∆t

where  ∆NTA is the timing drift rate and ∆t is the time interval from last timing adjustment indication.

Due to the large transmission delay and timing drift, the TA value carried in the TA update signalling is outdated when it arrives at the UE side. This happens in LEO-based NTN and is more severe at low elevation angle. To address this issue, the network can pre-correct the indicated TA value with the drifted parts when it sends out the TA update signalling based on the timing drift rate and the transmission delay.

Proposal 5: The common timing drift rate indicated by network should be supported.

UL frequency synchronization

The radial speed of NGSO satellite as seen by the UE on the service link can reach significant values. As a consequence, the Doppler shifts experienced on the service DL and UL link become critical in the NGSO scenarios [2] (cf. Table 2). DL and UL Doppler shift can be observed:

- DL RX signal at UE side is going to be affected by the un-compensated Doppler shift due to satellite mobility;

- UL RX signal at gNB side is going to be affected by the Doppler shift due to satellite mobility.

Table 9 : Max Doppler shift/drift to be supported in NTN scenarios

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27 ppm/s (600km) |

Since it is expected to reuse the same SSB design as per Release 15/16, Doppler shift pre-compensation on the DL is needed to help the DL synchronization at UE side. In case UEs can’t compensate for Doppler shifts experienced on both DL and UL service links, it is expected that UL frequency misalignment will be observed between the UL transmissions received at the gNB leading to critical performance losses. So the pre-compensation of the common frequency offset on DL transmission should be considered as a baseline assumption.

To support UE calculating pre-compensation value for UL transmission based on DL signal, it is necessary that pre-compensation value on the Doppler shift for DL transmission is signalled to UE. If the residual UL frequency offset at reference point is informed by network, UE can improve the accuracy of pre-compensation value for UL transmission based on the information.

Proposal 6: Pre-compensation value for DL frequency should be indicated by network.

Proposal 7: The residual offset value of UL frequency at the reference point should be indicated by network.

Conclusions

In this contribution, we discuss UL time and frequency synchronization for NTN, the following proposals have been made:

Proposal 1: With consideration on the signalling overhead, the large granularity of NTA,common i.e. 2048 Tc is preferred.

Proposal 2: The UE should assume that   NTA,common=0, when common TA is not broadcasted by network.

Proposal 3:  NTA  is controlled by RAR and TAC in MAC-CE as in Release-16.

Proposal 4:  Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode assume  NTA =0.

Proposal 5: The common timing drift rate indicated by network should be supported.

Proposal 6: Pre-compensation value for DL frequency should be indicated by network.

Proposal 7: The residual offset value of UL frequency at the reference point should be indicated by network.

References

- RP- 210908 “Solutions for NR to support non-terrestrial networks (NTN)”, Thales

- TR 38.821 v16.0.0 "Solutions for NR to support non-terrestrial networks (NTN)"

- Chairman’s Notes RAN1#104-e

- R1-2102215	 FL Summary on enhancements on UL time and frequency synchronization for NR NTN	Moderator (Thales)