3GPP TSG RAN WG1 #104bis-e                                                         R1-2103014

e-Meeting, April 12th – 20th, 2021

Source:	Intel Corporation

Title:	Enhancements to Multi-Beam Operation

Agenda item:	8.1.1

Document for:	Discussion and Decision

# Introduction

This paper is related to the new WID on further enhancements to MIMO in 5G NR Release 17 [1]. In the WID, the following objectives are outlined for beam management enhancements for feMIMO.

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signalling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signalling (as opposed to RRC) - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

In this paper, we present discussion on the objectives agreed in RAN1#102-e and corresponding agreements from RAN1#103-e and RAN1#104e, as well as analysis of SLS results for intra and inter-cell mobility according to simulation assumptions agreed in [2].

# Issue 1: Unified TCI Framework

Agreements from RAN1#104-e: Conclusion On Rel.17 unified TCI framework, based on the agreements in RAN1#102-e and 103-e, the following terms are defined as follows (at least for the purpose of discussion and reaching agreements). For M=1: - DL TCI: The source reference signal(s) (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC For N=1: - UL TCI: The source reference signal in the UL TCI provides a reference for determining UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC For M=N=1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. - Separate DL/UL TCI: The DL TCI and UL TCI are distinct (therefore, separate). For M>1: - DL TCI: Each of the M source reference signals (or 2M, if qcl_Type2 is configured in addition to qcl_Type1) in the M DL TCIs provides QCL information at least for one of the M beam pair links for UE-dedicated receptions on PDSCH and/or subset of CORESETs in a CC For N>1: - UL TCI: Each of the N source reference signals in the N UL TCIs provide a reference for determining UL TX spatial filter at least for one of the N beam pair links associated with dynamic-grant(s)/configured-grant(s) based PUSCH, and/or subset of dedicated PUCCH resources in a CC For M>1 and/or N>1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. In this case, M=N. - Separate DL/UL TCI: The M DL TCIs and N UL TCIs are distinct (therefore, separate). Note: Other TCI types/terms such as “common TCI” are not used. Agreement On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D. - Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported: - CSI-RS for beam management - CSI-RS for tracking - FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types Agreement On Rel.17 unified TCI framework, the following source RS types for UL TX spatial filter are supported: - CSI-RS for tracking - Note: SRS for BM, SSB, and CSI-RS for BM have been agreed in RAN1#102-e - FFS (to be decided by RAN1#104bis-e): non-BM CSI-RS other than for tracking, non-BM SRS Agreement On Rel.17 unified TCI framework: - For joint and separate DL/UL TCI, DL large scale QCL properties are inferred from one (qcl-Type1) or two RSs (qcl-Type1 and qcl-Type2) analogous to Rel.15/16 - For joint DL/UL TCI, UL spatial filter is derived from the RS of DL QCL Type D Agreement On Rel.17 unified TCI framework, by RAN1#104bis-e, down select or modify at least one from the following alternatives: - Alt1. A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI - Details on dynamic indication are FFS - FFS: UE capability for the support of joint DL/UL TCI and/or separate DL/UL TCI - Alt2A. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via RRC signaling - Alt2B. A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling - Alt3. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via MAC CE signaling - Details on how this is signaled in relation to TCI activation are FFS Agreement On Rel.17 unified TCI framework, decide by RAN1#104bis-e: - Whether DL or, if applicable, joint TCI also applies to the following signals. If not, FFS any other enhancement over Rel.15/16: - CSI-RS resources for CSI - Some CSI-RS resources for BM, if so, which ones (e.g. aperiodic, repetition ‘ON’) - CSI-RS for tracking - Whether UL or, if applicable, joint TCI also applies to the following signals - Some SRS resources or resource sets for BM Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework: - The setting of (P0, alpha, closed loop index) is at least associated with UL channel or UL RS - Select or modify from one of the following alternatives by RAN1#104bis-e for PUCCH, PUSCH, and SRS separately: - Alt1. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state - Alt2. The setting of (P0, alpha, closed loop index) is included with UL or (if applicable) joint TCI state - Alt3. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state - Alt4. The setting of (P0, alpha, closed loop index) is determined as in Rel-16 without enhancement Agreement On Rel.17 unified TCI framework: - Select at least one of the following alternatives by RAN1#104bis-e for path-loss measurement (PL-RS): - Alt1. PL-RS can be included in UL TCI state or (if applicable) joint TCI state. - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state. - Alt2. PL-RS can be associated with (but not included in) UL TCI state or (if applicable) joint TCI state - FFS: Exact association mechanism - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state - Alt3. The periodic DL-RS used as a source RS for determining spatial TX filter can be used as PL-RS. In case the periodic DL-RS used as a source RS for determining spatial TX filter is not used as PL-RS, reuse Rel.16 procedure with the same signaling structure (MAC CE+SRI field in UL-related DCI) to indicate PL-RS for UL transmission with minimum enhancement (e.g. pertaining to the use for PUCCH, or using default PL-RS) - PL-RS is not additionally configured in or associated to UL TCI state or (if applicable) joint TCI state - Alt4. UE calculates path-loss based on periodic DL RS configured as the source RS or a periodic QCL-Type-D/spatialRelationInfo source of the source RS in UL TCI state or (if applicable) joint TCI state - FFS: Whether UE can calculate path-loss based on DL periodic RS for path-loss calculation for UL RS in the UL TCI - FFS: Application time of PL-RS - NOTE: As in Rel-16, a UE does not expect to simultaneously maintain more than four path-loss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions FFS: investigate the condition(s) agreed in Rel-17 and, if needed, study whether a UE can simultaneously maintain more than four path-loss estimates

## Unified TCI Framework for Uplink

- In Rel-16 uplink spatial relation information framework, the PUCCH-SpatialRelationInfo includes a PUCCH Pathloss Reference RS ID, a P0 and closed loop index as mandatory parameters. For SRS and PUSCH, MAC-CE signaling using SRS and PUSCH pathloss reference RS update MAC-CE respectively can provide the relevant pathloss reference RS. Therefore in order to ensure similar capability as Rel-16 NR, the joint DL/UL or separate UL TCI state should include the pathloss reference RS ID as well P0 and closed loop index for PUCCH as part of the TCI state configuration. The SRS and PUSCH pathloss reference RS ID can either be separately updated via MAC-CE or optionally included in the TCI state. Therefore, for PUCCH pathloss RS, we prefer Alt 1 and for power control parameters we prefer Alt 3. Note that in the current proposal on power control parameters Alt 4 may not work for PUCCH. For joint DL/UL TCI state configuration, a similar approach can be used wherein uplink related parameters can be introduced into the DL TCI framework.

Proposal 1: Joint DL/UL and separate UL TCI configuration should include the pathloss reference RS and UL power control parameters (P0, alpha, closed loop index) for PUCCH as a mandatory configuration.

Proposal 2: SRS and PUSCH pathloss reference RS ID can be optionally included in the TCI state configuration or updated by MAC-CE

## Unified TCI Framework for Downlink

- In RAN1#104-e it was agreed that supported source/target QCL relations in Rel-16 are also valid for the Rel-17 unified TCI framework. There was additional discussion whether SSB and SRS for beam management should be supported as additional source RS for DL QCL Type D. On the issue of using SSB directly as a source RS for DL, the technical benefits of such a configuration do not seem very clear since CSI-RS for tracking is mandatorily configured and it is quasi co-located with an SSB. Hence configuring the TRS as a QCL source should serve the same purpose without additional support of SSB as a direct QCL source. For CA cases, the TRS can be QCL-ed with the same SSB and hence motivation to additionally support the SSB is unclear.

- Proposal 3: SSB is not required to be supported as an additional DL QCL source RS for the unified TCI framework.

- Similarly, in the case of SRS for BM, it is expected that a periodic DL RS is configured at least for pathloss estimation. Therefore, this periodic DL RS can equally be used as the QCL source without the need for configuring SRS as a QCL source and we see no benefit for such configuration.

- Proposal 4: SRS for BM is not is not required to be supported as an additional DL QCL source RS for the unified TCI framework.

- Another open issue is whether DL or joint DL/UL TCI is also applicable for CSI-RS for CSI, some CSI-RS for BM and TRS. When a joint TCI state is indicated, the UE should be capable of further beam tracking and CSI reporting with respect to the indicated beam and therefore we do not see the need for restricting the applicability of DL or joint TCI for these CSI-RS resources.

- Proposal 5: DL or joint DL/UL TCI also applies to CSI-RS resources for CSI, CSI-RS resources for BM and CSI-RS for tracking

## TCI State Pool Design and Configuration of TCI states

### TCI State Pool

Joint DL/UL TCI states for common DL/UL beam update, as well UL TCI state for separate UL beam update have been agreed. However, how these TCI states are configured to the UE is still under discussion. Currently, the configuration and indication of TCI state for DL works via MAC-CE which activates up to 8 TCI states, one of which can be activated by the DL scheduling DCI. Similar operation is envisioned for Rel-17 beam indication and therefore to simplify operation and reduce overhead of configuration, UL TCI states for separate beam indication should share the same TCI state pool along with DL TCI states. If separate TCI state pools are chosen, then UE will need to be configured with two separate activated TCI state lists by different MAC-CEs leading to higher DL overhead while the benefits for such configuration is unclear. If necessary, it is more preferable to increase the number of activated TCI states from 8 to 16 bits and use a 4-bit DCI field to indicate TCI states.

- Proposal 6: Separate UL TCI states and joint DL/UL TCI states will share the same TCI state pool as DL TCI states.

- For common TCI state pool design, certain fields in the TCI state IE can be optional for different types of TCI states i.e., UL power control related parameters are omitted from DL TCI states.

### TCI State Configuration and Usage Indication

The Rel-17 unified TCI framework is envisioned to be a flexible framework enabling fast beam switching across a multitude of scenarios. Since the framework now includes three different types of TCI states namely, joint DL/UL, separate DL and separate UL TCI state, each with its own applicable use case, switching between such TCI states should be kept flexible. If RRC based configuration is required to indicate a joint DL/UL vs separate DL or UL TCI state, the benefit of fast beam switching cannot be realized. Therefore, by default, a UE should be capable of being indicated with any type of TCI state through dynamic signaling. Based on previous agreement, we prefer Alt1 for maximum flexibility.

Proposal 7: A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI

The detailed indication can include the configuration of TCI states to TCI codepoints via MAC-CE where the MAC-CE also includes TCI state usage indication i.e., the MAC-CE will additionally configure TCI usage per codepoint such that the UE knows the type of TCI state when indicated with a DCI codepoint. In this framework, UE can also be configured with more than one TCI state per codepoint in case separate DL and UL TCI states are simultaneously activated. An use case for such activation is when UE receives DL from a macro cell but sends UL to small cell for load balancing and therefore beam correspondence does not apply.

Proposal 8: TCI state usage can be configured via MAC-CE where the type of TCI state configured to each codepoint is separately configured. The UE when indicated with a TCI codepoint associates the codepoint with joint DL/UL or separate DL or UL TCI state based on MAC-CE configuration.

## Intra and Inter-Band CA

For the case of CA, from a RAN1 perspective it is desirable to support the feature without restrictions i.e., the inter-band CA case should be also supported. For the case of TCI state pool configuration for CA, two options were listed in RAN1#103-e. Option 1 shares a single RRC TCI state ID pool across CCs which can save some RRC overhead depending on the number of configured CCs. Since CCID for a QCL Type A is absent in the TCI state, it needs to be implicitly determined. Alternately, the RRC TCI state ID pool can be configured per CC. While both options present a valid solution, Option 1 is slightly preferred since it has the potential to save overhead when multiple CC’s are configured.

Proposal 9: A shared RRC TCI state ID pool is supported for a set of configured CCs. The QCL Type A RS reference is implicitly determined based on the target CC of the TCI state.

It was also agreed that “same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs”. However, some companies pointed out that based on current agreement and restrictions of Rel-15 QCL chain, the QCL Type-D chain may enforce the gNB to transmit CSI-RS with repetition. While it is not ideal, it should be clarified if CSI-RS with repetition is actually needed to be triggered or can be simply used a configuration in the QCL chain which is QCL’ed with the same SSB. Otherwise, the same rule for Type A derivation i.e., QCL Type D RS can be determined in each CC from the common TCI state and the determined RSs can be QCL’ed with the same QCL Type D RS as shown in Figure 2 from the email discussion

Proposal 10: Separate QCL-TypeD RS can be determined from the common TCI state(s) for each of the CCs. The determined QCL-TypeD RSs for the set of CCs are further associated with a same QCL-TypeD RS.

# Issue 2: L1/L2 based Inter Cell Mobility

Agreements from RAN1#104-e: Agreement On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP: - A quality of up to K beams associated at least with non-serving cell(s) can be reported in a single CSI reporting instance - For each beam, the UE can report at least: (1) a Measured RS Indicator, and (2) a Beam Metric associated with the Measured RS Indicator - FFS: Maximum value of K - FFS: If K is fixed, configured, reported by UE capability, or dynamically selected - FFS: The type of beam metric (e.g. L1-RSRP, L3-RSRP, or hybrid L1/L3-RSRP) and related measurement behavior - FFS: Whether or not beam reporting associated with non-serving cell(s) can be mixed with that with serving-cell in one reporting instance At the end of RAN1#104-e, send an LS to RAN2 with all the RAN1-related inter-cell mobility agreements done so far during Rel17. LS is endorsed in R1- 2102209. Agreement On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP: - Rel.15 L1-RSRP is used as reporting quantity for measurement and reporting of non-serving-cell(s) - Support SSB as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP, and Rel.15 SS-RSRP calculated from SSB of non-serving cell(s) - FFS: Whether the measurement for SS-RSRP is limited within SMTC - FFS: Detailed reporting method, e.g. via including existing L1-RSRP report, UE-initiated report etc. - FFS: Whether or not to support CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP. If the support of CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP is confirmed, Rel.15 CSI-RSRP is also supported - Whether the support applies to CSI-RS with or without QCL source, or both - FFS: The number of non-serving cell(s) for measurement/reporting - FFS: time behavior of the reporting, i.e. periodic, semi-persistent, aperiodic, or UE-initiated - FFS: If other reporting quantities are supported, e.g. L3-RSRP, hybrid L1/L3-RSRP - FFS: Dynamic activation/deactivation/selection of the beam measurement on the RS(s) associated with non-serving cell(s) via MAC CE - FFS: Timing assumption (e.g. time of arrival and time of the measurement) for measurement of non-serving cell RS measurement Agreement On Rel.17 enhancements for L1/L2-centric inter-cell mobility, - Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s): - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references - SSB configured for non-serving cell(s) for UL TX spatial references - SRS for BM configured for non-serving cell(s) for UL TX spatial references - FFS: whether to support CSI-RS for mobility - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source - Send an LS to RAN2 on TCI state update (beam indication) using source RS configured for non-serving cell(s) for DL reception and UL transmission. The following topics are considered for the LS: - RRC configuration issues - Serving cell issues - C-RNTI issues - Issues related to CU-DU split - Inter-band CA issues - Inter-frequency issues

## Timing Advance for Uplink in L1/L2 Mobility

In RAN1#104-e meeting, it was agreed that beam reporting from non-serving cells for up to K beams will be supported. A UE which reports measurements from a SSB, or potentially other RSs associated with non-serving cell may be configured with a TCI state (joint DL/UL or separate UL) corresponding to the non-serving cell. While measurement and reporting have been discussed thus far, uplink transmission has not been discussed in detail. Specifically, in case the UE is switched to a new cell using only L1 measurements, the UE may also need to update its TA in the uplink assuming a synchronous network and within CP uplink reception in the new cell. In this case, if additional signaling is needed following TCI state indication i.e., RRC reconfiguration of TA groups (TAG) and additional MAC-CE signaling of TA for the new cell, it may lead to added latency.

Figure 1: Timing Advance update for L1/L2 centric inter-cell mobility

Figure 1, shows the scenario where the UE may be misaligned with respect to uplink timing if the TA is not updated for TCI states corresponding to a non-serving cell. In this case an option would be to pre-configure TAGs with some association to the PCID of the serving and non-serving cells. When a UE is indicated with a TCI state associated with a particular PCID, the UE may be able to update the TA according to the preconfigured TAG whose associated PCID matches the TCI state. Alternately, the groups of TCI states can be associated with a particular TAG-ID such that the UE, when indicated with a non-serving cell TCI state can implicitly associate the correct TA value with the TCI state.

Proposal 11: For DCI based TCI state indication associated with a non-serving cell for L1/L2 centric inter cell mobility, TA should be adjusted for UL transmission where the TAG could be associated with a PCID or a group of TCI states corresponding to a PCID.

## Mobility Enhancement in RAN1

In the rather lengthy agreements from the last two RAN1 meetings, a lot of different options have been listed for support of L1/L2 based mobility enhancement. However, the scope of such enhancement first needs to be clarified at a functional level in terms of what RAN1 can achieve independently of RAN2. As discussed in the last meeting and the LS sent to RAN2, their guidance is imperative to design a practical feature. However, RAN1 also needs to clarify the scope to be handled in RAN1 as we wait for the response LS from RAN2. To this end, we outline the following options for the scope of L1/L2 based mobility enhancement.

- Option 1: Serving cell change relies on existing L3 HO procedure i.e., L1/L2 mobility does not support serving cell change. Similar to CoMP Scenario 3 in LTE with minimal impact on RAN2. RAN1 impact is introduction of PCID in TCI state configuration. RRM/RLM is not impacted since serving cell does not change.

- Option 2: Serving cell change via L1/L2 signaling (intra-DU). From RAN2 perspective, no MAC/RLC reconfiguration but RRM/RLM needs to be updated.

- Option 3: Serving cell change via L1/L2 signaling (both intra- and inter-DU). From RAN2 perspective multiple non-serving cells can be pre-configured and RRM/RLM needs to be updated

For Option 1, serving cell ID is not changed and L1/L2 mobility only supports TCI state change from one TRP to another. Under this option, the only enhancement required is to include the PCID in TCI state. L1 measurements and RRM/RLM measurements and reporting are not impacted since serving cell remains the same.

For Options 2 and 3, serving cell change is handled using L1/L2 signalling which then replaces L3 HO procedure. Both Option 2 (intra DU) and Option 3 (inter- and intra-DU) needs additional work from RAN2 and should be ideally handled or led by RAN#2 as part of mobility enhancements.

Based on the discussion above, and irrespective of the reply to the LS from RAN2, we believe only Option 1 falls within the scope of enhancements which can be independently handled by. Any other enhancement involving L1/L2 based handover whether it co-exists with or replaces current procedure should be left to RAN2.

Proposal 12: The scope enhancement for RAN1-led L1/L2 mobility is limited to introduction of PCID in the unified TCI state configuration or SSB configuration which serves as QCL source for a TCI state. No impact to L1 measurement or RRM/RLM is supported.

In the case that Option 2 or 3 is envisioned as the scope for this agenda, we prefer that this is handed to RAN2 since there is major impact to higher layer specification. Additionally, as highlighted below, updates to L1 measurement and reporting may also be needed based on SLS results for inter-cell mobility.

### L1 Measurement and Reporting for Options 1

In RAN1#104-e a framework for supporting L1-measurement based reporting for non-serving cell beams was agreed. To this end, a UE can report up to K non-serving cell beams in a single CSI reporting instance. The value of K can be a UE capability which is reported. Additionally, if PCID is associated with SSB or TCI state configuration, then the UE should be able to report both serving and non-serving cell measurements in the same CSI reporting instance without any restrictions and the gNB can indicate a TCI state based on such measurement report.

Proposal 13: The value of K non-serving beams which can be reported is based on UE capability

Proposal 14: Both serving and non-serving cell reports can be combined within a single CSI reporting instance

The scope of enhancement in RAN1, if limited to Option 1 as discussed above, the level specification changes to support the L1/L2 based mobility should be minimized and to that end, the current beam reporting metric of L1-RSRP should be sufficient if the UE is reporting single beams without the scope of handover to a non-serving cell

Proposal 15: L1-RSRP is used as the beam reporting metric and the reporting is done via existing L1-RSRP report

### L1 Measurement and Reporting for Options 2 and 3

In this section we present initial evaluation results on inter-cell mobility to highlight current issues which need to be addressed in order to enable L1 based mobility procedures. For the purpose of this evaluation, we did not model the random handover delay and interruption time in [2] since it would render the objective of the evaluation useless if we were not able to observe the impacts of L1 impairments on the performance.

In the follow we show the case of L1-handover modeled using a handover threshold of 3dB i.e., handover is assumed when the RSRP of a non-serving cell is 3dB higher than the RSRP of a serving cell. We perform evaluation with and without UE rotation modeling as before. Note that the front panel of the UE faces the direction of motion.

Figure 2: (L) Handover locations for UE with no rotation. (R) Handover locations for UE with 1 RPM rotation. The impact of rotation is a significant ping-pong effect where the UE performs multiple handovers.

In this evaluation, Tx beam averaging is used from the gNB but time domain filtering of RSRP is not assumed. When time domain filtering of RSRP is implemented with a low pass filter such that RSRPavg = α*RSRPnew + (1-α)*RSRPavg with α=0.5, the number of handovers is shown to be reduced from 33 to 20 while RSRP performance on the best beam pair link remains almost identical.

Figure 3: Time domain RSRP filtering improves handover performance by reducing ping pong effect

Based on the evaluation results it is seen that L1-RSRP averaging needs to be performed for L1 handover modelling to avoid ping pong effect. This is similar to the L3 Handover case which uses a cell-level and a beam level metric. Additionally, spatial domain filtering across beams at the UE is also needed to smoothen the L1-RSRP. Such filtering needs to be accounted for before the L1-RSRP is reported back to the gNB.

For L1-handover modelling the spatial averaging can be performed at the UE and the UE can trigger can a handover event i.e., an event driven handover mechanism can be considered. However, before pursuing normative work on this model, further details need to be clarified. For example, how L1 inter-cell mobility coexists with L3 handover framework if Option 2 or 3 leading to serving cell change is considered. The assumptions on how many neighbour cells the UE can monitor is also important. As we have observed from our SLS results, it can often be the case, especially under UE rotation that UE can receive a strong signal when beam alignment happens with a cell which geographically far from the UE. Therefore, measurements on such cells as well as spatial domain filtering across beams needs to be enabled for best performance. Furthermore, the measurement latency is determined by the number of cells UE has to monitor. In our simulations, we have assumed a somewhat ideal scenario where UE can monitor all the cells in the system with a measurement periodicity which is identical to that of the serving cell. Even with such an ideal assumption, we note the performance degradation that is observed. In case this latency is increased, the handover performance and ping-pong effect worsen. Finally, it is unclear even when the UE monitors and performs measurement on neighbour cell RSs, what would be the assumption on random access when the handover event is triggered.

Observation 1: L1 based handover modelling suffers from ping-pong effect especially when impairments such as UE rotation is considered. To this end, time and spatial domain filtering for L1-RSRP reporting and UE triggered serving cell change event might need to be considered. Additionally, the following needs to be addressed:

- How L1 based serving cell change co-exists with current L3 handover framework?

- How many neighbor cells can be monitored by the UE and its implication on L1-RSRP and RRM/RLM measurements?

- Assumptions on measurement periodicity from neighbor cells

- Assumptions on random access procedure or lack thereof when UE triggers a beam switch to a non-serving cell

# Issue 3: Dynamic TCI State Update Indication

Agreements from RAN1#104-e: Agreement On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - FFS: the exact minimum values of beam application time supported by UE - FFS: whether existing UE capability can be reused as this UE capability. - FFS: whether different beam application time values are supported for uplink and downlink - FFS: whether UE capability needs to be introduced for the maximum value of beam application time - FFS: the reference for defining the UE capability (e.g. from DCI reception or ACK transmission) - FFS: whether a UE is allowed to report more than 1 values in case of MPUE FFS: the application time when DCI and applied channel(s) are on different CCs with same/different SCS(s)s Agreement On the Rel.17 DCI -based beam indication, in RAN1#104bis-e, down-select at least one of the following alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e): - Alt0: No additional DCI format is supported - Alt1: DCI formats 1_1 and 1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : How to identify DCI formats 1_1/1_2 used for beam indication only (not for scheduling a PDSCH reception, not indicating a SPS PDSCH release, or not indicating SCell dormancy), considering impacts on PDCCH coverage and scheduling mechanism - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt2: Dedicated DCI format other than 1_1/1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : If the format is based on an existing DCI format, how to identify the DCI format used for beam indication only - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt3: UL-related DCI formats 0_1/0_2 with UL grant, applicable only for UL-only TCI of separate DL /UL TCI Agreement On Rel.17 DCI-based beam indication, regarding application time of the beam indication: if beam indication is successfully received and the newly indicated beam in the beam indication is different from the previously indicated beam, down-select (no later than RAN1#105-e) one from the following. No other alternatives will be considered: - Alt1: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the DCI with the joint or separate DL/UL beam indication - Alt2A: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication - Alt 2B: the first slot that is at least X ms or Y symbols after the [first/last] symbol of the acknowledgment of the joint or separate DL/UL beam indication, except that the (new) TCI state update can be applied to the PDSCH, if it exists, (scheduled by the beam indication DCI) and corresponding ACK transmission (provided that the time offset between the DCI and the scheduled PDSCH exceed the threshold, analogous to Rel.15/16) - Alt2C: Support both Alt1 and Alt2A, and introduce a UE capability that indicates the support of Alt1 or Alt2A - Alt3: the first slot that is at least X1 ms or Y1 symbols after the [first/last] symbol of the DCI with beam indication and X2 ms or Y2 symbols after the [first/last] symbol of the acknowledgment of the beam indication FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y

## DCI Formats for Dynamic Beam Indication

### Re-using Current DCI Formats

DL DCI formats without DL Grant for joint DL/UL or Separate DL/UL Beam Indication

In the previous meeting alternatives for DCI based beam indication were listed for further down-selection. While DCI 1_1 and 1_2 have already been agreed to be supported for beam indication, the use of grant-free DCIs should also be allowed. Especially for UL only beam indication, tying the beam indication to a DL grant seems artificial. Therefore, we support Alt 1 from the above agreement.

Proposal 16: DCI formats 1_1 and 1_2 without DL assignment are supported for joint TCI as well as separate DL /UL TCI along with a DCI acknowledgment mechanism

DCI Formats UL Only Beam Indication

For the case of UL only beam indication, DCI format 0_1, 0_2 can be used with the SRI field being re-interpreted as the TCI indication field similar to the TCI field in DL DCI formats. Based on the UL DCI format the UE is able to apply the beam update to all UL channels and signals e.g., PUCCH, PUSCH and SRS.

DL DCI formats 1_1, 1_2 can also be used to indicate UL TCI state (both with and without a DL grant). When the UL TCI state shares the same TCI state pool with DL and joint TCI states, the UE is able to differentiate the UL TCI state based on the indicated codepoint i.e., the UE is able to map the codepoints to types of TCI states based on MAC-CE configuration as discussed in Section 2.3.2.

Proposal 17: For separate UL beam indication using UL TCI state

- When current DCI formats are re-used, the TCI state is applicable to all UL signals and channels

- UL DCI formats 0_1, 0_2 can be used with SRI field being reinterpreted as the TCI codepoint in DCI.

- DL DCI formats 1_1,1_2 can be used with implicit indication of UL TCI using TCI in DCI field where UE maps the indicated codepoints to UL TCI states based on the MAC-CE activation of TCI states in DCI codepoints

Group-Common Beam Indication DCI

For the case of using DCI based indication for TCI state activation of a group of UEs, a design based on dependent DCI is not desirable since if the first DCI is missed, it may severely degrade system performance. Instead, a group-common PDCCH based approach leveraging the NR MBS framework can be further considered. The aforementioned beam indication RNTI can be used to address this set of UEs and the CRC of group-common PDCCH can be scrambled by this RNTI.

Proposal 18: Consider NR MBS framework for group-common DCI transmission to a UE group for joint TCI state activation using a new beam indication RNTI for scrambling the CRC of the group-common DCI.

### Introduction of New DCI Format

In order to simplify signaling and to reduce the dependence on MAC-CE based TCI state activation, a new DCI format may be introduced to signal the necessary information for TCI state indication. Additionally, the bit field required to activate one of the RRC configured TCI states will need to much larger than the current 3 bits, possibly 6-8 bits depending on the number of configured TCI states. Additionally, a new DCI may also have the flexibility to indicate which channels and RSs are the target for the TCI state indication along with the serving cell ID, BWP ID and other specific information which are conveyed by MAC-CE or configured via spatial relation information for UL but currently absent in TCI state configuration. In such case, the DCI size needs to be provisioned for the worst-case payload. Furthermore, while DCI is easily envisioned for common beam indication, it should also support the case of separate beam indication for different channels and RSs. In such a case a careful design of the DCI format is required, and it might also have impact on current DCI size budget and might require size alignment with other non-scheduling DCI. Furthermore, DCI indication for CORESET beams needs special consideration. Additionally, HARQ might need to be supported and this may have further impact on the overall latency of the DCI based beam indication process.

Proposal 19: For DCI based indication, a new DCI format should be considered with the capability to dynamically indicate individually, the channels and RSs to which the TCI state is applicable. Additionally, both common and separate beam indication should be supported.

## Beam Application Time

For DCI based beam indication, the application time for the indicated beam was discussed in RAN1#103-e. The options to count the beam application are either from the reception of the PDCCH activating the TCI state or after the acknowledgement to the beam indication DCI is transmitted. In order for the acknowledgement mechanism to be effective and for the gNB and UE to maintain common understanding of the active TCI state, it is preferred that the beam indication time is calculated after the transmission of the acknowledgement for the DCI.

Additionally, it should be clarified how the acknowledgement transmission time is defined since the PUCCH carrying the acknowledgement may not transmitted in the configured resource if it’s subject to PUCCH multiplexing or dropping etc. In this case, it is preferred that the X ms or Y symbols is counted from the starting symbol of the actual PUCCH resource which carries the ACK feedback after applying the multiplexing rules.

Proposal 20: For DCI based beam indication, the beam application time is the first slot that is at least X ms or Y symbols after the starting symbol of the PUCCH resource carrying the acknowledgment of the joint or separate DL/UL beam indication.

# Issue 4: UE Panel Selection

Agreements from RAN1#104-e: Conclusion On Rel.17 enhancements to facilitate UL beam selection for MP-UE, the following terms are used at least for the purpose of discussion: - ‘Panel activation’ (at least for DL/UL measurement): activating L out of P available UE panel(s) at least for the purpose of DL and UL beam measurements (e.g. reception of DL measurement RS, transmission of SRS) - ‘Panel selection’ (for UL transmission): selecting 1 out of L activated UE panel(s) for the purpose of UL transmission - Note: UE-initiated panel activation and selection have been agreed in RAN1#103-e Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, - Rel.17 TCI state update (based on MAC CE + DCI along with the necessary TCI state activation, or MAC CE only) can be used for UE UL panel selection: - FFS : Whether specification support for this feature is necessary and if so the details of such spec support, e.g. - Additional spec support in TCI state definition to accommodate UL panel - UE reporting to facilitate UL panel selection - UE reporting, e.g. panel-specific report, including UE -panel state, e.g. inactive, active for DL /UL measurement, active for DL reception only, active for UL transmission, or other combination(s) of UE -panel states - Support for linking or association of UE panels with CSI-RS/SSB resources or resource sets, SRS resource sets, and/or PUCCH resource groups, etc.

For UE fast panel selection, the panel indication can be implicit and additional TCI update to capture UL panel is not needed.

# Issue 5: MPE Mitigation Techniques

Agreements from RAN1#104-e: Agreement On Rel.17 enhancements to facilitate MPE mitigation, - On further enhancing the P-MPR report in Rel.16 (already agreed RAN4 framework, including triggering), down select between beam-level and panel-select reporting - On SSBRI(s)/CRI(s) and/or indication of panel selection, focus study on the following: - Reporting of at least SSBRI(s)/CRI(s) to indicate gNB beam(s) that is feasible for UL transmission: additional reporting quantities are FFS - Reporting of at least an indicator associated with a UE ‘panel’ that is feasible for UL transmission: additional reporting quantities are FFS - Note: Just as agreed in RAN1#103-e, the purpose is to assess whether specification is needed or not Agreement On Rel.17 enhancements to facilitate MPE mitigation, decide in RAN1#104bis-e whether to support at least one the following (not necessarily, but can be, in one reporting instance): - {Rel.16 P-MPR based (beam/panel-level)} + {A}, where A is either Opt1A, Opt1B, Opt1C, or Opt1D: - Option 1A: Virtual PHR or a modified version associated with each activated UL TCI or, if applicable, joint TCI - Option 1B: {SSBRI(s)/CRI(s) and/or panel indication} - Option 1C: {SSBRI(s)/CRI(s) and/or panel indication} + virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - Option 1D: No additional reporting quantity - {SSBRI(s)/CRI(s) and/or panel indication} + {A}, where A is either Opt2A, Opt2B, Opt2A+ Opt2B, or Option 2C - Option 2A: L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - Option 2B: Virtual PHR or a modified version associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - Option 2C: No additional reporting quantity

## SLS Evaluations

We provide SLS results based on the evaluation assumptions in [2] to illustrate the need for MPE mitigation and the extent of specification support required based on the observed gains from the results. In the following figures, we provide evaluation for Dense Urban and Indoor Hotspot. Two panel blockage models are considered i.e., best panel blockage (which provides optimistic gains for MPE assuming ideal scenario) and random panel blockage (which is a more realistic scenario where a panel is randomly chosen for blockage). It is also assumed that the UE all transmit with full power over a sub-band size of 4PRBs.

Figure 4: MPE Mitigation under serving panel blocking model for Dense Urban Macro and Indoor Hotspot

Figure 5: MPE Mitigation under random panel blocking models for Dense Urban Macro and Indoor Hotspot

To illustrate the gains and tradeoffs for MPE, we carefully choose a realistic baseline scheme for UL panel selection without MPE mitigation. In this baseline scheme, it is assumed that the panel which may suffer losses due to MPE mitigation is also subject to blockage in the DL. To this end, the baseline scheme assumes that when a downlink panel suffers a blockage loss of 3dB, the UE selects an alternate panel and this panel is also used for UL. If the alternately selected panel after downlink loss is identical to UL serving panel or the randomly blocked UL panel (in each blockage model respectively), the corresponding UL panel is assumed to suffer an additional 10dB power back-off i.e., a total of 13dB loss. The resulting RSRP CDF is shown by the red curve in all figures.

Additionally, the last CDF (green) corresponds to the case when the serving/randomly blocked panel suffers a 13dB loss and there is no MPE mitigation resulting in a worst case RSRP. Conversely, the first curve (dashed black) is the RSRP from the best UE panel under the assumption that no MPE blockage occurs. The first set of results models the case when the best UE panel suffers blockage. It is seen that when MPE mitigation is allowed i.e., the UE can choose a potentially different panel, the RSRP performance shows significant improvement for Dense Urban and Indoor Hotspot. However, interestingly, when the UE is allowed to operate in a multi-TRP mode i.e., it can not only select the best alternate panel after MPE blockage event but can also select an alternate TRP, there is a major improvement in RSRP CDF especially for cell-edge UEs. Considering the fact that cell-edge UEs are the ones which tend to transmit with high PSD and are the most susceptible to suffer losses due to MPE, such gain is quite significant.

For the case of the random blockage model from [2] where a random panel is blocked and not necessarily the serving panel, results show that when DL suffers a loss in the panel which is subject to MPE regulation in the UL, the UE often selects an alternate panel and when this same panel is used in the UL most of the MPE related losses can be recovered. However, even in this case, when additional MPE mitigation is allowed, gains can be achieved for cell-edge UEs. Allowing multi-TRP based panel selection leads to significant improvement in cell-edge performance.

Observation 2: The following observations are made from the SLS results for MPE:

- Gains due to MPE mitigation techniques can be achieved especially for cell-edge users and when mutli-TRP operation is enabled for multi-panel UEs which suffer panel blockage due to MPE.

- While some gains can be recovered assuming downlink-based UL panel selection, simulation results justify the specification of MPE mitigation techniques for Rel-17 NR.

- Given the nature of gains which are relatively small, specification impact should be minimized, and simpler solutions adopted to support UE assisted recovery for MPE

## MPE Mitigation Related Enhancements

In RAN4#92bis the following agreement was reached in RAN4 regarding possible solutions [7].

- Rapid indication methods - P-MPR - Before P-MPR is taken - While P-MPR is applied - Can be one bit or include more information - Alert/Emergency signal to indicated back off is about to happen - Is this alert only or does it contain more information? - Assistance information methods - Energy headroom - How much energy UE has for transmissions until specific time - Power headroom - PHR reporting is in place already - Exposure headroom - Dynamic or Multiple maxUplinkdutycycle

Additionally, in RAN4#96e, for Rel-16, RAN4 agreed to introduce P-MPR reporting via MAC CE [3]. P-MPR reporting will be introduced, most likely reusing current PHR MAC CE (an approved LS was sent to RAN2 [4]):

- There will be 2-bits for P-MPR granularity, and value options are:

  - 3 ≤ P-MPR < 6

  - 6 ≤ P-MPR < 9

  - 9 ≤ P-MPR < 12

  - P-MPR ≥  12

- There will be a prohibit timer, with the value options listed below (which are the same as the PHR prohibit timer) {sf0, sf10, sf20, sf50, sf100, sf200, sf500, sf1000}

- P-MPR reporting is an optional UE capability, applicable to all FR2 power classes

- Triggering conditions: A configurable absolute P-MPR threshold will trigger the report; the values:

  - Absolute P-MPR threshold à {3dB, 6dB, 9dB, 12dB}

Although the above approaches are capable to indicate the MPE problem existing in the UL transmission, they don’t support recovery procedures helping to avoid radio link failure and connection releases. It is, therefore, necessary to amend the above approaches with reporting of additional information related to recovery of the UL link. Rel-15 beam reporting framework for FR2 is MPE-unaware – the optimal beam report doesn’t differentiate DL and UL transmission links and there is no mechanism (except explicit SRS transmission) to indicate UL beam, which doesn’t have significant Tx power constraint at the UE side due to the FR2 UE RF exposure compliance reasons.

As a result, even when MPE limited situation is reported from the UE to gNB using RAN4 solutions [3][14], the gNB may not be able to improve UL transmission performance because the solutions do not contain the recovery information. RAN4 solution however can be improved if the recovery information is included in the MAC-CE based P-MPR report facilitating selection of less MPE-limited (with small P-MPR) beams for UL transmission at gNB (see Figure 6).

Figure 6: Illustration of the need of beam selection enhancement to avoid MPE issue

In particular if large P-MPR, alert/emergency signal or duty cyclic constraint is transmitted by the UE for the active UL beam, UE may also report the alterative communication beam (e.g., with lower P-MPR or less duty cycle constraints) for UL transmission, similar to BFR recovery response specified in Rel-15/Rel-16 NR to cope with blockage issue. The framework of BFR can be largely reused to support UL beam recovery due to MPE. This method is also justified by the SLS evaluations in the previous section, where the choice of alternate panel/beam combination yields gains. However, since the nature of significant gains are shown to be limited to an extent to cell-edge users, a more complex solution which includes additional reporting is not required. Addition of recovery information is enough to achieve the gains related to MPE mitigation.

Proposal 21: For MPE mitigation for Rel-17,

- RAN4 solution for MAC-CE based P-MPR reporting is augmented with MPE recovery information indicating alterative UL beam with lower P-MPR or duty cycle constraints, where UE panel indication can be implicit.

- No additional reporting is needed since the gains compared to DL based UL panel selection are not very large.

Based on Proposal 21, we support {Rel.16 P-MPR based (beam/panel-level)} + {Option 1B} from the agreement in RAN1#104-e which should suffice for MPE mitigation.

# Issue 6: Latency Reduction for NR Beam Management Framework

Agreements from RAN1#104-e: Agreement On Rel.17 enhancements based on the unified TCI framework, perform study and, if needed, specify the following: - Group1: Beam management with reduced DL signaling to reduce latency - Group2: Reducing activation delay of TCI states and PL-RSs (including other WGs, e.g. RAN4) - On RAN4-related matters, assessment/study phase can be done in RAN1. If RAN4-based enhancements are found necessary, a LS to RAN4 will be sent (to prepare RAN4 work) Note: Given its dependence on the maturity compared to other issues (1 to 5), when to start the work and how much work is done on issue 6 should depend on the progress on the other issues. Note: Aim for at most one solution for each of the group in Rel-17 to address issue 6

## Identifying Performance Bottlenecks from SLS Evaluations

In order to efficiently support low latency beam management with reduce overhear in Rel-17 NR, especially with a large number of configured TCI states, the first goal should be to identify the major performance bottlenecks in the current beam management framework. To this end, two major latency issues exist with current beam management framework namely (i) TCI state indication and application latency and beam acquisition, refinement and tracking latency.

The TCI state indication and application latency is limited by the current MAC-CE based TCI state indication framework. In RAN1, signaling latency due to MAC-CE is defined as 3ms. However, in RAN4, the defined delay for TCI state switching is more conservatively defined [1] based on a concept of known and unknown TCI states. If the target TCI state is known, the delay is defined as

and if the TCI state is unknown it is defined as

where is the time to first SSB transmission after the MAC-CE is decoded by the UE and = 2ms and the additional processing times are required if the TCI state being activated is not in the active TCI state list. It is clear that TCI state application latency in the first case when TCI states are known is limited by the MAC-CE decoding latency. For the case of unknown TCI state additional time is required for L1-RSRP measurement and reporting which is in the realm of beam acquisition since the beams are not tracked. Therefore, the latency in this case can potentially be much higher than the 3ms defined in RAN1.

In this paper, we first provide SLS simulation results to investigate whether the major performance bottleneck stems from MAC-CE based TCI state application latency or beam acquisition latency due to SSB periodicity and measurements across multiple SSB periods. For this analysis we provide results based on the intra-cell mobility assumptions agreed in [2]. For initial analysis it is enough to use fast fading RSRP as the metric and therefore we simulated a single UE moving across the cell with the trajectory in Figure 1.

Figure 7: UE Trajectory for Intra Cell Mobility

For the UE shown in Figure 1, geographic association is assumed with cell 1. The detailed antenna configuration is provided in the Appendix. The gNB has an 8x4 sub-array and 32 DFT beams with 2x horizontal oversampling producing a total of 64 narrow beams corresponding to CSI-RS beams. We also assume that the gNB has a smaller 4x2 sub-array for forming wider DFT analog beams with 2x horizontal and 2x vertical oversampling corresponding to 32 SSB beams. Similarly, the UE has a 4x1 subarray on each panel and 8 narrow DFT beams with 2x horizontal oversampling corresponding to CSI-RS beams. Additionally, the UE is also capable of forming wider receive beams with a 2x1 subarray with 4 wide beams with 2x horizontal oversampling.

In our evaluations, we model impairments in the form of beam application latency and measurement/reporting latency. The application latency ensures the identified best beams are used only after the configured latency, while the measurement/reporting latency modelling makes beam reports available only after the configured latency.

### Impact of Beam Application Latency

First, we look at the impact of beam application latency when ideal and exhaustive narrow beam scanning is assumed. In this case, we assumed a measurement latency of 40ms and application latencies of 1ms and 3ms corresponding to TCI state indication faster than MAC-CE indication and the latency associated with MAC-CE indication of known TCI states respectively. The RSRP traces shown in Figure 2 are almost identical.

Figure 8: RSRP Trace of UE with and without rotation along β axis. It can be seen that RSRP is almost identical for application latency 1ms vs 3ms even with UE rotation.

The fact that application latency has minimal impact on the RSRP can be also seen from the comparison of median RSRPs across the trajectory in Figure 3.

Figure 9: Median RSRP for application latencies of 1ms and 3ms with and without UE rotation

From the above figures, it is seen that application latency of 3ms from MAC-CE vs sub-3ms case which would correspond to DCI based TCI state indication (possibly with HARQ) is not the performance bottleneck for beam management.

Observation 3: TCI state application latency has negligible impact to performance for known TCI states compared to DCI based TCI indication latency.

### Impact of Measurement Periodicity

Next, for a fixed application latency of 3ms, we compare the performance for different measurement periodicities. The fast fading RSRP trace with and without UE rotation is shown in Figure 4. In this evaluation we consider a measurement periodicity of 20ms as the ideal case and upper bound to the RSRP performance.

Figure 10: Fast Fading RSRP trace for different measurement periodicities. It is seen that measurement periodicity of 160ms significantly degrades performance even without UE rotation.

Figure 11: (L) Difference in median RSRP across the trajectory with the ideal case of 20ms periodicity. (R) The percentage of TTIs along the trajectory of the UE where 20ms periodicity has higher RSRP than the respective measurement periodicities.

From the above figures, we can clearly see that even a measurement periodicity of 80ms yields significant RSRP difference with the ideal case of 20ms measurement periodicity, while a 160ms periodicity leads to major performance degradation even without UE rotation

Observation 4: Beam acquisition latency due to measurement periodicity is a major performance bottleneck in current beam management.

### Performance of Hierarchical Beam Management

Next, we investigate the performance of hierarchical beam management schemes wherein the UE can perform initial beam acquisition through wide beam scanning and then refine the beams using narrow beam scanning in the direction of the acquired wide beams. In our evaluation, we model a mapping between narrow and wide beams based on circular cross correlation of the DFT analog beamforming vectors. For this evaluation, we assume that based on the results of the wide beam scan the gNB and UE can perform narrow Tx and Rx beam refinement on the narrow beams corresponding to best 8 wide beams. Compared to an exhaustive search of 64 Tx beams and 8 Rx beams across 3 UE panels, the hierarchical beam management scheme provides significant complexity and latency reduction. Note that for the hierarchical scheme simulated here, the UE performs panel selection based on the wide beam scanning.

Figure 12: RSRP trace for hierarchical beam management with wide and narrow beam scanning. The exhaustive search scheme performs narrow beam scanning only.

From Figure 6, we can see that hierarchical beam management has almost identical performance as the exhaustive narrow beam search scheme with respect to narrow beam RSRP even with UE rotation. Since hierarchical beam management brings significant latency and complexity reduction due to smaller number of beam measurements, it motivates us to propose further enhancements to the hierarchical beam management schemes to enable faster beam tracking and refinement.

Observation 5: Hierarchical beam management performs almost identical to exhaustive beam search thereby offering significant complexity and latency reduction.

Based on the simulation results in this section, we next propose latency reduction mechanisms for beam acquisition and beam refinement and tracking.

## Beam Acquisition Latency Reduction

From the simulation results in the previous section, it was seen that measurement periodicity is a major performance bottleneck in current beam management. In order to reduce the beam search latency particularly when a large number of TCI states are configured, some signaling enhancements can be additionally considered, wherein QCL relation between different SSB beams as well as QCL indication between SSB and CSI-RS resource sets can be enhanced.

As an example, for SSB based wide Tx beam acquisition, the UE can be signaled with beam relations between different SSB beams such that the Rx beam refinement can be handled without exhaustive beam search.

Figure 13: Beam acquisition enhancements

In Figure 4, the first 4 SSB can correspond to the red highlighted beams which roughly covers the overall coverage area of the gNB. Then the UE can further measure only those related TCI indexes and not scan the entire set of SSB. Such flexible indication can be beneficial especially when very large number of SSB beams need to be used. Additionally, QCL relation between SSB indices and the narrower CSI-RS beams within a CSI-RS resource set can also help the UE scan only a sub-set of CSI-RS beams for finer beam acquisition. Such QCL information between SSB and CSI-RS resource sets can be configured via MAC-CE. This can also potentially reduce the Tx beam acquisition latency.

If the UE is signaled QCL information between different SSB beams, the UE can potentially also utilize this information to perform Rx beam refinement across different SSBs which share neighboring beams. This could mean that the UE does not have to wait an entire SSB periodicity to complete Rx beam refinement and it can lead to significant latency reduction.

Proposal 22: To reduce beam acquisition latency QCL relations between different SSB beams can be indicated to the UE

Proposal 23: QCL relation between SSB and CSI-RS resource sets can be configured via MAC-CE signaling to further speed up narrow beam acquisition latency.

## Beam Refinement and Tracking Latency Reduction

In this section, we propose further enhancements for latency reduction for the hierarchical beam management framework which is based on wide SSB beam scanning and related refinement on narrower CSI-RS beams. The beam indication mechanism currently supports a combination of semi-static RRC signaling in conjunction with MAC-CE indication and finally further down selection via DCI. While the framework is flexible and the gNB is able to fully control the beam indication and configuration at the UE for both DL and UL, there is also a cost paid in terms of latency especially when TCI states need to switched multiple times. The overall signaling overhead and latency due to MAC-CE based activation is shown in Figure 1.

Figure 14: Static TX beam mapping for beam indication with MAC-CE based TCI update

From the figure it is evident the static beam mapping to resources requires TCI state update based on MAC-CE activation for change in TX beam or spatial relation information (or new UL TCI) update for uplink beam changes. It is possible to further refine the beam indication mechanism to reduce the latency of multiple TCI state switches and to minimize MAC-CE based TCI state updates as much as possible. To this end, we propose the following dynamic beam indication and tracking mechanisms wherein the process of beam refinement at gNB and UE can be combined with the help of flexible beam mapping.

Figure 15: CSI-RS and SRS based beam tracking

In the first method P3 and U3 procedures are used for beam tracking and updates, based assumptions of beam correspondence. In this case, the gNB initiates P3 based on CSI-RS with repetition. The UE uses this to update UE Rx (Tx) beam and then transmits SRS for U3 where the gNB updates Rx (Tx) beams. Based on this procedure, the gNB updates the CSI-RS Tx beam and the UE updates the Rx beam for TCI state and Tx beam spatial relation update. No additional signalling is needed to indicate the updated beam.

Figure 16: CSI-RS and CRI based beam tracking

In the second proposed method, P3 and P3 procedures are used. The gNB initiates CSI-RS based P2 procedure and the UE reports CRI after gNB Tx (Rx) beam acquisition. Based on this CRI, the gNB initiates P3 procedure using CSI-RS with repetition for the UE to update the Rx (Tx) beam. The UE is not expected to signal the choice of selected beam to gNB. These procedures make use of flexible beam mapping to the same TCI states and therefore reduce indication and tracking latency by foregoing the need for MAC-CE update of TCI states.

The SRS based scheme requires 1 DL-UL switch and the CRI based scheme requires 1 DL-UL and 1 UL-DL switch and avoids MAC-CE signaling. Scheduling offset for AP CSI-RS is required for P3 procedure in both the schemes. For the SRS based scheme, SRS-spatialrelationInfo is configured and PUSCH/PUCCH/SRS follows spatial filter of the SRS resource of configured spatial relation information. For the CRI based scheme, PUCCH/SRS-spatialRelationInfo is not configured. PUCCH/PUSCH/SRS follows the QCL-D of the TCI state of configured CORESET with lowest index.

For flexible beam tracking MAC-CE based update of TCI states can be performed. The update is based on hierarchical beam management where the UE measures L1-RSRP on SS/PBCH block and reports a SSB-RI. The gNB can update the active TCI state list of CSI-RS resource with the target SS/PBCH blocks. This can help the UE to keep track of the CSI-RS QCL Type D with respect to the SS/PBCH blocks without actually changing the TCI state.

Figure 17: TCI State update for CSI-RS for flexible beam mapping

Proposal 24: SRS/CRI based flexible beam mapping can be used to reduce beam refinement and tracking latency.

# Conclusion

In this paper, beam management enhancement for Rel-17 feMIMO has been discussed. The main proposals from this paper are outlined here:

Proposals:

- Joint DL/UL and separate UL TCI configuration should include the pathloss reference RS and UL power control parameters (P0, alpha, closed loop index) for PUCCH as a mandatory configuration.

- SRS and PUSCH pathloss reference RS ID can be optionally included in the TCI state configuration or updated by MAC-CE

- SSB is not required to be supported as an additional DL QCL source RS for the unified TCI framework.

- SRS for BM is not is not required to be supported as an additional DL QCL source RS for the unified TCI framework.

- DL or joint DL/UL TCI also applies to CSI-RS resources for CSI, CSI-RS resources for BM and CSI-RS for tracking

- Separate UL TCI states and joint DL/UL TCI states will share the same TCI state pool as DL TCI states

- A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI

- TCI state usage can be configured via MAC-CE where the type of TCI state configured to each codepoint is separately configured. The UE when indicated with a TCI codepoint associates the codepoint with joint DL/UL or separate DL or UL TCI state based on MAC-CE configuration.

- A shared RRC TCI state ID pool is supported for a set of configured CCs. The QCL Type A RS reference is implicitly determined based on the target CC of the TCI state.

- Separate QCL-TypeD RS can be determined from the common TCI state(s) for each of the CCs. The determined QCL-TypeD RSs for the set of CCs are further associated with a same QCL-TypeD RS.

- For DCI based TCI state indication associated with a non-serving cell for L1/L2 centric inter cell mobility, TA should be adjusted for UL transmission where the TAG could be associated with a PCID or a group of TCI states corresponding to a PCID.

- The scope enhancement for RAN1-led L1/L2 mobility is limited to introduction of PCID in the unified TCI state configuration or SSB configuration which serves as QCL source for a TCI state. No impact to L1 measurement or RRM/RLM is supported.

- The value of K non-serving beams which can be reported is based on UE capability

- Both serving and non-serving cell reports can be combined within a single CSI reporting instance

- L1-RSRP is used as the beam reporting metric and the reporting is done via existing L1-RSRP report

- DCI formats 1_1 and 1_2 without DL assignment are supported for joint TCI as well as separate DL /UL TCI along with a DCI acknowledgment mechanism

- For separate UL beam indication using UL TCI state

- When current DCI formats are re-used, the TCI state is applicable to all UL signals and channels

- UL DCI formats 0_1, 0_2 can be used with SRI field being reinterpreted as the TCI codepoint in DCI.

- DL DCI formats 1_1,1_2 can be used with implicit indication of UL TCI using TCI in DCI field where UE maps the indicated codepoints to UL TCI states based on the MAC-CE activation of TCI states in DCI codepoints

- Consider NR MBS framework for group-common DCI transmission to a UE group for joint TCI state activation using a new beam indication RNTI for scrambling the CRC of the group-common DCI.

- For DCI based indication, a new DCI format should be considered with the capability to dynamically indicate individually, the channels and RSs to which the TCI state is applicable. Additionally, both common and separate beam indication should be supported.

- For DCI based beam indication, the beam application time is the first slot that is at least X ms or Y symbols after the starting symbol of the PUCCH resource carrying the acknowledgment of the joint or separate DL/UL beam indication.

- For MPE mitigation for Rel-17,

- RAN4 solution for MAC-CE based P-MPR reporting is augmented with MPE recovery information indicating alterative UL beam with lower P-MPR or duty cycle constraints, where UE panel indication can be implicit.

- No additional reporting is needed since the gains compared to DL based UL panel selection are not very large.

- To reduce beam acquisition latency QCL relations between different SSB beams can be indicated to the UE

- QCL relation between SSB and CSI-RS resource sets can be configured via MAC-CE signaling to further speed up narrow beam acquisition latency

- SRS/CRI based flexible beam mapping can be used to reduce beam refinement and tracking latency

# References

- RP-193133, New WID: Further Enhancements on MIMO for NR, Samsung, 3GPP TSG RAN #86, Sitges, Spain, December 2019.

- R1-2007151, Moderator summary for multi-beam enhancement: EVM, Samsung, RAN1#102-e.

- R4-2011734, WF on MPE Enhancements, Oppo, RAN4#96e, August 2020.

- R4-2011733, LS in MPE Enhancements, Nokia, RAN4#96e, August 2020.

- 3GPP TS 38.133 v16.4.0, Requirements for Support of Radio Resource Management (Release 16)

- 3GPP TS 38.211 v16.2.0, NR Physical Channels and Modulation (Release 16)

- 3GPP TS 38.212 v16.2.0, NR Multiplexing and Channel Coding (Release 16)

- 3GPP TS 38.213 v16.2.0, NR Physical Layer Procedures for Control (Release 16)

- 3GPP TS 38.214 v16.2.0, NR Physical Layer Procedures for Data (Release 16)

- IST-WINNER II Deliverable 1.1.2 v.1.2, "WINNER II Channel Models", IST-WINNER2, Tech. Rep., 2007 (http://www.ist-winner.org/deliverables.html).

- 3GPP TR 38.901 v16.1.0, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

- 3GPP TR 36.839 v11.1.0, Mobility enhancements in heterogeneous networks (Release 11)

- RP-192227, Revised WID on NR RF Requirement Enhancements for FR2, Newport Beach, US, Sept 16-20, 2019 (revision of RP-191290).

- R4-1913057 WF on FR2 MPE Enhancement, Qualcomm, 3GPP TSG-RAN WG4 Meeting #92-Bis, Chongqing, China, 14-18 Oct 2019.

# Appendix: SLS Evaluation Assumptions

The general evaluation assumptions agreed in [2] are used. The following are a few specific details considered in our evaluations.

| Simulation Parameters | Dense Urban Macro, Indoor Hotspot |
| --- | --- |
| Carrier Frequency | 30GHz |
| Simulation BW | 50MHz |
| Sub-carrier Spacing and Slot Length | 120 kHz SCS, 14 OFDM Symbol slot |
| Channel Model | 5G-UMA (LOS), InH |
| Inter-Site Distance | UMA = 200m, InH = 20m |
| BS Antenna Configuration | Narrow Beam - (M, N, P, Mg, Ng) = (4,8,2,1,1), 64 DFT Beams (2x oversampling in horizontal domain) Wide Beam - (M, N, P, Mg, Ng) = (2,4,2,1,1), 32 DFT Beams (2x oversampling in horizontal and vertical domains) |
| UE Antenna Configuration | Narrow Beam - (M, N, P, Mg, Ng) = (1,4,2,1,3): 2 Panels (0°,90°, 270°) 8 DFT Beams per panel (2x horizontal oversampling) Wide Beam - (M, N, P, Mg, Ng) = (1,2,2,1,3): 2 Panels (0°,90°, 270°) 4 DFT Beams per panel (2x horizontal oversampling) UE front panel points towards the direction of motion |
| BS Height | UMA - 20m, InH – 3m Ceiling Mounted |
| UE Deployment | 1 outdoor UE @ 120 kmph |
| Latency Impairment Modeling | UE Beam Acquisition Latency = (20,40,80,160) ms UE Beam Application Latency = (1,3) ms |
| UE Rotation | X RPM Rotation along the β-axis of the LCS with X = 0,1 |
| UE Trajectory Sampling | Linear Trajectory Sampled every 160 slots |
| Spatial Consistency Modelling | Decorrelation distance-based Autocorrelation for LSP Spatial Consistency Model B from TR38.901 for SSP |