3GPP TSG- RAN WG1 Meeting #104b-e                                                 R1-2103090

# e-Meeting, April 12th – 20th, 2021

Agenda Item:	8.1.2.2

Source:	Apple Inc.

Title:	On Inter-Cell Multi-TRP Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #104 meeting, the following agreement on inter-cell multi-TRP operation have been achieved.

Agreement Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation - FFS: Whether the indication of PCI is implicit or explicit Conclusion Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel. Agreement At least following non-serving cell SSB information are needed in inter-cell MTRP operation - SSB time domain position - SSB transmission periodicity - SSB transmission power FFS: Other non-serving cell information FFS: Whether indication of these information is implicit or explicit Agreement For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e - Option1: Indicate/associate non-serving cell PCI in the TCI state - FFS other non-serving cell information - Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell - FFS: how the flag is linked to non-serving cell - Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively. - FFS: Each group is associated with a CORESETPoolIndex value. - FFS: how to link the group of TCI states to non-serving cell. - Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, … - FFS: detailed re-indexing rule(s) of non-serving cell RSs - Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with - FFS: how the indicator is linked to non-serving cell - Note: when there is only one non-serving cell, it means the same as Option2. Agreement Agree on scheme1 - Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI - FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB - FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB Conclusion The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2. - Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.

In this contribution, we provide some discussion on inter-cell multi-TRP enhancement.

# Discussion

## Details on configuration of non-serving cell RS

In RAN1 #104, the following 5 options on TCI and non-serving cell RS association are agreed for further down-selection.

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - FFS: how the flag is linked to non-serving cell

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - FFS: Each group is associated with a CORESETPoolIndex value.

  - FFS: how to link the group of TCI states to non-serving cell.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

  - FFS: detailed re-indexing rule(s) of non-serving cell RSs

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - FFS: how the indicator is linked to non-serving cell

  - Note: when there is only one non-serving cell, it means the same as Option2.

Option 1 is to indicate a PCI in the TCI state. But SSB can only be provided in the TCI for CSI-RS, and this may be with higher overhead, since only few cells may be enabled for multi-TRP operation. Option 2, 3 and 5 are quite similar, and the only difference is whether the indication is based on a kind of 1-bit flag or group index or a multi-bit indicator. Option 4 may need additional signalling to maintain the variable N, and may not have a good forward compatibility to support more than 2 TRP.

Proposal 1: For inter-cell multi-TRP operation, support option 2/3/5 to define the association between TCI and non-serving cell information, where an indicator can be used to provide the linkage between non-serving cell information and a TCI

- The TCI with the same indicator should be associated with the same CORESETPoolIndex

## Rate matching

In RAN1 #104, it was agreed that PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI. Thus, the remaining issue is whether the cross-cell rate matching is needed. This should depend on whether UE uses advanced receiver for interference cancellation. Since the non-serving cell SSB may be from different beams and some may create strong interference to the UE. So, it is better to introduce a UE capability on whether the cross-cell rate matching is needed.

Proposal 2: Support to introduce a UE capability to report the following information

- Whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- Whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

# Conclusion

In this contribution, we discussed inter-cell multi-TRP enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: For inter-cell multi-TRP operation, support option 2/3/5 to define the association between TCI and non-serving cell information, where an indicator can be used to provide the linkage between non-serving cell information and a TCI

- The TCI with the same indicator should be associated with the same CORESETPoolIndex

Proposal 2: Support to introduce a UE capability to report the following information

- Whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- Whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB