3GPP TSG RAN WG1 #104b-e 	R1-2103196

e-Meeting, April 12th – 20th, 2021

Agenda item:	7.2.11

Source: 	Qualcomm Incorporated

Title: 	Discussion on NR Rel-16 UE features

Document for:	Discussion/Decision

- Introduction

In this contribution, we discuss and provide proposals on the remaining open issues in the NR Release-16 UE feature list [1].

- Discussion

# 2.1 NR Others: FGs that are not dedicated to a specific Rel-16 work item/TEI

The following agreement was reached in RAN1 #102-e meeting with regards to the simultaneous transmission of SRS Ant. Sw. with other SRS resources.

Agreement - In UL CA, for SRS for antenna switching + SRS for CB/NCB /BM/antenna switching case, the simultaneous transmission of SRS on different CCs is subject to UE FG 22-5a/22-5b/22-5c/22-5d - New UE FG 22-5a/22-5b/22-5c/22-5d are introduced - Note: For simultaneously Ant.Sw . + Ant.Sw SRS in intra-band CA, or in inter-band CAs with bands whose UL are switched together according to the reported UE capability, the UE expects the same configuration of xTyR across the different CCs and the SRS resources overlapped in time domain from UE perspective are from the same UE antenna ports. - Note: In UL CA, for SRS for CB/NCB /BM + SRS for CB/NCB /BM case, the simultaneous transmission of SRS on different CCs are supported in the supported combinations subject to UE capability in Rel-15 - No spec impact for this - Note: different spatial relation for SRS +SRS is a separate issue

Even though the new FGs were added in the LS to RAN2 [2], the following Note from the above agreement was not captured in the 38.306 specification:

  - Note: For simultaneously Ant.Sw . + Ant.Sw SRS in intra-band CA, or in inter-band CAs with bands whose UL are switched together according to the reported UE capability, the UE expects the same configuration of xTyR across the different CCs  and the SRS resources overlapped in time domain from UE perspective are from the same UE antenna ports.

It should be noted that this Note is important for both UE and gNBs to have a common understanding on what is expected from a UE that supports this feature. Without the note, there is a risk of misconfiguring the UEs with Ant Switching configurations which are not compatible.

Proposal 1: Send an LS to RAN2 to add in the description of FG 22-5c and 22-5d the following note:

- For simultaneously Ant.Sw . + Ant.Sw SRS in intra-band CA, or in inter-band CAs with bands whose UL are switched together according to the reported UE capability, the UE expects the same configuration of xTyR across the different CCs  and the SRS resources overlapped in time domain from UE perspective are from the same UE antenna ports.

- References

[1] R1-2102007 “LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#104-e“ Moderators (AT&T, NTT DOCOMO, INC.)

[2] R1-2102007 “Updated RAN1 UE features list for Rel-16 NR including remaining RAN1 issues“ Moderators (AT&T, NTT DOCOMO, INC.)