3GPP TSG RAN WG1 #104bis-e	R1-2103285

e-Meeting, April 12th - April 20th, 2021

Title:	LS on simultaneous UL transmissions for DAPS-like solution in IAB

Source: 	ZTE, Sanechips

Agenda Item:	5

Document for:	Discussion and Decision

Introduction

RAN3 sent to RAN2 an LS on DAPS-like solution for service interruption reduction [1], and RAN2 sent an reply LS[2] with a recommendation that RAN1 should be consulted as to whether simultaneous UL transmissions can be supported in Rel-17. The details of the reply LS are given below.

RAN2 would like to thank RAN3 for the LS on DAPS-like solution for service interruption reduction in Rel-17 IAB. RAN2 discussed whether simultaneous UL transmission can be supported in Rel-17 for IAB. RAN2 understanding is that in Rel.16 DAPS HO, simultaneous transmissions of UL new data in source and target cell is not supported. However, RAN2 did not understand what is intended for “DAPS-like” solution in the RAN3 LS, and what should be achieved with that. RAN2 also believe that RAN1 should be consulted as to whether simultaneous UL transmissions can be supported in Rel-17 from their point of view.

This contribution shares our views on  simultaneous UL transmissions for IAB, and make some clarification from RAN1’s perspective.

Discussion

According to the follow text from TS38.213 sector 15, simultaneous UL transmissions in Rel-16 DAPS HO is not supported for the listed cases.

# 15 Dual active protocol stack based handover >>>Non-relevant contents skipped For DAPS handover that is not intra-frequency, if - the UE does not indicate support of ul-TransCancellationDAPS, and - UE does not indicate a capability for power sharing between source and target MCG in DAPS handover or the UE is not provided with uplinkPowerSharingDAPS-Mode, the UE does not expect transmissions on the target and source cell in overlapping time resources. For DAPS handover that is not intra-frequency, if - the UE indicates support of ul-TransCancellationDAPS, and - UE does not indicate a capability for power sharing between source and target MCG in DAPS handover or the UE is not provided with uplinkPowerSharingDAPS-Mode, and - UE transmissions on the target cell and the source cell are in overlapping time resources, the UE transmits only on the target cell, and cancels the transmission to source cell For intra-frequency DAPS handover, if - UE transmissions on the target cell and the source cell are in overlapping time resources, the UE transmits only on the target cell and cancels the transmission on the source cell

Observation 1: UE does not support simultaneous UL transmission for the following cases:

- Intra-frequency DAPS handover.

- DAPS handover that is not intra-frequency and UE doesn’t have the capability of power sharing between source and target MCG.

From RAN1’s perspective, IAB MT should follow the similar UL transmission behavior as UE for DAPS HO or DAPS-like solution. And simultaneous UL transmission is only feasible for inter-frequency cases and with a condition that UL power sharing is also supported between the target and source MCG.

Based on the above analysis, we propose the following:

Proposal 1: If necessary, to clarify the following to RAN3/RAN2:

- Intra-frequency simultaneous UL transmission is not supported.

- Inter-frequency simultaneous UL transmission is also not supported if UL power sharing is not supported.

Conclusion

In this contribution, we have discussed the RAN2 LS on simultaneous UL transmissions in IAB in IAB. Based on this, the following proposal is provided:

Proposal 1: If necessary, to clarify the following to RAN3/RAN2:

- Intra-frequency simultaneous UL transmission is not supported.

- Inter-frequency simultaneous UL transmission is also not supported if UL power sharing is not supported.

Reference

- R3-207184, LS on DAPS-like solution for service interruption reduction in Rel-17 IAB, RAN3, Samsung

- R2-2102364, Reply LS on DAPS-like solution for service interruption reduction, RAN2, Ericssion