## Study on XR Evaluations for NR (RAN1)

Please refer to RP-201145 for detailed scope of the WI

R1-2103190	Updated Work Plan for Rel-17 SI on XR Evaluations for NR	Qualcomm Incorporated

R1-2103191	Updated TR for Study on XR Evaluation	Qualcomm Incorporated

Late submission

//This one is to use NWM – please use RAN1-104b-e-NWM-NR-XR-04 as the document name

[104b-e-NR-XR-04] Email discussion/approval whether/how to reply LS R1-2102308 till 4/16. To be managed under 8.14 (Eddy, Qualcomm)

### Traffic Model

R1-2102320	Traffic model for XR and Cloud Gaming	Huawei, HiSilicon

R1-2102418	Discussion on the XR traffic models for evaluation	OPPO

R1-2102546	Discussion on traffic models of XR	vivo

R1-2102616	XR traffic model	CATT

R1-2102686	Traffic Model for XR and CG	MediaTek Inc.

R1-2102769	XR traffic model	FUTUREWEI

R1-2102827	On Traffic Model for XR study	Nokia, Nokia Shanghai Bell

R1-2102955	Traffic model for XR	Ericsson

R1-2102969	Discussion on Traffic Model for XR services	Xiaomi

R1-2103054	Traffic Model for XR	Intel Corporation

R1-2103128	Views on XR traffic model	Apple

R1-2103192	Remaining Issues on XR Traffic Models	Qualcomm Incorporated

R1-2103264	Traffic model for XR	Samsung

R1-2103278	Further Discussion on Traffic Model for XR Evaluations	ZTE, Sanechips

R1-2103317	Considerations on XR traffic model	Sony

R1-2103360	Discussion on traffic models for XR evaluation	LG Electronics

R1-2103429	UL traffic flows for XR applications	InterDigital, Inc.

R1-2103437	XR Traffic Model Considerations	AT&T

R1-2103598	Discussion on traffic model for XR	NTT DOCOMO, INC.

[104b-e-NR-XR-01] Email discussion/approval on traffic model with checkpoints for agreements on Apr-15, Apr-20 – Eddy (Qualcomm)

Proposal:

Parameters of Truncated Gaussian distribution for packet size of DL video stream in case of single stream evaluation (note: these parameter values are those before the truncation)

- Mean packet size: Derived from average data rate and fps as follows.

  - (average data rate) / (fps for video stream, i.e., # packets per second in our statistical model) / 8 [bytes]

- STD: 10.5% of Mean packet size

- Max: 150% of Mean packet size

- Min: 50% of Mean packet size

Other parameter values for STD, Max, Min can be optionally evaluated.

Agreement:

Jitter for DL video stream for the case of a single stream per UE

- J is drawn from a truncated Gaussian distribution:

  - Mean: 0 ms

  - STD: 2 ms

  - Range: [-4, 4] ms (baseline), [-5, 5] ms (optional)

    - Note: The values are set to ensure that packet arrivals are in order (i.e., arrival time of next packet is always larger than that of the previous packet) rather than the real measurement

  - Other values can be optionally evaluated

- Note: The above parameters for random variable J are effectively identical to the following parameter values because air interface PDB (e.g., 10ms or 15ms) applies from the point when each packet arrives at gNB as agreed in RAN1#104-e.

  - Mean: 4 ms (baseline), 5ms (optional)

  - STD: 2 ms

  - Range: [0, 8] ms (baseline), [0, 10] ms (optional)

  - Other values can be optionally evaluated

### Evaluation Methodology

Including identification of KPIs of interest for relevant deployment scenarios

R1-2102321	Evaluation methodology for XR and Cloud Gaming	Huawei, HiSilicon

R1-2102419	Discussion on the XR evaluation methodology	OPPO

R1-2102547	Discussion on evaluation methodologies of XR	vivo

R1-2102613	Evaluation methodology and performance index for XR	CATT

R1-2102687	On Evaluation Methodology for XR and CG	MediaTek Inc.

R1-2102770	XR evaluation methodology	FUTUREWEI

R1-2102828	Development of the Evaluation Methodology for XR Study	Nokia, Nokia Shanghai Bell

R1-2102956	Evaluation methodology for XR	Ericsson

R1-2102970	Discussion on evaluation methodology for XR services	Xiaomi

R1-2103055	Evaluation Methodology	Intel Corporation

R1-2103129	Views on XR evaluatoin methodology	Apple

R1-2103193	Remaining Issues on Evaluation Methodology for XR	Qualcomm Incorporated

R1-2103265	Evaluation methodology and KPIs for XR	Samsung

R1-2103279	On XR Evaluation Methodology	ZTE, Sanechips

R1-2103361	Discussion on evaluation methodologies for XR	LG Electronics

R1-2103430	Remaining Issues on XR Evaluations and KPIs	InterDigital, Inc.

R1-2103438	XR Evaluation Assumptions	AT&T

R1-2103599	Discussion on evaluation methodology for XR	NTT DOCOMO, INC.

R1-2103827	Summary of email discussion for evaluation methodology and assumptions	Moderator (vivo)

[104b-e-NR-XR-02] Email discussion/approval on evaluation methodology with checkpoints for agreements on Apr-15, Apr-20 – Xiaohang (Vivo)

Capacity

Proposal:

For XR/CG capacity evaluation, DL and UL performances are evaluated independently, and the system capacity for DL capacity and UL capacity are reported respectively.

- FFS: How to determine the joint capacity for DL and UL after companies have submitted evaluation results

Possible proposal 2: No need to discuss which option is mandatory or optional. It is up to companies to choose the TDD configuration and do the evaluation.

Power consumption

Agreement:

Case 2, i.e. CDRX, is optionally evaluated for UE power consumption evaluation

Possible proposal 5:

- Power saving gain is evaluated using power saving scheme e.g. CDRX, over the baseline scheme i.e. UE-always-ON

Agreement:

For XR power consumption evaluation, CDRX parameters are reported by companies

Possible proposal 7:

For UL UE power consumption evaluation, Option 2 i.e., linear interpolation is the baseline

- Option 1 can be evaluated optionally

### Initial Performance Evaluation Results

R1-2102322	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

R1-2102420	Initial performance results for XR evaluation	OPPO

R1-2102548	Initial performance evaluation results of XR	vivo

R1-2102614	Evaluation results of XR performance	CATT

R1-2102707	8.14.3  Initial Performance and Evaluation Results for XR and CG	MediaTek Inc.

R1-2102771	XR initial evaluations	FUTUREWEI

R1-2102829	Performance results in indoor hotspot and dense urban deployments of CG and VR/AR applications		Nokia, Nokia Shanghai Bell

R1-2102904	Initial XR Evaluation Results	CMCC

R1-2102957	Initial XR performance evaluation results	Ericsson

R1-2103130	Initial performance evaluation on XR	Apple

R1-2103194	Initial Evaluation Results for XR Capacity and UE Power Consumption	Qualcomm Incorporated

R1-2103280	Initial Performance Evaluation Results for XR	ZTE, Sanechips

R1-2103431	Performance Evaluation Results on XR applications	InterDigital, Inc.

R1-2103439	XR Initial Performance Results	AT&T

[104b-e-NR-XR-03] Email discussion/approval on initial performance evaluation results with checkpoints for agreements on Apr-15, Apr-20 – Eddy (Qualcomm)

### Others

R1-2102421	Discussion on the  support of XR/CG service in sidelink-unlicensed	OPPO

R1-2102549	Challenges and potential enhancements of XR	vivo

R1-2102615	Potential area of NR enhancement for the support of XR services	CATT

R1-2102830	Potential enhancements for better XR support over NR	Nokia, Nokia Shanghai Bell

R1-2102958	Mobility and XR applications	Ericsson

R1-2102971	Discussion on potential enhancement for supporting XR	Xiaomi

R1-2103131	Views on potential enhancements for XR	Apple

R1-2103195	Potential Enhancements for XR	Qualcomm Incorporated

R1-2103281	Considerations on XR Specific Enhancement	ZTE, Sanechips

R1-2103390	Challenges and potential enhancements for XR and Cloud Gaming	Huawei, HiSilicon

R1-2103432	Potential enhancements for supporting XR	InterDigital, Inc.