3GPP TSG RAN WG1 Meeting #104bis-e		R1-210xxxx

e-Meeting, April 12th – April 20th, 2021

Title:	Session notes for 8.10

(Enhancements to Integrated Access and Backhaul)

Source:	Ad-hoc chair (Samsung)

## Enhancements to Integrated Access and Backhaul

Please refer to RP-210758 for detailed scope of the WI

R1-2103182	Workplan for Rel-17 IAB	Qualcomm Incorporated

### Enhancements to resource multiplexing between child and parent links of an IAB node

R1-2102340	Resource multiplexing between backhaul and access for IAB duplexing enhancements	Huawei, HiSilicon

R1-2102538	Enhancement to resource multiplexing between child and parent links	vivo

R1-2103046	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

R1-2103120	Enhanced resource multiplexing within an IAB node	Apple

R1-2103183	Resource management for enhanced duplexing	Qualcomm Incorporated

R1-2103255	Enhancements to Resource Multiplexing for NR IAB	Samsung

R1-2103330	Discussion on IAB resource multiplexing enhancements	ETRI

R1-2103374	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node		CEWiT

R1-2103435	Enhancements for resource multiplexing among IAB backhaul and access links	Nokia, Nokia Shanghai Bell

R1-2103436	Enhancements for IAB resource multiplexing	AT&T

R1-2103497	Enhancements to the IAB resource multiplexing	ZTE, Sanechips

R1-2103591	Resource multiplexing between child and parent links of an IAB node	NTT DOCOMO, INC.

R1-2103608	Resource multiplexing in enhanced IAB systems	Lenovo, Motorola Mobility

R1-2103629	Discussions on IAB resource multiplexing enhancements	LG Electronics

R1-2103712	Resource multiplexing and DC in enhanced IAB	Ericsson

/This one is to use NWM – please use RAN1-104b-e-NWM-NR-eIAB-01 as the document name

[104b-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: April 15

- 2nd check point: April 20

R1-2103837	Summary of [104b-e-NR-eIAB-01] – Kickoff	Moderator (AT&T)

Agreement

The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported

Agreement

Adaptation of an IAB-node’s multiplexing operation is supported. The adaptation may be based on multiple factors, for example (not necessary to support all of the following):

- Resource type (D/U/F) at the IAB-DU and IAB-MT

- Specific sets of time/frequency resources

- Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.)

FFS:  Mechanisms for informing/coordination the change in multiplexing operation(s) between child and parent nodes (including whether the adaptation is dynamic or semi-static)

FFS: Need for explicit linkage between indicated multiplexing operations and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported)

Agreement

For the semi-static DU resource configuration in the frequency domain within a carrier, the frequency-domain granularity is configurable

- FFS:  minimum resource size e.g. N PRBs/N RBGs

- FFS: Separate or joint TDM and FDM semi-static DU resource configurations

Agreement

Soft resource availability indications for frequency-domain resources are supported

- FFS enhancements to DCI Format 2_5

- FFS: Separate or joint TDM and FDM indications

R1-2103958	Summaryof[104b-e-NR-eIAB-01]	Moderator (AT&T)

Agreement

To facilitate simultaneous operations and interference management, dynamic indication for restriction/usage/availability of beams (in upstream and/or downstream directions) is supported

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Whether IAB-specific enhancements beyond the existing beam management framework are needed to the support the functionality

- FFS: Impact on the semi-static resource configurations (e.g., extending the H/S/NA resource attributes to the spatial domain)

- FFS: Whether panel-based granularity is additionally supported

Agreement

The following enhancements to support intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios are considered (in addition to reusing solutions for inter-band dual connectivity) to support simultaneous Tx and/or Rx at the child IAB-MT to/from both parent links:

- Extending the Rel-16 CA TDD conflict resolution framework for synchronous intra-band NR-DC operation

- Coordinating TDD configurations for the parent nodes (for both intra-donor and inter-donor operation) and coordinating H/S/NA configurations for the child node between donors (at least for inter-donor operation)

### Other enhancements for simultaneous operation of IAB-node’s child and parent links

Including IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed

Void (not be handled during this e-meeting). No contributions please.

R1-2103047	Other Enhancements for Simultaneous Operations	Intel Corporation

### Others

Void (not be handled during this e-meeting). No contributions please.