3GPP TSG RAN WG1 Meeting #104bis-e		R1-210xxxx

e-Meeting, April 12th – 20th, 2021

Title:	Session notes for 8.9 (Rel-17 enhancements for NB-IoT and LTE-MTC)

Source:	Ad-hoc chair (Samsung)

## Rel-17 enhancements for NB-IoT and LTE-MTC

Please refer to RP-201306 for detailed scope of the WI

R1-2103763	Work plan of Rel-17 enhancements for NB-IoT and LTE-MTC	Huawei, Ericsson

### Support of 16-QAM for unicast in UL and DL for NB-IoT

R1-2102357	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

R1-2102652	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

R1-2102680	Switching point between QPSK and 16QAM and DCI design for 16QAM in R17	MediaTek Inc.

R1-2102857	Discussion on UL and DL 16QAM for NB-IoT	ZTE

R1-2103067	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

R1-2103531	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

R1-2103723	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

[104b-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: April 15

- 2nd check point: April 20

R1-2103853

Agreement

Confirm the working assumption that the following TBS indices are introduced for downlink with modification in RED:

| ITBS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 14 | 256 | 552 | 840 | 1128 | 1416 | 1736 | 2280 | 2856 |
| 15 | 280 | 600 | 904 | 1224 | 1544 | 1800 | 2472 | 3112 |
| 16 | 328296 | 632 | 968 | 1288 | 1608 | 1928 | 2600 | 3240 |
| 17 | 336 | 696 | 1064 | 1416 | 1800 | 2152 | 2856 | 3624 |
| 18 | 376 | 776 | 1160 | 1544 | 1992 | 2344 | 3112 | 4008 |
| 19 | 408 | 840 | 1288 | 1736 | 2152 | 2600 | 3496 | 4264 |
| 20 | 440 | 904 | 1384 | 1864 | 2344 | 2792 | 3752 | 4584 |
| 21 | 488 | 1000 | 1480 | 1992 | 2472 | 2984 | 4008 | 4968 |

Agreement

Confirm the working assumption:

- For standalone and guardband deployments, the downlink TBS entries between 14 (TBS of 2856 for I_SF=7) and 21 are used for 16QAM.

Agreement

For both uplink and downlink

- 16-QAM is not applied to C-RNTI from CSS.

- 16-QAM is not applied to EDT.

Working Assumption

The DCI size is not increased to support 16-QAM in uplink and downlink.

For comeback

Proposal 1: It is down-selected from following options on the indication of downlink 16-QAM:

- Option 1: MCS is increased to 5 bits, and repetition field is reduced by 1 bit;

- Combination of Option 2, 3, 4: MCS is 4 bits and 16QAM is indicated by a combination of the MCS field and repetition field

Possible Agreement

If 16-QAM is configured for NPDSCH, the channel quality report for 16-QAM is the MCS index associated to an NPDSCH transport block that achieves an error probability not exceeding 10% BLER.

### Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

R1-2102358	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

R1-2102653	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

R1-2102858	Remaining issues on 14-HARQ processes in DL for eMTC	ZTE

R1-2103068	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

R1-2103464	Design considerations to support 14-HARQ Feature for LTE-M	Sierra Wireless, S.A.

R1-2103724	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

[104b-e-LTE-Rel17_NB_IoT_eMTC-02] Email discussion on support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC – Gerardo (Ericsson)

- 1st check point: April 15

- 2nd check point: April 20

3859

Agreement

In Rel-17, for the 14 HARQ processes feature, PUCCH repetition is not supported with HARQ-ACK bundling.

Possible Agreement

On HARQ-ACK delay is determination

Alt-1: The HARQ-ACK delay is determined through an expression consisting of different subframe types (Using a similar principle as the PDSCH scheduling delay).

Solutions according with Alt-1 can be found in:

- [2] Proposal 1, Option 2.

- [3] Proposal 1.

Alt-2: The HARQ-ACK delay is determined following the legacy approach. That is, the “HARQ-ACK delay” is kept expressed in terms of “absolute subframes”.

Solutions according with Alt-2 can be found in:

- [2] Proposal 1, Option 1.

- [6] Proposal 3.

- [7] Proposal 1.

- [4] Proposal 3.

Work in RAN1#104bis-e to clarify Alt-2

Conclusion

In Rel-17, the 14 HARQ processes feature is not supported when the multi-TB grant feature is enabled.

### Support a maximum DL TBS of 1736 bits as a Rel-17 optional UE capability

For HD-FDD Cat. M1 UEs in CE mode A only

R1-2102359	Support of a max DL TBS of 1736 bits in LTE-MTC	Huawei, HiSilicon

R1-2102654	Support of a maximum DL TBS of 1736 bits for eMTC	Nokia, Nokia Shanghai Bell

R1-2102859	Remaining issues on DL TBS increase for eMTC	ZTE

R1-2103069	Support of larger TBS for eMTC	Qualcomm Incorporated

R1-2103313	Remaining issues for support of DL TBS of 1736 bits for eMTC	Sony

R1-2103462	Design considerations to support DL TBS of 1736 bits for LTE-M	Sierra Wireless, S.A.

R1-2103725	Support of a maximum DL TBS of 1736 bits in LTE-MTC	Ericsson

//This one is to use NWM – please use RAN1-104b-e-NWM-LTE-Rel17_NB_IoT_eMTC-03 as the document name

[104b-e-LTE-Rel17_NB_IoT_eMTC-03] Email discussion on support of a max TBS of 1736 bits – Martin (Sony)

- 1st check point: April 15

- 2nd check point: April 20

### Others

Void (not be handled during this e-meeting). No contributions please.