3GPP TSG RAN WG1 #105-e	                        	      R1-2104223

e-Meeting, May 10th – May 27th, 2021

Source: 	ZTE, Sanechips

Title:	Additional simulation results of UE power consumption in RRC idle and inactive state

Agenda Item:	8.7.1.3

Document for:	Discussion and Decision

Introduction

In previous meetings, some potential paging enhancement schemes such as PEI (paging early indication), sub-grouping for paging were discussed. PEI can be used to inform UE whether the UE need to monitor the paging DCI or not in the subsequent PO. If the PEI indicates “no paging message”, UE can skip the unnecessary operation including PDCCH/PDSCH reception and SSB processing. In this contribution, more simulation results are provided for different paging enhancement schemes.

Discussion on performance and power saving gain of PEI

Performance of paging PDCCH/PDSCH and PEI

In RAN1 #104-e meeting, UE behavior of Behv-A and Behv-B when the PEI is not detected were discussed. For each behavior, the performance evaluation methodology of PEI candidate based on PDCCH, TRS/CSI-RS and SSS is provided. With the link level simulation assumptions in Table A1, the following sections describe the performance analysis of several PEI candidates with Alt 1 in [1].

DCI-based PEI with SCL decoding

The joint miss-detection rate (JMDR) performance of DCI-based PEI with SCL decoding and paging PDCCH/PDSCH is shown in Figure 1.

(a) Behv-A                                          (b) Behv-B

Figure 1 JMDR performance of DCI-based PEI (SCL decoding) and paging PDCCH

As shown in the Figure 1(a), for PDSCH with TB scaling 1, the JMDR of DCI-based PEI with AL 4 and paging PDCCH fulfills the performance requirement. For PDSCH with TB scaling 0.5, the DCI-based PEI with AL 8 can fulfill the performance requirement.

For the JMDR in the case of Behv-B, since the FAR of the DCI-based PEI is extremely small (i.e., 2-21), the JMDR performance of the DCI-based PEI is almost the same as the MDR performance of the paging PDCCH.

According to Figure 1(b), we can see that for PDSCH with TB scaling 1, the JMDR of DCI-based PEI with AL 4 and paging PDCCH fulfill the performance requirement. The required SNR of DCI-based PEI with AL 4 is smaller than the required SNR of PDSCH with TB scaling 1 when MDR equals to 1%. And the MDR performance and JMDR performance of DCI-based PEI with AL 8 can fulfill the performance requirement by PDSCH with TB scaling 0.5.

For Behv-A/B, DCI-based PEI with AL 4 fulfills the performance required by paging PDSCH without TB scaling. DCI-based PEI with AL 8 can fulfill the performance requirement by PDSCH with TB scaling 0.5.

DCI-based PEI with ML decoding

The ML decoding can effectively improve the decoding performance of DCI and has little impact on complexity. With ML decoding algorithm, the MDR and JMDR performance of DCI-based PEI are shown in Figure 2.

(a) Behv-A                                          (b) Behv-B

Figure 2 JMDR performance of DCI-based PEI and paging PDCCH (ML decoding)

As shown in the Figure 2, with ML decoding, DCI-based PEI with AL 2 fulfills the performance requirement of TB scaling 1 and DCI-based PEI with AL 4 fulfills the performance requirement of TB scaling 0.5 in the case of Behv-A and Behv-B. What’s more, when the information bits of the DCI-based PEI are reduced, the MDR performance of DCI-based PEI can be further improved.

With ML decoding, DCI-based PEI with AL 2 fulfills the performance requirement of TB scaling 1 and DCI-based PEI with AL 4 fulfills the performance requirement of TB scaling 0.5 in the case of Behv-A and Behv-B.

SSS-like PEI

For Behv-A, the JMDR performance of SSS-like PEI with FAR 0.1% and paging PDCCH is shown in Figure 3. For the one-symbol SSS-like PEI, 127REs are assumed. For the two-symbol SSS-like PEI, 254 REs are assumed.

Figure 3 JMDR performance of SSS-like PEI and paging PDCCH - Behv-A

According to Figure 3, we can see that the JMDR performance of one-symbol SSS-like PEI and paging PDCCH does not fulfill the required performance. The JMDR performance of two-symbol SSS-like PEI and paging PDCCH fulfills the required performance.

For Behv-A, a new SSS pattern, i.e., two-symbol SSS-like PEI with 127x2 REs@FAR=0.1%, fulfills the performance required by the paging PDSCH without scaling.

For Behv-B, the JMDR performance of SSS-like PEI and paging PDCCH is shown in Figure 4.

Figure 4 JMDR performance of SSS-like PEI and paging PDCCH - Behv-B

According to Figure 4, we can see that the JMDR performance of both one-symbol SSS-like PEI and two-symbol SSS-like PEI is better than that of PDSCH, but the MDR performance does not fulfill the required performance when one-symbol SSS-like PEI is used. Therefore, at least two-symbol SSS-like PEI is needed.

For Behv-B, a new SSS pattern, i.e., SSS-like PEI with 127x2 REs@FAR=0.1%, fulfills the performance requirement when the paging PDSCH is not configured with TB scaling.

When PDSCH with TB scaling 1 is considered, DCI-based PEI with aggregate level 2 can fulfill the performance requirement, and the corresponding resource overhead is 144REs. However, SSS-like PEI requires at least two symbols, so the resource overhead is 264REs. Therefore, DCI-based PEI occupies less resources for both Behv-A and Behv-B.

Compared with SSS-like PEI, the DCI-based PEI occupies less resources for both Behv-A and Behv-B.

Power saving gain

In our contribution [2], the power saving gain with the timeline of two SSB before PO are provided. In some special cases, such as Redcap UE or poor channel condition, two SSBs before the PO may not meet the synchronization and measurement requirements. In some other cases, only one SSB is needed. Therefore, in this section, we analyze the power saving gain of PEI in the cases that three SSBs and one SSBs are processed before PO.

- Three SSBs before PO

The UE processing timeline with three SSBs before PO is provided in Figure 5.

Figure 5 UE processing timeline (Three SSBs before PO)

Assume that the offset between PO and the nearest SSB is 10 ms, the power saving gain provided by PEI in different locations are given in Figure 6. It can be seen that the PEI located near the first SSB provides the maximum power saving gain. The power saving gains obtained by the PEI located in location 1 and location 2 are basically the same.

Figure 6 Power saving gain provided by PEI (Three SSB before PO)

The PEI located near the first SSB provides the maximum power saving gain.

The power saving gains obtained by the PEI located in location 1 and location 2 are basically the same.

Taking the PEI located in position 2 as an example, the power saving gain of the combination of the PEI and sub-grouping is shown in Figure 7. It can be observed that the combination of PEI and sub-grouping can provide higher power saving gain. Hence, the sub-grouping information should be carried on PEI.

Figure 7 Power saving gain provided by PEI with sub-grouping (Three SSB before PO)

The combination of PEI and sub-grouping can provide more power saving gain.

The combination of DCI-based PEI and sub-grouping should be supported.

Figure 8 provides the comparison of power saving gain between DCI-based PEI, sequence-based PEI without synchronization (sequence-w/o-sync) and sequence-based PEI with synchronization (sequence-w-sync).

Figure 8 Power saving gain of PEI

According to Figure 8, it can be seen that the power saving gain from DCI-based PEI and sequence-based PEI is almost the same.

The power saving gain from DCI-based PEI and sequence-based PEI is almost the same.

- One SSB before PO

Figure 9 Potential PEI location (one SSB before PO)

As shown in Figure 9, when there is only one SSB before PO, the serving cell measurement relies on this SSB. Therefore, regardless of the PEI position (i.e., position 1~3 in Figure 9) or the PEI type (i.e., DCI-based PEI or sequence-based PEI), UE cannot reduce the processing of SSB. Therefore, DCI-based PEI and sequence-based PEI have the same power saving gain in the same position.

Figure 10 Power saving gain provided by PEI (One SSB before PO)

According to the Figure 10, it can be seen the PEI near the SSB can provide higher power saving gain. And the power saving gain corresponding to location 1 and location 2 are basically the same.

When there is only one SSB before PO,

  - DCI-based PEI and sequence-based PEI have the same power saving gain in the same position;

  - PEI located before and after the SSB before PO brings similar power saving gain.

Conclusion

In this contribution, we discuss the detection performance and power saving gain of IDLE mode UE. We have the following observations and proposals.

Observation 1: For Behv-A/B, DCI-based PEI with AL 4 fulfills the performance required by paging PDSCH without TB scaling. DCI-based PEI with AL 8 can fulfill the performance requirement by PDSCH with TB scaling 0.5.

Observation 2: With ML decoding, DCI-based PEI with AL 2 fulfills the performance requirement of TB scaling 1 and DCI-based PEI with AL 4 fulfills the performance requirement of TB scaling 0.5 in the case of Behv-A and Behv-B.

Observation 3: For Behv-A, a new SSS pattern, i.e., two-symbol SSS-like PEI with 127x2 REs@FAR=0.1%, fulfills the performance required by the paging PDSCH without scaling.

Observation 4: For Behv-B, a new SSS pattern, i.e., SSS-like PEI with 127x2 REs@FAR=0.1%, fulfills the performance requirement when the paging PDSCH is not configured with TB scaling.

Observation 5: Compared with SSS-like PEI, the DCI-based PEI occupies less resources for both Behv-A and Behv-B.

Observation 6: The PEI located near the first SSB provides the maximum power saving gain.

Observation 7: The power saving gains obtained by the PEI located in location 1 and location 2 are basically the same.

Observation 8: The combination of PEI and sub-grouping can provide more power saving gain.

Observation 9: The power saving gain from DCI-based PEI and sequence-based PEI is almost the same.

Observation 10: When there is only one SSB before PO,

a) DCI-based PEI and sequence-based PEI have the same power saving gain in the same position;

b) PEI located before and after the SSB before PO brings similar power saving gain.

Proposal 1: The combination of DCI-based PEI and sub-grouping should be supported.

References

Chairman's Notes RAN1#104-e, 3GPP TSG RAN1#104-e, 2021-01.

R1-2004221, Discussion on power saving enhancements for paging, ZTE, Sanechips, 2021-05.

- Appendix A: Simulation assumptions

Table A1. Assumptions for link level simulation

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 4GHz |
| Transmission BW | 20MHz |
| SCS | 30KHz |
| Antenna Configuration | 2Tx and 2Rx |
| Channels | TDL-C 300 ns delay spread 100 Hz Doppler shift |
| Frequency error | 0 PPM for paging-PDCCH/PDSCH; [-0.5,0.5] PPM for PEI. |
| PDCCH configuration | Paging DCI: AL8, 41 info + 24 CRC bits, REG bundle size 6; DCI-based PEI:AL4/8, 12 info + 24 CRC bits, REG bundle size 6. |
| PDSCH configuration | Mapping type A, MCS 0, 48 PRB, TB scaling factor 1, DMRS type 1 with 2 additional DMRS |