3GPP TSG RAN WG1 #105-e                                        R1-2104485

e-Meeting, May 19th – 27th, 2021

Source:	CATT

Title:	Enhancements on PUCCH and PUSCH

Agenda Item:	8.1.2.1

Document for:	Discussion and Decision

Introduction

As approved in RAN #86, improvement of reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel was identified in the WID on further enhancement of MIMO in Rel-17 [1]:

Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

- Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline.

In RAN1 #104bis e-meeting, the following agreements were achieved on M-TRP PUCCH transmission:

Agreement For the case of multi-TRP, to support per-TRP power control in FR1, the linking of PUCCH resource with [one or] two power control parameter sets, the following is supported - MAC-CE indicates RRC IE that configures power control parameter sets (p0, pathloss RS ID, and a closed-loop index). - The exact design of RRC IE is up to RAN2 but from RAN1 point of view, one possible example is to reuse PUCCH-SpatialRelationInfo except for the referenceSignal Note: It is common understanding in RAN1 that one PUCCH resource can be linked to one power control parameter set. Conclusion With reference to the normative work on NR-feMIMO: Related to the support of switching gap between UL transmissions towards two TRPs in RAN1 specifications, there is no consensus in RAN1 to specify symbol gap(s) for the following cases - PUSCH Type A - PUCCH scheme 1 - PUSCH Type B - PUCCH scheme 3 The above applies for the case included in the LS from RAN4 in R1-2102297. Agreement When inter-slot frequency hopping is configured with Scheme 1, decide one from the below options in RAN1#105-e meeting, - Option 1 - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15). - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam. - Option 2: - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact) - Option 3: - Frequency hopping is performed on slot level as in Rel-15 (no spec impact). Agreement Confirm the following Working Assumption: For PUCCH multi-TRP enhancements in Scheme 1, it is possible to configure either cyclic mapping or sequential mapping of spatial relation info’s over PUCCH repetitions. - FFS: Applicability of mapping patterns for different beam switching gaps - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Note: For Scheme 1, cyclical mapping pattern and sequential mapping pattern are as follows, - Cyclical mapping pattern: the first and second beam are applied to the first and second PUCCH repetition, respectively, and the same beam mapping pattern continues to the remaining PUCCH repetitions. - Sequential mapping pattern: the first beam is applied to the first and second PUCCH repetitions, and the second beam is applied to the third and fourth PUCCH repetitions, and the same beam mapping pattern continues to the remaining PUCCH repetitions. Agreement Confirm the following Working Assumption (with small correction of typo and clarification on UE capability in RED): - For beam mapping /power control parameter set mapping for PUCCH repetitions, - For M-TRP PUCCH Scheme 1 in FR1, it is possible to configure either cyclic mapping or sequential mapping of power control parameter sets over PUCCH repetitions (similar to spatial relation info’s over PUCCH repetitions). - For M-TRP PUCCH Scheme 3, reuse the same methods as Scheme 1 (by replacing slots with sub-slots) for beam mapping or power control resource parameter set mapping to sub-slots. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2.

In RAN1 #104bis e-meeting, the following agreements were achieved on M-TRP PUSCH transmission:

Agreement When SRS resources from two SRS resource sets indicated in DCI format 0_1/0_2, for linking SRI fields to two power control parameters, it is up to RAN2 to finalize the RRC details related to linking. RAN1 identified that the following options could be used. - Alt. 1: Add second sri-PUSCH-MappingToAddModList, and select two SRI-PUSCH-PowerControl from two sri-PUSCH-MappingToAddModList - Alt. 2: Add SRS resource set ID in SRI-PUSCH-PowerControl, and select SRI-PUSCH-PowerControl from sri-PUSCH-MappingToAddModList considering the SRS resource set ID Agreement For PHR reporting related to M-TRP PUSCH repetition, select one from the following options in RAN1 #105-e meeting. - Option 1: Calculate one PHR associated with the first PUSCH occasion (earliest repetition that overlaps with the first slot in which the PUSCH that carries the PHR MAC-CE is transmitted) - Option 2: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, but report one of them - FFS: How to select the PHR for reporting. - Option 4: Calculate two PHRs, each associated with a first PUSCH occasion to each TRP, and report two PHRs - Option 5: No changes to legacy PHR reporting Agreement When MAC-CE indicates a PL-RS ID for one or more SRI IDs, it also indicates whether the SRI IDs are associated with the first or the second SRS resource set. Agreement For multiplexing A-CSI on two PUSCH repetitions in the case of multi-TRP PUSCH repetition, - For S-DCI based multi-TRP PUSCH repetition Type B, support multiplexing A-CSI on the first PUSCH repetition corresponding to the first beam and the first (X = 1) PUSCH repetition corresponding to the second beam. - The UE is expected to follow the above operation for multiplexing A-CSI on two PUSCH repetitions only if - the first actual repetition corresponding to the first beam and the first actual repetition corresponding to the second beam does not have the same number of symbols, and - UCIs other than the A-CSI are not multiplexed on any of the two PUSCH repetitions. - When the UE does not follow the above operation, UE multiplexes A-CSI only on the first PUSCH repetition similar to Rel. 15/16. - The content for the two A-CSI should be the same - Note: RAN1 has the assumption on CSI timelines are followed as rel-15/16, including UE shall expect the timeline for the first A-CSI meets Z and Z’ requirement - FFS: For s-DCI based multi-TRP PUSCH repetition Type A and B, support multiplexing of A-CSI on the first PUSCH repetition corresponding to the first beam and the first PUSCH repetition corresponding to the second beam when there is no TB carried in the PUSCH. - The UE assumes that the number of repetitions is 2 regardless of the indicated number of repetitions. - For PUSCH repetition Type B, the first and second nominal repetitions are expected to be the same as the first and second actual repetitions, respectively (no segmentation). Working Assumption For indicating STRP/MTRP dynamic switching for non-CB/CB based MTRP PUSCH repetition, - Introduce a new field in DCI to indicate at least the S-TRP or M-TRP operation - FFS: Whether the new field is 1 bit or 2 bits Working Assumption For non-codebook based multi-TRP PUSCH, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field. The number of bits, N2N2, for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field. For each rank x, the first KxKx codepoint(s) are mapped to Kx SRIs of rank x associated with the first SRS field, the remaining (2N2-Kx)(2N2- Kx) codepoint(s) are reserved. Agreement For the indication of open-loop power control parameter (OLPC) in DCI format 0_1/0_2, support enhanced open-loop power control parameter (OLPC) set indication by indicating per-TRP OLPC set. - FFS: Details of indication. Agreement For CB based M-TRP PUSCH repetition, the first TPMI field is used to determine the entry of the second TPMI field which only contains TPMIs corresponding to the indicated rank (number of layers) of the first TPMI field. The second TPMI field’s bit width, M2, is determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field. For each rank y, the first Ky codepoint(s) of the second TPMI field are mapped to Ky KyTPMI(s) of rank y associated with the first TPMI field in increasing order codepoint index, the remaining (2M2-Ky) codepoint(s) are reserved. - How to describe/capture this in 38.212 is up to the editor. Agreement Confirm the following working assumption (with removing the last bullet): For single DCI based M-TRP PUSCH repetition Type A and B, it is possible to configure either cyclic mapping or sequential mapping of UL beams. - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - FFS: Support of half-half mapping. - FFS: Additional considerations on mapping patterns (including required beam switching gaps) Agreement For single DCI based M-TRP PUSCH Type B repetition, the indication of PTRS-DMRS association for maxRank > 2 is supported, down select one of the following options in RAN1 #105-e meeting, - The support of cyclic mapping can be optional UE feature for the cases when the number of repetitions is larger than 2. - Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs. - Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP. - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports. - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port. Agreement For type 1 or type 2 CG based multi-TRP PUSCH repetition, - Introduce the second fields of 'p0-PUSCH-Alpha' and 'powerControlLoopToUse' in 'ConfiguredGrantConfig’ - For type 1 CG based m-TRP PUSCH repetition, introduce the second fields of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator' and 'precodingAndNumberOfLayers' in 'rrc-ConfiguredUplinkGrant'. - For type 2 CG based M-TRP PUSCH, two SRIs/TPMIs are indicated via the activating DCI. - FFS1: UL PT-RS port(s) and DM-RS port(s) for CG type 1 - FFS3: Details on RV mapping. - FFS4: Possible transmission occasion for initial transmission - FFS5: Other TRP specific parameters in 'rrc-ConfiguredUplinkGrant', e.g., 'dmrs-SeqInitialization'.

In this contribution, we share our views on the enhancements of M-TRP PUCCH and PUSCH transmission.

Enhancements on PUCCH

Maximum repetition number

In RAN1 #104 e-meeting, the following agreement on the repetition number for M-TRP PUCCH transmission was achieved:

Agreement For M-TRP PUCCH scheme 1, - For PUCCH formats 1/3/4, values for the total number of repetitions at least contain values 2, 4, and 8. - FFS: maximum repetition number can be extended to 16. - For PUCCH formats 0/2, the total number of repetitions at least contain 2. - FFS: other values. - RRC configured number of slots (repetitions) are applied across both TRPs (e.g if the number of repetitions given by nrofSlots in PUCCH-config is 8, per TRP limit is 4).

For M-TRP PUCCH transmission, each TRP is expected to be associated with half of the repetition numbers. In Rel-16, the maximum repetition number for PUCCH formats 1/3/4 is 8 and repetition number for PUCCH format 0/2 is not supported. Since the motivation of M-TRP PUCCH transmission is to improve reliability and robustness, the maximum repetition number for each TRP is expected to be no less than that for Rel-16, i.e. repetition number of 16 for PUCCH format 1/3/4 and 2 for PUCCH format 0/2 should be supported. Since PUCCH format 0 and 2 are mainly designed for scenarios with low latency, they are not expected to have large repetition numbers. More than 2 repetitions for PUCCH format 0 and 2 are not preferred.

Proposal 1:

- For the repetition number for M-TRP PUCCH scheme 1,

  - 16 is supported for PUCCH format 1/3/4.

Power control and switching of S-TRP/M-TRP transmission.

In RAN1 #104 e-meeting, it was agreed to select one out of the following solutions to support per TRP closed-loop power control for PUCCH:

- Option.1: A single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for both PUCCH beams

- Option.2: A single TPC field (the existing TPC field) is used in DCI formats 1_1 / 1_2, and the TPC value applied for one of two PUCCH beams at a slot. The TPC value may be applied for the other PUCCH beam at another slot.

- Option 3: A second TPC field (similar to the existing TPC field) is added in DCI formats 1_1 / 1_2.

- Option 4: A single TPC field is used in DCI formats 1_1 / 1_2, and indicates two TPC values applied to two PUCCH beams, respectively.

Although option 1 has lowest overhead, it is not preferred since it can’t realize TRP-specific TPC adjustment.

For option 2, only one PUCCH beam can adjust its power according to the TPC command at one time. The TPC values for two PUCCH beams have to be adjusted by indicating TPC command for each beam alternately. In order to indicate which TRP the TPC command is associated to, one alternative is adding an additional bit for explicitly indication. Then 3 bits are needed. Another alternative is to associate TRPs with slot indexes, for example, slots with odd indexes are associated with TRP 1 and slots with even indexes are associated with TRP 2. Then the TPC command in the DCI is for TRP 1 when the PUCCH is triggered by a DCI in a slot with odd index, and the TPC command in the DCI is for TRP 2 when the PUCCH is triggered by a DCI in a slot with even index. Although such association can save one bit compared to explicitly indication, it is not preferred since it restricts the flexibility of PDSCH scheduling.

Both option 3 and option 4 can indicate TPC values for the two TRP separately at the same time. Option 3 has best TPC indication accuracy but requires twice overhead compared to Rel-16 TPC indication. For option 4, two TPC values are jointly encoded. Compared to option 3, option 4 can consume less overhead by using coarser TPC indication granularity. It can be a compromised solution of performance and DCI overhead.

Proposal 2:

- For per TRP closed-loop power control for PUCCH,

  - Option 3 or option 4 is supported.

  - If option 4 is adopted, 2 or 3 bits are used.

Beam mapping and frequency hopping

In last meeting, both cyclical mapping pattern and sequential mapping pattern are confirmed to be supported for M-TRP PUCCH transmission. Note that if the repetition number of a PUCCH resource with sequential mapping pattern is configured to be two, the two PUCCH transmission occasions would be associated with the same TRP. Then beam diversity gain can’t be achieved. For M-TRP PUCCH transmission, we prefer to always have beam diversity gain for PUCCH resources configured with 2 repetitions, with one of the following alternatives supported:

- Alt 1: For M-TRP PUCCH transmission, if a PUCCH resource is configured with sequential mapping pattern and two repetitions, the two transmission occasions are associated with two TRPs respectively;

- Alt 2: For M-TRP PUCCH transmission, if a PUCCH resource is configured with two repetitions, the PUCCH resource is not expected to be configured with a beam mapping pattern other than cyclical mapping.

Proposal 3:

- For M-TRP PUCCH transmission, when the repetition number of a PUCCH resource is set to two, one of the following alternatives is supported:

  - Alt 1: The two transmission occasions are associated with two TRPs respectively, regardless of the configured beam mapping pattern;

  - Alt 2: The PUCCH resource is not expected to be configured with a beam mapping pattern other than cyclical mapping.

In last meeting, the following 3 options on inter-slot frequency hopping for scheme 1 were proposed:

- Option 1

  - If sequential mapping pattern is configured, frequency hopping is performed on slot level (as in Rel-15).

  - If cyclical mapping pattern is configured, frequency hopping is performed among the repetitions with the same beam.

- Option 2:

  - gNB always configures sequential mapping pattern and frequency hopping is performed on slot level. (no spec impact)

- Option 3:

  - Frequency hopping is performed on slot level as in Rel-15 (no spec impact).

Figure 1 Comparison of different frequency hopping schemes for inter-slot PUCCH repetition

Fig. 1 shows the comparison of different frequency hopping schemes for different beam mapping patterns. In the figure, repetition number of 4 is assumed, “ Alt 1” denotes the frequency hopping scheme with frequency hopping performed among the repetitions with the same beam, and “Alt 2” denotes slot level frequency hopping scheme as in Rel-15. It can be seen from figure that:

- For sequential beam mapping, Alt 1 is equivalent to Alt 2.

- For cyclical beam mapping,

  - frequency hopping can be achieved by both Alt 1 and Alt 2.

  - Alt 2 achieves frequency hopping one slot earlier than Alt 1.

  - frequency hopping inner a beam is supported by Alt 1 and is not supported by Alt 2.

In our view, frequency hopping should be supported for both cyclical mapping and sequential mapping. For sequential mapping, there is no difference of Alt 1 and Alt 2, hence any one of Alt 1 and Alt 2 can be supported. For cyclical mapping, Alt 1 is preferred in the example since frequency hopping inner a beam is supported by Alt 1. Note that if the repetition number is 2, frequency hopping can’t be achieved by cyclical beam mapping + Alt 1. Therefore, for cyclical mapping, Alt 2 is preferred if the repetition number is 2 and Alt 1 is preferred if the repetition number is larger than 2. Considering that a PUCCH resource configured with sequential beam mapping and 2 repetitions would be transmitted with cyclical beam mapping, when the repetition number of a PUCCH resource is set to 2, only frequency hopping for cyclical beam mapping needs to be supported.

Proposal 4:

- For frequency hopping for M-TRP PUCCH transmission with scheme 1,

  - Frequency hopping is performed among the repetitions with the same beam for both sequential mapping pattern and cyclical mapping pattern when the repetition number is larger than 2;

  - Slot level frequency hopping is supported for cyclical mapping pattern when the repetition number is 2.

Default beam for PUSCH scheduled by DCI format 0_0

In Rel-15, the UE shall transmit PUSCH scheduled by DCI format 0_0 according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell. In Rel-17, it is possible that the PUCCH resource with the lowest ID within the active UL BWP is activated with two spatial relation info’s for M-TRP transmission, a clarification on how to transmit PUSCH scheduled by DCI format 0_0 is needed. In our opinion, if the PUSCH is not configured with M-TRP transmission, the most straight forward way is to determine the spatial filter of the PUSCH according to the spatial relation info with lower ID if PUCCH resource with the lowest ID is activated with two spatial relation info’s.

Proposal 5:

- If PUSCH is not configured with M-TRP transmission,

  - The PUSCH scheduled by DCI format 0_0 is transmitted according to the spatial relation info with lower ID of the PUCCH resource with the lowest ID if the PUCCH resource is activated with two spatial relation info’s.

Enhancements on PUSCH

Dynamic switching of S-TRP/M-TRP transmission and SRI/TPMI indication

For dynamic switching of S-TRP/M-TRP transmission of PUSCH, several schemes that aimed to reduce the indication overhead had been proposed in previous meetings. Most schemes use SRI field(s) or TPMI field(s) for dynamic switching of S-TRP/M-TRP.

For codebook based M-TRP PUSCH transmission, if SRI field(s) is used for dynamic switching of S-TRP/M-TRP, the following alternatives can be considered:

- Alt 1: Each SRI field is enhanced with a reserved state or an additional state indicating that the TRP corresponding to the SRI field is not transmitted to.

- For this solution, the association of SRI fields and TRPs are fixed, i.e. the first SRI field is always for the first TRP and the second SRI field is always for the second TRP;

- Alt 2: Only the second SRI field is enhanced

- Alt 2-1: The association of SRI fields and TRPs are fixed; the enhancement of the second SRI field is as follows: N sates indicate the SRS resource index for the second TRP in M-TRP transmission, N sates indicate S-TRP transmission to the second TRP and the SRS resource index for the transmission, and one state indicates S-TRP transmission to the first TRP, where N is the number of candidate SRI values for the second TRP.

- Alt 2-2: The first SRI field always indicates an SRS resource index for the transmission; for the second SRI field, 2 reserved states or 2 additional states are used to indicate S-TRP transmission and which TRP the transmission is associated to.

Table 1 shows the total number of bits for the two SRI fields for the above alternatives. In the table, the total number of bits for 2 SRI fieds with each SRI field same as Rel-16 is also provided as Alt 0 for better comparison. The case with SRI absent is not provided in the table since it may use other solutions. It can be seen from the table that if SRI field(s) is used for dynamic switching of M-TRP and S-TRP, at least 1 more bit is needed for most cases.

Table 1 Total number of bits for two SRI fields for different alternatives

| - | Number of SRS resources (NSRS) |  |  |
| --- | --- | --- | --- |
|  | NSRS=2 | NSRS=3 | NSRS=4 |
| Alt 0 | 2 | 4 | 4 |
| Alt 1 | 4 | 4 | 6 |
| Alt 2-1 | 4 | 5 | 6 |
| Alt 2-2 | 3 | 5 | 5 |

For codebook based M-TRP PUSCH transmission, the first TPMI field is used to determine the entry of the second TPMI field which only contains TPMIs corresponding to the indicated rank (number of layers) of the first TPMI field. The second TPMI field’s bit width is determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field. If the second TPMI field is used for dynamic switching of S-TRP/M-TRP, the following alternatives can be considered:

- Alt 1: The association of TPMI fields and TRPs are fixed; the enhancement of the second TPMI field is as follows: N sates indicate M-TRP transmission and the TPMI index for the second TRP, N sates indicate S-TRP transmission to the second TRP and the TPMI index for the transmission, and one state indicates S-TRP transmission to the first TRP, where N is the number of candidate TPMI indexes for the second TRP.

- Alt 2: The first TPMI field always indicates a TPMI and the transmission layer for the transmission; for the second TPMI field, 2 reserved states or 2 additional states are used to indicate S-TRP transmission and which TRP the transmission is associated to.

Table 2 shows the bit width for the second TPMI field for different alternatives. In the table, PUSCH transmission with different full power transmission modes have been considered, Nport represents the maximum number of antenna ports for the PUSCH transmission. The bit width for the second TPMI field only determined by the maximum number of TPMIs per rank among all ranks associated with the first TPMI field is also provided as Alt 0 for better comparison. The cases with the second TPMI absent are not provided in the table since they may use other solutions.

It can be seen from the table that if the second TPMI field is used to indicate dynamic switching of S-TRP/M-TRP transmission:

- For Alt 2, most cases require one more bits compared to Alt 0.

- Alt 1 requires same or more bits compared to Alt 2.

Table 2 Bit widths of the second TPMI field for different alternatives, for codebook based PUSCH

For non-codebook based M-TRP PUSCH transmission, the first SRI field is used to determine the entry of the second SRI field which only contains the SRI(s) combinations corresponding to the indicated rank (number of layers) of the first SRI field. The number of bits for the second SRI field is determined by the maximum number of codepoint(s) per rank among all ranks associated with the first SRI field. If SRI field(s) is used for dynamic switching of S-TRP/M-TRP, the following alternatives can be considered:

- Alt 1: Each SRI field is enhanced with a reserved state or an additional state indicating that the TRP corresponding to the SRI field is not transmitted to.

- For this solution, the association of SRI fields and TRPs are fixed, i.e. the first SRI field is always for the first TRP and the second SRI field is always for the second TRP;

- Alt 2: Only the second SRI field is enhanced,

- Alt 2-1: The association of SRI fields and TRPs are fixed; the enhancement of the second SRI field is as follows: N sates indicate M-TRP transmission and the SRI combination for the second TRP, N sates indicate S-TRP transmission to the second TRP and the SRI combination for the transmission, and one state indicates S-TRP transmission to the first TRP, where N is the number of candidate SRI values for the second TRP.

- Alt 2-2: The first SRI field always indicates a SRI combination for the transmission; for the second SRI field, 2 reserved states or 2 additional states are used to indicate S-TRP transmission and which TRP the transmission is associated to.

Table 3 shows the bit width for the second SRI field for the above alternatives. In the table, the bit width for the second SRI field only determined by the maximum number of SRIs per rank among all ranks associated with the first SRI field is also provided as Alt 0 for better comparison, and the cases with the second SRI absent is not present in the table since they may use other solutions.

It can be seen from Table 3 that

- For most cases, Alt 1 requires one more bits compared to Alt 0.

- For most cases, Alt 2-2 requires one more bits compared to Alt 0.

- Alt 2-1 requires same or more bits compared to Alt 2-2.

Table 3 Bit widths of the second SRI field for different alternatives, non-codebook based PUSCH

Since at least one more bit is needed for most cases if only SRI field(s) or TPMI field(s) is used for dynamic S-TRP/M-TRP PUSCH transmission indication, we prefer to introduce a new field in DCI for dynamic S-TRP/M-TRP transmission indication directly. The new field can be a 1-bit field to indicate whether the PUSCH transmission is S-TRP transmission or M-TRP transmission. If S-TRP transmission is indicated by the new field, the first TPMI field/SRI field can be used to indicate the TPMI/SRI for the PUSCH transmission, and two states of the second TPMI filed (for codebook based PUSCH)/the second SRI field (for non-codebook based PUSCH) can be used to indicate which TRP the transmission is associated to. According to this solution, no more bit is needed to indicate which TRP the PUSCH transmission is associated to when S-TRP transmission is indicated. For example, considering a codebook based PUSCH transmission with 4Tx for a non-coherent UE, if the maximum layer of codebook based PUSCH transmission is configured to be 1, UE determines the precoding matrix for the second TRP according to Table 4 when M-TRP transmission is indicated and determines the TRP according to Table 5 when S-TRP transmission is indicated.

Table 4 The second TPMI field for 4Tx, if non-coherent codebook subset is configured, maxRank=1, and M-TRP transmission is indicated

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 |
| … | … |
| 3 | 1 layer: TPMI=3 |

Table 5 The second TPMI field for 4Tx, if non-coherent codebook subset is configured, maxRank=1, and S-TRP transmission is indicated

| Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- |
| 0 | TRP 1 |
| 1 | TRP 2 |
| 2-3 | reserved |

If the second TPMI field/SRI field is absent, a new 2-bit field in DCI for dynamic S-TRP/M-TRP transmission indication can be supported, with one state indicate M-TRP transmission and 2 states indicate S-TRP transmission to each TRP respectively.

Proposal 6:

- For dynamic S-TRP/M-TRP transmission indication for non-CB/CB based PUSCH,

  - A new field in DCI is introduced to indicate S-TRP or M-TRP operation;

    - The new filed is a 1-bit field if TPMI fields are present for CB based PUSCH or SRI fields are present for non-CB based PUSCH;

      - If S-TRP transmission is indicated,

        - for CB based PUSCH transmission, two states of the second TPMI filed are used to indicate which TRP the PUSCH transmission is associated to;

        - for non-CB based PUSCH transmission, two states of the second SRI filed are used to indicate which TRP the PUSCH transmission is associated to.

    - The new filed is a 2-bit field if TPMI fields are absent for CB based PUSCH or SRI fields are absent for non-CB based PUSCH.

Beam mapping and frequency hopping

On beam mapping and frequency hopping for PUSCH, we have similar views as that for PUCCH, same solution as that for PUCCH can be applied. Our preference is that if the repetition number of a PUCCH resource with sequential mapping pattern is configured to be two, the two PUCCH transmission occasions are associated with two TRPs respectively. If the repetition number of the PUSCH transmission is 2, slot level frequency hopping is supported; otherwise, frequency hopping is performed among the repetitions within the same beam.

Proposal 7:

- For M-TRP PUSCH transmission with 2 repetitions, one of the following clarifications is supported:

  - Alt 1: The two transmission occasions are associated with two TRPs respectively, regardless of the configured beam mapping pattern;

  - Alt 2: The PUSCH transmission is not expected to be configured with a beam mapping pattern other than cyclical mapping pattern.

Proposal 8:

- For frequency hopping of M-TRP PUSCH transmission, the following scheme is supported:

  - Frequency hopping is performed among the repetitions with the same beam for both sequential mapping pattern and cyclical mapping pattern when the repetition number is larger than 2;

  - Slot level frequency hopping is supported for cyclical mapping pattern when the repetition number is 2.

PUSCH scheduled by DCI format 0_0

For M-TRP PUSCH transmission, repetitions among multiple TRPs are supported by PUSCH scheduled by DCI format 0_1 or DCI format 0_2 and CG PUSCH. In our opinion, in order to have better performance for PUSCH scheduled by DCI format 0_0, repetitions among multiple TRPs also should be supported for PUSCH scheduled by DCI format 0_0.

Proposal 9:

- Repetition for PUSCH scheduled by DCI format 0_0 is supported in Rel-17.

For PUSCH scheduled by DCI format 0_1 or DCI format 0_2, the repetition type and the number of repetitions can be configured and indicated by TDRA. This scheme can be extended to be applied for PUSCH scheduled by DCI format 0_0. For the time domain resource allocation of a PUSCH transmission scheduled by DCI format 0_0, it is possible that default PUSCH time domain resource allocation A is applied, which can’t indicate the number of repetitions. In this condition, whether the PUSCH transmission is S-TRP transmission or M-TRP transmission can be determined by the number of spatial relation info that the PUCCH resource with the lowest ID is associated to. If the PUCCH resource with the lowest ID is activated with two spatial relation info’s, the PUSCH scheduled by DCI format 0_0 has two transmission occasions with each of them corresponding to one spatial relation info; otherwise, the the PUSCH scheduled by DCI format 0_0 has only one transmission occasion.

Proposal 10:

- For M-TRP PUSCH scheduled by DCI format 0_0,

  - The spatial relation of the PUSCH is determined by the PUCCH resource with the lowest ID;

    - if the PUCCH resource with the lowest ID is activated with two spatial relation info’s, the number of transmission occasions of the PUSCH is two, and each occasion is associated with one spatial relation info;

    - otherwise, the number of transmission occasion of the PUSCH transmission is 1.

Power control for PUSCH

TPC indication

In RAN1 #104 e-meeting, the following agreements were achieved on TPC command for M-TRP PUSCH transmission:

Further study following alternatives to support per TRP closed-loop power control for PUSCH, select from the below options during the RAN1 #104-e-bis meeting. - Option.1: A single TPC field (the existing TPC field) is used in DCI formats 0_1 / 0_2, and the TPC value applied for both PUSCH beams - Option.2: A single TPC field (the existing TPC field) is used in DCI formats 0_1 / 0_2, and the TPC value applied for one of two PUSCH beams at a slot. - Option 3: A second TPC field (similar to the existing TPC field) is added in DCI formats 0_1 / 0_2. - Option 4: A single TPC field is used in DCI formats 0_1 / 0_2, and indicates two TPC values applied to two PUSCH beams, respectively.

In our opinion, for TPC command enhancement for M-TRP PUSCH transmission, same solution as that for M-TRP PUCCH transmission can be applied. For the above options:

- Option 1 is not preferred since it can’t realize TRP-specific TPC adjustment.

- Option 2 is not preferred since only one PUCCH beam can adjust its power according to the TPC command at one time. Besides, option 2 may restricts the flexibility of PDSCH scheduling or require one additional bit to indicate which TRP the TPC command is associated to.

- Option 3 and option 4 are preferred since they can indicate TPC values for the two TRP separately at the same time.

Proposal 11:

- For per TRP close-loop power control for PUSCH,

  - Option 3 or option 4 is supported.

  - If option 4 is adopted, 2 or 3 bits are considered.

Power control when SRI is absent

For S-TRP PUSCH transmission scheduled by DCI format 0_1 or DCI format 0_2, if the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE determines the path loss RS resource for the PUSCH transmission according to the SRI field. If the PUSCH transmission is scheduled by DCI format 0_1 or DCI format 0_2 that does not include an SRI field, or SRI-PUSCH-PowerControl is not provided to the UE, the UE determines the path loss RS resource for the PUSCH transmission according to the RS resource index with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero.

For M-TRP PUSCH transmission, TRP-specific default power control parameter should be supported, i.e. if the PUSCH transmission is scheduled by DCI format 0_1 or DCI format 0_2 that does not include an SRI field, or SRI-PUSCH-PowerControl is not provided to the UE, the UE would determine a path loss RS resource for each TRP respectively. The following two solutions can be considered:

- Alt 1: Two path loss reference RS lists are configured for two TRPs respectively, UE determines the path loss RS resource for each TRP according to the RS resource index with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero in the corresponding list;

- Alt 2: Only one path loss reference RS list is configured, UE determines the path loss RS resource for TRPs according to the RS resource index with respective PUSCH-PathlossReferenceRS-Id values being equal to zero and one in the list.

Proposal 12:

- For M-TRP PUSCH transmission scheduled by DCI format 0_1 or 0_2, if SRI filed is absent or SRI-PUSCH-PowerControl is not provided,

  - TRP-specific default power control parameters are supported .

Proposal 13:

- For M-TRP PUSCH transmission scheduled by DCI format 0_1 or 0_2, if SRI filed is absent or SRI-PUSCH-PowerControl is not provided, one of the following alternatives for path loss RS determination is supported:

  - Alt 1: Two path loss reference RS lists are configured for two TRPs respectively, UE determines the path loss RS resource for each TRP according to the RS resource index with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero in the corresponding list;

  - Alt 2: Only one path loss reference RS list is configured, UE determines the path loss RS resource for TRPs according to the RS resource index with respective PUSCH-PathlossReferenceRS-Id values being equal to zero and one in the list.

Association of PTRS and DMRS ports

In last meeting, the following options on the indication of PTRS-DMRS association for single DCI based M-TRP PUSCH Type B repetition when maxRank > 2 were proposed:

- Option 1 (4 bits): with a second PTRS-DMRS association field (similar to the existing field), and each field separately indicating the association between PTRS port and DMRS port for two TRPs.

- Option 2 (2 bits): using the existing PTRS-DMRS association field in DCI for the first TRP, and using reserved entries/bits in DM-RS port indication field for the second TRP.

- Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP

  - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports.

  - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

Option 1 is a solution with the largest overhead and the highest flexibility. For option 1, each PTRS for each TRP can be associated with any DMRS port.

For M-TRP PUSCH transmission, same DMRS port indexes can be used by the repetitions for the two TRPs, therefore a new DM-RS port indication field for the second TRP is not needed. Option 2 is not a good solution since it would increase unnecessary overhead of DCI by adding a new DMRS port indication filed for the second TRP.

For option 3, the overhead of PTRS-DMRS association field is the same as that for S-TRP PUSCH transmission. The performance of phase noise estimation may be affected since only partial DM-RS ports can be associated to. However, since at least the new SRI field and the new TPMI field would increase the payload of DCI that scheduling PUSCH, and the performance of DCI would be damaged by increasing too many bits, option 3 is slightly preferred compared to option 1.

Proposal 14:

- For PTRS-DMRS association for single DCI based M-TRP PUSCH Type B repetition, option 3 is supported  when maxRank > 2, where option 3 is as follows:

  - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP

    - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports.

    - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

PUSCH with configured grant

For RV indication for M-TRP PUSCH transmission scheduled by DCI, similar solution as that for M-TRP PDSCH transmission scheme 4 can be  used, i.e. the DCI indicates the first RV for the first PUSCH repetition, and the RV pattern (0 2 3 1) is applied separately to PUSCH repetitions of different TRPs with a configuring RV offset for the starting RV for the second TRP. Similarly, for the RV pattern for configured grant based M-TRP PUSCH transmission, TRP-specific RV pattern can be supported.

Proposal 15:

- For configured grant M-TRP PUSCH transmission, the first RV for the first PUSCH repetition and a RV offset for the starting RV for the second TRP are configured.

For a S-TRP PUSCH transmission corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PTRS port(s) and DMRS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in TS38.212, i.e. a PTRS port is associated with the 1st scheduled DMRS port sharing the PTRS port. For Type 1 configured grant based M-TRP PUSCH transmission, a default association relationship between UL PTRS port(s) and DMRS port(s) also can be assumed, with the same assumption as that in Rel-15.

Proposal 16:

- For PTRS-DMRS association for Type 1 CG based M-TRP PUSCH transmission, each PTRS port is associated with the 1st scheduled DMRS port sharing the PTRS port.

Conclusions

In this contribution we provide our views on remaining issues for supporting multi-TRP/panel-based PUCCH/PUSCH in Rel-17. We have the following proposals:

Proposal 1:

- For the repetition number for M-TRP PUCCH scheme 1,

  - 16 is supported for PUCCH format 1/3/4.

Proposal 2:

- For per TRP closed-loop power control for PUCCH,

  - Option 3 or option 4 is supported.

  - If option 4 is adopted, 2 or 3 bits are used.

Proposal 3:

- For M-TRP PUCCH transmission, when the repetition number of a PUCCH resource is set to two, one of the following alternatives is supported:

  - Alt 1: The two transmission occasions are associated with two TRPs respectively, regardless of the configured beam mapping pattern;

  - Alt 2: The PUCCH resource is not expected to be configured with a beam mapping pattern other than cyclical mapping.

Proposal 4:

- For frequency hopping for M-TRP PUCCH transmission with scheme 1,

  - Frequency hopping is performed among the repetitions with the same beam for both sequential mapping pattern and cyclical mapping pattern when the repetition number is larger than 2;

  - Slot level frequency hopping is supported for cyclical mapping pattern when the repetition number is 2.

Proposal 5:

- If PUSCH is not configured with M-TRP transmission,

  - The PUSCH scheduled by DCI format 0_0 is transmitted according to the spatial relation info with lower ID of the PUCCH resource with the lowest ID if the PUCCH resource is activated with two spatial relation info’s.

Proposal 6:

- For dynamic S-TRP/M-TRP transmission indication for non-CB/CB based PUSCH,

  - A new field in DCI is introduced to indicate S-TRP or M-TRP operation;

    - The new filed is a 1-bit field if TPMI fields are present for CB based PUSCH or SRI fields are present for non-CB based PUSCH;

      - If S-TRP transmission is indicated,

        - for CB based PUSCH transmission, two states of the second TPMI filed are used to indicate which TRP the PUSCH transmission is associated to;

        - for non-CB based PUSCH transmission, two states of the second SRI filed are used to indicate which TRP the PUSCH transmission is associated to.

    - The new filed is a 2-bit field if TPMI fields are absent for CB based PUSCH or SRI fields are absent for non-CB based PUSCH.

Proposal 7:

- For M-TRP PUSCH transmission with 2 repetitions, one of the following clarifications is supported:

  - Alt 1: The two transmission occasions are associated with two TRPs respectively, regardless of the configured beam mapping pattern;

  - Alt 2: The PUSCH transmission is not expected to be configured with a beam mapping pattern other than cyclical mapping pattern.

Proposal 8:

- For frequency hopping of M-TRP PUSCH transmission, the following scheme is supported:

  - Frequency hopping is performed among the repetitions with the same beam for both sequential mapping pattern and cyclical mapping pattern when the repetition number is larger than 2;

  - Slot level frequency hopping is supported for cyclical mapping pattern when the repetition number is 2.

Proposal 9:

- Repetition for PUSCH scheduled by DCI format 0_0 is supported in Rel-17.

Proposal 10:

- For M-TRP PUSCH scheduled by DCI format 0_0,

  - The spatial relation of the PUSCH is determined by the PUCCH resource with the lowest ID;

    - if the PUCCH resource with the lowest ID is activated with two spatial relation info’s, the number of transmission occasions of the PUSCH is two, and each occasion is associated with one spatial relation info;

    - otherwise, the number of transmission occasion of the PUSCH transmission is 1.

Proposal 11:

- For per TRP close-loop power control for PUSCH,

  - Option 3 or option 4 is supported.

  - If option 4 is adopted, 2 or 3 bits are considered.

Proposal 12:

- For M-TRP PUSCH transmission scheduled by DCI format 0_1 or 0_2, if SRI filed is absent or SRI-PUSCH-PowerControl is not provided,

  - TRP-specific default power control parameters are supported .

Proposal 13:

- For M-TRP PUSCH transmission scheduled by DCI format 0_1 or 0_2, if SRI filed is absent or SRI-PUSCH-PowerControl is not provided, one of the following alternatives for path loss RS determination is supported:

  - Alt 1: Two path loss reference RS lists are configured for two TRPs respectively, UE determines the path loss RS resource for each TRP according to the RS resource index with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero in the corresponding list;

  - Alt 2: Only one path loss reference RS list is configured, UE determines the path loss RS resource for TRPs according to the RS resource index with respective PUSCH-PathlossReferenceRS-Id values being equal to zero and one in the list.

Proposal 14:

- For PTRS-DMRS association for single DCI based M-TRP PUSCH Type B repetition, option 3 is supported  when maxRank > 2, where option 3 is as follows:

  - Option 3 (2 bits): 1 bit MSB is used to indicate PTRS-DMRS association for the first TRP, and 1 bit LSB is used to indicate PTRS-DMRS association for the second TRP

    - if maxNrofPorts = 1, the 1 bit indicates one of the first two DMRS ports.

    - if maxNrofPorts = 2, the 1 bit indicates one of two DMRS ports sharing the same PTRS port.

Proposal 15:

- For configured grant M-TRP PUSCH transmission, the first RV for the first PUSCH repetition and a RV offset for the starting RV for the second TRP are configured.

Proposal 16:

- For PTRS-DMRS association for Type 1 CG based M-TRP PUSCH transmission, each PTRS port is associated with the 1st scheduled DMRS port sharing the PTRS port.

References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung