3GPP TSG RAN WG1 #105-e			R1-2104598

e-Meeting, May 19th – 27th, 2021

Agenda item:	5.1

Title:	Discussion on Rel-17 UL Tx switching

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN1#104b-e meeting, the agreements on the mapping between UL transmission ports and Tx chain were reached:

2 carriers scenario:

- the mapping for SUL and UL CA Option 1

- the mapping for UL CA Option 2

3 carriers (1 on Band A + 2 on Band B) scenario:

- the mapping for SUL and UL CA Option 1, for 1Tx-2Tx switching;

- the mapping for SUL and UL CA Option 1, for 2Tx-2Tx switching;

- the mapping for UL CA Option 2, for 1Tx-2Tx switching;

- the mapping for UL CA Option 2, for 2Tx-2Tx switching;

In this contribution, we would like to discuss our understanding on leftover issues.

- Tx switching between case 1 and case 3 for UL CA option 1 and SUL

It was discussed in RAN1#104b that whether Tx switching between case 1 and case 3 for UL CA option 1 and SUL is included in WID.

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 3 | 2T+0T |

This switching pattern implies that when band B is working, there is only 1 Tx assigned to it, even though the other Tx is just left there idling. We would like RAN1 to discuss the condition of such scenario.

We noticed the way forward text in R4-2103235(RAN4#98e):

- Mandate 2-layer PUSCH transmission in all the carriers supporting 2Tx, including the two carriers on band B in 1Tx-2Tx switching scenario and the three carriers on band A+B for 2Tx-2Tx switching scenario.

  - Clarify that the case with 1T on CC2@Band B and 1T on CC3@Band B is not considered in the WI discussion.

  - Mandate 2-layer PUSCH transmission is only under the precondition that 2T is supported already on the carrier.

It seems that case1-case3 switching is possible when 2T is not supported in band B.

At the same time, if  case1-case3 switching is valid for Band A + Band B scenario, should it also be valid for 2 carriers scenario? That is, the mapping table for Rel-17 2Tx-2Tx switching between two uplink carriers for SUL and UL CA Option 1 should be updated:

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |
| Case 3 | 2T+0T | 2P+0P, 1P+0P |

Proposal 1: Discuss and clarify whether the switching between case 1 and case 3 for SUL and UL CA Option 1 is valid, for both 3 carriers case and 2 carriers case.

- Tx chain state determination

It was discussed in RAN1#104b that, the state of Tx chain might not be unique sometimes.

“For instance, if the current state of Tx chains is 0T+2T and the next UL transmission is 1-port transmission on carrier 1, since 1P+0P can be mapped to either 1T+1T or 2T+0T, then what’s the state of Tx chains after Tx switching”——R1-2102860

And 3 solutions were introduced:

- Alt 1: The state of Tx chains after Tx switching is predefined in the specifications.

- Alt 2: The state of Tx chains after Tx switching is indicated by Network.

- Alt 3: The state of Tx chains after Tx switching is determined by UE.

In our view, there are 2 potential problems introduced by Tx chain state ambiguity.

- “Two steps” problem: Take the example above, 1T+1T works fine for 1P+0P transmission. It is only problematic if 1P+0P is immediately followed by 2P+0P transmission. Because then the switch will have to happen again, introducing another interruption unnecessarily. 

We fail to find an actual scenario for above two-steps problem in real network.

- “Switching time” problem: If the switching time for “0T+2T  1T+1T” is different from “0T+2T  2T+0T”, the state mismatch between gNB and UE may introduce the wrong configuration on gNB for how long it should wait before it can resume UL transmission scheduling again for that UE.  Another issue of “Switching time” problem is that it introduces under-optimized (i.e., longer) interruption time.

Our consideration for these 2 problems are listed in table below.

|  | Does Exist | Does not Exist |
| --- | --- | --- |
| “Two steps” problem | Alt 1 preferred, Alt 2 ok | Alt 3 |
| “Switching time” problem | Alt 1 preferred, Alt 2 ok | Alt 3 |

To summarize:

- If there is no actual case for either problems, then the mismatch between gNB and UE does not introduce any real problem. It is ok to let UE make a final call on Tx chain state.

- If any of the problems exists, we prefer the Tx chain state rule is predefined in spec, just like the way in R16, to save signaling overhead.

Proposal 2: The state of Tx chains after Tx switching is determined by UE, if there is no actual case for either “Two steps” problem or “Switching time” problem.

Proposal 3: The state of Tx chains after Tx switching is predefined in the specifications, if any of the 2 problems exists.

- Conclusions

Proposal 1: Discuss and clarify whether the switching between case 1 and case 3 for SUL and UL CA Option 1 is valid, for both 3 carriers case and 2 carriers case.

Proposal 2: The state of Tx chains after Tx switching is determined by UE, if there is no actual case for either “Two steps” problem or “Switching time” problem.

Proposal 3: The state of Tx chains after Tx switching is predefined in the specifications, if any of the 2 problems exists.

- References

- RP‑202799, New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1), RAN#90e.

- Section 5.1, RAN1 Aspects for RF requirements for NR frequency range 1, Chair's Notes RAN1#104b-e