| 3GPP TSG-RAN WG1 Meeting #105 e-Meeting, May 10th – 27th, 2021 | R1-2104677 |
| --- | --- |

Agenda item:	8.6.1.1

Source: 	Qualcomm Incorporated

Title: 	BW Reduction for RedCap UE

Document for:	Discussion/Decision

# Introduction

To meet the generic and use specific requirements of mid-range use cases, potential UE complexity reduction techniques were identified and analyzed for wearables, industry wireless sensors and surveillance cameras [1]. A revised WID [2] was approved in RAN-91e meeting, and a main objective of the WI is to specify the support for the following UE complexity reduction features in RAN1, RAN2 and RAN4:

- Reduced maximum UE bandwidth:

  - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz

  - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz

In this contribution, we continue the discussion about BWP operation for R17 RedCap UE. In general, the BWP configuration rules supported by non-RedCap UE should be re-used by RedCap UE during and after initial access. In terms of BWP switching, RedCap UE is not expected to support a timeline faster than NR R15/16 non-RedCap UE.

# Supporting BW Reduction of RedCap UE in FR1

## Initial BWP Configuration

### CORESET0 and Initial DL BWP

In FR1, 20 MHz is the maximum BW supported by a RedCap UE during and after initial access [2].  Since the max BW of SSB, CORESET0 and SIB1 of non-RedCap UE are contained within 20 MHz during initial access,  RedCap UE and non-RedCap UE can share the same SSB, CORESET0 and SIB1. Since the initial DL BWP of non-RedCap UE is defined by CORESET0, RedCap UE and non-RedCap UE can also share the initial DL BWP during initial access. The other SIBs of RedCap UE can either be scheduled by SIB1, or be transmitted on-demand within the initial DL BWP of RedCap UE.

Proposal 1: During initial access, RedCap and non-RedCap UEs share the same SSB, CORESET#0, SIB1 and initial DL BWP.

Proposal 2: Other SIBs of RedCap UE can either be scheduled by SIB1, or be transmitted on-demand within the initial DL BWP of RedCap UE.

### PRACH and Initial UL BWP

In NR R15/16, a non-RedCap UE is provided an initial UL BWP configuration by UplinkConfigCommonSIB IE in SIB1 and the BW of initial UL  BWP can be wider than 20 MHz. When PUSCH (msg3) or PUCCH (HARQ feedback for msg4) is scheduled during initial access, a RedCap UE cannot interpret the UL grant appropriately, if the FDRA and frequency hopping  are specified with respect to an initial UL BWP wider than 20 MHz. Therefore, when a RedCap UE is allowed to access a cell whose initial UL BWP for non-RedCap UE is wider than 20 MHz, it is necessary for the network to identify the RedCap UE before sending the RAR grant, so that the RedCap UE can be scheduled appropriately with a valid UL grant. To facilitate early identification of RedCap UE before msg2/msg3, dedicated PRACH resources should be allocated for RedCap UE within its initial UL BWP. Furthermore, with early indication in dedicated PRACH resources, BS could derive TA/RTT for the RedCap UE. Since the pathloss between BS and RedCap UE is a function of RTT, BS can enable DL and/or UL coverage recovery for RedCap UE, by using the solutions available in NR R15/16 and R17 coverage enhancement WI.

Observation 1: By configuring dedicated PRACH resources for RedCap UE, NW is able to identify the RedCap UE during initial access. Based on the PRACH preamble transmitted by RedCap UE, NW is able to:

- schedule msg3/PUCCH (HARQ feedback for msg4/msgB) properly within the initial UL BWP of RedCap UE

- enable necessary coverage recovery for msg2/msg3/msgB/msg4/PUCCH by re-using the solutions available in NR R15/16 and R17 coverage enhancement WI

In NR R15/16, the initial UL and DL BWPs are required to be aligned at center frequency in TDD bands. In NR R17, RedCap UE should follow the same rule for BWP configuration. In addition, the RACH occasions (ROs) configured for RedCap UE should be contained within its initial UL BWP, as shown in Figure 1.

Figure 1: Initial UL BWP and RO Configuration for RedCap UE

The initial UL BWP of non-RedCap UE can be wider than 20 MHz.  For the example shown in Figure 2, we assume the initial UL BWP of non-RedCap UE is 40 MHz, and there are 8 FDMed ROs shared by all UEs (non-RedCap and RedCap), which spans 17.28 MHz for SCS 15 kHz and 34.56 MHz for SCS 30 kHz. If the starting PRB of RedCap UE’s initial UL BWP is aligned with that of non-RedCap UE, and RedCap UE needs to select a RO/PRACH preamble outside its UL BWP (e.g. M>3), it is infeasible for RedCap UE to reach the desired RO through center frequency change/retuning. To illustrate, Figure 2 shows three BWPs (A, B and C) with 20 MHz BW. In particular, BWP B and C are obtained by retuning/switching the center frequency of initial BWP. However, the two BWPs partially overlap with the ROs configured by NW. Therefore, there is a conflict in the RO configurations for BWP B, C and the initial UL BWP of non-RedCap UE. To transmit on the desired RO, RedCap UE can switch to BWP A, which requires spec change in RAN1/RAN2/RAN4 to support BWP switching during initial access.

Figure 2: RO Configuration/Selection Outside Initial UL BWP of RedCap UE

Due to the spec impacts and complexity increase for RedCap UE, BWP switching should not be allowed during initial access. To illustrate the complexity increase incurred by BWP switching, Figure 3 shows the UL BWP switching occasions during 4-step RACH. If RedCap UE is configured with a “virtual” BWP wider than its max BW (e.g. virtual UL BWP of RedCap UE = initial UL BWP of non-RedCap UE in Figure 3) and BWP switching is allowed during initial access, RedCap UE needs to perform multiple switching in UL and DL before establishing a RRC connection with NW. In addition, the impacts on RACH procedures need to evaluated since the BWP switching time needs to be accounted for in the RAR and contention resolution windows. Furthermore, the DL/UL BWP pairing rule needs to be revisited for RedCap UE at least in TDD bands. Such standardization efforts are not justified for RedCap UE, especially when it involves an increase of UE’s complexity and power consumption.

Figure 3: BWP Switching During Initial Access Increases RedCap UE’s Complexity and Power Consumption

Observation 2: Fast RF switching (changing center frequency of BWP) cannot solve the RO selection issue of RedCap UE, if the ROs configured for RedCap UE are outside its initial UL BWP.

Observation 3: Specifying multiple initial UL BWPs for RedCap UE and allowing BWP switching during initial access will increase the signaling overhead of NW, complexity of UE, and the spec impacts of RAN1/RAN2/RAN4.

Observation 4: When the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE, NW should configure a separate initial UL BWP for RedCap UE and ensure the ROs configured for RedCap UE are within the initial UL BWP of RedCap UE.

Proposal 3: During initial access, a RedCap UE should not be configured with an initial UL BWP wider than its max BW.

Proposal 4: When RedCap UE and non-RedCap UE co-exist in a cell, and the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE:

- the initial UL BWP of RedCap UE should be separately configured

- the initial UL BWP of RedCap UE can be nested within the initial UL BWP of non-RedCap UE

- the initial UL BWP of RedCap UE can be configured by SIB1, or by rules defined in standards

- in TDD bands, the initial UL and DL BWPs of RedCap UE should be aligned at the center frequency

- dedicated PRACH resources for RedCap UE should be configured within its initial UL BWP, and the early indication in msg1 or msgA preamble can indicate the presence of RedCap UE and its TA/RTT to facilitate:

  - scheduling msg2/PUCCH (HARQ feedback for msg4/msgB) within the initial UL BWP of RedCap UE

  - DL/UL coverage recovery for RedCap UE

  - request for on-demand SI dedicated to RedCap UE

Proposal 5: When RedCap UE and non-RedCap UE co-exist in a cell, and the initial UL BWP of non-RedCap UE is no wider than the max BW of RedCap UE:

- RedCap UE and non-RedCap UE can share the same initial UL BWP

- dedicated PRACH resources for RedCap UE should be configured within its initial UL BWP, and the early indication in msg1 or msgA preamble can indicate the presence of RedCap UE and its TA/RTT to facilitate:

  - DL/UL coverage recovery for RedCap UE

  - request for on-demand SI dedicated to RedCap UE

To minimize the spec impacts, RedCap UE can re-use the long (LRA=839) and short (LRA=139) PRACH preamble sequences introduced in NR R15. To ensure the ROs of RedCap UE can be associated with the best/appropriate SSB beams, gNB can configure dedicated PRACH resources for RedCap UE within its initial UL BWP. When the initial UL BWP of non-RedCap UE is wider than 20 MHz, the RRC parameters for RACH procedure of RedCap UE, such as msg1-FDM, msg1-FrequencyStart, ra-ResponseWindow, ssb-perRACH-OccasionAndCB-PreamblesPerSSB, and SSB-to-RO association pattern period can be separately configured when necessary. Therefore, we have the following proposals:

Proposal 6: RedCap UE can re-use the long and short PRACH preamble sequences introduced in NR R15.

- In FD-FDD or Type-A HD-FDD operation, RedCap UE can re-use the PRACH configurations for FR1 on paired spectrum (Table 6.3.3.2-2, TS 38.211).

- In TDD operation, RedCap UE can re-use the PRACH configurations for FR1 on unpaired spectrum (Table 6.3.3.2-3, TS 38.211).

Proposal 7: To facilitate early indication of RedCap UE during initial access, dedicated PRACH resources should be configured in SIB1 for RedCap UE within its initial UL BWP. The following options can be considered:

- Option 1: dedicated ROs for RedCap UE

- Option 2: dedicated preamble group for RedCap UE on ROs shared with non-RedCap UE

- Option 3: dedicated PRACH preamble format for RedCap UE

## BWP Operation after Initial Access

- For non-RedCap UE, the BW of initial UL BWP can be wider than 20 MHz, which is beyond the BW capability of RedCap UE during initial access

- When intra-slot frequency hopping is enabled for msg3, and the frequency span of two hops is wider than 20 MHz:

  - RedCap UE requires Tgap for retuning

  - Non-RedCap UE does not require a retuning gap

- Without early RedCap indication by PRACH, gNB does not know the presence of RedCap UE and cannot identify the origin of msg3

- When gNB attempts to decode msg3 from a RedCap UE and interprets Tgap as part of the 2nd hop of a non-RedCap UE, channel estimation is messed up and msg3 decoding fails

- gNB cannot decode msg3 of RedCap UE

- RedCap UE cannot access the network

After RRC connection is established, a RedCap UE can switch to an active DL/UL BWP by re-using the BWP switching mechanism in NR R15/16. In the DL BWP (initial, active, default) configured for RedCap UE, it is desirable for NW to schedule a SSB (on or off the NR sync raster) to reduce UE’s complexity of RRM measurement. On the other hand, further relaxation of RRM/RLM measurements can be considered for RedCap UEs with low mobility [2].

In TDD bands, a DL BWP of RedCap UE with index provided by BWP-Id is linked with a UL BWP of the RedCap UE with the same BWP-Id. Besides, a RedCap UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP with the same BWP-Id.

Similar to a non-RedCap UE, a RedCap UE can be configured by higher layers a set of at most four BWPs for receptions by the UE (DL BWP set) in a DL bandwidth by RRC parameters BWP-Downlink-RedCap or initialDownlinkBWP-RedCap by BWP-DownlinkCommon-RedCap and BWP-DownlinkDedicated-RedCap, and a set of at most four BWPs for transmissions by the UE (UL BWP set) in an UL bandwidth by RRC parameters BWP-Uplink-RedCap or initialUplinkBWP-RedCap configured by BWP-UplinkCommon-RedCap and BWP-UplinkDedicated-RedCap.

Out of the considerations for power saving, interference management and traffic offloading, the BWP switching procedures specified for non-RedCap UE should be supported by RedCap UE. Since the use cases of RedCap UE are not latency-sensitive, the Type-2 timeline of DCI based BWP switching (shown in Table 1) should be adopted as a baseline. On UL or DL, a RedCap UE can be configured with BWPs with different starting RB, same BW and same numerology. BWPs with different starting PRB positions should be configured with different BWP-Id. By default, a RedCap UE shall support the intra-slot frequency hopping and inter-slot frequency hopping within its initial or active UL BWP.

BWP switching can occur within or beyond the max BW of RedCap UE. Switching between BWPs with different starting PRB position and same BW resembles sub-band based frequency hopping, but the guard time required for BWP switching is much longer than that required for intra-BWP frequency hopping.  It is not necessary for RedCap UE to support a BWP switching timeline faster than Type-1, since it is beyond the capabilities of non-RedCap UE and the latency of RRC re-configuration is more than the guard period required by RF re-tuning. Furthermore, frequent RF switching can compromise the performance of both NW and UE, since it increases UE’s power consumption, reduces the spectral efficiency of NW, and increases the complexity of measurements, channel estimation and scheduling.

Table 1: Type 1 and Type 2 BWP Switching Timeline Based on DCI

To illustrate the diversity gain achievable by frequency hopping within 100 MHz channel BW, Figure 3 shows two scenarios with different BW configurations. To decouple the coding gain from HARQ/IR combining, RV0 is applied to the two repetitions with and without frequency hopping. The example on the left is a RedCap UE with ~1 MHz BW (6 RBs, 15 kHz SCS)  hopping on 20 MHz channel BW, and the right one is a RedCap UE with ~20 MHz BW (48 RBs, 30 kHz SCS) hopping on 100 MHz channel BW. Three different MCS are evaluated. For two adjacent hops, the frequency offset between the starting PRBs is twice the UE BW.

Compared with the baseline of 2 repetitions without frequency hopping, frequency hopping provides ~2 dB gain for the 1 MHz  BW at 10% BLER, whereas the gain for 20 MHz BW is marginal. This is because in a TDL-C channel with 300ns delay spread, 20 MHz BW is sufficient to glean the gain in frequency diversity.

A similar observation holds for frequency hopping of PUSCH on a wider channel BW.

Figure 4: Gain of Frequency Hopping Under Two Scenarios. Left: 1 MHz/hop over 20 MHz Channel BW. Right: 20 MHz/hop over 100 MHz Channel BW.

Observation 5:  When a RedCap UE is operating in a RRC configured BWP without CORESET0, NW should support the following configurations for RedCap UE:

- Periodic TRS for time/frequency tracking

- Dedicated RRC signaling for SI update

- SSB or CSI-RS-Resource-Mobility for RRM measurement of the serving cell

Observation 6: For DL or UL coverage recovery of RedCap UE, the gain of frequency (BWP) hopping diminishes as the BW per hop increases. To improve the SINR of RedCap UE, HARQ combining (based on IR or chase combining) and repetitions with RV cycling can provide more significant gains and less overhead (RS for demodulation and CSI measurements, retuning gap, etc.) than frequency hopping.

Observation 7: BWPs with different starting PRB locations should be configured with different BWP-Id by higher layer. A RedCap UE is not expected to support more than 4 RRC configured DL/UL BWPs on the serving cell.

Observation 8:  Fast and frequent BWP/RF switching degrades the energy/spectral efficiency of both UE and NW, due to the  increased overhead of switching gap, increased UE power consumption, and increased complexity in channel estimation, measurements, and collision/priority handling.

Proposal 8: RedCap UE should support inter-slot or intra-slot frequency hopping within its initial or active UL BWP. Cross-carrier BWP switching is not supported.

Proposal 9: RedCap UE should re-use the BWP switching mechanism of non-RedCap UE. For DCI based switching, RedCap UE should support Type-2 switching delay capability as a baseline.

Proposal 10: RedCap UE should not support BWP switching/hopping/retuning faster than Type-1 switching delay for DCI based switching.

Proposal 11: RedCap UE is not expected to support BWP configuration procedure and switching mechanisms different from non-RedCap UE.

# Supporting BW Reduction of RedCap UE in FR2

In RAN#90e meeting, a Rel-17 work item for support of reduced capability NR devices was approved and the WID was updated in RAN#91e [2]. As part of the work item, it is agreed to specify support for the following for FR2 BW reduction:

- Reduced maximum UE bandwidth:

  - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz

In this section, based on the WID, we present our views on the FR2 reduced maximum UE bandwidth.

The WID defines the following data rates:

- Industrial Wireless Sensors: < 2 Mbps (UL heavy)

- Video Surveillance: UL dominated (Economic: 2 – 4 Mbps, High End: 7.5 - 25 Mbps)

- Wearables: Reference DL/UL = 5-50/2-5 Mbps, peak DL/UL = 150/50 Mbps

Table 2 shows the maximum data rates that can be achieved for different BWs for 1 MIMO layer.

Table 2: Peak Data Rates (Mbps) for SCS 120 kHz (1 Layer) Based on TDD DL:UL = 3:1

| BW (MHz) | DL 64QAM | UL 64QAM |
| --- | --- | --- |
| 25 | 78 | 29 |
| 50 | 156 | 57 |
| 100 | 317 | 116 |

From the table, it can be noted that the data rates required for RedCap use cases may be achieved with smaller BW than the maximum UE BW of 100 MHz. For some use cases, it can be achieved with a BW much less than 100 MHz (e.g., 25 RB).

Observation 9: For FR2, the required data rates for RedCap use cases can be achieved with BW less than 100 MHz

Hence, a UE BW of 100 MHz may not be needed after initial cell search. To reduce the UE BW and thus save power, the UE may switch into a narrower BW active BWP (NBWP).

## Transitioning to a Narrow Active BWP (NBWP)

Due to the possibly large number of RedCap UEs, they may need to be distributed among several NBWPs (i.e., a carrier BW includes multiple NBWPs). To reduce signaling, the network may choose to allow the UEs to implicitly transition into a NBWP after initial access. This can be done by having the UE select a NBWP from a pool of NBWPs (e.g., with equal probability). Another approach is to have UEs select the NBWP based on some hashing function based on a UE ID. This may result in some uniform distribution of the UEs among the NBWPs and hence reduce the overloading and be more resource efficient.

Another aspect to consider is that a UE, based on its measurements (e.g., for interference) or capability may prefer a certain NBWP where the interference is low. Hence it may be desirable to have the option for the UE to initiate/request a preferred NBWP and/or BW.

The initial transition of a UE to one of the NBWPs can be:

- Network initiated/controlled

  - This is already existing in NR R15/16

- Implicit

  - Based on a random selection or some UE ID hashing function

- UE initialed/requested

  - UE may send a preferred max UE BW (≤ 100 MHz) to be used after initial access

  - UE may send a preferred BWP to be used after initial access

Proposal 12: For FR2, to save UE power and complexity, consider switching the UE to a narrow active BWP (NBWP) after initial access is complete. The switching may be:

- Network initiated/controlled (similar to existing NR)

- Implicit (e.g., based on a random selection or some UE ID hashing function)

- UE initiated/requested

## Virtual NBWP Hopping

There are however certain aspects that need to be considered due to the BW reduction of the active BWP.

- Reduce narrowband interference effects

- Get frequency diversity gains

- Optimize operation due to the reduced BW

For a narrow BW UE, to achieve frequency diversity gains, frequency hopping is one of the methods that can be used. However, in FR2, due to beamforming at both gNB and UE, in addition to smaller cells, the delay spread is smaller compared to FR1. This leads to a larger coherence BW and hence less gain using frequency hopping (if the hopping was within a limited frequency range). For FR2, to get the frequency diversity hopping gains, the UE may need to hop across a larger system frequency range (across larger system BW). For example, in case the network supports larger operation bandwidth, e.g., using CA, the UE which only supports up to 100 MHz and single CC may hop in frequency over larger frequency span which may include multiple CCs from gNB point of view.

In addition, for positioning, a design may be considered where the UE uses multiple frequency resources in TDM fashion, to achieve a higher BW capability (e.g., stitch 8x100 MHz to get 800 MHz) thus obtaining a higher positioning accuracy.

Hopping within a limited system BW, however, may still be beneficial to mitigate persistent interference because narrow BW operation may be more prone to such interference (affecting a large portion of the active BWP). This is even more exemplified for stationary devices where the interference is not randomized by the UE movements and may be persisting. It may be beneficial to have some sort of NBWP hopping mechanism, where we consider a “virtual” NBWP that is hopping in the frequency domain (Figure 4), where:

- Resources within the “virtual” BWP are relative to a reference point within the BWP hop that does not change

- The procedures (HARQ, timers, grants, etc.) are transparent to frequency hopping

This simplifies the scheduler and the specification development by having a transparent hopping mechanism.

To illustrate the gains of the NBWP hopping, link and system level evaluations were considered.

Figure 5: “Virtual” BWP Hopping for RedCap UEs

### UL Link-level Simulations

The following assumptions were used for the UL LLS:

| Parameter | Value |
| --- | --- |
| Physical Channel | PUSCH |
| SCS | 120 kHz |
| Channel (Delay Spread) | TDL-A (30 ns) / CDL-A (30 ns) / CDL-B (50 ns) |
| Carrier frequency | 28 GHz |
| Number of RBs (BW) | 14 (20.16 MHz) and 34 (48.96 MHz) |
| Hopping | Inter-slot |
| HARQ RV | 0213 (re-tx on different hops) |
| Num Antenna | 1 Tx /2 Rx |
| Num Layers | 1 |
| 100 MHz Hop RB Start | 20 MHz Allocation: 1, 33, 17, 49 (1st hop distance = 47.52 MHz) 50 MHz Allocation: 1, 17, 8, 32 (1st hop distance = 24.48 MHz) |
| 200 MHz Hop RB Start | 1, 65, 33, 97 (1st hop distance = 93.6 MHz) |
| 400 MHz Hop RB Start | 1, 129, 65, 193 (1st hop distance = 185.76 MHz) |

We have considered 2 allocation BWs (20 MHz and 50 MHz). We have considered the 50 MHz allocation to align with the TS 38.101 requirement of the minimum UE BW support for FR2. However, a UE may be allocated smaller BWs and can choose to reduce its BW to further reduce power. For example, a CORESET0 can be as small as 24 RBs (34.6 MHz) and a UE may choose to use that BW for its BB/RF if allocation does not require any more BW.

Table 3 shows the UL SNR gains that can be achieved from hopping over different BWs (100, 200, 400 MHz) using 1 Tx and 2 Rx. Figure 5 to Figure 10 show the corresponding BLER curves.

The following observations can be made from the results:

Observation 10: For FR2:

- UL hopping across 100 MHz can yield SNR link level PUSCH gains of few dBs (up to 3.5 dB)

  - Hopping for 50 MHz allocation also yields gains (up to 3 dB)

- Some channels (e.g., CDL-A) has larger coherence BW and needs larger BW hop (e.g., 200 MHz) to achieve more gains

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Table 3: PUSCH SNR Gains for Hopping over Different BWs

|  |  |  | SNR Gain (dB) @ 1% BLER |  |  |  |  | SNR Gain (dB) @ 10% BLER |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Allocation | Channel | MCS | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) |
| 20 MHz | CDL-A | 0 | 2.0 | 3.2 | 3.4 | 1.4 | 0.1 | 0.4 | 1.3 | 1.4 | 1.0 | 0.1 |
|  |  | 9 | 2.1 | 3.4 | 3.6 | 1.5 | 0.2 | 0.1 | 0.9 | 1.1 | 1.0 | 0.2 |
|  | CDL-B | 0 | 3.5 | 4.1 | 3.5 | 0.0 | -0.6 | 1.6 | 2.0 | 1.7 | 0.1 | -0.3 |
|  |  | 9 | 3.4 | 3.6 | 3.1 | -0.2 | -0.5 | 1.4 | 1.7 | 1.4 | 0.0 | -0.2 |
|  | TDL-A | 0 | 1.8 | 2.4 | 2.8 | 1.0 | 0.4 | 0.9 | 1.5 | 1.7 | 0.7 | 0.2 |
|  |  | 9 | 1.6 | 2.3 | 2.3 | 0.7 | 0.0 | 0.8 | 1.1 | 1.4 | 0.6 | 0.3 |
| 50 MHz | CDL-A | 0 | 1.0 | 2.8 | 3.0 | 2.0 | 0.2 | 0.2 | 1.3 | 1.4 | 1.3 | 0.1 |
|  |  | 9 | 0.8 | 2.7 | 2.9 | 2.1 | 0.2 | 0.2 | 1.2 | 1.4 | 1.2 | 0.2 |
|  | CDL-B | 0 | 0.8 | 2.1 | 1.8 | 1.1 | -0.3 | 0.4 | 1.2 | 1.2 | 0.7 | -0.1 |
|  |  | 9 | 0.5 | 1.6 | 1.4 | 0.9 | -0.2 | 0.3 | 1.0 | 0.9 | 0.6 | -0.1 |
|  | TDL-A | 0 | 0.5 | 1.4 | 1.9 | 1.5 | 0.5 | 0.5 | 1.1 | 1.3 | 0.9 | 0.2 |
|  |  | 9 | NA | 1.3 | 1.7 | NA | 0.3 | 0.2 | 0.5 | 0.9 | 0.8 | 0.4 |

| Figure 6: Hopping for CDL-A, 20 MHz Allocation | Figure 7: Hopping for CDL-A, 50 MHz Allocation |
| --- | --- |
| Figure 8: Hopping for CDL-B, 20 MHz Allocation | Figure 9: Hopping for CDL-B, 50 MHz Allocation |
| Figure 10: Hopping for TDL-A, 20 MHz Allocation | Figure 11: Hopping for TDL-A, 50 MHz Allocation |

### DL Link-level Simulations

The following assumptions were used for the DL LLS:

| Parameter | Value |
| --- | --- |
| Physical Channel | PDSCH |
| SCS | 120 kHz |
| Channel | TDL-A / CDL-A |
| Carrier frequency | 28 GHz |
| Number of RBs (BW) | 14 (20.16 MHz) |
| Desired delay spread | 30 ns |
| Hopping | Inter-slot |
| HARQ RV | 0213 (re-tx on different hops) |
| Num Layers/Rx Antenna | 1 |
| 100 MHz Hop RB Start | 1, 33, 17, 49 (1st hop distance = 47.52 MHz) |
| 200 MHz Hop RB Start | 1, 65, 33, 97 (1st hop distance = 93.6 MHz) |
| 400 MHz Hop RB Start | 1, 129, 65, 193 (1st hop distance = 185.76 MHz) |
| Antenna configuration for CDL channel model (indoor scenario of Rel-17 CE WI) |  |
| BS antenna | (Mg,Ng,M,N,P) = (1,1,8,8,2) with (0.5 dv, 0.5 dH) |
| UE antenna | (Mg,Ng,M,N,P) = (1,1,2,2,2) with (0.5 dv, 0.5 dH) * Only a single polarization is used for 1 Rx |

Table 4 shows the DL SNR gains that can be achieved from hopping of a 20 MHz allocation over different BWs (100, 200, 400 MHz) using 2 Tx and 1 or 2 Rx. Figure 11 to Figure 18 show the corresponding BLER curves.

The following observations can be made from the results:

Observation 11: For FR2:

- Hopping across over 100 MHz can yield SNR link level PDSCH performance gains of few dBs (1-5 dB)

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Table 4: PDSCH SNR Gains for Hopping over Different BWs

|  |  |  | SNR Gain (dB) @ 1% BLER |  |  |  |  | SNR Gain (dB) @ 10% BLER |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel | Num Rx | MCS | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) | 100 MHz (wrt no Hop) | 200 MHz (wrt no Hop) | 400 MHz (wrt no Hop) | 400 MHz (wrt 100 MHz) | 400 MHz (wrt 200 MHz) |
| TDL-A | 1 | 0 | 3.1 | 3.5 | 4.1 | 1.1 | 0.6 | 1.6 | 2.1 | 2.4 | 0.8 | 0.3 |
|  |  | 9 | 2.7 | 3.7 | 3.9 | 1.2 | 0.3 | 1.4 | 1.8 | 2.4 | 1.0 | 0.6 |
|  | 2 | 0 | 1.6 | 2.4 | 2.6 | 0.9 | 0.2 | 0.9 | 1.4 | NA | NA | NA |
|  |  | 9 | 1.5 | 2.1 | 2.3 | 0.8 | 0.2 | 0.7 | 1.2 | 1.3 | 0.6 | 0.1 |
| CDL-A | 1 | 0 | 5.0 | 6.8 | 6.9 | 1.9 | 0.1 | 1.6 | 2.8 | 2.9 | 1.3 | 0.1 |
|  |  | 9 | NA | NA | NA | 2.0 | -0.1 | 1.5 | 2.8 | 2.6 | 1.2 | -0.2 |
|  | 2 | 0 | 2.3 | 3.8 | 3.6 | 1.3 | -0.2 | 1.1 | 2.0 | 2.0 | 0.9 | 0.0 |
|  |  | 9 | 2.1 | 3.4 | 3.4 | 1.4 | 0.0 | 0.9 | 1.6 | 1.6 | 0.8 | 0.0 |

| Figure 12: Hopping for TDL-A, 1Rx, MCS0 | Figure 13: Hopping for CDL-A, 1Rx, MCS0 |
| --- | --- |
| Figure 14: Hopping for TDL-A, 1Rx, MCS9 | Figure 15: Hopping for CDL-A, 1Rx, MCS9 |
| Figure 16: Hopping for TDL-A, 2Rx, MCS0 | Figure 17: Hopping for CDL-A, 2Rx, MCS0 |
| Figure 18: Hopping for TDL-A, 2Rx, MCS9 | Figure 19: Hopping for CDL-A, 2Rx, MCS9 |

### UL System-level Simulations

The following assumptions were used for the UL SLS:

| Parameters | Value |
| --- | --- |
| Deployment | Indoor hotspot (8, 24 UEs/cell), UMi (10, 20 UEs/cell) |
| Beam Management | Disabled |
| Antenna array | Indoor hotspot (128), UMi (256) |
| Allocation BW | 20 MHz |
| Hopping | Hop over 4 sub-bands (25/100 MHz apart for 100/400 MHz hopping) |
| Antenna array | Indoor hotspot (32/128), UMi (64/256) |
| EIRP = Tx Power + Antenna gain + Array gain | 60 dBm = 28 dBm (TxP) + 8 dBi (ant gain) + 24 dB (array gain = 10*log10(256)) |
| Carrier frequency | 28 GHz |
| UE Tx Power | 14 dBm |
| Polarization | 2 |
| MIMO | SU-MIMO only (max Rank = 2) |
| Traffic | Video surveillance (480p@30fps = 2.5Mbps) |
| Frame structure | DDDSU |
| Direction | UL |
| Max Modulation | 64QAM |

Figure 19 and Figure 20 shown the SINR and mean UPT distributions for different deployments and number of UEs.

| Figure 20: SLS SINR Distributions | Figure 21: SLS UPT Mean Distributions |
| --- | --- |

The following observations can be made from the results:

Observation 12: For FR2:

- BWP hopping can yield overall SINR and mean UPT gains

- Hopping across larger BW (e.g., 400 MHz) may be needed to get SINR gains for some small delay spread cases (e.g., UMi)

Proposal 13: For FR2, consider introducing “virtual” BWP hopping to achieve frequency diversity gains and reduce the NB interference effects

- Resources within the “virtual” BWP are relative to a reference point within the BWP hop that does not change

- The procedures (HARQ, timers, grants, etc.) are transparent to frequency hopping

- Consider supporting hopping across larger BW (e.g., > 100 MHz) to achieve frequency diversity gains for low delay spread deployments

## Reducing the Effect of BWP Switching

UE hopping across frequency (e.g., using NBWP hopping) may lead to utilization issues in time due to the switching gaps defined in TS 38.133 (based on UE capability). RedCap is a delay tolerant system and some delays may be acceptable. However, it may be desirable to consider techniques to reduce the effect of the hopping switching gaps on messages within the gap. Some example methods to consider:

- Network implementation:

  - Other UEs may be scheduled during the gap, thus not affecting the overall system capacity

- Simplified BWP switching: Define simplified BWP switching by preconfiguring the switching/hopping and by using similar “virtual” BWP hop parameters

  - For DCI-based BWP switching, the switching time/gap is mainly due to UE DCI processing time, modem L1 processing time (loops re-initializations and settling, etc...), and RF retuning time.

  - Reduce the UE complexity by reducing the need for UE DCI and modem L1 processing. This can be achieved by pre-configuring the switches and using the same parameters

- BWP grouping:

  - A BWP group may contain some specific BWPs that when switching among them, the UE may be able to do faster RF retuning as compared to others​ (this can be based on UE capability)

  - For example, typically, a UE may be able to switch faster if the source and target BWP frequencies are close

- Variable BWP hop time (extension):

  - In case a transmission falls into a BWP switching gap, the previous BWP (before the switch) is extended to cover that transmission

- BWP hop skipping/modification:

  - UE may skip or modify BWP hops based on certain conditions that are either signaled to the UE (using RRC/MAC-CE/DCI) or specified. E.g.: no periodic/dynamic signals/messages scheduled in these hops

Proposal 14: For FR2, consider ways to reduce the impact of the BWP switching delays, examples include:

- Simplified BWP switching

  - By preconfiguring the switching/hops and by using similar BWP hop parameters

  - Consider RAN4’s input on this

- BWP grouping

  - Intra-BWP group switching times is smaller than inter-BWP group switching times

  - Consider RAN4’s input on this

- Variable BWP hop time (extension)

- BWP hop skipping/modification

# Conclusions

## BW Reduction in FR1

In this contribution, we have shared our views on the BW reduction for R17 RedCap devices. To summarize, we have the following observations and proposals for FR1:

Observation 1: By configuring dedicated PRACH resources for RedCap UE, NW is able to identify the RedCap UE during initial access. Based on the PRACH preamble transmitted by RedCap UE, NW is able to:

- schedule msg3/PUCCH (HARQ feedback for msg4/msgB) properly within the initial UL BWP of RedCap UE

- enable necessary coverage recovery for msg2/msg3/msgB/msg4/PUCCH by re-using the solutions available in NR R15/16 and R17 coverage enhancement WI

Observation 2: Fast RF switching (changing center frequency of BWP) cannot solve the RO selection issue of RedCap UE, if the ROs configured for RedCap UE are outside its initial UL BWP.

Observation 3: Specifying multiple initial UL BWPs for RedCap UE and allowing BWP switching during initial access will increase the signaling overhead of NW, complexity of UE, and the spec impacts of RAN1/RAN2/RAN4.

Observation 4: When the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE, NW should configure a separate initial UL BWP for RedCap UE and ensure the ROs configured for RedCap UE are within the initial UL BWP of RedCap UE.

Observation 5:  When a RedCap UE is operating in a RRC configured BWP without CORESET0, NW should support the following configurations for RedCap UE:

- Periodic TRS for time/frequency tracking

- Dedicated RRC signaling for SI update

- SSB or CSI-RS-Resource-Mobility for RRM measurement of the serving cell

Observation 6: For DL or UL coverage recovery of RedCap UE, the gain of frequency (BWP) hopping diminishes as the BW per hop increases. To improve the SINR of RedCap UE, HARQ combining (based on IR or chase combining) and repetitions with RV cycling can provide more significant gains and less overhead (RS for demodulation and CSI measurements, retuning gap, etc.) than frequency hopping.

Observation 7: BWPs with different starting PRB locations should be configured with different BWP-Id by higher layer. A RedCap UE is not expected to support more than 4 RRC configured DL/UL BWPs on the serving cell.

Observation 8:  Fast and frequent BWP/RF switching degrades the energy/spectral efficiency of both UE and NW, due to the  increased overhead of switching gap, increased UE power consumption, and increased complexity in channel estimation, measurements, and collision/priority handling.

Proposal 1: During initial access, RedCap and non-RedCap UEs share the same SSB, CORESET#0, SIB1 and initial DL BWP.

Proposal 2: Other SIBs of RedCap UE can either be scheduled by SIB1, or be transmitted on-demand within the initial DL BWP of RedCap UE.

Proposal 3: During initial access, a RedCap UE should not be configured with an initial UL BWP wider than its max BW.

Proposal 4: When RedCap UE and non-RedCap UE co-exist in a cell, and the initial UL BWP of non-RedCap UE is wider than the max BW of RedCap UE:

- the initial UL BWP of RedCap UE should be separately configured

- the initial UL BWP of RedCap UE can be nested within the initial UL BWP of non-RedCap UE

- the initial UL BWP of RedCap UE can be configured by SIB1, or by rules defined in standards

- in TDD bands, the initial UL and DL BWPs of RedCap UE should be aligned at the center frequency

- dedicated PRACH resources for RedCap UE should be configured within its initial UL BWP, and the early indication in msg1 or msgA preamble can indicate the presence of RedCap UE and its TA/RTT to facilitate:

  - scheduling msg3/PUCCH (HARQ feedback for msg4/msgB) within the initial UL BWP of RedCap UE

  - DL/UL coverage recovery of RedCap UE

  - request for on-demand SI dedicated to RedCap UE

Proposal 5: When RedCap UE and non-RedCap UE co-exist in a cell, and the initial UL BWP of non-RedCap UE is no wider than the max BW of RedCap UE:

- RedCap UE and non-RedCap UE can share the same initial UL BWP

- dedicated PRACH resources for RedCap UE should be configured within its initial UL BWP, and the early indication in msg1 or msgA preamble can indicate the presence of RedCap UE and its TA/RTT to facilitate:

  - DL/UL coverage recovery of RedCap UE

  - request for on-demand SI dedicated to RedCap UE

Proposal 6: RedCap UE can re-use the long and short PRACH preamble sequences introduced in NR R15.

- In FD-FDD or Type-A HD-FDD operation, RedCap UE can re-use the PRACH configurations for FR1 on paired spectrum (Table 6.3.3.2-2, TS 38.211).

- In TDD operation, RedCap UE can re-use the PRACH configurations for FR1 on unpaired spectrum (Table 6.3.3.2-3, TS 38.211).

Proposal 7: To facilitate early indication of RedCap UE during initial access, dedicated PRACH resources should be configured in SIB1 for RedCap UE within its initial UL BWP. The following options can be considered:

- Option 1: dedicated ROs for RedCap UE

- Option 2: dedicated preamble group for RedCap UE on ROs shared with non-RedCap UE

- Option 3: dedicated PRACH preamble format for RedCap UE

Proposal 8: RedCap UE should support inter-slot or intra-slot frequency hopping within its initial or active UL BWP. Cross-carrier BWP switching is not supported.

Proposal 9: RedCap UE should re-use the BWP switching mechanism of non-RedCap UE. For DCI based switching, RedCap UE should support Type-2 switching delay capability as a baseline.

Proposal 10: RedCap UE should not support BWP switching/hopping/retuning faster than Type-1 switching delay for DCI based switching.

Proposal 11: RedCap UE is not expected to support BWP configuration procedure and switching mechanisms different from non-RedCap UE.

## BW Reduction in FR2

For FR2, we have the following observations and proposals:

Observation 9: For FR2, the required data rates for RedCap use cases can be achieved with BW less than 100 MHz

Observation 10: For FR2:

- UL hopping across 100 MHz can yield SNR link level PUSCH gains of few dBs (up to 3.5 dB)

  - Hopping for 50 MHz allocation also yields gains (up to 3 dB)

- Some channels (e.g., CDL-A) has larger coherence BW and needs larger BW hop (e.g., 200 MHz) to achieve more gains

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Observation 11: For FR2:

- Hopping across over 100 MHz can yield SNR link level PDSCH performance gains of few dBs (1-5 dB)

- Hopping across larger BW (e.g., 200 MHz or 400 MHz) has an additional ~ 1-2 dB gain

- The main difference between the CDL and TDL gains is due to beamforming used for CDL channels

- The gain difference between hopping over 200 MHz and 400 MHz is small

Observation 12: For FR2:

- BWP hopping can yield overall SINR and mean UPT gains

- Hopping across larger BW (e.g., 400 MHz) may be needed to get SINR gains for some small delay spread cases (e.g., UMi)

Proposal 12: For FR2, to save UE power and complexity, consider switching the UE to a narrow active BWP (NBWP) after initial access is complete. The switching may be:

- Network initiated/controlled (similar to existing NR)

- Implicit (e.g., based on a random selection or some UE ID hashing function)

- UE initiated/requested

Proposal 13: For FR2, consider introducing “virtual” BWP hopping to achieve frequency diversity gains and reduce the NB interference effects

- Resources within the “virtual” BWP are relative to a reference point within the BWP hop that does not change

- The procedures (HARQ, timers, grants, etc.) are transparent to frequency hopping

- Consider supporting hopping across larger BW (e.g., > 100 MHz) to achieve frequency diversity gains for low delay spread deployments

Proposal 14: For FR2, consider ways to reduce the impact of the BWP switching delays, examples include:

- Simplified BWP switching

  - By preconfiguring the switching/hops and by using similar BWP hop parameters

  - Consider RAN4’s input on this

- BWP grouping

  - Intra-BWP group switching times is smaller than inter-BWP group switching times

  - Consider RAN4’s input on this

- Variable BWP hop time (extension)

- BWP hop skipping/modification

# References

- 3GPP TR 38.875, “Study on Support of Reduced Capability NR Devices,” December 2020.

- RP-210918, “Revised WID on Support of Reduced Capability NR Devices,” 3GPP TSG RAN#91e, March 22nd to 26th ,  2021.