3GPP TSG-RAN WG1 Meeting #105-e	R1-2104754

e-Meeting, May 10 – 27, 2021

Title:	[Draft] Reply LS R16 V2X for PUCCH reporting and for minimum time gap

Response to:	R2-2104475

Release:	Rel-16

Work Item:	5G_V2X_NRSL

Source:	OPPO [to be RAN1]

To:	RAN2

Cc:	-

Contact Person:

#### Name:	Kevin Lin

Tel. Number:

E-mail Address:	Kevin.Lin@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 thank RAN2’s LS in R2-2104475 (R1-2104559) and would like to provide our response to the following questions from RAN2.

Working assumption: “UE assumes that next retransmission(s) of the MAC PDU is required when FB is disabled, for CG, if sl-CG-MaxTransNumList is configured with a value not larger than the number of CG resources, when sl-CG-MaxTransNum is not reached”

Q1: RAN2 respectfully requests RAN1 to provide feedback on the working assumption above in case of any concern.

Answer to Q1: RAN1 has no concern on the working assumption made by RAN1 in Q1.

5.22.1.2 TX resource (re-)selection check […] 1> if retransmission of a MAC PDU on the selected sidelink grant has been dropped by either sidelink congestion control as specified in clause 8.1.6 of TS 38.214 or de-prioritization as specified in clause 16.2.4 of TS 38.213 [6], clause 5.4.2.2 of TS 36.321 [22] and clause 5.4.2.2: 2> remove the resource(s) from the selected sidelink grant associated to the Sidelink process, if the resource(s) of the selected sidelink grant is indicated for re-evaluation or pre-emption by the physical layer; 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of a SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];

Q2: RAN2 respectfully requests RAN1 to provide feedback in case of any concern on the MAC specification above.

Answer to Q2: RAN1 has no concern on the current MAC layer specification description raised in Q2.

2. Actions:

To RAN2 group

RAN1 respectfully requests RAN2 to take the above responses into account in the future work.

3. Date of Next TSG-RAN1 Meetings:

RAN1#106-e	16-27 August 2021	E-Meeting

RAN1#106b-e	11-19 October 2021   	E-Meeting