3GPP TSG RAN WG1 #105-e		R1-2104781

e-Meeting, May 10th -27th , 2021

Source:	OPPO

Title:	Discussion on other aspects

Agenda Item:	8.15.5

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide the details of the link budgets evaluation for the MEO Set-5 for NB-IoT/eMTC over NTN.

# Evaluation assumptions

The methodology provided in TR 38.821 [1] and the agreed reference scenario parameters for study of NB-IoT/eMTC over NTN in RAN1 meeting are considered to evaluate the link budget.

  - Agreed reference scenario parameters

In RAN1 meeting #104b-e, the following agreements regarding MEO Set-5 satellite parameters for study of NB-IoT/eMTC over NTN were made.

Agreement:

Include MEO Set-5 parameters for link budget analysis in a new Table 6.2-8 in TR 36.763, as a representative characterization of NTN-IoT scenarios with MEO altitude and characteristics:

Table 6.2-8: Sets of satellite parameters for link budget and system level evaluations

|  | Proposed MEO Scenarios (Set 5) |
| --- | --- |
| Satellite orbit | MEO |
| Satellite altitude | 10,000 km |
| Payload characteristics for DL transmission |  |
| Frequency band | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 1.5 m |
| Satellite EIRP density | 45.4 dBW/MHz |
| Satellite Tx max Gain | 28.1 dBi |
| 3dB beamwidth | 6.5 degrees |
| Satellite beam diameter (at nadir pointing) | 1140 km |
| Payload characteristics for UL reception |  |
| Frequency band | S-band (i.e. 2 GHz) |
| Equivalent satellite antenna aperture (NOTE1) | 1.5 m |
| G/T | 3.8 dB/K |
| Satellite Rx max Gain | 28.1 dBi |
| NOTE 1: This value is equivalent to the antenna diameter for the parabolic reflector modelled in Sec. 6.4.1 of TR 38.811. Other antenna models can be considered. |  |

Agreement:

Add MEO Set-5 satellite parameters for system level simulator calibration  in a new Table 6.2-9 in TR 36.763:

Table 6.2-9: Set-5 parameters for link budget analysis

| Set 5 | MEO |
| --- | --- |
| 3 dB Beam width (HPBW) | 6.5 degrees |
| Central beam center elevation | 90 degrees |
| Central beam edge elevation | 81.6 degrees |
| Central beam edge satellite-UE distance | 10042 km |

  - Link Budget Calculation

Section 6.1.3.1 in TR38.821 [1] is considered and cited here.

Carrier-to-noise-and-interference ratio (CNIR) of transmission link between satellite and UE can be derived by carrier-to-noise ratio (CNR) and carrier-to-interference ratio (CIR) as follows

CNIR dB=-10log1010-0.1CNR [dB]+10-0.1CIR [dB]	(6.1.3.1-1)

The formula for CNR calculation is

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]	(6.1.3.1-2)

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz, PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLSM is shadowing margin, PLSL[dB] is scintillation loss, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth.

Antenna-gain-to-noise-temperature GT can be derived by [2]

GTdB=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB]	(6.1.3.1-3)

where GR is receive antenna gain, Nf is noise figure, T0 is ambient temperature and Ta is antenna temperature. Receive antenna gain GR can be obtained by

GR dBi=GR,edBi+10log10NR,a-Lp[dB],  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna 	(6.1.3.1-4)

where GR,e is receive antenna element gain, NR,a is the number of receive antenna elements, Lp is polarization loss, η is the antenna aperture efficiency (a dimensionless parameter with typical values for parabolic antennas from 0.55 to 0.70), D is the equivalent antenna diameter, and λ is the wavelength.

EIRP can be calculated by

EIRP dBW=PT dBW-LC dB+GT [dBi]	(6.1.3.1-5)

where PT is antenna transmit power, LC is cable loss, and GT is transmit antenna gain and can be derived by

GT dBi=GT,e dBi+10log10NT,a,  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna	(6.1.3.1-6)

where GT,e is transmit antenna element gain and NT,a is the number of transmit antenna elements.

  - Parameters

The parameter configuration for link budget analysis is listed in Table 1, which is updated based on Table 6.1.3.2-1 in TR38.821.

Table 1. Parameter configuration for link budget analysis

| Parameters | Notes |
| --- | --- |
| Carrier frequency | 2 GHz for DL and UL (S-band) |
| System bandwidth | - NB-IoT: 180 kHz - eMTC: 1080 kHz |
| Channel bandwidth | - NB-IoT: 180 kHz (DL), Up to 180 kHz with all permissible smaller resource allocations 12*15 kHz, 6*15 kHz, 3*15 kHz, 1*15 kHz, 1*3.75 kHz - eMTC: 1080 kHz (DL), Up to 1080 kHz with all permissible smaller resource allocations , including 2*180 kHz, 180 kHz, 2*15 kHz or 3*15 kHz or 6*15 kHz (UL) |
| Satellite altitude | 10000 km |
| Target elevation angle | 10 |
| Atmospheric loss | Equation (6.6-8) in [2] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) |
| Additional loss | DL: 0 dB, UL: 3dB |
| Clear sky conditions | Yes |
| Frequency reuse factor | 1 |
| Average CIR within a satellite beam based on logarithmic mean | Based on single satellite system-level calibration methodology, statistics for average CIR are only collected for the UEs located in the central beam of the 19-beamlayout. The central beam boresight direction is computed based on the target elevation angle assumption. When the generated beam has a partial or full coverage outside the earth, it is discarded. For DL calibration, CIR is computed by averaging CIR over UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-1 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment). For UL calibration, For Handheld device, the channel bandwidth is 360 kHz. For VSAT, the channel bandwidth equals the system bandwidth allocated to each beam divided by 10. The devices in one beam are allocated on adjacent frequency resources. The same resource allocation is assumed for all the beams. CIR is computed by averaging over 10 simultaneously transmitting UEs randomly distributed over the reference beam (UE distribution assumption of Table 6.1.1.1-5). (See Figure 6.1.3.2-2 for UE bandwidth allocation, and Figure 6.1.1.1-1 and Figure 6.1.1.1-2 for beam deployment) The averaging should be performed over multiple realizations. |
| Satellite antenna polarization | Circular polarization |
| Terminal type | S band: (M, N, P) = (1,1,2) |
| Free space path loss | Equation (6.6-2) in [2] |
| Terminal RF parameters | Table 6.1.1-3 |
| Satellite RF parameters | Set-1 in Table 6.1.1-1 and Set-2 in Table 6.1.1-2 |
| Polarization loss | The considerations of Section 6.1.1.1 on Polarization loss apply. |
| Outcome | CNIR |
| NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. dB |  |

# Evaluation results

  - CNR results

The CNR results for both DL and UL for link budgets evaluation under satellite Set-5 are provided in Table 2 and Table 3.

Table 2. Link budget results for NB-IoT in Satellite set 5

| NB-IoT |  | DL | UL |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth (kHz) |  | 180 | 180 | 90 | 45 | 15 | 3.75 |
| Scenario D | CNR (dB) | -4.42 | -16.95 | -13.94 | -10.93 | -6.16 | -0.14 |

Table 3. Link budget results for eMTC in Satellite set 5

| eMTC |  | DL | UL |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth (kHz) |  | 1080 | 360 | 180 | 90 | 45 | 30 |
| Scenario D | CNR (dB) | -4.42 | -19.96 | -16.95 | -13.94 | -10.93 | -9.17 |

  - Agreed reference scenario parameters

The central beam edge DL CNR and UL CNR are considered for link budget analysis, where 0 dB and 3 dB additional loss due to beamwidth defined by HPBW is considered for DL CNR and UL CNR respectively. Moreover, the detailed evaluation results of the link budgets for the scenarios for NB-IoT/eMTC over NTN are provided in the accompanied excel file. The following assumptions are used for both DL and UL transmission of NB-IoT and eMTC.

- UE power class 3 with up to 200 mW (23 dBm), Noise Figure 7 dB, and Satellite parameter set 5.

- Others losses for MEO Set-5 in Table 4

Table 4. Others losses for MEO Set-5

| Other Losses | MEO (10000 km) |
| --- | --- |
| Scintillation losses | 2.2 |
| Atmospheric losses | 0.1 |
| Polarization loss | 3 |
| Shadow margin | 3 |

# Conclusion

This contribution provide the detailed evaluation assumptions and results of the link budgets for the MEO scenarios for NB-IoT/eMTC over NTN.

# Reference

| [1] | TR 38.821 V16.0.0, Solutions for NR to support non-terrestrial networks |
| --- | --- |