3GPP TSG RAN WG1 Meeting #105-e                                     R1-2104882

e-Meeting, 19th May – 27th May 2021

Source:               TCL Communication

Title:                    Discussion on joint channel estimation for PUSCH

Agenda item:      8.8.1.3

Document for:    Discussion and Decision

- Introduction

In the RAN1#104b-e meeting, the following agreements about joint channel estimation for PUSCH were achieved.

Agreements: - For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements. - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window - FFS the units the time domain window (e.g. repetitions, slots, and/or symbols) - FFS : association between the potential use case(s) and units of the time window - FFS: single or multiple time domain windows - FFS: relation with UE capability - FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms - FFS whether or not to further consider impacting of timing advance Agreement: - A new DMRS pattern equally spaced among PUSCH transmissions is not considered for joint channel estimation in Rel-17. Agreements: - For inter-slot frequency hopping with inter-slot bundling, down select on the following two options: - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size. - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size. - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined. - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD. - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size Conclusion: - For optimization of DMRS granularity in time domain with joint channel estimation, the proponents are encouraged to provide more simulation results in next meeting Agreements: - For the time domain window for joint channel estimation, down select on the following two options: - Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions: - PUSCH repetition type A - PUSCH repetition type B, if agreed - TBoMS, if agreed - Different TB, if agreed - Option 2: The unit of the time domain window is the same for the following PUSCH transmission: - PUSCH repetition type A - PUSCH repetition type B, if agreed - TBoMS, if agreed - Different TB, if agreed Agreement: - For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases: - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A. - FFS: additional specification enhancements on top of that defined to support repetition Type A - Only for single layer transmissions - Subject to UE capability - FFS: Over back-to-back PUSCH transmissions with different TBs

This contribution discusses the potential enhancement for joint channel estimation.

- Time-domain window for joint channel estimation

To enable joint channel estimation, a topic about a time domain window is introduced in AI 8.8.1.3, where a UE is expected to keep power consistency and phase continuity among PUSCH transmissions during the window.

In the RAN1#104b-e meeting, PUSCH transmissions with repetition type A or repetition type B, and back-to-back PUSCH transmission with different TBs were agreed to be considered for joint channel estimation.

For TBoMS, it is most likely to adopt PUSCH repetition type A like TDRA  and/or PUSCH repetition type B like TDRA, so it better to support the joint channel estimation, and perhaps reuse the joint channel estimation defined for PUSCH repetition type A/B.

Proposal 1: Joint channel estimation should be supported for TBoMS PUSCH transmission.

  - Non back-to-back PUSCH transmissions

Non back-to-back transmission, such as PUSCH repetition type A and TBoMS (with repetition type A like TDRA), with non-zero gap in-between adjacent transmissions, to keep phase consistency, according to RAN4 LS reply must meet the conditions, such as no change of modulation order and frequency resources of each repetition, no downlink reception in-between the PUSCH/PUCCH repetitions in TDD case.

In this way, the time domain window can be configured in terms of slot, where with zero-gap in-between adjacent windows, as shown in Fig. 1.

Figure 1 Non back-to-back transmission w/ inter-slot time domain window

Proposal 2: For non-back-to-back transmission with non-zero gap in-between adjacent transmission, the time domain window can be configured in terms of slot and with a zero gap in-between adjacent windows.

Considering the importance of time-domain resources for coverage enhancement, especially for TDD case, a non-zero gap in-between adjacent windows can be supported in which UE can use different phases and/or power level for other uplink transmissions or for downlink reception in TDD case, as shown in Figure 2 below. At this point, the time domain window can be configured in terms of repetition.

Figure 2 Non back-to-back transmissions with inter-transmission time domain window

Proposal 3: For non-back-to-back transmission with non-zero gap in-between adjacent transmission, the time domain window can be configured in terms of transmission and with a non-zero gap in-between adjacent windows.

- In this non-zero gap, UE can perform other uplink transmissions with different phase/power or downlink reception.

  - Back-to-back PUSCH transmission

Back-to-back transmission, such as PUSCH repetition type B and TBoMS (if repetition type A like TDRB) and muti-PUSCH scheduling, with zero gap in-between adjacent transmissions, it is easy to keep phase continuity.  The time domain window can be configured in terms of slot (as shown in Fig. 3).

Figure 3 Back-to-back transmission with inter-slot time domain window

Proposal 4: For back-to-back transmission with zero gap in-between adjacent transmission, the time domain window can be configured in terms of slot.

For PUSCH repetition type B, the time domain window can be further configured in terms of repetition; For multi-pusch scheduling and TBoMS with repetition type B like TDRA, the time domain window can be configured in terms of TB. All of them are shown in Figure 4.

Figure 4 Back-to-back transmission with inter-repetition/TB time domain window

Proposal 5: For back-to-back transmission with zero gap in-between adjacent transmission, the time domain window can be configured in terms of repetition or TB.

- Frequency hopping with bundling

According to the RAN4 LS Relay, phase and power should not be changed during joint channel estimation period. We think that the purpose of introducing bundling is the same as introducing the time domain window. Therefore, the design of Bundling should be consistent with the Time Domain Window.

There is an overlap for the time domain window and the bundle. A rule for avoiding or handling the collision between the time domain window and the bundling should be introduced. The following options can be considered:

- Opt1: Keep only one scheme, either the time domain window or the bundling

- Opt2: UE is not expected to be configured both time domain window(s) and the bundling.

- Opt3: If UE configured both of them, prefer the bundling configuration.

- Opt4: If UE configured both of them, prefer the configuration of the time domain window(s).

- Opt5: The gNB ensures that the two configurations do not conflict.

- Opt6: The bundling is only used for cases with frequency hopping, and the time domain window is used for cases without frequency hopping.

- Opt7. The bundling is only used for cases with frequency hopping, and the time domain window is used for cases with or without frequency hopping. Furthermore, the rules in opt2-opt5 are followed for cases with frequency hopping.

Proposal 6: A rule for avoiding or handling the collision between the time domain window and the bundling should be specified.

- Conclusion

In this contribution the following proposals and observations have been made:

Proposal 1: Joint channel estimation should be supported for TBoMS PUSCH transmission.

Proposal 2: For non-back-to-back transmission with non-zero gap in-between adjacent transmission, the time domain window can be configured in terms of slot and with a zero gap in-between adjacent windows.

Proposal 3: For non-back-to-back transmission with non-zero gap in-between adjacent transmission, the time domain window can be configured in terms of transmission and with a non-zero gap in-between adjacent windows.

- In this non-zero gap, UE can perform other uplink transmissions with different phase/power or downlink reception.

Proposal 4: For back-to-back transmission with zero gap in-between adjacent transmission, the time domain window can be configured in terms of slot.

Proposal 5: For back-to-back transmission with zero gap in-between adjacent transmission, the time domain window can be configured in terms of repetition or TB.

Proposal 6: A rule for avoiding or handling the collision between the time domain window and the bundling should be specified.

References

[1] 3GPP RAN1 Chair’s Notes RAN1#104b-e v012

[2] R4-2103393, “Reply on LS on PUCCH and PUSCH repetition”