3GPP TSG RAN WG1 #105-e			R1-2104890

e-Meeting, May 10th – 27th, 2021

Source:	Intel Corporation

Title:	Correction to PSFCH reception procedure for NACK-only case to mitigate half-duplex issue

Agenda item:	7.2.4

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss technical issue of the Rel.16 NR-V2X sidelink communication for groupcast transmissions with NACK only feedback and propose to address it through Rel.16 correction. It is well known that V2X applications are mission critical and require very high reliability within target communication range that may not be possible to achieve due to simple half-duplex problem.

# Technical Problem of NACK only Groupcast

In V2X sidelink resource allocation Mode-1 and Mode-2, UEs can use groupcast transmissions with NACK only feedback. In this mode of operation, receivers within target communication range from the transmitter provide NACK feedback in case of unsuccessful reception and do not provide ACK in case of successful reception. The problem can be illustrated by the following simple example:

- UE1, UE2, and UE3 operate with groupcast NACK only feedback and are group members (i.e., within target communication range from each other).

- UE1 and UE2 selected/were granted with resources in the same slot and transmitted in slot ‘n’.

- UE3 has successfully received UE1 and UE2 transmissions and thus has not provided HARQ feedback.

- UE1 and UE2 were not able to receive each other transmissions.

- Due to lack of NACK feedback UE1 and UE2 assume successful reception by UEs within target communication range.

- UE1 and UE2 stop transmissions of TBs without receiving each other transmissions.

The above problem was also confirmed by system level evaluations [1][2]. Figure 1 shows comparative analysis of the current Rel.16 design vs scenario when two blind retransmissions are used for groupcast communication with NACK only feedback. As it can be seen, the Rel.16 solution does not achieve PRR equal to 1 even at short communication distances.

Figure 1: Illustration of the Rel.16 groupcast communication with NACK only feedback

# Possible Solutions

To mitigate the illustrated half-duplex problem for groupcast NACK only sidelink communication, there could be different solution with difference spec impact. The following options are considered in descending order of spec impact / generalization:

- Option 1: Introduce a configurable minimum number of blind retransmissions N and support of mixing blind retransmissions and feedback-based retransmissions for a TB

  - In this case, it is fully controllable by configuration whether a UE explicitly performs N blind retransmissions first and then switches to the feedback-based regime. Furthermore, when the feedback is not requested in SCI, redundant PSFCH are not generated on receivers.

  - Specification in this case needs to introduce RRC signaling of the minimum number of blind retransmissions as well as MAC support for switching between blind and feedback-based modes for the same logical channel after the minimum number of blind retransmissions.

- Option 2: Introduce a configurable minimum number of retransmissions N performed w/o considering the feedback from receivers

  - In this case, it is fully controllable by configuration whether a UE implicitly performs N blind retransmissions first and then switches to the feedback-based regime.

  - Specification in this case needs to introduce signaling of the minimum number of blind retransmissions. But since the feedback can be ignored, there is no evident impact on MAC specification.

Figure 1: Illustration of Option 1 and Option 2 mitigation of half-duplex for NACK-only feedback regime

- Option 3: Introduce a fixed number of minimum two retransmissions and support of mixing blind retransmissions and feedback-based retransmissions for a TB

  - This option is based on Option 1 without considering configurability of the minimum number of blind retransmissions, thus reducing RRC spec impact.

  - The mechanism could be enabled/disabled by configuration or left up to UE implementation to decide.

- Option 4: Introduce a fixed number of minimum two retransmissions performed w/o considering the feedback from receivers

  - This option is based on Option 2 without considering configurability of the minimum number of blind retransmissions, thus reducing RRC spec impact.

  - The mechanism could be enabled/disabled by configuration or left up to UE implementation to decide.

If presented above options are not agreeable to the group as a part of the Rel.16 maintenance work, then the Rel.17 TEI framework should be considered to resolve the mentioned susceptibility of the NACK only sidelink groupcast communication to half-duplex events [3].

# Text Proposal

In this section, we provide example text proposal for Option 4.

## 16.5 UE procedure for reporting HARQ-ACK on uplink A UE can be provided PUCCH resources or PUSCH resources [12, TS 38.331] to report HARQ-ACK information that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions. The UE reports HARQ-ACK information on the primary cell of the PUCCH group, as described in Clause 9, of the cell where the UE monitors PDCCH for detection of DCI format 3_0. For SL configured grant Type 1 or Type 2 PSSCH transmissions by a UE within a time period provided by sl-PeriodCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources. For PSSCH transmissions scheduled by a DCI format 3_0, a UE generates HARQ-ACK information in response to PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource in a set of time resources provided by the DCI format 3_0. For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission. The UE can be indicated by a SCI format to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" - generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from a PSFCH reception in the PSFCH reception occasion and, if the UE determines that a PSFCH is not received at the PSFCH reception occasion, generate NACK - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - generate ACK if the UE determines ACK from at least one PSFCH reception occasion, from the number of PSFCH reception occasions, in PSFCH resources corresponding to every identity MID of the UEs that the UE expects to receive the PSSCH, as described in Clause 16.3; otherwise, generate NACK - if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - if the transmitted PSSCH scheduled by a SCI format 2-A or a SCI format 2-B is not the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is larger than one or the transmitted PSSCH scheduled by a SCI format 2-A or a SCI format 2-B is the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is equal to one, generate ACK when the UE determines absence of PSFCH reception for each PSFCH reception occasion from the number of PSFCH reception occasions; otherwise, generate NACK - if the transmitted PSSCH scheduled by a SCI format 2-A or a SCI format 2-B is the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is larger than one, generate a NACK value

# Conclusions

In this contribution, we have shown technical problem of NR sidelink groupcast communication with NACK only feedback and have shown simple changes that can address it with minimal changes in the Rel.16.

  - Agree to address half-duplex problem of NR-V2X sidelink groupcast communication with NACK only design

  - Discuss and identify the appropriate solution through the change of the Rel.16 specification

    - Solutions proposed in this contribution can be used as a starting point for discussion

References

- R1-2100673, “On Feasibility and Benefits of Inter-UE Coordination for Sidelink Mode-2 Design”, Intel Corporation, January 2021

- R1-2101910, “Reliability and Latency Enhancements for Mode 2”, Qualcomm Incorporated, January 2021 - R1-2105778, “Rel-17 TEI proposal for mitigating half-duplex issue in NR V2X groupcast NACK-only case regime”, Intel Corporation, May 2021

Annex: Evaluation Assumptions

In this section, we provide the list of evaluation assumptions used for analysis in this contribution. The set of main parameters used for NACK only evaluation assumptions is provided in Table 1.

Table 1: System level evaluation assumptions for NACK only groupcast communication studies

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario from NR V2X methodology - Vehicle speed = 70 km/h |
| Channel model | TR 37.885, NR V2X Channel Model |
| Spectrum allocation | Carrier frequency: 6 GHz Simulated Bandwidth: 40 MHz |
| Subcarrier spacing | 30 kHz |
| Tx/Rx Antenna Ports | 1Tx/2Rx |
| Communication Mode(s) | Groupcast, connection-less |
| UE-to-UE Association | - UE is considered as member of the group based on location principle - Each UE may be a member of a multiple groups |
| Traffic model | Aperiodic variable packet size traffic (TR 37.885 Aperiodic Model 1 traffic): - Packet size: uniform in the range [200..2000] B with quantization step of 200 B - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Latency requirement: 50 ms |
| TTI structure | NR Slot TTI: - Slot w/o PSFCH zone: 1 AGC symbol, 1 GAP symbol, 2 PSSCH symbols w/ pilots - Slot w/ PSFCH zone: 1 AGC symbol, 2 GAP symbol, 2 PSSCH symbols w/ pilots, 2 symbols for PSFCH |
| PSFCH Zone Allocation | Enabled, in every 2nd slot |
| Sidelink control TX parameters | 64 Bits QPSK Modulation |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB - PSSCH: 35 PRB |
| SCI/Data time resource allocation | - PSCCH: 3 Symbols - PSFCH: 2 Symbols - PSSCH: Remaining symbols in slot |
| Data Packet Tx parameters | Aperiodic variable packet size evaluations: - 200 B packet: QPSK, 2 TTI - 400 B packet: 16-QAM, 2 TTI - 600 B packet: 16-QAM, 2 TTI - 800 B packet: 16-QAM, 2 TTI - 1000 B packet: 16-QAM, 3 TTI - 1200 B packet: 16-QAM, 3 TTI - 1400 B packet: 64-QAM, 3 TTI - 1600 B packet: 64-QAM, 3 TTI - 1800 B packet: 64-QAM, 4 TTI - 2000 B packet: 64-QAM, 4 TTI |
| HARQ Feedback | Enabled, NACK-only |
| Feedback reporting distance | UEs within 200m report HARQ feedback |