3GPP TSG RAN WG1 #105e                                                             R1-2104907

e-Meeting, May 10th – 27th, 2021

Source: 	Intel Corporation

Title: 	NR Positioning DL-AOD Enhancements

Agenda item:	8.5.3

Document for: 	Discussion and Decision

# Introduction

The RAN WG1 agreed to support the DL-AOD enhancements that enable a gNB to share beam/antenna information with the LMF for UE-assisted positioning method and with the UE for UE-based positioning method.

The following agreement was captured in the chairman’s notes, [1]:

Agreement: Regarding support of angle calculation enhancement for DL-AoD: - Support gNB providing the beam/antenna information to the LMF. - The gNB beam/antenna information can be provided to the UE for UE-based DL-AoD - FFS: the details of contents of the beam/antenna information - FFS: the details of how to provide the beam/antenna information. - Note: The antenna information is related to reducing the overhead of beam information - Send an LS to RAN2/RAN3 regarding the option of angle report from gNB to LMF for UE-A DL-AoD requesting them to consider this option in Rel-17.

The proposals that were discussed but were not agreed yet suggest DL-AOD enhancements that enable a UE to measure and report information related to the first arrival path, including the PRS-RSRP, angle of departure, time of arrival, phase and amplitude of the channel impulse response corresponding to the first arrival path among the options.

The following proposals were captured in the FL’s summary, [2]:

Proposal 1.2: - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the PRS RSRP of the first path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the angle of departure of the first arriving path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the arrival time of the first path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the phase of the CIR corresponding to the first arriving path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the received signal value (amplitude and phase of the channel estimated from the first path) of the first arriving path

The additional assistance signaling from LMF to UE was discussed at the previous meeting to facilitate DL measurements of DL-AOD. The following agreement was captured in the chairman’s notes [1]:

Agreement: For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options: - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP] - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP] - Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced. - FFS: details of signaling FFS: Applicability of this agreement to other Positioning methods

In this contribution, we provide further details for each of the agreements above, including beam/antenna information signaling from gNB to LMF and UE, Network-based DL-AOD information reporting and supported options in Proposal 1.2, and additional assistance signaling to support DL-AOD/DL-AOA expected value and uncertainty range reporting from LMF to the UE.

The enhancements for the UL-AOA positioning method are proposed in the companion contribution [3] and our views on other enhancements are shared in [4] - [7].

Accuracy Improvements for DL-AOD Positioning Solutions

## Beam/Antenna Information Sharing

The gNB beam/antenna information sharing with the LMF for UE-assisted positioning method and with the UE for UE-based positioning method was discussed at the previous meeting. The following agreement has been captured in the chairman’s notes, [1]:

Agreement: Regarding support of angle calculation enhancement for DL-AoD: - Support gNB providing the beam/antenna information to the LMF. - The gNB beam/antenna information can be provided to the UE for UE-based DL-AoD - FFS: the details of contents of the beam/antenna information - FFS: the details of how to provide the beam/antenna information. - Note: The antenna information is related to reducing the overhead of beam information - Send an LS to RAN2/RAN3 regarding the option of angle report from gNB to LMF for UE-A DL-AoD requesting them to consider this option in Rel-17.

The proposal supports reporting of the gNB beam and antenna information to LMF and UE. The antenna information is reported separately to facilitate overhead reduction associated with the beam information reporting.

In this contribution we propose the following beam/antenna information reporting:

  - Support enhancement for the DL-AOD estimation in Rel-17 by utilizing the gNB/TRP beam/antenna information, including the following:

    - gNB/TRP beam information:

      - Phase value per antenna element / port

      - Amplitude value per antenna element (optionally)

    - gNB/TRP antenna array information:

      - Antenna array orientation in space with respect to the global coordinate system (when information is provided to UE)

      - Antenna pattern of the single antenna element (optionally)

      - For the uniform rectangular planar array, provide the total number of elements over horizontal and vertical dimension as well as the antenna spacing per dimension

      - In general case, provide the coordinates of the antenna array elements in the local coordinate system

## Network-based DL-AOD Information Reporting

The DL-AOD reporting enhancements that enable a UE to measure and report information related to the RSRP, DL-AOD, time of arrival, channel phase and amplitude associated with the first arrival path have been discussed and captured in the FL’s summary, [2]:

Proposal 1.2: - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the PRS RSRP of the first path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the angle of departure of the first arriving path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the arrival time of the first path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the phase of the CIR corresponding to the first arriving path - For both UE-based and UE-assisted DL-AOD, the UE can be requested to measure and report (for UE-assisted) the received signal value (amplitude and phase of the channel estimated from the first path) of the first arriving path

In this contribution, we propose the following steps, measurements, and reporting for the Network-based DL-AOD positioning method as summarized in Table 1. Two basic scenarios are considered assuming that the gNB/TRP beam/antenna assistance information:

- Option 1: gNB beam/antenna information is shared with LMF

- Option 2: gNB beam/antenna information is shared with UE

Table 1: Network-based DL-AOD solution – steps, measurements, and reporting

| Assumption on gNB/TRP beam/antenna assistance information sharing | Procedure steps |
| --- | --- |
| Option #1 Assistance information is shared with LMF | - UE receives DL-PRS resources, performs phase/amplitude measurements - UE reports phase/amplitude measurements to LMF - LMF computes the coordinates |
| Option #2 Assistance information is shared with UE | - UE receives DL-PRS resources, performs DL-AOD estimation - UE reports DL-AOD to LMF - LMF computes the coordinates |

Based on the considered steps, measurements and reporting for the Network-based DL-AOD solution, we have the following proposals:

- Support the network-based DL-AOD solution, where the gNB/TRP beam/antenna assistance information is shared with the LMF, including the following steps, measurements, and reporting (option #1 in Table 1):

  - UE receives DL-PRS resources, performs phase/amplitude measurements for the first arrival path per DL PRS resource

  - UE reports phase/amplitude measurements to LMF (Proposal 1.2 d/e in the FL’s summary)

  - LMF computes the coordinates

- Support the network-based DL-AOD solution, where the gNB/TRP beam/antenna assistance information is shared with the UE, including the following steps, measurements, and reporting (option #2 in Table 1):

  - UE receives DL-PRS resources, performs DL-AOD estimation for the first arrival path per DL PRS resource

  - UE reports DL-AOD to LMF (Proposal 1.2 b in the FL’s summary)

  - LMF computes the coordinates

## DL-AOD/DL-AOA Expected Value and Range Reporting

The additional assistance signaling from LMF to UE was discussed at the previous meeting to facilitate DL measurements of DL-AOD. The following agreement has been captured in the chairman’s notes, [1]:

Agreement: For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, study further whether to support at most one of the following options: - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoD/ZoD and uncertainty (of the expected DL-AoD/ZoD value) range(s) can be provided to the UE for each [TRP] - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE - Single Expected DL-AoA/ZoA and uncertainty (of the expected DL-AoA/ZoA value) range(s) can be provided to the UE for each [TRP] - Option 3: Indication of expected AoD/ZoD or AoA/ZoA value and uncertainty is not introduced. - FFS: details of signaling FFS: Applicability of this agreement to other Positioning methods

The agreement supports reduction of the angular search range by specifying the expected DL azimuth and zenith angles of departure/arrival as well as the uncertainty range.

Two options have been considered during the meeting.

In the first option, the expected DL-AOD/ZOD and uncertainty (of the expected DL-AOD/ZOD value) range(s) is signaled by the LMF to the UE. This option is reasonable if DL-AOD signaling is supported from the UE to the LMF (Proposal 1.2b). In opposite way, the solution will be incomplete.

In the second option, the expected DL-AOA/ZOA value and uncertainty (of the expected DL-AOA/ZOA value) range(s) is signaled by the LMF to the UE. The reporting of the DL-AOA/ZOA can be supported if the UE antenna orientation in space is known/calibrated. This can be a case for the reference UE with known coordinates and antenna orientation in space.

Based on the above considerations, we think that the Option 1 can be supported if the DL-AOD reporting from the UE to the LMF is introduced. The Option 2 can be supported for the reference UE with known coordinates and antenna orientation in space.

- For the purpose of UE-A and UE-B DL-AOD positioning solution support indication of the expected DL-AOD/ZOD value and uncertainty (of the expected DL-AOD/ZOD value) range(s) by LMF to the UE for each TRP, if DL-AOD reporting from the UE to LMF is supported

- For the purpose of UE-A and UE-B DL-AOD positioning solution support indication of the expected DL-AOA/ZOA value and uncertainty (of the expected DL-AOA/ZOA value) range(s) by LMF to the UE for each TRP, if UE antenna orientation in space is known/calibrated

  - Note: an example of such a UE can be a reference UE with known coordinates and antenna orientation in space

Conclusions

In this contribution, we have provided method, measurements, reporting and signaling to support accurate DL-AOD estimation. In summary, we have following list of proposals:

  - Support enhancement for the DL-AOD estimation in Rel-17 by utilizing the gNB/TRP beam/antenna information, including the following:

    - gNB/TRP beam information:

      - Phase value per antenna element / port

      - Amplitude value per antenna element (optionally)

    - gNB/TRP antenna array information:

      - Antenna array orientation in space with respect to the global coordinate system (when information is provided to UE)

      - Antenna pattern of the single antenna element (optionally)

      - For the uniform rectangular planar array, provide the total number of elements over horizontal and vertical dimension as well as the antenna spacing per dimension

      - In general case, provide the coordinates of the antenna array elements in the local coordinate system

- Support the network-based DL-AOD solution, where the gNB/TRP beam/antenna assistance information is shared with the LMF, including the following steps, measurements, and reporting (option #1 in Table 1):

  - UE receives DL-PRS resources, performs phase/amplitude measurements for the first arrival path per DL PRS resource

  - UE reports phase/amplitude measurements to LMF (Proposal 1.2 d/e in the FL’s summary)

  - LMF computes the coordinates

- Support the network-based DL-AOD solution, where the gNB/TRP beam/antenna assistance information is shared with the UE, including the following steps, measurements, and reporting (option #2 in Table 1):

  - UE receives DL-PRS resources, performs DL-AOD estimation for the first arrival path per DL PRS resource

  - UE reports DL-AOD to LMF (Proposal 1.2 b in the FL’s summary)

  - LMF computes the coordinates

- For the purpose of UE-A and UE-B DL-AOD positioning solution support indication of the expected DL-AOD/ZOD value and uncertainty (of the expected DL-AOD/ZOD value) range(s) by LMF to the UE for each TRP, if DL-AOD reporting from the UE to LMF is supported

- For the purpose of UE-A and UE-B DL-AOD positioning solution support indication of the expected DL-AOA/ZOA value and uncertainty (of the expected DL-AOA/ZOA value) range(s) by LMF to the UE for each TRP, if UE antenna orientation in space is known/calibrated

  - Note: an example of such a UE can be a reference UE with known coordinates and antenna orientation in space

References

| - [1] | - “Chairman's Notes RAN1#104b-e 8.5 v006,” April 12 – 20, 2021. |
| --- | --- |
| - [2] | - R1-2104047 “FL summary #2 for AI 8.5.3 Accuracy improvements for DL-AoD positioning,” April 12 – 20, 2021. |
| - [3] | - R1-2104906 “NR positioning UL-AOA Enhancements,” Intel Corporation, May 2021 |
| - [4] | R1-2104905 Mitigation of UE/gNB TX/RX timing errors, Intel Corporation, May 2021 |
| - [5] | - R1-2104908 NR positioning Latency Reduction, Intel Corporation, May 2021 |
| - [6] | R1-2104909 Mitigation of NLOS problem for NR Positioning, Intel Corporation, May 2021 |
| - [7] | R1-2104910 Support of On-Demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state, Intel Corporation, May 2021 |