3GPP TSG RAN WG1 Meeting #105-E                                              	 R1-2104911

e-Meeting, May 10th – 27th, 2021

Source:	Intel Corporation

Title:	On reduced max UE bandwidth for RedCap

Agenda item:	8.6.1.1

Document for:	Discussion and Decision

# Introduction

At RAN plenary meeting #91-E, the work item (WI) for the support of Reduced Capability (RedCap) NR devices was updated, and the following objectives related to UE complexity reduction in relation to number of Rx branches were identified [1]:

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Reduced maximum UE bandwidth: - Maximum bandwidth of an FR1 RedCap UE during and after initial access is 20 MHz. - Maximum bandwidth of an FR2 RedCap UE during and after initial access is 100 MHz.

Also, during RAN1 #104 meeting, the following were agreed [2]:

Agreements: - Sharing of the same SSB and CORESET#0 between RedCap and non-RedCap UEs is supported when the bandwidth is no wider than the RedCap UE bandwidth - The initial DL BWP (derived based on MIB/SIB) for RedCap UEs can be the same as the initial DL BWP for non-RedCap UEs at least when the initial DL BWP is no wider than the RedCap UE bandwidth. - FFS: after initial access, whether a RedCap UE is allowed to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth - Discuss further whether or not it is also applicable during initial access - The initial UL BWP (derived based on SIB) for RedCap UEs can be the same as the initial UL BWP for non-RedCap UEs at least when the initial UL BWP is no wider than the RedCap UE bandwidth. - FFS: during and after initial access, whether a RedCap UE is allowed to operate with an initial UL BWP wider than the maximum RedCap UE bandwidth - FFS whether or not to further introduce the following (e.g., for offloading purpose, for differentiation of RedCap vs. non RedCap UEs, for different BWP#0 configuration options, etc.) - Whether an additional CORESET can be configured for scheduling of RACH (msg2 & msg4)/Paging/SI messages for RedCap UEs - Whether the SIB-configured initial DL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial DL BWP for non-RedCap UEs. - Whether the SIB-configured initial UL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial UL BWP for non-RedCap UEs. Conclusion: RAN1 does not consider acquisition time improvements for FR2 RedCap UEs with SSB and CORESET#0 multiplexing patterns 2 and 3 as part of this WI. Agreements: - Study further how to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth, with the following options: - Option 1: Proper RF-retuning for RedCap - Option 2: Separate initial UL BWP(s) for RedCap UEs - Option 3: gNB configuration (e.g., restrictions on existing PRACH configurations, or FDM-ed ROs, or always restricting the initial UL BWP to within RedCap UE bandwidth) - Option 4: Dedicated PRACH configurations (e.g., ROs) for RedCap UEs - Other options are not precluded Conclusion: Discuss further in RAN1#104b-e whether or not to send LS to RAN4 regarding RF retuning time, and if so, the RAN1 details associated with question. Agreements: - Study further whether and how to enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, with the following options: - Option 1: Proper RF-retuning for RedCap (if feasible) - Option 2: Separate initial UL BWP(s) for RedCap - FFS more than one starting PRB position - Option 3: Separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation for the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping) - Option 4: gNB configuration (e.g., always restricting the initial UL BWP to within RedCap UE bandwidth, or restrictions on the frequency location and the amount of scheduled resource for Msg4/[MsgB] HARQ feedback and Msg3/[MsgA] PUSCH) - As an example, with restrictions on the frequency location and the amount of scheduled resource for Msg4/[MsgB] HARQ feedback and Msg3/[MsgA] PUSCH, when the initial UL BWP is the same for RedCap and non-RedCap UEs, the PUCCH (for Msg4/[MsgB] HARQ feedback) and PUSCH (for Msg3/[MsgA]) are within the RedCap UE bandwidth - Other options are not precluded

Subsequently, during RAN1 #104bis-E meeting, the following was agreed [3]:

Working assumption: - During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth. - The bandwidth and location of the initial DL BWP for RedCap UEs can be the same as the bandwidth and location of the MIB-configured initial DL BWP for non-RedCap UEs. - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth. - This does not preclude separate or additional bandwidth and location for initial DL BWP for RedCap UEs (FFS). Working assumption: After initial access, at least for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - FFS: BWP#0 configuration option 2 (as in 38.331, Appendix B2) Agreement: - During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Agreement: - After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e: - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Working assumption: A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

In this contribution, we present our views on the support of reduced UE BW for RedCap UEs considering the above WI objectives and decisions from previous RAN1 meetings.

# Aspects related to reduced BW support in RRC Idle/Inactive modes

In this section, we present our views on the remaining details related to support of RedCap UEs and operations in RRC Idle/Inactive modes.

Specifically, for DL:

- Whether SIB-indicated initial DL BWP can be different for RedCap and non-RedCap UEs?

- Whether to support configuration of an additional initial DL BWP in addition to the one defined by CORESET #0?

For UL:

- Whether and how to support the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth?

- How to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth?

- Whether and how to enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access?

DL: Whether SIB-indicated initial DL BWP can be different for RedCap and non-RedCap UEs?

According to Rel-15 NR specifications, a UE may be provided with a configuration for the locationAndBandwidth parameter for the initial DL BWP via SIB1 that then replaces the initial DL BWP defined by CORESET #0 once the UE is in RRC_CONNECTED mode, that is, for RRC_IDLE and RRC_INACTIVE modes, DL BWP #0 defined by CORESET #0 is used for DL reception.

With the introduction of RedCap UEs, the configuration of initial DL BWP provided via SIB1, may apply separately for non-RedCap and RedCap UEs. In particular, the configuration of initial DL BWP as indicated via SIB1 (in initialDownlinkBWP) may not be used by RedCap UEs. That is, the indication may only apply to non-RedCap UEs. Such a design approach can also allow for use of Config 2 for DL BWP #0 configuration for non-RedCap UEs in RRC_CONNCETED mode (see related discussions in Section 3).

Further, RedCap UEs may be optionally provided with a separate configuration of either the initialDownlinkBWP structure or the locationAndBandwidth parameter for DL BWP #0 configuration via SIB1. In the absence of separate configuration for RedCap UEs in SIB1 and if the BW indicated by locationAndBandwidth parameter for DL BWP #0 configuration via SIB1 exceeds max RedCap UE BW, RedCap UEs may continue to use the DL BWP #0 defined by CORESET #0 as they transition to RRC_CONNECTED mode.

Proposal 1:

- SIB-configured DL BWP can be provided to non-RedCap UEs as in Rel-15 with bandwidth that may be larger than max RedCap UE BW.

- RedCap UEs may be optionally provided with a separate configuration of either the initialDownlinkBWP structure or the locationAndBandwidth parameter for DL BWP #0 configuration via SIB1.

  - If not separately provided with such configuration, RedCap UE uses:

    - the configuration as in initialDownlinkBWP in SIB1 (common between RedCap and non-RedCap), if the BW is within max RedCap UE BW; and

    - CORESET #0 to define DL BWP #0, otherwise.

DL: Whether to support configuration of an additional initial DL BWP in addition to the one defined by CORESET #0?

When considering high density of RedCap UEs in the cell, there can be congestion in the DL BWP #0 defined by CORESET #0. In this regard, it can be beneficial to be able to offload common control reception for paging and/or random access for RedCap UEs to an additional CORESET, say CORESET #0A to define an additional initial DL BWP.

Since the same SI messages are expected to be shared between RedCap and non-RedCap UEs, it may not be as beneficial to offload SI messages (RMSI, OSI) to an additional BWP, instead such may only contribute to increased OH from system information delivery. As against this, for paging and RA, the loading on the network resources (esp. on DL BWP #0 defined by CORESET #0) scales with the number of RedCap UEs, and thus, being able to offload paging- and RA-related PDCCH/PDSCH to another BW defined by a CORESET #0A can be beneficial and effective in mitigating congestion in CORESET #0.

On the other hand, following working assumption from last meeting, per FG 6-1, it is mandated that each active DL BWP should include SSB and CORESET #0 of the cell. Thus, SSB and SI messages may need to be duplicated in the additional initial DL BWP, if not shared in the additional initial DL BWP (in practice, only SSB may be shared between the initial DL BWP and additional initial DL BWP), thereby potentially diminishing the benefits.

Observation 1:

- For a RedCap UE supporting only FG 6-1 and not FG 6-1a, if an additional CORESET (e.g., CORESET #0A) to define an additional initial DL BWP for offloading of common control messages (PDCCH and PDSCH) is provided via SIB1, Types 0/0A/1/2 PDCCH-CSS sets should be mapped to CORESET #0A and SSB should be included within the additional initial DL BWP.

UL: Whether and how to support the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth?

During RAN1 #104b-e meeting, the following options were identified [3]:

Agreement:

- During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e

  - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP.

  - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs.

  - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

For initial UL BWP, the following considerations are necessary:

- Need for a large (> 20 MHz for FR1 and > 100 MHz for FR2) initial UL BWP for non-RedCap UEs.

- Whether non-RedCap UEs may be provided with a larger initial UL BWP?

- Fragmentation of UL resources for non-RedCap UEs if configured with larger UL BWP due to pre-defined Frequency Hopping (FH) for Msg3 PUSCH and PUCCH with HARQ-ACK feedback in response to Msg4 PDSCH.

To the first question, the practical impact from restricting an initial UL BWP for a non-RedCap UE to a maximum bandwidth of 20 MHz (FR1) and 100 MHz (FR2) respectively may be negligible since there are very limited Tx opportunities for a UE in RRC Idle/Inactive states. For FR2, this restriction would limit a few instances of FDM-based RACH configurations, but the adverse impact from this (slightly increased RA latency) may be negligible, especially in the context of FR2 operations.

Thus, Option 3 can be the simplest option without a significant practical impact to RedCap or non-RedCap UEs.

On the other hand, if it is justified to maintain the same flexibility for RedCap UEs as in Rel-15/16, then Option 2 may be considered. As such, it should be possible to separately configure initial UL BWP for RedCap and non-RedCap UEs via SIB1, and such a feature can be used to provide a RedCap UE with initial UL BWP configuration limited to max RedCap UE BW, while for non-RedCap UEs, a larger initial UL BWP may be configured. An additional benefit/use-case for separate configuration of initial UL BWP for RedCap UEs is in realizing early identification of RedCap UEs.

With separate configuration of initial UL BWP while reusing initial DL BWP as defined by CORESET #0 for non-RedCap UEs, one change from Rel-15 behavior would be that, for TDD deployments, the center frequency of the DL BWP #0 and UL BWP #0 may not be aligned. This implies frequency retuning gaps need to be provisioned in addition to DL-to-UL and UL-to-DL switching times. However, for Idle/Inactive mode operations, considering the very limited cases of UL transmissions (mainly as part of the random access procedure), the impact from OH due to such retuning gaps for DL to UL transitions and vice-versa may be minimal. Note that, for non-RedCap UEs, it would need to be ensured that Rel-15 constraint on aligning center frequencies for DL and UL BWPs with same BWP index is satisfied.

On potential UL resource fragmentation for non-RedCap UEs, we note the following:

- Considering typical UL transmissions are more sensitive to large BW allocations due to link budget and the fact that initial UL BWP for RedCap UE can be as large as 20 MHz, the issue of UL resource fragmentation for non-RedCap UEs is not expected to have a significant impact.

- Achievable peak UL data rates for non-RedCap UEs are not impacted either – just that the peak rates may not be feasible in every slot. This is the fact in any multi-user system, and for Msg3 PUSCH and PUCCH with HARQ-ACK feedback in response to Msg4, the transmissions can be dynamically controlled by the gNB via the UL grant in the RAR or corresponding DCI formats as applicable, and in this regard, no different from another PUSCH scheduling in the cell.

- The narrower initial UL BWP for RedCap UE may be configured at an edge of the UL carrier, thereby minimizing impact from UL resource fragmentation.

In summary, Option 2 could be supported if the necessity of supporting a very large initial UL BWP (beyond 20 MHz/100 MHz) is justified for non-RedCap UEs.

On the other hand, with initial UL BWP larger than max UE BW (Option 1), frequency retuning gaps need to be provisioned for PUCCH transmissions (for which intra-slot FH applies by default for cell-common PUCCH resources) and for Msg3 PUSCH transmission if indicated to transmit with intra-slot FH. As discussed in Section 3, a frequency retuning gap of length between 50 – 200 us may be necessary, and this can amount to multiple symbols (e.g., around 2-4 symbols @ 30 kHz SCS) that need to be accommodated within the PUSCH or PUCCH durations, implying effective UL resource wastage.

Thus, support of Option 1 is not recommended.

Based on the above discussion, we do not see sufficient motivation to allow for initial UL BWP for RedCap or non-RedCap UEs that are larger than max RedCap UE BW.

Proposal 2:

- For a RedCap UE, the initial UL BWP is not expected to be larger than the max RedCap UE BW for that Frequency Range (FR).

Proposal 3:

- During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth,

  - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

Here, we would like to highlight that even if Option 3 is supported, it can be beneficial to be able to configure an initial UL BWP for RedCap UEs separate from that for non-RedCap UEs to support early identification of RedCap UEs during Msg1 transmission. Thus, the feature of providing a separate UL BWP configuration to RedCap UEs via SIB1, different from that for non-RedCap UEs, is relevant even for Option 3.

Observation 2:

- It can be beneficial and feasible to separately provide (via SIB1) initial UL BWP configurations for RedCap and non-RedCap UEs, regardless of the choice between Options 1, 2, or 3 on support of the scenario with non-RedCap UE with initial UL BWP larger than max RedCap UE BW.

UL: How to enable/support that a RACH occasion associated with the best SSB falls within the RedCap UE bandwidth?

In general, there can be cases wherein, with a large number of FDM-ed ROs, some of the ROs may fall outside of the max RedCap UE BW.

However, following straightforward options exist to ensure that a RO associated with the best SSB falls within the RedCap UE BW:

- Configuring separate initial UL BWP for RedCap UEs wherein such initial UL BWP configuration includes the RACH configuration with ROs configured such that all candidate ROs fall within max RedCap UE BW. This approach can be utilized if following Option 2 in previous question.

- Proper gNB configuration to avoid cases with FDM-ed ROs such that some ROs fall beyond max RedCap UE BW (e.g., Option 3 in previous question). Considering that access latency may not be an issue for RedCap UEs, the impact from multiplexing some of the ROs in time (rather than in frequency) may not have a significant impact on access latency for RedCap use-cases.

Note that separate configuration of ROs for RedCap UEs may be necessary for early identification of RedCap UE type(s) during Msg1 transmission, irrespective of whether or not all candidate ROs are contained within max RedCap UE BW in the initial UL BWP.

Proposal 4:

- That a RO associated with the best SSB falls within the RedCap UE BW can be ensured by:

  - Proper gNB configuration to avoid cases with FDM-ed ROs such that some ROs fall beyond max RedCap UE BW, e.g., not configure initial UL BWP larger than RedCap UE BW.

  - Configuring separate initial UL BWP for RedCap UEs with BW no larger than max RedCap UE BW and including the RACH configuration such that all candidate ROs fall within max RedCap UE BW, if initial UL BWP configuration for non-RedCap UE is supported to be larger than max RedCap UE BW.

UL: Whether and how to enable/support that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access?

It can be seen that if initial UL BWP is limited to max RedCap UE BW, that PUCCH (for Msg4/MsgB HARQ-ACK feedback) and Msg3 PUSCH can be automatically ensured to fall within RedCap UE BW during initial access. Further, separate configuration of RACH and common PUCCH resources should be configured if configuring separate initial UL BWP for RedCap UEs, and RedCap UE may be configured with PUCCH resources without mandating FH across the UL BWP if UL resource fragmentation is established as a significant issue.

Thus, we can summarize the operations as below in the following Proposal.

Proposal 5:

- Msg3 PUSCH and PUCCH (with HARQ-ACK in response to Msg4 PDSCH) are transmitted in the initial UL BWP configured for RedCap UEs.

  - This could be same/different from non-RedCap UEs, but within RedCap max UE BW.

- Msg3 PUSCH with FH is limited to within initial UL BWP for RedCap UEs (w/in RedCap UE max BW)

  - Rel-15 procedures and UE behavior apply.

  - If a separate initial UL BWP is configured for RedCap UEs, it also includes rach-ConfigCommon.

- PUCCH w/ HARQ-ACK in response to Msg4 PDSCH applies FH at edge of initial UL BWP configured for RedCap UEs

  - Rel-15 procedures and UE behavior apply.

  - If a separate initial UL BWP is configured for RedCap UEs, it also includes pucch-ConfigCommon to indicate common PUCCH resources, that may include only PUCCH resources without FH.

# Aspects related to reduced BW support in RRC Connected mode

In this section, we present our views on the following questions related to support of RedCap UEs and operations in RRC Connected mode:

For DL:

- Whether and how to support BWP#0 configuration option 2 supporting a single BWP in the cell, where the BWP is larger than max RedCap UE bandwidth?

- Support of FG #6-1 only for FR2?

For UL:

- Whether to support UL BWP #0 with BW larger than max RedCap UE BW?

- Whether and how to support mechanisms for frequency diversity and/or scheduling gain?

DL: Whether and how to support BWP#0 configuration option 2 supporting a single BWP in the cell, where the BWP is larger than RedCap UE bandwidth?

During RAN1 #104b-e meeting, RAN1 made the following working assumption [3]:

Working assumption: After initial access, at least for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

  - FFS: BWP#0 configuration option 2 (as in 38.331, Appendix B2)

BWP #0 configuration per “Option 2” in TS 38.331 allows a gNB to configure DL BWP #0 as a UE-specific DL BWP, and thereby enables a gNB to operate using only a single BWP spanning the DL carrier. Such configuration would be necessary, e.g., to enable use of non-fallback DCI formats in the DL BWP #0.

However, with introduction of RedCap UEs, such mechanism can still be preserved for non-RedCap UEs, while RedCap UEs may be scheduled either in the initial DL BWP defined by CORESET #0 (as for Idle/Inactive modes), or as a separately configured DL BWP that does not exceed the max RedCap UE BW (as elaborated in Proposal 1).

Note that, fundamentally, there is no difference compared to the case of allowing a RedCap UE to operate on a BWP larger than the max UE BW but then devising means to ensure that resource allocation for DL channels and signals are restricted to within RedCap UE’s max BW. On the other hand, the specification impact from such operation within a BWP larger than max UE BW would be significant, effectively define a parallel solution to BWP-based framework from Rel-15.

It should also be noted that in terms of realizing frequency diversity, FH schemes based on inter-BWP FH, that preserve the existing BWP framework, is much more desirable than specifying yet another solution similar to eMTC.

Observation 3:

- There is no material difference when considering serving RedCap UEs with restricted BW within a larger DL BWP or when a separate DL BWP, no larger than max RedCap UE BW, is provided to RedCap UEs, while non-RedCap UEs operate on a larger DL BWP.

Proposal 6:

- After initial access, for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

- BWP#0 configuration option 2 (supporting a single BWP in the cell) can be supported for a non-RedCap UE following Rel-15 specifications by allowing separate configuration of initial DL BWP via SIB1 for RedCap and non-RedCap UEs.

DL: Support of FG #6-1 only for FR2?

During RAN1 #104b-e meeting, the following working assumption was made :

Working assumption: A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE.

- At least for FR1, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

From the above WA, it can be seen that the case of FR2 is still open regarding support of basic BWP operation capabilities. For FR2, certain SSB-CORESET #0 configurations for multiplexing patterns 2/3 can result in a combined BW of SSB and CORESET #0 to exceed 100 MHz. Requiring mandatory support of FG 6-1a in such cases can avoid an effective DL BWP size greater than 100 MHz. However, considering that FG 6-1a implies that tfrequency retuning based reception between SSB and CORESET #0 (unlike non-RedCap UEs that can receive both, even when SSB may be outside active DL BWP, using a larger RF BW in the UE receiver), the impact on RedCap UE operations may be significant. On the other hand, not supporting these few configurations in FR2 in cells supporting RedCap UEs may not impose a significant practical constraint.

Thus, it is preferred that a RedCap UE is only expected to support FG #6-1 as basic BWP operation capability even in FR2.

Proposal 7:

- For FR2, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

UL: Whether to support UL BWP #0 with BW larger than max RedCap UE BW?

During RAN1 #104b-e meeting, the following options were identified [3]:

Agreement:

- After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e:

  - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP.

  - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs.

  - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

In addition to the points discussed in the previous section, we further note that, for connected mode, the motivation to configure UL BWPs larger than max RedCap UE BW is even lower since, post-RRC connection, PUCCH and PUSCH resources and their FH configurations for the RedCap UE can be flexibly adjusted to minimize potential impact from UL resource fragmentation for non-RedCap UEs. On the other hand, a non-RedCap UE can be configured with a non-initial UL BWP with larger BW (than max RedCap UE BW).

Furthermore, the impact due to frequency retuning gaps (for UL transmissions) as well as between DL and UL (for TDD) would be more frequent and significant in terms of increasing OH in the system if operating within a BWP larger than max UE BW. Depending on traffic, compared to procedures in Idle/Inactive modes, in RRC connected mode, the UL-to-DL and DL-to-UL switches could be significantly pronounced.

At the same time, the specification impact from allowing BWP to be larger than max UE BW would be significant, especially so, when it effectively would offer an alternative to the existing BWP framework. In our view, the Rel-15 BWP framework was designed keeping in mind coexistence of UEs with different BW capabilities or configurations (e.g., for UE power savings), and we do not see sufficient motivation to deviate from the existing framework. Thus, we do not see a necessity to pursue Option 1.

Thus, as in the case for Idle/Inactive states, UL BWP #0 for non-RedCap and RedCap UEs may be limited to max RedCap UE BW (Option 3).

However, if Option 2 is supported for during initial access, then Option 2 should be applicable for after initial access as well.

Proposal 8:

- After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth,

  - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

UL: Whether and how to support mechanisms for frequency diversity and/or scheduling gain?

Due to the degraded link performance, use of repetitions for PDSCH and PUSCH is expected to be more typical for RedCap UEs than their non-RedCap counterparts. In this context, to recover some of the potential frequency diversity losses when comparing against 100 MHz BWP sizes, and towards limiting some of the link performance degradations in DL and UL, enhanced frequency hopping mechanisms, based on inter-BWP frequency hopping (FH) could be introduced.

One of the fundamental considerations regarding support of inter-BWP FH is impact from transition times to accommodate BWP switches. The currently specified values (reproduced below in Table 1) appear prohibitively long to justify efficient inter-BWP FH.

Table 1. BWP switch delay (from Table 8.6.2-1 in 3GPP TS 38.133)

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

However, much shorter transition times can be expected if considering mainly RF retuning times and possibly AGC settling time. In this regard, it should be noted that original RAN WG4 feedback in Rel-15 in [4] included the following:

- How fast is the UE RF bandwidth adaptation? - Transition time (RF aspects) - For intra-band operation, at least for sub6, the transition time can be up to 20 µs if the center frequency is the same before and after the bandwidth adaptation, regardless other conditions listed in the LS - For intra-band operation, at least for sub6, the transition time is 50~200 µs if the center frequency is different before and after the bandwidth adaptation, regardless other conditions listed in the LS - For inter-band operation, at least for sub6, the transition time can be up to 900 µs, regardless the conditions listed in the LS. RAN1 should note that this time does not include AGC settling time which is covered in baseband aspects. - Transition time (baseband aspects) - For single-carrier operation, the total transition time includes the processing time of the bandwidth adaptation signaling, RF transition time and the waiting time for slot boundary alignment if DL signal from the same cell is assumed before and after the bandwidth adaptation - For multiple-carrier operation, the total transition time includes the processing time of the bandwidth adaptation signaling, RF transition time, the waiting time for slot boundary alignment and the waiting time for reference signals for AGC settling - It’s difficult for RAN4 to conclude quantitative values now because it highly depends on the bandwidth adaptation design as well as the final physical design of e.g. reference signals.

In the context of RedCap UEs, the above two highlighted bullets are of interest in consideration of support of inter-BWP frequency hopping (FH) for DL/UL.

As can be seen from Table 1, eventually, the BWP switching delays specified by RAN4 in Rel-15 are much longer than 50~200 us due to other components (e.g., some of these mentioned in the second set of bullets above). Of these, PDCCH decoding time and application of the RRC configuration of the new BWP being key contributors.

However, if PDCCH decoding latency can be obviated (e.g., following a configured hopping pattern) and the candidate BWPs have same configuration with exception of center frequency, the overall transition time for inter-BWP FH may be shortened to within a few OFDM symbols, within the RF retuning times previously indicated by RAN4. Considering that retuning times are typically impacted by the range of the candidate center frequencies, some restrictions on the maximum range of the candidate center frequencies (e.g., within 100 MHz) could help further.

As alluded to in the first two sub-bullets in the above-quoted response from RAN WG4, assuming that the same DL branch(es) (with common FFT timing) are used at the gNB side across the BWPs for a given DL cell, it may be sufficient to use CP for AGC settling time.

Proposal 9:

- Introduction of inter-BWP frequency hopping for DL and UL can be considered for RedCap UEs.

However, it would be imperative to send an LS to RAN4 requesting them to provide feedback on potential transition times for inter-BWP switching when assuming that application times may not need to be accounted for PDCCH decoding or for RRC configuration of the destination BWP. Note that such information can be relevant not only in the context of supporting inter-BWP FH but also necessary in a more general context, namely, (i) to confirm feasibility of switching times for RedCap UEs, (ii) confirm expected switching times between active DL and UL BWPs in TDD if they do not share a common center frequency; (iii) help assess the feasibility and challenges in supporting scenarios wherein an active DL BWP may not include SSB and/or CORESET #0, etc.

Towards this, the latest version of the LS draft that was discussed during RAN1 #104b-e (contents copied below) should be considered as the starting point.

# 1 Overall description RAN1 has discussed the RedCap WI objective on “Reduced maximum UE bandwidth” and would like to ask RAN4 whether it would be feasible to maintain the same RF switching times for RedCap UEs as currently specified for non-RedCap UEs or even reduce the RF switching times for RedCap UEs under the following assumptions with manageable impacts (to e.g. device cost, power consumption, and specifications): - The RF switching takes place between two frequency locations with different centre frequencies. - The maximum UE RF bandwidth is 20 MHz for FR1 and 100 MHz for FR2, and the frequency change is up to 80 MHz for FR1 and up to 300 MHz for FR2. - The RF bandwidth, SCS, QCL, and RRC configuration can be assumed to be the same before and after the RF switching, i.e. it is only the centre frequency that changes. - The RF switching may take place during initial access or after initial access. # 2 Actions To RAN4: ACTION: RAN1 respectfully asks RAN4 to provide feedback on the question above on RF switching time.

Proposal 10:

- RAN1 to send an LS to RAN WG4 for feedback on transition times for inter-BWP switching when assuming that PDCCH decoding and RRC configuration application times may be ignored and when maximum range of center frequencies of the candidate BWPs may be limited.

  - Use the latest version of the draft LS that was discussed during RAN1 #104b-e as a starting point.

# Conclusions

In this contribution, we presented our views on the normative specification work necessary for efficient support of RedCap UEs with reduced UE BW in existing and future NR deployments with minimal impact to non-RedCap UEs.

Based on the presented discussion, our views can be summarized via the following observations and proposals.

Proposal 1:

- SIB-configured DL BWP can be provided to non-RedCap UEs as in Rel-15 with bandwidth that may be larger than max RedCap UE BW.

- RedCap UEs may be optionally provided with a separate configuration of either the initialDownlinkBWP structure or the locationAndBandwidth parameter for DL BWP #0 configuration via SIB1.

  - If not separately provided with such configuration, RedCap UE uses:

    - the configuration as in initialDownlinkBWP in SIB1 (common between RedCap and non-RedCap), if the BW is within max RedCap UE BW; and

    - CORESET #0 to define DL BWP #0, otherwise.

Observation 1:

- For a RedCap UE supporting only FG 6-1 and not FG 6-1a, if an additional CORESET (e.g., CORESET #0A) to define an additional initial DL BWP for offloading of common control messages (PDCCH and PDSCH) is provided via SIB1, Types 0/0A/1/2 PDCCH-CSS sets should be mapped to CORESET #0A and SSB should be included within the additional initial DL BWP.

Proposal 2:

- For a RedCap UE, the initial UL BWP is not expected to be larger than the max RedCap UE BW for that Frequency Range (FR).

Proposal 3:

- During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth,

  - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

Observation 2:

- It can be beneficial and feasible to separately provide (via SIB1) initial UL BWP configurations for RedCap and non-RedCap UEs, regardless of the choice between Options 1, 2, or 3 on support of the scenario with non-RedCap UE with initial UL BWP larger than max RedCap UE BW.

Proposal 4:

- That a RO associated with the best SSB falls within the RedCap UE BW can be ensured by:

  - Proper gNB configuration to avoid cases with FDM-ed ROs such that some ROs fall beyond max RedCap UE BW, e.g., not configure initial UL BWP larger than RedCap UE BW.

  - Configuring separate initial UL BWP for RedCap UEs with BW no larger than max RedCap UE BW and including the RACH configuration such that all candidate ROs fall within max RedCap UE BW, if initial UL BWP configuration for non-RedCap UE is supported to be larger than max RedCap UE BW.

Proposal 5:

- Msg3 PUSCH and PUCCH (with HARQ-ACK in response to Msg4 PDSCH) are transmitted in the initial UL BWP configured for RedCap UEs.

  - This could be same/different from non-RedCap UEs, but within RedCap max UE BW.

- Msg3 PUSCH with FH is limited to within initial UL BWP for RedCap UEs (w/in RedCap UE max BW)

  - Rel-15 procedures and UE behavior apply.

  - If a separate initial UL BWP is configured for RedCap UEs, it also includes rach-ConfigCommon.

- PUCCH w/ HARQ-ACK in response to Msg4 PDSCH applies FH at edge of initial UL BWP configured for RedCap UEs

  - Rel-15 procedures and UE behavior apply.

  - If a separate initial UL BWP is configured for RedCap UEs, it also includes pucch-ConfigCommon to indicate common PUCCH resources, that may include only PUCCH resources without FH.

Observation 3:

- There is no material difference when considering serving RedCap UEs with restricted BW within a larger DL BWP or when a separate DL BWP, no larger than max RedCap UE BW, is provided to RedCap UEs, while non-RedCap UEs operate on a larger DL BWP.

Proposal 6:

- After initial access, for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

- BWP#0 configuration option 2 (supporting a single BWP in the cell) can be supported for a non-RedCap UE following Rel-15 specifications by allowing separate configuration of initial DL BWP via SIB1 for RedCap and non-RedCap UEs.

Proposal 7:

- For FR2, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

Proposal 8:

- After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth,

  - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

Proposal 9:

- Introduction of inter-BWP frequency hopping for DL and UL can be considered for RedCap UEs.

Proposal 10:

- RAN1 to send an LS to RAN WG4 for feedback on transition times for inter-BWP switching when assuming that PDCCH decoding and RRC configuration application times may be ignored and when maximum range of center frequencies of the candidate BWPs may be limited.

  - Use the latest version of the draft LS that was discussed during RAN1 #104b-e as a starting point.

# References

- RP-210918, “Revised WID on support of reduced capability NR devices,” RAN #91-e.

- Chairman’s Notes, 3GPP RAN1 #104-e.

- Chairman’s Notes, 3GPP RAN1 #104b-e.

- R1-1704179 (R4-1702029), “Reply LS on UE RF Bandwidth Adaptation in NR,” RAN WG4.