3GPP TSG RAN WG1 Meeting #105 -E				               R1-2104916

e-Meeting, May 10th – May 27th, 2021

Source:	Intel Corporation

Title:	On Paging Enhancements for UE Power Saving

Agenda item:	8.7.1.1

Document for:	Discussion and Decision

Introduction

The Rel-17 work item on UE power saving enhancements [1] includes the following objective for paging enhancements targeting enhanced UE power savings:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting

In RAN1 104bis-e [2], the following was agreed in connection to paging enhancements in idle/inactive mode.

Agreement: Observation 1a: For the evaluation and comparison of PEI candidate designs, the following observations for coexistence with legacy PDSCH are identified: - For coexistence with legacy PDSCH, semi-static resouce sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering PEI REs is supported for all PEI candidate designs. - For coexistence with legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1 - For PDCCH based PEI, CORESET-level rate matching can be realized for the PDSCH as per mandatory capability - For SSS-based PEI, CORESET-level rate matching may be realized for the PDSCH as per mandatory capability, depending on the design of SSS-based PEI and UE capability regarding number of supported CORESETs - For TRS/CSI-RS based PEI, RE-level rate matching can be realized for the PDSCH as per mandatory capability - When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource Observation 2a: For the evaluation and comparison of PEI candidate designs, the following summarize the identified configurations of PEI candidate designs, including pairs of the minimum required resource and maximum UE (sub)group indication capacity per PEI, that can comply with the mandatory performance metrics agreed in RAN1 #104-e: - If Behv-A is assumed, Paging Setting; PEI candidate design; Physical-layer configuration and resource; UE (sub)group indication capcity; Number of companies providing performance results. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 5 (HW/HiSi, OPPO, ZTE, CATT, MTK). AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo). AL8 PDCCH with 41-bit payload, occupies 576 REs; 41 bits; 1 (CATT). SSS-based PEI; 1-symbol SSS, occupying 132 REs (11 RB x 1 symbol); 3 bits; 1 (IDCC). 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung). 3 bits; 1 (IDCC). 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); ≥ 8 bits; 1 (Intel). 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi). 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (Samsung). 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 3 (vivo, ZTE, Ericsson). 6 bits; 1 (CATT). 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (OPPO, QC). 4 bits; 1 (MTK). PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 4 (OPPO, ZTE, MTK, Intel). SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); 3 bits; 1 (Intel). 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 8 bits; 1 (Intel). 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO). 4 bits; 1 (MTK). - If Behv-B is assumed, Paging Setting; PEI candidate design; Physical-layer configuration; UE (sub)group indication capcity; Number of companies providing performance results. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 4 (HW/HiSi, OPPO, ZTE, MTK). AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 2 (vivo, Samsung). SSS-based PEI; 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 3 (HW/HiSi, vivo, ZTE). 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi). 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (vivo, ZTE). 6 bits; 1 (CATT). 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO). 2 bits; 1 (MTK). PDSCH: MCS0, TB scaling 0.5 PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 3 (OPPO, ZTE, MTK). SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK). TRS/CSI-RS-based PEI; 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO). 2 bits; 1 (MTK). Observation 3a: For the evaluation and comparison of PEI candidate designs, the following summarize average resource overheads per PO for PEI candidate designs, considering the configurations identified from performance observation. - The average overhead results are based on PO settings without impact from UE sub-grouping indication within the PO. - Note: For comparison purpose, single-beam transmission for PEI is assumed, and results with multi-beam transmission for PEI is scaled. This doesn’t preclude any beam-forming related design for PEI. - If Behv-A is assumed: Paging Setting; PEI candidate design; Physical-layer configuration and resource; UE (sub)group indication capacity; Number of companies providing performance results; Average resource overhead per PO (REs); PO and PEI related assumptions; Resource sharing assumption. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 5 (HW/HiSi, OPPO, ZTE, CATT, MTK); 17.2; OPPO; 1 PEI for up to 12 PO's; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. 17.2; ZTE; 1 PEI for up to 12 PO's. 17.6; HW/HiSi; 1 PEI for up to 12 PO's. 21.8; MTK; 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle. 28.8; CATT; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 288.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo); 49.5; vivo; 1 PEI for 4 PO; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. 57.6; vivo; 1 PEI for 1 PO. 57.6; QC; 1 PEI for 1 PO. 57.6; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam. AL8 PDCCH with 41-bit payload, occupies 576 REs; 41 bits; 1 (CATT); 57.6; CATT; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 576.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. SSS-based PEI; 1-symbol SSS, occupying 132 REs (11 RB x 2 symbols); 3 bits; 1 (IDCC). 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung); 25.4; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam; Dynamic rate-matching in PDSCH. 25.4; vivo; 1 PEI for 1 PO. 26.4; ZTE; 1 PEI for 1 PO. 28.8; CATT; 1 PEI for 1 PO. 28.8; QC; 1 PEI for 1 PO. 254.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 264.0; ZTE; 1 PEI for 1 PO. 288.0; QC; 1 PEI for 1 PO. 3 bits; 1 (IDCC). 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK); 34.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; Dynamic rate-matching in PDSCH. 437.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); ≥ 8 bits; 1 (Intel); 14.4; Intel; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi); 123.4; HW/HiSi; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 168.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (Samsung); 21.6; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam; Dynamic rate-matching in PDSCH. 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 3 (vivo, ZTE, Ericsson); 28.8; vivo; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 28.8; ZTE; 1 PEI for 1 PO. 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 6 bits; 1 (CATT); 28.8; CATT; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 288.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (OPPO, QC); 30.0; OPPO; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 30.0; QC; 1 PEI for 1 PO. 300.0; QC; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 4 bits; 1 (MTK); 26.0; MTK; 1 PEI for up to 4 PO's; Dynamic rate-matching in PDSCH. PDSCH: MCS0, TB scaling 0.5; PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 4 (OPPO, ZTE, MTK, Intel); 34.4; OPPO; 1 PEI for up to 12 PO's; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. 43.6; MTK; 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle. 57.6; Intel; 1 PEI for 1 PO; PEI is transmitted as a Rel-15 PDCCH in a CORESET when a UE group is paged. SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 4 bits; 1 (MTK); 34.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; Dynamic rate-matching in PDSCH. 437.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols); 3 bits; 1 (Intel); 14.4; Intel; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols); 8 bits; 1 (Intel); 21.6; Intel; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO); 30.0; OPPO; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 4 bits; 1 (MTK); 26.0; MTK; 1 PEI for up to 4 PO's; averaged all PO settings for 1.28-sec cycle; Dynamic rate-matching in PDSCH. - If Behv-B is assumed: Paging Setting; PEI candidate design; Physical-layer configuration; UE (sub)group indication capacity; Number of companies providing performance results; Average resource overhead per PO (REs); PO and PEI related assumptions; Coexistence assumption. PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload; PDCCH-based PEI; AL4 PDCCH with 12-bit payload, occupying 288 REs; 12 bits; 4 (HW/HiSi, OPPO, ZTE, MTK); 24.0; HW/HiSi; 1 PEI for up to 12 PO's; PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET. 24.0; OPPO; 1 PEI for up to 12 PO's. 24.0; ZTE; 1 PEI for up to 12 PO's. 51.0; MTK; 1 PEI for up to 12 PO's; averaged all PO settings for 1.28-sec cycle. AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 2 (vivo, Samsung); 518.4; vivo; 1 PEI for 1 PO; PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET. 518.4; Samsung; 1 PEI for 1 PO; PEI RE# scaled w.r.t. 1-beam. SSS-based PEI; 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols); 1 bit; 3 (HW/HiSi, vivo, ZTE); 228.6; vivo; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 228.6; ZTE; 1 PEI for 1 PO. 254.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 264.0; ZTE; 1 PEI for 1 PO. 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK); 561.0; MTK; 1 PEI for 1 PO; average over all PO settings for 1.28-sec cycle; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (HW/HiSi); 168.0; HW/HiSi; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 1 bit; 2 (vivo, ZTE); 259.2; ZTE; 1 PEI for 1 PO; Dynamic rate-matching in PDSCH. 259.2; vivo; 1 PEI for 1 PO. 6 bits; 1 (CATT); 288.0; CATT; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO); 279.0; OPPO; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 2 bits; 1 (MTK); 150.0; MTK; 1 PEI for 2 PO's; Semi-static rate-matching in PDSCH. PDSCH: MCS0, TB scaling 0.5 PDCCH: AL16, 41-bit payload; PDCCH-based PEI; AL8 PDCCH with 12-bit payload, occupying 576 REs; 12 bits; 3 (OPPO, ZTE, MTK); 48.0; OPPO; 1 PEI for up to 12 POs; PEI is ALWAYS transmitted as a Rel-15 PDCCH in a CORESET. 102.0; MTK; 1 PEI for up to 12 POs. SSS-based PEI; 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols); 1 bit; 1 (MTK); 561.0; MTK; 1 PEI for 1 PO; RB-symbol rate-matching pattern period up to 40 ms; Semi-static rate-matching in PDSCH. TRS/CSI-RS-based PEI; 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols); 1 bit; 1 (OPPO); 270.0; OPPO; 1 PEI for 1 PO; Semi-static rate-matching in PDSCH. 2 bits; 1 (MTK); 150.0; MTK; 1 PEI for 2 PO's; Semi-static rate-matching in PDSCH.

During RAN1 103e [2], the following was agreed in connection to paging enhancements in idle/inactive mode.

Agreements: Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication within a PO can provide the following power saving gains w.r.t. Rel-16: - If the original group paging rate is 10%: - [0.3%] - [1.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [0.4%] - [0.8%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [0.3%] - [1.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 80% and showed following results: - [0.7%] - [7.6%] where the baseline assumes 1 SS burst for synchronization before PO reception - [0.8%] - [3.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [0.5%] - [4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception The number of UE sub-groups evaluated ranges from 2 to 16. Some companies show concern on assuming group paging rate larger than 60%. Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement. Agreements: Observation: For NR idle/inactive-mode UEs, UE sub-grouping indication carried in paging early indication can provide the following power saving gains w.r.t Rel-16: - If the original group paging rate is 10%: - [10.6%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [16.0%] –[36.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [14.3%] –[46.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results: - [8.0%] –[19.1%] where the baseline assumes 1 SS burst for synchronization before PO reception - [18.1%] –[34.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [20.6%] –[42.0%] where the baseline assumes 3 SS bursts for synchronization before PO reception The additional power saving gains w.r.t. paging early indication without UE sub-grouping are given as follows: - If the original group paging rate is 10%: - [0.6%] –[2.7%] where the baseline assumes 1 SS burst for synchronization before PO reception - [0.6%] –[4.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [0.6%] –[4.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception - Some sources also evaluated performance if the original group paging rate is in the range between 20% and 60% and showed following results: - [1.3%] –[8.0%] where the baseline assumes 1 SS burst for synchronization before PO reception - [2.1%] –[13.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [3.3%] –[16.1%] where the baseline assumes 3 SS bursts for synchronization before PO reception The number of UE sub-groups evaluated ranges from 2 to 16. The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB. Note: It is FFS in RAN1 another group paging rate > 10% for the evaluation of Rel-17 paging enhancement. Note: Not all sources providing results for paging early indication without UE sub-grouping also provide results for paging early indication with UE sub-grouping. Agreements: Observation: For NR idle/inactive-mode UEs with 10% group paging rate, cross-slot scheduling with K0 = 1, which can be supported by Rel-15/Rel-16 for Type 2 CSS, can provide the following power saving gains w.r.t. same-slot scheduling (K0 = 0): - [<1%] –[2.5%] where the baseline assumes 1 SS burst for synchronization before PO reception - [<1%] -[1.6%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [<1%] -[1.44%] where the baseline assumes 3 SS bursts for synchronization before PO reception One source shows that cross-slot scheduling with K0 = 32, which cannot be supported by Rel-15/Rel-16 for Type 2 CSS, can provide the following power saving gains w.r.t. same-slot scheduling (K0 = 0): - [0%] where the baseline assumes 1 SS burst for synchronization before PO reception - [6.3%] where the baseline assumes 3 SS bursts for synchronization before PO reception The power saving gain will become lower with higher group paging rate. Agreements: For NR idle/inactive-mode paging enhancement, paging early indication before paging occasion is supported from RAN1 perspective - FFS: Physical layer design based on DCI, SSS or TRS/CSI-RS - Send LS to inform RAN2 and kindly ask RAN2 to inform RAN1 if there is anything that RAN1 should take into consideration in the physical layer design for this feature, including any other progress RAN2 has made in this WI which may has RAN1 impact Agreements: Observation: For NR idle/inactive-mode UEs with 10% group paging rate, paging early indication without UE sub-grouping can achieve the following power saving gains w.r.t. Rel-16: - [0%] - [22.8%] where the baseline assumes 1 SS burst for synchronization before PO reception - Note: [0%] means UE can apply the baseline behavior if the time offset between the utilized SS burst and PO is small. - [5.0%] - [32.0%] where the baseline assumes 2 SS bursts for synchronization before PO reception - [10.2%] - [67.7%] where the baseline assumes 3 SS bursts for synchronization before PO reception The power saving gains will become lower for higher group paging rate. The power saving gains are dependent on the assumptions about placement of PEI and PO relative to SSB. The power saving gains may vary with different paging early indication design.

In this contribution, we express our views on different candidate signal/channel to carry the PEI.

# Discussion on UE power consumption

In RAN1 103e, it was agreed that PEI is supported in Rel-17 for idle/inactive mode UE. Potential candidates include PDCCH based design and sequence-based designs such as using TRS/CSI-RS or SSS. In this section, we discuss and compare the candidates in terms of relative power consumption.

If a PDCCH based PEI is used, UE may need to process one or more SSBs before PDCCH based PEI to detect the PEI and again before PO to achieve sufficiently low CFO for paging reception. This may result in increased power consumption, depending on the total number of additional SSBs processed leading to the PO. The first two examples in Figure 1 show possible timelines when PDCCH based PEI indicates the UE to wake up.

On the other hand, if TRS based PEI is used adjacent to PO, UE may be able to stay in deep sleep for a longer duration, and TRS with a preceding SSB may be sufficient to achieve low CFO. Hence, there is a larger potential for increased power saving (PS) gain with TRS based design compared to PDCCH based PEI. The last example in Figure 1 shows a possible processing timeline when TRS based PEI is received.

Figure 1. Processing timeline when UE receives PEI.

While in Example 1, it is assumed that reception of a single SSB may be sufficient for achieving sufficiently low residual time/frequency errors for PDCCH PEI reception, this may not always be feasible. Thus, Example 2 is also considered that assumes that multiple SSBs may be needed before PDCCH (PDCCH PEI) may be decoded successfully.

In Table 1, we show power saving gain comparison for sequence and PDCCH based PEI, assuming different locations for PDCCH based PEI. Assumptions include GPR: 10%, Paging cycle: 1.28S, PO duration: 4ms, SSB duration and periodicity: 2ms and 20ms.

Table1: Power consumption of TRS and PDCCH based PEI with different number of sub-groups

| Number of sub-groups indicated by PEI | Seq based PEI (1 SSB before PEI, 0 SSB between PEI and PO) (values in % in indicate power consumption w.r.t. Seq-based PEI and a single sub-group) | PDCCH PEI is placed far from PO, with 1 SSB in front, and 1 or more SSBs between PEI and PO (example 1) (values in % indicate power consumption w.r.t. Seq-based PEI) |  |  | PDCCH PEI is placed closed to PO, with 1 or more SSBs in front (example 2) (values in % indicate power consumption w.r.t. Seq-based PEI) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PDCCH based PEI (1 SSB before PEI, 1 SSB between PEI and PO) | PDCCH based PEI (1 SSB before PEI, 2 SSBs between PEI and PO) | PDCCH based PEI (1 SSB before PEI, 3 SSBs between PEI and PO) | PDCCH based PEI (2 SSBs before PEI, 0 SSB between PEI and PO) | PDCCH based PEI (3 SSBs before PEI, 0 SSB between PEI and PO) |
| 1 | 1.4243 (0.00%) | 1.4477 (1.64% more) | 1.4712 (3.29% more) | 1.4946 (~ 5% more) | 1.6541 (16.13% more) | 1.885 (32.34% more) |
| 4 | 1.4108 (1.0% less) | 1.4166 (0.41% more) | 1.4225 (0.83% more) | 1.4283 (1.24% more) | 1.6451 (16.6% more) | 1.8795 (33.22% more) |
| 8 | 1.4085 (1.12% less) | 1.4114 (0.21% more) | 1.4144 (0.42% more) | 1.4173 (0.63% more) | 1.6429 (16.64% more) | 1.8773 (33.28% more) |
| 12 | 1.4078 (1.15% less) | 1.4097 (0.14% more) | 1.4117 (0.27% more) | 1.4137 (0.42% more) | 1.6421 (16.64% more) | 1.8765 (33.29% more) |
| 16 | 1.4074 (1.19% less) | 1.4089 (0.1% more) | 1.4103 (0.21% more) | 1.4118 (0.32% more) | 1.6418 (16.66% more) | 1.8761 (33.30% more) |

As can be seen from Table 1, Example 2 can result in significantly higher UE power consumption compared to sequence-based PEI or for PDCCH based PEI per Example 1. This is because, unlike in Example 1, in Example 2, the UE cannot skip SSB processing, regardless of successful detection of a PEI or not.

Further, it can be observed by comparing the power consumption values for a given PEI design with increasing number of sub-groups (relative values shown in Table 1 for sequence-based PEI) that the relative gains as a function of the number of UE sub-groups diminish as the number of sub-groups increases. In fact, the gains saturate beyond 4 sub-groups.

Observation 1: For the agreed evaluation assumptions, TRS based PEI results in more power saving gain than PDCCH based PEI

Observation 2: For the agreed evaluation assumptions, relative PS gain diminishes as number of sub-groups is increased.

- The PS gains saturate from 4 sub-groups onwards.

Accordingly, considering the evaluated scenarios, a maximum of 4 sub-groups may be sufficient. Even when considering other use-cases, e.g., involving higher paging rates, it is not expected from the trend of results in Table 1 that more than 4 to 8 sub-groups can provide any additional gains. Thus, we have the following proposal.

Proposal 1: A maximum of 4 sub-groups per PO is preferred. Considering feedback from RAN2, at most 8 sub-groups may be supported per PO.

# UE Behavior upon detecting PEI

In this section, we compare the detection performance of PDCCH and TRS or SSS based PEI.

For evaluation of the PEI candidates, two behaviors were identified in RAN1 104e.

- Behv-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

- Behv-B:

  - PEI indicates whether or not UE should monitor a PO

  - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

In Behv-A, PEI is only transmitted if a UE needs to monitor PO, otherwise PEI is not transmitted, i.e., DTX of PEI implies not monitor PO from a UE perspective. On the other hand, Behv-B allows to transmit PEI even if the UE is not required to monitor PO. DTX of PEI implies UW would monitor PO. Hence, in Behv-B, UE could potentially monitor PO more often than it would in Behv-A. Moreover, Behv-B could cause more signaling overhead since PEI can be transmitted even if the UE is not required to monitor PO.

Observation 3: For 1 PEI to 1 PO association, Behv-B could cause more signalling overhead for PEI transmission and/or increased UE power consumption compared to Behv-A. This makes benefits of Behv-B over Behv-A questionable.

Behv-B may potentially require mapping of sub-groups across multiple PO. In Section 4, we discuss the issues with that consideration.

Proposal 2: Support Behv-A only as PEI functionality.

- Behv-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

# MDR and RE number comparison

From the observation tables in the agreement from RAN1 #104b-e meeting (Observation 3a), we observe that TRS based PEI may require much less number of REs than PDCCH based PEI to meet a given MDR target.

In particular, the following observations from analyses reported to RAN1 #104b-e are highlighted.

Observation 4: Both TRS and PDCCH-based PEI could meet the MDR requirements however TRS-based PEI provides significant resource overhead advantage over PDCCH-based PEI.

- For example, to meet SNR -7.79dB at 1% BLER for PDSCH with TB scaling 1 and CFO 0ppm, 24 RBs (144 REs) suffices for TRS whereas AL8 (576 REs) are needed for PDCCH-based PEI.

Observation 5: TRS-based PEI may include UE subgrouping information for at least 8 sub-groups where TRS BW can be as low as 24 RBs

Observation 6: AL 8+ is necessary for PDCCH-based PEI in most cases.

Observation 7: Joint MDR is mostly dominated by paging PDCCH performance, i.e., at the target SNR, MDR of TRS with FAR 1% is much lower than that of paging PDCCH.

Although only TRS-based PEI is considered in the above, SSS-based PEI is also expected to perform similarly as TRS, if not better. In [4], it was shown that SSS-based PEI also offers very good MDR performance. Hence, joint MDR based on SSS-based PEI (254 REs over two symbols in a slot) is also expected to outperform PDCCH-based PEI at least in terms of resource overhead.

# Consideration on coexistence, system, and specification impacts

None of the PEI design seems to impact legacy UE functionality, i.e., legacy paging can be performed as usual.

Next, we discuss two matters that we subject to extensive debates during last RAN1 meeting.

PEI relates to one or multiple PO?

In our view, 1 PEI to 1 PO association should be baseline consideration for comparison. It has been heavily debated in RAN1 104e regarding 1 PEI to multiple PO association. In our view, 1 PEI to multiple PO has the following issues:

- In such case, PEI may indicate sub-groups which span multiple POs. Hence, gap between PEI monitoring occasion and PO for a given UE can be large which can increase paging latency and also feasibility/practicality of such a-priori determination by gNB much ahead of a PO is questionable.

- Spanning sub-groups across multiple PO have significant RAN2 impact and also makes the UE sub-grouping process more complex

Observation 8: Spanning sub-groups over multiple PO causes following issues:

- Gap between PEI monitoring occasion and PO for a given UE can be large which can increase paging latency and also feasibility/practicality of such a-priori determination by gNB much ahead of a PO is questionable.

- It makes the UE sub-grouping process significantly more complex.

Coexistence with legacy signal/channels

An observation was captured in RAN1 104bis that only TRS based PEI supports dynamic RE level rate matching with PDSCH, whereas PDCCH supports CORESET level rate matching. It has been argued that the number of ZP-CSI-RS patterns for RE-level rate-matching is limited. However, this is not an issue

Observation 9: TRS-based PEI achieves most efficient rate matching when coexisting with PDSCH.

Moreover, some companies have assumed dynamic CORESET sharing for PDCCH based PEI. It needs to be noted that in some cases, gNB may need to use a CORESET only to transmit the PEI. Hence, in such cases, the entire CORSET may be translated as overhead in the context of dynamic CORESET-level resource sharing with PDSCH.

Observation 10: At a given instance, the CORESET where PDCCH based PEI is transmitted may not have other PDCCH candidates transmitted, resulting in much larger overhead than resource occupied by PDCCH based PEI when dynamic CORESET-level rate matching is considered.

Further, we note that TRS can coexist with CORESETs and SSB following Rel-15 design and additional spec efforts are not necessary in this regard.

Observation 11: TRS can coexist with CORESET and SSB according to Rel-15 design.

Specification Impact

TRS based PEI design can be based on Rel-15 legacy TRS pattern, whereas PDCCH based PEI will likely require introduction of a new DCI format.

Overall comparison

To summarize the above discussion, in Table 2, we compare the overall comparison between TRS-like sequence-based PEI and PDCCH based PEI based on MDR performance, resource OH, coexistence considerations, and specification efforts, under the agreed evaluation methodology and assumptions.

Table 2: Comparison of PEI designs

| PEI designs Metrics | TRS-like sequence-based PEI | DCI-based PEI |
| --- | --- | --- |
| MDR Performance | Winner |  |
| Resource overhead | Winner |  |
| Coexistence | Winner |  |
| Spec effort | Tie | Tie |

# Design of Paging Early Indication

Based on the discussion above, we propose the following:

Proposal 3: Support sequence-based PEI (TRS/CSI-RS or SSS based) in Rel-17 for idle/inactive mode UE.

Sub-grouping indication by Sequence based PEI can be achieved in different ways, such as

- (Sequence-selection) All combinations of N subgroups can be indicated via sequence transmission, i.e., sequence transmission to emulate N-bit bitmap

  - UE needs to check for only 2^(N-1) sequences

- (CDM-based) A time-frequency resource is shared to indicate number of sub-groups in CDM manner, e.g., OCC can be used to distinguish a first set of sub-group from second set of sub-groups

  - In this approach, UE may need to do fewer hypothesis testing than first method

- (Sequence-selection + orthogonal resources) Using a combination of sequence-selection-based approach with orthogonal time-frequency resources

  - In this approach, the number of sequences to be searched from can be reduced (reduced number of hypotheses) on a given time-frequency resource at the cost of increased OH.

- Other combinations of the above may also be possible.

Considering indication of paging UE sub-groups via PEI, as one option, 2^N sequences may be defined to indicate all combinations of N sub-groups. However, for a given UE, only those sequences corresponding to which the UE’s sub-group is indicated for waking up are relevant. Thus, the UE may only need to check 2^(N-1) sequences, thereby further improving the detection performance compared to the case with 2^N sequences.

Observation 12: If N sub-groups are indicated via sequence transmission, UE needs to check 2^(N-1) sequences, i.e., only the sequences that would wake up UE’s subgroup.

Moreover, we observed that a reasonable number of UE sub-groups can be indicated directly via TRS -based PEI without any significant degradation in the detection performance. If a large number of UE sub-groups need to be indicated, joint indication by PEI and paging DCI can be considered. PEI may indicate a set of UE sub-groups, and subsequently, paging DCI indicates which sub-groups within the indicated set would receive PDSCH.

In Table 3, we show power consumption difference when two stage UE sub-grouping indication is provided, compared to when all sub-grouping information is indicated via PEI. We observe that power consumption is only marginally increased. Hence, we have the following observation and proposal.

Table 3: Power consumption (average value/slot) of different UE sub-grouping indication methods. 1 SSB is monitored before PEI. No additional SSB monitoring is assumed between PEI and PO. GPR 10% and 1.28S cycle are assumed.

| # subgroups | PEI indication (number of sub-groups/sets) | PO (number of sub-groups per set) | Power Consumption - No Cross-Slot Sch. | Power Consumption – w/ Cross-Slot Sch. |
| --- | --- | --- | --- | --- |
| 8 | 8 | 1 | 1.4085 | 1.4081 |
|  | 2 | 4 | 1.4142 | 1.4126 |
|  | 4 | 2 | 1.4104 | 1.4096 |
| 16 | 16 | 1 | 1.4074 | 1.4072 |
|  | 2 | 8 | 1.4141 | 1.4125 |
|  | 8 | 2 | 1.4083 | 1.4079 |
|  | 4 | 4 | 1.4102 | 1.4094 |

Observation 13: Although PDCCH-based PEI may potentially include information of a larger number of UE sub-groups, this does not seem to offer critical PS advantage compared to the case, when both PEI and Paging PDCCH jointly indicate the UE sub-groups.

Proposal 4: Both PEI and paging DCI may jointly indicate UE sub-grouping information, especially when number of sub-groups is large and PEI is sequence based.

Signalling design for UE subgrouping can be discussed after making progress on the PEI design and corresponding UE behavior.

Proposal 5: Signalling design for UE sub-grouping can be postponed until after PEI signal/channel design is confirmed.

# Conclusions

In summary, we have following list of proposals and observations:

Observation 1: For the agreed evaluation assumptions, TRS based PEI results in more power saving gain than PDCCH based PEI

Observation 2: For the agreed evaluation assumptions, relative PS gain diminishes as number of sub-groups is increased.

- The PS gains saturate from 4 sub-groups onwards.

Observation 3: For 1 PEI to 1 PO association, Behv-B could cause more signalling overhead for PEI transmission and/or increased UE power consumption compared to Behv-A. This makes benefits of Behv-B over Behv-A questionable.

Observation 4: Both TRS and PDCCH-based PEI could meet the MDR requirements however TRS-based PEI provides significant resource overhead advantage over PDCCH-based PEI.

- For example, to meet SNR -7.79dB at 1% BLER for PDSCH with TB scaling 1 and CFO 0ppm, 24 RBs (144 REs) suffices for TRS whereas AL8 (576 REs) are needed for PDCCH-based PEI.

Observation 5: TRS-based PEI may include UE subgrouping information for at least 8 sub-groups where TRS BW can be as low as 24 RBs

Observation 6: AL 8+ is necessary for PDCCH-based PEI in most cases.

Observation 7: Joint MDR is mostly dominated by paging PDCCH performance, i.e., at the target SNR, MDR of TRS with FAR 1% is much lower than that of paging PDCCH.

Observation 8: Spanning sub-groups over multiple PO causes following issues:

- Gap between PEI monitoring occasion and PO for a given UE can be large which can increase paging latency and also feasibility/practicality of such a-priori determination by gNB much ahead of a PO is questionable.

- It makes the UE sub-grouping process significantly more complex.

Observation 9: TRS-based PEI achieves most efficient rate matching when coexisting with PDSCH.

Observation 10: At a given instance, the CORESET where PDCCH based PEI is transmitted may not have other PDCCH candidates transmitted, resulting in much larger overhead than resource occupied by PDCCH based PEI when dynamic CORESET-level rate matching is considered.

Observation 11: TRS can coexist with CORESET and SSB according to Rel-15 design.

Observation 12: If N sub-groups are indicated via sequence transmission, UE needs to check 2^(N-1) sequences, i.e., only the sequences that would wake up UE’s subgroup.

Observation 13: Although PDCCH-based PEI may potentially include information of a larger number of UE sub-groups, this does not seem to offer critical PS advantage compared to the case, when both PEI and Paging PDCCH jointly indicate the UE sub-groups.

Proposal 1: A maximum of 4 sub-groups per PO is preferred. Considering feedback from RAN2, at most 8 sub-groups may be supported per PO.

Proposal 2: Support Behv-A only as PEI functionality.

- Behv-A:

  - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged

  - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO

Proposal 3: Support sequence-based PEI (TRS/CSI-RS or SSS based) in Rel-17 for idle/inactive mode UE.

Proposal 4: Both PEI and paging DCI may jointly indicate UE sub-grouping information, especially when number of sub-groups is large and PEI is sequence based.

Proposal 5: Signalling design for UE sub-grouping can be postponed until after PEI signal/channel design is confirmed.

References

- RP-193239, “New WID: UE Power Saving Enhancements,” MediaTek.

- 3GPP RAN1 # 104-e Chairman Notes.

- 3GPP RAN1 # 103-e Chairman Notes.

R1-2100662, On paging enhancements for UE power saving, Intel Corporation, RAN1-104e.

Appendix

Table 3: Link level Simulation parameters for TRS and SSS.

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4 GHz |
| System BW | 20 MHz |
| Number of Tx/Rx antennas | 2 / 4 |
| SCS | 30 kHz |
| Modulation | TRS – QPSK, |
| Channel | TDL-C |
| Delay Spread | 300ns |
| UE speed | 27 km/hr |
| WUS sequence types | TRS |
| OS mapping | {4, 8} |
| WUS Burst Length | 1, 2 slots |
| BW | TRS – 24, 36, 48 RBs |
| # of sub-groups | 2, 3, 4, 6, 8 |
| Carrier Frequency offset | 𝒰(-X, X), X = 0, 0.1, 0.5, 1 ppm of carrier frequency |

Table 4: Link level Simulation parameters for PDCCH and PDSCH

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4 GHz |
| System BW | 20 MHz |
| Number of Tx/Rx antennas | 2 / 4 |
| SCS | 30 kHz |
| Modulation | QPSK for PDCCH, MCS 0 for PDSCH |
| Channel | TDL-C |
| Delay Spread | 300ns |
| PDCCH configurations | AL4, AL8, AL16 12 or 40 info + 24 CRC bits, REG bundle size 6 |
| PDSCH configurations | Mapping type A, MCS 0, 48 PRB, TB scaling 1, 0.5 and 0.25, DMRS type 1 with 2 additional DMRS |
| UE speed | 27 km/hr |
| Duration | CORESET: 2OS, PDSCH: 12 OS |
| CCE to PRB / VRB to PRB mapping | Distributed |
| BW | Up to 48RBs |
| Carrier Frequency offset | 𝒰(-X, X), X = 0, 0.1, 0.5, 1 ppm of carrier frequency |