3GPP TSG RAN WG1 #105-e			R1-2104927

e-Meeting, May 10th – 27th, 2021

Source:	Intel Corporation

Title:	Inter-UE Coordination Schemes for Sidelink Communication

Agenda item:	8.11.1.2

Document for:	Discussion and Decision

Introduction

The RAN WG approved work item on NR Sidelink Enhancements [1] with the following objective:

- Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. Note: RAN2 work will start after RAN#89.

At the RAN1#104bis-e meeting, the support of the following inter-UE coordination schemes was agreed by RAN WG1:

- Support the following schemes of inter-UE coordination in Mode 2: - Inter-UE Coordination Scheme 1: - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information - FFS condition(s) in which Scheme 1 is used - Inter-UE Coordination Scheme 2: - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict - FFS condition(s) in which Scheme 2 is used

In this contribution, we express our views on details of the agreed inter-UE coordination schemes. Our views on sidelink power saving objectives are provided in companion contribution [2].

# Inter-UE Coordination for Mode-2 Sidelink Communication

## Mode-2 Sidelink Communication Conflicts

From the single TX UE perspective, the Rel.16 Mode-2 sidelink communication design may have the following conflicts that can be potentially addressed by inter-UE coordination signaling:

- Half-duplex in transmission (HD-TX)

  - TX UE1 and TX UE2 transmitted in the same slot. UE1 is a target RX of UE2 and/or UE2 is a target RX of UE1

- Half-duplex in reservation (HD-RSV)

  - TX UE1 and TX UE2 reserved resource in the same slot. UE1 is a target RX of UE2 and/or UE2 is a target RX of UE1

- Half-duplex in reception (HD-RX)

  - TX UE1 reserved resource for transmission to UE2. UE2 prioritized its own transmission over reception from TX UE1

- Co-channel collision in transmission (CC-TX)

  - TX UE1 and TX UE2 transmitted in the same slot on overlapped frequency resources. UE1 is not a target RX of UE2 and UE2 is not a target RX of UE1

- Co-channel collision in reservation (CC-RSV)

  - TX UE1 and TX UE2 reserved resource in the same slot on overlapped frequency resources. UE1 is not a target RX of UE2 and UE2 is not a target RX of UE1

Each of the described above sidelink conflicts can be semi-persistent or dynamic depending on transmission mode. The described above sidelink conflicts can be mitigated by means of inter-UE coordination feedback signaling, further considered in this document.

  - RAN WG1 to discuss which of the following sidelink conflicts should be addressed by inter-UE coordination signaling

    - Half-duplex in transmission (HD-TX)

    - Half-duplex in reservation (HD-RSV)

    - Half-duplex in reception (HD-RX)

    - Co-channel collision in transmission (CC-TX)

    - Co-channel collision in reservation (CC-RSV)

  - Inter-UE coordination signaling to at least address sidelink conflicts of half-duplex in transmission

## Inter-UE Coordination Information and Signalling

Two types of inter-UE coordination schemes were agreed by RAN WG1. The following inter-UE coordination signaling options can be considered for NR sidelink communication:

- Scheme 1: Higher layer signaling over PSSCH for sidelink conflict avoidance

  - Pros:

    - Flexible payload size

  - Cons:

    - Relatively high latency

- Scheme 2: Physical layer signaling over PSFCH for sidelink conflict resolution

  - Pros:

    - Low latency dynamic feedback, that can mitigate sidelink conflict during transmission of a given TB

    - Similarity with HARQ ACK/NACK feedback mechanism

  - Cons:

    - Relatively small payload

The physical layer signaling can dynamically address sidelink conflicts during transmission of a given TB and can be used to mitigate impact of the following sidelink conflicts: half-duplex in transmission/reservation/reception and co-channel collisions in transmission/reservation.

The higher layer signaling should be considered as assistance information for TX UE resource selection and should mainly target enhancements for semi-persistent resource (re)-selection by TX UE.

  - For inter-UE coordination scheme 1, support higher layer signaling to optimize semi-persistent resource allocation

    - PSSCH is considered as a baseline for higher layer inter-UE coordination feedback design

  - For inter-UE coordination scheme 2, support physical layer signaling to enable low-latency inter-UE coordination feedback with small payload

    - PSFCH is considered as a baseline for physical layer inter-UE coordination feedback design

### Scheme 1: Higher Layer Signalling for Sidelink Collision Avoidance

The higher layer signaling can be used as inter-UE coordination feedback to TX UEs to assist in semi-persistent resource selection. For that purpose, assisting UEs may share information about candidate resources for resource selection or resources for resource exclusion, i.e. not recommended for sidelink transmission using PC5 RRC or MAC CE signaling. Both sets can be formed by reusing Rel.16 resource allocation procedure as much as possible. The only change that needs to be done is filtering of semi-persistent transmission, since only later ones should be considered for inter-UE coordination feedback preparation. In terms of higher layer signaling, the MAC CE design option seems to be a reasonable approach considering that MAC handles resource allocation aspects.

To generate candidate resource set for resource exclusion or (re-)selection, based on Rel.16 procedures, the reference configuration settings such as priority, resource size, min resource selection window, sensing window, periodicity of transmissions, etc. need to be configured to UE providing inter-UE coordination feedback. Part of these parameters can be preconfigured while some of them can be dynamically acquired from TX UEs.

  - Further study whether higher layer inter-UE coordination signaling for scheme 1 should provide information on

    - Alt.1: Candidate resources for resource (re)-selection procedure

    - Alt.2: Candidate resources for resource exclusion procedure

  - For inter-UE coordination scheme 1,

    - MAC CE is used to carry inter-UE coordination feedback

    - Reuse Rel.16 resource (re)-selection procedure (or resource exclusion procedure) for generation of higher layer inter-UE coordination feedback associated with a given timestamp (i.e. slot)

### Scheme 2: Physical Layer Signalling for Sidelink Conflict Resolution

The physical layer signaling can be used to provide inter-UE coordination feedback to TX UEs on the following sidelink conflicts:

  - Half-duplex in transmission (HD-TX)

  - Half-duplex in reservation (HD-RSV)

  - Half-duplex in reception (HD-RX)

  - Co-channel collision in transmission (CC-TX)

  - Co-channel collision in reservation (CC-RSV)

The described above sidelink conflicts can be differentiated at the RX UE side through SCI processing and (pre)-configured or pre-defined criteria / conditions. The differentiation of conflicts types may be essential from the perspective of prioritization of feedback transmission. In addition, TX UEs utilizing inter-UE coordination feedback may apply different behaviors to handle different types of conflicts when inter-UE coordination feedback is provided.

  - Physical layer inter-UE coordination signaling for scheme 2, supports indication of sidelink conflict type including

    - differentiation of sidelink conflict in transmission from conflict in reservation at TX UE side based on received feedback

    - differentiation of sidelink half-duplex conflict from co-channel collision conflicts at TX UE side based on received feedback

  - The sidelink conflict types indicated by inter-UE coordination feedback can be pre-configured

## Inter-UE Coordination Feedback Request

RX UE when it provides inter-UE coordination feedback should be aware that TX UE can use it to resolve sidelink conflict or improve resource selection procedure. Therefore, it is desirable to support mechanism when feedback is provided based on request from TX UE. To support request of inter-UE coordination feedback, the SCI signaling can be used to trigger feedback.

  - NR supports dynamic SCI signaling to request inter-UE coordination feedback

## Determination of UEs for Inter-UE Coordination Feedback

RAN1 made the following agreement to determine the conditions for UEs to become/serve as UEs providing inter-UE coordination feedback:

- Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination: - Details include applicable scenario(s)/inter-UE coordination scheme(s) - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc. - Including the possibility of being subject to certain conditions and/or capability

### Conditions to Serve as UE-A in Case of Scheme 1

When inter-UE coordination scheme 1 is considered, in our view any UE can be a UE-A to provide inter-UE coordination feedback subject to the following conditions:

Semi-persistent transmissions are enabled (allowed) per sidelink resource pool with a set of periodicities

Inter-UE coordination feedback for collision avoidance is enabled (allowed) per sidelink resource pool

  - This condition can be a function of CBR

UE supports inter-UE coordination scheme 1

UE intends to transmit data (a TB including inter-UE coordination payload)

Amount of time / number of sidelink slots passed from the previous feedback by given UE exceeds pre-configured threshold

UE intends to transmit a TB (that includes inter-UE coordination payload) and supports inter-UE coordination scheme 1

  - For inter-UE coordination scheme 1,

    - Feedback is generated based on semi-persistent transmissions

  - TX UE can apply feedback for both dynamic and semi-persistent transmissions and arbitrary cast type

  - For inter-UE coordination scheme 1, any UE can be a UE-A providing inter-UE coordination feedback subject to following conditions

      - Semi-persistent transmissions are enabled (allowed) per sidelink resource pool

      - Inter-UE coordination feedback for collision avoidance is enabled per sidelink resource pool

      - UE supports inter-UE coordination scheme 1

      - UE intends to transmit data (a TB including inter-UE coordination payload)

      - Amount of time / number of sidelink slots passed from the previous feedback by given UE exceeds pre-configured threshold

### Conditions to Serve as UE-A in Case of Scheme 2

In case of inter-UE coordination for conflict resolution, intended receivers (e.g. group members) should be primary candidates for feedback generation. However, from system perspective limiting the feedback only to group members is too restrictive and therefore additional criteria need to be discussed. For instance, distance or SL-RSRP range from TX UE can be used as additional criteria to serve as UE-A for a given TX UE.

If all UEs within communication range are expected to generate feedback potentially for each slot, it can significantly increase UE power consumption, loading in feedback channel. To avoid these it is possible to distribute UEs across predefined time intervals (feedback cycles) for transmission of inter-UE coordination information.

When inter-UE coordination scheme 2 is considered, in our view any UE can be a UE-A to provide inter-UE coordination feedback subject to the following conditions:

Inter-UE coordination scheme 2 for conflict resolution is enabled per sidelink resource pool

  - This condition can be a function of CBR

Inter-UE coordination feedback is requested by TX UE for given transmission

UE supports sidelink conflict detection (scheme 2) and sidelink conflict is detected

UE is intended receiver or non-intended receiver which is within pre-configured SL-RSRP or distance range from TX UE, if pre-configuration is provided

Amount of time / number of sidelink slots, passed from the previous feedback by given UE, exceeds pre-configured threshold

UE is intended receiver or any other UE which is within pre-configured RSRP or distance range from TX UE

  - For inter-UE coordination scheme 2,

    - Feedback is generated based on semi-persistent and dynamic transmissions

      - TX UE can apply feedback for both dynamic and semi-persistent transmissions and arbitrary cast type

    - Any UE can be a UE-A providing inter-UE coordination feedback subject to following conditions

      - Inter-UE coordination feedback for conflict resolution is enabled per sidelink resource pool

      - Inter-UE coordination feedback is requested by TX UE for a given transmission

      - UE supports inter-UE coordination scheme 2 and detects sidelink conflict

      - UE is intended receiver or non-intended receiver which is within pre-configured SL-RSRP or distance range from TX UE, if pre-configuration is provided

      - Amount of time / number of sidelink slots, passed from the previous feedback by given UE, exceeds pre-configured threshold

## Information for Inter-UE Coordination Feedback

Information used for inter-UE coordination feedback was discussed but not agreed by RAN WG1 and left for further discussion. In this section we provide our views on this aspect.

- For inter-UE coordination in Mode 2, consider at least one of the following information (with details FFS) - For Scheme 1 to determine by UE-A the set of resources preferred and/or non-preferred for UE-B’s transmission: - Other UEs’ reserved resources based on UE-A’s sensing result and/or coordination information (e.g., non-preferred resource set) received from other UEs - Coordination information (e.g., preferred resource set) received from other UEs - Information on UE-B’s traffic requirements (e.g., conveyed via triggering information from UE-B, if any) - Location information on UE-B and other UEs - Subset or all of UE-A’s NR SL resources selected for its transmission(s) of TB(s) - UE-A’s scheduled/configured resources for UL - LTE SL transmission and/or reception of UE-A - Resource set selected by UE-A for other UE-Bs’ transmissions - PSFCH transmission and/or reception - UE-A’s candidate resource set based on UE-A’s sensing - UE-B’s ability to use coordination information - Etc. - For Scheme 2 to determine by UE-A the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI: - Other UEs’ reserved resources and/or existing transmission (i.e. used resources) based on UE-A’s sensing result (e.g., measurement, information extracted from SCI) - Information on UE-B’s traffic requirements - Location information on UE-B and other UEs - Subset or all of UE-A’s NR SL resources selected for its transmission(s) of TB(s) - UE-A’s scheduled/configured resources for UL - LTE SL transmission and/or reception of UE-A - PSFCH transmission and/or reception - UE-B’s ability to use coordination information - Etc.

### Information for Inter-UE Coordination in Scheme 1

To determine by UE-A the set of resources preferred and/or non-preferred for UE-B’s transmission, the following information can be used in case of Scheme 1:

- Other UEs’ reserved resources based on UE-A’s sensing result (i.e. information extracted from SCI processing and SL RSRP measurements)

- Subset or all of UE-A’s NR SL resources selected for its transmission(s) of TB(s) (i.e. can be excluded from candidate resource set report)

- UE-A’s scheduled/configured resources for UL (i.e. can be excluded from candidate resource set report)

- UE-A’s candidate resource set based on UE-A’s sensing

- Information for resource exclusion and/or resource selection procedure used for inter-UE coordination feedback (reference configuration)

### Information for Inter-UE Coordination in Scheme 2

To determine by UE-A the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI:

- Other UEs’ reserved resources and/or existing transmission (i.e. used resources) based on UE-A’s sensing result (e.g., measurement, information extracted from SCIs – occupied/reserved resources, priority, source / destination IDs)

  - It is needed to determine sidelink conflict and its type

- Information on UE-B’s traffic requirements

  - We assume sidelink transmission priority should be used and can be extracted directly from SCI

- SL resources selected for UE-A’s transmission(s) of TB(s)

  - It is needed to address potential HD-RX sidelink conflict if UE-A is intended receiver of UE-B

- UE-A’s scheduled/configured resources for UL

  - It is needed to address potential HD-RX sidelink conflict if UE-A is intended receiver of UE-B

- PSFCH transmission and/or reception

  - This aspect can be further considered and evaluated for inter-UE coordination scheme 2

- Location information on UE-B and other UEs

  - It is needed to determine sidelink conflict type (HD-TX/RSV or CC-TX/CC-RSV) at least in groupcast NACK only sidelink communication. Can be also used to prioritize inter-UE coordination feedback

- UE-B’s ability to use coordination information

  - It is needed to determine whether feedback is requested

## Resources for Inter-UE Coordination Feedback Transmission

### Determination of Resources for Inter-UE Coordination Scheme 1

If higher layer inter-UE coordination feedback over PSSCH is agreed by the group, then resource for transmission should be determined based on Mode-2 resource allocation framework, including potential Rel.17 enhancements. RAN1 may need to discuss the priority associated with transmission carrying inter-UE coordination feedback.

  - If higher layer inter-UE coordination feedback over PSSCH is agreed, the Mode-2 resource allocation framework (procedures), including potential Rel.17 enhancements, should be used for inter-UE coordination feedback

### Determination of Resources for Inter-UE Coordination Scheme 2

For the case of PSFCH-like inter-UE coordination feedback, the timing / slot for inter-UE coordination feedback should be separately discussed for each type of sidelink collision. The selected resource should at least carry information about Source ID, sidelink conflict type (e.g. conflict in transmission or reservation), so that TX UEs can distinguish inter-UE coordination feedback directed to him and its type.

  - In case of physical layer inter-UE coordination feedback using PSFCH resources, the PSFCH resource (time/frequency/code) selected for feedback should be associated with Source ID, sidelink feedback type (conflict type)

## Prioritization of Inter-UE Coordination Feedback Transmission

### Inter-UE Coordination Feedback Transmission in Scheme 1

For transmission of inter-UE coordination information in Scheme 1, there is no need to differentiate TX UEs (i.e. feedback is a broadcast signaling). When UE multiplexes data and inter-UE coordination information, it needs to be discussed how destination ID and sidelink transmission priority are determined.

### Inter-UE Coordination Feedback Transmission in Scheme 2

In case of scheme 2, RX UE provides inter-UE coordination feedback to specific TX UE. Depending on amount of detected conflicts, it may need to deliver feedback to multiple TX UEs. RX UE may have limitations in terms of max number of feedbacks that can be transmitted in each slot and thus need to determine subset of feedbacks. Priority rules need to be discussed by RAN WG for inter-UE coordination feedback transmission. Given that RX UE is aware about priorities of sidelink transmissions signaled in SCI by TX UEs as well as about type of detected sidelink conflict, these parameters can be used to define priority rules for inter-UE coordination feedback transmission. The priority rules for inter-UE coordination feedback should also consider HARQ feedback from coordinating UE and necessity of UL transmission, if any.

  - For inter-UE coordination scheme 2, define priority rules for transmission of inter-UE coordination feedbacks considering sidelink transmission priority signaled by TX UEs

    - Further discuss behavior in case of tie break (i.e. in case of conflict among sidelink transmission with equal priority)

## Inter-UE Coordination Feedback & TX UE Behaviour

When UE-B receives the inter-UE coordination information from UE-A, its transmission behavior needs to be determined.

### Scheme 1: TX UE Behaviour for Higher Layer Inter-UE Coordination Feedback

The following options were identified in terms of UE-B’s behavior at the last RAN WG1 meeting.

- For scheme 1: - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

In our view, Option 1-1 is more general. If TX UE has received inter-UE coordination feedback using higher layer signaling it may use it for enhanced resource (re)-selection/exclusion in case of dynamic and semi-persistent processes on top of Rel.16 sensing and resource selection procedures.

For instance, TX UE may exclude resources provided in inter-UE coordination feedback or prioritize selection of resources from the inter-section of candidate resource sets generated by TX UE and provided by coordinating UE.

  - Option 1-1 is supported for inter-UE coordination scheme 1:

    - UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

### Scheme 2: TX UE Behaviour for Physical Layer Inter-UE Coordination Feedback

- For scheme 2: - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

Depending on sidelink conflict addressed by inter-UE coordination feedback the TX UE behavior can be different. In this section, we assume that all types of physical layer feedback are supported: i.e. feedback indicating half-duplex in transmission/reservation/reception and co-channel collisions in transmission / reservations.

Conflicts in transmission

If feedback is provided for the conflict in transmission the only possible TX UE behavior is to adjust retransmission behavior, i.e. increase number of retransmissions planned for a given TB.

Conflicts in reservation

If feedback is provided for the conflict in reservation, the potential TX UE behaviors are:

- Perform resource reselection

- Exclude resource/slot under conflict and perform resource re-selection/re-evaluation

- Continue transmission on reserved resource

- Skip transmission on reserved resource under conflict and use other reserved resource

In addition, UE can also adjust retransmission behavior/strategy (e.g. increase number of intended retransmissions or increment max number of retransmissions per priority level).

Conflicts in reception

If conflict in reception occurs and is conveyed by inter-UE coordination feedback, then it seems the most reasonable TX UE behavior is to skip transmission, since it is not expected to be processed by RX UE anyway. Besides dropping of transmission TX UE may perform resource reselection or increase number of intended retransmissions.

Based on discussion, we have following proposal:

  - For scheme 2, support the following options for UE-B’s behaviour

    - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

    - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

  - Further discuss and define details of TX UE behavior depending on type of physical layer inter-UE coordination feedback:

    - Half-duplex and/or co-channel collision in transmission

      - Increase amount of intended (re)-transmission or increment max number of retransmissions

    - Half-duplex and/or co-channel collision in reservation

      - Perform resource (re)-selection

      - Exclude resource and perform resource reselection

      - Continue transmission on reserved resource

      - Skip transmission on reserved resource with detected conflict and use other reserved resource

    - Half-duplex in reception

      - Skip transmission on reserved resource with detected conflict

In all above cases, UE can also adjust its sidelink retransmission behavior.

## NACK Signalling as Scheme 2 Inter-UE Coordination Feedback

For the case of detected sidelink conflict in transmission, the NACK signaling can be used to trigger retransmission by TX UE. The benefit of this solution is that it is transparent and consistent with Rel.16 UE behavior. Therefore, should be considered as a primary candidate to address half-duplex or co-channel collision issues in transmission.

  - Adopt sidelink NACK signaling as inter-UE coordination feedback to address sidelink conflicts in transmission for groupcast communication with NACK only transmission

# Evaluation of Inter-UE Coordination Solutions

## Performance Evaluation of Inter-UE Coordination Scheme 1

In this section, we provide analysis of the inter-UE coordination scheme 1 for broadcast communication (i.e. w/o HARQ feedback) for periodic traffic. The following scenarios are analyzed:

- Reference 1: Rel.16 design w/ blind retransmissions

- Reference 2: Rel.16 design w/ inter-UE coordination feedback for resource selection

  - In this case assisting UEs share inter-UE coordination information together with data transmissions. UEs performing resource selection form TX candidate resource set and take into account candidate resource sets from assisting UEs in [300-400]m range from TX. Up to 8 assisting UEs are used (4 in front and 4 behind). The intersection among candidate resource sets is prioritized for resource selection with a fallback to TX candidate resource set if the size of intersection set is not sufficient for TB transmission.

The results of evaluation are shown in Figure 1:

Figure 1: Performance analysis of inter-UE coordination scheme 1

Based on analysis of the presented results we have the following observations:

Observations:

  - Inter-UE coordination feedback scheme 1 can provide performance improvement of the Rel.16 design and increase communication range

  - The extension of communication range comes at the expense of minor degradation at short communication range, that can be explained by the inter-section mechanism (used for resource selection) which effectively decreases the size of candidate resource set for resource selection/randomization

## Performance Evaluation of Inter-UE Coordination Scheme 2

In this section, we provide analysis of the inter-UE coordination scheme 2 for groupcast communication with NACK only feedback and target communication range of 200m:

- Reference 1: Rel.16 design w/o HARQ feedback (blind retransmissions)

- Reference 2: Rel.16 design with HARQ NACK feedback

  - Note: Rel.16 design, if it is not corrected, is sensitive to half-duplex [4]-[6]

- Inter-UE coordination feedback for half-duplex in transmission HD-TX (feedback by RX UEs in range 200m)

- Inter-UE coordination feedback for co-channel collision in reservation CC-RSV (feedback by RX UEs in range 200m from TX UE; collision is detected if TX UEs are located within 400 m from each other)

- Inter-UE coordination feedback for half-duplex in transmission HD-TX (feedback by RX UEs in range 200m) & co-channel collision in reservation CC-RSV (feedback by RX UEs in range 200m from TX UE; collision is detected if TX UEs are located within 400 m from each other)

Figure 2: Analysis of inter-UE coordination feedback for NACK only groupcast 
and comparison with the Rel.16 design

Based on provided results we have the following observations:

Observations:

  - Rel.16 design for groupcast communication with HARQ NACK only feedback is sensitive to half-duplex problem

  - Rel.16 design for groupcast communication with HARQ NACK only feedback can be substantially improved by inter-UE coordination feedback indicating conflicts of half-duplex in transmission and outperforms Rel.16 design with blind retransmissions

  - In considered evaluation scenario, inter-UE coordination feedback indicating both sidelink conflicts: co-channel collisions in reservations and half-duplex in transmission does not provide significant incremental gain over inter-UE coordination feedback indicating half-duplex in transmissions only

# Conclusions

In this contribution, on enhancements of Mode-2 sidelink communication by means of inter-UE coordination feedback. In summary, we have following proposals:

  - RAN WG1 to discuss which of the following sidelink conflicts should be addressed by inter-UE coordination signaling

    - Half-duplex in transmission (HD-TX)

    - Half-duplex in reservation (HD-RSV)

    - Half-duplex in reception (HD-RX)

    - Co-channel collision in transmission (CC-TX)

    - Co-channel collision in reservation (CC-RSV)

  - Inter-UE coordination signaling to at least address sidelink conflicts of half-duplex in transmission

  - For inter-UE coordination scheme 1, support higher layer signaling to optimize semi-persistent resource allocation

    - PSSCH is considered as a baseline for higher layer inter-UE coordination feedback design

  - For inter-UE coordination scheme 2, support physical layer signaling to enable low-latency inter-UE coordination feedback with small payload

    - PSFCH is considered as a baseline for physical layer inter-UE coordination feedback design

  - Further study whether higher layer inter-UE coordination signaling for scheme 1 should provide information on

    - Alt.1: Candidate resources for resource (re)-selection procedure

    - Alt.2: Candidate resources for resource exclusion procedure

  - For inter-UE coordination scheme 1,

    - MAC CE is used to carry inter-UE coordination feedback

    - Reuse Rel.16 resource (re)-selection procedure (or resource exclusion procedure) for generation of higher layer inter-UE coordination feedback associated with a given timestamp (i.e. slot)

  - Physical layer inter-UE coordination signaling for scheme 2, supports indication of sidelink conflict type including

    - differentiation of sidelink conflict in transmission from conflict in reservation at TX UE side based on received feedback

    - differentiation of sidelink half-duplex conflict from co-channel collision conflicts at TX UE side based on received feedback

  - The sidelink conflict types indicated by inter-UE coordination feedback can be pre-configured

  - NR supports dynamic SCI signaling to request inter-UE coordination feedback

  - For inter-UE coordination scheme 1,

    - Feedback is generated based on semi-persistent transmissions

  - TX UE can apply feedback for both dynamic and semi-persistent transmissions and arbitrary cast type

  - For inter-UE coordination scheme 1, any UE can be a UE-A providing inter-UE coordination feedback subject to following conditions

      - Semi-persistent transmissions are enabled (allowed) per sidelink resource pool

      - Inter-UE coordination feedback for collision avoidance is enabled per sidelink resource pool

      - UE supports inter-UE coordination scheme 1

      - UE intends to transmit data (a TB including inter-UE coordination payload)

      - Amount of time / number of sidelink slots passed from the previous feedback by given UE exceeds pre-configured threshold

  - For inter-UE coordination scheme 2,

    - Feedback is generated based on semi-persistent and dynamic transmissions

      - TX UE can apply feedback for both dynamic and semi-persistent transmissions and arbitrary cast type

    - Any UE can be a UE-A providing inter-UE coordination feedback subject to following conditions

      - Inter-UE coordination feedback for conflict resolution is enabled per sidelink resource pool

      - Inter-UE coordination feedback is requested by TX UE for a given transmission

      - UE supports inter-UE coordination scheme 2 and detects sidelink conflict

      - UE is intended receiver or non-intended receiver which is within pre-configured SL-RSRP or distance range from TX UE, if pre-configuration is provided

      - Amount of time / number of sidelink slots, passed from the previous feedback by given UE, exceeds pre-configured threshold

  - If higher layer inter-UE coordination feedback over PSSCH is agreed, the Mode-2 resource allocation framework (procedures), including potential Rel.17 enhancements, should be used for inter-UE coordination feedback

  - In case of physical layer inter-UE coordination feedback using PSFCH resources, the PSFCH resource (time/frequency/code) selected for feedback should be associated with Source ID, sidelink feedback type (conflict type)

  - For inter-UE coordination scheme 2, define priority rules for transmission of inter-UE coordination feedbacks considering sidelink transmission priority signaled by TX UEs

    - Further discuss behavior in case of tie break (i.e. in case of conflict among sidelink transmission with equal priority)

  - Option 1-1 is supported for inter-UE coordination scheme 1:

    - UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

  - For scheme 2, support the following options for UE-B’s behaviour

    - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

    - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

  - Further discuss and define details of TX UE behavior depending on type of physical layer inter-UE coordination feedback:

    - Half-duplex and/or co-channel collision in transmission

      - Increase amount of intended (re)-transmission or increment max number of retransmissions

    - Half-duplex and/or co-channel collision in reservation

      - Perform resource (re)-selection

      - Exclude resource and perform resource reselection

      - Continue transmission on reserved resource

      - Skip transmission on reserved resource with detected conflict and use other reserved resource

    - Half-duplex in reception

      - Skip transmission on reserved resource with detected conflict

  - Adopt sidelink NACK signaling as inter-UE coordination feedback to address sidelink conflicts in transmission for groupcast communication with NACK only transmission

References

- RP-202846, “WID revision: NR sidelink enhancement”, LGE

- R1-2104926, “Sidelink power saving solutions”, Intel Corporation, Mai 2021.

- R1-2005897, “On feasibility and benefits of sidelink enhancements targeting Mode 2 reliability and latency”, Intel Corporation, August 2020

- R1-2100673, “On Feasibility and Benefits of Inter-UE Coordination for Sidelink Mode-2 Design”, Intel Corporation, January 2021

- RP-210469, “Discussion on WID Update for Rel-17 Sidelink Enhancements”, Intel Corporation, March 2021

- R1-2101910, “Reliability and Latency Enhancements for Mode 2”, Qualcomm Incorporated, January 2021

Annex – A: Evaluation Assumptions

In this section in Table 1 and Table 2 we provide evaluation assumptions used for Inter-UE coordination Scheme 1 and Scheme 2 studies. In all studies resource selection window of 32 slots length is used for data resource selection.

Table 1: System level evaluation assumptions for Scheme 1 (Higher Layer Signalling for Sidelink Collision Avoidance) studies

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario from NR V2X methodology - Vehicle speed = 70 km/h |
| Channel model | TR 37.885, NR V2X Channel Model |
| Spectrum allocation | Carrier frequency: 6GHz Simulated Bandwidth: 40 MHz |
| Subcarrier spacing | 30 kHz |
| Tx/Rx Antenna Ports | 1Tx/2Rx |
| Communication Mode(s) | Broadcast |
| Traffic model | Periodic variable packet size traffic: - Packet size: [800, 1200] Byte with probabilities [0.8, 0.2] accordingly - Inter-packet arrival time: 50 ms - Latency requirement: 50 ms |
| TTI structure | NR Slot TTI: - Slot w/o PSFCH zone: 1 AGC symbol, 1 GAP symbol, 2 PSSCH symbols w/ pilots - Slot w/ PSFCH zone: 1 AGC symbol, 2 GAP symbol, 2 PSSCH symbols w/ pilots, 2 symbols for PSFCH |
| PSFCH Zone Allocation | Enabled, in every 2nd slot |
| Sidelink control TX parameters | 64 Bits QPSK Modulation |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB - PSSCH: 35 PRB |
| SCI/Data time resource allocation | - PSCCH: 3 Symbols - PSFCH: 2 Symbols - PSSCH: Remaining symbols in slot |
| Data Packet Tx parameters | Periodic variable packet size evaluations: - 800 B packet: 16-QAM, 2 TTI - 1200 B packet: 64-QAM, 2 TTI |
| HARQ Feedback | Disabled |

Table 2: System level evaluation assumptions for Scheme 2 (Physical Layer Signalling for Sidelink Conflict Resolution) studies

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario from NR V2X methodology - Vehicle speed = 70 km/h |
| Channel model | TR 37.885, NR V2X Channel Model |
| Spectrum allocation | Carrier frequency: 6GHz Simulated Bandwidth: 40 MHz |
| Subcarrier spacing | 30 kHz |
| Tx/Rx Antenna Ports | 1Tx/2Rx |
| Communication Mode(s) | Groupcast, connection-less |
| UE-to-UE Association | - UE is considered as member of the group based on location principle - Each UE may be a member of a multiple groups |
| Traffic model | Aperiodic variable packet size traffic (TR 37.885 Aperiodic Model 1 traffic): - Packet size: uniform in the range [200..2000] B with quantization step of 200 B - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Latency requirement: 50 ms |
| TTI structure | NR Slot TTI: - Slot w/o PSFCH zone: 1 AGC symbol, 1 GAP symbol, 2 PSSCH symbols w/ pilots - Slot w/ PSFCH zone: 1 AGC symbol, 2 GAP symbol, 2 PSSCH symbols w/ pilots, 2 symbols for PSFCH |
| PSFCH Zone Allocation | Enabled, in every 2nd slot |
| Sidelink control TX parameters | 64 Bits QPSK Modulation |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB - PSSCH: 35 PRB |
| SCI/Data time resource allocation | - PSCCH: 3 Symbols - PSFCH: 2 Symbols - PSSCH: Remaining symbols in slot |
| Data Packet Tx parameters | Aperiodic variable packet size evaluations: - 200 B packet: QPSK, 4 TTI Max - 400 B packet: 16-QAM, 4 TTI Max - 600 B packet: 16-QAM, 4 TTI Max - 800 B packet: 16-QAM, 4 TTI Max - 1000 B packet: 16-QAM, 4 TTI Max - 1200 B packet: 16-QAM, 4 TTI Max - 1400 B packet: 64-QAM, 4 TTI Max - 1600 B packet: 64-QAM, 4 TTI Max - 1800 B packet: 64-QAM, 4 TTI Max - 2000 B packet: 64-QAM, 4 TTI Max |
| HARQ Feedback | Enabled, NACK-only |
| HARQ Feedback reporting distance | UEs within 200 m range report feedback |
| HD-TX Inter-UE coordination feedback reporting distance | UEs within 200 m range report feedback |
| CC-RSV Inter-UE coordination feedback reporting distance | UEs within 200 m range report feedback |
| CC-RSV Maximum collision Distance | Resource collision between TX UEs within 400 m range is reported |