3GPP TSG RAN WG1#105e       	        	                                   R1-2105151     E-meeting May.10th-27th, 2021

Agenda Item:					    8.1.1

Source:	Sony

Title:	Further enhancement on multi-beam operation

Document for:	Discussion

# Introduction

In RANP#86 meeting, the WID on multi-beam enhancement of Rel.17 was established as below

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management to support higher intra- and L1/L2-centric inter-cell mobility and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

In this contribution, we present our considerations and thoughts on how to step forward.

# Unified TCI state framework

In Rel.15/16, the DL beam indication depends on the signaling of TCI states. It is configured in PDSCH-Config per BWP and can be viewed as a resource pool of DL Tx beams. The essential function of TCI state is to conduct QCL relation between two DL RSs, e.g. QCL-TypeD between TRS and PDSCH DMRS.

As for UL, the beam indication is built on spatial relations which are separately configured and/or activated for UL channels or signals in a per PUCCH/SRS resource level. The spatial relation may contain an SRS resource or a DL RS as spatial source for determining UL Tx spatial filter, whereas a TCI state could only contain a DL RS (either SSB or CSI-RS) as a QCL source.

Next, we begin our discussion with source RS for conveying QCL-TypeD in the framework of unified TCI state.

## Source RS for unified TCI state

DL QCL

In RAN1#104e, the following agreement was achieved which implies that the basic QCL rule in Rel.16 can be reused for unified TCI state.

Agreement On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D. - Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported: - CSI-RS for beam management - CSI-RS for tracking - FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types

However, in Rel.15/16, only DL RS can be referred for DL reception. With indicated DL RS, UE determines the corresponding DL Rx beam. In other words, UE has flexibility in implementing its Rx beam. But if SRS for BM is indicated as a source RS for determining its Rx beam, the UE should revert the Tx beam of the indicated SRS resource as Rx beam. Therefore, the flexibility in implementing UE’s Rx beam would be impaired. Not to mention that once the UE experiences unpredicted rotation, mobility change, etc, the spatial Tx filter of indicated SRS resource may not be suitable for DL reception anymore. When applying DL RS as source RS, this can be less than an issue, since UE could track the DL RS and refine its Rx beam accordingly.

- : For determining DL Rx beam, reverting the Tx beam of indicated SRS for BM as the DL Rx beam seems restrictive for UE implementation and may cause DL beam pair misalignment.

In Rel.15/16, when UE enters the state of RRC connected, CSI-RS for CSI can serve as source RS for QCL-TypeA and TypeD for either PDCCH DMRS or PDSCH DMRS. In RAN1#104, RAN1 achieved consensus on reusing the existing QCL rule in Rel.16 as in following agreement. But it seems that the interpretation on Rel.16 QCL rule is not completely correct, by missing CSI-RS for CSI which was left to FFS.

Agreement On Rel.17 unified TCI framework, the supported source/target QCL relations in the current TS38.214 V16.4.0 is supported for QCL Type D. - Note: This implies that the following source RS types for DL QCL (Type D, for DL RX spatial filter reference) information for DL UE-dedicated reception on PDSCH and all/subset of CORESETs are supported: - CSI-RS for beam management - CSI-RS for tracking - FFS (to be decided by RAN1#104bis-e): If SSB, CSI-RS for CSI, and/or SRS for BM are also supported as source RS types

In the framework of unified TCI state in Rel.17, we believe that CSI-RS for CSI can serve as QCL source RS without any issue and it’s time for RAN1 to fix it, at least for above agreement.

- : For backward compatibility with Rel.15/16 beam management, support CSI-RS for CSI as QCL source RS in unified TCI state.

## TCI state pooling

With the unified TCI states, what remains questionable is whether DL and UL share a common TCI state pool, e.g. tci-StatesToAddModList configured in PDSCH-Config, or whether NW configures a similar unified TCI state pool in PUSCH-Config for UL separately. In most recent meetings (from RAN1#102e to RAN1#104e), agreements were made as below

b) In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives for accommodating the case of separate beam indication for UL and DL - Alt1. Utilize the joint TCI to include references for both DL and UL beams - Alt2. Utilize two separate TCI states, one for DL and one for UL. The TCI state for the DL is the same as agreed in 1a. The TCI state for the UL can be newly introduced. - Alt 2-1: The UL TCI state is taken from the same pool of TCI states as the DL TCI state - Alt 2-2: The UL TCI state is taken from another pool of TCI states than the DL TCI state - Note: The resulting beam indication directly refers to the associated source RS(s) - FFS (RAN1#103-e): Details on extension to intra- and inter-band CA - Note: This may be related to issue 5 as well as other reasons for different TCIs such as network flexibility/scheduling Agreement On Rel-17 unified TCI framework: - A pool of joint DL/UL TCI state is used for joint DL/UL TCI state update (beam indication). - FFS: The pool for separate DL and UL TCI state update (beam indication) - Note: Here, TCI state pool refers to a pool configured via higher-layer (RRC) signaling FFS: Whether joint TCI may include UL specific parameter(s) such as UL PC/timing parameters, PL RS, panel-related indication, etc. and if it is included, it is used only for UL transmission of the DL and UL transmissions to which the joint TCI is applied

Our thought is to maintain flexibility of DL and UL beam indication as much as possible. We illustrate the methodology of TCI state pooling in Figure 1. Two separate TCI state pools can be configured via RRC signaling, meaning one for DL and the other one for UL. Each configured TCI state pool can be activated/deactivated by MAC CE and then indicated by DCI. In addition, we noticed that in separate DL/UL beam indication, only DL RS can be applied for DL beam indication, meaning SRS for UE to determine DL Rx beam is not supported yet. But both DL RS (SSB and CSI-RS) and UL RS (SRS) can be configured into UL TCI state. From this sense, it is reasonable to separately configured TCI state pools for DL and UL.

- : Unified TCI state framework for both DL and UL

- : To achieve the flexibility of DL and UL beam updating/indication, support separately configured unified TCI state pools per BWP.

For UE supporting joint DL/UL beam indication, it could be dynamically indicated with joint TCI state. Assuming MPE event happens unpredictably with indicated joint TCI and only UE is able to detect such RF exposure, UE should be able to apply UL Tx beam with separated UL TCI as soon as possible.

Agreement On Rel.17 unified TCI framework, by RAN1#104bis-e, down select or modify at least one from the following alternatives: - Alt1. A UE can be dynamically indicated with either joint DL/UL TCI or separate DL/UL TCI - Details on dynamic indication are FFS - FFS: UE capability for the support of joint DL/UL TCI and/or separate DL/UL TCI - Alt2A. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via RRC signaling - Alt2B. A UE can be configured with either joint DL/UL TCI, separate DL/UL TCI, or both via RRC signaling - Alt3. A UE can be configured with either joint DL/UL TCI or separate DL/UL TCI via MAC CE signaling - Details on how this is signaled in relation to TCI activation are FFS

- : Support (Alt.1) UE can be dynamically indicated with joint TCI or separate TCI.

## Common beam operation for CA

Intra-band CA

In RAN1#103e, the common beam operation for intra-band CA was established as in following agreement.

Agreement On Rel-17 unified TCI framework, support common TCI state ID update and activation to provide common QCL information and/or common UL TX spatial filter(s) across a set of configured CCs: - The above applies to intra-band CA - The above applies to joint DL/UL and separate DL/UL beam indications - Just as Rel.16, the RS in the TCI state that provides QCL-TypeA [or QCL-TypeB] shall be in the same CC as the target channel or RS - The common TCI state ID implies that the same/single RS determined according to the TCI state(s) indicated by a common TCI state ID is used to provide QCL Type-D indication and to determine UL TX spatial filter across the set of configured CCs - FFS: The above also applies to inter-band CA - FFS: TCI state pool for CA - Opt-1: sharing a single RRC TCI state pool for the set of configured CCs, e.g., cell-group TCI state pool, or reuse TCI state pool for PDSCH in a reference cell; A CC ID for QCL-Type A RS is absent in a TCI state, and the CC ID for QCL-Type A RS is determined according to a target CC of the TCI state. - FFS: Whether it is possible that a single TCI state in the pool includes all source RSs from different CCs - Opt-2: configuring RRC TCI state pool per individual CC - FFS: Whether the Rel-17 common beam update across multiple CCs applies to beam indication for single channel (e.g. PDSCH only, single CORESET), a subset of channels, or all channels

For TCI states pooling for CA, there are two options which can be simply put as 1) single shared TCI state pool among CCs; 2) per individual CC TCI state pooling. In Rel.15/16, the DL beam indication mechanism across multiple CCs were designed by using one common TCI state ID which is analogous to Option 2 in our understanding. In Rel.17, it’s a chance to further reduce DL overhead associated with RRC configuration. From TCI state pooling aspects, we have

- : For common DL/UL beam operation, support (Opt-1) to configure a single TCI state pool across multiple CCs.

If Proposal 4 is supported, only a single TCI state pool is configured for multiple CCs. One of RRC parameter implementation can be that this TCI state pool is configured for PCell and shared for some SCell(s). Recall the fact that QCL-TypeA source RS conveying fine time/frequency QCL information should come from the same CC as target RS. But the QCL-TypeD source RS conducting spatial Rx QCL information could be located on a CC different from the CC of target RS. When configuring CC ID and BWP ID of Option 1, we would like to have following

- : When configuring a single TCI state pool across multiple CCs (Opt-1), the CC ID and BWP ID for QCL-TypeA RS should be absent in a TCI state. But such restraint should not be mandatorily applied to QCL-TypeD RS.

Inter-band CA

For the case of multiple CCs in different bands, i.e., inter-band CCs, there are, as well, opportunities to reduce the burden of signaling overhead. With no optimizations, multiple beam management procedures may be required in the inter-band case to cater for multiple CCs, BWPs and UE panels, as illustrated for the intra-band case. This has several adverse effects:

- Time-frequency resources need to be reserved for initial beam establishment and beam refinement on multiple CCs.

- The burden of signaling overhead, e.g., to configure suitable TCI states, increases with the number of CCs and UE panels.

- The UE burden and energy consumption associated with additional beam management procedures scales up with the number of CCs, UE panels and BWPs.

It is possible, however, to reuse beam management procedures across bands. For instance, when TRPs of different bands (e.g., 28 GHz and 39 GHz bands) are co-located, signals between NW and UE tend to propagate through the same physical directions. In this case, a beam sweep procedure common to multiple CCs may be feasible, including the reuse of TCI states. To harbor opportunities for optimizations and also considering above agreement, we make the following proposal

- : For inter-band CAs, support common beam operation for data/control, DL/UL, multiple BWPs, multiple TRPs and multiple UE panels.

The feasibility of the above proposal depends, of course, on the UE ability to simultaneously produce beams in different bands which roughly point in the same directions. It is apparent that the ability to do so will depend on the UE implementation. For example, some UEs might not be able to simultaneously control directions of beams in different bands, or they might be able to do so for certain band combinations, but not for others. Therefore, the achievable savings depend on certain UE capabilities. RAN4 has been discussing a potential new UE capability on independent beam management (IBM) and/or common beam management (CBM) for inter band CA operation. Note that if such capability would be eventually agreed, Proposal 6 supporting a common beam management procedure states across bands would then become necessary.

- : The UE capability related to simultaneously steering beams in the same direction belonging to CCs in different bands should be reported.

Furthermore, RAN4 is discussing possible test setups for inter-band CCs wherein the beams of different CCs may be configured to use different polarization directions. In addition, some UEs may only be able to have independent beam control on different CCs with different polarizations due to limitations from the RF implementation. Therefore, when optimizing beam indication across bands, polarization needs to be taken into account. In particular, the polarization property of beams shall not impair optimizations of cross CC beam indication while, at the same time, it should be possible to select orthogonal polarizations for beams in different bands pointing in the same/different directions.

- : For the optimization of beam indication across bands, the polarization property of beams should be considered.

## UL power control associated with unified TCI

In Rel.15/16, different power control mechanisms were designed for different UL channel/signal. Specifically, the whole set of RRC configured power control parameters (i.e. alpha/P0/PL RS/closed-loop index) of PUSCH highly depends on SRI in UL grant DCI. Only the mapping between SRI and PL RS can be updated by MAC CE. For PUCCH, each PUCCH resource can be activated by MAC CE with single PUCCH-SpatialRelationInfo within which the above-mentioned PC parameters can be found. For SRS, those PC parameters are configured in each SRS resource set and only PL RS can be updated within an SRS resource set by MAC CE.

- : In Rel.15/16, some of UL PC parameters, i.e. alpha/P0/PL RS/closed-loop index are separately configured for UL channel/signal, but only PL RS can be updated by MAC CE.

For UL, UE transmission power by nature links with Tx-Rx beam pair established between NW and UE. In Rel.17, both joint and separate TCI state can be dynamically indicated to UE via DCI. It is reasonable to consider UL Tx beam and Tx power association. From PUSCH power control perspective, the indicated TCI state (if no SRI existed in DCI) should provide the same function as SRI.

In RAN1#104bis-e, the UL PC parameters other than PL RS was further narrowed down.

Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework: - The setting of (P0, alpha, closed loop index) is at least associated with UL channel or UL RS - Select or modify from one of the following alternatives by RAN1#104bis-e for PUCCH, PUSCH, and SRS separately: - Alt1. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state - Alt2. The setting of (P0, alpha, closed loop index) is included with UL or (if applicable) joint TCI state - Alt3. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state - Alt4. The setting of (P0, alpha, closed loop index) is determined as in Rel-16 without enhancement Agreement On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, for each of PUSCH, PUCCH, and SRS, in RAN1#105-e, further discuss to down-select or combine from the following alternatives: - AltA. The setting of (P0, alpha, closed loop index) is also associated with UL or (if applicable) joint TCI state - AltB. The setting of (P0, alpha, closed loop index) is also included with UL or (if applicable) joint TCI state - AltC. The setting of (P0, alpha, closed loop index) is neither associated with nor included in UL or (if applicable) joint TCI state Note: It has been agreed that the setting of (P0, alpha, closed loop index) is associated with UL channel or UL RS (therefore the setting is channel- and signal-specific).

Given the framework of unified TCI state in Rel.17, PC parameters, i.e. P0/alpha/closed-loop index, were agreed to be at least associated with UL channel or signal. The next down selection is whether such set of parameters can be further associated with UL or joint TCI state. In our understanding, there is no need to associate this setting of parameters on a per TCI state level. It implies following the conventional Rel.15/16 way would be good enough.

- : For Rel.17 UL power control, support (Alt.3/AltC) the setting of P0/alpha/closed-loop index is neither associated with nor included into UL or joint TCI state.

As for PL RS, the following agreement was brushed in RAN1#104bis-e and there will be a down selection in upcoming meeting.

Agreement On Rel.17 unified TCI framework, in RAN1#105-e, further discuss to down select or combine from the following three alternatives for PL-RS (note: the text below is based on the agreed description in RAN1#104-e): - AltA. PL-RS can be included in UL TCI state (or, if applicable, joint TCI state). - FFS: Whether it is always included or not. If not included, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state. - AltB. PL-RS can be associated with (but not included in) UL TCI state (or, if applicable, joint TCI state) - FFS: Exact association mechanism - FFS: Whether it is always associated or not. If not associated, PL-RS is the periodic DL-RS used as a source RS for determining spatial TX filter or the PL RS used for the UL RS in UL or (if applicable) joint TCI state - AltC. UE calculates path-loss based on periodic DL RS configured as the source RS for determining spatial TX filter in UL or (if applicable) joint TCI state - FFS: If a PL RS is not included in or associated with the UL TCI state (or, if applicable, joint TCI state), whether the UE can estimate path-loss based on the PL-RS of an UL RS provided in an UL TCI state (or, if applicable, joint TCI state) as a source RS for determining the spatial TX filter. In addition: - FFS (to be decided in RAN1#105-e) whether a fallback scheme is needed and, if so, the details - FFS: Support additional UE capability to report whether above PLRS determination mechanism is supported - Note: As agreed in RAN1#104-e, the total number of maintained PL-RSs per CC is no more than 4 - FFS: investigate the condition(s) agreed in Rel-17 and, if needed, study whether a UE can simultaneously maintain more than four path-loss estimates based on UE capability - FFS: UE capability for maximum number of active PL-RS across CCs per band

The reason why we prefer PL RS not included into TCI state lies in the fact that if the TCI state is for DL beam indication, it seems the UL PC parameters are not useful but may consume extra overhead during configuration. Moreover, the association relation between TCI state and PL RS can be adjusted or updated via MAC CE. It cannot be easily done, if PL RS is configured into TCI state.

- : For Rel.17 UL power control, support (AltB) that PL RS can be associated  with either UL TCI state or joint TCI state.

# L1/L2 inter-cell mobility

In Rel.15/16 NR, the intra-cell beam level mobility was designed and specified. In the WID of Rel.17, RAN plans to support the inter-cell mobility with as less latency as possible, i.e. by using the L1/L2 signaling. The motivation for introducing the L1/L2-centric inter-cell mobility is to address mobility issue associated with moderate- to high-speed UE. When a UE moves across the border of its SpCell and enters the coverage of a non-serving cell (NSC), the benefit comes from early beam-level activation/indication from NSC before any RRC signaling involved (if needed).

## RRC pre-configuration on NSC info.

After RAN1#104e, an LS related to RRC (re)configuration was prepared by RAN1 and will be sent to RAN2 in upcoming meeting. The reason is simply because that RAN1 itself may not have the full specialty to determine the RRC impact to completely support L1/L2 inter-cell mobility. Before we get any feedback from RAN2, our understanding on RRC pre-configuration can be illustrated in Figure 2.

- : RRC pre-configuration on UE with SC (#0) and NSC (#A and #B)

Specifically, to avoid RRCReconfiguration, one NW implementation would be that NW pre-configures UE with all necessary RRC parameters of target cells along UE’s predicted trajectory or UE’s nearby location. These configured target cells belong to UE’s NSC. Only when necessary, non-serving cell can turn into serving cell via handover or SCell addition where the latter has been supported in CA operation. This approach seems suitable for UEs with predictable trajectory, e.g. UEs in highway or high-speed train. But the downside comes from the requirement of larger storage of RRC pre-configurations at UE side. Note that these NSC can be differentially configured to UE when compared with its SC. More specifically, UE can be configured with one serving cell as configuration benchmark, while as for NSC configuration only different RRC parameters should be signaled for the sake of saving RRC overhead and UE’s storage.

Another method is to configure all the cells with the same RRC parameters. But from NW deployment perspective, it seems too restrictive to do so, not to mention that some key parameters should be configured differently from cell to cell, e.g. PCI, scrambling ID. So, we don’t think this approach is practical if not impossible and thus have

- : RRC reconfiguration during handover could be replaced by either RRC pre-configurations or same RRC configuration for both SC and NSC, where the latter seems too restrictive from the aspects of NW deployment.

Along with cell-level configuration, another issue to be addressed is a constrain introduced in Rel.15/16. In particular, a UE is only allowed to be configured with TCI states containing source RS(s) from its serving cells. But to enable beam indication/activation from NSC, one simple and necessary action is to support pre-configuration on TCI states which include source RS from NSC. Taking DL as example, it can be done by optionally allowing PCI (either from SC or NSC) included into TCI state. Consequently, when pre-configuration on TCI states (some contains source RS from NSC) are activated/indicated, beam indication/activation can be done without involving RRC reconfiguration.

Considering the progress of unified TCI so far, we have following

- : For L1/L2 inter-cell mobility, support to pre-configure unified TCI state with source RS from non-serving cell by incorporating PCI into TCI state.

## DL measurement for NSC RS

In RAN1#104e, the following agreement on DL measurement and reporting was achieved

Agreement On Rel.17 multi beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP: - Rel.15 L1-RSRP is used as reporting quantity for measurement and reporting of non-serving-cell(s) - Support SSB as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP, and Rel.15 SS-RSRP calculated from SSB of non-serving cell(s) - FFS: Whether the measurement for SS-RSRP is limited within SMTC - FFS: Detailed reporting method, e.g. via including existing L1-RSRP report, UE-initiated report etc. - FFS: Whether or not to support CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP. If the support of CSI-RS (for e.g. mobility and/or tracking) of non-serving cell(s) as a measurement RS for L1/L2-centric inter-cell mobility and inter-cell mTRP is confirmed, Rel.15 CSI-RSRP is also supported - Whether the support applies to CSI-RS with or without QCL source, or both - FFS: The number of non-serving cell(s) for measurement/reporting - FFS: time behavior of the reporting, i.e. periodic, semi-persistent, aperiodic, or UE-initiated - FFS: If other reporting quantities are supported, e.g. L3-RSRP, hybrid L1/L3-RSRP - FFS: Dynamic activation/deactivation/selection of the beam measurement on the RS(s) associated with non-serving cell(s) via MAC CE - FFS: Timing assumption (e.g. time of arrival and time of the measurement) for measurement of non-serving cell RS measurement

In addition to SSB from NSC, a UE normally would be configured with CSI-RS for mobility for RRM purpose. In comparison with SSB, CSI-RS for mobility can be allowed with larger bandwidth in frequency domain and more density in time domain for more accurate measurement results. In addition, to get time-domain filtered results in L3, UE first needs to produce measurement results of mobility RS in L1 which can be reused for L1/L2 inter-cell mobility. Furthermore, CSI-RS for mobility can be configured with an SSB from the same NSC as its QCL-TypeD source RS. In such case, UE can track back the QCL chain of this non-serving cell and retune its proper DL Rx beam accordingly. Therefore, we failed to see a reason to refuse CSI-RS for mobility as measurement RS from NSC

- : Support CSI-RS for mobility as measurement RS for L1/L2 inter-cell mobility.

## Beam reporting for NSC RS

In RAN1#104bis-e, the value K of beam reporting in one reporting instance has been discussed and finally agreed in following agreement.

Agreement On Rel.17 multi-beam measurement/reporting enhancements for L1/L2-centric inter-cell mobility and inter-cell mTRP, - On the value of K (defined in RAN1#104-e as the number of beam qualities associated at least with non-serving cell(s) can be reported in a single CSI reporting instance), - For the supported maximum value(s) of K, down-select at least one from the following candidates {4, 8, 16} - FFS: whether the maximum value of K is a UE capability - Periodic, semi-persistent, and aperiodic reporting (and the respective measurements) are supported. - Note: Semi-persistent and aperiodic reporting (and their respective measurements) are NW-initiated

From UE complexity point of view, how many RS from NSC can be measured and then reported should be a UE capability. Hence, we would have

- : Support optional UE capability on maximum value of K (beams associated at least with non-serving cell(s)).

In Rel.15/16, higher layer events are defined in TS 38.331 for L3 inter-cell mobility. When the triggering condition of those L3 events satisfied, UE reports to NW and then possibly triggers L3 handover procedure. Note that from time-domain perspective, such reporting is not periodic/semi-persistent/aperiodic, but UE triggered. From latency perspective, L3 filtered RSRP reporting normally takes more time than the L1-RSRP reporting due to the nature of time domain filtering. On the other hand, the stability of L3-RSRP outperforms that of L1-RSRP in making handover decision. But since RAN1 manages to set up L1/L2 inter-cell mobility, we believe L1 metric would be more suitable.

For L1/L2 inter-cell mobility, it seems straight-forward to define L1 events by using L1 metric, e.g. L1-RSRP or L1-SINR. For example, one L1 event could be defined as when the quality of NSC is better than current NS with an offset. Once such newly defined L1 event happens, UE triggers corresponding L1 reporting which contains RS from NSC and corresponding L1 metric.

- : Define L1-metric based events for L1/L2 inter-cell mobility and support UE triggered beam reporting based on L1 events.

## Beam activation/indication

For L1/L2 inter-cell mobility, when it’s necessary, NW sends beam-related signaling to UE. The signaling of beam activation in L2 MAC CE and/or beam indication in L1 DCI conveys unified TCI state(s) which includes QCL-TypeD source RS from NSC. In RAN1#104e, the following agreement was achieved to further discuss on the source RS from NSC.

Agreement On Rel.17 enhancements for L1/L2-centric inter-cell mobility, - Discuss whether to support at least the source RS types already agreed for intra-cell mobility for the purpose of referencing to non-serving cell(s). Note: This implies the following source RS(s): - CSI-RS for BM configured for non-serving cell(s) for DL QCL and UL TX spatial references - CSI-RS for tracking (TRS) configured for non-serving cell(s) for DL QCL and UL TX spatial references - SSB configured for non-serving cell(s) for UL TX spatial references - SRS for BM configured for non-serving cell(s) for UL TX spatial references - FFS: whether to support CSI-RS for mobility - FFS: whether to support other source RS(s) potentially agreed later for intra-cell mobility - FFS: whether to support CSI-RS for BM and tracking configured for non-serving cell(s) and without non-serving cell SSB as QCL-TypeD source

First in our understanding the Rel.15/16 QCL rule of intra-cell mobility can be extended to inter-cell mobility. Specifically, CSI-RS for BM and TRS from NSC can be applied to DMRS and other RS as direct QCL source. But as for SSB from NSC, it can be applied to CSI-RS as direct QCL source, and to DMRS as indirect QCL source.

- : At least support to extend Rel.15/16 QCL rule of intra-cell mobility to inter-cell mobility

- CSI-RS (from NSC) for BM and for tracking as direct QCL source for DMRS

- SSB (from NSC) as indirect QCL source for DMRS

In section 3.3, we discuss the possibility to include CSI-RS for mobility as measurement RS. If newly defined L1 event occurs, CSI-RS for mobility could be reported by UE in existing L1 reporting. Considering the benefits, we think it would be reasonable to support such feature in UE’s measurement and reporting. If so, it also seems nature to indicate CSI-RS for mobility as source RS in TCI state along with other CSI-RS, e.g. CSI-RS for BM or for tracking.

- : Support CSI-RS for mobility as source RS for conducting QCL info from non-serving cell.

# Dynamic TCI state update/indication

In RAN1#102e, Item a) of Issue 3 was identified as following

[Issue 3] For Rel.17 NR FeMIMO, on dynamic TCI state update signaling medium: a) In RAN1#103-e, investigate, for the purpose of down selection, the following alternatives: - Alt1. DCI - Alt2. MAC CE - Note: Combination between DCI and MAC CE for, e.g. different use cases or control information partitioning can also be considered - Note: The study should consider factors such as feasibility for pertinent use cases, performance (based on at least the agreed EVM), overhead (including PDCCH capacity), latency, flexibility, reliability including the support of retransmission - Note: This may be related to outcome of issue 1a), 1b), and 6a)

During RAN1#103e, the DCI based TCI state updating was discussed when considering the impacts on reliability, overhead, and latency, and finally supported. In this section, we would like to discuss pending issues on DCI based approach.

## DCI formats for signaling TCI state(s)

In RAN1#103e, RAN1 agreed to reuse DL DCI format 1_1 and 1_2 with DL assignment for beam indication and leave other possibility open for further discussion. In RAN1#104e, this DCI format down selection was formulated with more concrete details on its HARQ-ACK, beam application timing, etc.

In RAN1#104bis-e, DCI format 1_1 and 1_2 without DL assignment (Alt1) was supported and all related design details, including HARQ mechanism, RNTI, DCI field combinations, were almost done. However, there were a few points FFS, for example how to handle the case when only UL data is available.

Agreement On the Rel.17 DCI-based beam indication, in RAN1#104bis-e, down-select at least one of the following alternatives regarding the support of DCI format(s) for beam indication in addition to the agreed DCI formats 1_1/1_2 with DL assignment (in RAN1#103-e): - Alt0: No additional DCI format is supported - Alt1: DCI formats 1_1 and 1_2 without DL assignment, applicable for joint TCI as well as separate DL /UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS , based on DCI indicating SCell dormancy - FFS : How to identify DCI formats 1_1/1_2 used for beam indication only (not for scheduling a PDSCH reception, not indicating a SPS PDSCH release, or not indicating SCell dormancy), considering impacts on PDCCH coverage and scheduling mechanism - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt2: Dedicated DCI format other than 1_1/1_2 without DL assignment, applicable for joint TCI as well as separate DL/UL TCI - Support DCI acknowledgment mechanism, e.g. based on SPS PDSCH release, based on triggered SRS, based on DCI indicating SCell dormancy - FFS : If the format is based on an existing DCI format, how to identify the DCI format used for beam indication only - FFS : Whether the UE can/shall assume the gNB configured application time is after ACK transmission - Alt3: UL-related DCI formats 0_1/0_2 with UL grant, applicable only for UL-only TCI of separate DL /UL TCI

DCI 0_1/0_2 with UL grant

As for UL scheduling, the function of UL beam indication was designed to be dependent on SRS resource in Rel.15/16. Specifically, the SRI in DCI 0_1/0_2 indicates SRS resource(s) which was/were configured in SRS resource set with usage set as either ‘codebook’ or ‘non-codebook’. Besides UL Tx beam(s), SRI also couples with other UL aspects, e.g. UL power control, precoder and rank selection, which cannot be provided by TCI state in DL DCI. In addition, when UL scheduling comes, the other fields (rather than TCI state) in DL DCI are not identical with that of UL scheduling DCI. Hence, it seems not feasible to only apply DL DCI to convey TCI state(s) when separate UL beam indication is needed. Therefore, we have

- : The multiple roles of SRI in UL DCI are very important to UL scheduling, which could not be fully replaced by the TCI state.

If DL DCI is the only signaling medium to indicate UL Tx beam, then the UL transmission depends on two DCIs, one for UL beam indication and the other one for UL grant. In other words, at most two DCIs should be signaled to UE for one PUSCH transmission. Unless the DCI for UL beam indication lasts for a few of slots, it would be not efficient when compared with UL scheduling DCI 0_1/0_2, hence we still prefer

- : Support (Alt.3) UL DCI format 0_1/0_2 with UL grant to indicate UL Tx beam by joint DL/UL TCI state or UL-only TCI state.

GC-DCI

In RAN1#103e, the possibility of using group-common DCI to carry TCI state(s) for a group of UE has been discussed and it is now for further study. From signaling perspective, assuming K users in a group and instead of sending K UE-dedicated DCI to each of K UEs, NW only need to send one GC-DCI in the best case. Hence, a certain amount of DL overhead (i.e. (K-1)/K) can be saved, therefore alleviating congestion for DL control channel.

In our view, for some specific deployment scenarios, e.g. high-speed train or vehicles in highway in which a group of UEs are located in close proximity. Though due to different UE rotation or UE position the DL Rx beams can be different from UE to UE, the DL Tx beam(s) from NW seems common for the group of UEs. Therefore, as inspired by GC-PDCCH designed in Rel.15/16, it seems reasonable to apply GC-DCI to carry DL TCI state to the group of UEs.

Agreement On beam indication signaling medium to support joint or separate DL/UL beam indication in Rel.17 unified TCI framework: - Support L1-based beam indication using at least UE-specific (unicast) DCI to indicate joint or separate DL/UL beam indication from the active TCI states - The existing DCI formats 1_1 and 1_2 are reused for beam indication - Support a mechanism for UE to acknowledge successful decoding of beam indication - The ACK/NAK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI - FFS: Whether any additional specification support is needed - Support activation of one or more TCI states via MAC CE analogous to Rel.15/16: - At least for the single activated TCI state, the activated TCI state is applied - The content for the MAC CE is determined based on the outcome of issue 1 - FFS: If supported, default TCI state when more than one TCI states are activated by MAC CE - Note: There is no implications on the support of single TRP or multi-TRP - FFS: Additional enhancement such as L1-based beam indication with group-common DCI - FFS: Whether the Rel.17 beam indication can also apply to beam indication for single channel (e.g. PDSCH only, single CORESET) or a subset of channels - FFS: Additional details on extending the support of L1-based beam indication when separate UL (from DL) common beam indication is configured

Moreover, this GC-DCI conveying DL TCI states should be identifiable by UE. Different from repurposing DL DCI with assignment, there are not many different fields in GC-DCI, therefore special combinations of some fields would not work. But one way to do so is to scramble the GC-PDCCH with a new RNTI.

- : Study standard impacts of introducing group common DCI to indicate/update DL TCI state(s) for a group of UEs.

## HARQ mechanism for DCI

It does matter that both NW and UE synchronously maintain the same knowledge of Tx-Rx beam pair. As for DCI based beam indication/updating, NW should know whether UE decodes the DCI carrying TCI state(s) successfully or not. Consequently, the HARQ mechanism for this type of DCI should be specified.

DCI 0_1/0_2 with UL grant

For DCI format 0_1/0_2 with UL grant, there is no explicit HARQ procedure specified, since NW by nature is aware of whether PUSCH has been decoded successfully or not as recipient. NW only sends UL grant again with same HARQ process ID and un-toggled NDI to UE for retransmitting PUSCH, if NW failed in decoding previously transmitted PUSCH or UE failed in decoding UL grant DCI. It seems straight forward to reuse this UL implicit HARQ mechanism for DCI 0_1/0_2 with UL grant.

Let’s below take the examples in Figure 3. At the beginning, NW sends UL grant DCI to schedule PUSCH transmission. If UL DCI has been successfully decoded by a UE, the UE would transmit PUSCH accordingly. No matter PUSCH is decoded successfully or not at NW corresponding to case a) and b), NW can assume that the UE has decoded UL grant DCI. If no PUSCH transmitted from UE corresponding to case c), NW infers that UE did not decode the UL grant DCI successfully. Then it would be up to NW to re-schedule the UE with UL grant DCI carrying unified TCI state(s). In summary, even for UL grant DCI (no explicit HARQ feedback), the conventional implicit HARQ mechanism can be reused or slightly enhanced to support UL grant DCI carrying TCI state(s).

- : HARQ procedure for dynamic granted PUSCH

- : For UL grant DCI carrying TCI state(s), if supported, then reuse the implicit HARQ mechanism of PUSCH for UL grant DCI.

GC-DCI

For GC-DCI carrying TCI state(s) for a group of UEs, different from other GC-DCI, e.g. containing SFI or TPC, HARQ mechanism is needed anyway to maintain the alignment of beam pair between NW and UE.

- : For GC-DCI carrying unified TCI state(s) for a group of UEs, if supported, study its HARQ mechanism for each UE within the indicated group.

## Beam application time

Due to the newly introduced L1 signaling on TCI state(s) indication/updating, a UE needs certain time to decode DCI and then retunes its RF for DL Rx beam(s) and/or UL Tx beam(s). It is reasonable for UE to report its capability on the minimum time interval to get every related aspect ready. In RAN1#103e and RAN1#104e, following agreements on beam application time (BAT) were achieved

Agreement On Rel.17 DCI-based beam indication: - Regarding application time of the beam indication: if beam indication is received, down-select from the following: - Alt1: the first slot that is at least X ms or Y symbols after the DCI with the joint or separate DL/UL beam indication - Alt2: the first slot that is at least X ms or Y symbols after the acknowledgment of the joint or separate DL/UL beam indication - FFS: whether any existing timing defined for DCI based TCI/spatial relation update can be used for X/Y - FFS: When to apply the minimum indication delay (e.g., when the newly indicated beam is different with the previously indicated beam) Agreement On the beam application time for Rel.17 DCI-based beam indication, the beam application time can be configured by the gNB based on UE capability - Support a UE capability for the minimum value of beam application time - FFS: the exact minimum values of beam application time supported by UE - FFS: whether existing UE capability can be reused as this UE capability. - FFS: whether different beam application time values are supported for uplink and downlink - FFS: whether UE capability needs to be introduced for the maximum value of beam application time - FFS: the reference for defining the UE capability (e.g. from DCI reception or ACK transmission) - FFS: whether a UE is allowed to report more than 1 values in case of MPUE - FFS: the application time when DCI and applied channel(s) are on different CCs with same/different SCS(s)s

In our view, same as MAC CE based TCI state activation, the beam application time should be started from the first slot after UE has acknowledged the L1 signaling. Otherwise, if a UE fails in decoding DCI, the slot counting shouldn’t be even started from NW and UE side. Hence, we think Alt2 is more agreeable when compared with Alt1.

- : For beam application time, support the 1st slot that is X ms or Y symbols after the UE acknowledges DCI carrying joint or separate DL/UL beam indication (Alt2).

# Uplink multi-panel fast operation

Regarding the UL panel-specific beam selection, certain progress was made until RAN1#98 in Rel.16 and then one conclusion came that this feature cannot be supported in Rel.16 due to no consensus in RAN1. In Rel.17, we would like to continue to discuss, design and specify (if necessary) the feature of fast UL panel selection. In what follows, we would like to touch a fundamental issue for UL fast panel selection based on unified TCI state, i.e. association between panel and TCI state, then discuss how to update such association in a dynamic manner.

## Association with UE panels

Intuitively for designing a panel-specific UL transmission, one may create a panel ID for each panel entity. Afterwards UE reports its panel-related information to NW associated with such panel ID. Note that such ID may not directly reflect UE’s physical antenna panel implementation, it could map to proper terminology in specification. One good example in our view would be SRS resource set with usage as ‘Beam Management’. From Rel.15, UE report its capability on how many SRS resource set(s) it supports for simultaneous UL beam sweeping. Such capability can be viewed by NW as how many UL antenna panels deployed at this UE.

Hence, we would like to keep our proposal to get align with the highlighted part in following agreement made in RAN1#104e.

Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, - Rel.17 TCI state update (based on MAC CE + DCI along with the necessary TCI state activation, or MAC CE only) can be used for UE UL panel selection: - FFS : Whether specification support for this feature is necessary and if so the details of such spec support, e.g. - Additional spec support in TCI state definition to accommodate UL panel - UE reporting to facilitate UL panel selection - UE reporting, e.g. panel-specific report, including UE -panel state, e.g. inactive, active for DL /UL measurement, active for DL reception only, active for UL transmission, or other combination(s) of UE -panel states - Support for linking or association of UE panels with CSI-RS/SSB resources or resource sets, SRS resource sets, and/or PUCCH resource groups, etc.

- : Support to link or associate UL panels with SRS resource sets for UL fast panel selection.

However, as some UE vendors pointed out in a few of previous meetings, the specification should not be designed in a way that forces UE to expose its antenna implementation. It implies that such explicit signaling on panel ID would be not be acceptable. In addition, when NR system operates based on DL beam sweeping and beam correspondence, there could be no SRS for BM configured for UE. With this compromised understanding, RAN1 achieved following agreements on implicit method for panel-specific selection.

Agreement On Rel.17 enhancement for facilitating fast uplink panel selection, for discussion purpose, a panel entity corresponds to one or more RS resources: - For CSI/beam reporting, the RS resource is an RS associated with measurement and/or reporting - For beam indication, the RS resource is a source RS for UL TX spatial filter information - Note: For one RS resource, the corresponding panel entity may vary and is controlled by the UE, and whether/how to maintain a common understanding between gNB and UE can be further discussed/decided - Note: The above does not preclude possibility that an RS resource can be mapped to multiple panels - Note: The one or more RS resources may correspond to one or more RS resource set(s) depending on further discussion/decision - Note: Specification should not be designed in such a way that the UE is required to disclose its antenna implementation

The implicit logic chain of associating UE panel(s) can be depicted as follows. In beam measurement/reporting procedure, a UE measures DL RS resources with its antenna panel(s) and reports specific DL RS resource(s) which can be deemed as associated with specific UE panel(s). We have to note that those selected DL RS could be already configured in joint TCI states or UL-only TCI states. Then in beam indication, UE can be indicated with such TCI state(s) to prepare UL transmission with specific UL panel. In summary, the implicit approach of conducting panel-specific selection is to link or associate UE panel(s) with joint or UL-only TCI state via DL RS which is measured by specific UE panel(s) and then reported.

To this end, RAN1 may need to check current beam reporting format on whether panel status and/or panel ID (or let’s say a Tag) should be reported too.

- : Support to implicitly link or associate UE panel(s) with joint or UL-only TCI state(s) via beam reporting.

Furthermore, in RAN1#104bis-e, 3 options were listed to associate a panel entity with potential terminology in spec, such as reported CSI-RS and/or SSB resource, or a new panel ID within CSI/beam reports. From the discussion above, both Opt1-1 and Opt1-2 can provide such linking function.

Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, for CSI/beam measurement/ reporting, down select and/or modify from the following candidates: - Opt1-1: A panel entity corresponds to a reported CSI-RS and/or SSB resource index in a beam reporting instance - The correspondence between a panel entity and a reported CSI-RS and/or SSB resource index is informed to NW - FFS: How to inform through CSI/beam reporting framework - FFS: Detailed design of the correspondence including the conveyed information - Note: the correspondence between a CSI-RS and/or SSB resource index and a panel entity is determined by the UE (analogous to Rel-15/16) - Opt1-2: A panel entity is referring to a new panel ID within CSI/beam reports - FFS: Detailed design of the new panel ID including the information conveyed by the new panel ID - Note: The association between the new panel ID and the panel entity is determined by the UE - Opt1-3: No additional specification support - The duration in which the above panel entity reference is valid and the respective setting are FFS Note: “panel entity” is only used for discussion purpose

- : Support to link a panel entity with a new panel ID (Opt1-2, 1st priority) or with CSI-RS and/or SSB resource in beam reporting (Opt1-1, 2nd priority).

## UE panel status update

In RAN1#103e, the UE-initiated UL panel control was agreed as

Agreement In Rel.17 enhancement for facilitating fast uplink panel selection, UE-initiated UL panel selection/activation are supported: - FFS: Whether NW-initiated panel selection/activation is also supported - FFS: Whether specification support for this feature is necessary and if so the details of such spec support.

Considering power saving purpose or MPE issue, a UE may turn off or let’s say deactivate some of its antenna panels. If NW schedules UL transmission involving such inactive antenna panels at UE, the UE may consume more time to turn on the inactive antenna panel(s) than the scheduled PDCCH to PUSCH time gap or for aperiodically triggered SRS. Consequently, the scheduled UL transmission cannot be fulfilled in time by UE which is surely undesirable either from NW side or UE side. In other words, both NW and UE should be on the same page on the ‘ON/OFF’ status of UE panels. Specifically, when a UE would turn off/on some UL panel(s), it should let NW know in a synchronous manner.

- : For scheduled UL transmission, it matters that both NW and UE should be mutually aware of status of UE panel(s) in a synchronous manner.

- : UE panel status updating due to rotation

In Figure 4, one may observe that the UE equipped with multiple panels rotates by certain angle. From L1 metric wise, such rotation may result in association change from CSI-RS resource #3 linking “green” panel to CSI-RS resource #4 linking “red” panel. When detected necessary association modification, it would be better for UE to report it to NW either via UCI (similar as beam reporting) or MAC CE.

- : Support UE-initiated UL panel(s) linking/association status update via UCI or MAC CE.

A UE has been granted to charge its antenna panel(s) for UL transmission. Not only because UE antenna panels are UE implementation, but the reason also lies in the fact that only UE can observe its battery lifetime and then determine to turn off some panel(s) to increase standby time, or detect potential panel-specific extensive RF emission to human tissue and turn off specific panel(s) to avoid MPE. This kind of local information at UE cannot be easily known by NW in a timely manner.

- : Do not support NW-initiated panel selection/activation, unless the benefits for supporting this feature can be fully justified.

# MPE mitigation

In Rel.16, RAN1 received an LS from RAN4 to handle the MPE issue to comply with regulatory RF exposure limits. In RAN4, two approaches to handle the MPE issue have been agreed. One approach is to directly back off UE Tx power from the configured maximum transmit power by a quantity P-MPR. The other one is to control the ratio of UL transmission based on UE capability, i.e. maxUplinkDutyCycle.

Though aforementioned RAN4 solutions in Rel.16 have a touch on the MPE issue, they came with a price of UL coverage deterioration, especially at FR2. For instance, when power back-off applied by a UE, the UE may need to lower its maximum power by, i.e.10dB to satisfy RF exposure limits. This can possibly result in dissatisfactory measurements of an UL Tx beam at NW side, though the corresponding DL beam selected still brings high L1-RSRP at UE.

- : Due to UL Tx power back-off, the Rel.15/16 beam management established on beam correspondence may result in suboptimal UL Tx beam selection.

As a further observation, we notice that the MPE issue only affects UL. Therefore, it is possible to only switch the UL beam or panel to overcome the UL coverage limitation caused by the MPE restriction. Depending on whether beam correspondence holds or not at the UE side, the UE may be scheduled with an UL Tx beam which is not the corresponding DL Rx beam. From Rel.15/16 spec,  different UL Tx beam and DL Rx beam, as mentioned above, is surely not restricted, thanks to two parallel beam management mechanisms, i.e. TCI states for DL and SpatialRelationInfo for UL in Rel.15/16 and separate UL/DL TCI states in Rel.17. So we have following observation

- : Depending on whether beam correspondence holds or not at UE side, the UE can be scheduled with an UL Tx beam which is not the corresponding DL Rx beam due to UL MPE issue.

As early as in Rel.16 (RAN1#97), the MPE issue was brought up to RAN1. But due to lack of consensus in RAN1#98, the discussion on MPE was concluded unfortunately. In Rel.17, the WID formally includes the MPE issue and Issue 5 was identified to handle it in the way below

- [Issue 5] For Rel.17 NR FeMIMO, on MPE mitigation (that is, minimizing the UL coverage loss due to the UE having to meet the MPE regulation), in RAN1#103-e: - If needed, identify candidate solutions to be down-selected in future meeting(s). The following sub-categories can be used: - CAT0. The need for specification support for MPE event detection and, if needed, candidate solutions - CAT1. The need for UE reporting associated with an MPE and/or a potential/anticipated MPE event if the UE selects a certain UL spatial resource, e.g., corresponding to DL or UL RS - CAT2. The need for NW signaling in response to the reported MPE event (taking into account issue 1) and UE behavior after receiving the NW signalling - Note: RAN4 has agreed to specify P-MPR reporting (cf. CRs for TS 38.101/102/133) which can be used as a baseline scheme for further enhancement - Note: This may be related to outcome of issue 4b) - Companies are encouraged to submit evaluation results based on the agreed EVM to justify the benefits of the candidate solutions

## MPE-aware beam reporting

In RAN1#104bis-e, to facilitate MPE mitigation, all options were thoroughly discussed and listed to be down selected in RAN1#105e.

Agreement On Rel.17 enhancements to facilitate MPE mitigation, in RAN1#105-e, further discuss to down-select at least one or combine from the following options: - Opt 1A. {Rel.16 P-MPR based (beam/panel-level)} + Virtual PHR or a modified version - The modified version may be associated with each activated UL TCI or, if applicable, joint TCI, or associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) from candidate pool, if reported. - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - FFS: how to determine the virtual PHR or the modified version. - Opt 1D. {Rel.16 P-MPR based (beam/panel-level)} - The reporting reuses the event-driven mechanisms from the Rel-16 P-MPR reporting - Opt 2A. {SSBRI(s)/CRI(s) and/or panel indication} + L1-RSRP [L1-SINR] or a modified version that accounts for MPE effect associated with each of the reported SSBRI(s)/CRI(s) and/or panel indication (if configured) - FFS: How panel-level L1-RSRP [L1-SINR] is reported if L1-RSRP [L1-SINR] is associated with panel - FFS: Whether/how to account for MPE effect in L1-RSRP [L1-SINR] report, e.g. by using scaled L1-RSRP [L1-SINR] - FFS: Whether/how to enhance existing beam reporting format to support Option 2A - FFS: When multiple SSBRIs/CRIs and their corresponding metrics are reported in the same reporting instance, whether to allow mixture between the SSBRI(s)/CRI(s)) intended for MPE mitigation and for DL beam reporting - FFS: Whether the reporting is UE-initiated (event-driven) and/or NW-initiated - FFS: If Opt2A is selected and there is no consensus on a modified L1-RSRP definition, at least the Rel-15 L1-RSRP definition is reused and virtual PHR may be added FFS: If gNB acknowledges MPE report from UE for UE-initiated (event-driven) reporting FFS: If differential report is supported when multiple UL beams are reported in the same report

Regarding the progress for Issue 4, UE panel entity can be either associated with DL RS resource in CSI/beam reporting. Hence, we think the same rule could also apply to Issue 5. There is no need for UE to report panel, if the report contains SSBRI(s)/CRI(s) which are associated with UE panel already.

- : To facilitate MPE mitigation, support UE reports (a subset of Opt 2A) SSBRI(s)/CRI(s) + modified L1-RSRP that accounts for MPE effect.

For some other NW, SRS based UL beam sweeping is necessary tool at NW to determine UL Tx beam. When a UE carries out this UL beam sweeping procedure, the UE should keep the MPE issue in mind and therefore apply power back-off on a per Tx beam basis. More specifically, different SRS resources within the SRS resource set with usage set to ‘beam management’ may have different P-MPR values. SRS resources with their Tx beams experiencing the MPE issue should apply a power back-off value, e.g. 10dB, when compared to other SRS resources not experiencing MPE. Therefore, considering the MPE impact, NW may select the ‘best’ SRS resource for UL transmission.

- : For UL beam sweeping, when taking MPE into account, UE should transmit SRS resource(s) by certain power back-off, if needed.

## Beam/panel-specific P-MPR reporting

The UL transmit power is strictly limited by regulators’ limitations on MPE. Therefore, a power back off (P-MPR) can be used by UEs to meet the MPE limitation defined in the Rel-15 RAN4 specification. However, since the P-MPR value can be significant and unpredictable in real life, it will severely reduce the UL coverage and potentially result in radio link failures (RLFs). In Rel-16, enhancements of the MPE solution target avoiding RLFs and connection releases caused by FR2 UE RF exposure compliance. In RAN4#93, it was agreed that the P-MPR shall be indicated to the network through MAC-CE [2].

For Rel-17 multiple-beam operation with MPE impact, it is reasonable for RAN1 to consider supporting the existing RAN4 solutions. As we mentioned earlier, RAN4 has agreed in RAN4#93 to indicate the P-MPR value to the network, so that the network can estimate a reasonable UL duty cycle for the UE to avoid potential radio link failure and UL coverage loss. From RAN1 aspect, it can further associate the P-MPR value with spatial relation, e.g., reporting P-MPR of multiple candidate UL beams, so that the gNB can select the optimal beam pair for the uplink transmission.

For multi-panel operation, a straightforward way is to let the UE report a P-MPR value associated to a UE panel. Please note here that the MPE level is related to the physical antenna panels. Therefore, a panel ID likely needs to be related to a single physical antenna panel. If the P-MPR of each UE-panel is indicated to the network, the gNB can specify which panel to use and further optimize of the UL scheduling, resource allocation, etc.

- : Support UE reports P-MPR value(s) associated with panel(s) (Opt 1D) to mitigate UL coverage loss due to MPE.

## UL beam/panel switching to avoid MPE

Enhancements from RAN4 are limited to transmitted power control and uplink scheduling. However, even for the same antenna panel, the exposure level can be different between different beams, and even larger differences can be observed between different panels. An example from [3] is shown below in Figure 5. The free space power density of three different array designs averaged over 4 cm2 is plotted: For the same array, the power density level varies for each spatial filter (beam). For different array designs, the difference is even more pronounced. As a result, one can expect the actual max UL duty cycle, and P-MPR to be different for each beam in a real implementation. Therefore, we believe that beam/panel switching can be an effective solution and enhancement to mitigating the UL coverage loss due to MPE.

(a)

(b)

- : The view of simulation models of 28 GHz UE mock-ups of three different 4×1 arrays, and phase excitation scheme with a progressive phase shift βi (b) maximum 4 cm 2-averaged incident power density when d (distance to the antenna array)  and βi vary for 28 GHz 4×1 array.

- : Beam/panel switching can be an effective solution to mitigate the UL coverage loss due to MPE.

In addition to the efforts on enhancing MPE related beam reporting and P-MPR reporting, another way to avoid MPE is to switch from a beam which exceeds the maximum exposure limit. For instance, when a UE is served by two TRPs, if one UL Tx beam with one TRP shall back-off Tx power due to MPE, this UE could choose the other TRP for UL transmission. In such case it would be beneficial to study the UE triggered UL Tx beam change. Hence, we have

- : To avoid UL MPE issue, it would be beneficial for RAN1 to study and, if necessary, specify the mechanism of UE triggered UL beam/panel switch.

# Definition on UE antennas

## Antenna panel definition

During RAN1#104b_e meeting it was agreed to further study possible need for gNB awareness of UE panel configuration.

Because different UE vendors may implement UE antenna panels in different ways, the conventional panel definition, a physical antenna array with either single or dual polarization, doesn’t cover all aspects for panel-specific beam selection.

Agreement On Rel.17 enhancements for MPUE, investigate and, if needed, specify the following: - UE reporting of panel-specific information as a UE capability, for example: - Information related to the total number of DL/UL panel entities - Information related to the number of (max) antenna ports/layers per panel entity - Information related to the maximum number of resources per panel entity for SRS BM - Information related to panel selection delay - Information related to panel activation delay - UE reporting information related to minimal activation/selection delay for a panel based on L1 or L2 signaling - UE reporting of panel activation/selection status of a panel entity, e.g. active state for both DL and UL, or active state for DL only - FFS: details of this information (e.g. minimal activation/selection delay for a panel) and signaling (e.g. L1 or L2 signaling) - UE-reported information in MPE report (if supported) is used to indicate the minimal activation/selection delay and panel activation/selection status Note: above ‘panel entity’ is a logical entity and how to map physical panels to the logical entities is up to UE implementation. Note: This will depend on the final outcome of whether specification support for UE-initiated panel activation/selection is agreed.

The “if needed” reflect that there is no consensus among companies regarding the need for gNB awareness of UE panel configuration. Some companies expressed that it was not clear how a gNB could use this information. It is our understanding that current networks are fully operational, while in practice, being far from optimal from a UE beam management (BM) perspective. With progress in technology (e.g. number of layers supported, beam refinement steps, 1D arrays vs. 2D arrays, etc.) and increase in the UE variety (e.g. introduction of red-cap UEs), UEs with different capabilities will be more common. Awareness of the UE-panel properties at gNB side is a first step toward enhanced BM algorithms.

- : Awareness of the UE-panel properties is a first step toward enhanced BM algorithms.

- : We see a clear benefit of specifying UE reporting of panel-specific information.

## Analog beam definition

Besides UE antenna panel definition, we also observe that there is no explicit definition of what a beam is. Though analog beamforming is highly up to UE implementation, but in our understanding, a beam can be defined as

- : A beam can be defined as a spatial filtering associated with one or two antenna ports carrying one or two layers separated in the polarization domain.

With the beam definition in hand, next one may consider whether to signal the UE capability on whether single or dual layer(s) are supported by a panel. In RAN4#90bis main session’s chairman notes, it was agreed to use 8 beams for an UL beam sweep in beam-correspondence test. In addition it was also agreed to further study whether there is an advantage to specify the polarization aspect in the UL beam sweep. This may have system benefits both from a MU perspective as well as from a link level capacity perspective. Hence we have following

- : RAN1 needs to study and specify (if necessary) whether additional signaling is necessary when a beam can support up to two independent layers separated by polarization.

# Conclusions

Finally, allow us to repeat our proposals to draw attention.

- : For DL QCL, support CSI-RS for CSI as QCL source RS in unified TCI state.

- : To achieve the flexibility of DL and UL beam updating/indication, support separately configured unified TCI state pools per BWP.

- : Support (Alt.1) UE can be dynamically indicated with joint TCI or separate TCI.

- : For common DL/UL beam operation, support (Opt-1) to configure a single TCI state pool across multiple CCs.

- : When configuring a single TCI state pool across multiple CCs (Opt-1), the CC ID and BWP ID for QCL-TypeA RS should be absent in a TCI state. But such restraint should not be mandatorily applied to QCL-TypeD RS.

- : For inter-band CAs, support common beam operation for data/control, DL/UL, multiple BWPs, multiple TRPs and multiple UE panels.

- : The UE capability related to simultaneously steering beams in the same direction belonging to CCs in different bands should be reported.

- : For the optimization of beam indication across bands, the polarization property of beams should be considered.

- : For Rel.17 UL power control, support (Alt.3/AltC) the setting of P0/alpha/closed-loop index is neither associated with nor included into UL or joint TCI state.

- : For Rel.17 UL power control, support (AltB) that PL RS can be associated  with either UL TCI state or joint TCI state.

- : For L1/L2 inter-cell mobility, support to pre-configure unified TCI state with source RS from non-serving cell by incorporating PCI into TCI state.

- : Support CSI-RS for mobility as measurement RS for L1/L2 inter-cell mobility.

- : Support optional UE capability on maximum value of K (beams associated at least with non-serving cell(s)).

- : Define L1-metric based events for L1/L2 inter-cell mobility and support UE triggered beam reporting based on L1 events.

- : At least support to extend Rel.15/16 QCL rule of intra-cell mobility to inter-cell mobility

- CSI-RS (from NSC) for BM and for tracking as direct QCL source for DMRS

- SSB (from NSC) as indirect QCL source for DMRS

- : Support CSI-RS for mobility as source RS for conducting QCL info from non-serving cell.

- : Support (Alt.3) UL DCI format 0_1/0_2 with UL grant to indicate UL Tx beam by joint DL/UL TCI state or UL-only TCI state.

- : Study standard impacts of introducing group common DCI to indicate/update DL TCI state(s) for a group of UEs.

- : For UL grant DCI carrying TCI state(s), if supported, then reuse the implicit HARQ mechanism of PUSCH for UL grant DCI.

- : For GC-DCI carrying unified TCI state(s) for a group of UEs, if supported, study its HARQ mechanism for each UE within the indicated group.

- : For beam application time, support the 1st slot that is X ms or Y symbols after the UE acknowledges DCI carrying joint or separate DL/UL beam indication (Alt2).

- : Support to link or associate UL panels with SRS resource sets for UL fast panel selection.

- : Support to implicitly link or associate UE panel(s) with joint or UL-only TCI state(s) via beam reporting.

- : Support to link a panel entity with a new panel ID (Opt1-2, 1st priority) or with CSI-RS and/or SSB resource in beam reporting (Opt1-1, 2nd priority).

- : Support UE-initiated UL panel(s) linking/association status update via UCI or MAC CE.

- : Do not support NW-initiated panel selection/activation, unless the benefits for supporting this feature can be fully justified.

- : To facilitate MPE mitigation, support UE reports (a subset of Opt 2A) SSBRI(s)/CRI(s) + modified L1-RSRP that accounts for MPE effect.

- : For UL beam sweeping, when taking MPE into account, UE should transmit SRS resource(s) by certain power back-off, if needed.

- : Support UE reports P-MPR value(s) associated with panel(s) (Opt 1D) to mitigate UL coverage loss due to MPE.

- : To avoid UL MPE issue, it would be beneficial for RAN1 to study and, if necessary, specify the mechanism of UE triggered UL beam/panel switch.

- : We suggest that definition of a UE panel and the related properties are addressed prior to further advancing discussions related to UE panel enhancements.

- : A beam can be defined as a spatial filtering associated with one or two antenna ports carrying one or two layers separated in the polarization domain.

- : RAN1 needs to study and specify (if necessary) whether additional signaling is necessary when a beam can support up to two independent layers separated by polarization.

# Reference

- RP-193133, “New WID: Further enhancements on MIMO for NR”, Samsung

- 3GPP TS 38.321, “Medium Access Control (MAC) protocol specification”

- B. Xu, K. Zhao, Z. Ying, D. Sjöberg, W. He and S. He, "Analysis of Impacts of Expected RF EMF Exposure Restrictions on Peak EIRP of 5G User Equipment at 28 GHz and 39 GHz Bands," in IEEE Access, vol. 7, pp. 20996-21005, 2019.