- 3GPP TSG RAN WG1 #105e	 		                                       R1-2105196

- e-Meeting, May 10th –27th, 2021

Source:	ZTE

Title:	Discussion on HARQ for IoT-NTN

Agenda Item:	8.15.4

Document for:  Discussion

- Introduction

In RAN1#104b-e [1], the followings have been agreed on HARQ for IoT-NTN.

Agreement:

Increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17.

In this contribution, the issues related to HARQ are elaborated with corresponding analysis.

- Discussion on necessity for HARQ enhancement

        - DL HARQ

In [2], the required repetition number and throughput were evaluated under 10% BLER constraint. From the results, it can be observed that the DL SNR is high enough and the maximum throughput of PDSCH transmission is achieved under small number of repetition number if continuous transmission is allowed. For example, in GEO with 90 degree elevation angle, the throughput will decrease when repetition number is larger than 8. However, the RTT in this case can be longer than 500 ms. Since the transmission duration of each TB is much shorter than the RTT, significant throughput reduction will happen due to HARQ stalling. In GEO, the loss of throughput can even be larger than 90%. It is reasonable to enhance DL HARQ to avoid such large performance loss.

In RAN1#104b-e, increasing HARQ process number is agreed not to supported, since it will increase the UE complexity, which is contradict to the lower complexity feature of IoT UEs. However, HARQ feedback disabling for DL can be supported. As discussed in NR-NTN, HARQ stalling due to long RTT can be resolved when HARQ feedback is disabled, so that the throughput can be increased with limited specification effort and complexity. Moreover, with disabling of HARQ feedback, the ACK-NACK feedback can be avoid, which will further reduce the power consumption for NB-IoT UE since it’s carried by NPUSCH in legacy system.

Proposal 1: HARQ feedback disabling for DL only should be supported in IoT-NTN.

The configuration of HARQ feedback disabling can considered to be configured per HARQ process through RRC signaling, which is semi-static, or DCI signaling, which can be dynamic. The dynamic configuration on HARQ feedback disabling through DCI is preferred. In LEO, the achievable SNR and RTT can change with different elevation angles. In low SNR scenario, where the required repetition number is high, HARQ stalling is not significant so that HARQ feedback can be enabled. With the movement of satellite, the achievable SNR can be increased and HARQ feedback may need to be disabled to improve throughput. Based on above analysis, it can be observed that dynamic configuration is required to indicate UE when to disable HARQ feedback in order to adapt to variant scenarios.

Proposal 2: Dynamic configuration of HARQ feedback disabling should be supported in IoT-NTN.

        - UL HARQ

In UL, the link budget is significantly smaller than DL, which indicates that more repetitions are required to ensure BLER is below certain threshold. As analyzed in [2], under 10% BLER constraint, the transmission duration of TB is comparable to the RTT. That is, the throughput loss due to HARQ stalling is not significant. Hence, there is no need to enhance UL HARQ to increase throughput.

Proposal 3: Enhancement on UL HARQ to increase throughput is not needed in IoT-NTN.

Reduced PDCCH monitoring was also discussed in RAN1#104b-e for the purpose of power saving. When only one HARQ process is supported, UE will not receive the HARQ feedback of a PUSCH transmission until RTT time has elapsed. Therefore, PDCCH monitoring for UL transmission during this period can be avoided. In traditional TN, the PDCCH monitoring for UL retransmission is controlled by the following two timers for DRX transmission:

- drx-ULRetransmissionTimer: Specifies the maximum number of consecutive PDCCH-subframe(s) until a grant for UL retransmission or the HARQ feedback is received.

- UL HARQ RTT Timer: This parameter specifies the minimum amount of subframe(s) before a UL HARQ retransmission grant is expected by the MAC entity.

Based on the definitions of above two timers, it can be observed that PDCCH monitoring for UL HARQ feedback is performed when drx-ULRetransmissionTimer is running. And drx-ULRetransmissionTimer will be started/restarted when UL HARQ RTT timer expires. Therefore, the start of PDCCH monitoring for HARQ feedback can be delayed by adding an additional offset corresponding to RTT to the UL HARQ RTT timer. In this case, the controlling on the PDCCH monitoring can be well achieved by simply enhancing UL HARQ RTT Timer.

Proposal 4: Reduced PDCCH monitoring can be achieved by enhancing UL HARQ RTT Timer in RAN2.

- Conclusions

In this contribution, discussion of the enhancement on HARQ for IoT-NTN is conducted with following observations and proposals:

Proposal 1: HARQ feedback disabling for DL only should be supported in IoT-NTN.

Proposal 2: Dynamic configuration of HARQ feedback disabling should be supported in IoT-NTN.

Proposal 3: Enhancement on UL HARQ to increase throughput is not needed in IoT-NTN.

Proposal 4: Reduced PDCCH monitoring can be achieved by enhancing UL HARQ RTT Timer in RAN2.

- Reference

- R1-2103987, Session notes for 8.15 (Study on NB-IoT/eMTC support for Non-Terrestrial Network)

- R1-2102919, Discussion on HARQ for IoT-NTN, ZTE