3GPP TSG RAN WG1 #105-e		                                  R1-2105252

e-Meeting, May 10th – 27th, 2021

Agenda Item:	7.2.4

Source: 	NEC

Title:	Remaining issues on resource allocation mode 2

Document for:	Discussion/Decision

- Introduction

This contribution shared our concerns on the current sensing procedure in Rel.16 sidelink which wrongly captures Re.16 agreements.

- Discussion

At RAN1#99, in terms of SPS resources (i.e., mapping of periodic semi-persistent resources into the resource selection window), we agreed to reuse LTE design [1]:

- (working assumption) Procedure of mapping of periodic semi-persistent resources into the resource selection window is reused from LTE

  - By reusing TS 36.213, section 14.1.1.6, steps 5 and 6 of non-partial sensing, as applicable

Back to LTE design, the related agreement was agreed in RAN1#87 [2]:

R1-1613655 WF on Support of Small Transmission Periods for V2V Communication Intel, Qualcomm Inc. Agreement: - Confirm that reselection UE scales the number of reservations of other UE within selection window by 1/i when 0<i<1 for the SCI received in the last i*P_step logical subframes in the sensing window. Here i denotes the resource reservation interval in the received SCI.

So, the boundary as we agreed to scale the number of reservation is "SCI received in the last i*P_step logical subframes in the sensing window".

- In LTE spec, the end of the sensing window is n' , the last i*P_step logical subframes in the sensing window is n'-Prsvp_RX' . Hence, the spec is captured as n'-m≤Prsvp_RX' (i.e., n'-Prsvp_RX' ≤m)

- In NR spec, the end of the sensing window is n'-Tproc,0, the last i*P_step logical subframes in the sensing window obviously is n'-Tproc,0-Prsvp_RX' .

Hence, the current text in NR spec was wrongly capturing the agreement to reuse LTE design and we suggest adopt the text proposal in section 3.

Observation 1: Current NR spec is wrongly capturing the agreement to reuse LTE design in terms of SPS resources.

Proposal 1: Adopt the text proposal in section 3.

Proposal 2: Tscal is set to selection window size T2 converted to units of ms plus Tproc,0SL converted to units of ms if proposal 1 is agreed.

- Conclusion

In this contribution, we shared our concerns on current resource excluding procedure for NR sidelink and proposed that:

Observation 1: Current NR spec is wrongly capturing the agreement to reuse LTE design in terms of SPS resources.

Proposal 1:

- Adopt following text to TS 38.214 8.1.4 [3]:

c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …,Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-Tproc,0SL-m≤Prsvp_RX', where tn'-Tproc,0SLSL = n-Tproc,0SL if slot n-Tproc,0SL belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'- Tproc,0SLSL is the first slot after slot n-Tproc,0SL belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

Proposal 2:

- Tscal is set to selection window size T2 converted to units of ms plus Tproc,0SL converted to units of ms if proposal 1 is adopted.

- Reference

[1] 3GPP TSG RAN WG1 Meeting #99 RAN1 Chairman’s Notes

[2] Final Report of 3GPP TSG RAN WG1 #87 v1.0.0 (Reno, USA, 14th – 18th November 2016)

[3] 3GPP TS 38.214 V16.5.0 (2021-03)