3GPP TSG RAN WG1 #105-e		                   R1-2105479

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.4.3

Source:	LG Electronics

Title:	Discussions on HARQ enhancements in NTN

Document for:	   Discussion and Decision

- Introduction

In the RAN1#104-e meeting, following working assumption and agreements were made and captured in the chairman’s note as:

Agreement:

For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

Agreement:

For Type-2 HARQ codebook in NTN: Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes

- FFS: The details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes

In this contribution, we further discuss on the HARQ enhancements in NTN.

- Discussions

HARQ process id identification

In RAN1 #103-e meeting, agreement on HARQ process id identification was made as follows:

Agreement:

- Enhanced HARQ process ID indication is supported for DCI 0-2/1-2 and DCI 0-1/1-1 by at least one of following:

  - Option 1: Slot index as the MSB

  - Option 1-a:Slot index as the LSB

  - Option 2: Reusing one bit from other bit field

  - Option 3: Extending the HARQ process ID field up to 5 bits

- FFS: DCI 0-0/1-0

- Note: 32 is taken as maximal supported HARQ processes number for both UL and DL

Option 1 or 1-a can provide additional 1 bit based on the slot index at the expense of scheduling flexibility or additional UE complexity. For example, in the slot based solution, additional 1bit can be identified by the scheduled slot index (e.g., according to the even slot index or odd slot index), but the scheduling flexibility at gNB can be restricted compared to other options. In option 2, other field can be reused for obtaining additional 1 bit. In case of HARQ feedback is disabled, some fields such as NDI, RV, PRI, etc., can be a candidate for option 2. However, it is not clear how this option work when HARQ feedback is enabled.

Option 3 can be a straightforward and clean way for supporting up to 32 HARQ processes. For non-fallback DCI (DCI 0-2/1-2 and DCI 0-1/1-1) cases, increasing 1bit payload in DCI can be acceptible for explicit HARQ process indication. However, in case of fall back DCI (DCI 0-0/1-0), it is not desirable to increase the payload, since fall back DCI is designed for supporting basic NR operations. Thus, in this case, it is desirable to limit the number of HARQ processes up to 16 for fallback operation. Based on above discussion, for enhanced HARQ process id identification, our preference is supporting option 3 with non-fallback DCI (DCI 0-2/1-2 and DCI 0-1/1-1) only. Note that, DCI 0-2/1-2 is originally introduced for URLLC purpose for reliability enhancement using compact DCI. In NTN, DCI 0-2/1-2 can also be applicable especially when HARQ feedback is disabled. In this case, some redundant fields related to HARQ feedback (e.g., NDI, PRI, RV and etc.) can be omitted.

Proposal 1: For enhanced HARQ process id identification in NTN, support followings enhancement.

- For non-fallback DCI (DCI format 0-2/1-2 and DCI format 0-1/1-1), increase the HARQ process ID field up to 5 bits

- For fallback DCI (DCI format 0-0/1-0), support no enhancement

Enabling/disabling HARQ feedback

Performance enhancements

In order to increase the link reliability performance, some enhancement on the transmission can be considered when the HARQ feedback is disabled. If the HARQ feedback is disabled, slot aggregation based transmission can be a good option to ensure the DL performance. For NTN scenario, larger number of repetition may provide good DL BLER performance at the expense of spectral efficiency. In [1], the trade-off between spectral efficiency and BLER according to the number of repetition was presented, and it is desired to select proper repetition number according to SINR and channel condition. To this end, repetition factor can be recommended by UE.

Another consideration point is how to properly determine MCS level when HARQ feedback is disabled. Without HARQ feedback, in order to determine the proper MCS level, gNB may rely on the RLC feedback which requires additional delay. To this end, PDSCH decoding results or decoding probability as new CSI contents can be considered. For example, UE may report its PDSCH decoding results using bit-map for a certain period configured by gNB. With this information, gNB can configure proper MCS level, repetition factor for PDSCH repetition and so on.

Proposal 2. For transmission enhancement when HARQ feedback is disabled, consider following enhancements:

- repetition factor is recommended by UE

- PDSCH decoding results or probability is reported by UE

PDSCH scheduling restriction

In the RAN1#104-e meeting, a working assumption regarding minimum gap value, X = T_proc,1, was made. The FFS point is whether X should be changed to X = max(T_proc,1, K1). Since T_proc,1 is the relatively relaxed processing time compared to PDSCH processing time, N1, it seems sufficient as a gap between two PDSCH with same HARQ process. Therefore, further relaxation by additionally introducing network controlled gap timing K1, which is the minimum k1 if configured, is not needed. Based on the above discussion, we propose to confirm working assumption of X=T_proc,1.

Proposal 3. Confirm working assumption of X = T_proc,1.

PDSCH scheduling restriction between different HARQ process ids

When two PDSCHs are associated with different HARQ process id, PDSCH scheduling restriction can also be considered. In NR, if there are two PDSCHs where PDSCH1 is scheduled in slot n and PDSCH2 is scheduled in slot n+k, ACK/NACK feedback timing for PDSCH1 should be before that of PDSCH2. So, if one of HARQ process is feedback-disabled, there might be some ambiguity on out-of-order constraint check. One simple solution can be introducing default/virtual ACK/NACK timing for disabled HARQ process.

Proposal 4. Discuss PDSCH scheduling restriction if two PDSCHs are associated with different HARQ process id and one of two PDSCHs is HARQ feedback disabled.

- HARQ-ACK codebook enhancement

Type-1 HARQ-ACK codebook

In this subsection, we discuss on how to construct HARQ-ACK codebook when HARQ feedback is disabled. Regarding this issue, in the last meeting, several options for enhancing HARQ-ACK codebook were identified. For Type-1 HARQ-ACK codebook following three options were listed as follows:

- Option-1: No enhancement;

- Option-2: Report NACK on disabled process

- Option-3: Reduce codebook size with criteria

In Type-1 HARQ-ACK codebook, the location of ACK/NACK feedback bit is determined based on the SLIV of the scheduling DCI and the codebook size is determined by semi-static configuration. In NTN, some HARQ processes can be configured as HARQ feedback disabled. In this case, the UE is not required to read some fields (e.g., K1 and PRI field) used for ACK/NACK feedback even though those fields are presented. Then, Type-1 HARQ-ACK codebook could be constructed only for the serving cells having at least one feedback-enabled HARQ process, by excluding the serving cells not having any feedback-enabled HARQ process. For example, in case of all HARQ processes in a certain CC are disabled for feedback, ACK/NACK feedback corresponding to this CC is not needed. If only one HARQ process is enabled for HARQ feedback in a certain CC, the UE is only required to feedback the information of ACK/NACK for the enabled HARQ process, i.e., 1bit feedback. It is quite desirable since Type-1 codebook size would be large compared to other codebook types, and even this reduction, obviously there would be no ambiguity on codebook size between UE and gNB.

Proposal 5. For Type-1 HARQ-ACK codebook enhancement, option 3 (reduced codebook size with criteria) is preferred.

Type-2 HARQ-ACK codebook

For Type-2 HARQ-ACK codebook, it is agreed to reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes. The remaining issue is details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-enable/disabled HARQ processes. In this regard, several detailed alternatives on C-DAI and T-DAI counting for DCI of PDSCH with enabled/disabled HARQ processes were identified during the email discussion in RAN1#103-e. Among the alternatives, in order to ensure the reliability for HARQ feedback-disabled processes, it is preferred that the C-DAI and T-DAI are given their true values (i.e., the count of feedback-enabled processes), despite they are not incremented. If all of DCIs scheduling PDSCHs are with disabled HARQ processes or the first DCI scheduling PDSCH is with disabled HARQ process, there might be some issues such as 1) whether or not to read the C-DAI value in scheduling DCI, 2) what is the initial C-DAI value if UE reads the C-DAI in scheduling DCI, and 3) how to construct corresponding ACK/NACK payload and transmit ACK/NACK feedback. One simple solution can be to set initial C-DAI as 0, if all of DCIs scheduling PDSCHs are with disabled HARQ processes or the first DCI scheduling PDSCH is with disabled HARQ process.

Proposal 6. For Type-2 HARQ-ACK codebook enhancement, the C-DAI and T-DAI are given their true values (i.e., the count of feedback-enabled processes) in the DCI of PDSCH with enabled/disabled HARQ processes.

Type-3 HARQ-ACK codebook

Type-3 HARQ-ACK codebook was introduced for NR-U (unlicensed band), and it was further agreed to be supported in licensed bands. In Type-3 HARQ-ACK codebook, HARQ-ACK feedback for all HARQ processes is provided in one shot manner. Therefore, if Type-3 HARQ-ACK codebook is constructed based only on enabled HARQ processes, the overhead can be reduced effectively. Compared to Type-2 HARQ-ACK codebook, Type-3 HARQ-ACK codebook has a merit in terms of DCI overhead and no codebook ambiguity caused by DCI missing, while it requires larger UCI overhead. In that sense, support of enhancement on Type-3 HARQ-ACK codebook is beneficial when HARQ feedback is disabled.

Proposal 7. Support enhancements on Type-3 HARQ-ACK codebook by including enabled HARQ processes only.

SPS PDSCH activation/release

In NR, HARQ-ACK feedback is used to indicate whether the UE is correctly received SPS PDSCH activation/release command or not. However, if HARQ feedback is disabled, gNB cannot know that information, and thus it may cause redundant DL resource reservation or DL buffer corruption. To resolve these issues, if the first PDSCH after reception of the SPS activation DCI is with disabled HARQ process, UE feedbacks acknowledgement for the reception of SPS activation DCI. If the first PDSCH after reception of the SPS activation DCI is with enabled HARQ process, UE transmits ACK/NACK information according to the decoding results of that PDSCH. Similarly, for SPS PDSCH release, UE feedbacks acknowledgement for the reception of SPS release DCI even though HARQ feedback is disabled. As another solution, UE does not expect the SPS PDSCH configuration with any disabled HARQ process. However, it may restrict the gNB scheduling flexibility.

Proposal 8. UE feedbacks acknowledgement for the reception of SPS activation DCI, if the first PDSCH after reception of the SPS activation DCI is with disabled HARQ process.

- Conclusion

In this contribution, we discussed on the HARQ enhancements in NTN. Based on the above discussion, we have following proposals:

Proposal 1: For enhanced HARQ process id identification in NTN, support followings enhancement.

- For non-fallback DCI (DCI format 0-2/1-2 and DCI format 0-1/1-1), increase the HARQ process ID field up to 5 bits

- For fallback DCI (DCI format 0-0/1-0), support no enhancement

Proposal 2. For transmission enhancement when HARQ feedback is disabled, consider following enhancement:

- repetition factor is recommended by UE

- PDSCH decoding results or probability is reported by UE

Proposal 3. Confirm working assumption of X = T_proc,1.

Proposal 4. Discuss PDSCH scheduling restriction if two PDSCHs are associated with different HARQ process id and one of two PDSCHs is HARQ feedback disabled.

Proposal 5. For Type-1 HARQ-ACK codebook enhancement, option 3 (reduced codebook size with criteria) is preferred.

Proposal 6. For Type-2 HARQ-ACK codebook enhancement, the C-DAI and T-DAI are given their true values (i.e., the count of feedback-enabled processes) in the DCI of PDSCH with enabled/disabled HARQ processes.

Proposal 7. Support enhancements on Type-3 HARQ-ACK codebook by including enabled HARQ processes only.

Proposal 8. UE feedbacks acknowledgement for the reception of SPS activation DCI, if the first PDSCH after reception of the SPS activation DCI is with disabled HARQ process.

Reference

[1] R1-1913019, “Considerations on HARQ in NTN”, Nokia