3GPP TSG RAN WG1 #105-e		R1-2105511

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.8.1.1 Enhancements on PUSCH repetition type A

Source: 	Sierra Wireless

Title: 	Design considerations for PUSCH repetition Type A Enhancements

Document for: 	Discussion and decision

# Introduction

This tdoc discusses design considerations for PUSCH repetition Type A Enhancements for the following work item [1] objectives:

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

Agreements made in previous meetings:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

FFS details

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

# Increasing the maximum number of repetitions

This section discusses this objective stated in the WID[1]:

Increasing the maximum number of repetitions up to a number to be determined during the course of the work

As stated in the WID, the maximum number of repetitions is FFS. There are many factors to consider when deciding how much to increase the maximum number of repeats.

eMBB Use Case:

The minimum data rate for the eMBB use case was agreed to be 100 kbps and is also documented in the WID. The figure below shows the LLS result for repeats [8 and 16] and TBS [888 and 1800] which corresponds to a data rate of 100kbps (detailed simulation assumptions are in appendix A):

Figure 2: LLS Result for 100kbps, Repeats=8 and Repeats=16

As seen from the above figure, 16 repeats does NOT provide coverage gain compared to 8 repeats. This is expected, because the energy per bit is the same since the data rate is kept constant at 100kbps. In fact, there is a small loss in coverage with 16 repeats. This is due to the loss of coding gain when using the larger TBS = 1800 since a higher code rate is needed. Some of this loss is recovered at Fdop=25Hz and 50Hz due to the increase in time diversity with 16 repeats but this is not enough to make up for the code gain loss.

For a data rate of >=100kbps (eMBB), increasing repeats beyond 8 does not provide any coverage gain

VoIP Use Case:

The following FR1 simulation assumptions for VoIP were agreed upon:

Vocoder generate one 320 bits TB every 20ms

Maximum latency 50/100ms

FR1 TDD frame structure: DDDSU and DDDDDDDSUU with SCS = 30kHz

FR1 FDD SCS = 15kHz

Since coverage is generally proportional to the time and power of transmission, we can use these factors to determine the maximum number of repeats needed. If latency beyond 20ms is acceptable, TBs need to be aggregated (i.e. a larger TBS is used) to avoid the need for the UE to transmit two TBs simultaneously which would split the UE’s power and decrease coverage. To avoid TB transmissions overlapping, the transmission time needs to be less than or equal to the vocoder packet aggregation time. Given 100ms was agreed to be the maximum latency, the maximum number of aggregations of the 20ms vocoder packets is 3. The figure below shows an example of 3 aggregated vocoder packets which result in 60ms of transmission time and a latency of 100ms:

| 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet | 20ms Vocoder 320bit Packet |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Aggregated TB #1 TBS = 960 bits |  |  | Aggregated TB #2 TBS = 960 bits |  |  | Aggregated TB #3 TBS = 960 bits |  |  |
|  |  |  | Max Transmit time = 60ms #1 TBS = 960 bits |  |  | Max Transmit time = 60ms #2 TBS = 960 bits |  |  |

Figure 3: Example of 3 aggregated vocoder packets resulting in 100ms latency

Given the above observation, and given the TDD formats of DDDSU and DDDDDDDSUU, the following observation can be made for TDD:

For FR1 TDD VoIP with 100ms latency, the maximum number of repeats that can be utilized is 24.

Interaction with TBoMS (TB over multi-slots)

How TBoMS interacts with repeats is still FFS. The concern is that if TBoMS supports repeats, then the transmission times could be very long. For example, if the TB is coded over 4 slots and is repeated 24 times, the total transmission time is 4*24 = 96 slots. Also, if TBoMS does support repeats, TBoMS can be used to increase coverage and even if it is only across 2 slots all coverage requirements including VoIP can be addressed and thus there is no need to increase # of repeats to increase coverage.

If TBoMS supports repeats, there is no need to increase number of repeats to improve coverage.

Thus, before deciding to increase the number of repeats and to what number, TBoMS needs to conclude whether it supports repeats or not:

Before deciding whether or not to increase number of repeats from 16, TBoMS need to decide whether it supports repeats or not.

# Repetitions counted basis on available UL slots

This section discusses this objective stated in the WID[1]:

The number of repetitions counted on the basis of available UL slots.

Even when repeats are counted based on available slots, the legacy 16 repeats coverage does not provide optimal coverage for the VoIP use case where  24 repeats are needed. 16 repeats with TBoMS over 2 slots can effectively provide 16 X2 = 32 repeats which meets the VoIP coverage requirement; thus these features should be allowed to be enabled at the same time.

The (“Increase repeats” feature or the TBoMS feature) and the “Counting based on available UL slots” feature should be allowed to be enabled at the same time.

The following related conclusion was made:

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

The following related agreement was made:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

The RedCap WI [4] includes the following note stating the CovEnh work shall be available for RedCap UEs:

- Uplink coverage enhancement solutions specified in the NR Coverage Enhancement WI (NR_cov_enh) shall be assumed to be available also to RedCap UEs by default (with small modifications for RedCap UEs if found necessary).

The RedCap WI [4] also states HD-FDD shall be specified:

  - Duplex operation:

    - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

When deciding on a method for determination of available slots, RAN1 should consider support for RedCap HD-FDD UEs.

In some FDD scenarios, Alt2/Alt-b will require a FD-FDD UE to be able to decode dynamic control information (e.g. SFI, CI, PUSCH priority) while transmitting, which is not possible for RedCap HD-FDD UEs. It may be possible for gNB to avoid this situation for HD-FDD RedCap UE’s but this would require complex scheduling algorithms or complex specification changes. Given this, the following proposal is made:

The method for determination of available slots should not depend on dynamic signalling (e.g. SFI, CI) – Alt-a and Alt1 should be agreed

Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

# Conclusions

- For a data rate of >=100kbps (eMBB), increasing repeats beyond 8 does not provide any coverage gain

For FR1 TDD VoIP with 100ms latency, the maximum number of repeats that can be utilized is 24.

If TBoMS supports repeats, there is no need to increase number of repeats to improve coverage.

- Before deciding whether or not to increase number of repeats from 16, TBoMS needs to decide whether it supports repeats or not.

The (“Increase repeats” feature or the TBoMS feature) and the “Counting based on available UL slots” feature should be allowed to be enabled at the same time.

When deciding on a method for determination of available slots, RAN1 should consider support for RedCap HD-FDD UEs.

The method for determination of available slots should not depend on dynamic signalling (e.g. SFI, CI) – Alt-a and Alt1 should be agreed

Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

# References

RP-202928 China Telecom, “New WID on NR coverage enhancements”

RP-200861 China Telecom, “Study on NR coverage enhancements”

TR 38.830 “Study on NR coverage enhancements”

RP-210918 WI “Support of reduced capability NR devices”

# Appendix A: LLS Simulation Assumptions:

| Parameter | Value |
| --- | --- |
| System bandwidth | 20 MHz |
| Configuration | FDD |
| Carrier frequency | 700MHz |
| Antenna configuration | NA |
| RX Antenna ports | 2 |
| Channel model | TDL-C 37ns |
| Doppler frequency | 2, 25, 50, 100 Hz |
| Number of PRBs | 4 |
| Frequency tracking error | 0Hz |
| Channel Estimation | Perfect |
| Frequency Hopping | None |
| DMRS Configuration | 2 DMRS per slot |
| Transform Pre-coding | No |
| SCS | 15 kHz |
| Modulation | QPSK |
| TBS | 888 and 1800 bits |
| Repeats | 8 and 16 |