3GPP TSG RAN WG1 #105-e	R1-2106117

# e-Meeting, May 10th – May 27th, 2021

Agenda Item:		8.7.1.2

Source:				Moderator (Samsung)

Title:					Final summary for TRS/CSI-RS occasion(s) for idle/inactive UEs

Document for:	Discussion/Decision

# Introduction

This document provides the summary of the contributions for TRS/CSI-RS occasion(s) for idle/inactive UEs in Section 8.7.1.2. The proposes from submitted contributions are categorized into the following topics for discussion in RAN1#105-e meeting:

- Topic #1. Availability indication

  - Issue 1-1: L1 based signaling

  - Issue 1-2: SIB based signaling

  - Issue 1-3: Indication content

  - Issue 1-4: Validity time

  - Issue 1-5: Application delay

- Topic #2. Configuration details

  - Issue 2-1: Application values for supported parameters

  - Issue 2-2: Other parameters

  - Issue 2-3: Details of QCL information

  - Issue 2-4: Methods for configuration overhead reduction

- Topic #3: Others

The following email thread for TRS/CSI-RS occasion(s) for idle/inactive UEs is announced by Chairman in RAN1#105 e-meeting:

//This one is to use NWM – please use RAN1-105-e-NWM-NR-R17-PowSav-02 as the document name

[105-e-NR-R17-PowSav-02] Email discussion regarding TRS/CSI-RS occasions for idle/inactive UEs – Qiongjie (Samsung)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

# Topic #1: Availability indication

According to the following agreement in RAN1#104bis-e meeting, a working assumption has been agreed to support L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) for idle/inactive UEs. Details are open for further disucssion. For indication content, two alterantives were agreed for downselection.

Working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported: - Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources - FFS whether and how to indicate the ‘availability’ in beam selective manner. - Other alternatives are not precluded

The following proposals regarding the availability indication are submitted in RAN1#105-e:

| ZTE [1] | Proposal 1: Confirm the working assumption that supporting L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. Proposal 2: The TRS availability indication should be carried by PEI. Proposal 3: The TRS availability indication carried by paging DCI can be also considered in addition to PEI based solution. Proposal 4: Alt1 with a bitmap can be used to indicate the availability/unavailability information for each configured TRS resource or resource set. Proposal 5: An application delay can be considered at least in the case that TRS resource or resource set availability indication is updated. |
| --- | --- |
| Huawei, HiSilicon[2] | - The validity time for PEI based availability indication is the time duration between the PEI and the associated PO. - The validity time for paging DCI based availability indication is the time duration after the transmission of the paging DCI and before the next PO of the same UE. - An indication periodicity is introduced which can be configured N paging cycles, during which the availability of assistance TRS is assumed to be the same. - If indication periodicity is additionally configured, the validity time of availability indication is the same as the indication periodicity. - Support indication of availability of assistance TRS before the start of PO through PEI and legacy paging DCI. - Bitmap is the baseline for availability indication, where each bit indicates a RS or a group of RS. - Support to indicate the availability of assistance TRS(s) per beam direction by a bitmap, where each bit corresponds to the assistance TRS(s) that are QCLed with the same associated SSB index. - Support to indicate the availability of assistance TRS in a window before the PO. |
| TCL Communication Ltd[3] | Proposal 1: Support Alt1; the explicit availability/unavailability information for all or some of the configured RS resources using a bitmap or codepoint. Proposal 2: Support the validity time for availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UE, and consider the following alternatives to measure the accurate validity time: Alt1: Single timer based validity time mechanism Alt2: Dual timer based validity time mechanism Proposal 3: Application delay can be considered based on the availability/unavailability status of TRS in the network as given below. - Application delay is necessary when the availability/unavailability status of TRS is changing - Application delay is not necessary when the availability/unavailability status of TRS remains the same Proposal 4: Beam selective transmission of TRS availability indication should be associated to the specific beams which are used to transmit the same TRS resources and TRS resources should be configured in beam specific manner for each SSB beams. |
| Vivo[4] | Proposal 2: The availability indication can be delievered at least through paging DCI. - TRS availability indication through PEI also depends on the signal/channel design of PEI, and it can be discussed after the details are clear. Proposal 3: SIB based TRS avsilsbility update can be supported. - It is up to RAN2 to decide whenther the same SI update mechanism is reused. Proposal 4: UE assumes the TRS available before receiving signaling indicating the TRS as NOT available. Proposal 5: The L1 availability indication takes effect once it is received from UE perspective. Proposal 6: Availability/unavailability information for all or some of configured RS resources is indicated using a bitmap, where each bit is associated with at least one resource or a set/group of resources. |
| Spreadtrum Communications[5] | Proposal 1: Confirm the working assumption on signalling method of availability indication of TRS/CSI-RS. Proposal 2: The availability indication of TRS/CSI-RS in paging DCI or PEI DCI shall be supported. Proposal 3: Availability/unavailability information for all or some of configured RS resources using a bitmap shall be supported. Proposal 4: Validity time for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UE shall be supported. Proposal 5: Application delay for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UE is not need, the availability indication becomes effective once received. |
| CATT[6] | Proposal 6: Working assumption that support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs should not be confirmed Proposal 8: The availability indication should be one bit or one code point to indicate all TRS/CSI-RS resources within a cell. UE could not assume any TRS/CSI-RS resource if the availability indication is only indicated the selected TRS/CSI-RS resources within a cell. Proposal 9: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-X. |
| CMCC[7] | Proposal 1. Both paging DCI and PEI can be considered as L1 based signaling for the availability indication of TRS/CSI-RS. SIB can be used to indicate which L1 based signalling to be used. Proposal 2. Don’t allow indicating the availability of TRS/CSI-RS only in paging DCI without short message and/or scheduling information. Proposal 3. Support Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap. Proposal 4. A pre-defined validity time can be considered for PEI based availability indication signalling, e.g., the time before the associated PO. Proposal 5. Both pre-defined validity time and explicit validity time indication can be considered for paging DCI based availability indication signalling. |
| Qualcomm Incorporated[8] | Proposal 3: Use the paging PDCCH to carry the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs - By the unused and/or reserved bits in the paging DCI - This includes cross-slot scheduling paging PDCCH as PEI. Proposal 4: For availability/unavailability information provided by a physical layer availability indication for TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs - First preference based on Alt. 1: Availability and unavailability information for all the configured RS resources using a bitmap - Second preference based on Alt. 2: Codepoint to indicate one or more resource/configuration indices that correspond to all the available RS resources. |
| OPPO[9] | Proposal 1: Paging DCI or PEI can be used to indicate the availability of TRS/CSI-RS. Proposal 2: At least paging DCI shall be used for TRS availability indication. Proposal 3: When PEI is configured for idle/inactive UEs, PEI can alternatively be used (e.g., based on network’s configuration) for TRS availability indication. Proposal 5: Availability/unavailability information is for all configured RS resources using a bitmap. - each bit is associated with one resource or a set/group of resources |
| Intel[10] | Proposal 2: Support paging DCI as the L1 indication for availability indication update - SIB provides a default value - Upon receiving change indication by L1, UE assumes the newly indicated availability characteristic - UE can be configured to switch back to default value upon expiry of a timer Proposal 3: Support availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint. |
| Apple[11] | Proposal 3: Support PEI (if PDCCH is adopted for PEI), paging DCI, and SIB-based signaling to carry the availability indication of the TRS occasion(s). Proposal 4: When the availability indication is carried in a DCI, it only carries the information for TRS/CSI-RS configuration(s) that correspond to the same beam as the DCI. Proposal 5: When the availability indication is carried in a DCI, a bitmap is used to carry the availability indication, with one bit per TRS configuration in the same beam. Proposal 6: When a TRS configuration is indicated as available, the idle/inactive UEs assumes that only a certain number of TRS occasion(s) before a PO is available. |
| Sony[12] | Proposal 1: Availability information of TRS/CSI-RS is signaled in the paging DCI. Proposal 2: Support to provide additional availability information (e.g. availability duration, which active TRS/CSI-RS are currently available). Proposal 4: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, use value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources. Proposal 5: Indication the ‘availability’ of TRS/CSI-RS occasions in beam selective manner is not supported. |
| Samsung[13] | Proposal 1: Support paging PDCCH based explicit availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Proposal 2: Whether or not to support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs should be discussed after the completion of L1 signal/channel design of PEI. Proposal 3: Support SIB based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. If the same availability indication is provided via L1 signal/channel, no need to provide SI update notification in paging PDCCH for the change of the availability indication. Proposal 4: Support indication of available TRS/CSI-RS resource set(s) for idle/inactive UEs. value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources Proposal 5: An availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be provided to UEs explicitly by SIB or L1 based availability indication. |
| MediaTek[14] | Proposal 1: Confirm the following working assumption to support L1-based signalling for TRS/CSI-RS availability indication. - Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. Proposal 2: SIB-based signalling with existing SI update procedure is not supported for TRS/CSI-RS availability indication. Proposal 3: If paging early indication (PEI) is configured, gNB indicates the TRS/CSI-RS availability information to idle/inactive mode UE(s) through PEI. Proposal 4: UE assumes the same TRS/CSI-RS availability indication in multi-beam operation. Proposal 5: for the availability indication of TRS/CSI-RS at the configured occasion(s), the application delay is not needed if validity time is supported. |
| LG[15] | Proposal 1: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE at least by the PEI. - FFS: whether to support indication using paging DCI Proposal 2: The L1 based signaling can indicate available duration for the TRS/CSI-RS occasion(s). - FFS: Details including the length of duration and its indication method |
| Panasonic[16] | Proposal 1: Confirm the working assumption on L1 based signaling for TRS/CSI-RS availability indication. Both PEI and paging based signaling are supported. Proposal 2: Before agreeing on the signaling details, the maximum number of TRS configurations supported by TRS/CSI-RS occasion(s) should be agreed first. This has direct impact on the detailed L1 signaling design. Proposal 4: Validity period needs to be defined for L1 availability/unavailability indication of TRS/CSI-RS occasion(s) for IDLE/INACTIVE UEs. |
| Nokia, Nokia Shanghai Bell[17] | Proposal: Support indicating the availability in static manner via SI without SI update and physical layer presence/availability indication. Proposal: Support providing static TRS availability configuration, e.g. in a form of a time table. Proposal: Physical layer indication should not increase the network overhead by requiring frequent/periodic signal transmission. Proposal: Support paging DCI based and PEI based beam specific aperiodic availability indication with validity timer as an additional availability indication. The value of validity timer should be configurable by network. |
| Xiaomi[18] | Proposal 2: Paging DCI or PDCCH based available/unavailable indication is preferred from the L1 point of view. Proposal 3: Alt1 is preferred for the information provided by a physical layer availability indication of TRS/CSI-RS, and in addition QCL association may be updated in the same time. |
| Sharp[19] | Proposal 3: Alt1 using a bitmap should be supported for availability/unavailability information for configured RS resources Proposal 4: The physical layer indication only indicates availability/unavailability commands for the resources with the same QCL reference |
| NTT DOCOMO, INC[20] | Proposal 1: When the availability is informed e.g., by paging PDCCH, the timer (re)starts, and then after the timer expires, i.e., the availability indication has not been received for the timer period, the UE assumes no TRS/CSI-RS can be obtained. - The time period can be configured, e.g., via SIB. Proposal 2: Only Paging DCI and/or paging early indication should be adopted to indicate the availability of TRS/CSI-RS for idle/inactive mode UE. |
| InterDigital, Inc.[21] | Proposal 1: Downselect between paging PDCCH or paging early indication for availability signaling after more progress is achieved in the design of the paging early indication. Proposal 2: Availability is indicated using a bitmap where each bit associated to a group of (including one) resources. Proposal 3: SIB-based signaling of availability indication is not supported. |
| Lenovo, Motorola Mobility[22] | Proposal 5: Availability of TRS transmission is indicated in paging DCI or DCI carrying the PEI (if DCI based PEI is supported). Proposal 6: gNB can configure a validity time interval for a TRS configuration. Upon expiry of the validity time, UE assumes that previous TRS configuration is unavailable. |
| Ericsson[23] | Proposal 1 Support L1-based TRS availability indication with associated validity timer via a bitfield in Paging DCI. Proposal 2 For L1-based TRS availability indication via Paging DCI, higher layers can configure multiple validity timer value(s) and the applied validity timer value is indicated via Paging DCI. Proposal 3 For L1-based TRS availability indication via Paging DCI, the bitfield within the paging DCI is explicitly configured using a start and length field (Details FFS). Proposal 4 For L1-based TRS availability indication via Paging DCI, a codepoint/bitmap based approach can be used to indicate TRS availability of different resources and/or for different validity timer values. Proposal 5 For L1-based TRS availability indication via Paging DCI, support beam selective TRS availability indication, i.e., if UE detects Paging DCI in a beam X, the availability bitfield in the Paging DCI is associated to a group of beams corresponding to beam X. a. Grouping is configured via higher layers (Details FFS) |
| Nordic Semiconductor ASA[24] | Proposal-2: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support - Alt1: Availability/unavailability information for a set of configured TRS resources is indicated in the PEI by a single bit. - The set of TRS resources is given by TRS resources associated to SSBs indicated in ssb-PositionsInBurst, unless configured by gNB otherwise. |

## First round discussion

According to the contributions submitted in RAN1#105b-e, the proposals regarding availability indication of TRS/CSI-RS at the configured occasion(s) for idle/inactive UEs can be categorized into five aspects

- L1 based signaling

- SIB based signaling

- Indication content

- Validity time

- Application delay

### Issue 1-1: L1 based signalling

A remaining issue about L1 based signaling is whether or not to confirm the following working assumption from last meeting.

Only one company are not OK to confirm the working assumption. In [6], CATT raised following concerns:

- When IDLE/Inactive UE moving to a new cell, L1 signaling of TRS/CSI-RS availability indication received from original cell is invalid.

- L1 signaling has backward compatibility issues when DCI is used as the L1 signaling. For instance, the reserved bits in the paging DCI could wake up legacy UE without the need of receiving paging/SI update, which will cause additional power consumption of legacy UE.

- It will be concatenated effects in limiting UE power saving gain and missing the paging message when L1 signaling is missed detected. Since the miss-detection occurs for IDLE/Inactive UE with L1 signaling, it will create misalignment between gNB and UE’s understandings on the availability of TRS/CSI-RS.

For 1), the availability indication from original cell will be invalid regardless of the signaling type when cell reselection occurs. This seems to be a common issue for any signaling type. When cell reselection happens, UE has to receive a new availability indication from the new cell, otherwise the UE cannot assume any availability by default. It’s possible that SIB based signaling could be better than L1 based signaling for this case, since UE receives L1 based signaling after SIB reception. However, the latency doesn’t seem to be a critical issue. Whether or not to further consider SIB based signaling is still open.

For 2), it’s not clear what’s the backward compatibility issue. The legacy UEs can ignore the reserved bits of paging PDCCH.

For 3), the impact of miss-detection of L1 based availability indication is still under discussion. For instance, many companies propose to support validity time for the availability indication. With validity time for L1 based availability indication, UE cannot assume any availability of TRS/CSI-RS when miss-detection happens.

Based on the analysis above, it’s suggested to consider the following proposal.

Moderator proposal #1

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Discussion take place on NWM.

In addition, there are many proposals discussing down selection from the two alternatives for L1 signaling based availability indication. The motivations and concerns for PEI based and paging PDCCH based signaling are summarized below.

|  | Motivations | Concerns |
| --- | --- | --- |
| Alt 1: PEI | - Low UE power consumption. UE can skip receiving paging DCI and paging PDSCH when there is no paging message for the UE. | - Feasibility depends on the signal/channel design of PEI - Couples the PEI and idle/inactive TRS/CSI-RS features |
| Alt2: paging PDCCH | - Decouples PEI and idle/inactive TRS/CSI-RS features. It allows an inactive/idle mode UE to support the TRS/CSI-RS but not the PEI. - Enough DCI bits to indicate the availability information, | - UE still needs to detect PO even when it is not paged, which will degrade the power saving gain - May cause “always-on” signal if the paging DCI is transmitted each PO to indicate the TRS availability information. |

Companies’ views are summarized as follows:

  - Alt1: ZTE, Huawei, HiSilicon, Spreadtrum, CMCC, OPPO, Apple (if PDCCH based), MediaTek, LG, Xiaomi, Docomo, Lenovo(if PDCCH based) (12)

  - Alt2: ZTE, Huawei, HiSilicon, Vivo, Spreadtrum, CMCC, Qualcomm, OPPO, Intel, Apple, Sony, Samsung, Xiaomi, Docomo, Lenovo, Ericsson (16)

  - Others:

  - CATT: An 1-bit explicit indication of enable/disable L1 signaling for TRS/CSI-RS availability indication can be configured together with TRS/CSI-RS resource configuration in SIB-

  - CMCC: SIB can be used to indicate which L1 based signalling to be used

The following proposal is suggested for discussion based on the summarized views above.

Moderator proposal #2

Support at least paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- Whether or not to support PEI based availability indication can be discussed after L1 signal/channel of PEI is confirmed

- FFS enable/disable the feature configurable by SIB

Discussion take place on NWM.

### Issue 1-2: SIB based signalling

On the one hand, some companies [Vivo, CATT, Intel, Apple, Samsung, Nokia] propose to support SIB based signaling of the availability indications, which are beneficial for the following use cases:

- Reuse SI update mechanism for infrequent update.

- A default mode, and UE switches back to the default mode when L1 based availability indication expires

- Cell reselection

On the other hand, some companies [HW, Docomo, InterDigital] proposed to not support SIB based signaling of the availability indication due to the following concerns:

- Availability indication carried in SIB may introduce additional paging DCI transmission, which leads more resource overhead for frequent update of availability for assistance TRS.

- Restriction of the flexibility of using TRS send for connected UEs.

For SIB based signaling, there are proposals to further clarify the details, s.t.

- Alt1: the availability of TRS/CSI-RS at a given cell should be informed to the UE by the present/not present of SIB-X TRS/CSI-RS configuration [CATT].

- Alt2: static TRS availability configuration, e.g. in a form of a time table [Nokia].

In addition, there are some proposals regarding the SI update mechanism associated with SIB-based signaling.

- It is up to RAN2 to decide whether the same SI update mechanism is reused [vivo]

- without SI update [Samsung, Nokia]

To conclude, it seems SIB based signaling and L1 based signaling have different suitable use cases. It’s beneficial to consider both of them. So, it’s suggested to further discuss SIB based signaling based on the following proposal.

Moderator proposal #3

Support SIB based signaling for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- FFS details,

  - E.g. whether and how SIB based signaling and L1 based signaling can be supported simultaneously

  - E.g. whether or not to provide SI update notification

Discussion take place on NWM.

### Issue 1-3: Indication content

For the details of information of the availability indication, the following two alternatives were provided in last RAN1 meeting.

- Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources

One remaining issue about the indication content is down-select from alternatives for determining the actual information carried in the L1 based signaling. Companies’ views are summarized as follows:

|  | Motivations | Others |
| --- | --- | --- |
| Alt 1 | - More flexibility There can be multiple active resources or resource sets simultaneously, which provides more TRS occasions and is beneficial to UE power saving in the case of varied SNR condition or UE with different capabilities | - Alt1-1: indicate the availability of TRS resources in a window before an PO to reduce signaling overhead - Huawei, HiSilicon, TCL - Alt1-2: one bit or one code point to indicate all TRS/CSI-RS resources within a cell - CATT |
| Alt2 | - Lower signaling overhead. Alt 2 only indicates identities for configured RS resources that are available, while the RS resources are that are not available can be derived by UE implicitly | - UE still needs to detect PO even when it is not paged, which will degrade the power saving gain - May cause “always-on” signal if the paging DCI is transmitted each PO to indicate the TRS availability information. |

Companies’ views are summarized as follows:

  - Alt1: ZTE, Huawei, HiSilicon, TCL, vivo, Spreadtrum, CATT, CMCC, Qualcomm, OPPO, Intel, Apple, Xiaomi, Sharp, InterDigital, Ericsson (16)

  - Alt2: Sony, Samsung (2)

  - Others:

  - Panasonic: Before agreeing on the signaling details, the maximum number of TRS configurations supported by TRS/CSI-RS occasion(s) should be agreed first. This has direct impact on the detailed L1 signalling design

As the majority supports Alt1, the following proposal based on the majority view is suggested for further discussion.

Moderator proposal #4

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per availability indication to support.

Discussion take place on NWM.

Also, there are many proposals to follow up the FFS point regarding whether and how to indicate the ‘availability’ in beam selective manner. The views are summarized as follows:

  - Alt1: Support to indicate the availability of assistance TRS(s) per beam direction by a bitmap, where each bit corresponds to the assistance TRS(s) that are QCLed with the same associated SSB index

  - Huawei, HiSilicon, Lenovo

  - Alt2: The physical layer indication only indicates availability/unavailability commands for the resources with the same QCL reference

  - TCL, Apple, Sharp, Ericsson

  - Alt3: beam selective manner is not supported

  - Sony

  - MediaTek: introduce some restrictions on UE implementation.

Therefore, it’s suggested to discuss all potential alternatives and down-select if possible.

Discuss whether and how to support L1 based signalling of the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UE in beam selective manner.

- Alt 1: To indicate the availability per beam direction by a bitmap, where each bit corresponds to the assistance TRS(s) that are QCLed with the same associated SSB index

- Alt2: The physical layer indication only indicates availability/unavailability commands for the resources with the same QCL reference

- Alt3: Beam selective manner is not supported.

Discussion take place on NWM.

### Issue 1-4: Validity time

There are many proposals regarding how to determine a validity time for an availability indication, such that a UE can be provided with a time duration in which an availability indication is assumed to be valid, otherwise the UE may assume an availability indication is always valid before the UE receives another or the next availability indication. The main motivations are a) not be need to specifically indicate ‘unavailability’ via physical layer indication, and b) to address scenarios wherein the UE may miss the L1 signaling

The alternatives to support validity time are summarized as follows:

  - Alt1: configured by higher layer, e.g. N paging cycles

  - Huawei, HiSilicon, [Spreadtrum], CMCC, Samsung, DOCOMO, Lenovo

  - Alt2: A window before a PO. e.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO.

  - Huawei, HiSilicon, CMCC

  - Alt3: Included in the availability indication, and multiple validity timer value(s) can be configured as candidates

  - Huawei, HiSilicon, CMCC Sony, Samsung, LG, Panasonic, Nokia, Ericsson

As the majority are positive to support the validity time for L1 signaling based availability indication. The following proposal is suggested for discussion.

Moderator proposal #5

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be determined based on one or more alternatives from the following:

- Alt 1: configured by higher layer

- Alt 2: a window before a PO.

  - E.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO.

- Alt3:  Included in the availability indication.

Discussion take place on NWM.

### Issue 1-5: Application delay

There are some proposals about whether or not to introduce an application delay to determine when an L1 based availability indication can take effect. The views are summarized as follows:

  - Alt1: Need when availability indication indicates change of availability/unavailability status for the relevant RS resources

  - ZTE, TCL

  - Alt1: No need. L1 based availability indication takes effect once received.

  - Vivo, spreadtrum

  - MediaTek: If the validity time is supported, only RS Off-to-On can happen because UE will assume there is no available TRS/CSI-RS at configured occasion(s) after the expiration of validity time. For RS Off-to-On, it can be up to UE implementation to determine the required time for TRS/CSI-RS usage, therefore, the application delay is not needed.

Based on the summary above, no much interests for introducing application delay. Therefore, it’s suggested to consider the following proposal based on the view that no applicable delay is needed.

Moderator proposal #6

A UE assumes a L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs takes effects once received.

Discussion take place on NWM.

## First round discussion summary

### Issue 1-1: L1 based signalling

Issue 1-1.1: whether or not to confirm WA regarding support at least L1 based signaing for the arability indication.

Companies’ views are summarized as follows:

- Confirm: Nordic, Nokia, QC, TCL, CMCC, Sharp, OPPO, Spreadtrum, Intel, Ericsson, Samsung, MediaTek, HW, HiSilicon, DOCOMO, Xiaomi, ZTE, Sanechips, LG, Sony, Vivo, Panasonic, Motorola, Apple (24)

- Not confirm: CATT (1)

Concern #1 (@CATT): We cannot agree to confirm the working assumption until the issues raised being clear without any questions.

Moderator: several companies [Samsung, TCL, Ericsson] have provide comments to resolve CATT’s concerns.

- Regarding “cell reselection issue”, after cell reselection UE can decode paging DCI and paging message in new cell like it does in legacy case. SIB based signaling can be further considered to if relatively longer delay of L1 based signaling is not preferred.

- Regarding “backward compatibility issue”, there is no issue since all UEs decode the same DCI payload. The legacy UEs can ignore the reserved bits of paging PDCCH.

- Regarding “miss-detection”, there is no issue since UE does not assume TRS is available unless it detects an availability indication. False detection is extremely rare due to 24-bit CRC on PDCCH.

- UE might not be able to achieve power saving unless UE could assume TRS being available at next paging occasion, which UE could wake up late with power saving

Moderator: I think you assume some bad implementation or design, which will not be considered. It’s impossible for one design work perfectly for any cases. The group have proposed valid motivations/use cases for both PEI based design and paging PDCCH. Also, SIB-based signaling have been considered by the majority as supplementary to L1 based signaling. The detailed procedure has been discussed step by step in other issues, such as indication content, validity timer, and application delay,

So, the moderator proposal #1 remains same.

Moderator proposal #1

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Issue 1-1.2: selection of L1 based signalling

The following proposal was discussed

Moderator proposal #2

Support at least paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- Whether or not to support PEI based availability indication can be discussed after L1 signal/channel of PEI is confirmed

- FFS enable/disable the feature configurable by SIB

Companies’ views are summarized as follows:

- Support: QC, OPPO, Spreadtrum, Intel, Sharp, Samsung, Ericsson, Xiaomi, Sony, Vivo

- Not support: Nordic, Nokia*, CMCC, CATT, MediaTek, HW*, HiSilicon*, ZTE*, Sanechips*

  - * paging PDCCH based signalling can be supported if PEI is not configured

- Support both paging PDCCH and PEI: Nokia, TCL, LG, Apple

Concern #1: UE behavior regarding paging PDCCH reception when PEI is configured

- Nordic: If UE supports PEI it may not read Paging DCI, therefore TRS indication/validation is in PEI for such a UE

- Nokia: counter intuitive to require the UE to monitor paging DCI for availability indication when PEI is configured.

- CMCC: we have concerns about “always-on” signal if paging DCI is used.

Moderator: The condition when PEI is not configured is added.

Concern #2(CATT): paging PDCCH based signaing doesn’t work

1) gNB needs to wake up all UEs if it would change the resource availability since UE would be at different paging occasions and UE-specific wakeup time

Moderator: it’s not a problem if NW has to wake up all UEs. Same principle has been considered in legacy paging PDCCH, such as for other short message.

2) doesn’t work for Cell reselection.

Moderator: The concern has been addressed before. After cell reselection UE can decode paging DCI and paging message in new cell like it does in legacy case. SIB based signaling can be further considered to if relatively longer delay of L1 based signaling is not preferred.

Concern #3(MediaTek, CMCC, HW, HiSilicon): Prioritize PEI based signaling. If PEI is configured, the availability indication should be signaled by PEI. PEI-based indication can achieve the largest power saving gain without introducing additional signaling overhead.

Moderator: PEI based signalling is added

Concern #4(Panasonic): Propose to firstly agree on the number of TRS configurations in the SIB, which has direct impact on the L1 availability signaling design.

Moderator: the number of TRS configuration will impact the details of indication content or information size to carry. But the selection of L1 signaling methods is still high level. It’s better to determine the high level direction ASAP.

Discussion in email reflector:

Concern #5 (Intel): Still not convinced on the need to provide availability indication in PEI when it can be provided by paging DCI.

Moderator: The main difference for PEI based signalling and paging PDCCH based is when UE is not paged, but gNB needs to transmit the arability information. In this case PEI based siganling has low signalling overhead, as gNB only needs to transmit PEI, while paging PDCCH based signalling requires NW transmit both paging PDCCH and PEI. Because PEI has to indicate paging PDCCH reception

Suggestion #1(Xiaomi): Support at least paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs when PEI is not configured.

Moderator: reasonable

Suggestion #2(Ericsson): While coupling PEI and TRS availability (i.e. in PEI) should be possible, it should not be mandated, i.e. NW can configure whether PEI carries TRS availability indicator.

Moderator: FFS is modified to consider both paging PDCCH and PEI based signalling

There are large supports for both paging PDCCH based solution and PEI based solution. And many companies support both. So, the proposal is updated to capture all most consensus in the group as follows.

Updated proposal #2_v1

if PEI is not configured, support at least paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs; otherwise support PEI based arability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- Whether or not to support PEI based availability indication can be discussed after L1 signal/channel of PEI is confirmed

- FFS enable/disable the feature (i.e. PEI based or paging PDCCH based availability indication) configurable by SIB

Clean version of updated proposal #2_v1

if PEI is not configured, support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs; otherwise support PEI based arability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- FFS enable/disable the feature (i.e. PEI based or paging PDCCH based availability indication) configurable by SIB

Intel: it’s always possible and the OH difference/UE power consumption would not really have any significant impact in a practical system. It is simply very unlikely that the TRS availability information keeps changing every paging cycle. Thus, we are not convinced that a second solution is needed to optimize for the PEI case when we would anyway need a solution using paging DCI.

Updated proposal #2_v2

If PEI is not configured, support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs when PEI is not configured; otherwise support PEI based arability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- FFS whether and how to enable/disable L1 the feature (i.e. PEI based or paging PDCCH based availability indication) configurable by SIB

- FFS L1 based availability indication when PEI is configured

### Issue 1-2: SIB based signalling

The following proposal was provided by moderator for 1st round discussion.

Moderator proposal #3

Support SIB based signaling for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- FFS details,

  - E.g. whether and how SIB based signaling and L1 based signaling can be supported simultaneously

  - E.g. whether or not to provide SI update notification

Companies’ views are summarized as follows:

- Support: Nordic, Nokia, QC, TCL, Intel, CATT, Samsung, Sony, Panasonic, Motorola, Apple (11)

- Not support: OPPO, MediaTek, Ericsson, HW, HiSilicon, ZTE, Sanechips, DOCOMO (8)

Concern #1 (CMCC, MediaTek, Ericssion, ZTE, DOCOMO): Does the second bullet conflicts with “It is RAN1 understanding that existing SI update procedure is used for SIB based signalling”

Moderator: the second bullet is only necessary when both SIB based signaling and L1 based signaling are supported. The proposal is updated to clarify that based on suggestion from Xiaomi

Concern #2 (LG, Sony): be okay to consider the SIB-based signaling if we take it as an additional mechanism and if the benefit is justified

Moderator: yes, it can be considered as additional mechanism by NW. The motivations are summarized in section 2.1. There are many use cases for SIB-based signaling, such as default mode, cell re-selection, or infrequent update.

Concern #3 (HW): it is not clear regarding the terminology of “SIB-based signaling for availability indication of TRS/CSI-RS occasions”

Moderator: details are up to RAN2 decision. Two notes are added to clarify RAN1 understanding about how it works

Suggestion #1: Clarify UE is not required to decode the SIB all the time

- Nordic: should we also specify that UE does not expect availability configuration to be updated more than once an X hours. One could of course rely on good gNB behaviour, but ... :)

- Nokia: it could be also noted that RAN1 expectation is that the availability indication would be more static so that negative implications of frequent SI updates can be avoided

- QC: to clarify that the UE is not required to decode the SIB all the time on all SIB occasions to read the latest TRS availability indication

Moderator: a note is added as suggested by Nokia.

Suggestion #2(Nokia): Clarify details are up to RAN2.

Moderator: Added. But companies can further check whether the details including both SIB_x to use and information carried are up to RAN2

Suggestion #3: Clarify the FFS

- Intel: change the first sub-bullet to ”how” instead of ”whether and how”

- Xiaomi): add note “It is RAN1 understanding that existing SI update notification is not used for availability indication of TRS/CSI-RS occasions for idle/inactive UEs when SIB based signaling and L1 based signaling can be supported simultaneously”.

Moderator: replaced the old FFS by the note suggested by Xiaomi

HW: Also regarding the SIB based signaling, we have commented in the NWM. It is not clear on how to understand SIB based signaling. It is one explicit bit in SIB to indicate the TRS availability, or just the assumption that if SIB configures assistance TRS occasions the UE can assume the TRS is available if L1 signalling is not configured to be used. This is critical for us to agree the proposal #3_2, otherwise we don’t know what we will agree on

Moderator: the details of SIB based signaling are up to RAN2. In RAN1 perspective, we propose this based on the valid motivations that it’s beneficial for many use cases as discussed before, a) cell reselection, b) infrequent update, c) default mode. We can continue FFS any RAN1 impact if any. But, till now, the majority think all details are up to RAN2 design.

Proposal is updated as follows.

Updated proposal #3_v1

Support SIB based signaling for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- FFS details, E.g. whether and how SIB based signaling and L1 based signaling can be supported simultaneously

- It is RAN1 understanding that existing SI update notification is not used for availability indication of TRS/CSI-RS occasions for idle/inactive UEs when SIB based signaling and L1 based signaling can be supported simultaneously.

  - E.g. whether or not to provide SI update notification

- It is RAN1 expectation that the availability indication would be more static if only SIB based signaling is enabled, so that negative implications of frequent SI updates can be avoided

- Details (e..g. SIB_X to use and information to carry) are up to RAN2

Concern #4(Apple, Intel): it seems not correct to say that the gNB should never change it. For longer term changes, the gNB can still modify the SIB based signaling.

Moderator: The original intention is gNB doesn’t need to trigger SI update procedure by sending SI update notification in L1 for the availability indication specifically if SIB based signaling and L1 based signaling are supported simultaneously to avoid unnecessary UE power consumptions when the UE can get the availability update by L1 based signaling. But it’s true gNB can still modify the SIB based signaling with SI update procedure, for example, if SIB based signaling indicates different information, e.g. default mode, from L1 based signaling.

The proposal is further updated based on suggestion from Apple

Updated proposal #3_v2

Support SIB based signaling for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- FFS details, E.g. whether and how SIB based signaling and L1 based signaling can be supported simultaneously

- It is RAN1 understanding that existing SI update notification is not used for availability indication of TRS/CSI-RS occasions for idle/inactive UEs should be avoided as much as possible when SIB based signaling and L1 based signaling can be supported simultaneously.

  - E.g. whether or not to provide SI update notification

- It is RAN1 expectation that the availability indication would be more static if only SIB based signaling is enabled, so that negative implications of frequent SI updates can be avoided

- Details (e..g. SIB_X to use and information to carry) are up to RAN2

Clean version of updated proposal #3_v2

Support SIB based signaling for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- It is RAN1 understanding that existing SI update notification for availability indication of TRS/CSI-RS occasions for idle/inactive UEs should be avoided as much as possible when SIB based signaling and L1 based signaling can be supported simultaneously.

- It is RAN1 expectation that the availability indication would be more static if only SIB based signaling is enabled, so that negative implications of frequent SI updates can be avoided

- Details (e..g. SIB_X to use and information to carry) are up to RAN2

### Issue 1-3: Indication content

The following proposal was provided by moderator for 1st round discussion.

Moderator proposal #4

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per availability indication to support.

Companies’ views are summarized as follows:

Support: Nordic, Nokia, TCL, CMCC, OPPO, LG, MediaTek, Ericsson, HW, HiSilicon, Xiaomi, ZTE, Sanechips, Sony, Vivo, Motorola, Apple (17)

Have concern: QC, CATT, Samsung, Panasonic

Concern#1(QC, CATT): to clarify ”some” of configured RS resources, not OK with some

Moderator: It’s originally suggested by HW to address the use case when availability indication is provided per UE group associated with a PO, so the bitmap is only for partial resources before a PO not all broadcasted resources. Since we haven’t discussed whether or not to support some or all. It’s removed to FFS.

Concern#2 (Samsung): We are not fully convinced by the benefit of bitmap based indication. The flexibility relies on bitmap size.

Moderator: For the sake of progress, Samsung is fine to make the compromise.

Question #1 (LG): We would like to clarify the meaning of ‘per availability indication’ in the last sub-bullet; whether it means ‘per signal/channel’ or ‘per bit in a bitmap/codepoint’

Moderator: my understanding it’s per signal/channel, i.e. the bitmap size, which impact the L1 signaling design. The motivation to add the FFS point is to address Panasonic as follow

  - Panasonic: Before agreeing on the signaling details, the maximum number of TRS configurations supported by TRS/CSI-RS occasion(s) should be agreed first. This has direct impact on the detailed L1 signalling design.

It’s clarified in the updated proposal.

Suggestion#1(Intel): e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

Moderator: I think we already discussed the wording in last meeting. It’s better keep the original wording. It should be fine as it’s just examples.

Suggestion#2(Panasonic): the FFS bullet should be settled first and then we can compare alternative 1&2 in terms of flexibility and overhead. If even a maximum number is still within a certain acceptable level, we are okay for sake of progress.

Moderator: thanks for the compromise.

The Moderator proposal #4 is updated as follows:

Updated proposal #4_v1

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Clean version of updated proposal #4_v1

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

We also discussed the following issue

Discuss whether and how to support L1 based signalling of the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UE in beam selective manner.

- Alt 1: To indicate the availability per beam direction by a bitmap, where each bit corresponds to the assistance TRS(s) that are QCLed with the same associated SSB index

- Alt2: The physical layer indication only indicates availability/unavailability commands for the resources with the same QCL reference

- Alt3: Beam selective manner is not supported.

Moderator: Companies’ views are quite divergent. Will be revised it after more progress on Moderator proposal #4-v1 in next round.

### Issue 1-4: Validity time

The following proposal was provided by moderator for 1st round discussion.

Moderator proposal #5

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be determined based on one or more alternatives from the following:

- Alt 1: configured by higher layer

- Alt 2: a window before a PO.

  - E.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO.

- Alt3:  Included in the availability indication.

Companies’ views collected during the 1st round discussion are summarized as follows:

For Alt1

- OK to consider/support: Nokia, QC, Spreadtrum, Intel, Sharp, LG, MediaTek, Huawei, HiSilicon, DOCOMO, Ericsson, Samsung, ZTE, Sanechips, Vivo, Motorola, Panasonic, Apple

- Object: CATT

For Alt2

- OK to consider/support: Nordic, CMCC, OPPO, LG, MediaTek, Huawei, HiSilicon, DOCOMO, Motorola, Panasonic, Apple

-Object: Nokia, QC, Intel, CATT

For Alt3:

- OK to consider/support: Nokia, QC, TCL, LG, MediaTek, DOCOMO, Ericsson, Samsung, ZTE, Sanechips, Sony, Panasonic, Apple

- Object: CATT, Huawei, HiSilicon

Concerns for different alternatives

| Alt1 | Alt2 | Alt3 | Others |
| --- | --- | --- | --- |
| CATT: Since paging occasion and paging duration is way longer than SI cycle, different UE might retrieve SI at different time. Since system information is same at different broadcast time, UE would have different understanding on the TRS availability duration when different UE retrieves SI at different time. Thus, Alt 1 would NOT work. | Nokia, Ericsson: we have not agreed to introduce these TRS occasions specifically only for paging, but could be used in more generic manner to maintain the synchronization to the serving cell, we don’t see such restriction necessary | Intel: Alt 3 cannot work by itself since one of the reasons for defining a validity timer is to address scenarios wherein the UE may miss the L1 signaling. So, at least 22 Alt 1 is necessary. | Vivo: we prefer that UE can assume the TRS available before receiving another new L1 availability indication. The benifits such as less signalling overhead. As a compromise, we are fine with Alt 1, if the time duration can be configured as infinite. I |
|  | QC, Intel: TRS is a connected mode UE’s TRS. Transmission of this TRS cannot be aligned with PO. Reply by Apple: We do not think Alt 2 means the alignment of PO and TRS. It just means that the TRS within a window before each PO is available. E.g. if the paging cycle is 1.28 sec, the UE assumes that the TRS within [60] ms window before the PO is available. The motivation is that the UE would only use the TRS before the PO reception, so this is sufficient for the UE. From the network perspective, it provides the network slightly more flexibility. Moderator: it seems Alt2 is supported mainly for PEI based availability indication so the time alignment can be reused. | HW: can increase the signaling overhead of L1 based availability indication since it requires more bits for duration indication |  |
|  | CATT: it is not clear whether the window period is associated with one paging occasion or multiple paging occasions. Since the paging rate is usually low (e.g., 10%), UE could not use most of TRS resource if the window of TRS availability is for one PO. If the window is for more than one PO (e.g., 10 POs), UE might not get to use TRS when paging message arrive more than 10 POs. Thus, Alt 2 window duration is not useful to ensure UE power saving with TRS.l |  |  |
|  | ZTE, Sanechips: it seems a more frequent indication before each PO is required. |  |  |

Suggestions/Clarification

Intel: we need to discuss further on UE behavior upon expiry of the timer - i.e., what is the default assumption

CATT: Alt 3 - Duration of TRS availability is implicit indicated by availability signaling without additional timer or duration constraint.

LG: add a note that combination of alternatives are not precluded. For example, timer value can be configured via higher layer while actual timer value can be indicated by L1 based signaling.

Vivo: Alt 1: configured by higher layer, including configured as ‘infinite’

Panasonic: It is proposed to revise the main bullet as “L1 based availability/unavailability indication of …” Also in the Alt3, it should be “availability/unavailability indication” Moderator: “L1 based availability indication” is used in the WA from last meeting. It should be fine to reuse the same wording.

Moderator: Views are not converging yet. Can be deprioritized in the first round of discussion. Will revisit in next round.

### Issue 1-5: Application delay

The following proposal was provided by moderator for 1st round discussion.

Moderator proposal #6

A UE assumes a L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs takes effects once received.

Companies’ views are summarized as follows:

- Support (no need for application dealy): Nokia, CMCC, spreadtrum, Intel, CATT, Ericsson, Samsung, Xiaomi, Vivo

- Not support (need application delay): Nordic, QC, TCL, LG, ZTE, Sanechips

- Others:

  - MediaTek: If the validity time is supported, then we think the application delay is not needed

  - Panasonic: This can be covered by the discussion of validity time in above

Moderator: Views are not converging yet. Will revisit in next round together with Issue 1-4.

## Second round discussion

### Issue 1-1: L1 based signalling

Regarding the selection of L1 based signaling of availability indication, there are two alternatives, i.e. paging PDCCH based availability indication and PEI based availability indication.

For paging PDCCH based indication, there is a consensus that it is needed at least for the case when PEI is not configured. Because the entire feature of idle/inactive mode TRS/CSI-RS occasions should be able to supported independently regardless PEI is supported or not.

The only debate is about the case when PEI is configured, the views didn’t converge yet.

On the one hand, several companies [Apple, HW, ZTE, LG, CMCC, Nokia, Interdigital, Nordic] proposed to support PEI based availability indication, due to benefits of saved UE energy and low resources overhead.

On the other hand, some companies still have concerns about PEI based signaling, including

- Paging PDCCH based availability indication is still workable when PEI is configured, and the performance difference of PEI based indication paging PDCCH based indication is not significant, considering that NW is not mandated to transmit L1 availability indication all the time. So, they don’t agree to support a second L1 based signaling for minor optimization.

  - Intel, QC, CATT

- So far there is no clear design of PEI, e.g., how PEI is conveyed, using sequences or DCI, using which DCI, it’s better to wait for the process of PEI design and then decide whether and how to use PEI for RS indication.

  - OPPO, Samsung

Since no company provide any evaluation results in this meeting, it’s hard to reach consensus for the case when PEI is configured. Thus, it’s suggested to focus on the case when PEI is not first, and FFS the case when PEI is configured.

Updated proposal #2_v4

At least for the case when PEI is not configured, support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

- FFS the case when PEI is configured

  - Performance comparison between PEI based availability indication and paging PDCCH based availability indication, e.g. power saving gain

After 2nd GTW on 05/24

The following proposal is suggested by Apple, CATT, HW, ZTE, LG, CMCC, Nokia, Interdigital, Nordic. We can check it first.

Option 1: (updated proposal #2_v3_CATT)

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

If there is still no consensus, I would suggest to discuss the two alternatives separately in two proposals, and discuss them separately to make progress step by step.

Option 2: (Split into two proposals)

Proposal #2-1

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least for the case when PEI is not configured,

- FFS whether and how to enable/disable paging PDCCH based availability indication configurable by SIB

Proposal #2-2

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable PEI based availability indication configurable by SIB

Discussion take place on NWM.

### Issue 1-2: SIB based signaling

Regarding SIB based signalling, only Ericsson (1) company objects to support SIB based availability indication. The majority are fine to support it as as complementary mechanism in addition to L1 based signalling as it can addressee different use cases than L1 based signalling and has the benefits of avoiding unnecessary L1 based signalling overhead.

For the detailed design, the following examples are provided during the discussion, and can be divided into two cases: a) only SIB based signalling is supported, and b) SIB based signalling and L1 based signalling are supported simultaneously:

Case #1: For SIB based signalling only

- Alt 1: The presence/absence of SIB-X of TRS configuration can be used as explicit indication, corresponding to available/unavailable indication

  - at least Apple, CATT, Nokia, Nordic

Case #2: SIB based signalling and L1 based signalling are supported simultaneously:

- Alt 2-1: there could be separate indication for availability in SIB, other than the TRS configurations. E.g. with 10 TRS configurations, the SIB can indicate 3 of them are available, and the availability of the remaining 7 can be further dynamically indicated by L1 signaling. What is indicated as available in SIB are always assumed to be available by the UE.

  - Apple

- Alt 2-2: a SIB-based signaling indicate a default availability assumption, UE assumes the default assumption if not receives any L1 based availability indication., otherwise use L1 based availability indication.

  - Intel

- Alt 2-3: same availability indication can be carried in both SIB based signalling and L1 based signalling at different transmission time. UE in different use cases can choose to monitor L1 based signalling or SIB based signalling by UE implementation. No need to provide SI update notification in paging PDCCH.

  - Samsung

Updated proposal #3_v5

Support SIB based signaling for availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

- Note: It is RAN1 understanding that SIB based availability information can be used when the availability would be more static, so that frequent SI updates can be avoided. When frequent changes in availability are needed, RAN1 assumption is that L1 based signaling can be used.

- Details (e.g. SIB_X to use and information to carry) are up to RAN2 when only SIB based signaling for availability indication is supported.

- FFS whether and how SIB based signaling and L1 based signaling can be supported simultaneously.

  - E.g. Information to carry in SIB_X

After 2nd GTW on 05/24

Based on the online GTW discussion and guidance from Chairman, the proposal is updated to address the concern regarding the motivations and details of all possible alternatives.

Firstly, although L1 based signalling could be applicable for both frequent and infrequent update of the availability state, SIB based signalling is beneficial than L1 based signalling at least for the case when the availability state is more static. And, it can be supported without explicit bits in SIB by reusing the configuration in SIB_X. No any cost to at least support this alternative as specified in the main text.

Furthermore, a combination of L1 based signaling and SIB based signaling provides signaling flexibility to NW to cover more uses cases. But more discussion is needed. So, all alternatives are provided under FFS.

Based on the justification above, the proposal is updated as follows.

Updated proposal #3_v6

Support SIB based signaling for availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X if only SIB based signaling is supported.

- FFS whether and how SIB based signaling and L1 based signaling can be supported simultaneously based on following alternatives:

  - Alt1: SIB based signaling provides availability indication for a subset of all configured TRS/CSI-RS occasions, and L1 based signaling provide availability indication the remaining configured TRS/CSI-RS occasions.

  - Alt2: SIB based signaling provides availability indication for a default assumption of the availability information for all configured TRS/CSI-RS occasions, and L1 based signaling provide updates relatively to the default assumption.

  - Alt3: Both SIB based signaling and L1 based signaling provide the same availability information but transmitted in different time according to configured monitoring occasions. No SI update notification in paging PDCCCH for any update of the availability information in SIB.

  - Other alternatives are not precluded.

Discussion take place on NWM.

## Second round discussion summary

### Issue 1-1: L1 based signalling

Companies’ views on option 1 are summarized below:

- Support: TCL, Huawei, HiSilicon, Nokia, PANASONIC, ZTE, Sanechips, LG, MediaTek, InterDigital, Samsung, CATT, QC, Ericsson, DOCOMO, Apple

- Strong concern: Intel

- Fine/prefer to FFS PEI based signalling: Intel, Samsung, QC, CATT

Suggestions/clarification:

| Nokia/Ericsson | consider following modification: - “for idle/inactive UEs if PDCCH-based PEI is down selected” to “for idle/inactive UEs at least if PDCCH-based PEI is down selected” |
| --- | --- |
| CATT | We would like to clarify that SSS-based PEI or TRS-based PEI could be considered for availability indication if it is down selected. |
| Samsung | If it’s RS based PEI, we think the RS resources of PEI can be used directly for synchronization, no need for other RS resources. |
| Intel | we can accept either Options 1 or 2 but only with FFS for indication via PEI, and come back to it when we have better idea on the benefits |

The proposal is updated based on the majority view with minor change as suggested by Nokia. I think we have done comprehensive discussion. I will suggest #2_v3 for GTW handling, and also provided original #2_v4 as backup. At least one should be agreeable.

Updated proposal #2_v3

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

(backup)Updated proposal #2_v4

At least for the case when PEI is not configured, support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

- FFS the case when PEI is configured

  - Performance comparison between PEI based availability indication and paging PDCCH based availability indication, e.g. power saving gain

### Issue 1-2: SIB based signaling

Companies are required to provide views for Updated proposal #3_v6 using NWM. Whether or not to support? Any modification?

Companies’ views on option 1 are summarized below:

- Support: TCL, OPPO, Huawei, HiSilicon, Nokia, PANASONIC, Xiaomi, InterDigital, Samsung, CATT, Intel

- Object: Ericsson

| Suggestions/concerns | Reply |
| --- | --- |
| OPPO:”SIB signaling...based on the presence/absence...” is confusing for me, does it mean that when the configuration is present, the TRS is available? Otherwise, it is not available? If yes, we cannot accept it, because the configuration anyway shall be there if L1 signaling methods is used. | Moderator: the answer to your question is “yes”. But there is a condition, the L1 based signaling is not configured. This is address the use case when NW expects static availability information. |
| OPPO: we propose to remove all the sub-bullet and try to agree the main bullet first. After we accept the SIB mechanism, we can further discuss the details. Of course, we don’t object to use these alternatives as examples. | Moderator: the alternatives are requested by HW FFS on whether and how should be fine. |
| ZTE: For all the alternatives listed in the bullets, UE needs to detect both SI update for SIB based indication and L1 signaling, we think they are neither beneficial to resource reduction or UE power saving. | Moderator: For Alt1 and Alt2, SIB based indication is only used for updating the arability indication for static resources, e.g. default assumption, so SI update procedure can be reused, while L1 based signaling can be used to update more frequently changed resources. No increase on UE power consumption. For resource overhead, they provide more flexibly for NW, and allows tradeoff to use L1 based signaling and SIB based signaling. Compared with L1 based signaling only, the combined solutions have benefits of low resource overhead. |
| OPPO: the current system information update mechanism shall be reused | Moderator: that’s the case when only SIB based availability indication is supported. But, system information update mechanism may not be necessary for a combination of SIB based indication and L1 based indication as proposed in Alt3. |
| HW, ZTE, DOCOMO: We have some further comments regarding the main bullet, it mainly covers the case when L1 signalling is disabled. Therefore, it is related with the FFS bullet of proposal #2, i.e. ‘whether and how to enable/disable 31 L1 based indication configured by SIB’. So we suggest that we first settle down the FFS bullet in proposal #2, then we can discuss to agree the main bullet of proposal 3. | Moderator: The main text seems to be more stable than FFS, it’s better to reach consensus on the main text first. They can be discussed in parallel. |
| HW, ZTE, Senechips, MediaTek, Ericsson, DOCOMO: SIB based signalling and L1 based signalling seem duplicated. The benefit to introduce SIB based signalling only seems pretty weak. the SIB based solution to dedicated availability indication seems requires more resources and UE energy. SIB based signalling and L1 based signalling are duplicated | Moderator: SIB based signaling is better than L1 based signaling at least for more static availability indication. CATT: The SIB-based signaling is used when the availability of TRS configured for IDLE/Inactive UE would not change frequently to achieve UE power saving. Most companies showed the power saving gain with TRS did not assume TRS availability changing frequently. Using our evaluation assumption of paging cycle 1.28 seconds and 10% paging rate, the TRS availability should be at least 10.28 seconds in average in order to achieve the power saving gain from TRS. Otherwise, UE needs to wake up early. Regarding the concern of signaling overhead by some companies, it is a general paging procedure for any system information update. We didn’t agree with ZTE’s analysis of number of DCI decoding, which is not aligned with current system operation |
| Nokia: it would be better to discuss about ‘availability information’ for SIB, rather than ‘availability indication’, to separate more clearly that the intent is not to have dynamic, but more static availability configuration. | Moderator: OK |
| Nokia, Xiaomi: we would suggest adjust the first bullet as follows: - “Support SIB based availability information for TRS occasions for idle/inactive UES, in case L1 based availability indication is not configured” | Moderator: OK |
| Nokia: For example/Alt1, where SIB1 based availability is considered, it would be maybe better talk about availability information to separate the dynamic L1 availability indication. E.g.: “Alt1: SIB based availability information is used for a subset of configured TRS occasions …” | Moderator: OK |
| Nokia: For example/Alt2, it is not fully clear for me how the default in Alt2 would operate, i.e. after reading SIB, UE#1 might have different understanding than UE#2 which has received the L1 indication. CATT: We would like to clarify if this proposal is simply L1 signaling over-write when L1 signaling is enabled. Ericsson: This seems to imply that default can be ‘UE assumes always ON TRS signaling’, while NW must send even more L1 signaling to turn it OFF. This goes against the Note in the WID and NR principle of minimizing always ON signals. There is a default assumption– UE does not assume TRS is available unless it is informed otherwise via L1 based availability. | Moderator: @Intel, can you provide more insights and clarify? Intel: To Nokia’s question on Alt 2, this is typical operation and NW would have to ensure that the information is consistent across UEs - it’s exactly the same situation as w/o SIB indication - NW again has to ensure that both UEs are provided with the same indication via L1 signaling. Nothing changes in that regard. |
| Nokia: If I understand the alternative correctly this would require UE to acquire the SIB (SIB-x) in question frequently, and this would have negative impact to the UE power consumption. Or is it assumed that SIB-x acquisition is done only when so defined by 38.331 (e.g. after 3hrs/cell reselection etc.)? | Moderator: for Alt3, UE doesn’t need to receive SIB-x frequently due to the change of availability information. gNB transmits the consistent availability info in both SIB and L1 based signaling. Since gNB already transmit the availability info explicitly in L1 based signaling, no need to trigger SI update procedure specifically for availability information update. |
| MediaTek, Ericsson: we are not sure whether ”no SI update notification” of Alt 3 conflicts with the confirmed working assumption or not | Moderator: the note is under FFS bullet. Things are still open. The SI update notification is reused for some cases, e.g. the main text. |
| For Alt1: CATT: We would like to clarify how many TRS resources are configured for each beam and the additional power saving achieved by more than on TRS resource in this assumption. Ericsson: It is not clear why this kind of hybrid scheme is needed as L1-based availability can address such scenario. | Moderator: is this really necessary at this stage? [Apple] “Alt 1: on CATT's question regarding how many TRS configuration per beam, we also tend to think one configuration per beam might be sufficient already, and we are open to discuss further. But this question applies to all solutions, regardless of how availability indication is provided. So we do not think this affects the description of Alt 1 here.” [Apple] I don’t feel this is a comment specifically for Alt 1. My feeling is that this is basically why Ericsson is not in favor of any kind of SIB-based signaling. But if we just talk about Alt 1, and if validity timer is adopted in the end, this hybrid approach can be very useful, so that UE knows which TRS is available even after the validity timer expires. |
| CATT: Alt 3: We have concern on how this would work. UE does not read the SIB unless System information update by paging or move to new cell after cell reselection. The TRS configuration in SIB-X does not have time stamp. The UE assumption on the TRS availability would be ambiguous in Alt 3. | Moderator: validity timer relative to the time when the availability indication is provided can be considered. NW can make sure the availability information is consistent. |
| Intel: Does it intend to say, if ONLY SIB based indication is configured? If yes, then the FFS bullet should also be updated to say ”configured”? | Moderator: you are right. |

Based on the summary, the proposal is updated as follows,

Updated proposal #3_v6

Support SIB based signaling for availability information indication of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X if only SIB based signaling is supported in case L1 based availability indication is not configured.

- FFS whether and how SIB based signaling and L1 based signaling can be configured supported simultaneously based on following alternatives:

  - Alt1: SIB based signaling provides availability information indication is used for a subset of all configured TRS/CSI-RS occasions, and L1 based signaling provide availability indication the remaining configured TRS/CSI-RS occasions.

  - Alt2: SIB based signaling provides availability indication for a default assumption of the availability information for all configured TRS/CSI-RS occasions, and L1 based signaling if provided over-writes provide updates relatively to the default assumption.

  - Alt3: Both SIB based signaling and L1 based signaling provide the same availability information but transmitted in different time according to configured monitoring occasions. No SI update notification in paging PDCCCH for any update of the availability information in SIB.

  - Other alternatives are not precluded.

## Third round discussion

### Issue 1-4: Validity time

As there are quite a number of supports for all three alternatives, it’s suggested to list all of them and do down-selection in the future.

The proposal is updated as follows to address the concerns and suggestions based on summary in Section 2.2-Issue 1-4.

Proposal #5_v1 L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be determined based on one or more alternatives from the following: - Alt 1: Configured by higher layer - FFS values for the validity time, including ‘infinite’ - Alt 2: A window before a PO for PEI based signaling if supported - E.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO. - Alt3: Included in the availability indication - FFS explicit/implicit indication by availability signaling with/without additional timer or duration constraint. - A combination of alternatives is not precluded. - E.g. candidates of validity time can be configured via higher layer while actual validity time can be indicated by L1 based signaling. - FFS UE behavior upon expiry of the timer - E.g. whether or not to introduce a default assumption

## Third round discussion summary

For all the companies submitted views, all are fine to support. Please check the details comments and reply as below.

For all the companies submitted views, all are fine to support. Please check the details comments and reply as below.

| Comments | Reply |
| --- | --- |
| Apple: I would like to clarify if this means that we will support at least one of the alternatives | Moderator: Yes. |
| Apple, HW, OPPO: Alt2: we think ”for PEI-based signaling if supported” should be removed. Even for paging PDCCH based signaling, the same window can be defined before a PO. | Moderator: it is added to address concern during the first round discussion as follows: QC, Intel: TRS is a connected mode UE’s TRS. Transmission of this TRS cannot be aligned with PO. It’s removed temporarily. @QC, Intel, please check the reply from Apple/HW/OPP, to see if you still have concern. |
| Apple, TCL, LG: Alt 3: what does ”with/without additional timer or duration constraint” mean | Moderator: additional timer or duration constraint is needed for explicit indication carried by the L1 the availability indication But CATT proposes the following: CATT: Duration of TRS availability is implicit indicated by availability signaling without additional timer or duration constraint. |
| Alt1: LG, Ericsson: One minor concern is the ’infinite’ values for the validity time in the Alt 1 seems to conflict with the current spec. Although I may understand the intention of it, according to the 38.331 spec, the UE shall ”delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid”. | Moderator: it’s suggested by Vivo as follows: Vivo: Alt 1: configured by higher layer, including configured as ‘infinite’. @Vivo, can you follow up with this concern? It’s removed temporarily. |
| For the last FFS bullet, HW: We have concern on the FFS bullet. There is ongoing discussion on proposal 3 regarding whether the default assumption configuration is needed. Therefore we should remove the bullet of FFS before the Alt.2 in Proposal 3 is agreed. ZTE: (1)We are not sure what the timer is used for in this bullet, and also the relationship with other alternatives. (2) We think it should be clear as we are discussing about “validity time”, UE should assume the TRS occasions are invalid when the timer (if any) expires. OPPO: the last FFS bullet is under the FFS bullet in alt DOCOMO, Ericsson: FFS is not needed, Regarding default assumption, it is under discussion in the proposal 3. | Moderator: OK. |
| Ericson: Even when TRS is actually available (indicated by L1 signaling + validity time), Alt 2 precludes UE from using some of available TRS. Can proponents of Alt 2 clarify why this is OK or necessary? | @ Apple, HW, OPPO: can you clarify? |

The proposal is further updated based on the justification above.

Proposal #5_v2

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be determined based on at least one or more alternatives from the following:

- Alt 1: Configured by higher layer

  - FFS values for the validity time, including ‘infinite’

- Alt 2: A window before a PO for PEI based signaling if supported

  - E.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO.

- Alt3:  Included in the availability indication

  - FFS explicit/implicit indication by availability signaling with/without additional timer or duration constraint.

- A combination of alternatives is not precluded.

  - E.g. candidates of validity time can be configured via higher layer while actual validity time can be indicated by L1 based signaling.

- FFS UE behavior upon expiry of the timer

  - E.g. whether or not to introduce a default assumption

Proposal #5_v3

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be determined based on at least one or more alternatives from the following:

- Alt 1: Configured by higher layer

  - FFS values for the validity time, including ‘infinite’

- Alt 2: A window before a PO for PEI based signaling if supported

  - E.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO.

- Alt3:  Included in the availability indication

  - FFS explicit /implicit indication by availability signaling with/without additional timer or duration constraint.

- A combination of alternatives is not precluded.

  - E.g. candidates of validity time can be configured via higher layer while actual validity time can be indicated by L1 based signaling.

- FFS UE behavior upon expiry of the timer

  - E.g. whether or not to introduce a default assumption

Proposal #5_v4

L1 based availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is valid for a time duration that can be determined based on at least one alternative from the following:

- Alt 1: Configured by higher layer

  - FFS values for the validity time, ‘infinity’ is not precluded for study

- Alt 2: A window before a PO

  - E.g. for PEI based signaling if supported

  - E.g. Time duration between the end of L1 signal/channel carries the availability indication and the start of an associated PO.

- Alt3:  Included in the availability indication

  - FFS explicitly indicate one value among multiple options or implicitly derived from the period of the L1 based availability indication

- A combination of alternatives is not precluded.

  - E.g. candidates of validity time can be configured via higher layer while actual validity time can be indicated by L1 based signaling.

# Topic #2. Configuration Details

According to the following agreements from RAN1#104e meeting, it has been agreed to support configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include at least powerControlOffsetSS, scramblingID, firstOFDMSymbolInTimeDomain, startingRB, and nrofRBs, But the corresponding applicable values have no been confirmed yet. Based on the agreement from last meeting, we only need to support the configuration parameters and corresponding applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs. Furthermore, how to configure the QCL information for multiple RS resources for TRS/CSI-RS occasion(s) for idle/inactive UEs are open for FFS.

In RAN1#104-e: Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values In RAN1#104b-e Agreement: Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details of the QCL information, e.g. associated SSB index Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration

The following proposals regarding details of configuration are submitted in RAN1#105-e:

| ZTE [1] | Proposal 6: The QCL information of TRS for idle/inactive UE is associated with SSB. Proposal 7: Reducing signaling overhead should be taken into consideration when configuring TRS related parameters. parameters; Applicable values. powerControlOffsetSS /dB; Ex. {-3, 0, 3, 6}. scramblingID; Ex. 0 to 1023. firstOFDMSymbolInTimeDomain; Ex. 0 to 9. startingRB; Ex. 0 to 274. nrofRBs; INTEGER (24..maxNrofPhysicalResourceBlocksPlus1). |
| --- | --- |
| Huawei, HiSilicon[2] | - For the configurable parameters in Table 2, the value range for CONNECTED mode can be used as the baseline. - gNB configures the associated SSB index as the QCL information. - Further discuss the TRS occasion in slot-level granularity to be configured relative to PO. - The following way can be used to reduce signaling overhead for the TRS resources within a resource set - Option 1: The parameters with the same value can be configured only once instead of multiple times. - Option 2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one. Table 2 The related parameters of TRS for IDLE/INACTIVE mode UEs Parameter; Note. powerControlOffsetSS; Configured by gNB. scramblingID; Configured by gNB. firstOFDMSymbolInTimeDomain; Configured by gNB. startingRB; Configured by gNB, not restricted by initial BWP. nrofRBs; Configured by gNB, not restricted by initial BWP. QCL; Configured by gNB. SCS; Not configured, the same as the SCS of CORESET #0. Trs-info; Not configured, implicitly assumed to be configured. frequencyDomainAllocation {row1, row2, row4, others}; Not configured, it is row1 by default.. nrofPorts; Not configured, single port is assumed.. cdm-Type; Not configured, single port is assumed.. periodicityAndOffset; FFS. |
| TCL Communication Ltd[3] | Proposal 5: Include the configuration of IndicationCycle in the DownlinkConfigCommonSIB, similar to the paging cycle, to reduce the availability indication overhead. Proposal 6: Availability of a set/group of multiple TRS/CSI-RS can be indicated in a paging cycle to the UE or group of UE for the next paging cycle, which may reduce the availability indication overhead. Proposal 7: Study the configuration of multiple TRS/CSI-RS SMTC window to reduce the configuration overhead. |
| Vivo[4] | Proposal 1: TRS resources configured for idle/inactive UEs can be type-C or type-D qualsi co-located with SSB, and the QCL information of the TRS resource can be optionally configured. |
| Spreadtrum Communications[5] |  |
| CATT[6] | Proposal 1: TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit. Proposal 2: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s) to achieve good power saving gain with low SIB signaling overhead. Proposal 3: QCL information configuration of TRS for idle/inactive UE should be configured per CSI-RS resource set. Proposal 4: QCL information of TRS/CSI-RS occasion(s) should be explicitly indicated to be associated with SSB ID by SIB-X for IDLE/Inactive UEs. Proposal 5: The following procedure can be used for TRS/CSI-RS occasion(s) configuration: Step1) predefined parameters of TRS/CSI-RS resource grid; Step 2) SIB indicate parameters details; including - QCL assumption of the configured TRS/CSI-RS resources associated with a SSB; - Code points mapping of availability/not availability for a given TRS resources. Step 3）To derive TRS occasion(s) according to predefined rule and parameters provided by step1 and step 2. |
| CMCC[7] |  |
| Qualcomm Incorporated[8] | Proposal 1: Clarify whether value range of the configuration parameter for TRS can be extended when the TRS is configured to idle/inactive mode UEs. Proposal 2: Reuse Rel-16 QCL rule with SSB as the QCL source for periodic TRS configured to inactive/idle UEs. |
| OPPO[9] | Proposal 4: QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is derived from higher layer configuration, e.g., qcl-InfoPeriodicCSI-RS. - qcl-InfoPeriodicCSI-RS shall refer to a SSB index |
| Intel[10] | Proposal 1: - In addition to the parameters agreed in RAN1-104e and 104bis-e, the following parameters can be included in TRS/CSI-RS configuration - Repetition - periodicityAndOffset - frequencyDomainAllocation - Availability indication |
| Apple[11] | Proposal 1: The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index. Proposal 2: Use the TRS configuration parameters in Table 1 as the starting point. Further signaling overhead reduction/optimization can be considered. Table 1 TRS configuration parameters TRS configuration parameters; Value Range. TRS configuration index; 0 to (max # of TRS configurations – 1). Frequency domain allocation; {0, 1, 2, 3} For TRS, this can be simplified to the offset for the TRS REs in a RB.. firstOFDMSymbolInTimeDomain; 0 to 9. startingRB; Can reuse the existing range: 0 to 274. nrofRBs; Can reuse the existing range: 24 to 276. powerControlOffsetSS; Reuse the existing range: {-3, 0, 3, 6} dB. scramblingID; Reuse the existing range: 0 to 1023. periodicityAndOffset; Can reuse the existing structure of CSI-ResourcePeriodicityAndOffset, with periodicity limited to {10, 20, 40, 80} ms.. QCL information; 0 to 63, indicating the QCLed SSB index. Number of slots for TRS; {1, 2}. |
| Sony[12] | Proposal 3: Support providing multiple TRS/CSI-RS configurations to idle/inactive UEs. |
| Samsung[13] | Proposal 6: Support applicable values of {db-3, db0, db3, db6} for powerControlOffsetSS of TRS/CSI-RS occasion(s) for idle/inactive UEs. Proposal 7: Support a length of 10bits for scramblingID of TRS/CSI-RS occasion(s) for idle/inactive UEs. Proposal 8: Support value in range of 0 to 13 for firstOFDMSymbolInTimeDomain of TRS/CSI-RS occasion(s) for idle/inactive UEs, Proposal 9: Support applicable value for startingRB in range of (0..maxNrofPhysicalResourceBlocks-1), and applicable values for nrofRBs in range of (24, maxNrofPhysicalResourceBlocksPlus1). for TRS/CSI-RS occasion(s) for idle/inactive UEs. Proposal 10: Support the following fixed configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs: - nrofPorts of 1, - no cdm-type - density of three Proposal 11: Support periodicityAndOffset with applicable values of {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640} slots for TRS/CSI-RS occasion(s) for idle/inactive UEs. Proposal 12: QCL information is provided per RS resource for TRS/CSI-RS occasion(s) for idle/inactive UEs, where the QCL information includes reference signal of a SSB and a qcl-type from { typeA, typeB, typeC, typeD }. |
| MediaTek[14] | Proposal 6: Use Table 1 as the starting point for the discussion of applicable values for TRS configurations. Table 1: Applicable values for the parameters Parameters; Value range. powerControlOffsetSS; {-3, 0, 3, 6} dB. scramblingID; 0 to 1023. firstOFDMSymbolInTimeDomain; Limit to {0, 1, …, 9} according to TS38.214 Section 5.1.6.1.1. startingRB; 0 to 274. nrofRBs; 24 to 276. periodocityAndOffset; Limit to {10, 20, 40, 80} according to TS38.214 Section 5.1.6.1.1. qcl-InfoPeriodicCSI-RS; Limit to {0, 1, …, 63} for SSB index. frequencyDomainAllocation; The value range can be {0, 1, 2, 3} to indicate the offset of the first RS RE to RE#0 in a RB. |
| LG[15] |  |
| Panasonic[16] | Proposal 3: Supported number of TRS configurations in SIB should be minimized, for both operations with and without beam sweeping. |
| Nokia, Nokia Shanghai Bell[17] | Proposal: The configuration of TRS to the IDLE/INACTIVE mode UEs needs to support independent configuration for each broadcast/SSB beam. Proposal: When informing TRS occasions for the IDLE/INACTIVE mode UEs, parameters ‘nrofPorts’, ‘cdm-Type’ and ‘density’ in ‘CSI-RS-ResourceMapping’ can be omitted from the configuration and values assumed to be same as defined by specification TS38.214 for CSI-RS configured with ‘trs-info’. Proposal: Following parameters can be assume to be same/common for RS resources in a slot for TRS configuration, or could be used to implicitly derive other parameter(s): - ‘firstOFDMSymbolInTimeDomain’ would need to be provided only once for symbols in a slot, and location of the second symbol can be derived from it. - Note: if number of consecutive slots (1 or 2) is indicated separetly, in case of two (consecutive) slots, the symbol locations are same in both slots. - ’row1’, ‘startingRB’ and ‘nrofRBs’are common/same for both TRS symbols in a slot, thus would be provided only once per slot (RS resource set). - ‘CSI-ResourcePeriodicityAndOffset’, or similar IE would need to be provided only once for TRS symbols in same slot, or in two consecutive slots and only periods of {10,20,40,80} slots need to be supported. Proposal: Configure the QCL information for each RS resource for IDLE/INACTIVE mode explicitely e.g. via SSB index. |
| Xiaomi[18] | Proposal 1: The QCL information configuration of TRS/CSI-RS occasion(s) associated with SSB indices for idle/inactive UEs should be supported. |
| Sharp[19] | Proposal 1: The NZP-CSI-RS-Resource configuration for power-saving needs reconstruction to accommodate the size limitation of SIB Proposal 2: SSB index could be used as QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs |
| NTT DOCOMO, INC[20] | Proposal 3: The common configuration parameter per RS resource set, or group of sets should be supported to reduce configuration overhead by SIB. |
| InterDigital, Inc.[21] |  |
| Lenovo, Motorola Mobility[22] | Proposal 1: Support following methods to reduce the TRS configuration signalling overhead: - Predefine or fix a part of TRS parameters in specification - Update a subset of parameters of TRS configuration - Based on configuration parameters of one NZP-CSI-RS resource of an NZP-CSI-RS resource set, a UE derives configuration parameters of remaining NZP-CSI-RS resources of the NZP-CSI-RS resource set Observation 2: QCL indication methods in Rel-15/16 NR may not be directly applicable for TRS configured for idle/inactive UEs. Proposal 2: QCL information for TRS configured for idle/inactive UEs is indicated per TRS resource set. Proposal 3: Consider following QCL indication methods for TRS configured for idle/inactive UEs: - Alt 1: based on TRS resource set ID - Alt 2: based on a bitmap of a length same as the maximum number of SSBs per half frame. Each configured TRS resource set is sequentially mapped to a bit with value ‘1’ and a corresponding SSB index. - Alt 3: based on a SSB index provided in the configuration. Proposal 4: Support following TRS transmission modes: - Mode 1: UE may assume that TRS are present on all configured TRS occasions - Mode 2: UE shall further check availability of TRS on configured TRS occasions |
| Ericsson[23] | Proposal 6 QCL information of TRS occasions (provided to idle/inactive UEs) should be associated with SSB indices. Proposal 7 In cases where there is no SI size limitation, support reuse of existing periodic TRS configuration(s) for TRS occasion provisioning. Proposal 8 In cases where resulting SIB size is deemed excessive, support grouping of common parameters within a TRS resource set, and across configured TRS resource sets. a. Details FFS (E.g. such as frequencyDomainAllocation, nrofRBs, and startingRB). |
| Nordic Semiconductor ASA[24] | Proposal-1: TRS are configured 5ms or 10ms before paging frame/half-frame. - supported periodicities are thus 5ms (half-frame) and 10ms (frame) Proposal-3: iTRS shall overlap with - All RBs of 24- and 48-RB CORESET#0 - 48 contiguous RBs of 96-RB CORESET#0 Proposal-4: TCI-state ID points to a SS/PBCH block index only [0-63]. - UE assumes QCL-TypeC' with the SS/PBCH block and, when applicable, 'QCL-TypeD' with the same SS/PBCH block Proposal-5: For iTRS, support configuration of row1 of frequencyDomainAllocation Proposal-6: After all parameters necessary for iTRS are identified, study further how to reduce configuration overhead for iTRS resources. |

## First round discussion

Based on the contributions submitted in RAN1#105b-e, the proposals regarding configuration details of TRS/CSI-RS occasion(s) for idle/inactive UEs can be categorized into five aspects

- Applicable values for supported parameters

- Other parameters

- Details of QCL information

- Methods for configuration overhead reduction

### Issue 2-1: Applicable values for supported parameters

The proposals regarding applicable values for supported parameters are summarized as follows:

|  | powerControlOffsetSS | scramblingID | firstOFDMSymbolInTimeDomain | startingRB | nrofRBs, |
| --- | --- | --- | --- | --- | --- |
| ZTE | {-3, 0, 3, 6} | . 0 to 1023 | 0 to 9 | (24..maxNrofPhysicalResourceBlocksPlus1) |  |
| Huawei, HiSilicon | same range of that for CONNECTED mode | same range of that for CONNECTED mode | same range of that for CONNECTED mode | same range of that for CONNECTED mode | same range of that for CONNECTED mode |
| Apple | {-3, 0, 3, 6} | 0 to 1023 | 0 to 9 | 0 to 274 | 24 to 276 |
| Samsung | {-3, 0, 3, 6} | 0 to 1023 | 0 to 13 | 0..maxNrofPhysicalResourceBlocks-1 | 24, maxNrofPhysicalResourceBlocksPlus1 |
| MediaTek | {-3, 0, 3, 6} dB | 0 to 1023 | {0, 1, …, 9} | 0 to 274 | 24 to 276 |

Based on the above views, the following proposal is suggested for further discussion.

Moderator proposal #7

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Please provide the detailed views on Moderator proposal #7  in the following table.

| Company | Support (Y or N) | Comments/Suggestions |
| --- | --- | --- |

### Issue 2-2: Other parameters

There are proposals about other configuration parameters:

|  | Views |
| --- | --- |
| periodicityAndOffset | - Alt1: to support - Intel - Apple, MediaTek, nokia: {10, 20, 40, 80} ms/slots: - Samsung {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640} slots - Alt2: No need to support, TRS occasion in slot-level granularity to be configured relative to PO. - Huawei, HiSilicon, Nordic |
| Repetition | - Yes: Intel |
| frequencyDomainAllocation | - Yes for row1 - Intel, Nordic, - MediaTek: {0, 1, 2, 3} to indicate the offset of the first RS RE to RE#0 in a RB |
| TRS configuration index | - Yes: Panasonic, Sony, Apple |
| Parameters assumed to be same as defined by specification TS38.214 for CSI-RS configured with ‘trs-info’, and omitted in the configuration - ‘nrofPorts’, ‘cdm-Type’, ‘density’,‘CSI-RS-ResourceMapping’ | - Supported: Nokia |

Therefore, the following proposal is provided for discussion:

Discuss whether to support additional configuration parameters of TRS/CSI-RS occasion(s) for idle/inactive UEs, including, and applicable values if supported:

- periodicityAndOffset,

- repetition

- frequencyDomainAllocation

- configuration index

- parameters assumed to be same as defined by specification TS38.214 for CSI-RS configured with ‘trs-info’, and omitted in the configuration, including

  - ‘nrofPorts’, ‘cdm-Type’, ‘density’, ‘CSI-RS-ResourceMapping’

Please provide the detailed views in the following table.

| Company | Support (Y or N) | Comments/Suggestions |
| --- | --- | --- |

### Issue 2-3: Details of QCL information

According to the contributions submitted to this meeting, the proposals regarding the details of QCL information are summarized as follows.

| Views | Supported companies |
| --- | --- |
| QCL information is indicated as an SSB index, e.g. 0-63 | ZTE, Huawei, HiSilicon, CATT, QC, OPPO, Apple, Samsung, MediaTke, Nokia, Xiaomi, sharp, Ericsson, Nordic |
| QCL information configuration of TRS for idle/inactive UE should be configured per CSI-RS resource set. | CATT, Lenovo |
| The configuration of TRS to the IDLE/INACTIVE mode UEs needs to support independent configuration for each broadcast/SSB beam. | Nokia |
| UE assumes QCL-TypeC' with the SS/PBCH block and, when applicable, 'QCL-TypeD' with the same SS/PBCH block | Nordic |

According to the above views, the following proposal is suggested for further discussion.

Moderator proposal #8

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type

Please provide the detailed views on Moderator proposal #8 in the following table.

| Company | Support (Y or N) | Comments/Suggestions |
| --- | --- | --- |

### Issue 2-4: Methods for configuration overhead reduction

According to the contributions submitted to this meeting, the following alternatives are proposed to reduce configuration overhead for TRS/CSI-RS occasion(s) for idle/inactive UEs.

| Alternative | Supported companies |
| --- | --- |
| Alt1:.The common configuration parameter per RS resource set, or group of sets | Huawei, HiSilicon, nokia, Docomo, Ericsson |
| Alt2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one. | Huawei, HiSilicon, Lenovo |
| Alt3: Predefine or fix a part of TRS parameters in specification | Lenovo |
| Alt4: Number of RS resources/configurations be minimized | Panosonic |
| Alt5: Configure TRS/CSI-RS SMTC, e.g. similar as SMTC for mobility | TCL |
| Alt6: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s) | CATT |

Therefore, it’s suggested to discuss all potential alternatives for configuration overhead reduction and down-select if possible.

Discuss the following alternatives to reduce configuration overhead for TRS/CSI-RS at the configured occasion(s) to the idle/inactive UE

To reduce configuration overhead for TRS/CSI-RS occasion(s) for idle/inactive UEs, one or more of the following alternatives can be supported:

- Alt1: The common configuration parameter per RS resource set, or group of sets

- Alt2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one

- Alt3: Predefine or fix a part of TRS parameters in specification

- Alt4: Number of RS resources/configurations be minimized

- Alt5: Configure TRS/CSI-RS SMTC, e.g. similar as SMTC for mobility

- Alt6: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s)

Please provide the detailed views in the following table.

| Company | Alternative to support | Comments |
| --- | --- | --- |
|  |  | . |

## First round discussion summary

### Issue 2-1: Applicable values for supported parameters

The following proposal was provided by moderator for 1st round discussion.

Moderator proposal #7

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Companies’ views are summarized as follows:

- Support: Nordic, Nokia, QC, CMCC, spreadtrum, Intel, Sharp, LG, MediaTek, Huawei, HiSilicon. ZTE, Sanechips, Xiaomi, vivo, Panasonic (16)

- Not support:

Suggestion/Concern #1: on firstOFDMSymbolInTimeDomain

-Nokia: clarify that if the parameter is used to indicate the symbol location of the first TRS resource in the slot and second TRS resource in the slot can be derived based on the first.

-Sharp:have a different value range for FR1 and FR2 in the current spec for TRS

-Ericsson: we do not support 0 to 9. It should reuse same range (or 0-13) as that for CONNECTED mode (it is anyway 4-bit field).

Moderator: Due to the restriction of TRS only, there will be always two symbols in a slot, so the first symbol index cannot be larger than 9. A note is added based on suggestion from Nokia.

The proposal is updated as follows:

Updated proposal #7_v1

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Clean version of updated proposal #7_v1

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

### Issue 2-2: Other parameters

Discuss whether to support additional configuration parameters of TRS/CSI-RS occasion(s) for idle/inactive UEs, including, and applicable values if supported:

- periodicityAndOffset,

- repetition

- frequencyDomainAllocation

- configuration index

- parameters assumed to be same as defined by specification TS38.214 for CSI-RS configured with ‘trs-info’, and omitted in the configuration, including

  - ‘nrofPorts’, ‘cdm-Type’, ‘density’, ‘CSI-RS-ResourceMapping’

Moderator: Views are not converging yet. Can be deprioritized in the first round of discussion. Will revisit in next round.

### Issue 2-3: Details of QCL information

The following proposal was provided by moderator for 1st round discussion.

Moderator proposal #8

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type

Companies’ views are summarized as follows:

- Support: Nordic, Nokia, QC, CMCC,TCL, OPPO, Spreadtrum, Intel, Sharp, CATT, LG, MediaTek, DOCOMO, Huawei, HiSilicon, Xiaomi, Samsung, Ericsson, Sony, Panasonic (14)

- Not support:

Concern #1 (ZTE): We think only QCL-D is needed to identify the beam direction, and the signaling overhead can be also reduced.

Moderator: only SSB index will be indicated. QCL type will be predetermined. But whether it’s QCL-typeD can be FFS. My understanding is QCL-typeD is applicable for FR2, ‘ QCL-typeC’ is applicable for FR1. The FFS is modified to clarify that no additional signaling overhead for that.

Suggestion#1(Motorola): The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated implicitly or explicitly as a SSB index in range of 0 to 63.

Moderator: By “implicitly”, do you mean QCL type? we need to clarify that. But for QCL type it can be predetermined regardless of the number of SSB index indicated.

The proposal is updated as follows:

Updated proposal #8_v1

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

Clean version of updated proposal #8_v1

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

### Issue 2-4: Methods for configuration overhead reduction

Discuss the following alternatives to reduce configuration overhead for TRS/CSI-RS at the configured occasion(s) to the idle/inactive UE

To reduce configuration overhead for TRS/CSI-RS occasion(s) for idle/inactive UEs, one or more of the following alternatives can be supported:

- Alt1: The common configuration parameter per RS resource set, or group of sets

- Alt2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one

- Alt3: Predefine or fix a part of TRS parameters in specification

- Alt4: Number of RS resources/configurations be minimized

- Alt5: Configure TRS/CSI-RS SMTC, e.g. similar as SMTC for mobility

- Alt6: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s)

Moderator: Views are not converging yet. Deprioritized in the first round discussion.

## Third round discussion

### Issue 2-2: Other parameters

Companies’ views are summarized below:

|  | Support | Object/Concerns |
| --- | --- | --- |
| periodicityAndOffset | Nokia, Intel, LG, Apple: {10, 20, 40, 80}ms QC, TCL, SHARP, MediaTek, Huawei, HiSilicon, Ericsson, Sony, PANASONIC, Motorola | Noridc: is connected to TRS validation interval discussed in Proposal 5 |
| repetition | TCL, Sony, PANASONIC QC, Intel: may be needed for SNR boosting similar to paging PDCCH MOs | Nordic: it is used for narrow beam acquisition, this is not in scope of this WID. In IDLE only wide beams corresponding to SSBs are assumed. Nokia, LG, Huawei, HiSilicon, ZTE, Sanechips, Samsung, Ericsson, Motorola, Apple, MediaTek: not supported for TRS |
| frequencyDomainAllocation, i.e. row1 w/ values from {0, 1, 2, 3}. | Nokia ,QC, TCL, Intel, SHARP, LG, MediaTek, Ericsson, Sony, PANASONIC, Motorola, Apple |  |
| configuration index | Nokia: making the resource ID the implicit QCL source indication. QC, TCL, Intel, SHARP, MediaTek, Ericsson, Sony, PANASONIC, Motorola LG, Apple: FFS | Nordic: need depends again on validation scheme design. If design is not based on Resource sets and there is one resource per SSB, then ID may not be needed. |
| parameters assumed to be same as defined by specification TS38.214 for CSI-RS configured with ‘trs-info’, and omitted in the configuration, including - ‘nrofPorts’, ‘cdm-Type’, ‘density’, ‘CSI-RS-ResourceMapping’ | Nordic, Nokia, QC, TCL, Intel, SHARP, LG, MediaTek, Huawei, HiSilicon, Sony, PANASONIC, Motorola | Apple: generally fine, But I think we are already doing this by agreeing on what parameters are needed. |

For periodicityAndOffset, the majority support it except Nordic. Nordic supports the periodicityAndOffset determined implicitly based on validity time. But, UE still needs to determine exact MOs within valid period.

For repetition, the majority think it’s not needed. Because it’s not supported for connected mode TRS based on current spec.

For frequencyDomainAllocation, the majority support it for row1 with values from {0, 1, 2, 3}.

For configuration index, the majority support it. But may need FFS more details. Two companies [Nokia, Nordic] commented it could be implicitly determined by QCL source/SSB index.

For fixed parameters, the majority are fine.

Based on the summary above, the following proposals regarding addition parameters can be considered.

Proposal #9_v1 Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset with applicable values from {10, 20, 40, 80}ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - Configuration index - FFS details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source

Proposal #10_v1 The following parameters can be omitted from the configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs, and idle/inactive UEs assume corresponding values to be same as defined by specification TS38.214 for CSI-RS configured with ‘trs-info’: - nrofPorts’, - ‘cdm-Type’, - ‘density’, - ‘CSI-RS-ResourceMapping’

### Issue 2-4: Methods for configuration overhead reduction

To reduce configuration overhead for TRS/CSI-RS occasion(s) for idle/inactive UEs, one or more of the following alternatives can be supported:

- Alt1: The common configuration parameter per RS resource set, or group of sets

- Alt2: gNB provides a ‘reference configuration’, and each configured resource can have a ‘delta-configuration’ compared with the reference one

- Alt3: Predefine or fix a part of TRS parameters in specification

- Alt4: Number of RS resources/configurations be minimized

- Alt5: Configure TRS/CSI-RS SMTC, e.g. similar as SMTC for mobility

- Alt6: TRS/CSI-RS configuration for Idle/Inactive mode should be associated with SSB/paging occasion(s)

Companies’ views submitted to NWM are summarized below:

|  | OK to consider/support | Object |
| --- | --- | --- |
| Alt1 | Nordic, Sharp, LG, MediaTek, DOCOMO, Huawei, HiSilicon, ZTE, Sanechips, Ericsson, Motorola |  |
| Alt2 | TCL, Huawei, HiSilicon, Motorola |  |
| Alt3 | Huawei, HiSilicon, ZTE, Sanechips, Motorola |  |
| Alt4 | Huawei, HiSilicon |  |
| Alt5 | TCL |  |
| Alt6 | CATT |  |
| All | QC, Samsung, Apple | Intel |

|  | Concerns | Clarification/suggestion |
| --- | --- | --- |
| Alt2 | Ericsson: seems to be RAN2 domain with delta signaling between configurations |  |
| Alt3 | Samsung; We already discuss fixed parameters for TRS in issue 2-2. Ericsson: Parts of Alt 3 have already been considered already (like trs-Info). |  |
| Alt4 | Ericsson: is NW implementation. | Panasonic: is the design principle in our view. |
| Alt5 | QC, Xiaomi: will work unless the TRS can be normally configured in this way (or almost this way) when it is configured to the connected mode UE. HW: we want to ask for clarification why the SMTC-like configuration can reduce overhead. |  |
| Alt 6 | QC: will work unless the TRS can be normally configured in this way (or almost this way) when it is configured to the connected mode UE. | CATT: TRS/CSI-RS resource configuration is only the frequency domain resource in NZP-CSI-RS-ResourceSet IE. The time domain resource should be associated with the PO (e.g., offset before PO) in order to achieve UE power saving HW: may be related to issue 2-2, under which we can discuss it. |
| Others | Nokia: It may not be possible to assume resource sets to share parameters or desirable to fix parameters beyond limitations provided by TS38.214 Intel: We are not quite certain at this stage whether there exists a need to define optimizations to control higher layer signaling OH. It may be better to revisit this question when configuration details, including how RS resources are defined and configured (as a baseline mechanism), are clear. Also, some of the potential optimizations (some of which are listed above) actually fall under RAN2 expertise on RRC signaling mechanisms. Panasonic: It is also good to conclude firstly the number of maximum RS configurations. Then whether and how to reduce the L2 configuration and L1 indication overhead would be clearer and next step discussion. | ZTE: a note that “Other alternatives are not precluded” Apple: the more fundamental discussion that should occur before this is to discuss whether to reuse CSI-RS resource set configuration signaling or create a new TRS configuration signaling to minimize the overhead. This helps us understand the size for each TRS configuration |

According to the summary, the majority is fine to support Alt1. For alt2, it’s more related to RAN2 topic, so can be deprioritized. For Alt3, it’s discussed in Issue 2-2 on other parameters. For Alt4, it can be sub-item of Alt1. For Alt5 and Alt6, configuration of some parameters, such as periodicity, have to be determined first.

Based on the analysis above, the proposal is updated as follows:

Proposal #11_v1 Support common configuration parameter per RS resource set and/or group of sets for configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS maximum number of RS resources sets or groups of sets - FFS details - E.g. whether or not to reuse NR Rel-15/16 CSI-RS resource set configuration

## Third round discussion summary

### Issue 2-2: Other parameters

For all the companies submitted views, all are fine to support. A few suggestions as follows.

Suggestions

Applicable values for periodicityAndOffset Apple, ZTE: for periodicityAndOffset, we should clarify the applicable values are for periodicity TCL:squeezing the periodicityAndOffset to lower values will allow network to consume more power by transmitting more TRS resources in a PO and the UE may receive more TRS resources and consume more power just like the UE consume power in SSB based synchronization procedure . we suggest to to keep the high values of PeriodicityAndoffset of CSI-RS in the periodicityAndOffset values of TRS/CSI-RS for idle/inactive UE e.g. {10, 20, 40, 80, 160, 320, 640} ms. SS: prefer to FFS applicable values for periodicityandoffset

Configuration index Apple: It may be possible to leave configurationIndex to a later decision, even though we also included it in our contribution. There is a good chance that we may need it, but it would be better to agree to it after we know where/how we will use the configuration index. HW:If the details are still FFS, we would prefer to keep the how bullet of configuration index as FFS

The proposal is updated based on the suggestions above.

Proposal #9_v2

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include:

- periodicityAndOffset with applicable values from {10, 20, 40, 80}ms

  - FFS applicable values

- frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB

- FFS Configuration index

  - FFS details,

    - E.g. Per resource or resource set or group of resource sets

    - E.g. explicit or implicit indication based on QCL source

For Proposal #10_v1. No much interests. Due to the time limitation, it’s not worth spending much time for this. We can focus on parameters needed.

### Issue 2-4: Methods for configuration overhead reduction

No much interests for this proposal. Some suggestions as follows:

Suggestions

Apple, ZTE: I understand that there is FFS bullet for whether to reuse or not, but in this case the term ”RS resource set” is not defined, and the main bullet becomes unclear. we should first determine whether the current signaling structure can be reused or not , and what we can do on the top of current signaling, which also includes set of common parameters

HW: At least support common configuration parameter per RS resource set and/or group of sets for configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs. · FFS miximum number of RS resource sets or groups of sets · FFS gNB provides a ‘reference configuration’, and each configured resource can have a‘delta-configuration compared with the reference one

Due to the time limit in this meeting, it’s better to focus on the small issue suggested by Apple and ZTE in this proposal. Whether and how to achieve configuration overhead reduction can be postponed.

Proposal #11_v2

Further study whether and how to reuse NR Rel-15/16 CSI-RS resource set configuration.

Proposal #11_v3

Further study whether and how to reuse NR Rel-15/16 CSI-RS resource set configuration, and if not, how the configuration is defined.

Proposal #11_v4

Further study whether to reuse NR Rel-15/16 CSI-RS resources set configuration, and if not, how the configuration is defined to minimize the signaling overhead.

# Topic #3: Others

The following proposals in addition to the three topics above are submitted in RAN1#104b-e

| ZTE [1] |  |
| --- | --- |
| Huawei, HiSilicon[2] | - Send LS to inform RAN2 about the progresses and agreements in RAN1 and trigger the discussion in RAN2 on the issues of signaling design for assistance TRS configuration. |
| TCL Communication Ltd[3] |  |
| Vivo[4] | Proposal 4: UE assumes the TRS available before receiving signaling indicating the TRS as NOT available. |
| Spreadtrum Communications[5] |  |
| CATT[6] | Proposal 1: TRS/CRS-RS resource/resource set configuration should meet the requirement of SIB message size limit. Proposal 5: The following procedure can be used for TRS/CSI-RS occasion(s) configuration: Step1) predefined parameters of TRS/CSI-RS resource grid; Step 2) SIB indicate parameters details; including - QCL assumption of the configured TRS/CSI-RS resources associated with a SSB; - Code points mapping of availability/not availability for a given TRS resources. Step 3）To derive TRS occasion(s) according to predefined rule and parameters provided by step1 and step 2. |
| CMCC[7] |  |
| Qualcomm Incorporated[8] |  |
| OPPO[9] |  |
| Intel[10] |  |
| Apple[11] | Proposal 6: When a TRS configuration is indicated as available, the idle/inactive UEs assumes that only a certain number of TRS occasion(s) before a PO is available. |
| Sony[12] | Proposal 6: In addition to SIB, support TRS/CSI-RS configuration via other high-layer signaling (e.g., dedicated RRC, RRC release message, etc.). |
| Samsung[13] | Proposal 10: Support the following fixed configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs: - nrofPorts of 1, - no cdm-type - density of three |
| MediaTek[14] |  |
| LG[15] | Proposal 3: Study how to handle PDSCH REs overlap with TRS/CSI-RS occasion(s). |
| Panasonic[16] |  |
| Nokia, Nokia Shanghai Bell[17] |  |
| Xiaomi[18] |  |
| Sharp[19] |  |
| NTT DOCOMO, INC[20] |  |
| InterDigital, Inc.[21] |  |
| Lenovo, Motorola Mobility[22] |  |
| Ericsson[23] |  |
| Nordic Semiconductor ASA[24] | Proposal-3: iTRS shall overlap with - All RBs of 24- and 48-RB CORESET#0 - 48 contiguous RBs of 96-RB CORESET#0 |

## First round discussion

Discussion take place on NWM.

## First round discussion summary

Two suggestions for other topics to discuss are provided as follows:

Suggestion #1 (LG): method for handling PDSCH REs overlap with the reference signals

Suggestion #2(HW, Nokia): Send LS to RAN2 considering in this meeting RAN1 would achieve progress regarding configuration information and configuration overhead reduction.

Moderator: For #1, will be visited in last round of discussion if time allows. For #2, will discuss in next round depending on the progress.

# Conclusion

In RAN1#105-e meeting, the following agreements are made.

Agreement:

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Agreement:

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Agreement:

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

Working assumption:

Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected.

- FFS whether and how to enable/disable L1 based availability indication configurable by SIB

Agreement:

Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include:

- periodicityAndOffset {10, 20, 40, 80} ms

- frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB

- FFS Configuration index

  - details,

    - E.g. Per resource or resource set or group of resource sets

    - E.g. explicit or implicit indication based on QCL source

Agreement:

Further study supporting SIB based signaling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured.

- FFS whether and how SIB based signaling and L1 based signaling can be configured simultaneously

# References

[1] R1-2104222	TRS for RRC idle and inactive UEs	ZTE, Sanechips

[2] R1-2104252	Assistance RS occasions for IDLE/inactive mode	Huawei, HiSilicon

[3] R1-2104308	TRS/CSI-RS occasions for idle/inactive UE	TCL Communication Ltd.

[4] R1-2104372	TRS/CSI-RS occasion(s) for idle/inactive UEs	vivo

[5] R1-2104433	Consideration on TRS/CSI-RS occasion(s) for idle/inactive UEs	Spreadtrum Communications

[6] R1-2104533	Configuration of TRS/CSI-RS for paging enhancement	CATT

[7] R1-2104623	Discussion on TRS/CSI-RS occasion(s) for IDLE/INACTIVE-mode UEs	CMCC

[8] R1-2104683	TRS/CSI-RS for idle/inactive UE power saving	Qualcomm Incorporated

[9] R1-2104788	Further discussion on RS occasion for idle/inactive UEs	OPPO

[10] R1-2104917	Discussion on TRS/CSI-RS Design in idle/inactive mode	Intel Corporation

[11] R1-2105117	Indication of TRS configurations for idle/inactive-mode UE power saving	Apple

[12] R1-2105175	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Sony

[13] R1-2105322	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Samsung

[14] R1-2105387	On TRS/CSI-RS occasion(s) for idle/inactive mode UE power saving	MediaTek Inc.

[15] R1-2105435	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	LG Electronics

[16] R1-2105475	Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs	Panasonic

[17] R1-2105506	On RS information to IDLE/Inactive mode Ues	Nokia, Nokia Shanghai Bell

[18] R1-2105574	On TRS/CSI-RS configuration and indication for idle/inactive UEs	Xiaomi

[19] R1-2105639	Discussion on TRS/CSI-RS occasions for idle/inactive UEs	Sharp

[20] R1-2105709	Discussion on TRS CSI-RS occasion for idleinactive UEs	NTT DOCOMO, INC.

[21] R1-2105743	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	InterDigital, Inc.

[22] R1-2105771	Provision of TRS/CSI-RS for idle/inactive UEs	Lenovo, Motorola Mobility

[23] R1-2105792	Provisioning of TRS occasions to Idle/Inactive UEs	Ericsson

[24] R1-2105887	On TRS design for idle/inactive UEs	Nordic Semiconductor ASA

# Summary of Agreements

## RAN1#102-e

Agreements: - New types/patterns of TRS/CSI-RS are not introduced specifically for idle/inactive mode UE. Agreements: The TRS/CSI-RS occasion(s) that may be for connected mode UEs can be shared to idle/inactive mode UEs. - Note: It is understood that gNB can potentially share the occasions to idle/inactive (which would just mean it up to NW whether to share or not share). - Note: It is understood that TRS/CSI-RS in the TRS/CSI-RS occasion(s) may or may not be transmitted. - Note: Always-on TRS/CSI-RS transmission by gNodeB is not required - At least TRS/CSI-RS occasion(s) corresponding to periodic TRS is supported - FFS for other RS types - FFS: Whether UE blind detection is required or not. Agreements: Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as: - AGC, time/frequency tracking - FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication Observation: It is up to gNB implementation whether or not to transmit a TRS/CSI-RS to idle/inactive UEs even when the TRS/CSI-RS is not needed by connected UEs (e.g., when there is a connected mode UE in a cell but the UE is no longer using the TRS/CSI-RS, or when there is no longer connected mode UE in a cell, etc.) Agreements: The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling - FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.) - FFS for other signalling candidates (e.g., pre-configuration, etc.) - FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.) Agreements: Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly). - Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

## RAN1#103-e

Agreement: - Functionality of RRM measurement for neighbour cell is not supported for TRS/CSI-RS for idle/inactive UE(s). Agreements: - SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s). - Up to RAN2 to decide which SIB is to be used. - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2 Send an LS to RAN2 informing the above agreements, and - To further add that RAN1 is working on the detailed physical layer design Agreement: - Aperiodic TRS and semi-persistent/aperiodic CSI-RS are not used as TRS/CSI-RS occasion(s) for idle/inactive UEs. Agreements: - Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting) - Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence Agreements: - Discuss further based on the following alternatives and down-select at RAN1#104-e: - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE. - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - The condition can be, e.g., existence of paging. - Alt 4. Combination of the above alternatives. - FFS for details - FFS for UE behavior when the availability is not informed. - Other techniques are not precluded. - Companies encourage to provide sufficient information for the proposal, e.g., - how to achieve power saving gain - how to minimize impact on NW how to minimize extra UE implementation complexity - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs - Proposals should be consistent with the WID objective. Conclusion: - TRS/CSI-RS based PEI is discussed in AI 8.7.1.1. - PEI functionality is not further discussed under AI 8.7.1.2. - Note: This does not prevent to potentially use PEI to carry the indication for TRS/CSI-RS presence.

## RAN1#104-e

Update on 1/28 email: Agreements: Configuration of TRS/CSI-RS occasion(s) for idle/inactive Ues include at least: - powerControlOffsetSS, - scramblingID - firstOFDMSymbolInTimeDomain, - startingRB. - nrofRBs, - FFS other parameters - FFS applicable values Agreements: The SCS configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs can be discussed and down-selected from following alternatives at RAN1#105-e: - Alt1: same as initial BWP - Alt2: configurable parameter Agreements: Multiple RS resources can be configured for TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details (including whether or not to restrict the RS to be TRS only) Update on 1/31: Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable). Conclusion From RAN1 perspective, there is no consensus on supporting RRM measurement for serving cell functionality for TRS/CSI-RS occasion(s) for idles/inactive UEs. Agreements: The configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs are discussed and down-selected from following alternatives at RAN1#104bis-e: - Alt-1: within initial DL BWP - Alt-2: is not restricted by initial BWP - IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. Agreements: To study QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs from following alternatives: - Alt-1: TCI state from higher layer configuration, e.g. qcl-InfoPeriodicCSI-RS - Alt-2: QCL assumptions associated with transmitted SSBs implicitly, e.g. similar to PDCCH monitoring in PO - FFS details - FFS details - Other alternatives are not precluded Conclusion: Decide at RAN1#104b-e, whether or not to support periodic CSI-RS in addition to periodic TRS for TRS/CSI-RS occasion(s) for idle/inactive UEs.

## RAN1#104b-e

Agreement: SCS of TRS/CSI-RS occasion(s) for idle/inactive UEs is same as SCS of CORESET#0. Agreement: Support higher layer configuration of the QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs. - FFS details of the QCL information, e.g. associated SSB index Agreement: IDLE/INACTIVE mode UE is not expected to receive TRS/CSI-RS outside the initial DL BWP. - Configuration of the frequency location of TRS/CSI-RS occasion(s) for idle/inactive UEs is not restricted by initial BWP. Working assumption: Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling To further check on 4/19 Agreement: Configuration for TRS/CSI-RS occasion(s) for idle/inactive UEs is based on periodic TRS only, including following limitations - Configuration parameters that are necessary to provide configuration of periodic TRS for idle/inactive UEs - Applicable values that are necessary to provide configuration of periodic TRS for idle/inactive UEs - If the configuration is provided, idle/inactive UEs can always implicitly assume that trs-info is configured. - The parameter trs-info does not need to be provided in the configuration Agreement: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, one or more alternatives from the following can be supported: - Alt1: Availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit from a bitmap or a codepoint is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - Alt2: value or codepoint to indicate one or more resource/configuration indices that correspond to the available RS resources - FFS whether and how to indicate the ‘availability’ in beam selective manner. - Other alternatives are not precluded