3GPP TSG RAN WG1 Meeting #105-e

e-Meeting, May 10th – 27th, 2021

Title:	RAN1 Chair’s Notes

1	Opening of the meeting (Day 1: UTC 9:00 AM)	3

1.1	Call for IPR	3

1.2	Competition Law Statement	3

1.3	Network Usage Conditions	4

1.4	Check-in for Registered Delegates	4

1.5	Aspects related to RAN1 Meeting Management	4

2	Approval of Agenda	4

3	Highlights from RAN plenary	4

4	Approval of Minutes from previous meetings	4

5	Incoming Liaison Statements (2)	4

5.1	RAN1 Aspects for RF requirements for NR frequency range 1 (FR1)	7

5.2	RAN1 Aspects for NR small data transmissions in INACTIVE state	8

6	E-UTRA (6)	8

6.1	Maintenance of E-UTRA Releases 8 – 15 (3)	8

6.2	Maintenance of E-UTRA Release 16 (2+1=3)	9

6.2.1	Maintenance of Additional MTC Enhancements (2)	9

6.2.2	Maintenance of Additional Enhancements for NB-IoT (1)	9

6.2.3	Maintenance of DL MIMO efficiency enhancements for LTE	9

6.2.4	Maintenance of LTE-based 5G Terrestrial Broadcast	9

6.2.5	LTE Rel-16 UE Features	9

6.2.6	Others	9

7	NR Maintenance (15+31=46)	9

7.1	Maintenance of Release 15 NR (15)	9

7.2	Maintenance of Release 16 NR (1+4+7+5+4+1+2+1+1+3+2=31)	12

7.2.1	Maintenance of Two step RACH for NR (1)	12

7.2.2	Maintenance of NR-based Access to Unlicensed Spectrum (4)	12

7.2.3	Maintenance of Integrated Access and Backhaul for NR	13

7.2.4	Maintenance for 5G V2X with NR sidelink (7)	13

7.2.5	Maintenance of Physical Layer Enhancements for NR URLLC (5)	14

7.2.6	Maintenance of Enhancements on MIMO for NR (4)	15

7.2.7	Maintenance of UE Power Saving for NR (1)	16

7.2.8	Maintenance of NR positioning support (2)	16

7.2.9	Maintenance of NR Mobility Enhancements (1)	17

7.2.10	Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (1)	17

7.2.11	NR Rel-16 UE Features (3)	17

7.2.12	Others (2)	17

8	Release 17 (5+4+4+5+5+5+3+5+3+2+3+3+3+3+4+1=58)	18

8.1	Further enhancements on MIMO for NR (5)	18

8.1.1	Enhancements on Multi-beam Operation	18

8.1.2	Enhancements for Multi-TRP Deployment	19

8.1.2.1	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	19

8.1.2.2	Enhancements on Multi-TRP inter-cell operation	19

8.1.2.3	Enhancements on beam management for multi-TRP	19

8.1.2.4	Enhancements on HST-SFN deployment	20

8.1.3	Enhancements on SRS flexibility, coverage and capacity	21

8.1.4	CSI enhancements: MTRP and FR1 FDD reciprocity	21

8.1.5	Others	21

8.2	Supporting NR from 52.6GHz to 71 GHz (4)	21

8.2.1	Initial access aspects	21

8.2.2	PDCCH monitoring enhancements	22

8.2.3	Enhancements for PUCCH formats 0/1/4	22

8.2.4	Beam management for new SCSs	23

8.2.5	PDSCH/PUSCH enhancements	23

8.2.6	Channel access mechanism	24

8.2.7	Others	24

8.3	Enhanced Industrial Internet of Things (IoT) and URLLC (4)	25

8.3.1	Necessity and Support of Physical Layer feedback enhancements	25

8.3.1.1	UE feedback enhancements for HARQ-ACK	25

8.3.1.2	CSI feedback enhancements	25

8.3.2	Enhancements for unlicensed band URLLC/IIoT	26

8.3.3	Intra-UE Multiplexing/Prioritization	27

8.3.4	Others	28

8.4	Solutions for NR to support non-terrestrial networks (NTN) (5)	28

8.4.1	Timing relationship enhancements	28

8.4.2	Enhancements on UL time and frequency synchronization	28

8.4.3	Enhancements on HARQ	29

8.4.4	Others	30

8.5	NR Positioning Enhancements (5)	30

8.5.1	Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays	30

8.5.2	Accuracy improvements for UL-AoA positioning solutions	31

8.5.3	Accuracy improvements for DL-AoD positioning solutions	31

8.5.4	Latency improvements for both DL and DL+UL positioning methods	32

8.5.5	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	32

8.5.6	Others	33

8.6	Support of Reduced Capability NR Devices (5)	33

8.6.1	UE complexity reduction	34

8.6.1.1	Aspects related to reduced maximum UE bandwidth	34

8.6.1.2	Aspects related to reduced number of Rx branches	34

8.6.1.3	Aspects related to duplex operation	35

8.6.1.4	Other aspects	36

8.6.2	RAN1 aspects for RAN2-led features for RedCap	36

8.6.3	Others	37

8.7	UE Power Saving Enhancements (3)	37

8.7.1	Enhancements for idle/inactive-mode UE power saving	37

8.7.1.1	Potential paging enhancements	37

8.7.1.2	TRS/CSI-RS occasion(s) for idle/inactive UEs	38

8.7.1.3	Others	38

8.7.2	Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime	38

8.7.3	Others	39

8.8	NR coverage enhancement (5)	39

8.8.1	PUSCH enhancements	39

8.8.1.1	Enhancements on PUSCH repetition type A	39

8.8.1.2	TB processing over multi-slot PUSCH	40

8.8.1.3	Joint channel estimation for PUSCH	41

8.8.2	PUCCH enhancements	41

8.8.3	Type A PUSCH repetitions for Msg3	42

8.8.4	Others	43

8.9	Rel-17 enhancements for NB-IoT and LTE-MTC (3)	43

8.9.1	Support of 16-QAM for unicast in UL and DL for NB-IoT	43

8.9.2	Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC	43

8.9.3	Others	43

8.10	Enhancements to Integrated Access and Backhaul (2)	44

8.10.1	Enhancements to resource multiplexing between child and parent links of an IAB node	44

8.10.2	Other enhancements for simultaneous operation of IAB-node’s child and parent links	44

8.10.3	Others	45

8.11	NR Sidelink enhancement (3)	45

8.11.1	Resource allocation enhancement	45

8.11.1.1	Resource allocation for power saving	45

8.11.1.2	Inter-UE coordination for Mode 2 enhancements	46

8.11.2	Others	47

8.12	NR Multicast and Broadcast Services (3)	47

8.12.1	Mechanisms to support group scheduling for RRC_CONNECTED UEs	47

8.12.2	Mechanisms to improve reliability for RRC_CONNECTED UEs	48

8.12.3	Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	48

8.12.4	Others	49

8.13	NR Dynamic spectrum sharing (DSS)  (3)	49

8.13.1	Cross-carrier scheduling (from Scell to Pcell)	49

8.13.2	Multi-cell PDSCH scheduling via a single DCI	50

8.13.3	Support efficient activation/de-activation mechanism for SCells in NR CA	51

8.13.4	Others	51

8.14	Study on XR Evaluations for NR (RAN1) (3)	51

8.14.1	Traffic Model	51

8.14.2	Evaluation Methodology	52

8.14.3	Initial Performance Evaluation Results	52

8.14.4	Others	53

8.15	Study on NB-IoT/eMTC support for Non-Terrestrial Network (4)	53

8.15.1	Scenarios applicable to NB-IoT/eMTC	53

8.15.2	Enhancements to time and frequency synchronization	53

8.15.3	Timing relationship enhancements	54

8.15.4	Enhancements on HARQ	55

8.15.5	Others	55

8.16	Others (1)	55

9	Elections	56

10	Closing of the meeting (Day 7: UTC 17:00 at the latest)	56

# Opening of the meeting (Day 1: UTC 9:00 AM)

Note:

- May 10th – May 18th are for RAN1 elections only. No technical discussion (other than discussion for the preparation phase of the e-meeting). Please see agenda 9 for more details of the elections.

- Nominal RAN1 technical work is from May 19th to May 27th, for a total of 7 work days. Therefore, Day 1 herein refers to May 19th and Day 7 refers to May 27th

## Call for IPR

I draw your attention to your obligations under the 3GPP Partner Organizations' IPR policies.  Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

The attention of the delegates to this meeting was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were thereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (http://www.etsi.org/WebSite/document/Legal/IPRForms.doc).

## Competition Law Statement

The attention of the delegates to this meeting was drawn to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel. The present meeting will be conducted with strict impartiality and in the interests of 3GPP. Furthermore, delegates were reminded that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network Usage Conditions

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Aspects related to RAN1 Meeting Management

Delegates are encouraged to check R1-1721392 for some thoughts on RAN1 meeting management.

# Approval of Agenda

R1-2104150	Draft Agenda of RAN1#105-e meeting	RAN1 Chair

Email approval till 5/20 – no dedicated email thread. Please use the general e-meeting management email thread for any comments. Automatically approved if no comments by 5/20  no comments, approved.

R1-2104152	RAN1#105-e Meeting Invitation, Timelines, Scope, Process	RAN1 Chair, ETSI MCC

R1-2104153	Additional Guidelines for RAN1#105-e-Meeting Management	RAN1 Chair

# Highlights from RAN plenary

Void for this meeting

# Approval of Minutes from previous meetings

R1-2104151	Report of RAN1#104bis-e meeting	ETSI MCC

Email approval till 5/21 - approved

# Incoming Liaison Statements (2)

R1-2105984	RAN1#105-e preparation phase on LSs	RAN1 Chair

All incoming LSs are noted. The following are for the next phase of email discussion/approval (either dedicated or as part of the normative work):

- To be handled under other AIs:

  - Email discussion for R1-2104159 is necessary, including a possible LS, till 5/24 (name TBD, Huawei). To be handled under 6.2.1

  - A reply LS for R1-2104155 is necessary – email discussion/approval till 5/24 (name TBD, Ericsson). To be handled under 8.4

  - A reply LS to R1-2104163 is necessary – email discussion/approval till 5/24 (name TBD, Apple). To be handled under 7.2.2

  - A reply LS to R1-2104164 is necessary – target 5/24 for email discussion/approval. To be handled under 7.2.11.

  - Discussion is necessary including a possible LS to R1-2104165. Can be taken as part of the normative work. To be handled under 8.12.3

  - No dedicated email thread discussion for R1-2104229. To be taken into account as part of the normative work under 8.7 (including a possible LS)

  - Email discussion/approval for R1-2104230 is necessary, from 5/21 -  5/26 (name TBD, OPPO). To be handled under 8.4.2

  - Email discussion/approval for a potential LS to R1-2104559, till 5/25 (name TBD, OPPO). To be handled under 7.2.4.

  - Email discussion/approval for the reply LS to R1-2100021 till 5/21-5/26, to be handled under 8.11 (Yuzhou Hu, ZTE)

  - Email discussion/approval for the reply LS to R1-2102306 till 5/25, to be handled under 8.5 (name TBD, Qualcomm)

  - For LS in R1-2104023, no dedicated email thread discussion; to be handled under 8.14 as part of the XR discussion, including a possible reply LS.

  - For LS in R1-2102308, no dedicated email thread discussion; to be handled under 8.14 as part of the XR discussion, including a possible reply LS

- To be handled under AI5:

  - A reply LS to R1-2104161 is necessary – email discussion/approval till 5/24 (Karri, Nokia)

  - A reply LS to R1-2104162 is necessary – email discussion/approval till 5/25 (Xingguang Wei, ZTE)

[105-e-AI5-LS-01] A reply LS to R1-2104161 is necessary – email discussion/approval till 5/24 (Karri, Nokia)

Update on 5/24: latest draft LS as in v001 is approved. Final LS in R1-2106108.

R1-2106107	[105-e-AI5-LS-01] Discussion summary on reply LS to R1-2104161	Moderator (Nokia)

R1-2106108	Reply LS on RI bit width for Cat5 UE in EN-DC mode	RAN1, Nokia

[105-e-AI5-LS-02] A reply LS to R1-2104162 is necessary – email discussion/approval till 5/25 (Xingguang Wei, ZTE)

Update on 5/25:

Agreement:

Regarding questions mentioned in RAN2 LS R1-2104162/R2-2104550,

- UE feature 6-24 (ul-TimingAlignmentEUTRA-NR) is applicable to Type 1 and Type 2 (NG)EN-DC BC types.

- UE feature 6-23 (pa-PhaseDiscontinuityImpacts) is applicable to Type 1 and Type 2 (NG)EN-DC/NE-DC BC types.

- Both 6-24 (ul-TimingAlignmentEUTRA-NR) and 6-23 (pa-PhaseDiscontinuityImpacts) are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part.

- RAN1 further discusses whether 6-24 (ul-TimingAlignmentEUTRA-NR) and 6-23 (pa-PhaseDiscontinuityImpacts) can be applicable to Type 5 (NG)EN-DC/NE-DC BC type by taking RAN4 discussion outcome into account.

Draft LS? 5/26

//LTE

R1-2104158	Reply LS on timing of neighbor cell RSS-based measurements	RAN2, Qualcomm

R1-2104159	Reply LS related to RSS based RSRQ for LTE-MTC	RAN2, Huawei

R1-2104709	Discussion on RAN2 reply LS on RSS based RSRQ	ZTE

R1-2104818	Discussion on RSS-based measurements	Qualcomm Incorporated

R1-2105941	Discussion on RSS based RSRQ for LTE-MTC	Huawei, HiSilicon

//NR

Rel-15

R1-2104161	LS on RI bit width for Cat5 UE in EN-DC mode	RAN2, Nokia

R1-2104226	On RI bit width for Cat5 UE in EN-DC mode	Nokia, Nokia Shanghai Bell

R1-2104578	Draft reply LS on RI bit width for Cat5 UE in EN-DC	ZTE

R1-2104640	Draft reply LS on Cat-5 with EN-DC	Qualcomm Incorporated

R1-2105280	Draft reply LS on RI bit width for Cat5 UE in EN-DC mode	Samsung

R1-2105446	Draft reply LS on RI bit width for Cat5 UE in EN-DC mode	vivo

R1-2105935	Discussion on RI bit width for Cat5 UE in EN-DC mode	Huawei, HiSilicon

R1-2104162	LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities	RAN2, ZTE

R1-2104319	Discussion on the Intra-band and Inter-band (NG)EN-DC NE-DC Capabilities	ZTE

R1-2104320	[DRAFT] Reply LS on the Intra-band and Inter-band (NG)EN-DC NE-DC Capabilities	ZTE

R1-2104466	Discussion on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities	CATT

R1-2105451	Draft reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities	vivo

R1-2105932	Discussion on EN-DC/NE-DC UE capabilities	Huawei, HiSilicon

Rel-16

R1-2104157	Reply LS on TCI state indication at Direct SCell activation	RAN2, MediaTek

R1-2104160	Reply LS to RAN1 on SL HARQ-ACK reporting to the gNB	RAN2, Huawei

R1-2104163	LS to RAN1 on random value generation for RMTC-SubframeOffset	RAN2, Apple

R1-2104459	Discussion on LS from RAN2 on random value generation for RMTC-SubframeOffset	Ericsson

R1-2104838	Draft reply LS on RMTC-subframeoffset	ZTE, Sanechips

R1-2104839	Discussion on the random value generation for RMTC-subframeoffset	ZTE, Sanechips

R1-2105271	Discussion on RAN2 LS on random value generation for RMTC-SubframeOffset	Nokia, Nokia Shanghai Bell

R1-2105279	Discussion on the random value generation for RMTC-SubframeOffset	Samsung

R1-2105414	Discussion on RAN2 LS on random value generation for RMTC-SubframeOffset	LG Electronics

R1-2105450	Draft Reply LS on random value generation for RMTC-SubframeOffset	vivo

R1-2105933	Discussion on random value generation for rmtc-SubframeOffset	Huawei, HiSilicon

R1-2104164	LS on fallback applicability for UE FeatureSetDownLinkPerCC capability fields	RAN2, Ericsson

R1-2104579	Draft reply LS on fallback applicability for FeatureSetDownLinkPerCC capability fields	ZTE

R1-2105281	Draft reply LS on fallback applicability for UE FeatureSetDownLinkPerCC capability fields			Samsung

R1-2105445	Draft reply LS on fallback applicability for UE FeatureSetDownLinkPerCC capability fields			vivo

R1-2105811	Discussion regarding LS reply on fallback applicability for UE FeatureSetDownLinkPerCC capability fields	Ericsson

R1-2105934	Discussion on fallback applicability for UE FeatureSetDownLinkPerCC capability fields	Huawei, HiSilicon

R1-2104559	LS on R16 V2X for PUCCH reporting and for minimum time gap	RAN2, OPPO

R1-2104641	Draft reply to RAN2 on minimum time gap	Qualcomm Incorporated

R1-2104753	Discussion on PUCCH reporting and for minimum time gap	OPPO

R1-2104754	Draft reply LS on R16 V2X for PUCCH reporting and for minimum time gap	OPPO

R1-2104843	Discussion on the LS from RAN2 on PUCCH reporting and for minimum time gap	CATT, GOHIGH

R1-2104883	Draft reply LS on R16 V2X for PUCCH reporting and for minimum time gap	Intel Corporation

R1-2104996	Draft reply LS on R16 V2X for PUCCH reporting and for minimum time gap	Intel Corporation

Withdrawn

R1-2105282	Draft reply LS on R16 V2X for PUCCH reporting and for minimum time gap	Samsung

R1-2105449	Draft Reply LS on R16 V2X for PUCCH reporting and for minimum time gap	vivo

R1-2105898	[Draft] LS on PUCCH reporting and minimum time gap for V2X	Ericsson

R1-2105899	Discussion on RAN2 LS on PUCCH reporting and for minimum time gap for V2X	Ericsson

R1-2105922	Discussion on HARQ feedback reporting and minimum time gap	Huawei, HiSilicon

Rel-17 NTN

R1-2104155	LS on PDB for new 5QI	SA2, Ericsson

R1-2104726	Discussion on SA2 LS on new 5QI for NTN	Ericsson

R1-2104749	Draft reply LS on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)	OPPO

R1-2104774	Discussion on LS on PDB for new 5Q	OPPO

R1-2105199	Discussion on LS on PDB for new 5QI	ZTE

R1-2105930	Discusion on PDB for new 5QI	Huawei, HiSilicon

R1-2105948	[Draft] LS response on New Standardized 5QIs for 5G-AIS	Qualcomm Inc.

R1-2104172	LS reply on NTN UL time and frequency synchronization requirements	RAN4, CATT

R1-2104230	LS on TA pre-compensation	RAN2, OPPO

R1-2104775	Discussion on LS on TA pre-compensation	OPPO

R1-2105198	Discussion on LS on TA pre-compensation	ZTE

R1-2105481	Draft reply LS on TA pre-compensation	LG Electronics

R1-2105931	Discussion on TA pre-compensation	Huawei, HiSilicon

Rel-17 MBS

R1-2104165	LS on broadcast session delivery and MCCH design	RAN2, Huawei

R1-2104576	[DRAFT]   Reply LS on broadcast session delivery and MCCH design	ZTE

R1-2104597	Discussion on RAN2 LS on broadcast session delivery and MCCH design	CMCC

R1-2105927	Discussion on LS on broadcast session delivery and MCCH design	Huawei, HiSilicon, CBN

Rel-17 1024QAM

R1-2104166	Reply LS on Introduction of DL 1024QAM for NR	RAN2, Ericsson

Rel-17 CovEnh

R1-2104168	Reply LS  on PUCCH and PUSCH repetition	RAN4, Qualcomm

R1-2105447	Discussion on RAN4 Reply LS on PUCCH and PUSCH repetition	vivo

Rel-17 MIMO

R1-2104169	Second reply LS on Beam switching gaps for Multi-TRP UL transmission	RAN4, Nokia

Rel-17 Scell Activation

R1-2104170	Reply LS on temporary RS for efficient SCell activation in NR CA	RAN4, Huawei

Rel-17 IIoT/URLLC

R1-2104171	Reply LS on UE transmit timing error	RAN4, Huawei

Rel-17 60GHz

R1-2104173	Reply LS on maximum/minimum channel bandwidths and channelization for NR operation in 52.6 to 71 GHz	RAN4, Intel

Rel-17 PowSav

R1-2104229	Reply LS on UE Sub-grouping for Paging Enhancement	RAN2, MediaTek

R1-2104467	Discussion on RAN2 Reply LS on UE Sub-grouping for Paging Enhancement	CATT

R1-2104642	Draft LS reply to RAN2 on UE Sub-grouping for Paging Enhancement	Qualcomm Incorporated

R1-2105385	[Draft] Reply LS on UE Sub-grouping for Paging Enhancement	MediaTek Inc.

R1-2105649	Discussion on LS about paging sub-grouping	ZTE, Sanechips

Related to R1-2100021 (LS to RAN1 on SL DRX design by RAN2, ZTE)

R1-2104298	[Draft] Reply LS on SL DRX design	Nokia, Nokia Shanghai Bell

R1-2104465	Discussion on LS from RAN2 on SL DRX design	CATT, GOHIGH

R1-2104719	Discussion on RAN2 LS on DRX impact	Nokia, Nokia Shanghai Bell

R1-2104757	Discussion on SL-DRX working assumption from RAN2	OPPO

R1-2104758	Draft reply LS to RAN2 on SL DRX design	OPPO

R1-2105448	Draft Reply LS on SL DRX design	vivo

R1-2105608	[draft]Reply LS on sidelink DRX	ZTE, Sanechips

R1-2105609	Further consideration of SL DRX	ZTE, Sanechips

R1-2105923	Discussion on RAN2 LS on DRX impact	Huawei, HiSilicon

Related to R1-2102306 (LS on Scheduling Location in Advance to reduce Latency, SA2, Qualcomm)

R1-2104643	Draft reply LS to SA2 on Scheduling Location in Advance	Qualcomm Incorporated

R1-2105937	Discussion on scheduling location in advance to reduce latency	Huawei, HiSilicon

Related to R1-2104023 (LS on Status Update on XR Traffic, SA4, Qulacomm)

R1-2105607	Reply LS to SA4 on Status Update on XR Traffic	ZTE, Sanechips

R1-2105610	About the LS on Status Update on XR Traffic Model	ZTE, Sanechips

R1-2105833	Discussion of SA4 input on XR traffic	Ericsson

R1-2105924	Discussion on LS on Status Update on XR Traffic	Huawei, HiSilicon

//LSs with RAN1 as “cc”, all noted, without dedicated email discussion, unless explicitly requested

R1-2104154	LS on Media-Related Services and Requirements	SA4, Qualcomm

R1-2104156	LS on Time Synchronization assistance parameters	SA2, Nokia

R1-2104167	Response LS on Scheduling Location in Advance to reduce Latency	RAN2, Qualcomm

R1-2104174	Reply LS on Introduction of Cell Grouping UE capability for NR-DC	RAN4, Qualcomm

R1-2104175	LS on Rel-16 updated RAN4 UE features lists for LTE and NR	RAN4, CMCC

R1-2104231	Reply LS to RAN4 on the capability of transparent TxD	RAN2, vivo

## RAN1 Aspects for RF requirements for NR frequency range 1 (FR1)

Refer to RP-202799 for details regarding RAN4-led WID on RF requirements for NR frequency range 1 (FR1)

Limit to maximum 1 contribution per company/organization/university.

Note: This is for Rel-17 only. For any Rel-16 related maintenance, please submit under 7.2.12

[105-e-NR-R17-TxSwitching-01] Email discussion on RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) – Jianchi (China Telecom)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2104245	Discussions on enhancements for UL Tx switching	Huawei, HiSilicon

R1-2104318	Discussion on Rel-17 UL Tx Switching	ZTE

R1-2104468	Discussion on R17 enhancements for UL TX switching	CATT

R1-2104598	Discussion on Rel-17 UL Tx Switching	CMCC

R1-2104644	Discussion on Rel-17 UL Tx switching	Qualcomm Incorporated

R1-2104737	Dsicussion on Rel-17 Tx switching enhancement	OPPO

R1-2104845	Discussion on Rel-17 uplink Tx switching	China Telecom

R1-2105452	Discussion on Rel-17 Tx switching enhancements	vivo

## RAN1 Aspects for NR small data transmissions in INACTIVE state

Refer to RP-210870 for details regarding RAN2-led WID on NR small data transmissions in INACTIVE state

Limit to maximum 1 contribution per company/organization/university.

[105-e-NR-R17-SDT-01] Email discussion on RAN1 Aspects for NR small data transmissions in INACTIVE state) – Li (ZTE)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2104227	On physical layer aspects of small data transmission	Nokia, Nokia Shanghai Bell

R1-2104282	Physical layer aspects of CG-SDT	Huawei, HiSilicon

R1-2104408	Discussion on physical layer aspects of small data transmission	Spreadtrum Communications

R1-2104469	Remaining issues on small data transmission	CATT

R1-2104798	Discussion on NR small data transmissions in INACTIVE state	OPPO

R1-2104840	Discussion on the remaining physical layer issues of small data transmission	ZTE, Sanechips

R1-2104884	Discussion on physical layer aspects of small data transmission	Intel Corporation

R1-2104940	Discussion on physical layer aspects of small data transmission	Intel Corporation

Withdrawn

R1-2104997	Discussion on physical layer aspects of small data transmission	Intel Corporation

Withdrawn

R1-2105073	Discussion on physical layer aspects of small data transmission	Apple

R1-2105283	Discussion on physical layer aspects for NR small data transmissions in INACTIVE state	Samsung

R1-2105415	Discussion on physical layer aspects of small data transmission	LG Electronics

R1-2105453	Discussion on RAN1 impacts for small data transmisison	vivo

R1-2105471	Physical layer aspects of small data transmission	InterDigital, Inc.

R1-2105508	Discussion on RAN1 aspects for NR small data transmissions in INACTIVE state	L.M. Ericsson Limited

# E-UTRA (7)

## Maintenance of E-UTRA Releases 8 – 15 (3)

[105-e-LTE-6.1CRs-01] Email discussion/approval on R1-2104575 by May 24 - Youjun (ZTE)

//This one is to use NWM – please use RAN1-105-e-NWM-LTE-6.1CRs-02 as the document name

[105-e-LTE-6.1CRs-02] Email discussion/approval on R1-2105953 (DCI format N0 when scrambled by SPS C-RNTI) and R1-2105954 (UL index for LTE-M) by May 24 - Youjun (ZTE)

[105-e-LTE-6.1CRs-03] Email discussion/approval on R1-2105398 and R1-2105940 by May 24 - Huiying (ZTE)

R1-2104575	Correction for MBSFN reference signal mapping to resource elements for 1.25 kHz	ZTE

R1-2105265	Correction on DCI format N0 when scrambled by SPS C-RNTI	ZTE

R1-2105266	Correction on UL index for LTE-M	ZTE

R1-2105398	Clarification on NPUSCH postponement for NB-IoT	ZTE, Sanechips, Lenovo, Motorola Mobility

R1-2105940	Discussion on NPUSCH postponement when overlapping with NPRACH	Huawei, HiSilicon

## Maintenance of E-UTRA Release 16 (2+1+1=4)

Only essential corrections

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”, unless otherwise noted

- In case when the number of corrections is within the contribution limit, either a TP format with necessary justification or a draft CR format is required

- In case when the number of corrections exceeds the contribution limit, please observe the contribution limit by including one or more TPs in the contribution(s), where each TP is provided necessary justification

### Maintenance of Additional MTC Enhancements (2)

For this agenda, no more than 2 contribution per 1 company/organization/university

//This one is to use NWM – please use RAN1-105-e-NWM-LTE-eMTC5-01 as the document name

[105-e-LTE-eMTC5-01] Reply LS related to RSS based RSRQ for LTE-MTC - ??? (Huawei)

- Discuss and decide on a potential reply LS to this incoming LS:

  - R1-2104159, “Reply LS related to RSS based RSRQ for LTE-MTC”, RAN2, Huawei, HiSilicon

- Related contributions:

  - R1-2104709, “Discussion on RAN2 reply LS on RSS based RSRQ”, ZTE

  - R1-2104818, “Discussion on RSS-based measurements”, Qualcomm Incorporated

  - R1-2105941, “Discussion on RSS based RSRQ for LTE-MTC”, Huawei, HiSilicon

- Decision by May 25

[105-e-LTE-eMTC5-02] Clarification of UE procedure for UL multi-TB scheduling in TDD for LTE-MTC – Johan (Ericsson)

- Discuss and decide on the potential clarification in 36.213 discussed in these contributions:

  - R1-2105267, “Clarification on UE procedure for uplink MTB scheduling in TDD”, ZTE

  - R1-2105268, “Discussion on UE procedure for uplink MTB scheduling in TDD”, ZTE

- Discussion and decision by May 24, TPs by May 27

R1-2105267	Clarification on UE procedure for uplink MTB scheduling in TDD	ZTE

R1-2105268	Discussion on UE procedure for uplink MTB scheduling in TDD	ZTE

### Maintenance of Additional Enhancements for NB-IoT (1)

For this agenda, no more than 2 contribution per 1 company/organization/university

[105-e-LTE-NB_IoTenh3-01] Correction on DMRS cyclic shift for PUR in NB-IoT – Xiang (Huawei)

- Discussion and decision by May 24, TPs by May 27

R1-2105269	Correction on DMRS cyclic shift for PUR in NB-IoT	ZTE

### Maintenance of DL MIMO efficiency enhancements for LTE

### Maintenance of LTE-based 5G Terrestrial Broadcast (1)

R1-2104574	Correction for 0.37kHz SCS	ZTE

[105-e-LTE-Terr-Bcast-01] Email/discussion/approval on R1-2104574 by May 21 – Alberto (Qualcomm)

### LTE Rel-16 UE Features

### Others

# NR Maintenance (15+32=47)

## Maintenance of Release 15 NR (15)

Only essential corrections – a rejected draft CR will be marked in red

Including CRs rejected for Rel-15, but can be considered for Rel-16. For each of these contributions, please clearly indicate that it’s originated from Rel-15 maintenance discussion (but not adopted), and is for Rel-16 maintenance consideration only

R1-2105972	RAN1#105-e preparation phase initial summary on NR Rel-15 CRs	Ad-hoc chair (Samsung)

R1-2105980	RAN1#105-e preparation phase final summary on NR Rel-15 CRs	Ad-hoc chair (Samsung)

Issue#5

R1-2104885	PDSCH processing time per Capability 2 and DCI format 1_0	Intel Corporation

[105-e-NR-7.1CRs-01] Issue#5: PDSCH processing time per Capability 2 and DCI format 1_0 – Debdeep (Intel) by May 25

Issue#6, Issue#10

R1-2105079	Discussions on PUSCH UCI Multiplexing without HARQ-ACK PUCCH in Rel-15	Apple

R1-2105390	Clarification on HARQ-ACK Information Feedback without Receiving DL assignment/PDSCH			MediaTek Inc.

[105-e-NR-7.1CRs-02] Issue#6 and Issue#10: Discussions on PUSCH UCI Multiplexing without HARQ-ACK PUCCH – Sigen (Apple) by May 25

Issue#7

R1-2105278	DRX interaction with CPU occupancy	Nokia

R1-2105349	Discussion on DRX interaction with CPU occupancy	Nokia

[105-e-NR-7.1CRs-03] Issue#7: DRX interaction with CPU occupancy – Filippo (Nokia) by May 25

- For Rel-16 only

Issue#9

R1-2105389	Clarification on PHR Report for SRS Carrier		MediaTek Inc.

[105-e-NR-7.1CRs-04] Issue#9: Clarification on PHR Report for SRS Carrier – Delegate (MediaTek) by May 25

Issue#15

R1-2105539	Correction on enabling configuration of time restriction over L1-RSRP and CSI measurement			Huawei, HiSilicon

[105-e-NR-7.1CRs-05] Issue#15: Correction on enabling configuration of time restriction over L1-RSRP and CSI measurement – Xinghua (Huawei) by May 21

Issue#16

R1-2105625	Correction on channel properties assumption of UL transmission	Sharp

[105-e-NR-7.1CRs-06] Issue#16: Correction on channel properties assumption of UL transmission – Delegate (Sharp) by May 25

Issue#17

R1-2105741	Clarification on back-to-back PUSCHs scheduling restriction in Rel-15	MediaTek Inc.

[105-e-NR-7.1CRs-07] Issue#17: Clarification on back-to-back PUSCHs scheduling restriction – Delegate (MediaTek) by May 25

Issue#20

R1-2105919	Correction on PDCCH blind detection capability for CA in TS38.213	Huawei, HiSilicon

[105-e-NR-7.1CRs-08] Issue#20: Correction on PDCCH blind detection capability for CA in TS38.213 – Delegate (Huawei) by May 25

Issue#23

R1-2105077	Correction on UL cancellation due to DCI format 2_0	Apple

R1-2105078	Correction on UL cancellation due to DCI format 2_0	Apple

[105-e-NR-7.1CRs-09] Issue#23: Correction on UL cancellation due to DCI format 2_0 – Delegate (Apple) by May 25

Issue#24

R1-2105456	Draft 38.212 CR on spreading factor for PUCCH format 4	vivo

[105-e-NR-7.1CRs-10] Issue#24: Draft 38.212 CR on spreading factor for PUCCH format 4 – Na (vivo) by May 21

Issue#25

R1-2104299	Remaining issues with PUSCH skipping (without LCH and PHY prioritization) (Rel-16)	Nokia, Nokia Shanghai Bell, Ericsson

R1-2104300	Draft CR on PUSCH skipping with repetition (Rel-16)	Nokia, Nokia Shanghai Bell

R1-2104316	Discussion on UL skipping for PUSCH	ZTE

R1-2104409	Discussion on UL skipping for PUSCH in Rel-16	Spreadtrum Communications

R1-2104471	Discussion on remaining issue of PUSCH skipping	CATT

R1-2104646	Discussion on PUSCH skipping	Qualcomm Incorporated

R1-2104809	Discussion on PUSCH skipping with UCI overlapping	OPPO

R1-2105076	Discussions on PUSCH skipping in Rel-16	Apple

R1-2105458	Discussion on PUSCH skipping with overlapping UCI on PUCCH in Rel-16	vivo

R1-2105459	Correction on 38.213 for UL skipping with PUSCH repetitions in Rel-16	vivo

R1-2105520	Discussion on UL skipping	Huawei, HiSilicon

R1-2105730	Discussion on UL skipping for PUSCH repetition	NTT DOCOMO INC.

[105-e-NR-7.1CRs-11] Issue#25: Remaining issues on UL skipping for PUSCH – Xiaohang (vivo) by May 27

- For Rel-16 only

Issue#26

R1-2104580	Discussion on SRS carrier switching	ZTE

R1-2104581	Draft CR on SRS carrier switching	ZTE

R1-2104647	Discussion on SRS carrier switch	Qualcomm Incorporated

R1-2105074	Timeline Considerations for SRS Carrier Switching and Dropping Procedure in Rel-16	Apple

R1-2105936	Correction on prioritization rules of SRS carrier switching	Huawei, HiSilicon

[105-e-NR-7.1CRs-12] Issue#26: SRS carrier switching – Chuangxin (ZTE) by May 25

- For Rel-16 only

Issue#28

R1-2105075	Correction for HARQ-ACK timing in Rel-16	Apple, Ericsson

[105-e-NR-7.1CRs-13] Issue#28: Correction for HARQ-ACK timing in Rel-16 – Sigen (Apple) by May 25

- For Rel-16 only

Issue#1, Issue#4, Issue#8, Issue#13, Issue#21, Issue#27

R1-2104317	Correction on multi-slot PUSCH transmission	ZTE

R1-2104810	Draft CR on PUCCH repetition	OPPO

R1-2105284	Correction on power control for TS 38.213	Samsung

R1-2105455	Draft 38.214 CR on reference correction for PDSCH EPRE to CSIRS EPRE assumption	vivo

R1-2105926	Correction on the MR-DC Uplink Power Control in 38.213	Huawei, HiSilicon

R1-2104645	Draft CR on Type-1 HARQ-ACK for PDSCH repetition with different DL and UL SCSs	Qualcomm Incorporated

[105-e-NR-7.1CRs-14] Discussion on editorial spec changes for recommendation to the editors (Issues #1, #4, #8, #13, #21, #27) – Sungjin (Samsung) by May 21

Issue#2, Issue#3, Issue#11, Issue#12, Issue#14, Issue#18, Issue#19, Issue#22

R1-2104470	Discussion on overlapping between positive SR and PUSCH without UL-SCH	CATT

R1-2104472	Draft CR on overlapping between PUSCH transmission and SRS transmission	CATT

R1-2105391	Clarification on PUSCH with UCI Only and DMRS Multiplexing	MediaTek Inc.

R1-2105454	Discussion on CSI-RS multiplexing with PDSCH scheduled by PDCCH with CRC scrambled by P-RNTI and SI-RNTI	vivo

R1-2105457	Disucssion on CSI-RS Resource counting for UE feature 2-33	vivo

R1-2105803	Discussion on PDCCH monitoring in overlapping CORESETs	ASUSTEK COMPUTER (SHANGHAI)

R1-2105835	UE feature for 2-port CSI-RS	Ericsson

R1-2105938	Correction of initial DL BWP configuration	Huawei, HiSilicon

[105-e-NR-7.1CRs-15] Review of issues not discussed under any of the emails threads [105-e-NR-7.1CRs-01] ~ [105-e-NR-7.1CRs-14] – Youngbum (Samsung) by May 24

- Decide whether to reject or continue discussions in future meetings

R1-2104941	PDSCH processing time per Capability 2 and DCI format 1_0	Intel Corporation

Withdrawn

R1-2104942	On UL skipping and PUSCH repetitions	Intel Corporation

Withdrawn

R1-2104998	PDSCH processing time per Capability 2 and DCI format 1_0	Intel Corporation

Withdrawn

R1-2104999	On UL skipping and PUSCH repetitions	Intel Corporation

Withdrawn

R1-2104886	On UL skipping and PUSCH repetitions	Intel Corporation

Withdrawn

R1-2105587	Discussion on PUSCH with ULSkipping and repetition	Ericsson

Withdrawn

## Maintenance of Release 16 NR (1+4+7+5+4+1+2+1+1+4+2=32)

Only essential corrections

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”, unless otherwise noted

- In case when the number of corrections is within the contribution limit, either a TP format with necessary justification or a draft CR format is required

- In case when the number of corrections exceeds the contribution limit, please observe the contribution limit by including one or more TPs in the contribution(s), where each TP is provided necessary justification

R1-2105395	Editorial corrections for 38.214	Nokia

[105-e-NR-AlignmentCRs-Spec#] Email discussion/approval on Rel-16 alignment CRs till 4/26 (Editors)

- For 38.214, including R1-2105395

### Maintenance of Two step RACH for NR (1)

R1-2105745	FL summary on the maintenance of 2-step RACH	Moderator (ZTE)

[105-e-NR-2step-RACH-01] Discuss the potential LS to RAN2 on the description of RRC parameter p0-AlphaSets and whether 4-step RACH can be absent for any of the BWPs, till 5/24 – Li (ZTE)

As part of the preparation, the draft CR in R1-2104474 is endorsed, to be included in 38.211 alignment CR.

Update on 5/24:

- Draft LS in R1-2106128 is approved. Final LS in R1-210xxxx.

R1-2104474	Correction on higher layer parameter for MsgA PUSCH DMRS	CATT

R1-2105507	Discussion on corrections for 2-step RACH	L.M. Ericsson Limited

### Maintenance of NR-based Access to Unlicensed Spectrum (4)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2105942	Feature lead summary for NR-U DL Signals and Channels	Moderator (Lenovo)

R1-2104271	Maintenance on channel access and HARQ procedures for NR Unlicensed	Huawei, HiSilicon

R1-2104272	Maintenance on DL signals and channels for NR Unlicensed	Huawei, HiSilicon

R1-2104458	Corrections related to HARQ	Ericsson

R1-2104459	Discussion on LS from RAN2 on random value generation for RMTC-SubframeOffset	Ericsson

R1-2104476	Discussion on enhanced dynamic HARQ-ACK codebook for secondary PUCCH group	CATT

R1-2104648	Invalid SSB in FBE	Qualcomm Incorporated

R1-2104763	Discussion on the remaining issues with shared spectrum operation determination	OPPO

R1-2104764	Discussion on the remaining issues of HARQ enhancements	OPPO

R1-2104831	Discussion on the UE behavior of CSI-RS report during SCell activation	ZTE, Sanechips

R1-2104832	Maintenance of channel access for NR-U	ZTE, Sanechips

R1-2105071	Correction on joint search space set group switching across multiple cells	DENSO CORPORATION

R1-2105080	Clarification on NR-U RRM measurement operation	Apple

R1-2105271	Discussion on RAN2 LS on random value generation for RMTC-SubframeOffset	Nokia, Nokia Shanghai Bell

R1-2105416	Discussion on RAN4 LS on measuring CSI-RS during SCell activation	LG Electronics

R1-2105460	TP on frequency hopping for NR-U configured grant	vivo

R1-2105461	Maintenance on HARQ operation for NR-U	vivo

R1-2105753	Correction of higher layer parameter name for NR-U	ITRI

R1-2105805	Initial UL BWP size restriction for NR-U	Sharp, LG Electronics

[105-e-NR-NRU-01] Email discussion/approval on measurement during SCell activation (DL-B2) until May 21 – Alex (Lenovo)

//This one is to use NWM – please use RAN1-105-e-NWM-NR-NRU-02 as the document name

[105-e-NR-NRU-02] Email discussion/approval on clarification of size of initial UL BWP (UL-01) until May 24 – Steve (Ericsson)

[105-e-NR-NRU-03] Email discussion/approval on HARQ enhancements focusing on the following from preparation phase summary until May 25 – David (Huawei)

- HARQ-1, HARQ-5, HARQ-6: Editorial

- HARQ-2 and HARQ-3

[105-e-NR-NRU-04] Email discussion/approval on reply LS to R1-2104163 until May 24 – Hong (Apple)

### Maintenance of Integrated Access and Backhaul for NR

### Maintenance for 5G V2X with NR sidelink (7)

For this agenda, no more than 3 contribution per 1 company/organization/university

[105-e-NR-5G_V2X-01] Email discussion/approval regarding

- Issue PS-0: Editorial corrections for physical layer structure

- Issue PS-2: SCS offset in SL BWP and UL BWP

till 5/24 with any follow-up TPs till 5/26 – Jeongho (Samsung)

Update on 5/24:

Agreements:

- Two TPs in Appendix 6.1 and 6.2 are adopted.

- 0 is the largest value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList for each of uplink and downlink and by sl-SCS-SpecificCarrierList for sidelink.

Draft CRs? 5/26

[105-e-NR-5G_V2X-02] Email discussion/approval regarding

- Issue M1-1-1: SL HARQ-ACK reporting when SL FB is not used (considering LS in R1-2104559)

till 5/24 with any follow-up TPs till 5/26 – Ricardo (Ericsson)

Update on 5/24:

Conclusion:

- Capture in TS 38.213 Clause 16.5 the necessary text to specify the UE behaviour for reporting HARQ-ACK on uplink when SL feedback is disabled for a TB (as per existing agreements):

  - The contents of the HARQ-ACK report on uplink is provided by higher layer.

  - The priority value of the HARQ-ACK report is the same as the priority value of the corresponding PSSCH transmission(s).

Draft CRs? 5/26

[105-e-NR-5G_V2X-03] Email discussion/approval regarding

- Issue M1-2-1: Value of n_CI

till 5/24 with any follow-up TPs till 5/26 – Ricardo (Ericsson)

Update on 5/24: to check on 5/25 (afternoon)

[105-e-NR-5G_V2X-04] Email discussion/approval regarding

- Issue M1-4: TPs corresponding to agreements in previous meetings (Agreement/LS from RAN1#104, reply LS received in R1-2104160, M1-2-2: DCI size alignment)

till 5/24 – Ricardo (Ericsson)

Update on 5/24:

- 38.214, 8.1.2.1 TP is endorsed  to be part of 38.214 alignment CR.

Draft CR for M1-4? 5/25

[105-e-NR-5G_V2X-05] Email discussion/approval regarding

- Issue M2-1: TP to implement the agreement from [104b-e-NR-5G_V2X-03]

till 5/24 – Sergey (Intel)

Update on 5/24: to check on 5/25

Update on 5/25:

Conclusion:

- In the following agreement made in [104b-e-NR-5G_V2X-03], “a UE skips step 5” is interpreted as that the UE re-initializes S_A to the set of all the candidate single-slot resources as in step 4 and proceeds to step 6.

- Update the specification of identification of candidate resources for Mode-2 resource allocation in section 8.1.4 of TS 38.214 to handle the case when X·M_total number of identified resources could not be reached after any number of loop iterations - If the number of the excluded resources in step 5 is larger than (1-X)·M_total, a UE skips step 5

The latest TP is endorsed.

CR for approval? 5/26

[105-e-NR-5G_V2X-06] Email discussion/approval regarding

- Issue PP-1: TP for multiplexing SL HARQ-ACK reports on a PUSCH

till 5/24 – Hanbyul (LGE)

Update on 5/24:

Conclusion:

Clarification is necessary on which bit value, 0 or 1, in sl-PSFCH-RB-Set is used for determination of PSFCH PRBs and leaves RAN2 to make the corresponding spec update.

- Send an LS to RAN2

Latest draft LS is approved. Final LS in R1-210xxxx.

The first two editorial 38.213 TPs (section 16.2.1., section 16.2.4.3.1) are endorsed. To be forward to 38.213 editor.

Other proposals? 5/25

[105-e-NR-5G_V2X-07] Email discussion/approval regarding

- LS reply to R1-2104559 taking into account the outcome of Thread #2 and Issue M2-5 (HARQ RTT time gap capturing issue in MAC)

till 5/25 – Name Kevin (OPPO)

R1-2104194	TP to address infinite loop due to excessive resource exclusion for Rel. 16 V2X	FUTUREWEI

R1-2104235	Remaining issues for sidelink physical layer procedure	Huawei, HiSilicon

R1-2104477	Discussion and TP on Mode1 resource allocation	CATT, GOHIGH

R1-2104478	Correction on SL HARQ-ACK report piggybacked on PUSCH	CATT, GOHIGH

R1-2104649	Remaining Issues in physical layer procedure	Qualcomm Incorporated

R1-2104750	Remaining open issues and corrections for mode 1 RA	OPPO

R1-2104751	Discusssion on TPs for skipping step 5 in mode 2 RA	OPPO

R1-2104752	Remaining open issues and corrections for physical layer procedure	OPPO

R1-2104887	Correction to sidelink resource identification procedure to prevent infinite loop issue – implementation of the agreement from [104b-e-NR-5G_V2X-03]	Intel Corporation

R1-2104890	Correction to PSFCH reception procedure for NACK-only case to mitigate half-duplex issue			Intel Corporation

R1-2104943	Correction to sidelink resource identification procedure to prevent infinite loop issue – implementation of the agreement from [104b-e-NR-5G_V2X-03]	Intel Corporation

Withdrawn

R1-2105000	Correction to sidelink resource identification procedure to prevent infinite loop issue – implementation of the agreement from [104b-e-NR-5G_V2X-03]	Intel Corporation

Withdrawn

R1-2105056	Maintenance for mode-1 resource allocation for NR sidelink	Fujitsu

R1-2105057	Maintenance for physical layer procedures for NR sidelink	Fujitsu

R1-2105081	On Remaining Issue of Mode 2 Resource Allocation	Apple

R1-2105082	Maintenance of Sidelink Physical Layer Procedure	Apple

R1-2105201	Discussion on essential corrections in physical layer procedure	LG Electronics

R1-2105202	Discussion on essential corrections in resource allocation procedure	LG Electronics

R1-2105210	Corrections for transmitting sidelink reference signals in TS 38.214	ETRI

R1-2105232	Corrections for transmitting sidelink reference signals in TS 38.214	ETRI

Withdrawn

R1-2105251	Remaining issues on physical layer structure	NEC

R1-2105252	Remaining issues on resource allocation mode 2	NEC

R1-2105462	Maintenance on NR sidelink mode-1 resource allocation mechanism	vivo

R1-2105463	Maintenance on NR sidelink mode-2 resource allocation mechanism	vivo

R1-2105464	Maintenance on NR sidelink synchronization and procedure	vivo

R1-2105611	Remaining issues on mode 1	ZTE, Sanechips

R1-2105612	Remaining issues on mode 2	ZTE, Sanechips

R1-2105613	Miscellaneous corrections of TS38.212_214	ZTE, Sanechips

R1-2105626	Remaining issues on physical layer structure for NR sidelink	Sharp

R1-2105627	Remaining issues on resource allocation for NR sidelink	Sharp

R1-2105628	Remaining issues on synchronization mechanism and QoS management for NR sidelink	Sharp

R1-2105680	Maintenance for resource allocation mechanism mode 1	NTT DOCOMO, INC.

R1-2105681	Maintenance for sidelink physical layer procedure	NTT DOCOMO, INC.

R1-2105740	Remaining issues on resource allocation mode-1 and sidelink procedure	ASUSTeK

R1-2105841	Remaining issues on sidelink mode 2	ASUSTeK

R1-2105895	Corrections to SL procedures	Ericsson

R1-2105896	Corrections to Mode 1	Ericsson

R1-2105897	Condition to stop the infinite loop for Mode 2 RA	Ericsson

R1-2105920	Correction on resource exclusion for other TBs	Huawei, HiSilicon

R1-2105921	Correction on PSSCH-DMRS time-domain OCC	Huawei, HiSilicon

R1-2105943	Maintenance for Resource allocation for sidelink - Mode 1	Nokia, Nokia Shanghai Bell

R1-2105944	Maintenance for Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

### Maintenance of Physical Layer Enhancements for NR URLLC (5)

Including maintenance for IIoT

For this agenda, no more than 3 contribution per 1 company/organization/university

[105-e-NR-L1enh-URLLC-01] Email discussion/approval on remaining issues on PDCCH enhancements – Chengyan (Huawei):

- Issue #2: Correction on relative SLIV reference for Type 1 HARQ codebook

- Issue #4: Remove DCI format 1_1 indicating SCell dormancy in case of 1-bit C-DAI

- Issue #6: Correction on the number of SRS resource set configuration

- Issue #3 (editor CR): Correction on RRC parameter in DCI formats when two HARQ-ACK codebooks are configured

- Issue #5 (editor CR): Editorial correction on

- Discussion and decision by May 24, TPs by May 27

[105-e-NR-L1enh-URLLC-02] Email discussion/approval on remaining issues on UCI enhancements – Jia (OPPO):

- Issue #1: Correction for sub-slot based PUCCH

- Issue #2: Conflict between the first PUCCH repetition and semi-static configuration

- Issue #3: Clarification on reference point of sub-slot based PUCCH resource

- Issue #5 (editor CR): Editorial correction on timing for secondary cell activation/deactivation

- Discussion and decision by May 24, TPs by May 27

[105-e-NR-L1enh-URLLC-03]  Email discussion/approval on remaining issues on Scheduling & HARQ enhancements – Wei (Qualcomm):

- Issue #2: Handling of collision between DL/SSB symbols and configured HP PUCCH and PUSCH

- Issue #1: Clarification on UE procedure for prioritization

- Discussion and decision by May 24, TPs by May 27

[105-e-NR-L1enh-URLLC-04] Email discussion/approval on remaining issues on intra-UE prioritization and multiplexing – Lihui (Vivo):

- Discuss whether to confirm RAN2’s working assumption on UL skipping vs. LCH-based prioritization.

- Discuss PHY impacts/behavior on intra-UE prioritization and multiplexing.

- Discussion and decision by May 24, TPs by May 27

[105-e-NR-L1enh-URLLC-05] Email discussion/approval on remaining issues on SPS enhancements – Duckhyun (LG):

- Issue #1: SPS PDSCH release and SPS receptions with slot aggregation

- Issue #3: CSI-PUCCH-ResourceList where SPS HARQ-ACK multiplexed

- Issue #2: Capture the agreement on SPS release that is not supported (to be discussed after decision made on issue #1)

- Discussion and decision by May 24, TPs by May 27

R1-2104214	Intra-UE Multiplexing and Prioritization for Rel-16 URLLC	Ericsson

R1-2104215	Maintenance of PDCCH and SPS for Rel-16 NR URLLC	Ericsson

R1-2104216	Maintenance of Scheduling and HARQ for Rel-16 NR URLLC	Ericsson

R1-2104311	Rel-16 URLLC/IIoT maintenance of UCI enhancements	Nokia, Nokia Shanghai Bell

R1-2104312	Rel-16 URLLC/IIoT maintenance of PDCCH, Scheduling/HARQ and SPS enhancements	Nokia, Nokia Shanghai Bell

R1-2104313	Rel-16 URLLC/IIoT PUSCH skipping (with LCH and/or PHY prioritization configured)	Nokia, Nokia Shanghai Bell

R1-2104321	Remaining issues on SPS enhancement in Rel-16 URLLC	ZTE

R1-2104322	Remaining issues on intra-UE multiplexing in Rel-16 URLLC	ZTE

R1-2104323	Discussion on PDSCH processing timeline for DCI format 1_2	ZTE

R1-2104410	Maintenance on PDCCH enhancements for Rel-16 NR URLLC	Spreadtrum Communications

R1-2104479	Discussion on overlapping between CG PUSCH and DG PUSCH	CATT

R1-2104480	Remaining issues on PUCCH transmission	CATT

R1-2104481	Remaining issues on HARQ codebook	CATT

R1-2104650	Remaining issues on eCG enhancements for URLLC	Qualcomm Incorporated

R1-2104799	Maintenance on PDCCH for UE PDSCH processing time for DCI format 1_2	OPPO

R1-2104800	Remaining issues on  scheduling and HARQ	OPPO

R1-2104801	Maintenance on SPS enhancements	OPPO

R1-2105083	UCI multiplexing and PUSCH skipping design in URLLC	Apple

R1-2105084	Remaining issues on intra-UE multiplexing/prioritization for eURLLC	Apple

R1-2105285	Discussion on PUSCH skipping for URLLC	Samsung

R1-2105286	Maintanence on PDCCH as PDSCH SLIV reference	Samsung

R1-2105417	PHR issues related to URLLC/IIOT WI	LG Electronics

R1-2105418	Remaining issues of other aspects for URLLC/IIOT	LG Electronics

R1-2105465	Maintenance on PDCCH enhancement	vivo

R1-2105466	Maintenance on UCI enhancement	vivo

R1-2105467	Maintenance on eCG enhancement and intra-UE prioritization	vivo

R1-2105531	Remaining issues on UCI enhancements and SPS	Huawei, HiSilicon

R1-2105532	On LCH prioritization and UL skipping	Huawei, HiSilicon

R1-2105682	Corrections on scheduling/HARQ for Rel-16 URLLC	NTT DOCOMO, INC.

R1-2105851	Release of UL grant type 2 PUSCH or SPS PDSCH configurations	ASUSTeK

R1-2105867	Maintenance of PDCCH enhancements for Rel-16 URLLC	WILUS Inc.

R1-2105928	Remaining issues on PDCCH enhancements	Huawei, HiSilicon

### Maintenance of Enhancements on MIMO for NR (4)

For this agenda, no more than 2 contribution per company/organization/university

R1-2105287	Summary for Rel.16 NR eMIMO maintenance	Moderator (Samsung)

[105-e-NR-eMIMO-01] Maintenance for Multi-Beam 1: addressing MB.2 (E), MB.3 (H) – Jiwon (LGE)

- Discussion and decision by May 21, TPs by May 27

[105-e-NR-eMIMO-02] Maintenance for Multi-Beam 2: addressing MB.5 (E), MB.4 (ND), MT.8 (ND) – Yushu (Apple)

- Discussion and decision by May 21, TPs by May 27

[105-e-NR-eMIMO-03] Maintenance for Multi-TRP 1: addressing MT.10 (E), MT.2 (H), MT.4 (ND), MT.9 (ND) – Li (OPPO)

- Discussion and decision by May 21, TPs by May 27

[105-e-NR-eMIMO-04] Maintenance for Multi-TRP 2: addressing MT.5 (H), MT.1 (ND), MT.6 (ND) – Li (OPPO)

- Discussion and decision by May 21, TPs by May 27

R1-2104407	Maintenance on multi-TRP transmission	Lenovo, Motorola Mobility

R1-2104482	Correction on power control for PUSCH	CATT

R1-2104582	Maintenance of multi-beam operation	ZTE

R1-2104583	Maintenance of Multi-TRP enhancements	ZTE

R1-2104651	Remaining Issues on Multi-TRP Enhancements	Qualcomm Incorporated

R1-2104728	Text proposals for overlapping between PUSCH/HARQ and CSI	OPPO

R1-2104729	Text proposals for TCI state activation	OPPO

R1-2105085	Remaining issues on Rel-16 Multi-TRP enhancement	Apple

R1-2105288	On Rel.16 multi-TRP/panel transmission	Samsung

R1-2105957	On Rel-16 multi-beam maintenance	Samsung

Revision of R1-2105289

R1-2105351	Discussion on DRX interaction with CPU occupancy	Nokia

R1-2105352	DRX interaction with CPU occupancy	Nokia

R1-2105468	Maintenance on number of SRS resource set(s) for DCI format 0_1 and DCI format 0_2	vivo

R1-2105469	Maintenance on beam related issues	vivo

R1-2105537	Correction on enabling configuration of time restriction over L1-SINR measurement	Huawei, HiSilicon

R1-2105538	Corrections on RRC names and interpretation for Multi-TRP	Huawei, HiSilicon

R1-2105809	Draft CR on DL SPS based PDSCH repetitions	Ericsson

R1-2105810	Maintenance for single-DCI based multi-TRP in Rel-16	Ericsson

R1-2105842	Interoperation between cross-carrier scheduling and multiple TRPs	ASUSTeK

### Maintenance of UE Power Saving for NR (1)

[105-e-NR_UE_Pow_Sav-01] Email discussion/approval regarding issues #1, #2, and #3 as in the summary – till 54/24 – Fangchen (CATT)

- Note: for issue #2, the conclusion from RAN1#104b-e still holds – there is no intention to further discuss the conclusion itself.

Update on 5/24:

Conclusion:

- Regarding the issue raised in R1-210425, the current text in the specification is sufficient for the scenario when SUL is configured without further clarification.

Agreements:

- The two 38.214 draft TPs are endorsed.

Draft CRs? 5/25

R1-2104254	Remaining issues for Rel-16 UE power saving	Huawei, HiSilicon

R1-2105086	Maintenace of UE power saving for NR	Apple

R1-2105788	Maintenance for Rel-16 UE power savings	Ericsson

### Maintenance of NR positioning support (2)

R1-2104276	Correction to PRS processing priority	Huawei, HiSilicon

R1-2104483	Discussion and TP on remaining issues in NR positioning	CATT

R1-2104584	Clarification on UE Rx-Tx time difference measurements	ZTE

R1-2104738	Corrections on DL PRS resource configruation	OPPO

R1-2105470	Maintenance on Rel-16 NR positioning	vivo

R1-2105518	Draft CR on measurement gap description for positioning	Nokia, Nokia Shanghai Bell

R1-2105907	Maintenance on Rel-16 NR positioning	Ericsson

[105-e-NR-Pos-01] Email discussion/approval on the following until May 25 – Alexey (Intel)

- Aspect #1: DL PRS processing priority

- Aspect #2: DL PRS numerology

- Aspect #3: Clarification on UE Rx-Tx time difference measurements

[105-e-NR-Pos-02] Email discussion/approval on the following until May 24 – Florent (Ericsson)

- Aspect #4: DL PRS periodicity and muting repetition factor

- Aspect #6: On MG request inside of the active DL BWP

- Aspect #7: On MG for NR Positioning

### Maintenance of NR Mobility Enhancements (1)

R1-2105834	Correction to DAPS HO	Ericsson

//This one is to use NWM – please use RAN1-105-e-NWM-NR-MobEnh-01 as the document name

[105-e-NR-MobEnh-01] Email discussion/approval on R1-2105834 until May 24 - Daewon (Intel)

### Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (1)

R1-2105864	Moderator summary of MR DC-CA pre-meeting preparation phase	Moderator (Nokia)

R1-2104324	Remaing issues for Rel-16 single uplink Tx	ZTE

R1-2104475	Correction for power control of NR-DC	CATT

R1-2105375	Remaining issues on Rel-16 carrier aggregation	MediaTek Inc.

R1-2105524	Discussion on half duplex operation for TDD CA	Huawei, HiSilicon

R1-2105789	Maintenance for Rel-16 MR-DCCA	Ericsson

R1-2105863	Removal of FR2-FR2 dual connectivity from Rel-16	Nokia, Nokia Shanghai Bell

R1-2105918	Corrections on CCS with different subcarrier spacings for PDCCH and PDSCH in TS 38.214			Huawei, HiSilicon

[105-e-NR-MRDC-CA-01] Email discussion/approval on the following until May-25 – Karri (Nokia)

- 1Tx: Discuss the changes proposed in R1-2104324 to LTE 36.212/213

- PC1: Discuss the proposed changes in R1-2104475 and if agreed, introduce them together with another change to 38.213, or include in the editor’s alignment CR

- UA: Discuss the proposed changes in R1-2105375 to TS38.214

- XCC: Discuss the proposed change in R1-2105918 and if agreed, could also be taken together with another change to 38.214 or include in the editor’s alignment CR

### NR Rel-16 UE Features (4)

[105-e-NR-UEFeature-LS-01] Update LS & spreadsheet regarding Rel-16 UE features to RAN2 till 5/27 – Hiroki (DCM)/Ralf (AT&T)

R1-2104652	Discussion on NR Rel-16 UE features	Qualcomm Incorporated

R1-2105790	Remaining details of Rel-16 NR UE features	Ericsson

R1-2105814	Remaining issues on NR Rel-16 UE features	Nokia, Nokia Shanghai Bell

[105-e-NR-UEFeatures-eMIMO-01] Email discussion/approval on reply LS to R1-2104164, till 5/24 (Ralf, AT&T)

Update on 5/21:

Agreement:

- RAN1 confirms RAN2’s understanding and furthermore, from RAN1 perspective, there is no impact to consider the capability as part of the fallback concept.

  - Use R1-2104579 as draft reply LS on fallback applicability for FeatureSetDownLinkPerCC capability fields (ZTE)  Final LS in R1-2106133.

[105-e-NR-UEFeature-Others-01] Email discussion/approval on UE features that are not dedicated to specific Rel-16 WI/TEI, till 5/24 (Hiroki, NTT DOCOMO)

- For FG22-10, add below sentence to the note.

  - For pdcch-MonitoringAnyOccasionsWithSpanGap, the supported set (set1, set2 or set 3) for cross-carrier scheduling with the different SCSs in the scheduling cell and the scheduled cell is still based on the indicated value for the band of the scheduling cell.

Update on 5/24:

Agreement:

- For FG22-10, add below sentence to the note in RAN1 UE features list to be sent to RAN2.

  - For pdcch-MonitoringAnyOccasionsWithSpanGap, the supported set (set1, set2 or set 3) for cross-carrier scheduling with the different SCSs in the scheduling cell and the scheduled cell is still based on the indicated value for the band of the scheduling cell.

- In the LS to be sent together with updated RAN1 UE features list, ask RAN2 to update corresponding description for FG22-10 (pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16) in TS 38.306

[105-e-NR-UEFeature-MRDCCA-01] Email discussion/approval on UE features regarding MR-DC/CA enhancement, till 5/24 (Hiroki, NTT DOCOMO)

- For FG18-1/1a/1b. following proposals are discussed in 7.2.11 if some high-level consensus is achieved based on the discussion in AI 7.2.10.

  - Change FR1-FR2 differentiation from ‘No’ to ‘Yes’

  - Add a note ‘If UE indicates support for a frequency range, this capability is applicable for power sharing between MCG and SCG cells with UL in that frequency range’

  - Ask RAN2 to modify the descriptions in TS38.306 accordingly

### Others (2)

Including any essential TPs related to the endorsed Rel-16 NR TEIs, Rel-16 RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) (used to be under 5.1)

For this agenda, no more than 2 contribution per 1 company/organization/university

R1-2104858	Summary of Rel-16 uplink Tx switching	Moderator (China Telecom)

[105-e-NR-TEI16-01] Email discussion/approval on Rel-16 NR TEI related issues, till 5/24 – Hiroki (DoCoMo)

- Discuss following proposal and corresponding specification impact

  - The DL/UL collision handling should be supported by a UE capable of such handling for each band within a band combination where the UE supports inter-band simultaneous transmission and reception.

Deadline extended to 5/26

[105-e-NR-R16-TxSwitching-01] Email discussion/approval regarding issues #1 and #2 as in the summary, till 5/24 – Jianchi (China Telecom)

Update on 5/24:

- The proposed 38.214 TP is endorsed. To be part of 38.214 alignment CR.

Other proposals? 5/25

R1-2104325	Remaining Issues of Rel-16 UL Tx Switching	ZTE

R1-2104653	Remaining issues for 1Tx-2Tx switching	Qualcomm Incorporated

R1-2104730	Enhancement on uplink power control for M-TRP	OPPO

R1-2104731	Text Proposals for Tx Switching between Two Uplink Carriers	OPPO

R1-2105925	Discussion on the remaining problems of supporting Tx switching between two uplink carriers			Huawei, HiSilicon

# Release 17 (5+5+4+6+6+5+3+5+3+2+3+3+3+3+4+1=61)

Limit to maximum 1 contribution per company/organization/university for any agenda item, including “Others”.

## Further enhancements on MIMO for NR (5)

Please refer to RP-202024 for detailed scope of the WI

### Enhancements on Multi-beam Operation

Mainly targeting FR2 while also applicable to FR1

R1-2105290	Moderator summary for multi-beam enhancement	Moderator (Samsung)

R1-2105296	Moderator summary for offline discussion on multi-beam enhancement: CA QCL and unified TCI for 'other signals/channels'	Moderator (Samsung)

R1-2104205	Enhancement on multi-beam operation	FUTUREWEI

R1-2104266	Enhancements on multi-beam operation	Huawei, HiSilicon

R1-2104292	Remaining Issues on Rel-17 Multi-beam Operation	InterDigital, Inc.

R1-2104343	Further discussion on multi beam enhancement	vivo

R1-2104404	Enhancements on Multi-beam Operation	Lenovo, Motorola Mobility

R1-2104411	Enhancements on Multi-beam Operation	Spreadtrum Communications

R1-2104484	Enhancements on multi-beam operation	CATT

R1-2104585	Enhancements on Multi-beam Operation	ZTE

R1-2104599	Enhancements on multi-beam operation	CMCC

R1-2104654	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2104732	Enhancements on Multi-beam Operation	OPPO

R1-2104888	Enhancements to Multi-Beam Operations	Intel Corporation

R1-2104944	Enhancements to Multi-Beam Operations	Intel Corporation

Withdrawn

R1-2105001	Enhancements to Multi-Beam Operations	Intel Corporation

Withdrawn

R1-2105058	Enhancements on Multi-beam Operation	Fujitsu

R1-2105087	Views on Rel-17 Beam Management enhancement	Apple

R1-2105151	Further enhancement on multi-beam operation	Sony

R1-2105231	Enhancements on multi-beam operation	Fraunhofer IIS, Fraunhofer HHI

R1-2105246	Discussion on multi-beam operation	NEC

R1-2105273	Enhancements on Multi-beam Operation	Nokia, Nokia Shanghai Bell

R1-2105291	Multi-Beam Enhancements	Samsung

R1-2105353	Enhancement on multi-beam operation	MediaTek Inc.

R1-2105540	Enhancements on multi-beam operation	Xiaomi

R1-2105588	Enhancements on Multi-beam Operation	Convida Wireless

R1-2105665	Enhancements on Multi-Beam Operations	AT&T

R1-2105683	Discussion on multi-beam operation	NTT DOCOMO, INC.

R1-2105779	Enhancements on Multi-beam Operation	LG Electronics

R1-2105816	Discussion on enhancements for Multi-beam Operation	Asia Pacific Telecom, FGI

R1-2105828	Enhancements on Multi-beam Operation	Ericsson

[105-e-NR-feMIMO-01] Email discussion on enhancements on multi-beam operation – Eko (Samsung)

- 1st check point: May 24

- 2nd check point: May 27

### Enhancements for Multi-TRP Deployment

#### Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

Only to handle UL part

- NO contributions to DL, please

R1-2104201	Multi-TRP/panel for non-PDSCH	FUTUREWEI

R1-2104267	Enhancements on multi-TRP for reliability and robustness in Rel-17	Huawei, HiSilicon

R1-2104293	Multi-TRP Enhancements for PUCCH and PUSCH	InterDigital, Inc.

R1-2104344	Further discussion on Multi-TRP for PUCCH and PUSCH enhancements	vivo

R1-2104405	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Lenovo, Motorola Mobility

R1-2104412	Discussion on enhancements on Multi-TRP for PUCCH and PUSCH	Spreadtrum Communications

R1-2104485	Enhancements on PUCCH and PUSCH	CATT

R1-2104586	Multi-TRP enhancements for PUCCH and PUSCH	ZTE

R1-2104600	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	CMCC

R1-2104655	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Qualcomm Incorporated

R1-2104733	Enhancements on Multi-TRP based enhancement for PUCCH and PUSCH	OPPO

R1-2104841	Enhancements on Multi-TRP for uplink channels	CAICT

R1-2104889	Multi-TRP enhancements for PUCCH and PUSCH	Intel Corporation

R1-2104945	Multi-TRP enhancements for PUCCH and PUSCH	Intel Corporation

Withdrawn

R1-2104946	Multi-TRP enhancements for PUCCH and PUSCH	Intel Corporation

Withdrawn

R1-2105002	Multi-TRP enhancements for PUCCH and PUSCH	Intel Corporation

Withdrawn

R1-2105003	Multi-TRP enhancements for PUCCH and PUSCH	Intel Corporation

Withdrawn

R1-2105059	Enhancements on Multi-TRP for PDCCH PUCCH and PUSCH	Fujitsu

R1-2105088	Views on Rel-17 multi-TRP reliability enhancement	Apple

R1-2105152	Considerations on Multi-TRP for PDCCH, PUCCH, PUSCH	Sony

R1-2105247	Discussion on multi-TRP for PUCCH and PUSCH	NEC

R1-2105274	Enhancements for Multi-TRP URLLC schemes	Nokia, Nokia Shanghai Bell

R1-2105292	Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH	Samsung

R1-2105350	On multi-TRP enhancements for PUSCH	Fraunhofer IIS, Fraunhofer HHI

R1-2105354	Enhancements on Multi-TRP for PUCCH and PUSCH	MediaTek Inc.

R1-2105541	Enhancements on Multi-TRP for PUSCH and PUCCH	Xiaomi

R1-2105589	Enhancements on Multi-TRP for PUCCH and PUSCH	Convida Wireless

R1-2105629	Enhancements on Multi-TRP for PUSCH	Sharp

R1-2105684	Discussion on MTRP for reliability	NTT DOCOMO, INC.

R1-2105731	Discussion on mTRP PUSCH	ASUSTeK

R1-2105780	Enhancements on Multi-TRP for PUCCH and PUSCH	LG Electronics

R1-2105808	On PUCCH and PUSCH enhancements for multi-TRP	Ericsson

R1-2105817	Discussion on enhancements for Multi-TRP for uplink channels	Asia Pacific Telecom, FGI

R1-2105837	Enhancements on Multi-TRP for PUCCH and PUSCH	TCL Communication Ltd.

[105-e-NR-feMIMO-02] Email discussion on enhancements on multi-TRP for PUSCH, PUCCH – Keeth (Nokia)

- 1st check point: May 24

- 2nd check point: May 27

#### Enhancements on Multi-TRP inter-cell operation

Void (not be handled during this e-meeting). No contributions please.

#### Enhancements on beam management for multi-TRP

R1-2104202	Beam management for simultaneous multi-TRP transmission with multi-panel reception				FUTUREWEI

R1-2104268	Discussion on Multi-panel reception for multi-TRP in Rel-17	Huawei, HiSilicon

R1-2104294	Further Discussion on M-TRP Beam Management	InterDigital, Inc.

R1-2104345	Further discussion on MTRP multibeam enhancement	vivo

R1-2104406	Enhancements on beam management for multi-TRP	Lenovo, Motorola Mobility

R1-2104413	Discussion on enhancements on beam management for multi-TRP	Spreadtrum Communications

R1-2104486	Beam management enhancements for multi-TRP	CATT

R1-2104587	Enhancements on beam management for Multi-TRP	ZTE

R1-2104601	Enhancements on beam management for multi-TRP	CMCC

R1-2104656	Enhancements on beam management for multi-TRP	Qualcomm Incorporated

R1-2104734	Enhancements on beam management for multi-TRP	OPPO

R1-2104891	Multi-TRP enhancements for beam management	Intel Corporation

R1-2104947	Multi-TRP enhancements for beam management	Intel Corporation

Withdrawn

R1-2105004	Multi-TRP enhancements for beam management	Intel Corporation

Withdrawn

R1-2105060	Enhancements on beam management for multi-TRP	Fujitsu

R1-2105089	Views on Rel-17 multi-TRP BM enhancement	Apple

R1-2105153	Enhancements on beam management for multi-TRP	Sony

R1-2105211	Enhancements on beam management for multi-TRP	ETRI

R1-2105233	Enhancements on beam management for multi-TRP	ETRI

Withdrawn

R1-2105248	Discussion on beam management for multi-TRP	NEC

R1-2105275	Enhancements on Beam Management for Multi-TRP/Panel Transmission	Nokia, Nokia Shanghai Bell

R1-2105293	Enhancements on beam management for multi-TRP	Samsung

R1-2105367	Enhancement on beam management for MTRP	MediaTek Inc.

R1-2105542	Enhancement on beam management for Multi-TRP	Xiaomi

R1-2105590	On Multi-TRP BFR	Convida Wireless

R1-2105666	Beam Management Enhancements for Multi-TRP	AT&T

R1-2105672	Discussion on beam management for multi-TRP	ASUSTEK COMPUTER (SHANGHAI)

R1-2105685	Discussion on beam management for MTRP	NTT DOCOMO, INC.

R1-2105754	Discussion on beam management for multi-TRP	ITRI

R1-2105781	Enhancements on beam management for multi-TRP	LG Electronics

R1-2105806	On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception	Ericsson

R1-2105818	Discussion of enhancements on beam management for Multi-TRP	Asia Pacific Telecom, FGI

R1-2105836	Enhancements on beam management for multi-TRP	TCL Communication Ltd.

[105-e-NR-feMIMO-03] Email discussion on enhancements on beam management for multi-TRP – Runhua (CATT)

- 1st check point: May 24

- 2nd check point: May 27

#### Enhancements on HST-SFN deployment

R1-2104203	Enhancement to support HST-SFN deployment scenario	FUTUREWEI

R1-2104269	Enhancements on high speed train for multi-TRP in Rel-17	Huawei, HiSilicon

R1-2104295	Further Discussion on HST-SFN	InterDigital, Inc.

R1-2104346	Further discussion and evaluation on HST-SFN schemes	vivo

R1-2104414	Discussion on enhancements on HST-SFN deployment	Spreadtrum Communications

R1-2104487	Discussion on HST-SFN transmission schemes	CATT

R1-2104588	Discussion on Multi-TRP HST enhancements	ZTE

R1-2104602	Enhancements on HST-SFN deployment	CMCC

R1-2104657	Enhancements on HST-SFN deployment	Qualcomm Incorporated

R1-2104735	Enhancements on HST-SFN deployment	OPPO

R1-2104892	Enhancements to HST-SFN deployments	Intel Corporation

R1-2104948	Enhancements to HST-SFN deployments	Intel Corporation

Withdrawn

R1-2105005	Enhancements to HST-SFN deployments	Intel Corporation

Withdrawn

R1-2105090	Views on Rel-17 HST enhancement	Apple

R1-2105154	Enhancement on HST-SFN deployment	Sony

R1-2105249	Discussion on HST-SFN deployment	NEC

R1-2105276	Enhancements for HST-SFN deployment	Nokia, Nokia Shanghai Bell

R1-2105294	Enhancements on HST-SFN	Samsung

R1-2105543	Enhancements on HST-SFN operation for multi-TRP PDCCH transmission	Xiaomi

R1-2105586	Enhancement on HST-SFN deployment	Ericsson

R1-2105591	On Enhancements for HST-SFN deployment	Convida Wireless

R1-2105686	Discussion on HST-SFN deployment	NTT DOCOMO, INC.

R1-2105761	Enhancements for HST-SFN deployment	Lenovo, Motorola Mobility

R1-2105782	Enhancements on HST-SFN deployment	LG Electronics

[105-e-NR-feMIMO-04] Email discussion on enhancements on HST-SFN deployment – Alexei (Intel)

- 1st check point: May 24

- 2nd check point: May 27

### Enhancements on SRS flexibility, coverage and capacity

Void (not be handled during this e-meeting). No contributions please.

### CSI enhancements: MTRP and FR1 FDD reciprocity

R1-2104204	CSI enhancement for multi-TRP and FDD	FUTUREWEI

R1-2104270	Discussion on CSI Enhancements for Rel-17	Huawei, HiSilicon

R1-2104296	Views on CSI Enhancements for NCJT MTRP	InterDigital, Inc.

R1-2104347	Further discussion and evaluation on MTRP CSI and Partial reciprocity	vivo

R1-2104415	Discussion on CSI enhancement for multi-TRP and FR1 FDD reciprocity	Spreadtrum Communications

R1-2104488	CSI enhancements for Rel-17	CATT

R1-2104589	CSI enhancements for Multi-TRP and FR1 FDD reciprocity	ZTE

R1-2104603	Enhancements on CSI reporting for Multi-TRP	CMCC

R1-2104658	CSI enhancements: MTRP and FR1 FDD reciprocity	Qualcomm Incorporated

R1-2104736	CSI enhancement for M-TRP and FDD reciprocity	OPPO

R1-2104893	On CSI enhancements for MTRP and FDD	Intel Corporation

R1-2104949	On CSI enhancements for MTRP and FDD	Intel Corporation

Withdrawn

R1-2105006	On CSI enhancements for MTRP and FDD	Intel Corporation

Withdrawn

R1-2105091	Views on Rel-17 CSI enhancement	Apple

R1-2105155	More considerations on CSI enhancements	Sony

R1-2105250	Discussion on CSI enhancement for multi-TRP	NEC

R1-2105264	CSI enhancements on Type II PS codebook and multi-TRP	Fraunhofer IIS, Fraunhofer HHI

R1-2105277	Enhancement on CSI measurement and reporting	Nokia, Nokia Shanghai Bell

R1-2105295	Views on Rel. 17 CSI enhancements	Samsung

R1-2105368	CSI enhancement for NCJT and FR1 FDD reciprocity	MediaTek Inc.

R1-2105687	Discussion on CSI enhancements	NTT DOCOMO, INC.

R1-2105762	CSI enhancements for multi-TRP and FDD reciprocity	Lenovo, Motorola Mobility

R1-2105783	CSI enhancements for Rel-17	LG Electronics

R1-2105807	CSI enhancements for MTRP and FR1 FDD reciprocity	Ericsson

//This one is to use NWM – please use RAN1-105-e-NWM-NR-feMIMO-05 as the document name

[105-e-NR-feMIMO-05] Email discussion on CSI enhancements: MTRP and FR1 FDD reciprocity – Min (Huawei)

- 1st check point: May 24

- 2nd check point: May 27

### Others

Void (not be handled during this e-meeting). No contributions please.

## Supporting NR from 52.6GHz to 71 GHz (5)

Please refer to RP-210862  for detailed scope of the WI

### Initial access aspects

Including SSB, initial BWP, PRACH, etc.

R1-2104210	Initial access for Beyond 52.6GHz	FUTUREWEI

R1-2104273	Initial access signals and channels for 52-71GHz spectrum	Huawei, HiSilicon

R1-2104348	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz	vivo

R1-2104416	Discussion on initial access aspects for NR for 60GHz	Spreadtrum Communications

R1-2104452	Initial access aspects	Nokia, Nokia Shanghai Bell

R1-2104460	Initial Access Aspects	Ericsson

R1-2104507	Initial access aspects for up to 71GHz operation	CATT

R1-2104659	Initial access aspects for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104765	Discusson on initial access aspects	OPPO

R1-2104833	Discussion on the initial access aspects for 52.6 to 71GHz	ZTE, Sanechips

R1-2104894	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

R1-2104950	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105007	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105061	Considerations on initial access for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2105092	Discussion on Initial access signals and channels	Apple

R1-2105156	Considerations on initial access aspects for NR from 52.6 GHz to 71 GHz	Sony

R1-2105260	Discussion on initial access aspects supporting NR from 52.6 to 71 GHz	NEC

R1-2105297	Initial access aspects for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105370	Discussion on initial access of 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105419	Initial access aspects to support NR above 52.6 GHz	LG Electronics

R1-2105495	Initial access aspects for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105555	On initial access aspects for NR from 52.6GHz to 71 GHz	Xiaomi

R1-2105581	Discussions on initial access aspects	InterDigital, Inc.

R1-2105592	NR Initial Access from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105630	Initial access aspects	Sharp

R1-2105660	On the importance of inter-operator PCI confusion resolution and ANR support in 52.6 GHz and beyond	AT&T

R1-2105688	Initial access aspects for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105786	Further details of initial access for NR above 52.6 GHz	Charter Communications

R1-2105868	Discussion on initial access aspects for NR beyond 52.6GHz	WILUS Inc.

[105-e-NR-52-71GHz-01] Email discussion/approval on initial access aspects with checkpoints for agreements on May-24, May-27 – Daewon (Intel)

### PDCCH monitoring enhancements

Void (not be handled during this e-meeting). No contributions please.

### Enhancements for PUCCH formats 0/1/4

R1-2104211	Enhancements of PUCCH formats for Beyond 52.6GHz	FUTUREWEI

R1-2104349	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

R1-2104417	Discussion on enhancements for PUCCH format 0/1/4 for above 52.6GHz	Spreadtrum Communications

R1-2104453	Enhanced PUCCH formats 0/1/4	Nokia, Nokia Shanghai Bell

R1-2104461	PUCCH enhancements	Ericsson

R1-2104508	Enhancements for PUCCH formats for up to 71GHz operation	CATT

R1-2104660	Enhancements for PUCCH for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104766	Discussion on enhancements for PUCCH format 0/1/4	OPPO

R1-2104834	Discussion on the PUCCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2104895	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2104951	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105008	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105093	Discussion on Enhancements for PUCCH formats 0/1/4	Apple

R1-2105157	Further considerations on enhancements for PUCCH formats 0/1/4	Sony

R1-2105298	Enhancements for PUCCH format 0/1/4 for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105369	On Enhancements for PUCCH formats 0/1	MediaTek Inc.

R1-2105420	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz	LG Electronics

R1-2105496	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105582	Discussions on enhancements for PUCCH formats 0/1/4	InterDigital, Inc.

R1-2105689	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105869	Discussion on PUCCH enhancement for PUCCH format 0/1/4	WILUS Inc.

R1-2105929	Enhancement on PUCCH formats	Huawei, HiSilicon

[105-e-NR-52-71GHz-02] Email discussion/approval on PUCCH format 0/1/4 enhancements with checkpoints for agreements on May 25, May 27 – Steve (Ericsson)

### Beam management for new SCSs

Including timing associated with beam-based operation

Void (not be handled during this e-meeting). No contributions please.

### PDSCH/PUSCH enhancements

Including defining maximum bandwidth for new SCSs, time line related aspects adapted to each of the new numerologies 480kHz and 960kHz, reference signals, scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, etc.

Only to handle multi-PDSCH/PUSCH with a single DCI and HARQ

- NO contributions please to other topics such as defining maximum bandwidth for new SCSs, timeline related aspects adapted to each of the new numerologies 480 kHz and 960 kHz and reference signals, etc.

R1-2105422	Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

R1-2104212	Enhancements to support PDSCH/PUSCH for Beyond 52.6GHz	FUTUREWEI

R1-2104274	PDSCH/PUSCH enhancements for 52-71GHz spectrum	Huawei, HiSilicon

R1-2104350	Discussions on multi-PDSCH/PUSCH scheduling for NR operation from 52.6GHz to 71GHz			vivo

R1-2104418	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz	Spreadtrum Communications

R1-2104454	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

R1-2104462	PDSCH-PUSCH Enhancements	Ericsson

R1-2104509	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

R1-2104661	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104767	Discussion on PDSCH/PUSCH enhancements	OPPO

R1-2104835	Discussion on the PDSCH/PUSCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

R1-2104896	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

R1-2104952	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105009	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105062	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

R1-2105094	Discussion on multi-PxSCH and HARQ Codebook Enhancements	Apple

R1-2105158	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Sony

R1-2105259	Discussion on PDSCH enhancements supporting NR from 52.6GHz to 71 GHz	NEC

R1-2105299	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105372	HARQ codebook design for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105396	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic Corporation

R1-2105421	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

R1-2105497	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105556	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

R1-2105583	Enhancing PDSCH/PUSCH Scheduling for 52.6 GHz to 71 GHz Band	InterDigital, Inc.

R1-2105596	PDSCH Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105690	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105784	PDSCH-PUSCH Enhancement for NR beyond 52.6 GHz	Charter Communications

R1-2105870	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz	WILUS Inc.

[105-e-NR-52-71GHz-03] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, with checkpoints for agreements on May 24, May 27 – Seonwook (LGE)

### Channel access mechanism

R1-2104213	Channel access for shared spectrum Beyond 52.6 GHz	FUTUREWEI

R1-2104275	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

R1-2104351	Discussions on channel access mechanism for NR operation from 52.6GHz to 71 GHz	vivo

R1-2104419	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

R1-2104455	Channel access mechanism	Nokia, Nokia Shanghai Bell

R1-2104463	Channel Access Mechanisms	Ericsson

R1-2104510	Channel access mechanism for up to 71GHz operation	CATT

R1-2104662	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

R1-2104720	Discussions on channel access mechanism enhancements for 52.6G-71 GHz	CAICT

R1-2104768	Discussion on channel access mechanism	OPPO

R1-2104836	Discussion on the channel access for 52.6 to 71GHz	ZTE, Sanechips

R1-2104897	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

R1-2104953	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105010	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

Withdrawn

R1-2105063	Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz	Fujitsu

R1-2105095	Channel access mechanism	Apple

R1-2105145	Channel access for multi-beam operation	Panasonic

R1-2105159	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

R1-2105261	Discussion on channel access mechanism supporting NR from 52.6 to 71GHz	NEC

R1-2105300	Channel access mechanism for NR from 52.6 GHz to 71 GHz	Samsung

R1-2105371	On the channel access mechanisms for 52.6-71 GHz NR operation	MediaTek Inc.

R1-2105423	Channel access mechanism to support NR above 52.6 GHz	LG Electronics

R1-2105498	Channel access mechanisms for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

R1-2105557	Discussion on channel access mechanism for NR on 52.6-71 GHz	Xiaomi

R1-2105584	Discussion on channel access mechanisms	InterDigital, Inc.

R1-2105597	On Channel Access Mechanism for NR from 52.6 GHz to 71 GHz	Convida Wireless

R1-2105661	On receiver assisted channel access and directional LBT	AT&T

R1-2105691	Channel access mechanism for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

R1-2105755	Discussion on multi-beam operation	ITRI

R1-2105785	Channel access mechanisms for above 52.6 GHz	Charter Communications

R1-2105871	Discussion on channel access mechanism for NR from 52.6GHz to 71GHz	WILUS Inc.

[105-e-NR-52-71GHz-04] Email discussion/approval on channel access mechanism with checkpoints for agreements on May 25, May 27 – Jing (Qualcomm)

### Others

Including analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range

[105-e-NR-52-71GHz-05] Email discussion/approval focusing on analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range with checkpoints for agreements on May 25, May 27 – Alex (Lenovo)

R1-2104352	Discussions on frequency range definition and sync raster for 52.6-71GHz	vivo

R1-2104456	Extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

R1-2104464	Introduction of the 52.6 to 71 GHz frequency range	Ericsson

R1-2104511	Views on Frequency Range for 52.6  to 71 GHz	CATT

R1-2104769	Discussion on other aspects	OPPO

R1-2104837	Discussion on the frequency range definition for above 52.6GHz	ZTE, Sanechips

R1-2104898	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

R1-2104954	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

Withdrawn

R1-2105011	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

Withdrawn

R1-2105096	On How to Introduce the 52.6-71GHz Frequency Range	Apple

R1-2105301	Discussion on defining frequency range for NR from 52.6 to 71 GHz	Samsung

R1-2105373	Frequency range naming discussion for 52.6-71 GHz	MediaTek Inc.

R1-2105424	Discussion on frequency range definition to support NR above 52.6 GHz	LG Electronics

R1-2105533	Discussion on the frequency range for bands in 52.6-71 GHz	Huawei, HiSilicon

R1-2105585	Efficient beam management for NR in 52.6 – 71 GHz	InterDigital, Inc.

R1-2105692	Views on how to introduce 52.6 - 71 GHz frequency range in NR	NTT DOCOMO, INC.

R1-2105787	Discussion on how to introduce the 52.6-71GHz frequency range	Charter Communications

## Enhanced Industrial Internet of Things (IoT) and URLLC (4)

Please refer to RP-210854 for detailed scope of the WI

Please also refer to RP-202872 for additional guidance for this e-meeting

### Necessity and Support of Physical Layer feedback enhancements

#### UE feedback enhancements for HARQ-ACK

Only to handle topics of PUCCH carrier switching and retransmission of cancelled HARQ

- No contributions to other topics please

[105-e-NR-R17-IIoT-URLLC-01] Email discussion on UE feedback enhancements for HARQ-ACK – Klaus (Nokia)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2104314	Moderator summary #1 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT			Moderator (Nokia)

R1-2106097

R1-2104217	HARQ-ACK Enhancements for IIoT/URLLC	Ericsson

R1-2104262	UE feedback enhancements for HARQ-ACK	Huawei, HiSilicon

R1-2104309	HARQ-ACK Feedback Enhancements for URLLC/IIoT	Nokia, Nokia Shanghai Bell

R1-2104326	Discussion on HARQ-ACK enhancements for eURLLC	ZTE

R1-2104353	HARQ-ACK enahncements for Rel-17 URLLC	vivo

R1-2104420	Discussion on HARQ-ACK feedback enhancements for Rel-17 URLLC	Spreadtrum Communications

R1-2104512	UE feedback enhancements for HARQ-ACK	CATT

R1-2104604	Discussion on UE feeback enhancements for HARQ-ACK	CMCC

R1-2104663	HARQ-ACK enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2104802	HARQ-ACK enhancements for Rel-17 URLLC/IIoT	OPPO

R1-2104854	Discussion on two aspects of UE HARQ-ACK feedback enhancements	China Telecom

R1-2104899	On dynamic carrier switching and dropped HARQ feedback retransmission	Intel Corporation

R1-2104955	On dynamic carrier switching and dropped HARQ feedback retransmission	Intel Corporation

Withdrawn

R1-2105012	On dynamic carrier switching and dropped HARQ feedback retransmission	Intel Corporation

Withdrawn

R1-2105097	Views on eIIoT/URLLC HARQ feedback enhancements	Apple

R1-2105160	Retransmission of dropped HARQ-ACK for URLLC	Sony

R1-2105188	Discussion on UE feedback enhancements for HARQ-ACK	PANASONIC R&D Center Germany

R1-2105212	UE feedback enhancements for HARQ-ACK	ETRI

R1-2105234	UE feedback enhancements for HARQ-ACK	ETRI

Withdrawn

R1-2105258	UE feedback enhancements for HARQ-ACK	NEC

R1-2105302	HARQ-ACK Reporting Enhancements for URLLC	Samsung

R1-2105355	UE feedback enhancements for HARQ-ACK	ETRI

Withdrawn

R1-2105399	HARQ feedback enhancements for IIoT and URLLC	InterDigital, Inc.

R1-2105425	Discussion on UE feedback enhancement for HARQ-ACK	LG Electronics

R1-2105631	UE feedback enhancements for HARQ-ACK	Sharp

R1-2105693	Discussion on HARQ-ACK feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2105732	On UE feedback enhancements for HARQ-ACK	MediaTek Inc.

R1-2105750	UE feedback enhancements for HARQ-ACK	CAICT

R1-2105766	HARQ-ACK feedback enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2105819	Discussion on UE feedback enhancements for HARQ-ACK	Asia Pacific Telecom, FGI

R1-2105872	Discussion on HARQ-ACK enhancement for URLLC/IIoT	WILUS Inc.

#### CSI feedback enhancements

[105-e-NR-R17-IIoT-URLLC-02] Email discussion on CSI feedback enhancements – Paul (InterDigital)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2105975

R1-2104199	CSI feedback enhancements for URLLC	FUTUREWEI

R1-2104218	CSI Feedback Enhancements for IIoT/URLLC	Ericsson

R1-2104263	CSI feedback enhancements	Huawei, HiSilicon

R1-2104327	Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2104354	CSI feedback enhancements for Rel-17 URLLC	vivo

R1-2104421	Discussion on CSI feedback enhancements for Rel-17 URLLC	Spreadtrum Communications

R1-2104513	CSI feedback enhancements	CATT

R1-2104605	Discussion on CSI feeback enhancements for URLLC	CMCC

R1-2104664	CSI enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2104803	CSI feedback enhancements for URLLC	OPPO

R1-2104900	Selection of enhanced CSI feedback schemes	Intel Corporation

R1-2104956	Selection of enhanced CSI feedback schemes	Intel Corporation

Withdrawn

R1-2105013	Selection of enhanced CSI feedback schemes	Intel Corporation

Withdrawn

R1-2105098	Views on eIIoT/URLLC CSI feedback enhancements	Apple

R1-2105161	Considerations on CSI feedback enhancements	Sony

R1-2105186	Discussion on CSI Feedback Enhancements	Quectel, Langbo

R1-2105303	Improving MCS Selection for URLLC	Samsung

R1-2105426	Discussion on CSI feedback enhancements for URLLC	LG Electronics

R1-2105472	CSI feedback enhancements	InterDigital, Inc.

R1-2105580	CSI feedback enhancements for URLLC/IIoT use cases	Nokia, Nokia Shanghai Bell

R1-2105694	Discussion on CSI feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2105733	CSI feedback enhancements for URLLC	MediaTek Inc.

R1-2105767	CSI feedback enhancements for URLLC/IIoT	Lenovo, Motorola Mobility

### Enhancements for unlicensed band URLLC/IIoT

Only to handle topics of CG harmonization and decision between Alt-a/Alt-b for semi-static channel access mode when a UE can operate as UE-initiated COT for both CG and DG (see sections 2.3/2.4/2.5 of R1-2103960)

- No contributions to other topics please

//This one is to use NWM – please use RAN1-105-e-NWM-NR-R17-IIoT-URLLC-03 as the document name

[105-e-NR-R17-IIoT-URLLC-03] Email discussion on enhancements for unlicensed band URLLC/IIoT – Sorour (Ericsson)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106044

R1-2106045

R1-2106046

Agreement:

- Option 1 is taken in the following agreement:

Agreement:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

  - If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled by X.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT..

- Alt-a is taken in the following agreement:

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

R1-2104200	UE initiated COT for semi-static channel access	FUTUREWEI

R1-2104219	Enhancements for IIoT/URLLC on Unlicensed Band	Ericsson

R1-2104265	Uplink enhancements for URLLC in unlicensed controlled environments	Huawei, HiSilicon

R1-2104328	Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2104355	Enhancements for unlicensed band URLLC/IIoT	vivo

R1-2104422	Discussion on enhancements for unlicensed band URLLC/IIoT	Spreadtrum Communications

R1-2104514	Enhancements for unlicensed band URLLC/IIoT	CATT

R1-2104665	Uplink enhancements for URLLC in unlicensed controlled environments	Qualcomm Incorporated

R1-2104804	Enhancements for unlicensed band URLLC/IIoT	OPPO

R1-2104901	On the Harmonization of UL CG between NR-U and URLLC and COT-initiator Determination			Intel Corporation

R1-2104957	On the Harmonization of UL CG between NR-U and URLLC and COT-initiator Determination			Intel Corporation

Withdrawn

R1-2105014	On the Harmonization of UL CG between NR-U and URLLC and COT-initiator Determination			Intel Corporation

Withdrawn

R1-2105099	URLLC uplink enhancements for unlicensed spectrum	Apple

R1-2105142	Further UL enhancements for IIoT/URLLC in unlicensed controlled environment	Nokia, Nokia Shanghai Bell

R1-2105143	Enhancements for unlicensed band URLLC/IIoT	Panasonic Corporation

R1-2105162	Considerations on unlicensed URLLC	Sony

R1-2105213	Enhancements for unlicensed band URLLC/IIoT	ETRI

R1-2105235	Enhancements for unlicensed band URLLC/IIoT	ETRI

Withdrawn

R1-2105304	Enhancements for unlicensed band URLLC/IIoT	Samsung

R1-2105356	Enhancements for unlicensed band URLLC/IIoT	ETRI

Withdrawn

R1-2105400	Enhancements for unlicensed band URLLC/IIoT	InterDigital, Inc.

R1-2105409	Enhancements for unlicensed band URLLC/IIoT	NEC

R1-2105427	Discussion on unlicensed band URLLC/IIOT	LG Electronics

R1-2105632	Enhancements for unlicensed band URLLC/IIoT	Sharp

R1-2105695	Discussion on enhancements for unlicensed band URLLC	NTT DOCOMO, INC.

R1-2105734	On the enhancements for unlicensed band URLLC/IIoT	MediaTek Inc.

R1-2105768	Enhancements for unlicensed band URLLC/IIoT	Lenovo, Motorola Mobility

R1-2105873	Discussion on enhancement for unlicensed URLLC/IIoT	WILUS Inc.

### Intra-UE Multiplexing/Prioritization

[105-e-NR-R17-IIoT-URLLC-04] Email discussion on intra-UE multiplexing/prioritization – Jia (OPPO)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106051

R1-2104220	Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

R1-2104264	Intra-UE multiplexing enhancements	Huawei, HiSilicon

R1-2104310	On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

R1-2104329	Discussion on enhanced intra-UE multiplexing	ZTE

R1-2104356	Intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

R1-2104423	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

R1-2104515	Intra-UE multiplexing and prioritization	CATT

R1-2104606	Discussion on intra-UE multiplexing/prioritization	CMCC

R1-2104666	Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

R1-2104805	Enhancements on intra-UE multiplexing/prioritization	OPPO

R1-2104855	Discussion on intra-UE multiplexing and prioritization	China Telecom

R1-2104902	Details of intra-UE multiplexing and prioritization	Intel Corporation

R1-2104958	Details of intra-UE multiplexing and prioritization	Intel Corporation

Withdrawn

R1-2105015	Details of intra-UE multiplexing and prioritization	Intel Corporation

Withdrawn

R1-2105100	Design of Rel-17 intra-UE multiplexing/prioritization	Apple

R1-2105144	Discussion on Intra-UE multiplexing and prioritization of different priority	Panasonic Corporation

R1-2105163	Considerations on intra-UE UL multiplexing	Sony

R1-2105187	Discussion on Intra-UE Multiplexing/Prioritization	Quectel, Langbo

R1-2105206	Intra-UE Multiplexing and Prioritization	TCL Communication Ltd.

R1-2105221	Intra-UE Multiplexing/Prioritization	ETRI

R1-2105236	Intra-UE Multiplexing/Prioritization	ETRI

Withdrawn

R1-2105262	Discussion on Intra-UE prioritization and multiplexing	NEC

R1-2105305	Uplink intra-UE multiplexing and prioritization	Samsung

R1-2105357	Intra-UE Multiplexing/Prioritization	ETRI

Withdrawn

R1-2105428	Discussion on Intra-UE multiplexing/prioritization	LG Electronics

R1-2105473	Intra-UE multiplexing and prioritization	InterDigital, Inc.

R1-2105558	Intra-UE multiplexing prioritization for URLLC IIoT	Xiaomi

R1-2105633	Intra-UE UCI multiplexing with different priorities and channel prioritization	Sharp

R1-2105696	Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2105735	Methods for intra-UE multiplexing and prioritization	MediaTek Inc.

R1-2105756	Discussion on intra-UE multiplexing	ITRI

R1-2105769	Intra-UE multiplexing enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2105874	Discussion on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

### Others

Including any RAN1 involvement in propagation delay compensation enhancements (including mobility issues, if any).

Void (not be handled during this e-meeting). No contributions please.

## Solutions for NR to support non-terrestrial networks (NTN) (6)

Please refer to RP-210908 for detailed scope of the WI

[105-e-NR-NTN-LS-01]  A reply LS for R1-2104155 is necessary – email discussion/approval till 5/24 (Xingqin, Ericsson).

### Timing relationship enhancements

R1-2104255	Discussion on timing relationship enhancements for NTN	Huawei, HiSilicon

R1-2104306	Discussion on timing relationship enhancement in NTN	THALES

Withdrawn

R1-2104424	Consideration on timing relationship enhancements for NTN	Spreadtrum Communications

R1-2104516	Timing relationship enhancement for NTN	CATT

R1-2104564	Timing relationship enhancements for NR-NTN	MediaTek Inc.

R1-2104607	Discussion on timing relationship enhancements for NTN	CMCC

R1-2104667	Enhancements on Timing Relationship for NTN	Qualcomm Incorporated

R1-2104708	Timing relationship enhancements for NTN	Zhejiang Lab

R1-2104721	Timing relationship enhancements to support NTN	CAICT

R1-2104727	On timing relationship enhancements for NTN	Ericsson

R1-2104770	Discusson on timing relationship enhancement	OPPO

R1-2104827	Further discussion of time relation aspects for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2104857	Timing Relationship Enhancements in NR-NTN	China Telecom

R1-2104903	On timing relationship enhancements for NTN	Intel Corporation

R1-2104959	On timing relationship enhancements for NTN	Intel Corporation

Withdrawn

R1-2105016	On timing relationship enhancements for NTN	Intel Corporation

Withdrawn

R1-2105101	Timing Relationship Enhancements for NR NTN	Apple

R1-2105164	Calculation and application of timing relationship offsets	Sony

R1-2105189	Discussion on timing relationship for NR-NTN	ZTE

R1-2105207	Discussion on Timing Relationship Enhancements for NTN	Fraunhofer IIS, Fraunhofer HHI

R1-2105208	Timing relationship enhancements for NTN	ITL

R1-2105306	Timing relationship enhancements for NTN	Samsung

R1-2105410	Discussion on timing relationship enhancements for NTN	NEC

R1-2105477	Discussions on timing relationship enhancements in NTN	LG Electronics

R1-2105559	Discussion on the timing relationship enhancement for NTN	Xiaomi

R1-2105619	Timing relationship for NTN	Panasonic Corporation

R1-2105623	Discussion on NTN timing relationship	Lenovo, Motorola Mobility

R1-2105667	Timing relationship enhancement for NTN	InterDigital, Inc.

R1-2105697	Discussion on timing relationship enhancements for NTN	NTT DOCOMO, INC.

R1-2105820	Timing relationship enhancements in NTN	Asia Pacific Telecom, FGI

[105-e-NR-NTN-01] Email discussion/approval on timing relationship enhancements with checkpoints for agreements on May 24, May 27 – Xingqin (Ericsson)

### Enhancements on UL time and frequency synchronization

R1-2104302	Feature lead summary #1 on enhancements on UL timing and frequency synchronization for NR NTN		THALES

R1-2104303	Feature lead summary #2 on enhancements on UL timing and frequency synchronization for NR NTN		THALES

R1-2104304	Feature lead summary #3 on enhancements on UL timing and frequency synchronization for NR NTN		THALES

R1-2104305	Feature lead summary #4 on enhancements on UL timing and frequency synchronization for NR NTN		THALES

R1-2104256	Discussion on UL time and frequency synchronization enhancement for NTN	Huawei, HiSilicon

R1-2104301	Considerations on UL timing and frequency synchronization in NTN	THALES

R1-2104315	Enhancements on UL time and frequency synchronization	PANASONIC R&D Center Germany

R1-2104425	Consideration on enhancements on UL time and frequency synchronization for NTN	Spreadtrum Communications

R1-2104517	UL time and frequency synchronization for NTN	CATT

R1-2105952	Enhancements on UL Time and Frequency Synchronisation for NR-NTN	MediaTek Inc.

Revision of R1-2104565

R1-2104608	Enhancements on UL time and frequency synchronization for NTN	CMCC

R1-2104668	UL time and frequency synchronization for NTN	Qualcomm Incorporated

R1-2104722	Considerations on Enhancements on UL Time Synchronization in NTN	CAICT

R1-2104771	Discussion on UL time and frequency synchronization	OPPO

R1-2104811	On UL time and frequency synchronization enhancements for NTN	Ericsson

R1-2104828	Discussion on time and frequency synchronization for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2104904	On UL synchronization for NTN	Intel Corporation

R1-2104960	On UL synchronization for NTN	Intel Corporation

Withdrawn

R1-2105017	On UL synchronization for NTN	Intel Corporation

Withdrawn

R1-2105102	Uplink Time and Frequency Synchronization for NR NT	Apple

R1-2105165	Considerations on UL time synchronisation	Sony

R1-2105190	Discussion on UL synchronization for NR-NTN	ZTE

R1-2105214	Discussion on NTN uplink time synchronization	Lenovo, Motorola Mobility

R1-2105272	Discussion on UL Time Synchronization for NTN	Fraunhofer IIS, Fraunhofer HHI

R1-2105307	Enhancements on UL time and frequency synchronization for NTN	Samsung

R1-2105478	Discussions on UL time and frequency synchronization enhancements in NTN	LG Electronics

R1-2105560	Discussion on UL time and frequency synchronization for NTN	Xiaomi

R1-2105634	Enhancements on frequency synchronization for NTN	Sharp

R1-2105668	UL time/frequency synchronization for NTN	InterDigital, Inc.

R1-2105698	Discussion on UL time and frequency synchronization enhancements for NTN	NTT DOCOMO, INC.

R1-2105812	On satellite pass predictions under NTN discontinuous coverage	Sateliot, Gatehouse, ESA, Kepler

R1-2105821	UL time and frequency synchronization in NTN	Asia Pacific Telecom, FGI

R1-2105947	UL time synchronization acquisition for NTN	Mitsubishi Electric RCE

[105-e-NR-NTN-02] Email discussion/approval on UL time and frequency synchronization with checkpoints for agreements on May 25, May 27 – Mohamed (Thales)

[105-e-NR-NTN-05] Email discussion/approval for R1-2104230 from May 21 to May 26 - Hao (OPPO)

### Enhancements on HARQ

R1-2104257	Discussion on HARQ enhancement for NTN	Huawei, HiSilicon

R1-2104357	Discussion on HARQ enhancements for NR-NTN	vivo

R1-2104402	Discussion on HARQ for NTN	Magister Solutions Ltd

R1-2104426	Consideration on enhancements on HARQ for NTN	Spreadtrum Communications

R1-2104518	HARQ operation enhancement for NTN	CATT

R1-2104566	Enhancements on HARQ for NR NTN	MediaTek Inc.

R1-2104609	Enhancements on HARQ for NTN	CMCC

R1-2104669	Enhancements on HARQ for NTN	Qualcomm Incorporated

R1-2104723	Enhancements on HARQ to support NTN	CAICT

R1-2104772	Discussion on HARQ enhancements	OPPO

R1-2104812	On HARQ enhancements for NTN	Ericsson

R1-2104829	Further discussion related to HARQ operation for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2105054	HARQ enhancements for NTN networks	CEWiT

R1-2105103	On HARQ Enhancements for NR NTN	Apple

R1-2105166	Enhancement on HARQ for NTN	Sony

R1-2105191	Discussion on HARQ for NR-NTN	ZTE

R1-2105215	Enhancements on HARQ for NTN	Lenovo, Motorola Mobility

R1-2105222	Discussion on HARQ Enhancements for NTN	ETRI

R1-2105237	Discussion on HARQ Enhancements for NTN	ETRI

Withdrawn

R1-2105308	Enhancements on HARQ for NTN	Samsung

R1-2105358	Discussion on HARQ Enhancements for NTN	ETRI

Withdrawn

R1-2105411	Discussion on HARQ enhancements for NR NTN	NEC

R1-2105479	Discussions on HARQ enhancements in NTN	LG Electronics

R1-2105561	Discussion on the HARQ enhancement for NTN	Xiaomi

R1-2105620	HARQ enhancement for NTN 	Panasonic Corporation

R1-2105669	HARQ enhancement for NTN	InterDigital, Inc.

R1-2105757	Enhancements on HARQ for NTN	ITRI

R1-2105822	Enhancements on HARQ in NTN	Asia Pacific Telecom, FGI

[105-e-NR-NTN-03] Email discussion/approval on enhancements on HARQ with checkpoints for agreements on May 24, May 27 – Nan (ZTE)

### Others

Including the list of topics to specify if beneficial and needed

R1-2104776	FL Summary of 8.4.4 Other Aspects of NR-NTN	OPPO

R1-2104307	Beam management and BWP operation in NTN	THALES

R1-2104358	Discussion on other aspects for NR-NTN	vivo

R1-2104427	Consideration on beam management and other aspects for NTN	Spreadtrum Communications

R1-2104519	Beam management and other aspects for NTN	CATT

R1-2104610	Other Aspects for NTN	CMCC

R1-2104670	BWP operation and other issues for NTN	Qualcomm Incorporated

R1-2104773	Discusson on beam management	OPPO

R1-2104813	On other enhancements for NTN	Ericsson

R1-2104830	Discussion of other aspects for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2105104	Discussions on Other Aspects of NR NTN	Apple

R1-2105148	Beam management and polarization signaling for NTN	Panasonic

R1-2105167	Discussion on beam management and polarization for NTN	Sony

R1-2105192	Discussion on additional enhancement for NR-NTN	ZTE

R1-2105209	Discussion on Beam Management for NTN	Fraunhofer IIS, Fraunhofer HHI

R1-2105216	Discussion on other aspects for NTN	Lenovo, Motorola Mobility

R1-2105223	Discussion on beam management for NTN	ETRI

R1-2105238	Discussion on beam management for NTN	ETRI

Withdrawn

R1-2105309	Remaining issues for NTN Samsung	Samsung

R1-2105359	Discussion on beam management for NTN	ETRI

Withdrawn

R1-2105480	Discussions on other aspects of NTN	LG Electronics

R1-2105529	Discussion on other design aspects for NTN	Huawei, HiSilicon

R1-2105562	Discussion on other design aspects for NTN	Xiaomi

R1-2105671	On beam management for NTN	InterDigital, Inc.

[105-e-NR-NTN-04] Email discussion/approval on enhancements on other topics with checkpoints for agreements on May 25, May 27 – Hao (Oppo)

## NR Positioning Enhancements (6)

Please refer to RP-210903 for detailed scope of the WI

[105-e-NR-ePos-LS-01] Email discussion/approval for the reply LS to R1-2102306 till 5/25 (Alex, Qualcomm)

### Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays

Including DL, UL and DL+UL positioning methods, and UE-based and UE-assisted positioning solutions.

R1-2104277	Enhancement to mitigate gNB and UE Rx/Tx timing error	Huawei, HiSilicon

R1-2104359	Discussion on  potential enhancements for RX/TX timing delay mitigating	vivo

R1-2104520	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays			CATT

R1-2104590	Positioning accuracy improvement by mitigating timing delay	ZTE

R1-2104611	Discussion on mitigation of gNB/UE Rx/Tx timing errors	CMCC

R1-2104671	Enhancements on Timing Error Mitigations for improved Accuracy	Qualcomm Incorporated

R1-2104739	Enhancement of timing-based positioning by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays		OPPO

R1-2104871	Discussion on accuracy improvements by mitigating timing delays	InterDigital, Inc.

R1-2104905	Mitigation of UE/gNB TX/RX Timing Errors	Intel Corporation

R1-2104961	Mitigation of UE/gNB TX/RX Timing Errors	Intel Corporation

Withdrawn

R1-2105018	Mitigation of UE/gNB TX/RX Timing Errors	Intel Corporation

Withdrawn

R1-2105105	Positioning accuracy enhancements under timing errors	Apple

R1-2105168	Discussion on mitigating UE Rx/Tx and gNB Rx/Tx timing delays	Sony

R1-2105310	Discussion on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays			Samsung

R1-2105482	Discussion on accuracy improvement by mitigating UE Rx/Tx and gNB Rx/Tx timing delays	LG Electronics

R1-2105512	Views on mitigating UE and gNB Rx/Tx timing errors	Nokia, Nokia Shanghai Bell

R1-2105699	Discussion on mitigating UE and gNB Rx/Tx timing delays	NTT DOCOMO, INC.

R1-2105759	Mitigation of RX/TX timing delays for higher accuracy	MediaTek Inc.

R1-2105856	On methods for Rx/Tx timing delays mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2105859	Enhancements for mitigation of Tx/Rx Delays	Lenovo, Motorola Mobility

R1-2105908	Techniques mitigating Rx/Tx timing delays	Ericsson

[105-e-NR-ePos-01] Email discussion/approval on accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays with checkpoints for agreements on May 25, May 27 – Ren Da (CATT)

### Accuracy improvements for UL-AoA positioning solutions

R1-2104278	Enhancement for UL AoA positioning	Huawei, HiSilicon

R1-2104360	Discussion on potential enhancements for UL-AoA method	vivo

R1-2104521	Discussion on accuracy improvements for UL-AoA positioning solutions	CATT

R1-2104591	Accuracy improvement for UL-AoA positioning solutions	ZTE

R1-2104612	Discussion on UL-AoA enhancements	CMCC

R1-2104672	Potential Enhancements on UL-AOA positioning	Qualcomm Incorporated

R1-2104740	Enhancements for UL AoA Positioning	OPPO

R1-2104872	Discussion on enhancements for UL-AoA positioning solutions	InterDigital, Inc.

R1-2104906	NR Positioning UL-AoA Enhancements	Intel Corporation

R1-2104962	NR Positioning UL-AoA Enhancements	Intel Corporation

Withdrawn

R1-2105019	NR Positioning UL-AoA Enhancements	Intel Corporation

Withdrawn

R1-2105106	Positioning Accuracy enhancements for UL-AoA	Apple

R1-2105169	Discussion on accuracy improvements for UL-AoA positioning method	Sony

R1-2105311	Discussion on accuracy improvements for UL-AoA positioning solutions	Samsung

R1-2105483	Discussion on accuracy improvement for UL-AoA positioning	LG Electronics

R1-2105513	Views on enhancing UL AoA	Nokia, Nokia Shanghai Bell

R1-2105658	Discussion on UL AoA positioning enhancements	PML

R1-2105700	Discussion on UL-AoA positioning enhancements	NTT DOCOMO, INC.

R1-2105857	UL-AoA positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2105909	Enhancements of UL-AoA positioning solutions	Ericsson

[105-e-NR-ePos-02] Email discussion/approval on accuracy improvements for UL-AoA positioning solutions and with lower priority any issues related to the Others section, with checkpoints for agreements on May 24, May 27 – Alexey (Intel)

### Accuracy improvements for DL-AoD positioning solutions

Including UE-based and network-based (including UE-assisted) positioning solutions

R1-2104228	Accuracy improvements for DL-AoD positioning solutions 	BUPT

R1-2104279	Enhancement for DL AoD positioning	Huawei, HiSilicon

R1-2104361	Discussion on potential enhancements for DL-AoD method	vivo

R1-2104522	Discussion on accuracy improvements for DL-AoD positioning solutions	CATT

R1-2104592	Accuracy improvements for DL-AoD positioning solutions	ZTE

R1-2104613	Discussion on DL-AoD enhancements	CMCC

R1-2104673	Potential Enhancements on DL-AoD positioning	Qualcomm Incorporated

R1-2104741	Enhancements for DL-AoD positioning	OPPO

R1-2104842	Discussion on enhancements for DL-AoD positioning	CAICT

R1-2104844	Carrier Phase Based Downlink Angle of Departure Measurement 	DanKook University

Late submission

R1-2104873	Discussion on enhancements for DL-AoD positioning solutions	InterDigital, Inc.

R1-2104907	NR Positioning DL-AoD Enhancements	Intel Corporation

R1-2104963	NR Positioning DL-AoD Enhancements	Intel Corporation

Withdrawn

R1-2105020	NR Positioning DL-AoD Enhancements	Intel Corporation

Withdrawn

R1-2105107	Positioning Accuracy enhancements for DL-AoD	Apple

R1-2105170	Discussion on accuracy improvements for DL-AoD positioning method	Sony

R1-2105312	Discussion on accuracy improvements for DL-AoD positioning solutions	Samsung

R1-2105484	Discussion on accuracy improvement for DL-AoD positioning	LG Electronics

R1-2105514	Views on enhancing DL AoD	Nokia, Nokia Shanghai Bell

R1-2105563	Accuracy improvements for DL-AoD positioning solutions	Xiaomi

R1-2105701	Discussion on DL-AoD positioning enhancements	NTT DOCOMO, INC.

R1-2105858	DL-AoD positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

R1-2105860	DL-AoD Positioning Enhancements	Lenovo, Motorola Mobility

R1-2105910	Enhancements of DL-AoD positioning solutions	Ericsson

[105-e-NR-ePos-03] Email discussion/approval on accuracy improvements for DL-AoD positioning solutions with checkpoints for agreements on May 25, May 27 – Florent (Ericsson)

### Latency improvements for both DL and DL+UL positioning methods

R1-2104280	Positioning latency enhancements	Huawei, HiSilicon

R1-2104362	Discussion on latency enhancement for NR positioning	vivo

R1-2104523	Discussion on latency improvements for both DL and DL+UL positioning methods	CATT

R1-2104593	Discussion on latency reduction for NR positioning	ZTE

R1-2104614	Discussion on latency improvement for positioning	CMCC

R1-2104674	Enhancements for Latency Improvements for Positioning	Qualcomm Incorporated

R1-2104742	Enhancements on Latency Reduction in NR Positioning	OPPO

R1-2104874	Discussion on latency improvements for DL and DL+UL positioning methods	InterDigital, Inc.

R1-2104908	NR Positioning Latency Reduction	Intel Corporation

R1-2104964	NR Positioning Latency Reduction	Intel Corporation

Withdrawn

R1-2105021	NR Positioning Latency Reduction	Intel Corporation

Withdrawn

R1-2105108	Views on Rel-17 positioning latency reduction	Apple

R1-2105171	Considerations on Latency Improvements for DL and DL+UL positioning methods	Sony

R1-2105313	Discussion on latency improvements for both DL and DL+UL positioning methods	Samsung

R1-2105485	Discussion on latency improvements for NR positioning	LG Electronics

R1-2105515	Views on PHY Latency Reductions	Nokia, Nokia Shanghai Bell

R1-2105564	Latency improvements for both DL and DL+UL positioning method	Xiaomi

R1-2105760	Aspects for physical latency improvement	MediaTek Inc.

R1-2105861	Positioning Latency Reduction Enhancements	Lenovo, Motorola Mobility

R1-2105911	Latency improvements for both DL and DL+UL positioning methods	Ericsson

[105-e-NR-ePos-04] Email discussion/approval on latency improvements for both DL and DL+UL positioning methods with checkpoints for agreements on May 24, May 27 – Su (Huawei)

### Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

R1-2104198	NLOS Mitigation Enhancements	FUTUREWEI

R1-2104281	Enhancements to support multi-path and NLOS mitigation	Huawei, HiSilicon

R1-2104363	Discussion on potential enhancements for multipath/NLOS mitigation	vivo

R1-2104524	Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	CATT

R1-2104594	Enhancements on NLOS mitigation for NR positioning	ZTE

R1-2104675	Multipath Reporting in NR Positioning	Qualcomm Incorporated

R1-2104743	Discussion on multipath/NLOS mitigation for NR positioning	OPPO

R1-2104856	Potential enhancements of information reporting from UE for multipath/NLOS mitigation	China Telecom

R1-2104875	Discussion on multipath/NLOS mitigation for positioning	InterDigital, Inc.

R1-2104909	Mitigation of NLOS Problem for NR Positioning	Intel Corporation

R1-2104965	Mitigation of NLOS Problem for NR Positioning	Intel Corporation

Withdrawn

R1-2105022	Mitigation of NLOS Problem for NR Positioning	Intel Corporation

Withdrawn

R1-2105109	Views on potential enhancements for NLOS mitigation in Rel-17 positioning	Apple

R1-2105172	Discussion on enhanced reporting from UE and gNB for Multipath/NLOS mitigation	Sony

R1-2105314	Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation	Samsung

R1-2105486	Discussion on multipath/NLOS mitigation for positioning	LG Electronics

R1-2105516	Views on LoS/NLoS Identification and Mitigation	Nokia, Nokia Shanghai Bell

R1-2105565	Potential enhancements for multipath/NLOS mitigation	Xiaomi

R1-2105702	Discussion on multipath/NLOS mitigation for NR positioning	NTT DOCOMO, INC.

R1-2105862	Accuracy enhancements based on NLOS/Multipath Information Reporting	Lenovo, Motorola Mobility

R1-2105865	Potential positioning enhancements for multipath/NLOS mitigation	Fraunhofer IIS, Fraunhofer HHI

R1-2105912	Potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation		Ericsson

[105-e-NR-ePos-05] Email discussion/approval on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation with checkpoints for agreements on May 25, May 27 – Ryan (Nokia)

### Others

Including aspects for RAN2-led on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, and RAN2-led methods/measurements/signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions

R1-2104364	Discussion on inactive state positioning and on-demand PRS	vivo

R1-2104525	Discussion on on-demand transmission and reception of DL PRS and positioning solutions for UEs in RRC_ INACTIVE state	CATT

R1-2104595	Discussion on items led by RAN2 for NR positioning	ZTE

R1-2104615	Discussion on NW/UE efficiency enhancements	CMCC

R1-2104676	Enhancements Related to On Demand PRS And Positioning in RRC Inactive State	Qualcomm Incorporated

R1-2104744	Discussion on positioning for UE in RRC_INACTIVE and on-demand PRS	OPPO

R1-2104876	Discussion on on-demand PRS and INACTIVE mode positioning	InterDigital, Inc.

R1-2104880	Carrier/Subcarrier Phase Based Enhancement for 5G NR Positioning	DanKook University

Late submission

R1-2104910	Support of On-demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

R1-2104966	Support of On-demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

Withdrawn

R1-2105023	Support of On-demand DL PRS and NR Positioning for UEs in RRC-INACTIVE state	Intel Corporation

Withdrawn

R1-2105315	Discussion on positioning in inactive state	Samsung

R1-2105487	Discussion on other enhancements for positioning	LG Electronics

R1-2105517	Additional views on Inactive Mode Positioning and on-demand PRS	Nokia, Nokia Shanghai Bell

R1-2105534	Discussion on UL and DL+UL positioning in INACTIVE state	Huawei, HiSilicon

R1-2105566	On-demand PRS and positioning for in-active state UE	Xiaomi

R1-2105892	Potential physical layer impact to the RAN2-led topics	MediaTek Inc.

R1-2105913	On-demand transmission and reception of DL PRS for DL and DL+UL positioning	Ericsson

## Support of Reduced Capability NR Devices (5)

Please refer to RP-210918 for detailed scope of the WI

R1-2104178	Revised WI work plan for RedCap	Rapporteur (Ericsson)

### UE complexity reduction

#### Aspects related to reduced maximum UE bandwidth

[105-e-NR-R17-RedCap-01] Email discussion regarding aspects related to reduced maximum UE bandwidth – Johan (Ericsson)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2105999      FL summary #1 on reduced maximum UE bandwidth for RedCap                 Moderator (Ericsson)

R1-2106000      FL summary #2 on reduced maximum UE bandwidth for RedCap                 Moderator (Ericsson)

R1-2106092 (LS)

Agreements: Replace the RAN1#104bis-e working assumption with the following working assumption (for option 1) and working assumption (for option 2):

- Working assumption: After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

- Working assumption: After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

Agreements:

- Both during and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the maximum RedCap UE bandwidth is allowed.

- Working assumption: Both during and after initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs.

  - FFS: whether/how to avoid or minimize PUSCH resource fragmentation due to PUCCH transmission for the above case

  - Support the case when the centre frequency is assumed to be the same for the initial DL and UL BWPs in TDD.

    - FFS whether or not to additionally support the case when the centre frequency is different; if so, how to minimize centre frequency retuning

R1-2106001      FL summary #3 on reduced maximum UE bandwidth for RedCap                           Moderator (Ericsson)

Agreement:Take the following as an agreement, revised from the RAN1#104bis-e working assumption:

- A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE.

  - At least for FR1, FG 6-1 (“Basic BWP operation with restriction” as described in TR 38.822) is used as a starting point for the mandatory RedCap UE type capability.

    - This does not preclude support of FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822) as a UE capability for RedCap UEs.

Working assumption: Both during and after initial access, even for the scenario where the initial UL BWP for non-RedCap UEs is not configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP can optionally be configured/defined for RedCap UEs.

- RO sharing between RedCap and non-RedCap is not precluded.

Working assumption: For enabling/supporting that the RACH occasion (RO) associated with the best SSB falls within the RedCap UE bandwidth, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth), and this separate initial UL BWP for RedCap includes ROs for RedCap UEs.

- Note: these ROs can be dedicated for RedCap UEs or shared with non-RedCap UEs.

Working assumption:

- For enabling/supporting that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth).

  - FFS: whether/how the specification also supports separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation of the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping)

R1-2104179	Reduced maximum UE bandwidth for RedCap	Ericsson

R1-2104188	Discussion on Bandwidth Reduction for RedCap UEs	FUTUREWEI

R1-2104283	Reduced maximum UE bandwidth	Huawei, HiSilicon

R1-2104365	Discussion on reduced maximum UE bandwidth	vivo, Guangdong Genius

R1-2104428	Discussion on reduced maximum UE bandwidth for RedCap	Spreadtrum Communications

R1-2104526	Discussion on reduced maximum UE bandwidth	CATT

R1-2104543	Aspects related to reduced maximum UE bandwidth	Nokia, Nokia Shanghai Bell

R1-2104553	Discussion on reduced maximum UE bandwidth for RedCap	China Telecommunications

Withdrawn

R1-2104616	Discussion on reduced maximum UE bandwidth	CMCC

R1-2104677	BW Reduction for RedCap UE	Qualcomm Incorporated

R1-2104710	Bandwidth reduction for reduced capability NR devices	ZTE, Sanechips

R1-2104782	Discussion on reduced UE bandwidth	OPPO

R1-2104851	Discussion on reduced maximum UE bandwidth for RedCap	China Telecom

R1-2104881	Discussion on reduced maximum UE bandwidth	TCL Communication Ltd.

R1-2104911	On reduced max UE bandwidth for RedCap	Intel Corporation

R1-2104967	On reduced max UE bandwidth for RedCap	Intel Corporation

Withdrawn

R1-2105024	On reduced max UE bandwidth for RedCap	Intel Corporation

Withdrawn

R1-2105072	Reduced maximum UE band width for RedCap Ues	DENSO CORPORATION

R1-2105110	On reduced maximum UE bandwidth for Redcap	Apple

R1-2105217	Reduced maximum UE bandwidth for RedCap	Lenovo, Motorola Mobility

R1-2105983	Bandwidth Reduction for RedCap UEs	Samsung

Revision of R1-2105316

R1-2105429	Aspects related to the reduced maximum UE bandwidth of RedCap	LG Electronics

R1-2105567	Discussion on the reduced maximum UE bandwidth for RedCap	Xiaomi

R1-2105593	Discussion on aspects related to reduced maximum UE bandwidth	NEC

R1-2105635	Discussion on reduced maximum UE bandwidth	Sharp

R1-2105679	Aspects related to reduced maximum UE bandwidth	Panasonic Corporation

R1-2105703	Discussion on reduced maximum UE bandwidth for RedCap	NTT DOCOMO, INC.

R1-2105736	On reduced maximum bandwidth for RedCap UEs	MediaTek Inc.

R1-2105746	Reduced maximum bandwidth for RedCap UEs	InterDigital, Inc.

R1-2105751	Discussion on reduced maximum UE bandwidth	China Unicom

R1-2105800	Discussion on aspects related to reduced maximum UE bandwidth	ASUSTEK COMPUTER (SHANGHAI)

R1-2105882	On aspects related to reduced maximum UE BW	Nordic Semiconductor ASA

#### Aspects related to reduced number of Rx branches

R1-2105112	FL summary #1 for reduced number of Rx branches for RedCap	Apple

//This one is to use NWM – please use RAN1-105-e-NWM-NR-R17-RedCap-02 as the document name

[105-e-NR-R17-RedCap-02] Email discussion regarding aspects related to reduced number of Rx branches – Hong (Apple)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2105112       FL summary #1 for reduced number of Rx branches for RedCap          Apple

R1-2106081      FL summary #2 for reduced number of Rx branches for RedCap          Moderator (Apple).

Agreement:

- Redcap UE is mandated to support at least DCI format 0_0/1_0.

Agreement:

For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework.

- Detailed signalling is up to RAN2

R1-2106125	FL summary #3 for reduced number of Rx branches for RedCap 	Apple

Conclusion

- No consensus to support early identification of the number of Rx branches in Msg1/Msg3/MsgA for Redcap UE in Rel-17

Agreement:

Regarding DCI format 0_1/1_1 and DCI format 0_2 and 1_2,

- DCI format 0_1/1_1 are mandatory as in legacy. DCI 0_2/1_2 are optionally supported.

R1-2104180	Reduced number of Rx branches for RedCap	Ericsson

R1-2104189	Discussion on RX Branch Reduction for RedCap UEs	FUTUREWEI

R1-2104284	Reduced number of Rx branches for RedCap	Huawei, HiSilicon

R1-2104366	Discussion on reduced number of Rx branches	vivo, Guangdong Genius

R1-2104527	Discussion on reduced number of Rx branches	CATT

R1-2104544	Aspects related to reduced number of Rx branches	Nokia, Nokia Shanghai Bell

R1-2104617	Discussion on reduced number of Rx branches	CMCC

R1-2104678	RX Branch Reduction for RedCap UE	Qualcomm Incorporated

R1-2104711	Discussion on reduced number of UE Rx branches	ZTE, Sanechips

R1-2104783	Discussion on reduced number of UE Rx branches	OPPO

R1-2104912	On reduced number of Rx branches for RedCap	Intel Corporation

R1-2104968	On reduced number of Rx branches for RedCap	Intel Corporation

Withdrawn

R1-2105025	On reduced number of Rx branches for RedCap	Intel Corporation

Withdrawn

R1-2105111	On reduced number of Rx branches for Redcap	Apple

R1-2105218	Reduced number or Rx branches for RedCap	Lenovo, Motorola Mobility

R1-2105317	Discussion on reduced number of RX branches for RedCap UEs	Samsung

R1-2105430	Aspects related to the reduced number of Rx branches of RedCap	LG Electronics

R1-2105568	Aspects on reduced number of Rx branches	Xiaomi

R1-2105594	Discussion on aspects related to reduced number of Rx branches	NEC

R1-2105636	Discussion on reduced minimum number of Rx branches	Sharp

R1-2105704	Discussion on reduced minimum number of Rx branches for RedCap	NTT DOCOMO, INC.

R1-2105728	Aspects related to reduced number of Rx branches	Panasonic Corporation

R1-2105737	On reduced number of Rx branches for RedCap UEs	MediaTek Inc.

R1-2105747	Reduced number of Rx branches for RedCap UEs	InterDigital, Inc.

R1-2105752	Discussion on reduced number of Rx branches	China Unicom

R1-2105883	On aspects related to reduced number of Rx branches	Nordic Semiconductor ASA

#### Aspects related to duplex operation

[105-e-NR-R17-RedCap-03] Email discussion regarding aspects related to duplex operation – Chao (Qualcomm)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106006	FL summary #1 on duplex operation for RedCap	Qualcomm Inc.

Agreement:

- For Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission), a HD-FDD RedCap UE is not required to monitor ULCI

  - No special handling on the priority rule for PDCCH carrying ULCI

R1-2106145        FL summary#2 on duplex operation for RedCap                  Moderator (Qualcomm Inc.)

Conclusion:

- No consensus of specification support of semi-static UL/DL pattern to HD-FDD RedCap UEs in Rel-17.

Agreement:

- For Case 8 of valid RO overlapping with PDCCH in Type 0/0A/1/2 CSS set, down-select from the following options

  - Option 1: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that valid RO is prioritized over configured PDCCH

  - Option 2: Leave to UE implementation whether to receive the configured PDCCH or transmit the PRACH on the valid RO

  - Option 3: If configured PDCCH is in a Type-2 CSS set, then PDCCH is prioritized; otherwise the valid RO is prioritized

  - Option 4: Configured PDCCH is prioritized over valid RO

  - Option 5: Configured by network, e.g. via a priority indicator

  - FFS: whether or not the set of symbols overlapping with PDCCH in CSS set includes also Ngap symbols before the valid RO and whether the same value for Ngap in current spec is reused for HD-FDD

  - FFS whether a valid RO follows TDD’s or FDD’s definition, and if so, the corresponding impact

- FFS: whether or not the same principle is applied to PUSCH occasion of MSGA in 2-step RACH, if supported

R1-2104181	Duplex operation for RedCap	Ericsson

R1-2104285	Duplex operation for RedCap	Huawei, HiSilicon

R1-2104367	Discussion on RedCap half-duplex operation	vivo, Guangdong Genius

R1-2104429	Discussion on duplex operation for RedCap	Spreadtrum Communications

R1-2104528	Discussion on HD-FDD operation	CATT

R1-2104545	Aspects related to duplex operation	Nokia, Nokia Shanghai Bell

R1-2104618	Discussion on collision handling of HD-FDD operation	CMCC

R1-2104679	Type-A HD-FDD for RedCap UE	Qualcomm Incorporated

R1-2104712	HD-FDD for reduced capability NR devices	ZTE, Sanechips

R1-2104784	On half-duplex operation	OPPO

R1-2104852	Discussion on duplex operation for RedCap	China Telecom

R1-2104913	On support of HD-FDD for RedCap	Intel Corporation

R1-2104969	On support of HD-FDD for RedCap	Intel Corporation

Withdrawn

R1-2105026	On support of HD-FDD for RedCap	Intel Corporation

Withdrawn

R1-2105053	Discussion on aspects related to duplex operation	Potevio Company Limited

R1-2105113	Duplex operation for RedCap	Apple

R1-2105219	Half duplex operation for RedCap	Lenovo, Motorola Mobility

R1-2105318	HD-FDD Operation for RedCap UEs	Samsung

R1-2105431	Aspects related to the duplex operation of RedCap	LG Electronics

R1-2105569	Discussion on Half-duplex FDD operation of Redcap UE	Xiaomi

R1-2105637	Discussion on the duplex operation of redcap UEs	Sharp

R1-2105705	Discussion on duplex operation for RedCap	NTT DOCOMO, INC.

R1-2105729	Aspects related to duplex operation	Panasonic Corporation

R1-2105738	On half duplex operation for RedCap UEs	MediaTek Inc.

R1-2105748	Duplex operation for RedCap UEs	InterDigital, Inc.

R1-2105801	Discussion on aspects related to duplex operation	ASUSTEK COMPUTER (SHANGHAI)

R1-2105823	Discussion on aspects related to duplex operation	Asia Pacific Telecom, FGI

R1-2105875	Discussion on duplex operation for RedCap UE	WILUS Inc.

R1-2105884	On aspects related to duplex operation	Nordic Semiconductor ASA

R1-2105900	Half-duplex FDD redcap operation	Sony

#### Other aspects

Including maximum number of DL MIMO layers and relaxed maximum modulation order

[105-e-NR-R17-RedCap-04] Email discussion regarding other aspects of UE complexity reduction – Debdeep (Intel)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106027	FL summary #1 on other aspects of UE complexity reduction for RedCap	Moderator (Intel)

Conclusion:

- For a RedCap UE, when motivated by reduced max number of DL MIMO layers modifications to CSI measurement and/or reporting mechanisms are not pursued in Rel-17.

Agreements:

- For a RedCap UE, 64QAM MCS tables (Table 5.1.3.1-1 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-1 in TS 38.214 for UL w/ transform precoding respectively) are the “default” ones and are mandatory.

- The following is optionally supported by RedCap UEs:

  - 256QAM MCS tables (Table 5.1.3.1-2 in TS 38.214 for DL and UL OFDM)

  - 64QAM low SE MCS tables (Table 5.1.3.1-3 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively)

Agreements:

- For a RedCap UE, “CQI table 1” (Table 5.2.2.1-2 in TS 38.214), that corresponds to MCS Table 5.1.3.1-1 in TS 38.214, is mandatory.

- The following is optionally supported by a RedCap UE:

  - “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) that corresponds to MCS Table 5.1.3.1-2 in TS 38.214 (256QAM MCS table)

  - “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) that corresponds to MCS Table 5.1.3.1-3 in TS 38.214 (64QAM low SE MCS table)

R1-2106166         FL summary #2 on other aspects of UE complexity reduction for RedCap Moderator (Intel)

Agreement:

- Both 256QAM MCS table for PDSCH and “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) are supported by a RedCap UE indicating support of 256QAM for PDSCH.

Agreement:

- For a RedCap UE, support of 64QAM low SE MCS table for PDSCH and support of “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) are not coupled and capability of each can be reported independent of the other.

Agreement:

- For a RedCap UE, support of 64QAM low SE MCS table for PDSCH (Table 5.1.3.1-3 in TS 38.214) and support of 64QAM low SE MCS tables for PUSCH (Table 5.1.3.1-3 in TS 38.214 for UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively) are not coupled and capability of each can be reported independent of the other.

R1-2104182	MIMO and modulation support for RedCap	Ericsson

R1-2104190	Discussion on Modulation order and parameters	FUTUREWEI

R1-2104286	Reduced maximum  MIMO layers and reduced maximum modulation order for RedCap	Huawei, HiSilicon

R1-2104368	Other feature reductions for RedCap NR devices	vivo, Guangdong Genius

R1-2104430	Discussion on relaxed maximum modulation order for RedCap	Spreadtrum Communications

R1-2104529	Discussion on other aspects related to complexity reduction	CATT

R1-2104554	Other UE Complexity Reduction Aspects	Nokia, Nokia Shanghai Bell

R1-2104619	Discussion on other aspects of reduced UE complexity	CMCC

R1-2104680	Other Aspects of UE Complexity Reduction	Qualcomm Incorporated

R1-2104713	Discussion on modulation order and MIMO layers for RedCap	ZTE, Sanechips

R1-2104914	On other complexity reduction features for RedCap	Intel Corporation

R1-2104970	On other complexity reduction features for RedCap	Intel Corporation

Withdrawn

R1-2105027	On other complexity reduction features for RedCap	Intel Corporation

Withdrawn

R1-2105114	On relaxed maximum modulation order	Apple

R1-2105319	Other aspects for complexity reduction for RedCap UEs	Samsung

R1-2105570	Discussion on relaxed maximum modulation order and relaxed MIMO layer	Xiaomi

R1-2105595	MIMO aspects for RedCap	NEC

R1-2105706	Discussion on other aspects of UE complexity reduction for RedCap	NTT DOCOMO, INC.

### RAN1 aspects for RAN2-led features for RedCap

Including RAN2-led aspects related ta one RedCap UE type, enabling RedCap UEs to be explicitly identifiable to networks, and a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not

[105-e-NR-R17-RedCap-05] Email discussion regarding RAN1 aspects for RAN2-led features – Shinya (DoCoMo)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2105981

R1-2106094          FL summary #2 on RAN1 aspects for RAN2-led features for RedCap              Moderator (NTT DOCOMO)

Working assumption:

- For 4-step RACH, support the early indication of RedCap UEs at least in Msg1.

  - The early indication in Msg1 can be configured to be enabled/disabled

    - FFS How to support enable/disable the early indication

  - FFS details e.g.:

    - separate initial UL BWP

    - separate PRACH resource

    - PRACH preamble partitioning

  - FFS the possibility of supporting Msg3 for the early indication

R1-2106146          FL summary#3 on RAN1 aspects for RAN2-led features for RedCap              Moderator (NTT DOCOMO)

Agreement:

- Early indication of RedCap UEs in Msg1 can be enabled/disabled via SIB

Send an LS to RAN2 informing them the above working assumption and the agreement for early indication, possibly also RAN2-related agreements – Shinya (DCM)

Working assumption:

- RedCap UE type is defined based on one of the following options

  - Option 2: Only include the reduced capabilities that the network needs to know during initial access, if any.

  - Option 4: The corresponding minimum set of the reduced capabilities that one RedCap UE type shall mandatorily support

  - FFS: details of the set of reduced capabilities

Conclusion:

- RAN1 postpones the discussion on constraining of reduced capabilities, and if deemed necessary, RAN1 can come back

Agreement:

- Support 2-step RACH for RedCap UEs as an optional feature

  - FFS details of early indication in MsgA, e.g.:

    - Separation of 2-step RACH resources or MsgA preambles

    - Separation of initial UL BWP

    - Using a new indication in MsgA PUSCH part

  - Note: Discussion on 4-step RACH for early indication should be prioritised

R1-2104183	RAN1 aspects for RAN2-led features for RedCap	Ericsson

R1-2104191	Discussion on the Identification of RedCap UEs	FUTUREWEI

R1-2104287	RAN1 aspects of RedCap UE type and identification	Huawei, HiSilicon

R1-2104369	Higher layer support for RedCap	vivo, Guangdong Genius

R1-2104431	Discussion on early indication for RedCap UE	Spreadtrum Communications

R1-2104530	Discussion on higher layer support of RedCap	CATT

R1-2104546	Higher layer support of Reduced Capability NR Devices	Nokia, Nokia Shanghai Bell

R1-2104562	Design consideration for Higher layer support of RedCap	Sierra Wireless, S.A.

R1-2104620	Discussion on higher layer support of RedCap UE	CMCC

R1-2104681	Cross Layer Design Considerations for RedCap Device	Qualcomm Incorporated

R1-2104714	Higher layer support of Reduced Capability NR devices	ZTE, Sanechips

R1-2104785	Mechanism in higher&PHY layer for Reduced Capability NR Devices	OPPO

R1-2104853	Discussion on RAN1 aspects for RAN2-led features for RedCap	China Telecom

R1-2104915	On RedCap UE types: Definition, access control, and identification	Intel Corporation

R1-2104971	On RedCap UE types: Definition, access control, and identification	Intel Corporation

Withdrawn

R1-2105028	On RedCap UE types: Definition, access control, and identification	Intel Corporation

Withdrawn

R1-2105115	On Higher Layer Support of Redcap Devices	Apple

R1-2105173	UE identification of redcap devices	Sony

R1-2105220	UE identification and access control for RedCap	Lenovo, Motorola Mobility

R1-2105320	RAN1 aspects for RAN2-led features for RedCap	Samsung

R1-2105432	RAN1 aspects for RAN2-led features for RedCap	LG Electronics

R1-2105571	Discussion on the early indication and access control	Xiaomi

R1-2105638	RAN1 aspects for RAN2-led features for RedCap	Sharp

R1-2105707	Discussion on RAN1 aspects for RAN2-led features for RedCap	NTT DOCOMO, INC.

R1-2105749	Identification and restriction of RedCap UEs	InterDigital, Inc.

R1-2105876	Discussion on higher layer support of Redcap UE	WILUS Inc.

R1-2105885	On RedCap UE early identification	Nordic Semiconductor ASA

### Others

R1-2104184	Ensuring coexistence between RedCap and non-RedCap UEs	Ericsson, Deutsche Telekom, NTT DOCOMO, Softbank, Telecom Italia, Telstra, Verizon Wireless, Vodafone

R1-2104370	Discussion on reduced capability signaling	vivo, Guangdong Genius

R1-2104531	Views on remaining issues of RedCap	CATT

R1-2104547	Coverage recovery aspects for RedCap UEs	Nokia, Nokia Shanghai Bell

R1-2104621	Discussion on potential modification  of existing DCI formats	CMCC

R1-2104715	NR UE features for RedCap	ZTE, Sanechips

R1-2104786	Other considerations for reduced UE capability	OPPO

R1-2105433	Discussion on other aspects of RedCap	LG Electronics

R1-2105535	On RedCap UL transmission	Huawei, HiSilicon

R1-2105572	Discussion on the transmission of system information for RedCap	Xiaomi

R1-2105802	Discussion on other aspects for reduced capability UE	ASUSTEK COMPUTER (SHANGHAI)

## UE Power Saving Enhancements (3)

Please refer to RP-200938 for detailed scope of the WI

### Enhancements for idle/inactive-mode UE power saving

#### Potential paging enhancements

[105-e-NR-R17-PowSav-01] Email discussion regarding potential paging enhancements – Weide (MediaTek)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106076

Agreement:

For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported.

R1-2104221	Discussion on power saving enhancements for paging	ZTE, Sanechips

R1-2104251	Paging enhancements for UE power saving in IDLE/inactive mode	Huawei, HiSilicon

R1-2104371	Paging enhancements for idle/inactive mode UE power saving	vivo

R1-2104432	Discussion on potential paging enhancements for UE Power Saving	Spreadtrum Communications

R1-2104532	Paging enhancement for UE power saving	CATT

R1-2104622	Discussion on paging early indication design	CMCC

R1-2104682	Paging enhancements for idle/inactive UE power saving	Qualcomm Incorporated

R1-2104787	Further discussion on Paging enhancements for power saving	OPPO

R1-2104916	On Paging Enhancements for UE Power Saving	Intel Corporation

R1-2104972	On Paging Enhancements for UE Power Saving	Intel Corporation

Withdrawn

R1-2105029	On Paging Enhancements for UE Power Saving	Intel Corporation

Withdrawn

R1-2105116	Paging early indication for idle/inactive-mode UE	Apple

R1-2105174	Considerations on potential paging enhancements	Sony

R1-2105321	Discussion on paging enhancements	Samsung

R1-2105386	On Paging Enhancements for Idle/Inactive Mode UE Power Saving	MediaTek Inc.

R1-2105434	Discussion on potential paging enhancements	LG Electronics

R1-2105474	On paging enhancement	Panasonic

R1-2105504	On paging enhancements for UE power saving	Nokia, Nokia Shanghai Bell

R1-2105573	Paging enhancements for power saving	Xiaomi

R1-2105708	Discussion on paging enhancements	NTT DOCOMO, INC.

R1-2105742	Paging enhancements for UE power saving	InterDigital, Inc.

R1-2105770	Paging enhancement for UE power saving	Lenovo, Motorola Mobility

R1-2105791	Design of Paging Enhancements	Ericsson

R1-2105886	On  paging early indication	Nordic Semiconductor ASA

#### TRS/CSI-RS occasion(s) for idle/inactive UEs

R1-2105323	Moderator summary for TRS/CSI-RS occasion(s) for idle/inactive Ues	Moderator (Samsung)

//This one is to use NWM – please use RAN1-105-e-NWM-NR-R17-PowSav-02 as the document name

[105-e-NR-R17-PowSav-02] Email discussion regarding TRS/CSI-RS occasions for idle/inactive UEs – Qiongjie (Samsung)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

Update on 5/22:

Agreement:

Confirm the following working assumption:

Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

- FFS details, including paging DCI and/or PEI for L1 based signaling

- FFS SIB-based signaling/configuration

  - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling

Agreement:

For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint

- e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources

- e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources

- FFS maximum number of configured RS resources per physical layer availability indication to support.

- FFS whether availability/unavailability information is for all or some of configured RS resources

Agreement:

Support applicable values for the following configuration parameters as below.

- powerControlOffsetSS: {-3, 0, 3, 6}dB

- scramblingID: 0 to 1023

- firstOFDMSymbolInTimeDomain: 0 to 9

  - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4

- startingRB: 0 to 274

- nrofRBs: 24 to 276

Agreement:

The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63.

- FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration

- FFS: QCL type, which is predetermined

R1-2104222	TRS for RRC idle and inactive UEs	ZTE, Sanechips

R1-2104252	Assistance RS occasions for IDLE/inactive mode	Huawei, HiSilicon

R1-2104308	TRS/CSI-RS occasions for idle/inactive UE	TCL Communication Ltd.

R1-2104372	TRS/CSI-RS occasion(s) for idle/inactive UEs	vivo

R1-2104433	Consideration on TRS/CSI-RS occasion(s) for idle/inactive UEs	Spreadtrum Communications

R1-2104533	Configuration of TRS/CSI-RS for paging enhancement	CATT

R1-2104623	Discussion on TRS/CSI-RS occasion(s) for IDLE/INACTIVE-mode UEs	CMCC

R1-2104683	TRS/CSI-RS for idle/inactive UE power saving	Qualcomm Incorporated

R1-2104788	Further discussion on RS occasion for idle/inactive UEs	OPPO

R1-2104917	Discussion on TRS/CSI-RS Design in idle/inactive mode	Intel Corporation

R1-2104973	Discussion on TRS/CSI-RS Design in idle/inactive mode	Intel Corporation

Withdrawn

R1-2105030	Discussion on TRS/CSI-RS Design in idle/inactive mode	Intel Corporation

Withdrawn

R1-2105117	Indication of TRS configurations for idle/inactive-mode UE power saving	Apple

R1-2105175	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Sony

R1-2105322	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	Samsung

R1-2105387	On TRS/CSI-RS occasion(s) for idle/inactive mode UE power saving	MediaTek Inc.

R1-2105435	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	LG Electronics

R1-2105475	Potential enhancements for TRS/CSI-RS occasion(s) for idle/inactive UEs	Panasonic

R1-2105506	On RS information to IDLE/Inactive mode Ues	Nokia, Nokia Shanghai Bell

R1-2105574	On TRS/CSI-RS configuration and indication for idle/inactive UEs	Xiaomi

R1-2105639	Discussion on TRS/CSI-RS occasions for idle/inactive UEs	Sharp

R1-2105709	Discussion on TRS CSI-RS occasion for idleinactive UEs	NTT DOCOMO, INC.

R1-2105743	Discussion on TRS/CSI-RS occasion(s) for idle/inactive UEs	InterDigital, Inc.

R1-2105771	Provision of TRS/CSI-RS for idle/inactive UEs	Lenovo, Motorola Mobility

R1-2105792	Provisioning of TRS occasions to Idle/Inactive UEs	Ericsson

R1-2105887	On TRS design for idle/inactive UEs	Nordic Semiconductor ASA

#### Others

R1-2104223	Additional simulation results of UE power consumption in RRC idle and inactive state	ZTE, Sanechips

R1-2104373	Discussion on paging grouping	vivo

R1-2104534	Details of PEI configuration	CATT

R1-2104789	Paging and TRS indication in idle/inactive modes	OPPO

R1-2105527	Analysis on power consumption for IDLE mode UE	Huawei, HiSilicon

R1-2105793	Modeling of Network Power Consumption	Ericsson

### Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime

[105-e-NR-R17-PowSav-03] Email discussion regarding potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime – Xiaodong (vivo)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106040

R1-2106041

R1-2104224	Extension to Rel-16 DCI-based power saving adaptation during DRX Active Time	ZTE, Sanechips

R1-2104253	Extensions to Rel-16 DCI-based power saving adaptation for an active BWP	Huawei, HiSilicon

R1-2104374	Discussion on DCI-based power saving adaptation in connected mode	vivo

R1-2104434	Discussion on power saving techniques for connected-mode UEs	Spreadtrum Communications

R1-2104535	PDCCH monitoring adaptation	CATT

R1-2104624	Discussion on PDCCH monitoring reduction during DRX active time	CMCC

R1-2104684	DCI-based power saving adaptation during DRX ActiveTime	Qualcomm Incorporated

R1-2104790	DCI-based power saving adaptation solutions	OPPO

R1-2104918	Discussion on DCI-based UE Power Saving Schemes during active time	Intel Corporation

R1-2104974	Discussion on DCI-based UE Power Saving Schemes during active time	Intel Corporation

Withdrawn

R1-2105031	Discussion on DCI-based UE Power Saving Schemes during active time	Intel Corporation

Withdrawn

R1-2105118	Enhanced DCI-based power saving adaptation	Apple

R1-2105263	DCI-based Power Saving Enhancements	Fraunhofer HHI, Fraunhofer IIS

R1-2105324	Discussion on DCI-based power saving techniques	Samsung

R1-2105388	On enhancements to DCI-based UE power saving during DRX active time	MediaTek Inc.

R1-2105436	Discussion on DCI-based power saving adaptation during DRX ActiveTime	LG Electronics

R1-2105476	Potential extension(s) to Rel-16 DCI-based power saving adaptation during DRX ActiveTime			Panasonic

R1-2105505	UE power saving enhancements for Active Time	Nokia, Nokia Shanghai Bell

R1-2105710	Discussion on extension to DCI-based power saving adaptation	NTT DOCOMO, INC.

R1-2105744	PDCCH monitoring reduction in Active Time	InterDigital, Inc.

R1-2105758	Discussion on DCI-based power saving adaptation	ITRI

R1-2105772	Enhanced DCI based power saving adaptation	Lenovo, Motorola Mobility

R1-2105794	Design of active time power savings mechanisms	Ericsson

R1-2105824	Discussion on extension(s) to Rel-16 DCI-based power saving adaptation	Asia Pacific Telecom, FGI

R1-2105850	A common framework for SSSG switching and PDCCH skipping	ASUSTeK

R1-2105888	On PDCCH monitoring adaptation	Nordic Semiconductor ASA

### Others

R1-2104225	Further discussion on potential power saving schemes for RRC connected UEs	ZTE, Sanechips

R1-2104375	Discussion on RLM/BFD relax	vivo

R1-2104435	Discussion on other power saving techniques	Spreadtrum Communications

R1-2104536	Link level performance for UE power saving	CATT

R1-2104791	Discussion on RLM relaxation	OPPO

R1-2105528	Other considerations on power saving in Rel-17	Huawei, HiSilicon

R1-2105795	Evaluation results for UE power saving schemes	Ericsson

## NR coverage enhancement (5)

Please refer to RP-210855 for detailed scope of the WI

### PUSCH enhancements

#### Enhancements on PUSCH repetition type A

[105-e-NR-R17-CovEnh-01] Email discussion regarding enhancements for PUSCH repetition type A – Toshi (Sharp)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2105992

Update on 5/22:

Agreement:

- RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17

R1-2106155

Agreement:

- Down-selection in RAN1#106-e:

- Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method,

- Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots.

FL proposal #2-1 after the 2nd round discussion

- Alt 1 of the following agreement in RAN1#104-e includes the following two steps

  - Step 1: Determination of available slot for counting K repetitions, where the determination is based on RRC configurations.

  - Step 2: If any additional collision happens (due to e.g., dynamic SFI, UL CI, priority indication, or some RRC configurations, with details to be further discussed) for the PUSCH repetition needs to be dropped,  the PUSCH repetition is omitted (still counted as part of the overall repetitions), but is not further postponed.

Agreements: Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded) - Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC). - Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Conclusion:

- The following agreement in RAN1#104-e is applied to all slots including special slots.

Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details

R1-2104240	Discussion on coverage enhancements for PUSCH repetition type A	Huawei, HiSilicon

R1-2104290	Discussion on enhancements on PUSCH repetition Type A for low latency requirement	NICT

Withdrawn

R1-2104291	Discussion on enhancements on PUSCH repetition Type A for low latency requirement	NICT, TOYOTA MOTOR CORPORATION

R1-2104330	Discussion on enhanced PUSCH repetition type A	ZTE

R1-2104376	Discussion on enhancement for PUSCH repetition type A	vivo

R1-2104537	Discussion on enhancements on PUSCH repetition type A	CATT

R1-2104625	Discussion on enhancements on PUSCH repetition type A	CMCC

R1-2104685	Enhancements on PUSCH Repetition Type A	Qualcomm Incorporated

R1-2104792	Enhancements on PUSCH repetition type A	OPPO

R1-2104846	Enhancements on PUSCH repetition type A	China Telecom

R1-2104859	Type-A PUSCH repetition for coverage enhancement	InterDigital, Inc.

R1-2104919	Enhancements on PUSCH repetition type A	Intel Corporation

R1-2104975	Enhancements on PUSCH repetition type A	Intel Corporation

Withdrawn

R1-2105032	Enhancements on PUSCH repetition type A	Intel Corporation

Withdrawn

R1-2105119	Discussion on PUSCH repetition type A enhancement	Apple

R1-2105146	Discussion on enhancements on PUSCH repetition Type A	Panasonic Corporation

R1-2105255	Discussion on PUSCH repetition type A	NEC

R1-2105325	Enhancements on PUSCH repetition type A	Samsung

R1-2105488	Discussions on PUSCH repetition type A enhancements	LG Electronics

R1-2105511	Design considerations for PUSCH repetition Type A Enhancements	Sierra Wireless, S.A.

R1-2105575	Enhancements on PUSCH repetition type A	Xiaomi

R1-2105640	Enhancements on PUSCH repetition type A	Sharp

R1-2105652	PUSCH Repetition Type A Enhancement	Ericsson

R1-2105711	Enhancement on PUSCH repetition type A	NTT DOCOMO, INC.

R1-2105773	Enhancements on PUSCH repetition type A	Lenovo, Motorola Mobility

R1-2105877	Discussion on enhancements on PUSCH repetition type A	WILUS Inc.

R1-2105901	Enhancements on PUSCH repetition type A	Nokia, Nokia Shanghai Bell

#### TB processing over multi-slot PUSCH

[105-e-NR-R17-CovEnh-02] Email discussion regarding TB processing over multi-slot PUSCH – Marco (Nokia)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2105996

Working assumption:

For TBS determination of TBoMS:

- NohPRB is configured by xOverhead and represents the overhead per slot.

- NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated.

Note: xOverhead configuration is as per Rel-15/16.

Agreement:

The following 2 options for time domain resource determination for TBoMS are considered for down-selection during RAN1 #105-e:

- Option 1: Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA.

- Option 2: Time domain resource determination for TBoMS can be performed via PUSCH repetition Type A like TDRA or via PUSCH repetition Type B like TDRA.

  - The use of PUSCH repetition Type B like TDRA for time domain resource determination is according to an additional UE capability for a TBoMS capable UE.

  - FFS DMRS pattern for PUSCH repetition Type B like TDRA

R1-2104242	Discussion on TB processing over multi-slot PUSCH	Huawei, HiSilicon

R1-2104297	On TB processing over multiple slots for PUSCH	Indian Institute of Tech (H)

R1-2104331	Discussion on TB processing over multi-slot PUSCH	ZTE

R1-2104377	Discussion on PUSCH TB processing over multiple slots	vivo

R1-2104436	Discussion on TB processing over multi-slot PUSCH	Spreadtrum Communications

R1-2104538	Discussion on TB processing over multi-slot PUSCH	CATT

R1-2104626	Discussion on TB processing over multi-slot PUSCH	CMCC

R1-2104686	TB processing over multi-slot PUSCH	Qualcomm Incorporated

R1-2104793	Issues for TB over multi-slot PUSCH	OPPO

R1-2104847	Discussion on TB processing over multi-slot PUSCH	China Telecom

R1-2104860	TB processing over multiple slots	InterDigital, Inc.

R1-2104920	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

R1-2104976	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

Withdrawn

R1-2105033	Discussion on TB processing over multi-slot PUSCH	Intel Corporation

Withdrawn

R1-2105064	Views on TB processing over multi-slot PUSCH	Fujitsu

R1-2105120	Discussion on TB processing over multi-slot PUSCH	Apple

R1-2105147	Discussion on TB processing over multi-slot PUSCH	Panasonic Corporation

R1-2105256	Discussion on TB processing over multi-slot PUSCH	NEC

R1-2105326	TB processing over multi-slot PUSCH	Samsung

R1-2105968	Discussion on TB processing over multi-slot PUSCH	MediaTek Inc.

Revision of R1-2105384

R1-2105489	Discussions on TB processing over multi-slot PUSCH	LG Electronics

R1-2105510	Design Considerations for TB processing over multi-slot PUSCH	Sierra Wireless, S.A.

R1-2105576	Discussion on TB processing over multi-slot PUSCH	Xiaomi

R1-2105641	TB processing over multi-slot PUSCH	Sharp

R1-2105653	TB Processing over Multi-Slot PUSCH	Ericsson

R1-2105712	TB processing over multi-slot PUSCH	NTT DOCOMO, INC.

R1-2105774	Enhancements for TB processing over multi-slot PUSCH	Lenovo, Motorola Mobility

R1-2105878	Discussion on TB processing over multi-slot PUSCH	WILUS Inc.

R1-2105902	Transport block processing for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

#### Joint channel estimation for PUSCH

//This one is to use NWM – please use RAN1-105-e-NWM-NR-R17-CovEnh-03 as the document name

[105-e-NR-R17-CovEnh-03] Email discussion regarding joint channel estimation for PUSCH – Jianchi (China Telecom)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2105979

Agreement:

- Joint channel estimation over non-back-to-back PUSCH transmissions within one slot is not supported.

Agreement:

- Definition of the maximum duration: a maximum time duration during which UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- FFS whether or not such a definition is necessary for RAN1 specifications.

  - Note: whether such a definition is to be specified in RAN4 specifications is up to RAN4.

- FFS the maximum duration may be reported by UE.

- Note: it is understood that for a UE, the maximum duration is no less than the time domain window duration

Agreement: Send LS to RAN4 asking the following questions

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

Agreement:

- Optimization of DMRS granularity in time domain for PUSCH is not considered for joint channel estimation in Rel-17.

R1-2104241	Discussion on joint channel estimation for PUSCH	Huawei, HiSilicon

R1-2104332	Discussion on joint channel estimation for PUSCH	ZTE

R1-2104378	Discussion on Joint channel estimation for PUSCH	vivo

R1-2104437	Discussion on joint channel estimation for PUSCH	Spreadtrum Communications

R1-2104539	Discussion on joint channel estimation for PUSCH	CATT

R1-2104627	Discussion on joint channel estimation for PUSCH	CMCC

R1-2104687	Joint channel estimation for PUSCH	Qualcomm Incorporated

R1-2104794	Consideration on Joint channel estimation for PUSCH	OPPO

R1-2104848	Discussion on joint channel estimation for PUSCH	China Telecom

R1-2104861	Joint channel estimation for PUSCH	InterDigital, Inc.

R1-2104882	Discussion on joint channel estimation for PUSCH	TCL Communication Ltd.

R1-2104921	Discussion on joint channel estimation for PUSCH	Intel Corporation

R1-2104977	Discussion on joint channel estimation for PUSCH	Intel Corporation

Withdrawn

R1-2105034	Discussion on joint channel estimation for PUSCH	Intel Corporation

Withdrawn

R1-2105121	Discussion on joint channel estimation for PUSCH	Apple

R1-2105176	Joint channel estimation for PUSCH	Sony

R1-2105327	Joint channel estimation for PUSCH	Samsung

R1-2105394	Discussion on joint channel estimation for PUSCH	MediaTek Inc.

R1-2105397	Discussion on joint channel estimation for PUSCH	Panasonic Corporation

R1-2105490	Discussions on joint channel estimation for PUSCH	LG Electronics

R1-2105509	Design Considerations for Joint channel estimation for PUSCH	Sierra Wireless, S.A.

R1-2105577	Joint channel estimation for PUSCH	Xiaomi

R1-2105642	Joint channel estimation for multi-slot PUSCH	Sharp

R1-2105654	Joint Channel Estimation for PUSCH	Ericsson

R1-2105713	Joint channel estimation for PUSCH	NTT DOCOMO, INC.

R1-2105775	Enhancements for joint channel estimation for multiple PUSCH	Lenovo, Motorola Mobility

R1-2105879	Discussion on joint channel estimation for PUSCH	WILUS Inc.

R1-2105903	Joint channel estimation for PUSCH coverage enhancements	Nokia, Nokia Shanghai Bell

### PUCCH enhancements

Including dynamic PUCCH repetition factor indication, and DMRS bundling across PUCCH repetitions (When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH)

[105-e-NR-R17-CovEnh-04] Email discussion regarding PUCCH enhancements – Yi (Qualcomm)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106014

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements.

- Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible.

R1-2106154

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication.

- Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor.

  - FFS: RRC signaling enhancement details

Conclusion: For PUCCH repetitions, the following use cases are deprioritized in RAN1 work on PUCCH DMRS bundling

   Use case 1: back-to-back PUCCH repetitions within one slot.

   Use case 2: non-back-to-back PUCCH repetitions within one slot.

‐   Use case 2a: no uplink transmission in the middle of two PUCCH repetitions

‐   Use case 2b: other uplink transmissions in the middle of two PUCCH repetitions

R1-2104243	Discussion on PUCCH coverage enhancement	Huawei, HiSilicon

R1-2104333	Discussion on coverage enhancements for PUCCH	ZTE

R1-2104379	Discussion on PUCCH enhancements	vivo

R1-2104438	Discussion on PUCCH enhancements	Spreadtrum Communications

R1-2104540	Discussion on PUCCH enhancement	CATT

R1-2104628	Discussion on PUCCH enhancements	CMCC

R1-2104688	PUCCH enhancements	Qualcomm Incorporated

R1-2104795	PUCCH enhancements for coverage	OPPO

R1-2104849	Discussion on PUCCH enhancements	China Telecom

R1-2104862	Discussions on PUCCH enhancements	InterDigital, Inc.

R1-2104922	Discussion on PUCCH enhancements	Intel Corporation

R1-2104978	Discussion on PUCCH enhancements	Intel Corporation

Withdrawn

R1-2105035	Discussion on PUCCH enhancements	Intel Corporation

Withdrawn

R1-2105122	PUCCH coverage enhancement	Apple

R1-2105149	Discussion on PUCCH enhancement for NR coverage enhancement	Panasonic Corporation

R1-2105224	PUCCH enhancements	ETRI

R1-2105239	PUCCH enhancements	ETRI

Withdrawn

R1-2105257	Discussion on PUCCH enhancements	NEC

R1-2105328	PUCCH enhancements	Samsung

R1-2105360	PUCCH enhancements	ETRI

Withdrawn

R1-2105491	Discussions on coverage enhancement for PUCCH	LG Electronics

R1-2105578	PUCCH coverage enhancement	Xiaomi

R1-2105643	PUCCH coverage enhancement	Sharp

R1-2105655	PUCCH Dynamic Repetition and DMRS Bundling	Ericsson

R1-2105714	PUCCH enhancements	NTT DOCOMO, INC.

R1-2105776	Enhancements for PUCCH repetition	Lenovo, Motorola Mobility

R1-2105904	PUCCH coverage enhancements	Nokia, Nokia Shanghai Bell

### Type A PUSCH repetitions for Msg3

[105-e-NR-R17-CovEnh-05] Email discussion regarding Type A PUSCH repetitions for Msg 3 – Xianghui (ZTE)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106007

Update on 5/22:

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold.

- FFS the determination of the RSRP threshold.

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options.

- Option 1: Use the same mechanism as supported for Msg3 initial transmission.

- Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission.

- The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214.

- The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214.

- FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Agreement:

- For requesting Msg3 PUSCH repetition, support the following:

  - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs.

    - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

    - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH).

  - FFS whether or not to additionally support one (& only one) more option:

    - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

    - E.g., Option 3: Use separate RO, which include

      - the separate RO configured by a separate RACH configuration index from legacy UE, and

      - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated.

Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon.

- A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols.

  - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition.

  - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

R1-2104244	Msg3 repetition for coverage enhancement	Huawei, HiSilicon

R1-2104334	Discussion on support of Type A PUSCH repetitions for Msg3	ZTE

R1-2104380	Discussion on Type A PUSCH repetitions for Msg3	vivo

R1-2104439	Discussion on type A PUSCH repetitions for Msg3	Spreadtrum Communications

R1-2104541	Discussion on Type A PUSCH repetitions for Msg3	CATT

R1-2104629	Discussion on type A PUSCH repetitions for Msg3	CMCC

R1-2104689	Type A PUSCH repetition for Msg3	Qualcomm Incorporated

R1-2104796	Type A PUSCH repetitions for Msg3 coverage	OPPO

R1-2104850	Discussion on type A PUSCH repetitions for Msg3	China Telecom

R1-2104863	Type A PUSCH repetitions for Msg3	InterDigital, Inc.

R1-2104923	On Msg3 PUSCH repetition	Intel Corporation

R1-2104979	On Msg3 PUSCH repetition	Intel Corporation

Withdrawn

R1-2105036	On Msg3 PUSCH repetition	Intel Corporation

Withdrawn

R1-2105123	Discussion on Msg3 Coverage Enhancement	Apple

R1-2105150	Discussion on Type A PUSCH repetitions for Msg.3	Panasonic Corporation

R1-2105225	Type A PUSCH repetitions for Msg3	ETRI

R1-2105240	Type A PUSCH repetitions for Msg3	ETRI

Withdrawn

R1-2105329	Type A PUSCH repetitions for Msg3	Samsung

R1-2105361	Type A PUSCH repetitions for Msg3	ETRI

Withdrawn

R1-2105492	Discussion on coverage enhancement for Msg3 PUSCH	LG Electronics

R1-2105579	Discussion on Type A PUSCH repetition for Msg3	Xiaomi

R1-2105644	Type A repetition for msg3 PUSCH	Sharp

R1-2105656	Type A PUSCH Repetition for Msg3	Ericsson

R1-2105715	Type A PUSCH repetitions for Msg3	NTT DOCOMO, INC.

R1-2105777	Type A PUSCH repetitions for Msg3	Lenovo, Motorola Mobility

R1-2105880	Discussion on Type A PUSCH repetitions for Msg3	WILUS Inc.

R1-2105905	Approaches and solutions for Type A PUSCH repetitions for Msg3	Nokia, Nokia Shanghai Bell

### Others

R1-2104335	Performance impacts of residual frequency error for joint channel estimation of PUSCH and PUCCH transmissions	ZTE

R1-2104381	Enhanced Contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition			vivo

R1-2104542	Views on reusing PUSCH enhancements for Msg3	CATT

R1-2104797	Other considerations for coverage enhancement	OPPO

R1-2104864	Deterministic PUSCH repetition for coverage enhancements	InterDigital, Inc.

R1-2105330	Discussion on PRACH enhancements for msg3 beam improvement	Samsung

R1-2105523	Other issues for coverage enhancement	Huawei, HiSilicon

R1-2105657	Other Coverage Enhancements for PUSCH	Ericsson

R1-2105906	Implications of DMRS bundling for PUSCH and PUCCH	Nokia, Nokia Shanghai Bell

## Rel-17 enhancements for NB-IoT and LTE-MTC (3)

Please refer to RP-201306 for detailed scope of the WI

### Support of 16-QAM for unicast in UL and DL for NB-IoT

R1-2104288	Support of 16QAM for unicast in UL and DL in NB-IoT	Huawei, HiSilicon

R1-2104548	Support of 16-QAM for NB-IoT	Nokia, Nokia Shanghai Bell

R1-2104716	Discussion on UL and DL 16QAM for NB-IoT	ZTE

R1-2104819	Support of 16-QAM for NB-IoT	Qualcomm Incorporated

R1-2105374	Support 16QAM in NB-IOT Release 17	MediaTek Inc.

R1-2105622	Support 16QAM for NBIoT	Lenovo, Motorola Mobility

R1-2105889	Support of 16-QAM for unicast in UL and DL in NB-IoT	Ericsson

[105-e-LTE-Rel17_NB_IoT_eMTC-01] Email discussion on support of 16-QAM for unicast in UL and DL for NB-IoT – Yubo (Huawei)

- 1st check point: May 24

- 2nd check point: May 27

### Support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC

R1-2104289	Support of 14-HARQ processes in DL for HD-FDD MTC UEs	Huawei, HiSilicon

R1-2104549	Support of 14-HARQ processes in DL for eMTC	Nokia, Nokia Shanghai Bell

R1-2104717	Remaining issues on 14-HARQ processes in DL for eMTC	ZTE

R1-2104821	Support of 14 HARQ processes and scheduling delay	Qualcomm Incorporated

R1-2105890	Support of 14 HARQ processes in DL in LTE-MTC	Ericsson

[105-e-LTE-Rel17_NB_IoT_eMTC-02] Email discussion on support additional PDSCH scheduling delay for introduction of 14-HARQ processes in DL for eMTC – Gerardo (Ericsson)

- 1st check point: May 24

- 2nd check point: May 27

### Others

Including any remaining issues in supporting a maximum DL TBS of 1736 bits as a Rel-17 optional UE capability

[105-e-LTE-Rel17_NB_IoT_eMTC-03] Email discussion on support of a max TBS of 1736 bits – Martin (Sony)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104718	Remaining issues on DL TBS of 1736 bits for CE mode A	ZTE

R1-2105891	On the L2 Buffer Size for NB-IoT and LTE-M UEs	Ericsson

R1-2105939	Discussion on DL PAPR for 16-QAM of NB-IoT	Huawei, HiSilicon

## Enhancements to Integrated Access and Backhaul (2)

Please refer to RP-210758 for detailed scope of the WI

R1-2104690	Workplan for Rel-17 IAB	Qualcomm Incorporated

### Enhancements to resource multiplexing between child and parent links of an IAB node

[105-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104246	Resource multiplexing between backhaul and access for IAB duplexing enhancements	Huawei, HiSilicon

R1-2104382	Enhancement to resource multiplexing between child and parent links	vivo

R1-2104691	Resource management for enhanced duplexing	Qualcomm Incorporated

R1-2104877	Enhancements to the IAB resource multiplexing	ZTE, Sanechips

R1-2104924	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

R1-2104980	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

Withdrawn

R1-2105037	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

Withdrawn

R1-2105055	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node		CEWiT

R1-2105124	Enhanced resource multiplexing within and across IAB nodes	Apple

R1-2105226	Discussion on IAB resource multiplexing enhancements	ETRI

R1-2105241	Discussion on IAB resource multiplexing enhancements	ETRI

Withdrawn

R1-2105331	Enhancements to Resource Multiplexing for NR IAB	Samsung

R1-2105362	Discussion on IAB resource multiplexing enhancements	ETRI

Withdrawn

R1-2105493	Discussions on IAB resource multiplexing enhancements	LG Electronics

R1-2105617	Enhancements for resource multiplexing among IAB backhaul and access links	Nokia, Nokia Shanghai Bell

R1-2105662	Enhancements for IAB resource multiplexing	AT&T

R1-2105716	Resource multiplexing between child and parent links of an IAB node	NTT DOCOMO, INC.

R1-2105763	Resource multiplexing in enhanced IAB systems	Lenovo, Motorola Mobility

R1-2105852	Resource multiplexing and DC in enhanced IAB	Ericsson

### Other enhancements for simultaneous operation of IAB-node’s child and parent links

Including IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed

//This one is to use NWM – please use RAN1-105-e-NWM-NR-eIAB-02 as the document name

[105-e-NR-eIAB-02] Email discussion on other enhancements for simultaneous operation of IAB-node’s child and parent links – Luca (Qualcomm)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104247	Enhancements for simultaneous operation of MT and DU	Huawei, HiSilicon

R1-2104383	Other enhancements for simultaneous operation of child and parent links	vivo

R1-2104692	Enhancements for simultaneous operation of IAB-node's child and parent links	Qualcomm Incorporated

R1-2104878	Enhancements for simultaneous operation of child and parent links	ZTE, Sanechips

R1-2104925	Other Enhancements for Simultaneous Operations	Intel Corporation

R1-2104981	Other Enhancements for Simultaneous Operations	Intel Corporation

Withdrawn

R1-2105038	Other Enhancements for Simultaneous Operations	Intel Corporation

Withdrawn

R1-2105065	Other enhancements for simultaneous operation of IAB-node’s child and parent links	Fujitsu

R1-2105068	Discussion on simultaneous operation of IAB-node’s child and parent links	CEWiT

R1-2105125	Other enhancements for simultaneous operation of an IAB node's links	Apple

R1-2105227	Discussion on simultaneous operation enhancements for eIAB	ETRI

R1-2105242	Discussion on simultaneous operation enhancements for eIAB	ETRI

Withdrawn

R1-2105332	Enhancements to Timing, Power Control and CLI for NR IAB	Samsung

R1-2105363	Discussion on simultaneous operation enhancements for eIAB	ETRI

Withdrawn

R1-2105494	Discussions on other enhancement for simultaneous operation	LG Electronics

R1-2105618	Other enhancements for simultaneous operation of IAB-node’s child and parent links	Nokia, Nokia Shanghai Bell

R1-2105663	Enhancements for IAB interference management	AT&T

R1-2105717	Other enhancements for simultaneous operation of IAB-node’s child and parent links	NTT DOCOMO, INC.

R1-2105764	Timing, interference, and power control in enhanced IAB systems	Lenovo, Motorola Mobility

R1-2105839	Timing, interference management and power control in enhanced IAB	Ericsson

### Others

R1-2104384	Other enhancements to Rel-17 IAB nodes	vivo

R1-2104879	Consideration on reference carrier for semi-static IAB-DU resource configuration	ZTE, Sanechips

R1-2105525	Other enhancements for IAB	Huawei, HiSilicon

R1-2105840	Discussion on the payload size for DCI format 2_5	Ericsson

## NR Sidelink enhancement (3)

Please refer to RP-202846 for detailed scope of the WI

R1-2105203	Consolidation of agreements on sidelink evaluation methodology update for power saving	LG Electronics

//From AI5

[105-e-NR-R17-Sidelink-LS-01] Email discussion/approval for the reply LS to R1-2100021 from 5/21 to 5/26 - Yuzhou Hu (ZTE)

### Resource allocation enhancement

R1-2105333	On Resource Allocation Enhacements	Samsung

#### Resource allocation for power saving

Including consideration of the impact of sidelink DRX, if any

[105-e-NR-R17-Sidelink-01] Email discussion regarding resource allocation for power saving – Kevin (OPPO)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106031

Agreement:

- For the set of Preserve values in periodic-based partial sensing,

  - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList.

  - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

    - This is per mode 2 Tx resource pool (pre-)configuration

    - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset

      - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx

        - FFS whether the monitoring can be mandatory

R1-2104176	Sidelink resource allocation for power saving	Nokia, Nokia Shanghai Bell

R1-2104192	Power consumption reduction for sidelink resource allocation	FUTUREWEI

R1-2104236	Sidelink resource allocation to reduce power consumption	Huawei, HiSilicon

R1-2104385	Resource allocation for sidelink power saving	vivo

R1-2104440	Discussion on sidelink resource allocation for power saving	Spreadtrum Communications

R1-2104489	Discussion on resource allocation for power saving	CATT, GOHIGH

R1-2104560	NR Sidelink Resource Allocation for UE Power Saving	Fraunhofer HHI, Fraunhofer IIS

R1-2104630	Discussion on resource allocation for power saving	CMCC

R1-2104693	Power Savings for Sidelink	Qualcomm Incorporated

R1-2104706	Discussion on resource allocation for power saving	Zhejiang Lab

R1-2104724	Considerations on partial sensing in NR V2X	CAICT

R1-2104755	Power saving mechanisms in NR sidelink	OPPO

R1-2104869	Sidelink resource allocation for power saving	Lenovo, Motorola Mobility

R1-2104926	Sidelink Power Saving Schemes	Intel Corporation

R1-2104982	Sidelink Power Saving Schemes	Intel Corporation

Withdrawn

R1-2105039	Sidelink Power Saving Schemes	Intel Corporation

Withdrawn

R1-2105066	Considerations on partial sensing and DRX in NR Sidelink	Fujitsu

R1-2105070	Discussion on Sidelink Resource Allocation for Power Saving	Panasonic Corporation

R1-2105126	On Sidelink Resource Allocation for Power Saving	Apple

R1-2105177	Discussion on sidelink resource allocation for power saving	Sony

R1-2105204	Discussion on resource allocation for power saving	LG Electronics

R1-2105228	Discussion on resource allocation for power saving	ETRI

R1-2105243	Discussion on resource allocation for power saving	ETRI

Withdrawn

R1-2105253	Discussion on resource allocation for power saving	NEC

R1-2105334	On Resource Allocation for Power Saving	Samsung

R1-2105364	Discussion on resource allocation for power saving	ETRI

Withdrawn

R1-2105380	Discussion on sidelink power saving	MediaTek Inc.

R1-2105544	Discussion on sidelink resource allocation enhancement for power saving	Xiaomi

R1-2105598	NR SL Resource Allocation for Power Saving	Convida Wireless

R1-2105614	Discussion on resource allocation for power saving	ZTE, Sanechips

R1-2105615	Discussion on resource allocation for power saving	Hyundai Motors

R1-2105645	Discussion on resource allocation for power saving	Sharp

R1-2105651	Resource allocation for power saving with partial sensing in NR sidelink enhancement	ITL

R1-2105674	Sidelink resource allocation for power saving	InterDigital, Inc.

R1-2105718	Discussion on sidelink resource allocation for power saving	NTT DOCOMO, INC.

R1-2105845	Discussion on partial sensing and SL DRX impact	ASUSTeK

R1-2105866	Further discussion on power saving for sidelink	ROBERT BOSCH GmbH

R1-2105893	Resource allocation procedures for power saving	Ericsson

#### Inter-UE coordination for Mode 2 enhancements

[105-e-NR-R17-Sidelink-02] Email discussion regarding inter-UE coordination for Mode 2 enhancements – Seungmin (LGE)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

R1-2106062

R1-2104177	Inter-UE coordination in mode 2 sidelink resource allocation	Nokia, Nokia Shanghai Bell

R1-2104193	Discussion on techniques for inter-UE coordination	FUTUREWEI

R1-2104237	Inter-UE coordination in sidelink resource allocation	Huawei, HiSilicon

R1-2104386	Discussion on mode-2 enhancements	vivo

R1-2104441	Discussion on inter-UE coordination in sidelink resource allocation	Spreadtrum Communications

R1-2104457	Inter-UE Coordination for Mode 2 Enhancements	Kyocera Corporation

R1-2104490	Discussion on inter-UE coordination in mode 2 enhancement	CATT, GOHIGH

R1-2104561	Resource Allocation Enhancements for Mode 2	Fraunhofer HHI, Fraunhofer IIS

R1-2104631	Discussoin on reliability and latency enhancements for mode-2 resource allocation	CMCC

R1-2104694	Reliability and Latency Enhancements for Mode 2	Qualcomm Incorporated

R1-2104707	Inter-UE coordination schemes in mode 2	Zhejiang Lab

R1-2104725	Considerations on mode 2 enhancements	CAICT

R1-2104756	Inter-UE coordination in mode 2 of NR sidelink	OPPO

R1-2104870	Discussion on inter-UE coordination for Mode 2 enhancements	Lenovo, Motorola Mobility

R1-2104927	Inter-UE Coordination Schemes for Sidelink Communication	Intel Corporation

R1-2104983	Inter-UE Coordination Schemes for Sidelink Communication	Intel Corporation

Withdrawn

R1-2105040	Inter-UE Coordination Schemes for Sidelink Communication	Intel Corporation

Withdrawn

R1-2105067	Considerations on inter-UE coordination for mode 2 enhancements	Fujitsu

R1-2105127	On Inter-UE Coordination	Apple

R1-2105178	Discussion on inter-UE coordination for Mode 2 enhancements	Sony

R1-2105200	Discussion on the inter-UE coordination	ZTE

R1-2105205	Discussion on inter-UE coordination for Mode 2 enhancements	LG Electronics

R1-2105229	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

R1-2105244	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

Withdrawn

R1-2105254	Discussion on mode 2 enhancements	NEC

R1-2105270	Inter-UE coordination for enhanced resource allocation	Mitsubishi Electric RCE

R1-2105335	On Inter-UE Coordination for Mode2 Enhancements	Samsung

R1-2105365	Discussion on inter-UE coordination for Mode 2 enhancements	ETRI

Withdrawn

R1-2105393	Discussion on Mode 2 enhancements	MediaTek Inc.

R1-2105545	Discussion on inter-UE coordination	Xiaomi

R1-2105599	NR SL Inter-UE Coordination for Mode 2 Enhancements	Convida Wireless

R1-2105616	Discussion on inter-UE coordination for Mode 2 enhancements	Hyundai Motors

R1-2105646	Discussion on inter-UE coordination for Mode 2 enhancements	Sharp

R1-2105650	Inter-UE coordination for Mode 2 enhancements	Panasonic Corporation

R1-2105659	Inter-UE coordination for mode 2 enhancements	ITL

R1-2105675	On inter-UE coordination for Mode 2 enhancement	InterDigital, Inc.

R1-2105719	Resource allocation for reliability and latency enhancements	NTT DOCOMO, INC.

R1-2105848	Discussion on V2X mode 2 enhancements	ASUSTeK

R1-2105881	Discussion on inter-UE coordination for sidelink mode-2	ROBERT BOSCH GmbH

R1-2105894	Feasibility and benefits of mode 2 enhancements for inter-UE coordination	Ericsson

### Others

Void (not be handled during this e-meeting). No contributions please.

## NR Multicast and Broadcast Services (3)

Please refer to RP-201038 for detailed scope of the WI

### Mechanisms to support group scheduling for RRC_CONNECTED UEs

R1-2104195	Group Scheduling Aspects for Connected UEs	FUTUREWEI

R1-2104248	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

R1-2104336	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

R1-2104387	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

R1-2104442	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2104491	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2104550	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

R1-2104632	Discussion on group scheduling mechanisms	CMCC

R1-2104695	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2104759	Group scheduling for NR Multicast and Broadcast Services	OPPO

R1-2104865	On group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

R1-2104928	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

R1-2104984	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105041	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105069	Discussion on Group Scheduling and Simultaneous MBS and Unicast Reception 	TCL Communication Ltd.

R1-2105128	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

R1-2105179	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

R1-2105336	Support of group scheduling for RRC_CONNECTED Ues	Samsung

R1-2105381	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

R1-2105437	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

R1-2105600	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

R1-2105647	Discussion on common frequency resource for multicast of RRC_CONNECTED UEs	ETRI

R1-2105670	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Google Inc.

R1-2105720	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2105838	Discussion on group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2105844	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

R1-2105914	Mechanisms to support MBS group scheduling for RRC_CONNECTED Ues	Ericsson

[105-e-NR-MBS-01] Email discussion/approval on mechanisms to support group scheduling for RRC_CONNECTED UEs with checkpoints for agreements on May 24, May 27 – Fei (CMCC)

### Mechanisms to improve reliability for RRC_CONNECTED UEs

R1-2104196	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

R1-2104249	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2104337	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2104388	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2104443	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

R1-2104492	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

R1-2104551	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2104633	Discussion on reliability improvement	CMCC

R1-2104696	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2104760	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

R1-2104866	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2104929	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

R1-2104985	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105042	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

Withdrawn

R1-2105129	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

R1-2105337	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2105382	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

R1-2105438	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2105601	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

R1-2105648	Discussion on HARQ-ACK feedback for multicast of RRC_CONNECTED UEs	ETRI

R1-2105721	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

R1-2105843	Discussion on reliability for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

R1-2105915	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

[105-e-NR-MBS-02] Email discussion/approval on mechanisms to improve reliability for RRC_CONNECTED UEs with checkpoints for agreements on May 25, May 27 – Jinhuan (Huawei)

### Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

R1-2104197	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

R1-2104250	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

R1-2104338	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs		ZTE

R1-2104389	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			vivo

R1-2104444	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

R1-2104493	Discussion on basic functions for MBS for RRC_IDLEINACTIVE UEs	CATT, CBN

R1-2104552	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

R1-2104634	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

R1-2104697	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

R1-2104761	Discussion on support for IDLE and INACTIVE state Ues	OPPO

R1-2104867	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

R1-2104930	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

R1-2104986	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

Withdrawn

R1-2105043	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

Withdrawn

R1-2105130	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Apple

R1-2105180	Considerations on MBS functions for RRC_IDLE/RRC_INACTIVE UEs	Sony

R1-2105338	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

R1-2105383	Discussion on broadcast or multicast for RRC_IDLE or INACTIVE UEs	MediaTek Inc.

R1-2105439	Basic function for broadcast/multicast	LG Electronics

R1-2105602	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

R1-2105673	Discussion on MBS for RRC_IDLE/INACTIVE UE	Google Inc.

R1-2105722	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			NTT DOCOMO, INC.

R1-2105849	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

R1-2105916	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

[105-e-NR-MBS-03] Email discussion/approval on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs with checkpoints for agreements on May 24, May 27 – David (BBC)

### Others

R1-2104339	Consideration on performance enhancement for RRC_IDLE/INACTIVE UEs	ZTE

R1-2104390	Other issues for Rel-17 MBS	vivo

R1-2104494	Discussion on other issues in Rel-17 MBS	CATT

R1-2104762	PUCCH resource allocation for  NACK-only based HARQ-ACK feedback in MBMS	OPPO

R1-2105440	Other aspects for MBS	LG Electronics

R1-2105526	Impact from MCCH and MTCH on broadcast reception	Huawei, HiSilicon

R1-2105853	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

Withdrawn

R1-2105854	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

Withdrawn

R1-2105855	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

R1-2105917	Assumptions for performance evaluations of NR-MBS	Ericsson

## NR Dynamic spectrum sharing (DSS)  (3)

Please refer to RP-193260 for detailed scope of the WI

Also include RAN1 impact from RP-201040 (LTE_NR_DC_enh2)

### Cross-carrier scheduling (from Scell to Pcell)

[105-e-NR-DSS-01] Email discussion/approval for CCS – Ravi (Ericsson)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104185	On cross-carrier scheduling from SCell to Pcell	Nokia, Nokia Shanghai Bell

R1-2104232	Discussion on SCell PDCCH scheduling P(S)Cell PDSCH or PUSCH	Huawei, HiSilicon

R1-2104340	Discussion on Cross-Carrier Scheduling from SCell to PCell	ZTE

R1-2104391	Discussion on Scell scheduling Pcell	vivo

R1-2104445	Discussion on cross-carrier scheduling from SCell to Pcell	Spreadtrum Communications

R1-2104495	Discussion on cross-carrier scheduling from Scell to Pcell	CATT

R1-2104635	Discussion on cross-carrier scheduling from SCell to Pcell	CMCC

R1-2105970	Cross-carrier scheduling from an SCell to the PCell/PSCell	Qualcomm Incorporated

Revision of R1-2104698

R1-2104806	Discussion on cross-carrier scheduling from Scell to Pcell	OPPO

R1-2104931	On SCell scheduling PCell transmissions	Intel Corporation

R1-2104987	On SCell scheduling PCell transmissions	Intel Corporation

Withdrawn

R1-2105044	On SCell scheduling PCell transmissions	Intel Corporation

Withdrawn

R1-2105131	Views on Rel-17 DSS SCell scheduling PCell	Apple

R1-2105230	Cross-carrier scheduling from SCell to Pcell	ETRI

R1-2105245	Cross-carrier scheduling from SCell to Pcell	ETRI

Withdrawn

R1-2105339	Cross-carrier scheduling from SCell to PCell	Samsung

R1-2105366	Cross-carrier scheduling from SCell to Pcell	ETRI

Withdrawn

R1-2105378	On Cross-Carrier Scheduling from SCell to PCell/PSCell	MediaTek Inc.

R1-2105401	Search space monitoring in sSCell and PCell	InterDigital, Inc.

R1-2105441	Discussion on cross-carrier scheduling from SCell to Pcell	LG Electronics

R1-2105546	Discussion on Cross-carrier scheduling from SCell to PCell	Xiaomi

R1-2105723	Discussion on cross-carrier scheduling enhancements for NR DSS	NTT DOCOMO, INC.

R1-2105765	Cross-carrier scheduling (from Scell to Pcell)	Lenovo, Motorola Mobility

R1-2105796	Enhanced cross-carrier scheduling for DSS	Ericsson

R1-2105847	Discussion on cross-carrier scheduling from sSCell to PCell/PSCell	ASUSTeK

### Multi-cell PDSCH scheduling via a single DCI

Focusing on study whether or not to support the feature first

[105-e-NR-DSS-02] Email discussion/approval for multi-cell PDSCH scheduling via a single DCI – Haipeng (Lenovo)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104186	Way Forward On single DCI scheduling two cells	Nokia, Nokia Shanghai Bell

R1-2104233	Discussion on multi-carrier scheduling using single PDCCH	Huawei, HiSilicon

R1-2104341	Discussion on Multi-cell PDSCH Scheduling via a Single DCI	ZTE

R1-2104392	Discussion on joint scheduling	vivo

R1-2104446	Discussion on multi-cell PDSCH scheduling via a single DCI	Spreadtrum Communications

R1-2104496	Discussion on multi-cell PDSCH scheduling via a single DCI	CATT

R1-2104807	Discussion on multi-cell PDSCH scheduling via a single DCI	OPPO

R1-2104868	On multi-cell PDSCH scheduling via a single DCI	Lenovo, Motorola Mobility

R1-2104932	On 2-cell scheduling via single DCI	Intel Corporation

R1-2104988	On 2-cell scheduling via single DCI	Intel Corporation

Withdrawn

R1-2105045	On 2-cell scheduling via single DCI	Intel Corporation

Withdrawn

R1-2105132	Views on Rel-17 DSS Multi-cell PDSCH scheduling via a single DCI	Apple

R1-2105340	On a single DCI format scheduling on multiple cells	Samsung

R1-2105402	On the support of single DCI scheduling two cells	InterDigital, Inc.

R1-2105412	Multi-cell PDSCH scheduling via a single DCI	NEC

R1-2105442	Discussion on multi-cell PDSCH scheduling via a single DCI	LG Electronics

R1-2105724	Discussion on multi-cell PDSCH scheduling via a single DCI for NR DSS	NTT DOCOMO, INC.

R1-2105797	Study on single DCI scheduling PDSCH on multiple cells	Ericsson

### Support efficient activation/de-activation mechanism for SCells in NR CA

[105-e-NR-DSS-03] Email discussion/approval for efficient activation/de-activation mechanism – Frank (Huawei)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104187	On low latency Scell activation	Nokia, Nokia Shanghai Bell

R1-2104206	Support efficient activation/de-activation mechanism for Scells	FUTUREWEI

R1-2104234	Discussion on efficient activation/de-activation mechanism for SCells	Huawei, HiSilicon

R1-2104342	Discussion on Support Efficient Activation De-activation Mechanism for SCells in NR CA			ZTE

R1-2104393	Discussion on efficient activation/de-activation mechanism for Scells	vivo

R1-2104447	Discussion on efficient activation/de-activation mechanism for SCells in NR CA	Spreadtrum Communications

R1-2104497	Discussion on efficient activation and de-activation mechanism for SCell in NR CA	CATT

R1-2104699	Efficient activation/de-activation mechanism for SCells in NR CA	Qualcomm Incorporated

R1-2104808	Discussion on efficient activation/de-activation for Scell	OPPO

R1-2104933	On efficient activation/de-activation for SCells	Intel Corporation

R1-2104989	On efficient activation/de-activation for SCells	Intel Corporation

Withdrawn

R1-2105046	On efficient activation/de-activation for SCells	Intel Corporation

Withdrawn

R1-2105133	On efficient SCell Activation/Deactivation	Apple

R1-2105341	Remaining Issues on Scell Activation/Deactivation	Samsung

R1-2105379	Discussion on temporary RS	MediaTek Inc.

R1-2105403	Efficient SCell Activation	InterDigital, Inc.

R1-2105413	Discussion on efficient activation mechanism for SCells	NEC

R1-2105725	Discussion on efficient activation deactivation mechanism for Scells	NTT DOCOMO, INC.

R1-2105798	Reduced Latency SCell Activation	Ericsson

R1-2105846	Efficient activation/deactivation of SCell	ASUSTeK

### Others

R1-2104394	Discussion on other aspects on DSS	vivo

R1-2104498	Simulation results for multi-cell PDSCH scheduling via a single DCI	CATT

R1-2104577	Further analysis and simulation for Multi-cell scheduling for both downlink and uplink	ZTE

R1-2105519	Remaining issues on the efficient activation/de-activation mechanism for SCells	Huawei, HiSilicon

R1-2105799	CRS Rate-matching enhancement for DSS	Ericsson

## Study on XR Evaluations for NR (RAN1) (3)

Please refer to RP-201145 for detailed scope of the WI

R1-2104700	Updated Work Plan for Rel-17 SI on XR Evaluations for NR	Qualcomm Incorporated

### Traffic Model

[105-e-NR-XR-01] Email discussion/approval on traffic model – Eddy (Qualcomm)

- Including discussion and possible reply LS for R1-2104023 and R1-2102308

- 1st check point: May 24

- 2nd check point: May 27

R1-2104207	XR traffic model	FUTUREWEI

R1-2104238	Traffic model for XR and Cloud Gaming	Huawei, HiSilicon

R1-2104395	Remaining issues on traffic models of XR	vivo

R1-2104502	XR traffic model	CATT

R1-2104555	On Traffic Model for XR study	Nokia, Nokia Shanghai Bell

R1-2104701	Remaining Issues on XR Traffic Models	Qualcomm Incorporated

R1-2104745	Discussion on the XR traffic models for evaluation	OPPO

R1-2104934	Traffic Model for XR	Intel Corporation

R1-2104990	Traffic Model for XR	Intel Corporation

Withdrawn

R1-2105047	Traffic Model for XR	Intel Corporation

Withdrawn

R1-2105134	Considerartions on XR traffic model	Apple

R1-2105181	Considerations on XR traffic model	Sony

R1-2105342	Traffic Models for XR	Samsung

R1-2105376	Traffic Model for XR and CG	MediaTek Inc.

R1-2105443	Discussion on traffic models for XR evaluation	LG Electronics

R1-2105499	Discussion on UL traffic models	InterDigital, Inc.

R1-2105547	Discussion on remaining issues of traffic Model for XR services	Xiaomi

R1-2105603	Remaining Issues of XR Traffic Model	ZTE, Sanechips

R1-2105726	Discussion on traffic model for XR	NTT DOCOMO, INC.

R1-2105829	Traffic model for XR	Ericsson

### Evaluation Methodology

Including identification of KPIs of interest for relevant deployment scenarios

/This one is to use NWM – please use RAN1-105-e-NWM-NR-XR-02 as the document name

[105-e-NR-XR-02] Email discussion/approval on evaluation methodology – Xiaohang (vivo)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104208	XR evaluation methodology	FUTUREWEI

R1-2104239	Evaluation methodology for XR and Cloud Gaming	Huawei, HiSilicon

R1-2104396	Discussion on evaluation methodologies for XR	vivo

R1-2104499	Evaluation methodology and performance index for XR	CATT

R1-2104556	Development of the Evaluation Methodology for XR Study	Nokia, Nokia Shanghai Bell

R1-2104702	Remaining Issues on Evaluation Methodology for XR	Qualcomm Incorporated

R1-2104746	Discussion on the XR evaluation methodology	OPPO

R1-2104935	Evaluation Methodology for XR	Intel Corporation

R1-2104991	Evaluation Methodology for XR	Intel Corporation

Withdrawn

R1-2105048	Evaluation Methodology for XR	Intel Corporation

Withdrawn

R1-2105135	Remaining issues in XR evaluation methodology	Apple

R1-2105343	Evaluation methodology and KPIs for XR	Samsung

R1-2105377	On Evaluation Methodology for XR and CG	MediaTek Inc.

R1-2105444	Discussion on evaluation methodologies for XR	LG Electronics

R1-2105500	Discussion on additional issues on XR Evaluations Methodology and KPI	InterDigital, Inc.

R1-2105548	Discussion on remaining issues of evaluation methodology for XR services	Xiaomi

R1-2105604	Further Discussion on XR Evaluation Methodology	ZTE, Sanechips

R1-2105727	Discussion on evaluation methodology for XR	NTT DOCOMO, INC.

R1-2105830	Evaluation methodology for XR	Ericsson

### Initial Performance Evaluation Results

[105-e-NR-XR-03] Email discussion/approval on initial performance evaluation results – Eddy (Qualcomm)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104209	XR initial evaluations	FUTUREWEI

R1-2104397	Initial performance evaluation results on XR	vivo

R1-2104500	Evaluation results of XR performance	CATT

R1-2104557	Performance results in indoor hotspot and dense urban deployments of CG and VR applications			Nokia, Nokia Shanghai Bell

R1-2104703	Initial Evaluation Results for XR Capacity and UE Power Consumption	Qualcomm Incorporated

R1-2104747	Evaluation results for XR evaluation	OPPO

R1-2104936	Initial results for XR	Intel Corporation

R1-2104992	Initial results for XR	Intel Corporation

Withdrawn

R1-2105049	Initial results for XR	Intel Corporation

Withdrawn

R1-2105136	Performance evaluation on XR	Apple

R1-2105392	Initial Performance and Evaluation Results for XR and CG	MediaTek Inc.

R1-2105501	Initial Performance Evaluation Results on XR	InterDigital, Inc.

R1-2105521	Initial evaluation results for XR and Cloud Gaming	Huawei, HiSilicon

R1-2105605	Performance Evaluation Results for XR	ZTE, Sanechips

R1-2105664	XR Initial Performance Results	AT&T

R1-2105831	Initial XR performance evaluation results	Ericsson

### Others

R1-2104398	Challenges and potential enhancements of XR	vivo

R1-2104501	Potential area of NR enhancement for the support of XR services	CATT

R1-2104558	Enhancements for better XR support over NR	Nokia, Nokia Shanghai Bell

R1-2104704	Potential Enhancements for XR	Qualcomm Incorporated

R1-2104748	Discussion on the  support of XR/CG service in sidelink-unlicensed	OPPO

R1-2105137	Considerations on potential enhancements for XR	Apple

R1-2105344	Potential enhancements for better XR support in NR	Samsung

R1-2105502	Discussion on potential enhancements for XR	InterDigital, Inc.

R1-2105522	Challenges and potential enhancements for XR and Cloud Gaming	Huawei, HiSilicon

R1-2105549	Discussion on potential enhancement for supporting XR	Xiaomi

R1-2105606	Further Discussion on Capacity and Power Working Areas for XR	ZTE, Sanechips

R1-2105832	Discussion on enhancements for XR	Ericsson

## Study on NB-IoT/eMTC support for Non-Terrestrial Network (4)

Please refer to RP-202689 for detailed scope of the WI

R1-2104573	Link budget result calibration Spreadsheet for IoT NTN	MediaTek Inc.

R1-2105815	Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)	Rapporteur (MediaTek)

### Scenarios applicable to NB-IoT/eMTC

R1-2104571	Summary #1 of AI 8.15.1  Scenarios applicable to NB-IoT/eMTC NTN	MediaTek Inc.

R1-2104258	Application scenarios of IoT in NTN	Huawei, HiSilicon

R1-2104403	Discussion on eMTC enabling High Value NTN IoT use-cases	Omnispace

R1-2104503	Applicable scenarios to NB-IoT/eMTC	CATT

R1-2104567	Scenarios applicable to IoT NTN	MediaTek Inc.

R1-2104636	Discussion on scenarios applicable to NB-IoT/eMTC	CMCC

R1-2104777	Discussion on scenarios applicable to NB-IoT/eMTC	OPPO

R1-2104814	On scenarios and evaluations for NB-IoT and eMTC based NTN	Ericsson

R1-2104822	Scenarios applicable to NB-IoT/eMTC 	Qualcomm Incorporated

R1-2105138	On Link Budget Analysis of IoT NTN	Apple

R1-2105182	IoT-NTN Link Budgets	Sony

R1-2105193	Discussion on the remaining issues of scenarios and assumption for IoT-NTN	ZTE

R1-2105345	Initial link budget evaluation for NB-IoT/eMTC	Samsung

R1-2105404	Link budget evaluations and deployment for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2105550	Discussion on the link budget of NBIoT and eMTC over NTN	Xiaomi

Withdrawn

R1-2105945	Discussion on eMTC enabling High Value NTN IoT use-cases	Omnispace

Withdrawn

R1-2105946	Discussion on eMTC enabling High Value NTN IoT use-cases	Omnispace

[105-e-NR-NB_IoT_eMTC-01] Email discussion/approval on scenarios applicable to NB-IoT/eMTC with checkpoints for agreements on May 24, May 27 – Gilles (MediaTek)

### Enhancements to time and frequency synchronization

Including random access procedure/signals

R1-2104572	"Summary #1 of AI 8.15.2  Enhancements to time and frequency

synchronization for IoT NTN"	MediaTek Inc.

R1-2104259	Discussion on time and frequency synchronization enhancement for IoT in NTN	Huawei, HiSilicon

R1-2104399	Discussion on enhancements to time and frequency synchronization on NB-IoT_eMTC for NTN			vivo

R1-2104448	Consideration on enhancements to time and frequency synchronization for IoT NTN	Spreadtrum Communications

R1-2104504	Time and frequency synchronization for NB-IoT/eMTC	CATT

R1-2104568	Enhancements to time and frequency synchronization for IoT NTN	MediaTek Inc.

R1-2104637	Enhancements to time and frequency synchronization for IoT NTN	CMCC

R1-2104778	Discussion on enhancements to time and frequency synchronization	OPPO

R1-2104815	On time and frequency synchronization enhancements for IoT NTN	Ericsson

R1-2104823	Enhancements to time and frequency synchronization	Qualcomm Incorporated

R1-2104937	On synchronization for NB-IoT and eMTC NTN	Intel Corporation

R1-2104993	On synchronization for NB-IoT and eMTC NTN	Intel Corporation

Withdrawn

R1-2105050	On synchronization for NB-IoT and eMTC NTN	Intel Corporation

Withdrawn

R1-2105139	Time and Frequency Synchronization in IoT NTN	Apple

R1-2105183	Enhancements to time and frequency synchronisation for IoT-NTN	Sony

R1-2105194	Discussion on the synchronization for IoT-NTN	ZTE

R1-2105346	On enhancements to time and frequency synchronization	Samsung

R1-2105405	Enhancement to time and frequency synchronization for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2105551	Discussion on time and frequency synchronization for IoT NTN	Xiaomi

R1-2105624	Time and frequency synchronization for IoT NTN	Lenovo, Motorola Mobility

R1-2105676	Time/Frequency Synchronization for IoT NTN	InterDigital, Inc.

R1-2105825	Time and Frequency Synchronization to NB-IoT in NTN	Asia Pacific Telecom, FGI

[105-e-NR-NB_IoT_eMTC-02] Email discussion/approval on enhancements to time and frequency synchronization with checkpoints for agreements on May 25, May 27 – Gilles (MediaTek)

### Timing relationship enhancements

R1-2104260	Discussion on timing relationship enhancement for IoT in NTN	Huawei, HiSilicon

R1-2104449	Consideration on timing relationship enhancements for IoT NTN	Spreadtrum Communications

R1-2104505	Timing relationship enhancement for NB-IoT/eMTC	CATT

R1-2104569	Timing relationship enhancements for IoT NTN	MediaTek Inc.

R1-2104638	Timing relationship enhancements for IoT NTN	CMCC

R1-2104779	Discussion on timing relationship enhancements	OPPO

R1-2104816	On timing relationship enhancements for IoT NTN	Ericsson

R1-2104824	Timing relationship enhancements	Qualcomm Incorporated

R1-2104938	On timing relationship for NB-IoT and eMTC NTN	Intel Corporation

R1-2104994	On timing relationship for NB-IoT and eMTC NTN	Intel Corporation

Withdrawn

R1-2105051	On timing relationship for NB-IoT and eMTC NTN	Intel Corporation

Withdrawn

R1-2105140	Timing Relationship Enhancement in IoT NTN	Apple

R1-2105184	Timing relationship enhancements for IoT-NTN	Sony

R1-2105195	Discussion on timing relationship for IoT-NTN	ZTE

R1-2105347	Timing relationship enhancements	Samsung

R1-2105406	Timing relationship enhancements for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2105503	RAR Window Offset	Fraunhofer IIS / Fraunhofer HHI

R1-2105552	Discussion on the timing relationship enhancement for IoT NTN	Xiaomi

R1-2105677	Timing relationship enhancement for IoT NTN	InterDigital, Inc.

R1-2105826	Timing relationship enhancements to NB-IoT in NTN	Asia Pacific Telecom, FGI

[105-e-NR-NB_IoT_eMTC-03] Email discussion/approval on timing relationship enhancements with checkpoints for agreements on May 24, May 27 – Sam (Sony)

### Enhancements on HARQ

R1-2104261	Discussion on HARQ enhancement for IoT in NTN	Huawei, HiSilicon

R1-2104400	Discussion on HARQ enhancements on NB-IoT_eMTC for NTN	vivo

R1-2104450	Consideration on enhancements on HARQ for IoT NTN	Spreadtrum Communications

R1-2104506	HARQ operation enhancement for NB-IoT/eMTC	CATT

R1-2104570	Enhancements on HARQ for IoT NTN	MediaTek Inc.

R1-2104639	Enhancements on HARQ for IoT NTN	CMCC

R1-2104780	Discussion on HARQ enhancements	OPPO

R1-2104817	On HARQ enhancements for IoT NTN	Ericsson

R1-2104825	Enhancements on HARQ	Qualcomm Incorporated

R1-2105141	HARQ Enhancement in IoT NTN	Apple

R1-2105185	HARQ enhancements for IoT-NTN	Sony

R1-2105196	Discussion on HARQ for IoT-NTN	ZTE

R1-2105348	On enhancements on HARQ	Samsung

R1-2105407	HARQ for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2105553	Discussion on the HARQ enhancement for IoT NTN	Xiaomi

R1-2105621	HARQ enhancement for IoT NTN	Lenovo, Motorola Mobility

R1-2105678	HARQ enhancement for IoT NTN	InterDigital, Inc.

R1-2105827	Enhancements on HARQ to NB-IoT in NTN	Asia Pacific Telecom, FGI

[105-e-NR-NB_IoT_eMTC-04] Email discussion/approval on enhancements on HARQ with checkpoints for agreements on May 25, May 27 – Carmela (Samsung)

### Others

R1-2104451	Consideration on other design aspects for IoT NTN	Spreadtrum Communications

R1-2104781	Discussion on other aspects	OPPO

R1-2104820	On evaluation assumptions for NB-IoT and eMTC based NTN	Ericsson

R1-2104826	Other aspects for NTN IOT	Qualcomm Incorporated

R1-2105197	Discussion on additional enhancement for IoT-NTN	ZTE

R1-2105408	Applicability of NB-IoT/eMTC features for NTN	Nokia, Nokia Shanghai Bell

R1-2105530	Other aspects to support IoT in NTN	Huawei, HiSilicon

R1-2105554	Discussion on the other design aspects for IoT NTN	Xiaomi

## Others (1)

Including potential Rel-17 TEIs. No GTW time is planned for any Rel-17 TEIs

[105-e-NR-R17-TEI-01] Email discussion/approval Rel-17 TEIs  – Hiroki (NTT DOCOMO)

- 1st check point: May 24

- 2nd check point: May 27

R1-2104401	2Tx codebook configuration for UE supporting 4Tx in UL	vivo, ZTE, CMCC

R1-2104473	Discussion on SSB resources for RLM	CATT

R1-2104596	TEI-17 proposal on NR codeword mapping	ZTE

R1-2104705	Rel-17 TEI	Qualcomm Incorporated

R1-2104939	On Rel-17 TEI enhancement	Intel Corporation

R1-2104995	On Rel-17 TEI enhancement	Intel Corporation

Withdrawn

R1-2105052	On Rel-17 TEI enhancement	Intel Corporation

Withdrawn

R1-2105536	Enhancements on the scheduling of PUSCH over multiple slots	Huawei, HiSilicon, China Unicom

R1-2105739	Views on TEI-17 proposal targeting the false PMI reporting issue	MediaTek Inc.

R1-2105778	Rel-17 TEI proposal for mitigating half-duplex issue in NR V2X groupcast NACK-only case regime		Intel Corporation, Qualcomm Inc.

R1-2105804	NR positioning support for TA-based positioning in E-CID (TEI)	NTT DOCOMO INC., Ericsson, Polaris Wireless, FirstNet, Deutsche Telekom, Intel Corporation, CATT

R1-2105813	TEI-17 proposal targeting the false PMI reporting issue	Ericsson, NTT DOCOMO, Softbank, Verizon, T-Mobile USA

# Elections

Elections for the position of RAN1 Chair, with details available at:

https://www.3gpp.org/news-events/elections/2176

Depending on the election result of RAN1 Chair, there may also be an election for a Vice Chair position, with details available at:

https://www.3gpp.org/news-events/elections/2176

More details about the election, whenever available, are to be announced.

R1-2104563	Guidance for elections during TSG RAN WG1#105-e	RAN1 Chair, ETSI MCC

RAN1 chair election:

- Candidates (in alphabetical order of last name)

  - Dr. Younsun KIM, Samsung Electronics Co., Ltd (TTA)

  - Dr. Havish KOORAPATY, Ericsson LM (ETSI)

- 1st ballot (May 10th – 11th)

  - Number of Individual Members with a right to vote: 272

  - Votes cast: 262 (3 abstentions)

  - Results:

    - Dr. Younsun KIM, Samsung Electronics Co., Ltd (TTA), 158 votes, 61%

    - Dr. Havish KOORAPATY, Ericsson LM (ETSI), 101 votes, 39%

Dr. Havish KOORAPATY kindly withdrew after the 1st ballot. Thus, Dr. Younsun KIM was elected as new RAN1 chair.

One RAN1 Vice Chair election:

- Candidates (in alphabetical order of last name)

  - Dr. Hiroki HARADA, NTT DOCOMO INC. (ARIB)

  - Dr. Xiaodong XU, China Mobile Com. Corporation (CCSA)

- 1st ballot (May 13th – 14th)

  - Number of Individual Members with a right to vote: 272

  - Votes cast: 236 (1 abstention)

  - Results:

    - Dr. Xiaodong XU, China Mobile Com. Corporation (CCSA), 150 votes, 64%

    - Dr. Hiroki HARADA, NTT DOCOMO INC. (ARIB), 85 votes, 36%

Dr. Hiroki HARADA kindly withdrew after the 1st ballot. Thus, Dr. Xiaodong XU was elected as one new RAN1 Vice Chair.

# Closing of the meeting (Day 7: UTC 17:00 at the latest)