3GPP TSG RAN WG1 Meeting #106-e 	R1-2106472

E-meeting, August 16th - 27th, 2021

Agenda Item:	8.13.1

Source:	Huawei, HiSilicon

Title:	Discussion on SCell PDCCH scheduling P(S)Cell PDSCH or PUSCH

Document for:	Discussion and Decision

Introduction

When a PCell can be cross-carrier scheduled by an SCell, there may be the possibility that two cells are configured to schedule one cell. The specification impact on UE procedure for receiving control information needs to be discussed. At the RAN1 #105-e meeting, two types of UEs (Type A and Type B) are agreed to be supported. In this contribution, we discuss the BD/CCE limit handling and overbooking of the two types of UEs firstly, then share our views on the other remaining issues.

Discussion

Type-0/0A/1/2-CSS sets configuration for Type A UE

At the RAN1 #105-e meeting, two types of UEs (Type A and Type B) are agreed to support CCS from sSCell to PCell

- Type A UE: at least USS sets/Type3-CSS sets on PCell and USS sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of PCell and sSCell

- Type B UE: monitors PDCCHs in overlapping [slot/symbol] of P(S)Cell and sSCell

A remaining issue for Type A UE is how to configure Type-0/0A/1/2-CSS sets on PCell.

For one thing, in current specification, except for overbooking handling, the BD/CCE limit handling has nothing to do with the type of SS sets (CSS sets or USS sets). The UE BD/CCE capability has nothing to do with the type of SS sets either. For another thing, if UE monitors Type-0/0A/1/2-CSS sets on PCell and USS sets on sSCell in overlapping [slot/symbol] of PCell and sSCell for Type A UE, the impacts on BD/CCE limit handling and UE implementation are similar to the impacts introduced by Type B UE. Therefore, considering the specification impact and UE implementation complexity, we propose to configure Type-0/0A/1/2-CSS sets on PCell and USS sets on sSCell so that the UE does not monitor them in overlapping [slot/symbol] of PCell and sSCell for Type A UE.

In the following discussion, we consider Type A UE with this restriction.

Proposal 1: For Type A UE, Type-0/0A/1/2-CSS sets on PCell and USS sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of PCell and sSCell.

BD/CCE limit handling and overbooking

In order to further discuss BD/CCE limit handling and overbooking for the two types of UEs, we first review the BD/CCE limit handling in current specification. For simplicity, we take the BD limit handling based on slot limits as an example. Non-overlapped CCEs limit handling is similar.

When a UE reports BD/CCE capability corresponding to Ncellscap downlink cells, and is configured with Ncells,0DL,μ scheduling cells having SCS configuration μ and only one CORESET pool, and Ncells,1DL,μ scheduling cells having SCS configuration μ and two CORESET pools.

More specifically,

- If μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor PDCCH candidates more than the below BD limits per slot for each scheduled cell, on the active DL BWP of the scheduling cell

-	MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ when the scheduling cell is from the Ncells,0DL,μ cells, or

-	MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ when the scheduling cell is from the Ncells,1DL,μ cells

-	MPDCCHmax,slot,μ  for each CORESET pool when the scheduling cell is from the Ncells,1DL,μ cells

- If μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap

- the UE is not required to monitor PDCCH candidates more than the below BD limit per slot, on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ cells

-	MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j

- and for each scheduled cell

-	minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ when the scheduling cell is from the Ncells,0DL,μ cells, or

-	minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ when the scheduling cell is from the Ncells,1DL,μ cells

-	minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ for each CORESET pool when the scheduling cell is from the Ncells,1DL,μ cells

- Ncells,1DL,μ is the number of cells with SCS configuration of μ and configured with two CORESET pools

Where γ is indicated by BDFactorR related to the UE reported blind detection factor R value. The blind detection factor R is reported per UE support of multi-DCI multi-TRP operation where the PDCCH can be received from multiple points within a carrier. Also, the limit is related to SCS for the corresponding scheduling cell(s)/point(s). Thus, we can see

Observation 1: The BD/CCE capability is related to the number of sources that PDCCH can be transmitted from as scaled from one cell and the SCS of the scheduling PDCCH(s).

Type A UE

BD/CCE limit handling

For Type A UE, since the UE does not monitor PDCCH candidates in overlapping [slot/symbol] of PCell and sSCell, the BD/CCE limit for a slot can be determined according to the PDCCH candidates configured on this slot. In a slot, if the PDCCH candidates are only configured on PCell, the BD/CCE limit on this slot is determined based on the PCell configurations, e.g., SCS configuration μ and BDFactorR. Similarly, in a slot, if the PDCCH candidates are configured only on sSCell, the BD/CCE limit on this slot is determined based on the sSCell configurations.

Overbooking

At the RAN1 #104b-e meeting, it is agreed that overbooking is not allowed on USS set(s) of SCell. For Type A UE, since the UE does not monitor PDCCH candidates in overlapping [slot/symbol] of PCell and sSCell, it is clear that overbooking can only be possible in the slot where the PDCCH candidates are configured on PCell.

Proposal 2: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of an SCell and the PCell, for determining the UE BD/CCE capability for Type A UE:

-	in a slot, if the PDCCH candidates are configured on PCell, the BD/CCE limit of this slot is determined based on the PCell configurations

-	in a slot, if the PDCCH candidates are configured on sSCell, the BD/CCE limit of this slot is determined based on the sSCell configurations

-	PDCCH overbooking is maintained only for USS set(s) in the slot where the PDCCH candidates are configured on PCell.

Type B UE

For determining the BD/CCE capability/distribution for Type B UE, the effect on the BD/CCE capability/distribution is similar to the case of serving cells with multi-TRP operation, which was specified with the use of blind detection factor. The difference is that the SCSs of two TRPs of a serving cell are always the same, while the SCSs of PCell and SCell can be same or different.

At the RAN1 #104b-e meeting, three BD/CCE limit handling options (Option A, Option B and Option C) were proposed. We discuss which option is suitable for Type B UE below

Case 1: same SCS between PCell and SCell

BD/CCE limit handling

When the SCSs of PCell and SCell are the same, the mechanism to determine the number of downlink cells corresponding to BD/CCE capability of a UE or distribute of BD/CCE capability in the downlink cells of the UE in multiple TRPs can be referred to. A single BDFactorR value can be configured which is used for scaling the BD/CCE limit per number of serving cells. Regarding the UE capability and gNB scheduling, the value of BDFactorR may be 1 or 2. Also in this case, assume Ncells,0DL,μ scheduling cells having SCS configuration μ without sSCell configuration, and scheduling sSCell having the same SCS configuration μ as well as PCell. For simplicity, multi-TRP operation is not assumed but can be simply added later since the general framework is the same.

Then, according to the agreed restriction that there would be only one sSCell used for scheduling the PCell, and the observation 1 above, the number of scheduling sources in this case will be scaled from 1 cell which is the PCell only. Thus, Ncells,1DL,μ=1 with the same framework as the current specification for determining the BD limit for cells with SCS configuration of μ (otherwise Ncells,1DL,μ=0 ).

More specifically, we could summarize

- If μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor PDCCH candidates more than the below BD limits per slot for each scheduled cell, on the active DL BWPs of the scheduling cells

-	MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ when the scheduling cell is from the Ncells,0DL,μ cells, or

-	MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ for the two cells scheduling PCell

-	MPDCCHmax,slot,μ for each of the two cells scheduling PCell

- If μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap

- the UE is not required to monitor PDCCH candidates more than the below BD limits per slot, on the active DL BWPs of scheduling cell(s) from the Ncells,0DL,μ cells and the scheduling cells for scheduling PCell

-	MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j

- and  for each scheduled cell

-	minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ when the scheduling cell is from the Ncells,0DL,μ cells, or

-	minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ for the two cells scheduling PCell

-	minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ for each of the two cells scheduling PCell

- Ncells,1DL,μ=1 for the cells scheduling PCell with SCS configuration of μ, otherwise  Ncells,1DL,μ=0 (in line with current specifications)

Note, the above BD/CCE limit handling is similar/same as Option B, where x1(m)+x2(m)+y(m) is less than or equal to an upper bound based on only the slot of SCS configuration of PCell.

Overbooking

Since overbooking is not allowed on USS set(s) of SCell, that is to say, to any slot of  PCell, UE will drop some PDCCH candidates on USS set(s) of PCell according to the dropping rules to guarantee x1(m)+x2(m) is less than or equal to BD limit. After the dropping handling of PCell, the BD limit of x1(m)+x2(m)+y(m) shall be guaranteed based on PDCCH candidates configuration of PCell and SCell.

Case 2: different SCSs between PCell and SCell

BD/CCE limit handling

When the SCSs of PCell and SCell are different, as observed that the UE implementation is differentiated per each SCS, separate BDFactorR values for respective PCell and SCell should be considered. Slightly different from the case of same SCS between PCell and SCell where a single BD factor is used to scale the BD limit for one TRP with same SCS as another TRP, the two BDFactorR values for different SCS case will be used in distributing of total BD/CCE capability among serving cells per SCS and determining the maximum BD/CCE limit, which provides the network with more flexibility to control the BD for each cell while still ensures the total BD under UE capability given each of the BDFactorR value no larger than 1.

For simplicity, we still explain the BD limit handling based on slot limits for the cells without multi-TRP operation as baseline and, γμ1 and γμ2 denote the separate BDFactorR values for PCell with SCS configuration  μ1 and SCell with SCS configuration μ2 respectively. As explained above, the value of γμ may not exceed 1 such that the BD limit for each of the two scheduling cell is a fraction of the original UE BD limit for one cell. If advanced UE capability is enabled by a UE, different R can be considered.

- If μ=03Ncells,0DL,μ+γμ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor PDCCH candidates more than the below BD limits per slot for each scheduled cell, on the active DL BWPs of the scheduling cells

-	MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ when the scheduling cell is from the Ncells,0DL,μ cells, or

-	MPDCCHtotal,slot,μ=γμ∙MPDCCHmax,slot,μ for each of the two cells scheduling the PCell

- If μ=03Ncells,0DL,μ+γμ∙Ncells,1DL,μ>Ncellscap

- the UE is not required to monitor PDCCH candidates more than the below BD limits per slot, on the active DL BWPs of scheduling cell(s)

-	MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γμ∙Ncells,1DL,μj=03Ncells,0DL,j+γj∙Ncells,1DL,j

- and for each scheduled cell,

-	minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ when the scheduling cell is from the Ncells,0DL,μ cells, or

-	minγμ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ for each of the two cells scheduling the PCell

- Ncells,1DL,μ=1 for the above for the cells scheduling PCell with SCS configuration of μ1 or μ2, otherwise  Ncells,1DL,μ=0.

The above BD/CCE limit handling is actually Option C:

- On every PCell slot m1, x1(m1)+x2(m1) is less than or equal to γμ1∙MPDCCHmax,slot,μ1 , where γμ1 is configured for the PCell with SCS configuration  μ1

- On every SCell slot m2, y(m2) is less than or equal to γμ2∙MPDCCHmax,slot,μ2 , where γμ2 is configured for the SCell with SCS configuration  μ2

Overbooking

In this case, since separate BDFactorR values are configured to determine separate BD/CCE limits for PCell and sSCell respectively, it is rather easier for network to control the BD limit on PCell only as existing has done.

In summary,

Proposal 3: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of an SCell and the PCell, for determining the UE BD/CCE capability for Type B UE:

-	if the SCS of the two cells are the same, a single BDFactorR is applied to both cells based on Option B

-	if the SCSs of two cells are different, separate BDFactorR values should be configured respectively for the PCell and the sSCell based on Option C

-	PDCCH overbooking is maintained only for USS set(s) on PCell.

PDCCH monitoring handling when sSCell is deactivated

At the RAN1 #105-e meeting, it is agreed to support CA activation/deactivation operation for the sSCell. In order to offload PDCCH from PCell to sSCell, USS sets for DCI formats 0_1, 1_1, 0_2, 1_2 may be configured so that the UE only monitors them on sSCell. In this case, once sSCell is deactivated, there is only fallback DCI scheduling PDSCH/PUSCH on PCell, which degrades the scheduling efficiency. Therefore, in this case, UE can be configured additional USS sets for DCI formats 0_1, 1_1, 0_2, 1_2 on PCell, once sSCell is deactivated, the additional USS sets on PCell are activated.

Proposal 4: Additional USS sets for DCI formats 0_1, 1_1, 0_2, 1_2 on PCell are activated when sSCell is deactivated in the case that the UE only monitors DCI formats 0_1, 1_1, 0_2, 1_2 on sSCell.

Conclusion

According to the above discussions, we have the following observation and proposals:

Observation 1: The BD/CCE capability is related to the number of sources that PDCCH can be transmitted from as scaled from one cell and the SCS of the scheduling PDCCH(s).

Proposal 1: For Type A UE, Type-0/0A/1/2-CSS sets on PCell and USS sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of PCell and sSCell.

Proposal 2: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of an SCell and the PCell, for determining the UE BD/CCE capability for Type A UE:

-	in a slot, if the PDCCH candidates are configured on PCell, the BD/CCE limit of this slot is determined based on the PCell parameters

-	in a slot, if the PDCCH candidates are configured on sSCell, the BD/CCE limit of this slot is determined based on the sSCell parameters

-	PDCCH overbooking is maintained only for USS set(s) in the slot where the PDCCH candidates are configured on PCell.

Proposal 3: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of an SCell and the PCell, for determining the UE BD/CCE capability for Type B UE:

-	if the SCS of the two cells are the same, a single BDFactorR is applied to both cells based on Option B

-	if the SCSs of two cells are different, separate BDFactorR values should be configured respectively for the PCell and the SCell based on Option C

-	PDCCH overbooking is maintained only for USS set(s) on PCell.

Proposal 4: Additional USS sets for DCI formats 0_1, 1_1, 0_2, 1_2 on PCell are activated when sSCell is deactivated in the case that the UE only monitors DCI formats 0_1, 1_1, 0_2, 1_2 on sSCell.

References

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”

Annex: RAN1 agreements on cross carrier scheduling from SCell to PCell

WID[1]:

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

RAN1 #102-e:

Agreements: - Following scheduling combinations are allowed/not allowed when cross-carrier scheduling from an SCell to PCell/PSCell is configured - self-scheduling on PCell/PSCell is allowed - cross-carrier scheduling from PCell/PSCell to another SCell is not allowed - self-scheduling on the ‘SCell used for scheduling PCell/PSCell’ is allowed - cross-carrier scheduling from the ‘SCell used for scheduling PCell/PSCell’ to another serving cell is allowed - cross-carrier scheduling from another serving cell to the ‘SCell used for scheduling PCell/PSCell’ is not allowed - FFS: Search space and DCI format handling for the allowed cases above Agreement: - Configuring 2 or more Scells to schedule the PCell/PSCell is not allowed.

RAN1 #103-e:

Agreements: - When CCS from an SCell (sSCell) to PCell/PSCell is configured, UE monitors Type 0/0A/1/2 CSS sets (for the DCI formats associated with those SS sets) only on the PCell/PSCell and not on the sSCell - Note: UE monitors Type 0/0A/2 CSS only on PCell while Type 1 CSS can be monitored on PCell/PSCell Agreements: - Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded) - Alt 1: UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s) - Alt 2: UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded) - Alt 2-1: - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-2: - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported - FFS: Details of switching mechanism - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-3: - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously - Alt 2-4: - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously - FFS following aspects - Impact of sSCell activation/deactivation and sSCell dormancy - Impact on BD/CCE limit handling - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a USS set configured for scheduling of PCell/PSCell)

RAN1 #104-e:

Agreement When CCS from sSCell to PCell/PSCell is configured, - Out of order scheduling is not allowed between a) PDSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PDSCH on PCell/PSCell scheduled by PDCCH on sSCell - Out of order scheduling is not allowed between a) PUSCH on PCell/PSCell scheduled by PDCCH on PCell/PSCell and b) PUSCH on PCell/PSCell scheduled by PDCCH on sSCell FFS: Whether this agreement requires RAN1 specification impact. Agreement When CCS from sSCell to PCell/PSCell is configured, - Simultaneous reception of a) unicast PDSCH on PCell/PSCell scheduled from PCell/PSCell and b) unicast PDSCH on PCell/PSCell scheduled from sSCell is not allowed - Simultaneous transmission of a) PUSCH on PCell/PSCell scheduled from PCell/PSCell and b) PUSCH on PCell/PSCell scheduled from sSCell is not allowed - Note: Simultaneous implies full/partial time overlapping FFS: Whether this agreement requires RAN1 specification impact. Agreement - When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported Working Assumption - When CCS from sSCell to PCell/PSCell is configured, UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 that schedule PDSCH/PUSCH on PCell/PSCell on PCell/PSCell USS set(s), and/or on sSCell USS set(s) - The WA to be confirmed after agreements are made on PDCCH BD/CCE handling and PDCCH overbooking handling for CCS from sSCell to PCell/PSCell - Specs also allow UEs supporting functionality of only Alt-1. Capability signaling details, if any, can be handled during the UE capability discussion for Rel17 - FFS: Whether the UE can monitor PDCCH from both cells in the same slot. Agreement - When CCS from sSCell to PCell/PSCell is configured, UE monitors ‘DCI formats 0_0 and 1_0 in CSS that schedule PDSCH/PUSCH on PCell/PSCell’ only on the PCell/PSCell and not on the sSCell

RAN1 #104b-e:

Agreement When CCS from sSCell to PCell/PSCell is configured, - CIF=0 used for sSCell self-scheduling, and CIF for sSCell to PCell cross-carrier scheduling is explicitly configured using RRC signalling Agreement PDCCH overbooking on sSCell USS set(s) is not allowed Agreement - When CCS from sSCell to PCell/PSCell is configured, CA activation/deactivation operation for the sSCell is supported For RAN1#105-e, companies are encouraged to consider: Further discuss PDCCH monitoring and BD/CCE limit handling in RAN1#105e considering below BD/CCE limit handling options - Option A - At least when P(S)Cell SCS is not higher than sSCell SCS, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that max of (x1(m1)+x2(m1))+max of y(m2) corresponding to any P(S)Cell slots m1 and m2 is less than or equal to Z1 - At least the case of Z1 = 44 is supported for P(S)Cell SCS 15kHz - FFS if Z1 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16) - FFS signalling details on how the limit Z1 is realized, e.g. - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m)) - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - separate BdfactorR for P(S)Cell and sSCell - SS configuration-based BD limit for max(x1(m)+x2(m)) and max(y(m)) - RRC configured BD limit/scaling factor-based limit for max(x1(m)+x2(m))+ max(y(m)) - Counting ‘sSCell-to-P(S)Cell’ scheduling as an additional scheduling cell with numerology given by sSCell numerology in determining the BD/CCE limits - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration µμ of the sSCell more than MPDCCHmax,slot,μ PDCCH candidates per slot of sSCell. - FFS how limit MPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured - Option B - At least when P(S)Cell SCS is not higher than sSCell SCS, For P(S)Cell slot m, PDCCH monitoring candidates on P(S)Cell and/or sSCell are configured such that x1(m)+x2(m)+y(m) is less than or equal to BD limit Z2 - At least the case of Z2 = 44 is supported for P(S)Cell SCS 15kHz - FFS if Z2 larger than above can also be supported based on UE capability (e.g. similar to BDFactorR in Rel16) - max of (x1(m1)+x2(m1)) + max of y(m2) corresponding to any P(S)Cell slots m1 and m2 can is allowed to be larger than BD limit Z2 - FFS signalling details on how the limit Z2 is realized - FFS reference SCS to use when P(S)Cell has higher SCS than sSCell (if supported) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration µ of the sSCell more than MPDCCHmax,slot,μ PDCCH candidates per slot of sSCell. - FFS how limit MPDCCHtotal,slot,μ is computed and applied when CCS from sSCell to P(S)Cell is configured - Option C - PDCCH monitoring candidates on P(S)Cell are configured such that max of (x1(m1)+x2(m1)) is less than or equal to Z3 - Z3 is derived by the PDCCH monitoring capability of PCell - PDCCH monitoring candidates on sSCell are configured such that max of y(m2) is less than or equal to Z4 - Z4 is derived by the PDCCH monitoring capability of sSCell - FFS details to define Z3 and Z4, e.g. - Separate RRC configured BD limits/scaling factor-based limits for max(x1(m)+x2(m)) and max(y(m)) - For sSCell scheduling P(S)Cell, the UE is not required to monitor on the active DL BWP with SCS configuration µ of the sSCell more than Z4 PDCCH candidates per slot of sSCell - Note - x1(m) is #BDs for PDCCH CSS(s) candidates monitored on P(S)Cell slot m - x2(m) is #BDs for PDCCH USS(s) candidates monitored on P(S)Cell slot m - y(m) is #BDs for PDCCH USS(s) candidates monitored on sSCell in all sSCell slot(s) that overlap slot m of P(S)Cell - USS(s) => USS(s) that can schedule PDSCH/PUSCH on P(S)Cell)

RAN1 #105-e:

Agreement Two types of UEs (Type A and Type B) can support CCS from sSCell to P(S)Cell - For Type A UE - At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported for Type A UE) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - FFS: BD/CCE handling - For Type B UE - Following search space sets on P(S)Cell and search space sets on sSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - For handling ‘USS sets for scheduling P(S)Cell’ on P(S)Cell and/or on sSCell for DCI formats 0_1,1_1,0_2,1_2 - Alt 2-1 is adopted - There is no restriction on Type-0/0A/1/2-CSS sets configurations - FFS: BD/CCE handling - For Type A and/or Type B UE - FFS: switching to ‘normal’ PDCCH monitoring on P(S)Cell when sSCell is deactivated - FFS: Whether Type A is specified or is Type-B with restrictions (as part of UE features discussion) - FFS: Whether the UE can be configured with unaligned CA - FFS: Whether the above applies for multicast PDSCH